Output Voltage: 5 V, 3.8 V, 3.3 V, 3.0 V, 2.8 V,

(TOP VIEW)

2.7 V, 2.5 V, 1.8 V, 1.6 V and Variable

Dropout Voltage, Typically 300 mV

at 150 mA

Over Current Limitation

Less Than 2-A Quiescent Current in

Shutdown Mode

−40C to 125C Operating Junction

Temperature Range

5-Pin SOT-23 (DBV) Package

description

IN	1	5

GND	2

EN	3	4

OUT

NC/FB

The TPS763xx family of low-dropout (LDO) voltage regulators offers the benefits of low-dropout voltage, low-power operation, and miniaturized packaging. These regulators feature low dropout voltages and quiescent currents compared to conventional LDO regulators. Offered in a 5-terminal, small outline integrated-circuit SOT-23 package, the TPS763xx series devices are ideal for cost-sensitive designs and for applications where board space is at a premium.

A combination of new circuit design and process innovation has enabled the usual pnp pass transistor to be replaced by a PMOS pass element. Because the PMOS pass element behaves as a low-value resistor, the dropout voltage is very low—typically 300 mV at 150 mA of load current (TPS76333)—and is directly proportional to the load current. Since the PMOS pass element is a voltage-driven device, the quiescent current is very low (140 A maximum) and is stable over the entire range of output load current (0 mA to 150 mA). Intended for use in portable systems such as laptops and cellular phones, the low-dropout voltage feature and low-power operation result in a significant increase in system battery operating life.

The TPS763xx also features a logic-enabled sleep mode to shut down the regulator, reducing quiescent current to 1 A maximum at TJ = 25C.The TPS763xx is offered in 1.6-V,1.8-V, 2.5-V, 2.7-V, 2.8-V, 3.0-V, 3.3-V, 3.8-V, and 5-V fixed-voltage versions and in a variable version (programmable over the range of 1.5 V to 6.5 V.

AVAILABLE OPTIONS

| TJ | VOLTAGE | PACKAGE | PART NUMBER |  | SYMBOL |
| --- | --- | --- | --- | --- | --- |
| −40C to 125C | Variable | SOT-23 (DBV) | TPS76301DBVT(1) | TPS76301DBVR(2) | PAZI |
|  | 1.6 V |  | TPS76316DBVT | TPS76316DBVR | PBHI |
|  | 1.8 V |  | TPS76318DBVT | TPS76318DBVR | PBAI |
|  | 2.5 V |  | TPS76325DBVT | TPS76325DBVR | PBBI |
|  | 2.7 V |  | TPS76327DBVT | TPS76327DBVR | PBCI |
|  | 2.8 V |  | TPS76328DBVT | TPS76328DBVR | PBDI |
|  | 3.0 V |  | TPS76330DBVT | TPS76330DBVR | PBII |
|  | 3.3 V |  | TPS76333DBVT | TPS76333DBVR | PBEI |
|  | 3.8 V |  | TPS76338DBVT | TPS76338DBVR | PBFI |
|  | 5.0 V |  | TPS76350DBVT | TPS76350DBVR | PBGI |

(1) The DBVT passive indicates tape and reel of 250 parts. (2) The DBVR passive indicates tape and reel of 3000 parts.

Please be aware that an important notice concerning availability, standard warranty, and use in critical applications of

Texas Instruments semiconductor products and disclaimers thereto appears at the end of this data sheet.

All trademarks are the property of their respective owners.

!"    # $% & " !#       '%() $!"     *!"&+      *%$"#

$      "  #'&$   $!"   # '&  ",& "&  #      &-!#  #" % & "# #"! *! * .!  ! "/+

Copyright  2001 − 2004 Texas Instruments Incorporated

FUNCTIONAL BLOCK DIAGRAM

TPS76301

IN EN

GND

VREF

Current Limit/ Thermal Protection

OUT

FB

TPS76316/ 18/ 25/ 27/ 28/ 30/ 33/ 38/ 50

IN EN

VREF

Current Limit/ Thermal Protection

OUT

GND

Terminal Functions

| TERMINAL NAME | DESCRIPTION |
| --- | --- |
| GND | Ground |
| EN | Enable input |
| FB | Feedback voltage (TPS76301 only) |
| IN | Input supply voltage |
| NC | No connection (fixed-voltage option only) |
| OUT | Regulated output voltage |

absolute maximum ratings over operating free-air temperature range (unless otherwise noted)1

Input voltage range(2)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

− 0.3 V to 10 V

Voltage range at EN

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . −0.3 V to VI + 0.3 V

Voltage on OUT, FB . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7 V

Peak output current

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

Internally limited

ESD rating, HBM

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2 kV

Continuous total power dissipation

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

See Dissipation Rating Tables

Operating junction temperature range, TJ

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . −40C to 150C

Storage temperature range, Tstg

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . −65C to 150C

(1) Stresses beyond those listed under absolute maximum ratings may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated under recommended operating conditions is not

implied. Exposure to absolute-maximum-rated conditions for extended periods may affect device reliability. (2) All voltage values are with respect to network ground terminal.

DISSIPATION RATING TABLE

BOARD	PACKAGE	RJC	RJA

DERATING FACTOR ABOVE TA = 25C

TA  25C POWER RATING

TA = 70C POWER RATING

TA = 85C POWER RATING

Low K(1)	DBV	65.8 C/W	259 C/W	3.9 mW/C	386 mW	212 mW	154 mW High K(2)	DBV	65.8 C/W	180 C/W	5.6 mW/C	555 mW	305 mW	222 mW

(1) The JEDEC Low K (1s) board design used to derive this data was a 3 inch x 3 inch, two layer board with 2 ounce copper traces on top of the

board.

(2) The JEDEC High K (2s2p) board design used to derive this data was a 3 inch x 3 inch, multilayer board with 1 ounce internal power and ground planes and 2 ounce copper traces on top and bottom of the board.

recommended operating conditions

|  | MIN NOM MAX | UNIT |
| --- | --- | --- |
| Input voltage, VI(1) | 2.7 10 | V |
| Continuous output current, IO | 0 150 | mA |
| Operating junction temperature, TJ | −40 125 | C |

(1) To calculate the minimum input voltage for your maximum output current, use the following equation:

VI(min) = VO(max) + VDO(max load)

| electrical characteristics over recommended operating free-air temperature range, VI = VO(typ) + 1 V, IO = 1 mA, EN = IN, Co = 4.7 F (unless otherwise noted) PARAMETER |  |  | TEST CONDITIONS | MIN TYP MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| VO | Output voltage | TPS76301 | 3.25 V > VI ≥ 2.7 V, IO = 1 mA to 75 mA, 2.5 V ≥ VO ≥ 1.5 V, TJ = 25C | 0.98 VO VO 1.02 VO | V |
|  |  |  | 3.25 V > VI ≥ 2.7 V, IO = 1 mA to 75 mA, 2.5 V ≥ VO ≥ 1.5 V | 0.97 VO VO 1.03 VO |  |
|  |  |  | VI ≥ 3.25 V, IO = 1 mA to 100 mA, 5 V ≥ VO ≥ 1.5 V TJ = 25C | 0.98 VO VO 1.02 VO |  |
|  |  |  | VI ≥ 3.25 V, IO = 1 mA to 100 mA, 5 V ≥ VO ≥ 1.5 V | 0.97 VO VO 1.03 VO |  |
|  |  |  | VI ≥ 3.25 V, IO = 1 mA to 150 mA, 5 V ≥ VO ≥ 1.5 V TJ = 25C | 0.975 VO VO 1.025 VO |  |
|  |  |  | VI ≥ 3.25 V, IO = 1 mA to 150 mA, 5 V ≥ VO ≥ 1.5 V | 0.9625 VO VO 1.0375 VO |  |
|  |  | TPS76316 | 1 mA< IO < 75 mA, | 1.568 1.6 1.632 | V |
|  |  |  | VI = 2.7 V, 1 mA< IO < 75 mA | 1.552 1.6 1.648 |  |
|  |  |  | 1 mA < IO < 100 mA, | 1.568 1.6 1.632 |  |
|  |  |  | VI = 3.25 V, 1 mA < IO < 100 mA | 1.552 1.6 1.648 |  |
|  |  |  | 1 mA < IO < 150 mA, | 1.560 1.6 1.640 |  |
|  |  |  | VI = 3.25 V, 1 mA < IO < 150 mA | 1.536 1.6 1.664 |  |
|  |  | TPS76318 | 1 mA< IO < 75 mA, | 1.764 1.8 1.836 | V |
|  |  |  | VI = 2.7 V, 1 mA< IO < 75 mA | 1.746 1.8 1.854 |  |
|  |  |  | 1 mA < IO < 100 mA, | 1.764 1.8 1.836 |  |
|  |  |  | VI = 3.25 V, 1 mA < IO < 100 mA | 1.746 1.8 1.854 |  |
|  |  |  | 1 mA < IO < 150 mA, | 1.755 1.8 1.845 |  |
|  |  |  | VI = 3.25 V, 1 mA < IO < 150 mA | 1.733 1.8 1.867 |  |
|  |  | TPS76325 | IO = 1 mA to 100 mA, TJ = 25C | 2.45 2.5 2.55 | V |
|  |  |  | IO = 1 mA to 100 mA | 2.425 2.5 2.575 |  |
|  |  |  | IO = 1 mA to 150 mA, TJ = 25C | 2.438 2.5 2.562 |  |
|  |  |  | IO = 1 mA to 150 mA | 2.407 2.5 2.593 |  |
|  |  | TPS76327 | IO = 1 mA to 100 mA, TJ = 25C | 2.646 2.7 2.754 | V |
|  |  |  | IO = 1 mA to 100 mA | 2.619 2.7 2.781 |  |
|  |  |  | IO = 1 mA to 150 mA, TJ = 25C | 2.632 2.7 2.767 |  |
|  |  |  | IO = 1 mA to 150 mA | 2.599 2.7 2.801 |  |
|  |  | TPS76328 | IO = 1 mA to 100 mA, TJ = 25C | 2.744 2.8 2.856 | V |
|  |  |  | IO = 1 mA to 100 mA | 2.716 2.8 2.884 |  |
|  |  |  | IO = 1 mA to 150 mA, TJ = 25C | 2.73 2.8 2.87 |  |
|  |  |  | IO = 1 mA to 150 mA | 2.695 2.8 2.905 |  |
|  |  | TPS76330 | IO = 1 mA to 100 mA, TJ = 25C | 2.94 3.0 3.06 | V |
|  |  |  | IO = 1 mA to 100 mA | 2.91 3.0 3.09 |  |
|  |  |  | IO = 1 mA to 150 mA, TJ = 25C | 2.925 3.0 3.075 |  |
|  |  |  | IO = 1 mA to 150 mA | 2.888 3.0 3.112 |  |

VI = 2.7 V,

TJ = 25C

VI = 3.25 V,

TJ = 25C

VI = 3.25 V,

TJ = 25C

VI = 2.7 V,

TJ = 25C

VI = 3.25 V,

TJ = 25C

VI = 3.25 V,

TJ = 25C

electrical characteristics over recommended operating free-air temperature range,

| VI = VO(typ) + 1 V, IO = 1 mA, EN = IN, Co = 4.7 F (unless otherwise noted) (continued) PARAMETER |  |  | TEST CONDITIONS | MIN TYP MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| VO | Output voltage | TPS76333 | IO = 1 mA to 100 mA, TJ = 25C | 3.234 3.3 3.366 | V |
|  |  |  | IO = 1 mA to 100 mA | 3.201 3.3 3.399 |  |
|  |  |  | IO = 1 mA to 150 mA, TJ = 25C | 3.218 3.3 3.382 |  |
|  |  |  | IO = 1 mA to 150 mA | 3.177 3.3 3.423 |  |
|  |  | TPS76338 | IO = 1 mA to 100 mA, TJ = 25C | 3.724 3.8 3.876 | V |
|  |  |  | IO = 1 mA to 100 mA | 3.705 3.8 3.895 |  |
|  |  |  | IO = 1 mA to 150 mA, TJ = 25C | 3.686 3.8 3.914 |  |
|  |  |  | IO = 1 mA to 150 mA | 3.667 3.8 3.933 |  |
|  |  | TPS76350 | IO = 1 mA to 100 mA, TJ = 25C | 4.875 5 5.125 | V |
|  |  |  | IO = 1 mA to 100 mA | 4.825 5 5.175 |  |
|  |  |  | IO = 1 mA to 150 mA, TJ = 25C | 4.750 5 5.15 |  |
|  |  |  | IO = 1 mA to 150 mA | 4.80 5 5.20 |  |
| Quiescent current I(Q) (GND terminal current) |  |  | IO = 0 to 150 mA, TJ = 25C (1) | 85 100 | A |
|  |  |  | IO = 0 to 150 mA (2) | 140 |  |
| Standby current |  |  | EN < 0.5 V, TJ = 25C | 0.5 1 |  |
|  |  |  | EN < 0.5 V | 2 |  |
| Vn Output noise voltage |  |  | BW = 300 Hz to 50 kHz, TJ = 25C, Co = 10 F (2) | 140 | V |
| PSRR Ripple rejection |  |  | f = 1 kHz, Co = 10 F, TJ = 25C (2) | 60 | dB |
| Current limit |  |  | TJ = 25C (3) | 0.5 0.8 1.5 | A |
| Output voltage line regulation (∆VO/VO) (see Note 3) |  |  | VO + 1 V < VI ≤ 10 V, VI ≥ 3.5 V, TJ = 25C | 0.04 0.07 | %/V |
|  |  |  | VO + 1 V < VI ≤ 10 V, VI ≥ 3.5 V | 0.1 |  |
| VIH EN high level input |  |  | (2) | 1.4 2 | V |
| VIL EN low level input |  |  | (2) | 0.5 1.2 |  |
| II EN input current |  |  | EN = 0 V | −0.01 −0.5 | A |
|  |  |  | EN = IN | −0.01 −0.5 |  |

(1) Minimum IN operating voltage is 2.7 V or VO(typ) + 1 V, whichever is greater.

(2) Test condition includes, output voltage VO=0 volts (for variable device FB is shorted to VO), and pulse duration = 10 mS. (3) If VO < 2.5 V and VImax = 10 V, VImin = 3.5 V:

Line Reg. (mV)      % V

VO VImax   3.5 V

100	  1000

If VO > 2.5 V and VImax = 10 V, VImin = VO + 1 V:

VO VImax    VO   1

Line Reg. (mV)      % V

100	  1000

electrical characteristics over recommended operating free-air temperature range,

| VI = VO(typ) + 1 V, IO = 1 mA, EN = IN, Co = 4.7 F (unless otherwise noted) (continued) PARAMETER |  | TEST CONDITIONS | MIN TYP MAX | UNIT |
| --- | --- | --- | --- | --- |
| VDO Dropout voltage | TPS76325 | IO = 0 mA, TJ = 25C | 0.2 | mV |
|  |  | IO = 1 mA, TJ = 25C | 3 |  |
|  |  | IO = 50 mA, TJ = 25C | 120 150 |  |
|  |  | IO = 50 mA | 200 |  |
|  |  | IO = 75 mA, TJ = 25C | 180 225 |  |
|  |  | IO = 75 mA | 300 |  |
|  |  | IO = 100 mA, TJ = 25C | 240 300 |  |
|  |  | IO = 100 mA | 400 |  |
|  |  | IO = 150 mA, TJ = 25C | 360 450 |  |
|  |  | IO = 150 mA | 600 |  |
|  | TPS76333 | IO = 0 mA, TJ = 25C | 0.2 | mV |
|  |  | IO = 1 mA, TJ = 25C | 3 |  |
|  |  | IO = 50 mA, TJ = 25C | 100 125 |  |
|  |  | IO = 50 mA | 166 |  |
|  |  | IO = 75 mA, TJ = 25C | 150 188 |  |
|  |  | IO = 75 mA | 250 |  |
|  |  | IO = 100 mA, TJ = 25C | 200 250 |  |
|  |  | IO = 100 mA | 333 |  |
|  |  | IO = 150 mA, TJ = 25C | 300 375 |  |
|  |  | IO = 150 mA | 500 |  |
|  | TPS76350 | IO = 0 mA, TJ = 25C | 0.2 | mV |
|  |  | IO = 1 mA, TJ = 25C | 2 |  |
|  |  | IO = 50 mA, TJ = 25C | 60 75 |  |
|  |  | IO = 50 mA | 100 |  |
|  |  | IO = 75 mA, TJ = 25C | 90 113 |  |
|  |  | IO = 75 mA | 150 |  |
|  |  | IO = 100 mA, TJ = 25C | 120 150 |  |
|  |  | IO = 100 mA | 200 |  |
|  |  | IO = 150 mA, TJ = 25C | 180 225 |  |
|  |  | IO = 150 mA | 300 |  |

TYPICAL CHARACTERISTICS

2.505

2.5

TPS76325

OUTPUT VOLTAGE

vs

VI = 3.5 V

CI = CO = 4.7 F

TJ = 25C

1.805

1.800

TPS76318

OUTPUT VOLTAGE

vs

2.495

2.49

2.485

1.795

1.790

1.785

VO− Output Voltage − V

1.780

2.48

| 1.775 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VI = 3.5 V CI = CO = 4.7 F TJ = 25C |  |  |  |

2.475

0

30	60

90	120

150

180

1.770

0

30	60

90	120

150

180

IO − Output Current − mA

Figure 1

IO − Output Current − mA

Figure 2

5.01

5

TPS76350

OUTPUT VOLTAGE

vs

OUTPUT CURRENT

2.53

2.52

TPS76325

OUTPUT VOLTAGE

vs

FREE-AIR TEMPERATURE

VI = 3.5 V

CI = CO = 4.7 F

4.99

4.98

VO− Output Voltage − V

2.51

2.5

IO = 1 mA

4.97

2.49

IO = 150 mA

4.96

2.48

4.95

| 0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VI = 6 V CI = CO = 4.7 F TJ = 25C |  |  |  |

30	60

90	120

150

180

2.47

−55

−35

−15

5	25

45	65

85	105

125

IO − Output Current − mA

TJ − Junction Temperature − C

VO− Output Voltage − V

VO− Output Voltage − V

Figure 3

Figure 4

TYPICAL CHARACTERISTICS

1.82

1.81

1.8

1.79

1.78

1.77

1.76

1.75

TPS76318

OUTPUT VOLTAGE

vs

FREE-AIR TEMPERATURE

IO = 1 mA

IO = 150 mA

VI = 3.5 V

CI = CO = 4.7 F

5.1

5.08

5.06

VO− Output Voltage − V

5.04

5.02

5

4.98

4.96

4.94

4.92

TPS76350

OUTPUT VOLTAGE

vs

FREE-AIR TEMPERATURE

VI = 6 V

CI = CO = 4.7 F

IO = 1 mA

IO = 150 mA

1.74

−55

−35

−15

5	25

45	65	85	105

125

4.9

−55

−35

−15

5	25

45	65

85	105

125

TJ − Junction Temperature − C

Figure 5

TJ − Junction Temperature − C

Figure 6

1000

TPS76350

GROUND CURRENT

vs

FREE-AIR TEMPERATURE

VI = 6 V

CI = CO = 4.7 F

IO = 0 mA and 150 mA

3  V  Hz

2.5  V  Hz

OUTPUT NOISE vs FREQUENCY

CO = 10 F IO = 150 mA

TJ = 25C

100

2  V  Hz

1.5  V  Hz

CO = 4.7 F IO = 150 mA

10

−55

−35

−15

5	25

45	65

85	105

125

1  V  Hz

0.5  V  Hz

0  V  Hz

250

CO = 4.7 F IO = 1 mA

CO = 10 F IO = 1 mA

1k	10k	100k

Ground Current −  A

VO− Output Voltage − V

TJ − Junction Temperature − C

Figure 7

f − Frequency − Hz

Figure 8

TYPICAL CHARACTERISTICS

OUTPUT IMPEDANCE

vs

FREQUENCY

10

600

500

TPS76325

DROPOUT VOLTAGE

vs

FREE-AIR TEMPERATURE

VI = EN = 2.7 V CI = CO = 4.7 F

VDO − Dropout Voltage − mV

400

150 mA

IO = 1 mA

1

300

0.1

0.01

IO = 150 mA CI = CO = 4.7 F

ESR = 1 

TJ = 25C

0.1	1

10	100

1000

200

100

0

−55

−35

−15

0 mA

5	25

1 mA

45	65

85	105

125

f − Frequency − kHz

TJ − Junction Temperature − C

Figure 9

Figure 10

TPS76325

RIPPLE REJECTION

vs

FREQUENCY

70

60	IO = 1 mA

Ripple Rejection − dB

50

40	IO = 150 mA

30

20

10

0

−10

10

CO = 4.7 F ESR = 1  TJ = 25C

100	1 k

10 k

100 k

1 M	10 M

Zo − Output Impedance − 

f − Frequency − Hz

Figure 11

TYPICAL CHARACTERISTICS

TPS76318

LINE TRANSIENT RESPONSE

5

200

TPS76318

LOAD TRANSIENT RESPONSE

I O − Output Current − mA

4	100

3

2

CO = 4.7 F

1	ESR = 0.25 

TJ = 25C

20

0

dv	  1V

0

 VO − Change in

Output Voltage − mV

50

0

−50

CO = 4.7 F ESR = 0.25  TJ = 25C

−20

dt        10   s

−100

−30

0

20	40

60   80	100

120

140 160   180  200

−150

0

20	40

60   80	100

120

140 160   180  200

t − Time − s

t − Time − s

Figure 12

Figure 13

TPS76350

LINE TRANSIENT RESPONSE

8

200

TPS76350

LOAD TRANSIENT RESPONSE

CO = 4.7 F ESR = 0.25 

7	dv

1V

100

dt        10   s

6

I O − Output Current − mA

0

TJ = 25C

5

CO = 4.7 F

ESR = 0.25 

50	TJ = 25C

0

 VO − Change in

Output Voltage − mV

150

100

0

−50

−100

−100

0

50    100

150 200   250

300

350 400   450  500

−200

0

20	40

60   80	100

120

140 160   180  200

t − Time − s

t − Time − s

 VO − Change in

Output Voltage − mV

 VO − Change in

Output Voltage − mV

VI − Input Voltage − V

VI − Input Voltage − V

Figure 14

Figure 15

TYPICAL CHARACTERISTICS

TYPICAL REGIONS OF STABILITY COMPENSATION SERIES RESISTANCE (CSR)(1) vs

OUTPUT CURRENT

100

TYPICAL REGIONS OF STABILITY COMPENSATION SERIES RESISTANCE (CSR)(1) vs

ADDED CERAMIC CAPACITANCE

100

Region of Instability

Region of Instability

CSR − Compensation Series Resistance − 

10	10

CO = 4.7 F

1	TJ = 25C

I = 150 mA

1	CO = 4.7 F TJ = 25C

CSR − Compensation Series Resistance − 

0.1

Region of Instability

0.1

Region of Instability

0.01

0	50	100	150	200	250

0.01

0	0.1   0.2   0.3   0.4   0.5

0.6   0.7   0.8   0.9	1

IO − Output Current − mA

Added Ceramic Capacitance − F

Figure 16

Figure 17

TYPICAL REGIONS OF STABILITY COMPENSATION SERIES RESISTANCE (CSR)(1) vs

OUTPUT CURRENT

100

TYPICAL REGIONS OF STABILITY COMPENSATION SERIES RESISTANCE (CSR)(1) vs

ADDED CERAMIC CAPACITANCE

100

Region of Instability

10

Region of Instability

CSR − Compensation Series Resistance − 

10

CO = 10 F

1

1	CO = 10 F

CSR − Compensation Series Resistance − 

0.1

Region of Instability

0.1

Region of Instability

0.01

0	50	100	150	200	250

0.01

0	0.1   0.2   0.3   0.4   0.5

0.6   0.7   0.8   0.9	1

IO − Output Current − mA

Added Ceramic Capacitance − F

Figure 18

Figure 19

(1) CSR refers to the total series resistance, including the ESR of the capacitor, any series resistance added externally, and PWB trace resistance to CO.

APPLICATION INFORMATION

The TPS763xx low-dropout (LDO) regulators are new families of regulators which have been optimized for use in battery-operated equipment and feature extremely low dropout voltages, low quiescent current (140 A), and an enable input to reduce supply currents to less than 2 A when the regulator is turned off.

device operation

The TPS763xx uses a PMOS pass element to dramatically reduce both dropout voltage and supply current over more conventional PNP pass element LDO designs. The PMOS pass element is a voltage-controlled device that, unlike a PNP transistor, does not require increased drive current as output current increases. Supply current in the TPS763xx is essentially constant from no-load to maximum load.

Current limiting and thermal protection prevent damage by excessive output current and/or power dissipation. The device switches into a constant-current mode at approximately 1 A; further load reduces the output voltage instead of increasing the output current. The thermal protection shuts the regulator off if the junction temperature rises above 165C. Recovery is automatic when the junction temperature drops approximately 25C below the high temperature trip point. The PMOS pass element includes a back diode that safely conducts reverse current when the input voltage level drops below the output voltage level.

A logic low on the enable input, EN shuts off the output and reduces the supply current to less than 2 A. EN should be tied high in applications where the shutdown feature is not used.

A typical application circuit is shown in Figure 20.

TPS763xx(1)

1

VI	IN

NC/FB   4

C1

1 F

3

EN

GND

5

OUT

VO

+

4.7 F

2	CSR = 1 

(1) TPS76316, TPS76318, TPS76325, TPS76327, TPS76328,

TPS7630 TPS76333, TPS76338, TPS76350 (fixed-voltage options).

Figure 20. Typical Application Circuit

APPLICATION INFORMATION

external capacitor requirements

Although not required, a 0.047 F or larger ceramic bypass input capacitor, connected between IN and GND and located close to the TPS763xx, is recommended to improve transient response and noise rejection. A higher-value electrolytic input capacitor may be necessary if large, fast-rise-time load transients are anticipated and the device is located several inches from the power source.

Like all low dropout regulators, the TPS763xx requires an output capacitor connected between OUT and GND to stabilize the internal loop control. The minimum recommended capacitance value is 4.7 F and the ESR (equivalent series resistance) must be between 0.3  and 10 . Capacitor values 4.7 F or larger are acceptable, provided the ESR is less than 10 . Solid tantalum electrolytic, aluminum electrolytic, and multilayer ceramic capacitors are all suitable, provided they meet the requirements described above. Most of the commercially available 4.7 F surface-mount solid tantalum capacitors, including devices from Sprague, Kemet, and Nichico, meet the ESR requirements stated above.

CAPACITOR SELECTION

PART NO.	MFR.	VALUE	MAX ESR(1)	SIZE (H  L  W)†

T494B475K016AS	KEMET	4.7 F	 1.5 	1.9  3.5  2.8

195D106x0016x2T	SPRAGUE	10 F	 1.5 	1.3  7.0  2.7

695D106x003562T	SPRAGUE	10 F	 1.3 	2.5  7.6  2.5

TPSC475K035R0600	AVX	4.7 F	 0.6 	2.6  6.0  3.2

(1) Size is in mm. ESR is maximum resistance in ohms at 100 kHz and TA = 25C. Listings are sorted by height.

output voltage programming

The output voltage of the TPS76301 adjustable regulator is programmed using an external resistor divider as shown in

Figure 21. The output voltage is calculated using:

VO      0.995     Vref       1      R1 	(1)

R2

Where:

Vref = 1.192 V typ (the internal reference voltage)

0.995 is a constant used to center the load regulator (1%)

Resistors R1 and R2 should be chosen for approximately 7-A divider current. Lower value resistors can be used but offer no inherent advantage and waste more power. Higher values should be avoided as leakage currents at FB increase the output voltage error. The recommended design procedure is to choose R2 = 169 k to set the divider current at 7 A and then calculate R1 using:

R1

VO

0.995     Vref

1   R2	(2)

APPLICATION INFORMATION

OUTPUT VOLTAGE PROGRAMMING GUIDE

VI

1 F

 2 V

 0.5 V

TPS76301

1

IN

5

OUT

3

EN

4

FB

VO R1

+

4.7 F

| (1) 1% values shown. OUTPUT VOLTAGE (V) | DIVIDER RESISTANCE (kΩ)(1) |  |
| --- | --- | --- |
|  | R1 | R2 |
| 2.5 | 187 | 169 |
| 3.3 | 301 | 169 |
| 3.6 | 348 | 169 |
| 4 | 402 | 169 |
| 5 | 549 | 169 |
| 6.45 | 750 | 169 |

GND

2

R2

CSR = 1 

Figure 21. TPS76301 Adjustable LDO Regulator Programming

power dissipation and junction temperature

Specified regulator operation is assured to a junction temperature of 125C; the maximum junction temperature allowable to avoid damaging the device is 150C. This restriction limits the power dissipation the regulator can handle in any given application. To ensure the junction temperature is within acceptable limits, calculate the maximum allowable dissipation, PD(max), and the actual dissipation, PD, which must be less than or equal to PD(max).

The maximum-power-dissipation limit is determined using the following equation:

max     T

TJ PD(max)

Where:

A R  JA

TJmax is the maximum allowable junction temperature

RJA is the thermal resistance junction-to-ambient for the package, see the dissipation rating table. TA is the ambient temperature.

The regulator dissipation is calculated using:

PD       VI     VO    IO

Power dissipation resulting from quiescent current is negligible.

APPLICATION INFORMATION

regulator protection

The TPS763xx pass element has a built-in back diode that safely conducts reverse currents when the input voltage drops below the output voltage (e.g., during power down). Current is conducted from the output to the input and is not internally limited. If extended reverse voltage is anticipated, external limiting might be appropriate.

The TPS763xx also features internal current limiting and thermal protection. During normal operation, the TPS763xx limits output current to approximately 800 mA. When current limiting engages, the output voltage scales back linearly until the overcurrent condition ends. While current limiting is designed to prevent gross device failure, care should be taken not to exceed the power dissipation ratings of the package. If the temperature of the device exceeds 165C, thermal-protection circuitry shuts it down. Once the device has cooled down to below 140C, regulator operation resumes.

PACKAGING INFORMATION

Orderable Device Status (1) Package Package Pins Package Eco Plan (2) Lead/Ball Finish MSL Peak Temp (3) Type Drawing Qty

TPS76301DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76301DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76301DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76301DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76316DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76316DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76316DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76316DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76318DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76318DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76318DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76318DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76325DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76325DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76325DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76325DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76327DBV OBSOLETE SOT-23 DBV 5 TBD Call TI Call TI

TPS76327DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76327DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76327DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76327DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76328DBV OBSOLETE SOT-23 DBV 5 TBD Call TI Call TI

TPS76328DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76328DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76328DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76328DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM

PACKAGE OPTION ADDENDUM

www.ti.com	8-Aug-2005

Orderable Device Status (1) Package Package Pins Package Eco Plan (2) Lead/Ball Finish MSL Peak Temp (3) Type Drawing Qty

no Sb/Br)

TPS76330DBV OBSOLETE SOT-23 DBV 5 TBD Call TI Call TI

TPS76330DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76330DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76330DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76330DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76333DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76333DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76333DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76333DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76338DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76338DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76338DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76338DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76350DBVR ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76350DBVRG4 ACTIVE SOT-23 DBV 5 3000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76350DBVT ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

TPS76350DBVTG4 ACTIVE SOT-23 DBV 5 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

(1) The marketing status values are defined as follows:

ACTIVE: Product device recommended for new designs.

LIFEBUY: TI has announced that the device will be discontinued, and a lifetime-buy period is in effect.

NRND: Not recommended for new designs. Device is in production to support existing customers, but TI does not recommend using this part in a new design.

PREVIEW: Device has been announced but is not in production. Samples may or may not be available.

OBSOLETE: TI has discontinued the production of the device.

(2)    Eco   Plan   -   The   planned   eco-friendly   classification:   Pb-Free   (RoHS)   or   Green   (RoHS   &   no   Sb/Br)   -   please   check http://www.ti.com/productcontent for the latest availability information and additional product content details.

TBD: The Pb-Free/Green conversion plan has not been defined.

Pb-Free (RoHS): TI's terms "Lead-Free" or "Pb-Free" mean semiconductor products that are compatible with the current RoHS requirements for all 6 substances, including the requirement that lead not exceed 0.1% by weight in homogeneous materials. Where designed to be soldered at high temperatures, TI Pb-Free products are suitable for use in specified lead-free processes.

Green (RoHS & no Sb/Br): TI defines "Green" to mean Pb-Free (RoHS compatible), and free of Bromine (Br) and Antimony (Sb) based flame retardants (Br or Sb do not exceed 0.1% by weight in homogeneous material)

(3)  MSL, Peak Temp. -- The Moisture Sensitivity Level rating according to the JEDEC industry standard classifications, and peak solder temperature.

Addendum-Page 2

Important Information and Disclaimer:The information provided on this page represents TI's knowledge and belief as of the date that it is provided. TI bases its knowledge and belief on information provided by third parties, and makes no representation or warranty as to the accuracy of such information. Efforts are underway to better integrate information from third parties. TI has taken and continues to take reasonable steps to provide representative and accurate information but may not have conducted destructive testing or chemical analysis on incoming materials and chemicals. TI and TI suppliers consider certain information to be proprietary, and thus CAS numbers and other limited information may not be available for release.

In no event shall TI's liability arising out of such information exceed the total purchase price of the TI part(s) at issue in this document sold by TI

to Customer on an annual basis.

IMPORTANT NOTICE

Texas Instruments Incorporated and its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products and services at any time and to discontinue any product or service without notice. Customers should obtain the latest relevant information before placing orders and should verify that such information is current and complete. All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment.

TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI components. To minimize the risks associated with customer products and applications, customers should provide adequate design and operating safeguards.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products or services are used. Information published by TI regarding third-party products or services does not constitute a license from TI to use such products or services or a warranty or endorsement thereof. Use of such information may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

Reproduction of information in TI data books or data sheets is permissible only if reproduction is without alteration and is accompanied by all associated warranties, conditions, limitations, and notices. Reproduction of this information with alteration is an unfair and deceptive business practice. TI is not responsible or liable for such altered documentation.

Resale of TI products or services with statements different from or beyond the parameters stated by TI for that product or service voids all express and any implied warranties for the associated TI product or service and is an unfair and deceptive business practice. TI is not responsible or liable for any such statements.

Following are URLs where you can obtain information on other Texas Instruments products and application solutions:

| Products Amplifiers Data Converters | amplifier.ti.com dataconverter.ti.com | Applications Audio Automotive | www.ti.com/audio www.ti.com/automotive |
| --- | --- | --- | --- |
| DSP | dsp.ti.com | Broadband | www.ti.com/broadband |
| Interface | interface.ti.com | Digital Control | www.ti.com/digitalcontrol |
| Logic | logic.ti.com | Military | www.ti.com/military |
| Power Mgmt | power.ti.com | Optical Networking | www.ti.com/opticalnetwork |
| Microcontrollers | microcontroller.ti.com | Security | www.ti.com/security |
|  |  | Telephony Video & Imaging | www.ti.com/telephony www.ti.com/video |
|  |  | Wireless | www.ti.com/wireless |

Mailing Address:	Texas Instruments

Post Office Box 655303 Dallas, Texas 75265

Copyright  2005, Texas Instruments Incorporated