XC61C Series

Low Voltage Detectors (VDF= 0.8V～1.5V) Standard Voltage Detectors (VDF 1.6V～6.0V)

ETR0201_013a

■GENERAL DESCRIPTION

The XC61C series are highly precise, low power consumption voltage detectors, manufactured using CMOS and laser trimming technologies.

Detect voltage is extremely accurate with minimal temperature drift.

Both CMOS and N-channel open drain output configurations are available.

■APPLICATIONS

●Microprocessor reset circuitry

●Memory battery back-up circuits

●Power-on reset circuits

●Power failure detection

●System battery life and charge voltage monitors

■FEATURES

Highly Accurate                 : ± 2%

: ± 1%(Standard Voltage VD: 2.6V~5.0V)

Low Power Consumption : 0.7μA (TYP.) [VIN=1.5V] Detect Voltage Range       : 0.8V ~ 6.0V in 0.1V increments Operating Voltage Range : 0.7V ~ 6.0V (Low Voltage)

0.7V～10.0V (Standard Voltage)

Detect Voltage Temperature Characteristics

: ±100ppm/℃ (TYP.)

Output Configuration        : N-channel open drain or CMOS

Packages                           : SSOT-24

SOT-23

SOT-89

TO-92

Environmentally Friendly : EU RoHS Compliant, Pb Free

■TYPICAL APPLICATION CIRCUITS

■TYPICAL PERFORMANCE CHARACTERISTICS

1/17

XC61C Series

■PIN CONFIGURATION

| ■PIN ASSIGNMENT PIN NUMBER |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PIN NAME | FUNCTION |
| SSOT-24 | SOT-23 | SOT-89 | TO-92 (T) | TO-92 (L) |  |  |
| 2 | 3 | 2 | 2 | 1 | VIN | Supply Voltage |
| 4 | 2 | 3 | 3 | 2 | VSS | Ground |
| 1 | 1 | 1 | 1 | 3 | VOUT | Output |
| 3 | - | - | - | - | NC | No Connection |

■PRODUCT CLASSIFICATION

●Ordering Information

XC61C①②③④⑤⑥⑦-⑧(*1)

(SIDE VIEW)                           (SIDE VIEW)

| DESIGNATOR | DESCRIPTION | SYMBOL | DESCRIPTION |
| --- | --- | --- | --- |
| ① | Output Configuration | C | CMOS output |
|  |  | N | N-ch open drain output |
| ② ③ | Detect Voltage | 08 ~ 60 | e.g.0.9V → ②0, ③9 |
|  |  |  | e.g.1.5V → ②1, ③5 |
| ④ | Output Delay | 0 | No delay |
| ⑤ | Detect Accuracy | 1 | Within ±1% (VDF(T)=2.6V~5.0V) |
|  |  | 2 | Within ±2% |
| ⑥⑦-⑧ | Packages (*2) Taping Type | NR | SSOT-24 (SC-82) |
|  |  | NR-G | SSOT-24 (SC-82) (Halogen & Antimony free) |
|  |  | MR | SOT-23 |
|  |  | MR-G | SOT-23 (Halogen & Antimony free) |
|  |  | PR | SOT-89 |
|  |  | PR-G | SOT-89 (Halogen & Antimony free) |
|  |  | TH | TO-92 (Standard) Taping Type: Paper type |
|  |  | TB | TO-92 (Standard) Taping Type: Bag |
|  |  | LH | TO-92 (Custom pin configuration) Taping Type: Paper type (Discontinued Product) |
|  |  | LB | TO-92 (Custom pin configuration) Taping Type: Bag (Discontinued Product) |

(*1)   The “-G” suffix indicates that the products are Halogen and Antimony free as well as being fully RoHS compliant.

(*2)   The device orientation is fixed in its embossed tape pocket.   For reverse orientation, please contact your local Torex sales office or representative. (Standard orientation: ⑥R-⑧, Reverse orientation: ⑥L-⑧)

2/17

XC61C

Series

■BLOCK DIAGRAMS

(1) CMOS Output                                                                      (2) N-ch Open Drain Output

■ABSOLUTE MAXIMUM RATINGS

Ta = 25OC

| PARAMETER |  |  | SYMBOL | RATINGS | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Voltage |  | *1 | VIN | 9.0 | V |
|  |  | *2 |  | 12.0 |  |
| Output Current |  |  | IOUT | 50 | mA |
| Output Voltage | CMOS |  | VOUT | VSS -0.3 ~ VIN +0.3 | V |
|  | N-ch Open Drain Output *1 |  |  | VSS -0.3 ~ 9.0 |  |
|  | N-ch Open Drain Output *2 |  |  | VSS -0.3 ~ 12.0 |  |
| Power Dissipation | SSOT-24 |  | Pd | 150 | mW |
|  | SOT-23 |  |  | 150 |  |
|  | SOT-89 |  |  | 500 |  |
|  | TO-92 |  |  | 300 |  |
| Operating Temperature Range |  |  | Topr | -40～+85 | OC |
| Storage Temperature Range |  |  | Tstg | -40～+125 | OC |

*1: Low voltage: VDF(T)=0.8V~1.5V

*2: Standard voltage: VDF(T)=1.6V~6.0V

3/17

XC61C Series

■ELECTRICAL CHARACTERISTICS

| VDF (T) = 0.8V to 6.0V ± 2% VDF (T) = 2.6V to 5.0V ± 1% PARAMETER | SYMBOL | CONDITIONS |  |  | MIN. | TYP. | MAX. | UNITS | CIRCUITS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Detect Voltage | VDF | VDF(T)=0.8V~1.5V *1 VDF(T)=1.6V~6.0V *2 |  |  | VDF(T) x 0.98 | VDF(T) | VDF(T) x 1.02 | V | 1 |
|  |  | VDF(T)=2.6V~5.0V *2 |  |  | VDF(T) x 0.99 | VDF(T) | VDF(T) x 1.01 | V | 1 |
| Hysteresis Range | VHYS |  |  |  | VDF x 0.02 | VDF x 0.05 | VDF x 0.08 | V | 1 |
| Supply Current | ISS | VIN = 1.5V |  |  | - | 0.7 | 2.3 | μA | 2 |
|  |  | VIN = 2.0V |  |  | - | 0.8 | 2.7 |  |  |
|  |  | VIN = 3.0V |  |  | - | 0.9 | 3.0 |  |  |
|  |  | VIN = 4.0V |  |  | - | 1.0 | 3.2 |  |  |
|  |  | VIN = 5.0V |  |  | - | 1.1 | 3.6 |  |  |
| Operating Voltage *1 | VIN | VDF(T) = 0.8V to 1.5V |  |  | 0.7 | - | 6.0 | V | 1 |
| Operating Voltage *2 |  | VDF(T) = 1.6V to 6.0V |  |  | 0.7 | - | 10.0 |  |  |
| Output Current *1 | IOUT | N-ch VDS = 0.5V |  | VIN = 0.7V | 0.10 | 0.80 | - | mA | 3 |
|  |  |  |  | VIN = 1.0V | 0.85 | 2.70 | - |  |  |
|  |  | CMOS, P-ch VDS = 2.1V VIN = 6.0V |  |  | - | -7.5 | -1.5 |  | 4 |
| Output Current *2 |  | N-ch VDS = 0.5V |  | VIN = 1.0V | 1.0 | 2.2 | - |  | 3 |
|  |  |  |  | VIN = 2.0V | 3.0 | 7.7 | - |  |  |
|  |  |  |  | VIN = 3.0V | 5.0 | 10.1 | - |  |  |
|  |  |  |  | VIN = 4.0V | 6.0 | 11.5 | - |  |  |
|  |  |  |  | VIN = 5.0V | 7.0 | 13.0 | - |  |  |
|  |  | CMOS, P-ch VDS = 2.1V VIN = 8.0V |  |  | - | -10.0 | -2.0 |  | 4 |
| Leak Current | Ileak | VIN=6.0V, VOUT=6.0V*1 VIN=10.0V, VOUT=10.0V*2 | CMOS |  | - | 10 | - | nA | 3 |
|  |  |  | N-ch Open Drain |  | - | 10 | 100 |  |  |
| Temperature Characteristics | ΔVDF ΔTopr･VDF | -40℃ ≦ Topr ≦ 85℃ |  |  | - | ±100 | - | ppm/ ℃ | - |
| Delay Time (VDR→VOUT inversion) | tDLY | Inverts from VDR to VOUT |  |  | - | 0.03 | 0.20 | ms | 5 |

NOTE:

*1: Low Voltage: VDF(T)=0.8V~1.5V

*2: Standard Voltage: VDF(T)=1.6V~6.0V VDF (T): Setting detect voltage

Release Voltage: VDR = VDF + VHYS

Ta=25℃

4/17

XC61C

Series

■OPERATIONAL EXPLANATION

(Especially prepared for CMOS output products)

① When input voltage (VIN) rises above detect voltage (VDF), output voltage (VOUT) will be equal to VIN. (A condition of high impedance exists with N-ch open drain output configurations.)

② When input voltage (VIN) falls below detect voltage (VDF), output voltage (VOUT) will be equal to the ground voltage

(VSS) level.

③ When input voltage (VIN) falls to a level below that of the minimum operating voltage (VMIN), output will become unstable. In this condition, VIN will equal the pulled-up output (should output be pulled-up.)

④ When input voltage (VIN) rises above the ground voltage (VSS) level, output will be unstable at levels below the

minimum operating voltage (VMIN). Between the VMIN and detect release voltage (VDR) levels, the ground voltage (VSS)

level will be maintained.

⑤ When input voltage (VIN) rises above detect release voltage (VDR), output voltage (VOUT) will be equal to VIN. (A condition of high impedance exists with N-ch open drain output configurations.)

⑥ The difference between VDR and VDF represents the hysteresis range.

●Timing Chart

5/17

XC61C Series

■NOTES ON USE

1.   Please use this IC within the stated maximum ratings. Operation beyond these limits may cause degrading or permanent damage to the device.

2.   When a resistor is connected between the VIN pin and the input with CMOS output configurations, oscillation may occur as a result of voltage drops at RIN if load current (IOUT) exists. (refer to the Oscillation Description (1) below)

3.   When a resistor is connected between the VIN pin and the input with CMOS output configurations, irrespective of N-ch

output configurations, oscillation may occur as a result of through current at the time of voltage release even if load current (IOUT) does not exist. (refer to the Oscillation Description (2) below )

4.   With a resistor connected between the VIN pin and the input, detect and release voltage will rise as a result of the IC's supply current flowing through the VIN pin.

5.   In order to stabilize the IC's operations, please ensure that VIN pin's input frequency's rise and fall times are more than several μ sec / V.

6.   Please use N-ch open drains configuration, when a resistor RIN is connected between the VIN pin and power source.

In such cases, please ensure that RIN is less than 10kΩ and that C is more than 0.1μF.

●Oscillation Description

(1) Output current oscillation with the CMOS output configuration

When the voltage applied at IN rises, release operations commence and the detector's output voltage increases. Load current (IOUT) will flow at RL. Because a voltage drop (RIN x IOUT) is produced at the RIN resistor, located between the input

(IN) and the VIN pin, the load current will flow via the IC's VIN pin. The voltage drop will also lead to a fall in the voltage level

at the VIN  pin. When the VIN  pin voltage level falls below the detect voltage level, detect operations will commence. Following detect operations, load current flow will cease and since voltage drop at RIN will disappear, the voltage level at the VIN pin will rise and release operations will begin over again.

Oscillation may occur with this " release - detect - release " repetition.

Further, this condition will also appear via means of a similar mechanism during detect operations.

(2) Oscillation as a result of through current

Since the XC61C series are CMOS IC S, through current will flow when the IC's internal circuit switching operates (during release and detect operations). Consequently, oscillation is liable to occur as a result of drops in voltage at the through

current's resistor (RIN) during release voltage operations. (refer to Figure 3)

Since hysteresis exists during detect operations, oscillation is unlikely to occur.

6/17

XC61C

Series

100kΩ*

7/17

XC61C Series

■TYPICAL PERFORMANCE CHARACTERISTICS

●Low Voltage

8/17

XC61C

Series

■TYPICAL PERFORMANCE CHARACTERISTICS (Continued)

●Low Voltage (Continued)

(4) N-ch Driver Output Current vs. VDS

ＸＣ６１ＣＣ０９０２ (０.９Ｖ)

1.4

Ta=25℃

Output Current: IOUT (mA)

1.2

VIN =0.8V

1.0

0.8

1.4

Output Current: IOUT (mA)

1.2

1.0

0.8

ＸＣ６１ＣＣ１１０２(１.１Ｖ)

Ta=25℃

VIN =0.8V

3.0

Output Current: IOUT (mA)

2.5

2.0

1.5

ＸＣ６１ＣＣ１１０２(１.１Ｖ)

Ta=25℃

VIN =1.0V

0.6

0.4

0.2

0.7V

0.6

0.4

0.2

0.7V

0.5

0

0         0.2       0.4       0.6       0.8       1.0

VDS (V)

0

0         0.2       0.4       0.6       0.8       1.0

VDS (V)

0

0         0.2       0.4       0.6       0.8       1.0

VDS (V)

1.4

Output Current: IOUT (mA)

1.2

1.0

0.8

0.6

0.4

0.2

ＸＣ６１ＣＣ１５０２(１.５Ｖ)

Ta=25℃

VIN =0.8V

0.7V

Output Current: IOUT (mA)

8.0

6.0

4.0

2.0

ＸＣ６１ＣＣ１５０２(１.５Ｖ)

Ta=25℃

VIN =1.4V

1.2V

1.0V

0

0         0.2       0.4       0.6       0.8       1.0

VDS (V)

0

0     0.2    0.4    0.6    0.8    1.0    1.2    1.4

VDS (V)

(5) N-ch Driver Output Current vs. Input Voltage

ＸＣ６１ＣＣ０９０２ (０.９Ｖ)                         ＸＣ６１ＣＣ１１０２ (１.１Ｖ)                         ＸＣ６１ＣＣ１５０２(１.５Ｖ)

2.5

Output Current: IOUT (mA)

2.0

1.5

VDS=0.5V

5.0

Output Current: IOUT (mA)

4.0

3.0

VDS=0.5V                          Ta=-40℃

25℃

10

VDS=0.5V

Output Current: IOUT (mA)

8

6

Ta=-40℃

25℃

1.0

0.5

0

Ta=85℃

25℃

-40℃

2.0

1.0

0

4

80℃

2

0

85℃

0         0.2       0.4       0.6       0.8       1.0

0       0.2     0.4     0.6     0.8     1.0     1.2

0    0.2   0.4   0.6   0.8   1.0   1.2   1.4   1.6

Input Voltage: VIN (V)                                             Input Voltage: VIN (V)                                             Input Voltage: VIN (V)

(6) P-ch Driver Output Current vs. Input Voltage

ＸＣ６１ＣＣ０９０２ (０.９Ｖ)                         ＸＣ６１ＣＣ１１０２　(１.１Ｖ)

ＸＣ６１ＣＣ１５０２(１.５Ｖ)

12

Ta= 25℃

Output Current: IOUT (mA)

10

VDS=2.1V

12

Ta= 25℃

10

VDS=2.1V

12

Ta= 25℃

Output Current: IOUT (mA)

10

VDS=2.1V

8                                                          1.5V                       8

1.5V                        8

1.5V

6                                                          1.0V                       6

1.0V                        6

1.0V

4                                                          0.5V                      4

0.5V                        4

0.5V

Output Current: IOUT (mA)

2                                                                                        2                                                                                          2

0

0        1        2        3        4        5        6

0

0        1        2        3        4        5        6

0

0        1        2        3        4        5        6

Input Voltage: VIN (V)                                             Input Voltage: VIN (V)                                             Input Voltage: VIN (V)

9/17

XC61C Series

■TYPICAL PERFORMANCE CHARACTERISTICS (Continued)

●Standard Voltage

(1) Supply Current vs. Input Voltage

ＸＣ６１ＣＣ１８０２ (１.８Ｖ）                                                         ＸＣ６１ＣＣ２７０２ (２.７Ｖ)

3.5

Supply Current: ISS (μA)

3.0

3.5

Supply Current: ISS (μA)

3.0

2.5

2.0

1.5

1.0

0.5

2.5

2.0

1.5

1.0

0.5

25℃

Ta=85℃

-40℃

0

| 0 2 4 6 8 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 25℃ |  | Ta=85℃ |  |  |  |  |
|  |  |  |  |  | -40℃ |  |  |  |

Input Voltage: VIN (V)

0

0           2           4           6           8          10

Input Voltage: VIN (V)

3.5

ＸＣ６１ＣＣ３６０２ (３.６Ｖ)                                                                ＸＣ６１ＣＣ４５０２ (４.５Ｖ)

3.5

Supply Current: ISS (μA)

3.0

Supply Current: ISS (μA)

3.0

2.5

2.0

1.5

25℃

Ta=85℃

2.5

2.0

1.5

25℃

Ta=85℃

1.0

0.5

-40℃

1.0

0.5

-40℃

0

0           2           4           6           8          10

Input Voltage: VIN (V)

0

0           2           4           6           8          10

Input Voltage: VIN (V)

(2) Detect, Release Voltage vs. Ambient Temperature

Detect, Release Voltage: VDF, VDR (V)

1.90

ＸＣ６１ＣＣ１８０２ (１.８Ｖ)                                                               ＸＣ６１ＣＣ２７０２ (２.７Ｖ)

Detect, Release Voltage: VDF, VDR (V)

2.80

VDR

1.85

VDR

2.75

1.80

VDF

2.70

VDF

1.75

-50     -25       0        25       50       75      100

Ambient Temperature : Ta (℃)

2.65

-50     -25       0        25       50       75      100

Ambient Temperature : Ta (℃)

3.8

ＸＣ６１ＣＣ３６０２ (３.６Ｖ)

Detect, Release Voltage: VDF, VDR (V)

4.7

ＸＣ６１ＣＣ４５０２ (４.５Ｖ)

3.7

VDR

4.6

VDR

3.6

VDF

4.5

VDF

Detect, Release Voltage: VDF, VDR (V)

3.5

-50     -25       0        25       50       75      100

Ambient Temperature : Ta (℃)

4.4

-50     -25       0        25       50       75      100

Ambient Temperature : Ta (℃)

10/17

XC61C

Series

■TYPICAL PERFORMANCE CHARACTERISTICS (Continued)

●Standard Voltage (Continued)

(3) Output Voltage vs. Input Voltage

ＸＣ６１ＣＮ１８０２ （１.８V)                                                  ＸＣ６１ＣＮ２７０２ （２.７Ｖ)

| 2 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ta=25℃ |  |  |  |  |  |  |

Output Voltage: VOUT (V)

Ta=25℃

Output Voltage: VOUT (V)

2

1

1

0

0                              1                              2

Input Voltage: VIN (V)

0

0                   1                   2                   3

Input Voltage: VIN (V)

ＸＣ６１ＣＮ３６０２ (３.６V)

4

Ta=25℃

Output Voltage: VOUT (V)

3

ＸＣ６１ＣＮ４５０２ (４.５V)

5

Ta=25℃

4

3

2

2

1

Output Voltage: VOUT (V)

1

0

0              1              2              3              4

0

0           1           2          3           4           5

Input Voltage: VIN (V)                                                                         Input Voltage: VIN (V)

Note : The N-channell open draiin ppuullll uupp rreessiissttaannccee vvaalluuee iiss 110000kkΩ.Ω.

(4) N-ch Driver Output Current vs. VDS

ＸＣ６１ＣＣ１８０２ (１.８Ｖ)                                                  ＸＣ６１ＣＣ２７０２ (２.７V)

10                                                                                                                       30

Ta=25℃

Output Current: IOUT (mA)

8

VIN =1.5V

Ta=25℃

Output Current: IOUT (mA)

25

VIN =2.5V

6

4

1.0V

2

0

20

15

10

5

1.0V

0

1.5V

2.0V

0            0.5           1.0           1.5           2.0

VDS (V)

0       0.5     1.0     1.5     2.0     2.5     3.0

VDS (V)

ＸＣ６１ＣＣ３６０２ (３.６Ｖ)

40

XC61CC4502 (4.5V)

ＸＣ６１XCＣ6Ｃ1C４C５45０0２2 ((4４.5V.５品V))

80

80        Ta=25℃

Ta=25℃

Output Current: IOUT (mA)

30

20

10

0

1.5V

VIN =3.0V

2.5V

2.0V

Output Current: IOUT (mA)

7700

6600

5500

4400

3300

2200

1100

00

Ta=25℃

VIN =4.0V

1.15.5VV

VIN =4.0V

33.5.5VV

3.03V.0V

2.52.V5V

2.20.V0V

0       0.5     1.0     1.5     2.0     2.5     3.0

VDS (V)

0    0.5   1.0   1.5   2.0   2.5   3.0   3.5   4.0

0     0.5    1.0    1.5    2.0    2.5    3.0    3.5    4.0

VDS (V)

VDS (V)

11/17

XC61C Series

■TYPICAL PERFORMANCE CHARACTERISTICS (Continued)

●Standard Voltage (Continued)

(4) N-ch Driver Output Current vs. VDS

1000

ＸＣ６１ＣＣ１８０２ (１.８V)                                                  ＸＣ６１ＣＣ２７０２ （２.７V)

1000

Ta=25℃

VIN =0.8V

Ta=25℃

Output Current: IOUT (μA)

800

Output Current: IOUT (μA)

800

VIN =0.8V

600

400

0.7V

600

400

0.7V

200

200

0

0         0.2        0.4        0.6        0.8        1.0

VDS (V)

0

0         0.2        0.4        0.6        0.8        1.0

VDS (V)

1000

ＸＣ６１ＣＣ３６０２ (３.６V)                                                  ＸＣ６１ＣＣ４５０２ (４.５V)

1000

Output Current: IOUT (μA)

800

Ta=25℃

VIN =0.8V

Output Current: IOUT (μA)

800

Ta=25℃

VIN =0.8V

600

600

400

0.7V

400

0.7V

200

200

0

0         0.2        0.4        0.6        0.8        1.0

VDS (V)

0

0         0.2        0.4        0.6        0.8        1.0

VDS (V)

(5) N-ch Driver Output Current vs. Input Voltage

ＸＣ６１ＣＣ１８０２ (１.８V)                                                  ＸＣ６１ＣＣ２７０２ (２.７V)

15                                                                                                                          25

VDS=0.5V

Output Current: IOUT (mA)

10

Ta=-40℃

25℃

VDS=0.5V                    Ta=-40℃

Output Current: IOUT (mA)

20

25℃

15

5

85℃

10

85℃

5

0

0             0.5           1.0           1.5           2.0

0

0       0.5      1.0      1.5      2.0      2.5      3.0

Input Voltage: VIN (V)                                                                          Input Voltage: VIN (V)

ＸＣ６１ＣＣ３６０２ (３.６V)                                                  ＸＣ６１ＣＣ４５０２ (４.５V)

30                                                                                                                          40

VDS=0.5V

Output Current: IOUT (mA)

25

20

Ta=-40℃

25℃

VDS=0.5V

Output Current: IOUT (mA)

30

25℃

Ta=-40℃

15

10                                                  85℃

5

20

10                                                85℃

0

0              1              2              3              4

Input Voltage: VIN (V)

0

0           1           2           3           4           5

Input Voltage: VIN (V)

12/17

XC61C

Series

■TYPICAL PERFORMANCE CHARACTERISTICS (Continued)

●Standard Voltage (Continued)

(6) P-ch Driver Output Current vs. Input Voltage

ＸＣ６１ＣＣ１８０２ (１.８V)                                                   ＸＣ６１ＣＣ２７０２ (２.７V)

15                                                                                                                               15

VDS=2.1V

VDS=2.1V

1.5V

Output Current: IOUT  (mA)

Output Current: IOUT  (mA)

10                                                                                                                               10

1.5V

1.0V

1.0V

0.5V

0.5V

0

0              2             4              6              8            10

0

0              2             4              6              8            10

Input Voltage: VIN (V)                                                                             Input Voltage: VIN (V)

ＸＣ６１ＣＣ３６０２ (３.６V)

15

ＸＣ６１ＣＣ４５０２ (４.５V)

15

VDS=2.1V

VDS=2.1V

Output Current: IOUT  (mA)

Output Current: IOUT  (mA)

10                                                                     1.5V                                                  10

1.0V

1.0V

0.5V

0.5V

0

0              2             4              6              8            10

0

0              2             4              6              8            10

Input Voltage: VIN (V)

Input Voltage: VIN (V)

13/17

XC61C Series

■PACKAGING INFORMATION

●SSOT-24 (SC-82)                                                              ●SOT-23

●SOT-89                                                                             ●TO-92

14/17

XC61C

Series

■MARKING RULE

● SSOT-24, SOT-23, SOT-89

| ① represents integer of detect voltage and MARK | CONFIGURATION | VOLTAGE (V) |
| --- | --- | --- |
| B | CMOS | 1.X |
| C | CMOS | 2.X |
| D | CMOS | 3.X |
| E | CMOS | 4.X |
| F | CMOS | 5.X |
| H | CMOS | 6.X |

4          3                                         CMOS Output (XC61CC series)

①    ②    ④

1          2

3

①   ②   ③   ④

①    ②

③    ④

1                2

N-Channel Open Drain Output (XC61CN series)

| MARK | CONFIGURATION | VOLTAGE (V) |
| --- | --- | --- |
| K | N-ch | 0.X |
| L | N-ch | 1.X |
| M | N-ch | 2.X |
| N | N-ch | 3.X |
| P | N-ch | 4.X |
| R | N-ch | 5.X |
| S | N-ch | 6.X |

| ② represents decimal number of detect voltage MARK | VOLTAGE (V) | MARK | VOLTAGE (V) |
| --- | --- | --- | --- |
| 0 | X.0 | 5 | X.5 |
| 1 | X.1 | 6 | X.6 |
| 2 | X.2 | 7 | X.7 |
| 3 | X.3 | 8 | X.8 |
| 4 | X.4 | 9 | X.9 |

1            2            3

③ represents delay time

(Except for SSOT-24)

| MARK | DELAY TIME | PRODUCT SERIES |
| --- | --- | --- |
| 3 | No Delay Time | XC61Cxxx0xxx |

④ represents production lot number

Based on the internal standard.   (G, I, J, O, Q, W excluded)

15/17

XC61C Series

■MARKING RULE (Continued)

●TO-92

① represents output configuration

| MARK | OUTPUT CONFIGURATION |
| --- | --- |
| C | CMOS |
| N | N-ch |

②, ③ represents detect voltage (ex.)

| MARK |  | VOLTAGE (V) |
| --- | --- | --- |
| ② | ③ |  |
| 3 | 3 | 3.3 |
| 5 | 0 | 5.0 |

| ④ represents delay time MARK | DELAY TIME |
| --- | --- |
| 0 | No delay |

⑤ represents detect voltage accuracy

| MARK | DETECT VOLTAGE ACCURACY |
| --- | --- |
| 1 | Within ± 1% (Semi-custom) |
| 2 | Within ± 2% |

⑥ represents a least significant digit of production year

| MARK | PRODUCTION YEAR |
| --- | --- |
| 5 | 2005 |
| 6 | 2006 |

⑦ represents production lot number

0 to 9, A to Z repeated.   (G, I, J, O, Q, W excluded)

* No character inversion used.

16/17

XC61C

Series

1.   The products and product specifications contained herein are subject to change without notice to improve performance characteristics.   Consult us, or our representatives before use, to confirm that the information in this datasheet is up to date.

2.   We assume no responsibility for any infringement of patents, patent rights, or other rights arising from the use of any information and circuitry in this datasheet.

3.  Please ensure suitable shipping controls (including fail-safe designs and aging protection) are in force for equipment employing products listed in this datasheet.

4.   The products in this datasheet are not developed, designed, or approved for use with such equipment whose failure of malfunction can be reasonably expected to directly endanger the life of, or cause significant injury to, the user.

(e.g.  Atomic  energy;  aerospace;  transport;   combustion  and  associated  safety equipment thereof.)

5.   Please use the products listed in this datasheet within the specified ranges.

Should you wish to use the products under conditions exceeding the specifications, please consult us or our representatives.

6.   We assume no responsibility for damage or loss due to abnormal use.

7.   All rights reserved. No part of this datasheet may be copied or reproduced without the prior permission of TOREX SEMICONDUCTOR LTD.

17/17