R	Virtex™ 2.5 V

Field Programmable Gate Arrays

DS003-1 (v2.5 ) April 2, 2001	Product Specification

Features

•	Fast, high-density Field-Programmable Gate Arrays

-	Densities from 50k to 1M system gates

-	System performance up to 200 MHz

-	66-MHz PCI Compliant

-	Hot-swappable for Compact PCI

•	Multi-standard SelectIO™ interfaces

-	16 high-performance interface standards

-	Connects directly to ZBTRAM devices

•	Built-in clock-management circuitry

-	Four dedicated delay-locked loops (DLLs) for advanced clock control

-	Four primary low-skew global clock distribution nets, plus 24 secondary local clock nets

•	Hierarchical memory system

-	LUTs configurable as 16-bit RAM, 32-bit RAM,

16-bit dual-ported RAM, or 16-bit Shift Register

-	Configurable synchronous dual-ported 4k-bit

RAMs

-	Fast interfaces to external high-performance RAMs

•	Flexible architecture that balances speed and density

-	Dedicated carry logic for high-speed arithmetic

-	Dedicated multiplier support

-	Cascade chain for wide-input functions

-	Abundant registers/latches with clock enable, and dual synchronous/asynchronous set and reset

-	Internal 3-state bussing

-	IEEE 1149.1 boundary-scan logic

-	Die-temperature sensor diode

•	Supported by FPGA Foundation™ and Alliance

Development Systems

-	Complete support for Unified Libraries, Relationally

Placed Macros, and Design Manager

-	Wide selection of PC and workstation platforms

•	SRAM-based in-system configuration

-	Unlimited re-programmability

-	Four programming modes

•	0.22 mm 5-layer metal process

•	100% factory tested

Description

The	Virtex	FPGA	family	delivers	high-performance, high-capacity programmable logic solutions. Dramatic increases in silicon efficiency result from optimizing the new architecture for place-and-route efficiency and exploiting an aggressive 5-layer-metal 0.22 mm  CMOS process. These advances make Virtex FPGAs powerful and flexible alterna- tives to mask-programmed gate arrays. The Virtex family comprises the nine members shown in Table 1.

Building on experience gained from previous generations of FPGAs, the Virtex family represents a revolutionary step forward in programmable logic design. Combining a wide variety of programmable system features, a rich hierarchy of fast, flexible interconnect resources, and advanced process technology, the  Virtex family delivers a  high-speed and high-capacity programmable logic solution that enhances design flexibility while reducing time-to-market.

Table 1:  Virtex Field-Programmable Gate Array Family Members

| Device | System Gates | CLB Array | Logic Cells | Maximum Available I/O | Block RAM Bits | Maximum SelectRAM+™ Bits |
| --- | --- | --- | --- | --- | --- | --- |
| XCV50 | 57,906 | 16x24 | 1,728 | 180 | 32,768 | 24,576 |
| XCV100 | 108,904 | 20x30 | 2,700 | 180 | 40,960 | 38,400 |
| XCV150 | 164,674 | 24x36 | 3,888 | 260 | 49,152 | 55,296 |
| XCV200 | 236,666 | 28x42 | 5,292 | 284 | 57,344 | 75,264 |
| XCV300 | 322,970 | 32x48 | 6,912 | 316 | 65,536 | 98,304 |
| XCV400 | 468,252 | 40x60 | 10,800 | 404 | 81,920 | 153,600 |
| XCV600 | 661,111 | 48x72 | 15,552 | 512 | 98,304 | 221,184 |
| XCV800 | 888,439 | 56x84 | 21,168 | 512 | 114,688 | 301,056 |
| XCV1000 | 1,124,022 | 64x96 | 27,648 | 512 | 131,072 | 393,216 |

© 2001 Xilinx, Inc. All rights reserved. All Xilinx trademarks, registered trademarks, patents, and disclaimers are as listed at  http://www.xilinx.com/legal.htm.

All other trademarks and registered trademarks are the property of their respective owners. All specifications are subject to change without notice.

Virtex Architecture

Virtex devices feature a flexible, regular architecture that comprises an array of configurable logic blocks (CLBs) sur- rounded by programmable input/output blocks (IOBs), all interconnected by a rich hierarchy of fast, versatile routing resources. The abundance of routing resources permits the Virtex family to accommodate even the largest and most complex designs.

Virtex FPGAs are SRAM-based, and are customized by loading configuration data into internal memory cells. In some modes, the FPGA reads its own configuration data from an external PROM (master serial mode). Otherwise, the configuration data is written into the FPGA (Select- MAP™, slave serial, and JTAG modes).

The standard Xilinx Foundation™ and Alliance Series™ Development systems deliver complete design support for Virtex, covering every aspect from behavioral and sche- matic entry, through simulation, automatic design transla- tion and implementation, to the creation, downloading, and readback of a configuration bit stream.

Higher Performance

Virtex devices provide better performance than previous generations of FPGA. Designs can achieve synchronous system clock rates up to 200 MHz including I/O. Virtex inputs and outputs comply fully with PCI specifications, and interfaces can be implemented that operate at 33 MHz or 66

MHz. Additionally, Virtex supports the hot-swapping requirements of Compact PCI.

Xilinx thoroughly benchmarked the Virtex family. While per- formance  is  design-dependent, many  designs  operated internally at speeds in excess of 100 MHz and can achieve

200 MHz. Table 2 shows performance data for representa- tive circuits, using worst-case timing parameters.

| Table 2: Performance for Common Circuit Functions Function | Bits | Virtex -6 |
| --- | --- | --- |
| Register-to-Register |  |  |
| Adder | 16 64 | 5.0 ns 7.2 ns |
| Pipelined Multiplier | 8 x 8 16 x 16 | 5.1 ns 6.0 ns |
| Address Decoder | 16 64 | 4.4 ns 6.4 ns |
| 16:1 Multiplexer |  | 5.4 ns |
| Parity Tree | 9 18 36 | 4.1 ns 5.0 ns 6.9 ns |
| Chip-to-Chip |  |  |
| HSTL Class IV |  | 200 MHz |
| LVTTL,16mA, fast slew |  | 180 MHz |

Virtex™ 2.5 V Field Programmable Gate Arrays

R

Virtex Device/Package Combinations and Maximum I/O

Table 3:  Virtex Family Maximum User I/O by Device/Package (Excluding Dedicated Clock Pins)

| Package | XCV50 | XCV100 | XCV150 | XCV200 | XCV300 | XCV400 | XCV600 | XCV800 | XCV1000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CS144 | 94 | 94 |  |  |  |  |  |  |  |
| TQ144 | 98 | 98 |  |  |  |  |  |  |  |
| PQ240 | 166 | 166 | 166 | 166 | 166 |  |  |  |  |
| HQ240 |  |  |  |  |  | 166 | 166 | 166 |  |
| BG256 | 180 | 180 | 180 | 180 |  |  |  |  |  |
| BG352 |  |  | 260 | 260 | 260 |  |  |  |  |
| BG432 |  |  |  |  | 316 | 316 | 316 | 316 |  |
| BG560 |  |  |  |  |  | 404 | 404 | 404 | 404 |
| FG256 | 176 | 176 | 176 | 176 |  |  |  |  |  |
| FG456 |  |  | 260 | 284 | 312 |  |  |  |  |
| FG676 |  |  |  |  |  | 404 | 444 | 444 |  |
| FG680 |  |  |  |  |  |  | 512 | 512 | 512 |

Virtex Ordering Information

Example:

Device Type

Speed Grade

-4

-5

XCV300 -6 PQ 240 C

Temperature Range

C = Commercial (TJ = 0ﾰC to +85ﾰC) I = Industrial (TJ = –40ﾰC to +100ﾰC)

Number of Pins

-6	Package Type

BG = Ball Grid Array

FG = Fine-pitch Ball Grid Array

PQ = Plastic Quad Flat Pack

HQ = High Heat Dissipation QFP TQ = Thin Quad Flat Pack

CS = Chip-scale Package

Figure 1:  Virtex Ordering Information

Revision History

| Date | Version | Revision |
| --- | --- | --- |
| 11/98 | 1.0 | Initial Xilinx release. |
| 01/99 | 1.2 | Updated package drawings and specs. |
| 02/99 | 1.3 | Update of package drawings, updated specifications. |
| 05/99 | 1.4 | Addition of package drawings and specifications. |
| 05/99 | 1.5 | Replaced FG 676 & FG680 package drawings. |
| 07/99 | 1.6 | Changed Boundary Scan Information and changed Figure 11, Boundary Scan Bit Sequence. Updated IOB Input & Output delays. Added Capacitance info for different I/O Standards. Added 5 V tolerant information. Added DLL Parameters and waveforms and new Pin-to-pin Input and Output Parameter tables for Global Clock Input to Output and Setup and Hold. Changed Configuration Information including Figures 12, 14, 17 & 19. Added device-dependent listings for quiescent currents ICCINTQ and ICCOQ. Updated IOB Input and Output Delays based on default standard of LVTTL, 12 mA, Fast Slew Rate. Added IOB Input Switching Characteristics Standard Adjustments. |
| 09/99 | 1.7 | Speed grade update to preliminary status, Power-on specification and Clock-to-Out Minimums additions, “0” hold time listing explanation, quiescent current listing update, and Figure 6 ADDRA input label correction. Added TIJITCC parameter, changed TOJIT to TOPHASE. |
| 01/00 | 1.8 | Update to speed.txt file 1.96. Corrections for CRs 111036,111137, 112697, 115479, 117153, 117154, and 117612. Modified notes for Recommended Operating Conditions (voltage and temperature). Changed Bank information for VCCO in CS144 package on p.43. |
| 01/00 | 1.9 | Updated DLL Jitter Parameter table and waveforms, added Delay Measurement Methodology table for different I/O standards, changed buffered Hex line info and Input/Output Timing measurement notes. |
| 03/00 | 2.0 | New TBCKO values; corrected FG680 package connection drawing; new note about status of CCLK pin after configuration. |
| 05/00 | 2.1 | Modified “Pins not listed ...” statement. Speed grade update to Final status. |
| 05/00 | 2.2 | Modified Table 18. |
| 09/00 | 2.3 | • Added XCV400 values to table under Minimum Clock-to-Out for Virtex Devices. • Corrected Units column in table under IOB Input Switching Characteristics. • Added values to table under CLB SelectRAM Switching Characteristics. |
| 10/00 | 2.4 | • Corrected Pinout information for devices in the BG256, BG432, and BG560 packages in Table 18. • Corrected BG256 Pin Function Diagram. |
| 04/01 | 2.5 | • Revised minimums for Global Clock Set-Up and Hold for LVTTL Standard, with DLL. • Converted file to modularized format. See Virtex Data Sheet section. |

Virtex Data Sheet

The Virtex Data Sheet contains the following modules:

•	DS003-1, Virtex 2.5V FPGAs:

Introduction and Ordering Information (Module 1)

•	DS003-2, Virtex 2.5V FPGAs:

Functional Description (Module 2)

•	DS003-3, Virtex 2.5V FPGAs:

DC and Switching Characteristics (Module 3)

•	DS003-4, Virtex 2.5V FPGAs:

Pinout Tables (Module 4)