Ultralow Input Bias Current

Operational Amplifier

AD549

FEATURES

Ultralow input bias current

60 fA maximum (AD549L)

CONNECTION DIAGRAM

GUARD PIN, CONNECTED TO CASE

OFFSET  NULL

250 fA maximum (AD549J)

Input bias current guaranteed over the common-mode voltage range

Low offset voltage

0.25 mV maximum (AD549K)

1.00 mV maximum (AD549J) Low offset drift

5 μV/°C maximum (AD549K)

1

INVERTING    2

INPUT

3

NONINVERTING

INPUT

8

AD549

4

V–

10kΩ

V+

7

6   OUTPUT

5

OFFSET

NULL

20 μV/°C maximum (AD549J) Low power

700 μA maximum supply current

Low input voltage noise

4 μV p-p over 0.1 Hz to 10 Hz

MIL-STD-883B parts available

APPLICATIONS

Electrometer amplifier Photodiode preamp pH electrode buffer

Vacuum ion gauge measurement

1                                     5

4

VOS TRIM

Figure 1.

00511-001

–15V

GENERAL DESCRIPTION

The AD5491 is a monolithic electrometer operational amplifier with very low input bias current. Input offset voltage and input offset voltage drift are laser trimmed for precision performance. The ultralow input current of the part is achieved with Topgate™ JFET technology, a process development exclusive to Analog Devices, Inc. This technology allows fabrication of extremely low input current JFETs compatible with a standard junction isolated bipolar process. The 1015 Ω common-mode impedance, which results from the bootstrapped input stage, ensures that

the input current is essentially independent of the common- mode voltage.

The AD549 is suited for applications requiring very low input current and low input offset voltage. It excels as a preamp for a wide variety of current output transducers, such as photodiodes, photomultiplier tubes, or oxygen sensors. The AD549 can also be used as a precision integrator or low droop sample-and-hold. The AD549 is pin compatible with standard FET and electrometer op amps, allowing designers to upgrade the performance of present systems at little additional cost.

The AD549 is available in a TO-99 hermetic package. The case is connected to Pin 8, thus, the metal case can be independently

1 Protected by U.S. Patent No. 4,639,683.

Rev. H

Information furnished by Analog Devices is believed to be accurate and reliable. However, no responsibility is assumed by Analog Devices for its use, nor for any infringements of patents or other rights of thirdparties that may result from its use. Specifications subject tochange without notice. No license is granted by implication or otherwise under any patent or patent rights of Analog Devices. Trademarks andregisteredtrademarks arethe propertyoftheir respective owners.

connected to a point at the same potential as the input terminals, minimizing stray leakage to the case. The AD549 is available in four performance grades. The J, K, and L versions are rated over the commercial temperature range of 0°C to +70°C. The S grade is specified over the military temperature range of −55°C to +125°C and is available processed to MIL-STD-883B, Rev. C. Extended reliability plus screening is also available. Plus screening includes

168 hour burn-in, as well as other environmental and physical tests derived from MIL-STD-883B, Rev. C.

PRODUCT HIGHLIGHTS

1.    The AD549 input currents are specified, 100% tested, and guaranteed after the device is warmed up. They are guaran- teed over the entire common-mode input voltage range.

2.    The AD549 input offset voltage and drift are laser trimmed to 0.25 mV and 5 μV/°C (AD549K), and to 1 mV and

20 μV/°C (AD549J).

3.    A maximum quiescent supply current of 700 μA minimizes heating effects on input current and offset voltage.

4.    AC specifications include 1 MHz unity-gain bandwidth and 3 V/μs slew rate. Settling time for a 10 V input step is

5 μs to 0.01%.

One Technology Way, P.O. Box 9106,  Norwood,  MA 02062-9106, U.S.A. Tel: 781.329.4700                                                           www.analog.com Fax: 781.461.3113     ©2002–2008 Analog Devices, Inc. All rights reserved.

AD549

TABLE OF CONTENTS

Features .............................................................................................. 1

Applications....................................................................................... 1

Connection Diagram ....................................................................... 1

General Description ......................................................................... 1

Product Highlights ........................................................................... 1

Revision History ............................................................................... 2

Specifications..................................................................................... 3

Absolute Maximum Ratings............................................................ 5

ESD Caution.................................................................................. 5

Typical Performance Characteristics ............................................. 6

Functional Description .................................................................. 10

Minimizing Input Current ........................................................ 10

Circuit Board Notes ................................................................... 10

Offset Nulling ............................................................................. 11

AC Response with High Value Source and Feedback

Resistance .................................................................................... 12

Common-Mode Input Voltage Overload ............................... 12

Differential Input Voltage Overload ........................................ 13

Input Protection ......................................................................... 13

Sample and Difference Circuit to Measure Electrometer

Leakage Currents........................................................................ 13

Photodiode Interface ................................................................. 14

Log Ratio Amplifier ................................................................... 15

Temperature Compensated pH Probe Amplifier................... 16

Outline Dimensions ....................................................................... 18

Ordering Guide .......................................................................... 18

REVISION HISTORY

3/08—Rev. G to Rev. H

Changes to Features.......................................................................... 1

Changes to Figure 1 .......................................................................... 1

Deleted Package Option Parameter ............................................... 4

Inserted ESD Caution ...................................................................... 5

Changes to Figure 2, Figure 3, and Figure 7.................................. 6

Changes to Figure 11........................................................................ 7

Changes to Figure 17........................................................................ 8

Changes to Figure 41...................................................................... 14

7/07—Rev. F to Rev. G

Changes to Figure 45...................................................................... 16

Changes to Temperature Compensated pH Probe

Amplifier Section............................................................................ 17

Changes to Figure 46...................................................................... 17

Changes to Ordering Guide .......................................................... 18

5/06—Rev.  E to Rev. F

Removed ESD Caution .....................................................................5

8/05—Rev. D to Rev. E

Change to Figure 22 ..........................................................................9

5/04—Rev. C to Rev. D

Updated Format.................................................................. Universal

Changes to Features ..........................................................................1

Updated Outline Dimensions ....................................................... 18

Added Ordering Guide .................................................................. 18

10/02—Rev. B to Rev. C

Deleted Product Highlights #5 ........................................................1

Edits to Specifications .......................................................................3

Deleted Metallization Photograph ..................................................3

Updated Outline Dimensions ....................................................... 13

7/02—Rev. A to Rev. B

Edits to Specifications .......................................................................2

Rev. H | Page 2 of 20

AD549

SPECIFICATIONS

@ 25°C and VS = ±15 V dc, unless otherwise noted; all minimum and maximum specifications are guaranteed; specifications in boldface

are tested on all production units at final electrical test, and results from those tests are used to calculate outgoing quality levels.

Table 1.

| Parameter | AD549J Min Typ Max | AD549K Min Typ Max | AD549L Min Typ Max | AD549S Min Typ Max | Unit |
| --- | --- | --- | --- | --- | --- |
| INPUT BIAS CURRENT 1 Either Input, VCM = 0 V Either Input, VCM = ±10 V Either Input at TMAX, VCM = 0 V Offset Current Offset Current at TMAX | 150 250 150 250 11 50 2.2 | 75 100 75 100 4.2 30 1.3 | 40 60 40 60 2.8 20 0.85 | 75 100 75 100 420 30 125 | fA fA pA fA pA |
| INPUT OFFSET VOLTAGE2 Initial Offset Offset at TMAX vs. Temperature vs. Supply vs. Supply, TMIN to TMAX Long-Term Offset Stability | 0.5 1.0 1.9 10 20 32 100 32 100 15 | 0.15 0.25 0.4 2 5 10 32 10 32 15 | 0.3 0.5 0.9 5 10 10 32 10 32 15 | 0.3 0.5 2.0 10 15 10 32 32 50 15 | mV mV μV/°C μV/V μV/V μV/month |
| INPUT VOLTAGE NOISE f = 0.1 Hz to 10 Hz f = 10 Hz f = 100 Hz f = 1 kHz f = 10 kHz | 4 90 60 35 35 | 4 6 90 60 35 35 | 4 90 60 35 35 | 4 90 60 35 35 | μV p-p nV/√Hz nV/√Hz nV/√Hz nV/√Hz |
| INPUT CURRENT NOISE f = 0.1 Hz to 10 Hz f = 1 kHz | 0.7 0.22 | 0.5 0.16 | 0.36 0.11 | 0.5 0.16 | fA rms fA/√Hz |
| INPUT IMPEDANCE Differential VDIFF = ±1 Common Mode VCM = ±10 V | 1013¦¦1 1015¦¦0.8 | 1013¦¦1 1015¦¦0.8 | 1013¦¦1 1015¦¦0.8 | 1013¦¦1 1015¦¦0.8 | Ω¦¦pF Ω¦¦pF |
| OPEN-LOOP GAIN VOUT @ ±10 V, RL = 10 kΩ VOUT @ ±10 V, RL = 10 kΩ, TMIN to TMAX VOUT = ±10 V, RL = 2 kΩ VOUT = ±10 V, RL = 2 kΩ, TMIN to TMAX | 300 1000 300 800 100 250 80 200 | 300 1000 300 800 100 250 80 200 | 300 1000 300 800 100 250 80 200 | 300 1000 300 800 100 250 25 150 | V/mV V/mV V/mV V/mV |
| INPUT VOLTAGE RANGE Differential 3 Common-Mode Voltage Common-Mode Rejection Ratio −10 V ≤ VCM ≤ +10 V TMIN to TMAX | ±20 −10 +10 80 90 76 80 | ±20 −10 +10 90 100 80 90 | ±20 −10 +10 90 100 80 90 | ±20 −10 +10 90 100 80 90 | V V dB dB |

Rev. H | Page 3 of 20

AD549

| Parameter | AD549J Min Typ Max | AD549K Min Typ Max | AD549L Min Typ Max | AD549S Min Typ Max | Unit |
| --- | --- | --- | --- | --- | --- |
| OUTPUT CHARACTERISTICS VOUT @ RL = 10 kΩ, TMIN to TMAX VOUT @ RL = 2 kΩ, TMIN to TMAX Short-Circuit Current TMIN to TMAX Load Capacitance Stability, G = +1 | −12 +12 −10 +10 15 20 35 9 4000 | −12 +12 −10 +10 15 20 35 9 4000 | −12 +12 −10 +10 15 20 35 9 4000 | −12 +12 −10 +10 15 20 35 6 4000 | V V mA mA pF |
| FREQUENCY RESPONSE Unity Gain, Small Signal Full Power Response Slew Rate Settling Time, 0.1% Settling Time, 0.01% Overload Recovery, 50% Overdrive, G = −1 | 0.7 1.0 50 2 3 4.5 5 2 | 0.7 1.0 50 2 3 4.5 5 2 | 0.7 1.0 50 2 3 4.5 5 2 | 0.7 1.0 50 2 3 4.5 5 2 | MHz kHz V/μs μs μs μs |
| POWER SUPPLY Rated Performance Operating Quiescent Current | ±15 ±5 ±18 0.60 0.70 | ±15 ±5 ±18 0.60 0.70 | ±15 ±5 ±18 0.60 0.70 | ±15 ±5 ±18 0.60 0.70 | V V mA |
| TEMPERATURE RANGE Operating, Rated Performance Storage | 0 70 −65 +150 | 0 70 −65 +150 | 0 70 −65 +150 | −55 +125 −65 +150 | °C °C |

1 Bias current specifications are guaranteed after five minutes of operation at TA = 25°C. Bias current increases by a factor of 2.3 for every 10°C rise in temperature.

2 Input offset voltage specifications are guaranteed after five minutes of operation at TA = 25°C.

3 Defined as maximum continuous voltage between the inputs, such that neither input exceeds ±10 V from ground.

Rev. H | Page 4 of 20

AD549

ABSOLUTE MAXIMUM RATINGS

Table 2.

Parameter                                                         Rating Supply Voltage                                                   ±18 V Internal Power Dissipation                                500 mW Input Voltage1                                                                                     ±18 V Output Short-Circuit Duration                          Indefinite Differential Input Voltage                                  +VS and −VS

Storage Temperature Range                             −65°C to +125°C Operating Temperature Range

AD549J, AD549K, AD549L                           0°C to +70°C AD549S                                                         −55°C to +125°C

Lead Temperature (Soldering, 60 sec)              300°C

1 For supply voltages less than ±18 V, the absolute maximum input voltage is equal to the supply voltage.

Stresses above those listed under Absolute Maximum Ratings may cause permanent damage to the device. This is a stress rating only; functional operation of the device at these or any other conditions above those indicated in the operational section of this specification is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ESD CAUTION

Rev. H | Page 5 of 20

AD549

TYPICAL PERFORMANCE  CHARACTERISTICS

20

15

INPUT VOLTAGE (V)

10

5

800

AMPLIFIER QUIESCENT CURRENT (µA)

700

600

500

0

| 0 5 10 15 20 |  |  |  |
| --- | --- | --- | --- |
|  |  | VIN+ |  |
|  |  | VIN– |  |

SUPPLY VOLTAGE (±V)

Figure 2. Input Voltage Range vs. Supply Voltage

00511-002

400

00511-005

| 0 5 10 15 20 |  |  |  |
| --- | --- | --- | --- |

SUPPLY VOLTAGE (±V)

00511-006

Figure 5. Quiescent Current vs. Supply Voltage

| 20 120 25°C RL = 10kΩ |  |  | +VOUT |
| --- | --- | --- | --- |
|  |  |  | –VOUT |

COMMON-MODE REJECTION RATIO (dB)

110

OUTPUT VOLTAGE SWING (V)

15

100

10

90

5

80

0

0                         5                        10                       15                       20

SUPPLY VOLTAGE (±V)

Figure 3. Output Voltage Swing vs. Supply Voltage

30

00511-003

70

–20                     –10                       0                        10                       20

INPUT COMMON-MODE VOLTAGE (V)

00511-007

Figure 6. CMRR vs. Input Common-Mode Voltage

3000

VS = ±15V

20                                                                                                                                                               1000

15

OPEN-LOOP GAIN (V/mV)

10                                                                                                                                                                 300

OUTPUT VOLTAGE SWING (V p-p)

5

0

10                      100                      1k                      10k                    100k

LOAD RESISTANCE (Ω)

00511-004

100

| 0 5 10 15 20 |  |  |  |
| --- | --- | --- | --- |

SUPPLY VOLTAGE (±V)

Figure 4. Output Voltage Swing vs. Load Resistance

Figure 7. Open-Loop Gain vs. Supply Voltage

Rev. H | Page 6 of 20

00511-011

| AD549 |  |  |  |
| --- | --- | --- | --- |

| 3000 50 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

45

OPEN-LOOP GAIN (V/mV)

1000                                                                                                                                                                    40

35

INPUT CURRENT (fA)

300                                                                                                                                                                    30

00511-008

25

100

| –55 –25 5 35 65 95 125 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

TEMPERATURE (°C)

20

0                         5                        10                       15                       20

POWER SUPPLY VOLTAGE (±V)

Figure 8. Open-Loop Gain vs. Temperature

30

25

20

ΔIVOSI (µV)

15

10

5

0

00511-009

0             1             2             3             4             5             6             7

WARM-UP TIME (Minutes)

Figure 9. Change in Offset Voltage vs. Warm-Up Time

00511-012

Figure 11. Input Bias Current vs. Power Supply Voltage

| 160 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

NOISE SPECTRAL DENSITY (nV/  Hz)

140

120

100

80

60

40

20

10                               100                               1k                               10k

FREQUENCY (Hz)

Figure 12. Input Voltage Noise Spectral Density

| 50 100k |  |  |  |
| --- | --- | --- | --- |

INPUT NOISE VOLTAGE (µV p-p)

45                                                                                                                                                                 10k

INPUT CURRENT (fA)

40                                                                                                                                                                   1k

35                                                                                                                                                                 100

30                                                                                                                                                                   10

25                                                                                                                                                                     1

00511-010

20                                                                                                                                                                  0.1

WHENEVER JOHNSON NOISE IS GREATER THAN AMPLIFIER NOISE, AMPLIFIER NOISE CAN BE CONSIDERED NEGLIGIBLE FOR THE APPLICATION

1kHz BANDWIDTH RESISTOR

JOHNSON NOISE

00511-013

10Hz BANDWIDTH AMPLIFIER GENERATED NOISE

–10                      –5                        0                         5                        10

COMMON-MODE VOLTAGE (V)

Figure 10. Input Bias Current vs. Common-Mode Voltage

100k           1M            10M          100M           1G            10G          100G

SOURCE RESISTANCE (Ω)

Figure 13. Noise vs. Source Resistance

Rev. H | Page 7 of 20

AD549

100

80

OEPN-LOOP GAIN (dB)

60

40

20

0

–20

–40

| 100 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

80

PHASE MARGIN (Degrees)

60

40

20

0

00511-014

–20

–40

| 120 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | +PSS | R |  |
|  |  | –PSS | R |  |  |

POWER SUPPLY REJECTION RATIO (dB)

100

80

60

40

20

0

–20

10             100             1k             10k           100k           1M            10M

FREQUENCY (Hz)

Figure 14. Open-Loop Frequency Response

40

10             100             1k             10k           100k           1M

FREQUENCY (Hz)

Figure 17. PSRR vs. Frequency Response

10

00511-017

10M

35

OUTPUT VOLTAGE SWING (V)

30                                                                                                                                                                     5

25

OUTPUT VOLTAGE SWING (V)

20                                                                                                                                                                     0

15

10                                                                                                                                                                   –5

5

10mV

5mV

1mV

10mV

00511-018

1mV

0

10                100                1k                10k              100k               1M

00511-015

–10

0                   1                   2                   3                   4                   5

FREQUENCY (Hz)

Figure 15. Large Signal Frequency Response

SETTLING TIME (µs)

Figure 18. Output Voltage Swing and Error vs. Settling Time

| 100 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

COMMON-MODE REJECTION RATIO (dB)

80

60

40

20

0

–20

10             100             1k             10k           100k           1M

00511-016

FREQUENCY (Hz)

Figure 16. CMRR vs. Frequency

10M

Rev. H | Page 8 of 20

AD549

+VS

2               7

0.1µF

VIN

10kΩ

10kΩ

2

+VS

7

0.1µF

AD549      5

RL             CL

VOUT

SQUARE

AD549      5

VOUT

VIN           3

4

0.1µF

10kΩ

00511-019

100pF

WAVE INPUT

3               4

0.1µF

RL

10kΩ

CL

00511-022

100pF

SQUARE WAVE INPUT

–VS

–VS

Figure 19. Unity-Gain Follower

Figure 22. Unity-Gain Inverter

5V

5V

00511-020

5µs

Figure 20. Unity-Gain Follower Large Signal Pulse Response

00511-023

5µs

Figure 23. Unity-Gain Inverter Large Signal Pulse Response

10mV

10mV

00511-021

1µs

00511-024

1µs

Figure 21. Unity-Gain Follower Small Signal Pulse Response

Figure 24. Unity-Gain Inverter Small Signal Pulse Response

Rev. H | Page 9 of 20

AD549

FUNCTIONAL DESCRIPTION

MINIMIZING INPUT CURRENT

The AD549 is optimized for low input current and offset voltage. Careful attention to how the amplifier is used reduces input currents in actual applications.

Keep the amplifier operating temperature as low as possible to minimize input current. Like other JFET input amplifiers, the AD549 input current is sensitive to chip temperature, rising by a factor of 2.3 for every 10°C. Figure 25 is a plot of the AD549 input current vs. ambient temperature.

CIRCUIT BOARD  NOTES

A number of physical phenomena generate spurious currents that degrade the accuracy of low current measurements. Figure 27 is a schematic of a current to voltage (I-to-V) converter with these parasitic currents modeled.

CF

RF

2

| 1nA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

100pA

fS                                3

AD549

8

6                   + VOUT

–

INPUT BIAS CURRENT

10pA

V    dCP         dV

00511-027

RP                        CP          II' = R  + dT  V + dT CP

P

1pA

100fA

10fA

1fA

–55            –25              5               35              65

TEMPERATURE (°C)

00511-025

95             125

VS

Figure 27. Sources of Parasitic Leakage Currents

Finite resistance from input lines to voltages on the board, modeled by Resistor RP, results in parasitic leakage. Insulation resistance of more than 1015 Ω must be maintained between the amplifier signal and supply lines to capitalize on the low input currents of the AD549. Standard PCB material does not

have high enough insulation resistance; therefore, connect the

Figure 25. Input Bias Current vs. Ambient Temperature

On-chip power dissipation raises the chip operating tempera- ture, causing an increase in input bias current. Due to the low quiescent supply current of the AD549, the chip temperature is less than 3°C higher than its ambient temperature when the (unloaded) amplifier is operating with 15 V supplies. The difference in the input current is negligible.

00511-026

However, heavy output loads can cause a significant increase in chip temperature and a corresponding increase in the input current. Maintaining a minimum load resistance of 10 Ω is recommended. Input current vs. additional power dissipation due to output drive current is plotted in Figure 26.

| 6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BASED ON TYPICAL IB = 40fA |  |  |  |  |

NORMALIZED INPUT BIAS CURRENT

5

4

3

2

1

input leads of the AD549 to standoffs made of insulating material with adequate volume resistivity (that is, Teflon®). The surface of the insulator must be kept clean to preserve surface resistivity. For Teflon, an effective cleaning procedure consists of swabbing the surface with high grade isopropyl alcohol, rinsing with deionized water, and baking the board at 80°C for

10 minutes.

In addition to high volume and surface resistivity, other proper- ties are desirable in the insulating material chosen. Resistance

to water absorption is important because surface water films drastically reduce surface resistivity. The insulator chosen should also exhibit minimal piezoelectric effects (charge emission due to mechanical stress) and triboelectric effects (charge generated by friction). Charge imbalances generated by these mechanisms can appear as parasitic leakage currents. These effects are modeled by Variable Capacitor CP in Figure 27. Table 3 lists various insulators and their properties.1

Guarding the input lines by completely surrounding them with a metal conductor biased near the potential of the input lines has two major benefits. First, parasitic leakage from the signal line is reduced because the voltage between the input line and the guard is very low. Second, stray capacitance at the input node is minimized. Input capacitance can substantially degrade signal bandwidth and the stability of the I-to-V converter.

0          25         50         75        100       125       150       175       200

ADDITIONAL INTERNAL POWER DISSIPATION (mW)

Figure 26. Input Bias Current vs. Additional Power Dissipation

1 Electronic Measurements, pp. 15–17, Keithley Instruments, Inc., Cleveland, Ohio, 1977.

Rev. H | Page 10 of 20

AD549

The case of the AD549 is connected to Pin 8 so that it can be bootstrapped near the input potential. This minimizes pin leakage and input common-mode capacitance due to the case. Guard schemes for inverting and noninverting amplifier topologies are illustrated in Figure 28 and Figure 29.

CF

GUARD

RF

OFFSET NULLING

The AD549 input offset voltage can be nulled by using balance Pin 1 and Pin 5, as shown in Figure 30. Nulling the input offset voltage in this fashion introduces an added input offset voltage drift component of 2.4 μV/°C per mV of nulled offset (a maxi- mum additional drift of 0.6 μV/°C for the AD549K, 1.2 μV/°C for the AD549L, and 2.4 μV/°C for the AD549J).

+VS

2

IN                                            3

AD549      6

8

00511-028

+ VOUT

–

2              7

AD549      6

5

3                1

4

00511-030

+ VOUT

–

Figure 28. Inverting Amplifier with Guard

–VS

10kΩ

GUARD

+ VS

–

3

AD549

2               8

VOUT

6                   +

RF

Figure 30. Standard Offset Null Circuit

The approach in Figure 31 can be used when the amplifier is used as an inverter. This method introduces a small voltage referenced to the power supplies in series with the positive input terminal of the amplifier. The amplifier input offset voltage drift with temperature is not affected. However, variation of the power supply voltages causes offset shifts.

00511-029

RI

–

Figure 29. Noninverting Amplifier with Guard

Other guidelines include keeping the circuit layout as compact

RF I

R

2

+                         AD549      6

VI

3

+ VOUT

–

as possible and keeping the input lines short. Keeping the assembly rigid and minimizing sources of vibration reduces triboelectric

–

499kΩ      499kΩ

+VS

100kΩ

and piezoelectric effects. All precision, high impedance circuitry

requires shielding against interference noise. Use low noise coaxial

200Ω

0.1µF

00511-031

–VS

or triaxial cables for remote connections to the input signal lines.

Figure 31. Alternate Offset Null Circuit for Inverter

Table 3. Insulating Materials and Characteristics

| Material | Volume Resistivity (V to CM) | Minimal Triboelectric Effect 1 | Minimal Piezoelectric Effect1 | Resistance to Water Absorption1 |
| --- | --- | --- | --- | --- |
| Teflon Kel-F® Sapphire Polyethylene Polystyrene Ceramic Glass Epoxy PVC Phenolic | 1017 to 1018 1017 to 1018 1016 to 1018 1014 to 1018 1012 to 1018 1012 to 1014 1010 to 1017 1010 to 1015 105 to 1012 | W W M M W W W G W | W M G G M M M M G | G G G M M W W G W |

1 G: good with regard to property; M: moderate with regard to property; W: weak with regard to property.

Rev. H | Page 11 of 20

AD549

AC RESPONSE WITH HIGH VALUE SOURCE AND FEEDBACK RESISTANCE

Source and feedback resistances greater than 100 kΩ magnify the effect of the input capacitances (stray and inherent to

the AD549) on the ac behavior of the circuit. The effects of

common-mode and differential input capacitances should be taken into account because the circuit bandwidth and stability can be adversely affected.

In an inverting configuration, the differential input capacitance forms a pole in the loop transmission of the circuit. This can create peaking in the ac response and possible instability. A feedback capacitance can be used to stabilize the circuit. The inverter pulse response with RF and RS equal to 1 MΩ appears

in Figure 34. Figure 35 shows the response of the same circuit with a 1 pF feedback capacitance. Typical differential input capacitance for the AD549 is 1 pF.

00511-032

10mV                                                                5µs

Figure 32. Follower Pulse Response from 1 MΩ Source Resistance, Case Not Bootstrapped

00511-034

10mV                                                                5µs

Figure 34. Inverter Pulse Response with 1 MΩ Source and Feedback Resistance

10mV

5µs

00511-035

10mV                                                               5µs

Figure 33. Follower Pulse Response from 1 MΩ Source Resistance, Case Bootstrapped

In a follower, the source resistance and input common-mode capacitance form a pole that limits the bandwidth to ½πRSCS. Bootstrapping the metal case by connecting Pin 8 to the output minimizes capacitance due to the package. Figure 32 and Figure 33 show the follower pulse response from a 1 MΩ source resistance with and without the package connected to the output. Typical common-mode input capacitance for the AD549 is 0.8 pF.

00511-033

Figure 35. Inverter Pulse Response with 1 MΩ Source and Feedback Resistance, 1 pF Feedback Capacitance

COMMON-MODE INPUT VOLTAGE OVERLOAD

The rated common-mode input voltage range of the AD549 is from 3 V less than the positive supply voltage to 5 V greater than the negative supply voltage. Exceeding this range degrades the CMRR of the amplifier. Driving the common-mode voltage above the positive supply causes the amplifier output to saturate at the upper limit of the output voltage. Recovery time is typically 2 μs after the input has been returned to within the normal operating range. Driving the input common-mode voltage within 1 V of the negative supply causes phase reversal of the output signal. In this case, normal operation typically resumes within 0.5 μs of the

input voltage returning within range.

Rev. H | Page 12 of 20

AD549

DIFFERENTIAL INPUT VOLTAGE OVERLOAD

A plot of the AD549 input currents vs. differential input voltage (defined as VIN+ − VIN−) appears in Figure 36. The input current at either terminal stays below a few hundred femtoamps until one input terminal is forced higher than 1 V

SOURCE

RPROTECT

3

00511-038

AD549      6

2

to 1.5 V above the other terminal. Under these conditions, the input current limits at 30 μA.

| 100µ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | II | N– |  |  |  |  |  |  |  | II | N+ |  |  |  |  |

10µ

1µ

INPUT CURRENT (A)

100n

10n

1n

Figure 38. Follower with Input Current Limit

Figure 39 is a schematic of the AD549 as an inverter with an input voltage clamp. Bootstrapping the clamp diodes at the inverting input minimizes the voltage across the clamps and keeps the leakage due to the diodes low. Use low leakage diodes, such as the FD333s, and shield them from light to prevent photo- currents from being generated. Even with these precautions, the diodes measurably increase input current and capacitance.

100p

10p

1p

100f

10f

SOURCE

PROTECT DIODES

RF

2

00511-039

AD549      6

3

00511-036

–5      –4      –3      –2      –1       0        1        2        3        4        5

DIFFERENTIAL INPUT VOLTAGE (V) (VIN+ – VIN–)

Figure 36. Input Current vs. Differential Input Voltage

INPUT PROTECTION

The AD549 safely handles any input voltage within the supply voltage range. Subjecting the input terminals to voltages beyond the power supply can destroy the device or cause shifts in input current or offset voltage if the amplifier is not protected.

A protection scheme for the amplifier as an inverter is shown in Figure 37. RP is chosen to limit the current through the inverting input to 1 mA for expected transient (less than 1 sec) overvoltage conditions, or to 100 μA for a continuous overload. Because RP is inside the feedback loop and is much lower in value than the amplifier input resistance, it does not affect the dc gain of the inverter. However, the Johnson noise of the resistor adds root sum of squares to the amplifier input noise.

RF

Figure 39. Input Voltage Clamp with Diodes

SAMPLE-AND-DIFFERENCE CIRCUIT TO MEASURE ELECTROMETER LEAKAGE CURRENTS

There are a number of methods used to test electrometer leakage currents, including current integration and direct I-to-V con- version. Regardless of the method used, board and interconnect cleanliness, proper choice of insulating materials (such as Teflon or Kel-F), correct guarding and shielding techniques, and care

in physical layout are essential to making accurate leakage measurements.

Figure 40 is a schematic of the sample-and-difference circuit. It uses two AD549 electrometer amplifiers (A and B) as I-to-V converters with high value (1010 Ω) sense resistors (RSa and RSb). R1 and R2 provide for an overall circuit sensitivity of

10 fA/mV (10 pA full scale). CC and CF provide noise suppression and loop compensation. CC should be a low leakage polystyrene capacitor. An ultralow leakage Kel-F test socket is used for con-

SOURCE

RPROTECT

CF

2

00511-037

AD549      6

3

tacting the device under test. Rigid Teflon coaxial cable is used to make connections to all high impedance nodes. The use of rigid coaxial cable affords immunity to error induced by mechan- ical vibration and provides an outer conductor for shielding. The entire circuit is enclosed in a grounded metal box.

Figure 37. Inverter with Input Current Limit

In the corresponding version of this scheme for a follower, shown in Figure 38, RP and the capacitance at the positive input terminal produce a pole in the signal frequency response at a

f = ½πRC. Again, the Johnson noise, RP, adds to the input voltage noise of the amplifier.

Rev. H | Page 13 of 20

AD549

The test apparatus is calibrated without a device under test present. After power is turned on, a 5 minute stabilization period is required. First, VERR1 and VERR2 are measured. These voltages are the errors caused by the offset voltages and leakage currents of the I-to-V converters.

VERR1 = 10 (VOSA – IBA × RSa)

VERR2 = 10 (VOSB – IBB × RSb)

Although a series of devices can be tested after only one calibra- tion measurement, calibration should be updated periodically

to compensate for any thermal drift of the I-to-V converters or changes in the ambient environment. Laboratory results have shown that repeatable measurements within 10 fA can be realized when this apparatus is properly implemented. These results are achieved in part by the design of the circuit, which eliminates relays and other parasitic leakage paths in the high impedance

CC

20pF

RSa

1010Ω

2

A

CF

0.1µF

R2

9.01kΩ

R1

1kΩ

CAL/TEST

signal lines, and in part by the inherent cancellation of errors through the calibration and measurement procedure.

PHOTODIODE INTERFACE

The low input current and low input offset voltage of the AD549 make it an excellent choice for very sensitive photodiode preamps (see Figure 41). The photodiode develops a signal current, IS,

VOS

I (+)

–

DEVICE UNDER

AD549

3               8

6

GUARD

+ VERR1/VA

–

VOUT

–

equal to

IS = R × P

where P is light power incident on the diode surface, in watts, and R is the photodiode responsivity in amps/watt. RF converts the signal current to an output voltage

+

I (–)

TEST

R2

9.01kΩ

R1

1kΩ

CF

0.1µF

3               8

B

VERR2/VB

VOUT  = RF × IS

RF

109Ω

CF

AD549      6                      +

2

10pF

2

I

S

AD549      6

5

10kΩ

+

CC

20pF

RSb

1010Ω

CF

0.1µF

R2

9.01kΩ

R1

3                1

4

–VS

1µF              V

00511-041

–

OUT

1kΩ

Figure 40. Sample and Difference Circuit for Measuring

Electrometer Leakage Currents

Once measured, these errors are subtracted from the readings taken with a device under test present. Amplifier B closes the feedback loop to the device under testing in addition to pro- viding the I-to-V conversion. The offset error of the device

Figure 41. Photodiode Preamp

00511-040

The dc error sources and an equivalent circuit for a small area

(0.2 mm square) photodiode are indicated in Figure 42.

RF

109Ω

CF

10pF

under testing appears as a common-mode signal and does not affect the test measurement. As a result, only the leakage current of the device under testing is measured.

IS                RS

109Ω

CS

20pF

IS–

–  VOS  +

00511-042

A                         + VOUT

–

VA  – VERR1 = 10[RSa × IB(+)]

VX  – VERR2 = 10[RSb × IB(–)]

Figure 42. Photodiode Preamp DC Error Sources

Rev. H | Page 14 of 20

AD549

Input current, IB, contributes an output voltage error, VE1, proportional to the feedback resistance

VE1 = IB × RF

The input voltage offset of the op amp causes an error current through the photodiode shunt resistance, RS

I = VOS/RS

The error current results in an error voltage (VE2) at the amplifier output equal to

gain that multiplies the op amp input voltage noise contribu- tion. A single-pole filter at the output of the amplifier limits the op amp output voltage noise bandwidth to 26 Hz, comparable

to the signal bandwidth. This greatly improves the signal-to- noise ratio of the preamplifier (in this case, by a factor of 3).

10µ

IF AND CS, NO FILTERS

IF AND CS, WITH FILTERS

AD549

VE2 = (1 + RF/RS)VOS

Given typical values of photodiode shunt resistance (on the order of 109 Ω), RF/RS can easily be greater than 1, especially if a large feedback resistance is used. Also, RF/RS increases with tempera- ture because photodiode shunt resistance typically drops by a factor of 2 for every 10°C rise in temperature. An op amp with low offset voltage and low drift must be used to maintain accuracy.

The AD549K offers a guaranteed maximum 0.25 mV offset

1µ

VOLTAGE NOISE CONTRIBUTIONS NOISE SPECTRAL DENSITY (nV/  Hz)

100n

10n

EN CONTRIBUTION, WITH FILTER

OPEN-LOOP GAIN

00511-044

EN CONTRIBUTION, NO FILTER

voltage and 5 mV/°C drift for very sensitive applications.

Photodiode Preamp Noise

Noise limits the signal resolution obtainable with the preamp. The output voltage noise divided by the feedback resistance is the minimum current signal that can be detected. This mini- mum detectable current divided by the responsivity of the photodiode represents the lowest light power that is detectable by the preamp.

Noise sources associated with the photodiode, amplifier, and feedback resistance are shown in Figure 43; Figure 44 is the spectral density vs. frequency plot of the contribution of each of the noise sources to the output voltage noise (circuit parameters in Figure 42 are assumed). The rms contribution of each noise source to the total output voltage noise is obtained by

integrating the square of its spectral density function over frequency. The rms value of the output voltage noise is the square root of the sum of all contributions. Minimizing the total area under these curves optimizes the resolution of the preamplifier for a given bandwidth.

IF

RF

00511-043

CF

IS                RS             CS             IN                                   A

EN

1              10             100             1k             10k           100k           1M

FREQUENCY (Hz)

Figure 44. Spectral Density of the Photodiode Preamp Noise

Sources vs. Frequency

LOG RATIO AMPLIFIER

Logarithmic ratio circuits are useful for processing signals with wide dynamic range. The 60 fA maximum input current of the AD549L makes it possible to build a log ratio amplifier with

1% log conformance for input currents ranging from 10 pA to

1 mA, a dynamic range of 160 dB.

The log ratio amplifier in Figure 45 provides an output voltage proportional to the log base 10 of the ratio of Input Current I1 and Input Current I2. Resistor R1 and Resistor R2 are provided for voltage inputs. Because NPN devices are used in the feedback loop of the front-end amplifiers that provide the log transfer function, the output is valid only for positive input voltages and input currents. The input currents set the Collector Current IC1 and Collector Current IC2 of a matched pair of log transistors, Q1 and Q2, to develop Voltage VA and Voltage VB

VA, VB = –(kT/q)ln IC/IES

where IES is the saturation current of the transistor.

The difference of VA and VB is taken by the subtractor section to obtain

VC = (kT/q)ln(IC2/IC1)

VC is scaled up by the ratio of (R9 + R10)/R8, which is equal to approximately 16 at room temperature, resulting in the output

voltage

Figure 43. Photodiode Preamp Noise Sources

The photodiode preamp in Figure 41 can detect a signal current of 26 fA rms at a bandwidth of 16 Hz, which, assuming a photodiode responsivity of 0.5 A/W, translates to a 52 fW rms minimum detectable power. The photodiode used has a high source resistance and low junction capacitance. CF sets the

signal bandwidth with RF and also limits the peak in the noise

VOUT  = 1 V × log(IC2/IC1)

R8 is a resistor with a positive 3500 ppm/°C temperature coeffi- cient to provide the necessary temperature compensation. The parallel combination of R15 and R7 is provided to keep the gain of the subtractor section for positive and negative inputs matched over temperature.

Rev. H | Page 15 of 20

AD549

Frequency compensation is provided by R11, R12, C1, and C2. The bandwidth of the circuit is 300 kHz at input signals greater than 50 μA; bandwidth decreases smoothly with decreasing signal levels.

To trim the circuit, set the input currents to 10 μA and trim the A3 offset using the trim potentiometer of the amplifier for the output to equal 0. Next, set I1 to 1 μA and adjust the output to equal 1 V by trimming R10. Additional offset trims on Ampli- fier A1 and Amplifier A2 can be used to increase the voltage input accuracy and dynamic range.

The very low input current of the AD549 makes this circuit useful over a very wide range of signal currents. The total input current (which determines the low level accuracy of the circuit) is the sum of the amplifier input current, the leakage across the compensating capacitor (negligible if a polystyrene or Teflon capacitor is used), and the collector-to-collector and collector- to-base leakages of one side of the dual log transistors. The magnitudes of these last two leakages depend on the amplifier input offset voltage and are typically less than 10 fA with 1 mV offsets. The low level accuracy is limited primarily by the amplifier input current, only 60 fA maximum when the AD549L is used.

The effects of the emitter resistance of Q1 and Q2 can degrade circuit accuracy at input currents above 100 μA. The networks

composed of R13, D1, R16, R14, D2, and R17 compensate for these errors, so that this circuit has less than a 1% log confor- mance error at 1 mA input currents. The correct value for R13 and R14 depends on the type of log transistors used. The 49.9 kΩ resistors were chosen for use with LM394 transistors. Smaller resistance values are needed for smaller log transistors.

TEMPERATURE COMPENSATED pH PROBE AMPLIFIER

A pH probe can be modeled as an mV-level voltage source with a series source resistance dependent on the electrode composition and configuration. The glass bulb resistance of a typical pH electrode pair falls between 106 Ω and 109 Ω. It is therefore important to select an amplifier with low enough input currents such that the voltage drop produced by the amplifier input bias current and the electrode resistance does not become an appreciable percentage of a pH unit.

The circuit in Figure 46 illustrates the use of the AD549 as a pH probe amplifier. As with other electrometer applications, the use of guarding, shielding, and Teflon standoffs is necessary to capitalize on the AD549 low input current. If an AD549L (60 fA maximum input current) is used, the error contributed by the input current is held below 60 μV for pH electrode source impedances up to 109 Ω. Input offset voltages (which can be trimmed) are below 0.5 mV.

10kΩ

4                   V1

3                1          OFFSET

FOR EACH AMPLIFIER PIN 7

0.1µF

0.1µF

+VS

I1 IN                     2

R1

A1       5

AD549      6

C1

100pF

D3

R11

4.99kΩ

PIN 4                                   –VS

Q1, Q2 = LM394

DUAL LOG TRANSISTORS

R15

1kΩ

*

V1 IN

10kΩ

Q1  A                    R3

20kΩ

R5

20kΩ

R7

15kΩ

R14

49.9kΩ

D2

R16

10Ω

D1

3

A3

AD549  5

6

2                1

R10

VOUT

SCALE FACTOR

V2 IN

R2

10kΩ

R17

10Ω

Q2

R13

49.9kΩ

R4

4                  10kΩ  2kΩ

OUTPUT OFFSET

R6                       R9

ADJ

C2           B

100pF

20kΩ

4.99kΩ

20kΩ   *

R8

1kΩ

14.3kΩ

V2

VOUT = 1V × LOG10

V

1

I2

I2 IN

2

A2

AD549  5

VOUT = 1V × LOG10

I

D4                                                                               1

D1, D4 1N4148 DIODES

6

R8, R15 1kΩ + 350 ppm/°C TC RESISTOR

3                1

4                  10kΩ

V2

OFFSET

00511-045

*TELLAB QB1 OR PRECISION RESISTOR PT146

ALL OTHER RESISTORS ARE 1% METAL FILM

Figure 45. Log Ratio Amplifier

Rev. H | Page 16 of 20

AD549

The pH probe output is ideally 0 V at a pH of 7, independent

of temperature. The slope of the transfer function of the probe, though predictable, is temperature dependent (−54.2 mV/pH at

0°C and −74.04 mV/pH at 100°C). By using an AD590 tempera- ture sensor and an AD534 analog divider, an accurate temperature compensation network can be added to the basic pH probe ampli- fier. Table 4 shows voltages at various points, thereby illustrating

the compensation. The AD549 is set for a noninverting gain of

13.51. The output of the AD590 circuitry (Point C) is equal to

10 V at 100°C and decreases by 26.8 mV/°C. The output of the AD534 analog divider (Point D) is a temperature-compensated output voltage centered at 0 V for a pH of 7 and has a transfer function of –1.00 V/pH unit. The output range spans from

−7.00 V (pH = 14) to +7.00 V (pH = 0).

+15V

0.1µF

pH PROBE OUTPUT

(A)

0.1µF

3                7

(B)

14

AD534

(D)

AD549      6

10   Z2

OUT  12

OUTPUT

2

8

12kΩ

1kΩ

4

0.1µF

1kΩ

(C)

11   Z1

1    X1

2    X2

Y2   7

Y1   6

8

0.1µF

AD590      + IN STAINLESS

STEEL PROBE

OR AC2626      –

SCALE FACTOR ADJUST

+15V

26.6kΩ

00511-046

–15V

Figure 46. Temperature Compensated pH Amplifier

Table 4. Illustration of Temperature Compensation

| Probe Temperature (°C) | Point |  |  |  |
| --- | --- | --- | --- | --- |
|  | A (Probe Output) (mV) | B (A × 13.51) (V) | C (AD590 Output) (V) | D (10 × (B ÷ C)) (V) |
| 0 25 37 60 100 | 54.20 59.16 61.54 66.10 74.04 | 0.732 0.799 0.831 0.893 1.000 | 7.32 7.99 8.31 8.93 10.00 | 1.00 1.00 1.00 1.00 1.00 |

Rev. H | Page 17 of 20

AD549

OUTLINE DIMENSIONS

REFERENCE PLANE

0.1850 (4.70)

0.3700 (9.40)

0.3350 (8.51)

0.3350 (8.51)

0.3050 (7.75)

0.1650 (4.19)

0.5000 (12.70) MIN

0.2500 (6.35) MIN

0.0500 (1.27) MAX

0.2000 (5.08) BSC

0.1000 (2.54) BSC

5

4

3

0.1600 (4.06)

0.1400 (3.56)

6

7

0.0450 (1.14)

0.0190 (0.48)

0.0160 (0.41)

0.1000 (2.54) BSC

2                  8

1

0.0270 (0.69)

0.0400 (1.02) MAX                                                                                 0.0340 (0.86)

0.0400 (1.02)

0.0100 (0.25)

0.0210 (0.53)

0.0160 (0.41)

BASE & SEATING PLANE

0.0280 (0.71)

45° BSC

022306-A

COMPLIANT TO JEDEC STANDARDS MO-002-AK CONTROLLING DIMENSIONS ARE IN INCHES; MILLIMETER  DIMENSIONS (IN PARENTHESES) ARE ROUNDED-OFF INCH EQUIVALENTS FOR

REFERENCE ONLY AND ARE NOT APPROPRIATE FOR USE IN DESIGN.

Figure 47. 8-Lead Metal Can [TO-99] (H-08)

Dimensions shown in inches and (millimeters)

ORDERING GUIDE

| Model | Temperature Range | Package Description | Package Option |
| --- | --- | --- | --- |
| AD549JH AD549JHZ1 AD549KH AD549KHZ1 AD549LH AD549LHZ1 AD549SH/883B | 0°C to +70°C 0°C to +70°C 0°C to +70°C 0°C to +70°C 0°C to +70°C 0°C to +70°C −55°C to +125°C | 8-Lead Metal Can (TO-99) 8-Lead Metal Can (TO-99) 8-Lead Metal Can (TO-99) 8-Lead Metal Can (TO-99) 8-Lead Metal Can (TO-99) 8-Lead Metal Can (TO-99) 8-Lead Metal Can (TO-99) | H-08 H-08 H-08 H-08 H-08 H-08 H-08 |

1 Z = RoHS Compliant Part.

Rev. H | Page 18 of 20

AD549

NOTES

Rev. H | Page 19 of 20

AD549

NOTES

©2002–2008 Analog Devices, Inc. All rights reserved. Trademarks and registered  trademarks  are the  property  of their respective  owners.

D00511-0-3/08(H)

Rev. H | Page 20 of 20