Ultraprecision

Operational Amplifier

OP177

FEATURES

Ultralow offset voltage

PIN CONFIGURATION

TA = 25°C, 25 μV maximum

Outstanding offset voltage drift 0.1 μV/°C maximum

Excellent open-loop gain and gain linearity

VOS TRIM   1

–IN   2

+IN   3

OP177

TOP VIEW

8   VOS TRIM

7   V+

6   OUT

12 V/μV typical

CMRR: 130 dB minimum

PSRR: 115 dB minimum

Low supply current 2.0 mA maximum

Fits industry-standard precision op amp sockets

00289-001

V–   4                                  5   NC (Not to Scale)

NC = NO CONNECT

Figure 1. 8-Lead PDIP (P-Suffix),

8-Lead SOIC (S-Suffix)

GENERAL DESCRIPTION

The OP177 features one of the highest precision performance of any op amp currently available. Offset voltage of the OP177 is only 25 μV maximum at room temperature. The ultralow VOS of the OP177 combines with its exceptional offset voltage drift (TCVOS) of 0.1 μV/°C maximum to eliminate the need for external VOS  adjustment and increases system accuracy over temperature.

The OP177 open-loop gain of 12 V/μV is maintained over the full ±10 V output range. CMRR of 130 dB minimum, PSRR of

120 dB minimum, and maximum supply current of 2 mA are

just a few examples of the excellent performance of this

operational amplifier. The combination of outstanding specifications of the OP177 ensures accurate performance in high closed-loop gain applications.

This low noise, bipolar input op amp is also a cost effective alternative to chopper-stabilized amplifiers. The OP177

provides chopper-type performance without the usual problems

of high noise, low frequency chopper spikes, large physical size, limited common-mode input voltage range, and bulky external storage capacitors.

The OP177 is offered in the −40°C to +85°C extended industrial temperature ranges. This product is available in 8-lead PDIP, as well as the space saving 8-lead SOIC.

FUNCTIONAL BLOCK DIAGRAM

V+

R2A*

R1A

2B

(OPTIONAL NULL)

R2B* R1B

C1                                                                                         R7

Q19

NONINVERTING INPUT

INVERTING INPUT

V–

Q5

R3

Q21

R4          Q22

Q7

Q23

Q24

Q3     Q6

Q1

Q8

Q4

Q27

Q26

Q25

Q2

Q9

Q11      Q12

C3                      C2

R5

Q14

Q13

Q10

Q17

Q16

Q15

R6

Q18

00289-002

R8

R9

R10

Q20

OUTPUT

*R2A AND R2B ARE ELECTRONICALLY ADJUSTED ON CHIP AT FACTORY.

Figure 2. Simplified Schematic

Rev. F

Information furnished by Analog Devices is believed to be accurate and reliable. However, no responsibility is assumed by Analog Devices for its use, nor for any infringements of patents or other rights of thirdparties that may result from its use. Specifications subject tochange without notice. No license is granted by implication or otherwise under any patent or patent rights of Analog Devices. Trademarks andregisteredtrademarks arethe propertyoftheir respective owners.

One Technology Way, P.O. Box 9106,  Norwood,  MA 02062-9106, U.S.A. Tel: 781.329.4700                                                           www.analog.com Fax: 781.461.3113     ©1995–2009 Analog Devices, Inc. All rights reserved.

OP177

TABLE OF CONTENTS

Features .............................................................................................. 1

Pin Configuration............................................................................. 1

General Description ......................................................................... 1

Functional Block Diagram .............................................................. 1

Revision History ............................................................................... 2

Specifications..................................................................................... 3

Electrical Characteristics ............................................................. 3

Test Circuits................................................................................... 4

Absolute Maximum Ratings............................................................ 5

Thermal Resistance ...................................................................... 5

ESD Caution.................................................................................. 5

Typical Performance Characteristics ............................................. 6

Applications Information .................................................................9

Gain Linearity ................................................................................9

Thermocouple Amplifier with Cold-Junction

Compensation........................................................................................9

Precision High Gain Differential Amplifier  ........................... 10

Isolating Large Capacitive  Loads.............................................. 10

Bilateral Current Source ............................................................ 10

Precision Absolute Value Amplifier......................................... 10

Precision Positive Peak Detector.............................................. 12

Precision Threshold Detector/Amplifier ................................ 12

Outline Dimensions ....................................................................... 13

Ordering Guide .......................................................................... 14

REVISION HISTORY

3/09—Rev.  E to Rev. F

Added Figure 23, Renumbered Sequentially ................................ 8

Updated Outline Dimensions ....................................................... 13

5/06—Rev. D to Rev. E

Changes to Figure 1 .......................................................................... 1

Change to Specifications Table 1 .................................................... 3

Changes to Specifications Table 2................................................... 4

Changes to Table 3............................................................................ 5

Changes to Figure 23 and Figure 24............................................... 9

Changes to Figure 32...................................................................... 12

Updated the Ordering Guide ........................................................ 14

4/06—Rev. C to Rev. D

Change to Pin Configuration Caption........................................... 1

Changes to Features.......................................................................... 1

Change to Table 2 ............................................................................. 4

Change to Figure 2 ........................................................................... 4

Changes to Figure 10 and Figure 11............................................... 6

Changes to Figure 12 through Figure 17 ........................................7

Changes to Figure 18 through Figure 22 ........................................8

Change to Figure 27 ....................................................................... 10

Changes to Figure 30 and Figure 31............................................. 11

Updated Outline Dimensions ....................................................... 13

Changes to Ordering Guide .......................................................... 13

1/05—Rev. B to Rev. C

Edits to Features.................................................................................1

Edits to General Description ...........................................................1

Edits to Pin Connections..................................................................1

Edits to Electrical Characteristics .............................................. 2, 3

Global deletion of references to OP177E ............................ 3, 4, 10

Edits to Absolute Maximum Ratings ..............................................5

Edits to Package Type .......................................................................5

Edits to Ordering Guide ...................................................................5

Edit to Outline Dimensions .......................................................... 11

11/95—Rev. 0: Initial Version

Rev. F | Page 2 of 16

OP177

SPECIFICATIONS

ELECTRICAL CHARACTERISTICS

@ VS = ±15 V, TA = 25°C, unless otherwise noted.

Table 1.

| Parameter | Symbol | Conditions | OP177F Min Typ Max | OP177G Min Typ Max | Unit |
| --- | --- | --- | --- | --- | --- |
| INPUT OFFSET VOLTAGE | VOS |  | 10 25 | 20 60 | μV |
| LONG-TERM INPUT OFFSET 1 Voltage Stability | ΔVOS/time |  | 0.3 | 0.4 | μV/mo |
| INPUT OFFSET CURRENT | IOS |  | 0.3 1.5 | 0.3 2.8 | nA |
| INPUT BIAS CURRENT | IB |  | −0.2 +1.2 +2 | −0.2 +1.2 +2.8 | nA |
| INPUT NOISE VOLTAGE | en | fO = 1 Hz to 100 Hz2 | 118 150 | 118 150 | nV rms |
| INPUT NOISE CURRENT | in | fO = 1 Hz to 100 Hz2 | 3 8 | 3 8 | pA rms |
| INPUT RESISTANCE Differential Mode3 | RIN |  | 26 45 | 18.5 45 | MΩ |
| INPUT RESISTANCE COMMON MODE | RINCM |  | 200 | 200 | GΩ |
| INPUT VOLTAGE RANGE4 | IVR |  | ±13 ±14 | ±13 ±14 | V |
| COMMON-MODE REJECTION RATIO | CMRR | VCM = ±13 V | 130 140 | 115 140 | dB |
| POWER SUPPLY REJECTION RATIO | PSRR | VS = ±3 V to ±18 V | 115 125 | 110 120 | dB |
| LARGE SIGNAL VOLTAGE GAIN | AVO | RL ≥ 2 kΩ, VO = ±10 V5 | 5000 12,000 | 2000 6000 | V/mV |
| OUTPUT VOLTAGE SWING | VO | RL ≥ 10 kΩ RL ≥ 2 kΩ RL ≥ 1 kΩ | ±13.5 ±14.0 ±12.5 ±13.0 ±12.0 ±12.5 | ±13.5 ±14.0 ±12.5 ±13.0 ±12.0 ±12.5 | V V V |
| SLEW RATE2 | SR | RL ≥ 2 kΩ | 0.1 0.3 | 0.1 0.3 | V/μs |
| CLOSED-LOOP BANDWIDTH2 | BW | AVCL = 1 | 0.4 0.6 | 0.4 0.6 | MHz |
| OPEN-LOOP OUTPUT RESISTANCE | RO |  | 60 | 60 | Ω |
| POWER CONSUMPTION | PD | VS = ±15 V, no load VS = ±3 V, no load | 50 60 3.5 4.5 | 50 60 3.5 4.5 | mW mW |
| SUPPLY CURRENT | ISY | VS = ±15 V, no load | 1.6 2 | 1.6 2 | mA |
| OFFSET ADJUSTMENT RANGE |  | RP = 20 kΩ | ±3 | ±3 | mV |

1 Long-term input offset voltage stability refers to the averaged trend line of VOS vs. time over extended periods after the first 30 days of operation. Excluding the initial hour of operation, changes in VOS during the first 30 operating days are typically less than 2.0 μV.

2 Sample tested.

3 Guaranteed by design.

4 Guaranteed by CMRR test condition.

5 To ensure high open-loop  gain throughout the ±10 V output range, AVO is tested at −10 V ≤ VO ≤ 0 V, 0 V ≤ VO ≤ +10 V, and –10 V ≤ VO ≤ +10 V.

Rev. F | Page 3 of 16

OP177

@ VS = ±15 V, −40°C ≤ TA ≤ +85°C, unless otherwise noted.

Table 2.

| Parameter | Symbol | Conditions | OP177F Min Typ Max | OP177G Min Typ Max | Unit |
| --- | --- | --- | --- | --- | --- |
| INPUT Input Offset Voltage Average Input Offset Voltage Drift1 Input Offset Current Average Input Offset Current Drift2 Input Bias Current Average Input Bias Current Drift2 Input Voltage Range 3 | VOS TCVOS IOS TCIOS IB TCIB IVR |  | 15 40 0.1 0.3 0.5 2.2 1.5 40 −0.2 +2.4 +4 8 40 ±13 ±13.5 | 20 100 0.7 1.2 0.5 4.5 1.5 85 +2.4 ±6 15 60 ±13 ±13.5 | μV μV/°C nA pA/°C nA pA/°C V |
| COMMON-MODE REJECTION RATIO | CMRR | VCM = ±13 V | 120 140 | 110 140 | dB |
| POWER SUPPLY REJECTION RATIO | PSRR | VS = ±3 V to ±18 V | 110 120 | 106 115 | dB |
| LARGE-SIGNAL VOLTAGE GAIN4 | AVO | RL ≥ 2 kΩ, VO = ±10 V | 2000 6000 | 1000 4000 | V/mV |
| OUTPUT VOLTAGE SWING | VO | RL ≥ 2 kΩ | ±12 ±13 | ±12 ±13 | V |
| POWER CONSUMPTION | PD | VS = ±15 V, no load | 60 75 | 60 75 | mW |
| SUPPLY CURRENT | ISY | VS = ±15 V, no load | 20 2.5 | 2 2.5 | mA |

1 TCVOS is sample tested.

2 Guaranteed by endpoint limits.

3 Guaranteed by CMRR test condition.

4 To ensure high open-loop  gain throughout the ±10 V output range, AVO is tested at −10 V ≤ VO ≤ 0 V, 0 V ≤ VO ≤ +10 V, and −10 V ≤ VO ≤ +10 V.

TEST CIRCUITS

200kΩ

50Ω

–

OP177

+

VOS =

00289-003

VO

VO

4000

Figure 3. Typical Offset Voltage Test Circuit

20kΩ

V+

– INPUT

+

–

OP177

+

V–

00289-004

OUTPUT VOS TRIM RANGE IS

TYPICALLY ±3.0mV

Figure 4. Optional Offset Nulling Circuit

20kΩ

+20V

–

OP177

+            PINOUTS SHOWN FOR

00289-005

P AND Z PACKAGES

–20V

Figure 5. Burn-In Circuit

Rev. F | Page 4 of 16

OP177

ABSOLUTE MAXIMUM RATINGS

Table 3.

Parameter                                                             Ratings Supply Voltage                                                      ±22 V Internal Power Dissipation1                                                         500 mW Differential Input Voltage                                      ±30 V Input Voltage                                                         ±22 V Output Short-Circuit Duration                              Indefinite

Storage Temperature Range                                   −65°C to +125°C

Operating Temperature Range                             −40°C to +85°C Lead Temperature (Soldering, 60 sec)                     300°C

DICE Junction Temperature (TJ)                              −65°C to +150°C

1 For supply voltages less than ±22 V, the absolute maximum input voltage is equal to the supply voltage.

Stresses above those listed under Absolute Maximum Ratings may cause permanent damage to the device. This is a stress rating only; functional operation of the device at these or any other conditions above those indicated in the operational section of this specification is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

THERMAL RESISTANCE

θJA is specified for worst-case mounting conditions, that is, θJA is specified for device in socket for PDIP; θJA is specified for device soldered to printed circuit board for SOIC package.

Table 4. Thermal Resistance

| Package Type | θJA | θJC | Unit |
| --- | --- | --- | --- |
| 8-Lead PDIP (P-Suffix) 8-Lead SOIC (S-Suffix) | 103 158 | 43 43 | °C/W °C/W |

ESD CAUTION

ESD (electrostatic  discharge)  sensitive device.  Electrostatic charges  as high  as 4000 V readily accumulate  on the human  body  and  test  equipment and  can discharge  without  detection.  Although  this product  features proprietary  ESD protection circuitry, permanent damage may  occur  on  devices  subjected to  high  energy electrostatic   discharges.  Therefore,  proper   ESD precautions  are   recommended  to   avoid   performance degradation or loss of functionality.

Rev. F | Page 5 of 16

OP177

TYPICAL PERFORMANCE  CHARACTERISTICS

2

TA = 25°C

RL = 10kΩ

1

0

INPUT VOLTAGE (µV) (NULLED TO 0mV @ VOUT = 0V)

–1

20

25

DEVICE IMMERSED IN

70° OIL BATH (20 UNITS)

30

35

40

ABSOLUTE CHANGE IN INPUT OFFSET VOLTAGE (µV)

45

00289-009

VS = ±15V

00289-006

–2

–10                      –5                        0                        5                       10

OUTPUT VOLTAGE (V)

Figure 6. Gain Linearity (Input Voltage vs. Output Voltage)

50

0          10         20         30         40         50         60         70

TIME (Seconds)

00289-008

00289-010

Figure 9. Offset Voltage Change Due to Thermal Shock

POWER CONSUMPTION (mW)

100

10

1

TA = 25°C

0                    10                   20                   30                    40

TOTAL SUPPLY VOLTAGE, V+ TO V– (V)

Figure 7. Power Consumption vs. Power Supply

| 25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VS = ±15V |  |  |  |  |  |  |  |  |

20

OPEN-LOOP GAIN (V/µV)

15

10

5

00289-007

0

00289-011

–55      –35      –15        5        25        45        65       85       105      125

TEMPERATURE (°C)

Figure 10. Open-Loop Gain vs. Temperature

| 5 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | LOT A LOT B LOT C LOT D |  |  |  |  |  |  |  |  |  |  |

4

3

OPEN-LOOP GAIN (V/µV)

12

2

VOS (µV)

1

0                                                                                                                                                                     8

–1

–2

4

–3

TA = 25°C RL = 2kΩ

–4

–5

0        20       40       60       80      100     120     140     160     180

TIME (Seconds)

Figure 8. Warm-Up VOS Drift (Normalized) Z Package

0

0                        ±5                      ±10                    ±15

POWER SUPPLY VOLTAGE (V)

Figure 11. Open-Loop Gain vs. Power Supply Voltage

±20

Rev. F | Page 6 of 16

OP177

4

VS = ±15V

INPUT BIAS CURRENT (nA)

3

2

160

140

OPEN-LOOP GAIN (dB)

120

100

80

00289-015

TA = 25°C VS = ±15V

60

1                                                                                                                                                                    40

20

0

00289-012

–50                       0                       50                      100

TEMPERATURE (°C)

0

0.01       0.1          1          10        100        1k         10k

FREQUENCY (Hz)

100k       1M

Figure 12. Input Bias Current vs. Temperature

00289-016

Figure 15. Open-Loop Frequency Response

2.0

INPUT OFFSET CURRENT (nA)

1.5

1.0

0.5

VS = ±15V

150

140

130

CMRR (dB)

120

110

100

TA = 25°C

90

0

–50                       0                       50                      100

TEMPERATURE (°C)

Figure 13. Input Offset Current vs. Temperature

00289-013

80

1                  10                100                1k                10k              100k

FREQUENCY (Hz)

Figure 16. CMRR vs. Frequency

| 100 |  |  |  |  | TA = 25°C VS = ±15V |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

80

CLOSED-LOOP GAIN (dB)

60

40

20

0

130

120

110

PSRR (dB)

100

90

80

70

00289-017

TA = 25°C

–20

00289-014

10            100             1k            10k           100k           1M           10M FREQUENCY (Hz)

60

0.1                  1                  10                100                1k                10k

FREQUENCY (Hz)

Figure 14. Closed-Loop Response for Various Gain Configurations

Figure 17. PSRR vs. Frequency

Rev. F | Page 7 of 16

OP177

INPUT NOISE VOLTAGE (nV√Hz)

1000

RS1 = RS2 = 200kΩ

THERMAL NOISE OF SOURCE

RESISTORS INCLUDED

20

VS = +15V VIN = ±10mV

00289-021

NEGATIVE SWING

RS = 0

10

00289-018

EXCLUDED

MAXIMUM OUTPUT (V)

5

TA = 25°C VS = ±15V

1

1                                 10                              100                               1k

FREQUENCY (Hz)

Figure 18. Total Input Noise Voltage vs. Frequency

0

100                                                 1k                                                10k

LOAD RESISTANCE TO GROUND (Ω)

Figure 21. Maximum Output Voltage vs. Load Resistance

10

TA = 25°C VS = ±15V

RMS NOISE (µV)

1

40

OUTPUT SHORT-CIRCUIT CURRENT (mA)

35

+ISC

30

25

00289-022

–ISC

20

TA = 25°C VS = ±15V

0.1

00289-019

100                               1k                              10k                            100k

BANDWIDTH (Hz)

Figure 19. Input Wideband Noise vs. Bandwidth

(0.1 Hz to Frequency Indicated)

15

0                         1                         2                         3                        4

TIME FROM OUTPUT BEING SHORTED (Minutes)

00289-033

Figure 22. Output Short-Circuit Current vs. Time

32

TA = 25°C

28                                                                                        VS = ±15V

| 1.50 TA = 25°C VS = ±15V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | IB1– (nA) IB2– (nA) IB1+ (nA) IB3+ (nA) |  |  |  |  |  |

1.25

PEAK-TO-PEAK AMPLITUDE (V)

24

1.00

20

IB (nA)

16                                                                                                                                                                   0.75

12

0.50

8

4

0

1k                              10k                             100k                             1M

FREQUENCY (Hz)

00289-020

0

–16      –14      –10       –6        –2         2          6         10        14

VCM (V)

Figure 20. Maximum Output Swing vs. Frequency

Figure 23. Input Bias (IB) vs. Common-Mode Voltage (VCM)

Rev. F | Page 8 of 16

OP177

APPLICATIONS INFORMATION

GAIN LINEARITY

The actual open-loop gain of most monolithic op amps varies at different output voltages. This nonlinearity causes errors in high closed-loop gain circuits.

It is important to know that the manufacturer’s AVO specifica- tion is only a part of the solution because all automated testers use endpoint testing and, therefore, show only the average gain. For example, Figure 24 shows a typical precision op amp with a respectable open-loop gain of 650 V/mV. However, the gain is not constant through the output voltage range, causing non- linear errors. An ideal op amp shows a horizontal scope trace.

Figure 25 shows the OP177 output gain linearity trace with its truly impressive average AVO of 12,000 V/mV. The output trace is virtually horizontal at all points, assuring extremely high gain accuracy. Analog Devices also performs additional testing to ensure consistent high open-loop gain at various output voltages. Figure 26 is a simple open-loop gain test circuit.

THERMOCOUPLE AMPLIFIER WITH COLD- JUNCTION COMPENSATION

An example of a precision circuit is a thermocouple amplifier that must accurately amplify very low level signals without introducing linearity and offset errors to the circuit. In this circuit, an S-type thermocouple with a Seebeck coefficient of

10.3 μV/°C produces 10.3 mV of output voltage at a temperature of 1000°C. The amplifier gain is set at 973.16, thus, it produces an output voltage of 10.024 V. Extended temperature ranges beyond 1500°C are accomplished by reducing the amplifier

gain. The circuit uses a low cost diode to sense the temperature at the terminating junctions and, in turn, compensates for any ambient temperature change. The OP177, with its high open- loop gain plus low offset voltage and drift, combines to yield a precise temperature sensing circuit. Circuit values for other thermocouple types are listed in Table 5.

| Table 5. Thermocouple Type | Seebeck Coefficient | R1 | R2 | R7 | R9 |
| --- | --- | --- | --- | --- | --- |
| K J S | 39.2 μV/°C 50.2 μV/°C 10.3 μV/°C | 110 Ω 100 Ω 100 Ω | 5.76 kΩ 4.02 kΩ 20.5 kΩ | 102 kΩ 80.6 kΩ 392 kΩ | 269 kΩ 200 kΩ 1.07 MΩ |

VX

00289-023

–10V                0V                    +10V

+15V

2    REF01    6

10.000V

2.2µF       4

3

R7                              R9

AVO ≥ 650V/mV RL = 2kΩ

Figure 24. Typical Precision Op Amp

+                              47kΩ

R

1%

10µF

+

392kΩ

1%

1.07MΩ

0.05%

+15V

0.1µF

VY

– TYPES

00289-024

+

ISOTHERMAL COLD- JUNCTIONS

R2

20.5kΩ

1%

COPPER COPPER

R8

1.0kΩ

0.05%

R5

100Ω

10µF

–

OP177

VX

–10V                               0V                                  +10V

ISOTHERMAL BLOCK

COLD-JUNCTION

R1

100Ω

1%

(ZERO ADJUST- MENT)

R4

50Ω

+

10µF

10µF

0.1µF

AVO ≥ 12000V/mV RL = 2kΩ

Figure 25. Output Gain Linearity Trace

COMPENSATION

1%

ANALOG GROUND

–15V

00289-026

ANALOG GROUND

Figure 27. Thermocouple Amplifier with Cold Junction Compensation

VY

10kΩ

10kΩ

VIN = ±10V

1MΩ

10Ω

VX

–

OP177

00289-025

+                        RL

Figure 26. Open-Loop Gain Linearity Test Circuit

Rev. F | Page 9 of 16

OP177

PRECISION HIGH GAIN DIFFERENTIAL AMPLIFIER The high gain, gain linearity, CMRR, and low TCVOS of the OP177 make it possible to obtain performance not previously

available in single stage, very high gain amplifier applications.

See Figure 28.

ISOLATING LARGE CAPACITIVE LOADS

The circuit shown in Figure 29 reduces maximum slew rate but allows driving capacitive loads of any size without instability. Because the 100 Ω resistor is inside the feedback loop, its effect on output impedance is reduced to insignificance by the high

open loop gain of the OP177.

For best CMR,

R1 must equal  R3

R2                     R4

RF

10pF

In this example, with a 10 mV differential signal, the maximum errors are listed in Table 6.

R2

+15V

RS

0.1µF

1MΩ

INPUT

2  –      7

+15V

0.1µF

6

OP177

3

+

100Ω

OUTPUT

00289-028

CLOAD

R1

1kΩ           2          7

–

4   0.1µF

R3

1kΩ

6

OP177

3

+

–15V

Figure 29. Isolating Capacitive Loads

R4

1MΩ

4

–15V

00289-027

0.1µF

BILATERAL CURRENT SOURCE

The current sources shown in Figure 30 supply both positive and negative currents into a grounded load.

Figure 28. Precision High Gain Differential Amplifier

Table 6. High Gain Differential Amp Performance

Type                                                        Amount

Note that

R5⎜ R4      ⎟

Common-Mode Voltage                 0.1%/V

⎛       1⎞

Z             ⎝ R2       ⎠

Gain Linearity, Worst Case              0.02%

TCVOS                                                                                      0.0003%/°C TCIOS                                                                                        0.008%/°C

O          R5  R4     R3

−

R2         R1

and that for ZO to be infinite

R5  R4              R3 must 

R2                  R1

PRECISION ABSOLUTE VALUE AMPLIFIER

The high gain and low TCVOS assure accurate operation with inputs from microvolts to volts. In this circuit, the signal always appears as a common-mode signal to the op amps (for details, see Figure 31).

Rev. F | Page 10 of 16

OP177

BASIC CURRENT SOURCE                                                                100mA CURRENT SOURCE

R3

VIN

R1

100kΩ

R2

1kΩ

2  –

6

R1

VIN

R3

2  –

6   50Ω

+15V

2N2222

100kΩ

OP177

3

+                         R5

R2              3

OP177

+

2N2907

R5

R4

990Ω

10Ω

IOUT ≤ 15mA

R4                                     –15V

00289-029

IOUT ≤ 100mA R3

IOUT = VIN R  × R

1        5

Figure 30. Bilateral Current Source

GIVEN R3 = R4 + R5, R1 = R2

1kΩ

1kΩ

+15V

2         7

0.1µF

C1

30pF

D1

1N4148

+15V

7

2  –

0.1µF

6

VIN

–

6

OP177

3

+

2N4393

OP177

3

+

4

R3                        0.1µF

VOUT

00289-030

0 < VOUT < 10V

4   0.1µF

2kΩ

–15V

–15V

Figure 31. Precision Absolute Value Amplifier

1kΩ

+15V

0.1µF

1N4148

+15V

0.1µF

7                                   NC

2  –

2  –   7

VIN

1kΩ     3

+

1kΩ

AD820

3

00289-031

VOUT

4

0.1µF

+

CH                                         4

0.1µF

–15V

–15V

RESET

1kΩ

Figure 32. Precision Positive Peak Detector

Rev. F | Page 11 of 16

OP177

PRECISION POSITIVE PEAK DETECTOR

In Figure 32, CH must be polystyrene, Teflon®, or polyethylene

to minimize dielectric absorption and leakage. The droop rate is determined by the size of CH and the bias current of the AD820.

CC

+15V

RF

100kΩ

PRECISION THRESHOLD DETECTOR/AMPLIFIER

VTH

RS

1kΩ

2  –     7

0.1µF

D1

1N4148

In Figure 33, when VIN < VTH, amplifier output swings negative,

R1

2kΩ

6

OP177

3

00289-032

VOUT

reverse biasing diode D1. VOUT = VTH if RL = ∞. When VIN ≥

VTH, the loop closes.

VIN

+

4   0.1µF

V          V         V

V      ⎛1    R   ⎞

–15V

OUT  

TH   

IN  −

TH ⎜

⎝

    F  ⎟

RS  ⎠

Figure 33. Precision Threshold Detector/Amplifier

CC is selected to smooth the response of the loop.

Rev. F | Page 12 of 16

OP177

OUTLINE DIMENSIONS

0.400 (10.16)

0.365 (9.27)

0.355 (9.02)

8                    5          0.280 (7.11)

0.250 (6.35)

PIN 1

0.210 (5.33) MAX

0.150 (3.81)

0.130 (3.30)

1                    4

0.100 (2.54) BSC

0.240 (6.10)

0.015 (0.38) MIN

0.060 (1.52) MAX

0.015 (0.38) GAUGE

0.325 (8.26)

0.310 (7.87)

0.300 (7.62)

0.195 (4.95)

0.130 (3.30)

0.115 (2.92)

0.115 (2.92)

0.022 (0.56)

0.018 (0.46)

0.014 (0.36)

SEATING PLANE

0.005 (0.13) MIN

PLANE

0.430 (10.92)

MAX

0.014 (0.36)

0.010 (0.25)

0.008 (0.20)

0.070 (1.78)

0.060 (1.52)

0.045 (1.14)

COMPLIANT TO JEDEC STANDARDS MS-001-BA CONTROLLING DIMENSIONS ARE IN INCHES; MILLIMETER DIMENSIONS

(IN PARENTHESES) ARE ROUNDED-OFF INCH EQUIVALENTS FOR

REFERENCE ONLY AND ARE NOT APPROPRIATE FOR USE IN DESIGN. CORNER LEADS MAY BE CONFIGURED AS WHOLE OR HALF LEADS.

Figure 34. 8-Lead Plastic Dual In-Line Package (PDIP) P-Suffix

(N-8)

Dimensions show in inches and (millimeters)

5.00 (0.1968)

4.80 (0.1890)

4.00 (0.1574)

8                 5

6.20 (0.2440)

3.80 (0.1497)   1

4   5.80 (0.2284)

0.25 (0.0098)

0.10 (0.0040)

1.27 (0.0500) BSC

1.75 (0.0688)

8°

0.50 (0.0196)  45°

0.25 (0.0099)

| COPLANARITY |  |  |  |  | 1.35 (0.0532) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0.51 (0.0201) |  |  |

0.10

SEATING PLANE

0.31 (0.0122)

0.25 (0.0098) 0°

0.17 (0.0067)

1.27 (0.0500)

0.40 (0.0157)

COMPLIANT TO JEDEC STANDARDS MS-012-AA CONTROLLING DIMENSIONS ARE IN MILLIMETERS; INCH DIMENSIONS

(IN PARENTHESES) ARE ROUNDED-OFF MILLIMETER EQUIVALENTS FOR

REFERENCE ONLY AND ARE NOT APPROPRIATE FOR USE IN DESIGN.

Figure 35. 8-Lead Standard Small Outline Package (SOIC_N) S-Suffix

(R-8)

Dimensions shown in millimeters and( inches)

Rev. F | Page 13 of 16

OP177

ORDERING GUIDE

| Model | Temperature Range | Package Description | Package Option |
| --- | --- | --- | --- |
| OP177FP OP177FPZ1 OP177GP OP177GPZ1 OP177FS OP177FS-REEL OP177FS-REEL7 OP177FSZ1 OP177FSZ-REEL1 OP177FSZ-REEL71 OP177GS OP177GS-REEL OP177GS-REEL7 OP177GSZ1 OP177GSZ-REEL1 OP177GSZ-REEL71 | −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C −40°C to +85°C | 8-Lead PDIP 8-Lead PDIP 8-Lead PDIP 8-Lead PDIP 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N 8-Lead SOIC_N | P-Suffix (N-8) P-Suffix (N-8) P-Suffix (N-8) P-Suffix (N-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) S-Suffix (R-8) |

1 Z = RoHS Compliant Part.

Rev. F | Page 14 of 16

OP177

NOTES

Rev. F | Page 15 of 16

OP177

NOTES

©1995–2009 Analog Devices, Inc. All rights reserved. Trademarks and registered  trademarks  are the  property  of their respective  owners.

D00289-0-3/09(F)

Rev. F | Page 16 of 16