A3422

Hall Effect Direction Detection Sensor IC

Last Time Buy

These parts are in production but have been determined to be LAST TIME BUY. This classification indicates that the product is obsolete and notice has been given. Sale of this device is currently restricted to existing customer applications. The device should not be purchased for new design applications because of obsolescence in the near future. Samples are no longer available.

Date of status change: May 4, 2009

Deadline for receipt of LAST TIME BUY orders: November 4, 2009

Recommended Substitutions:

For existing customer transition, and for new customers or new appli-

cations, refer to the A3423.

NOTE: For detailed information on purchasing options, contact your local Allegro field applications engineer or sales representative.

Allegro MicroSystems, Inc. reserves the right to make, from time to time, revisions to the anticipated product life cycle plan for a product to accommodate changes in production capabilities, alternative product availabilities, or market demand. The information included herein is believed to be accurate and reliable. However, Allegro MicroSystems, Inc. assumes no respon- sibility for its use; nor for any infringements of patents or other rights of third parties which may result from its use.

3422

HALL-EFFECT, DIRECTION-DETECTION SENSOR IC

The A3422xKA Hall-effect, direction-detection sensor IC is a new generation of special-function integrated devices that is applicable to sens- ing the direction of rotation of a ring magnet. This transducer provides separate digital outputs that provide information on magnet rotation

speed, direction, and magnet pole count.

Data Sheet

27650.1d

This device eliminates the ma-

jor manufacturing hurdles encountered in fine-pitch direction-detection applications, namely maintaining accurate mechanical location between the two active Hall elements.  Here, the two Hall elements are photo- lithographically aligned to better than 1 μm, as contrasted with 100 μm or worse mechanical location tolerance when manufactured discretely. This highly sensitive, temperature-stable, magnetic transducer is ideal for use in digital-encoder systems in the harsh environments of auto-

motive or industrial applications.

The A3422xKA is a high-sensitivity

device optimized for use with high-density magnets.

The A3422xKA monolithic integrated circuit contains two inde- pendent Hall-effect bipolar switches whose digital outputs are internally coupled to CMOS logic circuitry that decodes signal speed and direc-

tion.

Extremely low-drift BiCMOS circuitry is used for the amplifiers

Pinning is shown viewed from branded side.

ABSOLUTE MAXIMUM RATINGS

Supply Voltage, VCC . . . . . . . . . . . . . . . 18 V Magnetic Flux Density, B  . . . . . Unlimited Output OFF Voltage, VOUT    . . . . . . . . .  VCC Output Sink Current, IOUT   . . . . . . . . 30 mA Package Power Dissipation,

PD   . . . . . . . . . . . . . . . . . . . . .  500 mW

Operating Temperature Range, TA

Suffix ‘E–’ . . . . . . . . . . -40°C to +85°C

Suffix ‘L–’ . . . . . . . . . -40°C to +150°C

Storage Temperature Range,

TS    . . . . . . . . . . . . . . . -65°C to +170°C

to ensure symmetry between the two switches so that signal quadrature can be maintained. An on-chip voltage regulator allows the use of this device from a 4.5 V to 18 V supply. The outputs are standard open-col- lector outputs.

Two operating temperature ranges are provided; suffix ‘E–’ is for

the automotive and industrial temperature range of -40°C to +85°C, suf- fix ‘L–’ is for the automotive and military temperature range of

-40°C to +150°C. The 5-pin ‘KA’ SIP package provides a cost-com- petitive solution to magnetic sensing in harsh environments.

FEATURES

•   Internal Direction-Decoding Circuitry

•   Two Matched Hall Bipolar Switches on a Single Substrate

•   Superior Temperature Stability

•   4.5 V to 18 V Operation

Electrically Defined Power-On State

Undervoltage Lockout

3422

HALL-EFFECT,

DIRECTION-DETECTION

SENSOR IC

Selection Guide

| Part Number | Packing* | Mounting | Ambient, TA (°C) |
| --- | --- | --- | --- |
| A3422EKA-T | Bulk, 500 pieces/bag | 5-pin SIP through hole | –40 to 85 |
| A3422LKA-T | Bulk, 500 pieces/bag | 5-pin SIP through hole | –40 to 150 |

*Contact Allegro for additional packing options.

FUNCTIONAL BLOCK DIAGRAM

1     SUPPLY

2    DIRECTION

UVLO

REG

LOGIC

POWER-ON LOGIC

3    GROUND

4    E1 OUTPUT

E2

X            X                                                                                  5

E1

SPEED

Dwg. FH-018

TIMING DIAGRAM

+B                                   BOP1

0

CHANGE IN DIRECTION

BRP1

-B

+B       BOP2

0

BRP2

-B

OUT E1

OUT E2

(INTERNAL)

SPEED (OUT E1 XOR OUT E2)

td

DIRECTION

Dwg. WH-012A

, Box 15036

2

Worcester, Massachusetts 01615-0036 (508) 853-5000

Copyright © 2001, 2003 Allegro MicroSystems, Inc.

3422

HALL-EFFECT,

DIRECTION-DETECTION

SENSOR IC

ELECTRICAL CHARACTERISTICS over operating temperature range.

| Characteristic | Symbol | Test Conditions | Limits |  |
| --- | --- | --- | --- | --- |
|  |  |  | Min. Typ. Max. | Units |
| Supply Voltage Range | VCC | Operating, TJ < 165°C1 | 4.5 — 18 | V |
| Output Leakage Current | IOFF | VOUT = VCC = 18 V | — <1.0 10 | μA |
| Output Saturation Voltage | VOUT(SAT) | IOUT = 20 mA | — 0.21 0.50 | V |
| Power-On State | POS | VCC = 0 → 5 V, BRP1 < B < BOP1, BRP2 < B < BOP2 | OFF OFF OFF | — |
| Undervoltage Lockout | VCC(UV) | IOUT = 20 mA, VCC = 0 → 5 V | — 3.5 — | V |
| Undervoltage Hysteresis | VCC(hys) | Lockout (VCC(UV)) - Shutdown | — 0.5 — | V |
| Power-On Time | tpo | VCC > 4.5 V | — — 50 | μs |
| Output Rise Time | tr | CL = 20 pF, RL = 820 | — 200 — | ns |
| Output Fall Time | tf | CL = 20 pF, RL = 820 | — 200 — | ns |
| Direction Change Delay | td | CL = 20 pF, RL = 820 | 0.5 1.0 5.0 | μs |
| Supply Current | ICC | VCC = 8 V, All outputs OFF | 5.0 9.0 18 | mA |

NOTES:  1.  Maximum supply voltage must be adjusted for power dissipation and ambient temperature.

2.  Typical Data is at VCC = 12 V and TA = +25°C and is for design information only.

0.6

0.5

0.4

0.3

0.2

0.1

0

| 25 50 75 100 125 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | R θJA = 164°C/W |  |

TEMPERATURE IN  °C

150

175

Dwg. GH-069

www.allegromicro.com                                                                                                                                                                                                               3

3422

HALL-EFFECT,

DIRECTION-DETECTION

SENSOR IC

MAGNETIC CHARACTERISTICS over operating voltage range.

| Characteristic | Symbol | Test Conditions | Limits |  |
| --- | --- | --- | --- | --- |
|  |  |  | Min. Typ. Max. | Units |
| Operate Point | BOP | TA = -40°C | — — 85 | G |
|  |  | TA = +25°C | — 29 75 | G |
|  |  | TA = Maximum | — — 75 | G |
| Release Point3 | BRP | TA = -40°C | -85 — — | G |
|  |  | TA = +25°C | -75 -17 — | G |
|  |  | TA = Maximum | -75 — — | G |
| Hysteresis | Bhys | TA = -40°C | 10 — — | G |
|  |  | TA = +25°C | 10 46 — | G |
|  |  | TA = Maximum | 10 — — | G |
| Operate Differential | — | BOP1 - BOP2 | — — ±60 | G |
| Release Differential | — | BRP1 - BRP2 | — — ±60 | G |

ux density is measured at most sensitive area of device,

nominally located 0.0165” (0.42 mm) below the branded face of the package.

2.  Typical Data is at VCC = 12 V and TA = +25°C and is for design information only. ned as less than zero (algebraic convention).

Typical Magnetic Characteristics

50                                                                                                                                                                                                                                                                                                                                              100

40

30                                                                                                                                                 OPERATE POINT                                                                                                                                                               80

20

10                                                                                                                                                                                                                                                                                                                                                 60

0                                V      = 8 V CC

-10

40

RELEASE POINT

-20

-30                                                                                                                                                                                                                                                                                                                                                  20

-40

-50

-50

-25

0                       25                      50

75                    100

125

150

0

| -50 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | V = 8 V CC |  |  |  |

-25

0                      25                   50                    75                  100

125

150

AMBIENT TEMPERATURE IN °C

Dwg. GH-026-1

AMBIENT TEMPERATURE IN °C

Dwg. GH-051-1

, Box 15036

4

Worcester, Massachusetts 01615-0036 (508) 853-5000

3422

HALL-EFFECT,

DIRECTION-DETECTION

SENSOR IC

Typical Electrical Characteristics

300                                                                                                                                                                                                                                                                                                                                 10

250

I OUT=  20 mA                                                                                                                                                                                                                                                                                                                                                                                                                                   CC

9.0

ALL OUTPUTS ON

200

150

8.0

100

7.0

ALL OUTPUTS OFF

50

0

-50

-25

0                       25                     50                       75                     100

AMBIENT TEMPERATURE IN °C

125

150

6.0

-50

-25

0                         25                     50                       75                     100

AMBIENT TEMPERATURE IN °C

125

150

Dwg. GH-029-2

Dwg. GH-053-1

13

12                                                                                                                              T    = 25°C A

11

10

9.0

ALL OUTPUTS ON

8.0

ALL OUTPUTS OFF

7.0

6.0

5.0

2.0                                     6.0                                        10                                        14                                        18

SUPPLY VOLTAGE IN VOLTS

Dwg. GH-058-3

www.allegromicro.com                                                                                                                                                                                                               5

3422

HALL-EFFECT,

DIRECTION-DETECTION

SENSOR IC

Functional Description

The integrated circuit contains an internal voltage regula- tor that powers the Hall elements and both the analog and digital circuitry.  This regulator allows operation over a wide supply voltage range and provides some immunity to supply noise.  The device also contains CMOS logic circuitry that decodes the direction of rotation of the ring magnet.

Quadrature/Direction Detection. Internal logic circuitry provides outputs representing speed and direction of the magnetic field across the face of the package.  For the direction signal to be appropriately updated, a quadra- ture relationship must be maintained between the ring magnet pole width*, the element-to-element spacing, and, to a lesser extent, the magnetic switch points.  For optimal design, the device should be actuated with a ring magnet pole width* two times the sensor-to-sensor spacing.  This will produce a sinusoidal magnetic field whose period (denoted as Τ) is then four times the element-to-element spacing.  A quadrature relationship can also be maintained for a ring magnet that has a period that satisfies the rela- tionship nΤ/4 = 1.5 mm, where n is any odd integer. Therefore, ring magnets with pole-pair spacings equal to 6 mm (n = 1), 2 mm (n = 3), 1.2 mm (n = 5), etc. are permit- ted.

The response of the device to the magnetic field produced by a rotating ring magnet is shown on page 2. Note the phase shift between the two elements.

*“Pole” refers to a single pole (North or South) unless stated as “pole pair” (North and South).

Outputs. The device provides three saturated outputs: DIRECTION, E1 OUTPUT, and SPEED.  DIRECTION provides the direction output of the device and is defined as OFF (high) for the direction E1 to E2 and ON (low) for

the direction E2 to E1.  SPEED provides an XOR’d output of the two elements.  Because of internal delays, DIREC- TION will always be updated before SPEED and is updated at every transition of E1 OUTPUT and E2 OUT- PUT (internal) allowing the use of up-down counters without the loss of pulses.

Power-On State. At power on, the logic circutry is reset to provide an OFF (high) at DIRECTION and an OFF (high) for E1 and E2 (internal) for magnetic fields less

than BOP.  This eliminates ambiguity when the device is powered up and either element detects a field between BOP

and BRP.  If either element is subjected to a field greater than

BOP, the internal logic will set accordingly.

, Box 15036

6

Worcester, Massachusetts 01615-0036 (508) 853-5000

3422

HALL-EFFECT,

DIRECTION-DETECTION

SENSOR IC

Applications Information

1.50 COS α (mm)

0.059 COS α (inch)

Operation with Fine-Pitch Ring Magnets. For targets with a circular pitch of less than 4mm, a perfor- mance improvement can be observed by rotating the front face of the device (see below).  This device rotation decreases the effective element-to-element spacing, provided that the Hall elements are not rotated beyond the width of the target.

Applications. It is strongly recommended that an external 0.01 µF bypass capacitor be connected (in close proximity to the Hall elements) between the supply and ground of the device to reduce both external noise and noise generated by the internal logic.

The simplest form of magnet that will operate these devices is a ring magnet.  Other methods of operation, such as linear magnets, are possible.  Extensive applica- tions information on magnets and Hall-effect devices is also available in the “Hsall-Effect IC Applications Guide” which can be found in the latest issue of the Allegro MicroSystems Electronic Data Book, AMS-702 or Appli- cation Note 27701, or at

www.allegromicro.com

0.059"

1.50 mm

α

TARGET FACE WIDTH, F

>1.50 SIN α (mm)

>0.059 SIN α (inch)

Rotated device for fine-pitch ring magnets

A

1            2            3            4            5

Dwg. MH-024

www.allegromicro.com                                                                                                                                                                                                               7

3422

HALL-EFFECT,

DIRECTION-DETECTION

SENSOR

Criteria for Device Qualiﬁcation

cation information.

Element Locations

ACTIVE AREA DEPTH

0.0165"

0.42 mm

NOM

0.059"

1.50 mm

0.096"

2.44 mm

0.072"

1.83 mm

E1                                     E2

A

BRANDED SURFACE

1                 2                  3                 4                  5

Dwg. MH-007-1A

, Box 15036

8

Worcester, Massachusetts 01615-0036 (508) 853-5000

3422

HALL-EFFECT,

DIRECTION-DETECTION

SENSOR IC

Dimensions in Inches

(controlling dimensions)

Package Designator 'KA'

Dimensions in Millimeters

(for reference only)

0.252

0.247

6.40

6.27

0.063

0.059

1.60

1.50

0.181

0.176

4.60

4.47

45°

45°

0.083         1                2

MAX

0.600

0.560

3                4                5

0.018

0.0173

0.0138

15.24

14.23

2.11         1                2

MAX

3                4               5

0.46

0.44

0.35

SEE NOTE

SEE NOTE

0.0189

0.0142

0.050

BSC

Dwg. MH-010H  in

0.48

0.36

1.27

BSC

Dwg. MH-010H mm

Surface-Mount Lead Form (add ‘-TL’ to part number)

0.095

±0.005

2.41

±0.13

0.002

MAX

0.051

MAX

0.004

MAX

0°–8°

0.020

MIN

FLAT                                                                                                                 Dwg. MH-015 in

0.10

MAX

0°–8°

0.51

MIN

FLAT                                                                                                                    Dwg. MH-015 mm

sets. Dimensions given are measured at the widest point (parting line). guration at vendor’s option within limits shown.

ash.

4.  Recommended minimum PWB hole diameter to clear transition area is 0.035” (0.89 mm).

ed, dimension is nominal.

6.  Supplied in bulk pack (500 pieces per bag).

www.allegromicro.com                                                                                                                                                                                                               9

3422

HALL-EFFECT,

DIRECTION-DETECTION

SENSOR IC

The products described herein are manufactured under one or more of the following U.S. patents: 5,045,920; 5,264,783; 5,442,283; 5,389,889;

5,581,179; 5,517,112; 5,619,137; 5,621,319; 5,650,719; 5,686,894;

5,694,038; 5,729,130; 5,917,320; and other patents pending.

Allegro MicroSystems, Inc. reserves the right to make, from time to time,

cations as may be required to permit improvements in the performance, reliability, or manufacturability of its

products. Before placing an order, the user is cautioned to verify that the information being relied upon is current.

Allegro products are not authorized for use as critical components in life- support appliances, devices, or systems without express written approval.

The information included herein is believed to be accurate and reliable. However, Allegro MicroSystems, Inc. assumes no responsibility for its use; nor for any infringements of patents or other rights of third parties that may result from its use.

, Box 15036

10

Worcester, Massachusetts 01615-0036 (508) 853-5000