A3515 and A3516

Ratiometric, Linear Hall-Effect Sensor ICs for High-Temperature Operation

Discontinued Product

These parts are no longer in production The device should not be purchased for new design applications. Samples are no longer available.

Date of status change: April 28, 2008

Recommended Substitutions:

For new customers or new applications:

• for the A3515, please refer to the Allegro A1321, and

• for the A3516, please refer to the Allegro A1323.

NOTE: For detailed information on purchasing options, contact your local Allegro field applications engineer or sales representative.

Allegro MicroSystems, Inc. reserves the right to make, from time to time, revisions to the anticipated product life cycle plan for a product to accommodate changes in production capabilities, alternative product availabilities, or market demand. The information included herein is believed to be accurate and reliable. However, Allegro MicroSystems, Inc. assumes no responsibility for its use; nor for any infringements of patents or other rights of third parties which may result from its use.

Data Sheet

27501.10E

3515 AND

3516

RATIOMETRIC, LINEAR HALL-EFFECT SENSOR ICS FOR HIGH-TEMPERATURE OPERATION

The A3515– and A3516– are sensitive, temperature-stable linear Hall-

effect sensor ICs with greatly improved offset characteristics.  Ratiometric, linear Hall-effect devices provide a voltage output that is proportional to the applied magnetic field and have a quiescent output voltage that is approximately 50%

X

of the supply voltage.  These magnetic devices are ideal for use in linear and rotary position sensing systems in the harsh environments of automotive and industrial applications over extended temperatures to -40°C and +150°C.  The A3515– features an output sensitivity of 5 mV/G, while the A3516– has an

V                                                       output sensitivity of 2.5 mV/G.  See the Magnetic Characteristics table for

CC                                                              complete, individual device parametrics.

SUPPLY

GROUND

OUTPUT

1            2             3

Dwg. PH-006

Pinning is shown viewed from branded side.

ABSOLUTE MAXIMUM RATINGS

Supply Voltage, VCC ..............................  8.0 V Output Voltage, VO ................................  8.0 V Output Sink Current, IO ....................... 10 mA Magnetic Flux Density, B .............. Unlimited Package Power Dissipation,

PD ........................................... See Graph

Operating Temperature Range*, TA

Suffix ‘E–’ ..................... -40°C to +85°C

Suffix ‘L–’ .................... -40°C to +150°C

Storage Temperature Range,

TS .................................. -65°C to +170°C

* Infrequent excursions permitted; see

Applications Information.

Each BiCMOS monolithic circuit integrates a Hall element, improved temperature-compensating circuitry to reduce the intrinsic sensitivity drift of the Hall element, a small-signal high-gain amplifier, and a rail-to-rail low- impedance output stage.

A proprietary dynamic offset cancelation technique, with an internal high-frequency clock, reduces the residual offset voltage, which is normally caused by device overmolding, temperature dependancies, and thermal stress. This technique produces devices that have an extremely stable quiescent output voltage, are immune to mechanical stress, and have precise recover- ability after temperature cycling.  Many problems normally associated with low-level analog signals are minimized by having the Hall element and amplifier in a single chip.  Output precision is obtained by internal gain and offset trim adjustments during the manufacturing process.

These devices are supplied in a 3-pin ultra-mini-SIP ‘UA’ package.

FEATURES

■  Temperature-Stable Quiescent Output Voltage

■  Precise Recoverability After Temperature Cycling

■  Output Voltage Proportional to Applied Magnetic Field

■  Ratiometric Rail-to-Rail Output

■  Improved Sensitivity

■  4.5 V to 5.5 V Operation

■  Immune to Mechanical Stress

■  Small Package Size

■  Solid-State Reliability

Always order by complete part number, e.g.,  A3515LUA  .

3515 AND 3516

RATIOMETRIC,

LINEAR

HALL-EFFECT SENSOR ICS

FUNCTIONAL BLOCK DIAGRAM

1     SUPPLY

Vcc

DYNAMIC OFFSET CANCELLATION

LOW-PASS FILTER

X                                                                                                                                      3        OUTPUT

–

+

2       GROUND

Vcc/2

θ°

Dwg. FH-016A

| 800 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Suffix "–UA" RθJA = 165°C/W |  |  |  |  |  |  |  |
| Suffix "–U" RθJA = 184°C/W |  |  |  |  |  |  |  |  |  |
|  |  | Suffix "E–" |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Suffix "L–" |  |  |  |

ALLOWABLE PACKAGE POWER DISSIPATION in MILLIWATTS

700

600

500

400

300

200

100

0

20              40              60

80             100

120

140

160

180

AMBIENT TEMPERATURE in °C

Dwg. GH-046A

115 Northeast Cutoff, Box 15036

2                                                                              Worcester, Massachusetts 01615-0036  (508) 853-5000

Copyright © 1996, 2003 Allegro MicroSystems, Inc.

3515 AND 3516

RATIOMETRIC,

LINEAR

HALL-EFFECT SENSOR ICS

ELECTRICAL CHARACTERISTICS over operating temperature range, at VCC = 5 V (unless otherwise noted).

| Characteristic | Symbol | Test Conditions | Limits |  |
| --- | --- | --- | --- | --- |
|  |  |  | Min. Typ. Max. | Units |
| Supply Voltage | VCC | Operating | 4.5 5.0 5.5 | V |
| Supply Current | ICC | B = 0, VCC = 6 V, IO = 0 | – 7.2 10 | mA |
| Quiescent Voltage Output | VOQ | B = 0, IO = 1 mA, TA = 25°C | 2.425 2.500 2.575 | V |
| Output Voltage | VOH | B = +X*, IO = 1 mA | – 4.7 – | V |
|  | VOL | B = -X*, IO = -1 mA | – 0.2 – | V |
| Output Source Current Limit | IOLM | B = -X*, VO = 0 | -1.0 -1.5 – | mA |
| Bandwidth (-3 dB) | BW |  | – 30 – | kHz |
| Clock Frequency | fC |  | – 170 – | kHz |
| Output Resistance | rO | IO ≤ -2 mA | – 1.0 – | Ω |
| Wide-Band Output Noise (rms) | eo | B = 0, BW = 10 Hz to 10 kHz, IO ≤ -1 mA, CO = 100 pF | – 400 – | µV |

NOTE 1 – Typical data is at  TA = 25°C and is for design information only. NOTE 2 – Negative current is defined as coming out of (sourcing) the output.

* This test requires positive and negative fields sufficient to swing the output driver between fully OFF and saturated (ON), r espec- tively.  It is NOT intended to indicate a range of linear operation.

www.allegromicro.com                                                                                                                                                                             3

3515 AND 3516

RATIOMETRIC,

LINEAR

HALL-EFFECT SENSOR ICS

MAGNETIC CHARACTERISTICS over operating temperature range, at VCC = 5 V, IO = -1 mA (unless otherwise noted).

| Characteristic* | Part Numbers |  |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  | A3515EUA | A3515LUA | A3516EUA | A3516LUA |  |
|  | Min. Typ. Max. | Min. Typ. Max. | Min. Typ. Max. | Min. Typ. Max. |  |
| Operating Temp. Range, TA | -40 – +85 | -40 – +150 | -40 – +85 | -40 – +150 | °C |
| Sensitivity at TA = 25°C | 4.50 5.00 5.50 | 4.50 5.00 5.50 | 2.25 2.50 2.75 | 2.25 2.50 2.75 | mV/G |
| ∆Sens(∆T) at TA = Max. | -2.5 2.5 7.5 | -2.5 2.5 7.5 | -2.5 2.5 7.5 | -2.5 2.5 7.5 | % |
| ∆Sens(∆T) at TA = Min. | -9.0 -1.3 1.0 | -9.0 -1.3 1.0 | -9.0 -1.3 1.0 | -9.0 -1.3 1.0 | % |
| ∆V † OQ(∆T) | – – ±10 | – – ±10 | – – ±10 | – – ±10 | G |
| Ratiometry, ∆VOQ(∆V) | – 100 – | – 100 – | – 100 – | – 100 – | % |
| Ratiometry, ∆Sens(∆V) | – 100 – | – 100 – | – 100 – | – 100 – | % |
| Positive Linearity, Lin+ | – 100 – | – 100 – | – 100 – | – 100 – | % |
| Negative Linearity, Lin– | – 100 – | – 100 – | – 100 – | – 100 – | % |
| Symmetry | – 100 – | – 100 – | – 100 – | – 100 – | % |

NOTE 1 – Magnetic flux density is measured at most sensitive area of device located 0.0195" (0.50 mm) below the branded face of the “UA” package.

NOTE 2 – 10 G = 1 mT, exactly.

NOTE 3 – Except for ∆Sens(∆T), typical data is at  TA = 25°C and is for design information only.

* See Characteristics Definitions for test conditions.

† This calculation (formula 1, next page) yields the device’s equivalent accuracy, over the operating temperature range, in gauss.

115 Northeast Cutoff, Box 15036

4                                                                              Worcester, Massachusetts 01615-0036  (508) 853-5000

3515 AND 3516

RATIOMETRIC,

LINEAR

HALL-EFFECT SENSOR ICS

CHARACTERISTICS DEFINITIONS

Quiescent Voltage Output.  In the quiescent state (no mag- netic field), the output is ideally equal to one-half of the supply voltage over the operating voltage and temperature range   (VOQ

∪ VCC/2).  Due to internal component tolerances and thermal

considerations, there is a tolerance on the quiescent voltage

output and on the quiescent voltage output as a function of supply voltage and ambient temperature.  For purposes of specification, the quiescent voltage output as a function of

Ratiometry.  The A3515xUA and A3516xUA feature a ratiometric output.  The quiescent voltage output and sensitivity are proportional to the supply voltage (ratiometric).

The per cent ratiometric change in the quiescent voltage output is defined as

VOQ(VCC)  / VOQ(5V)

temperature is defined as

VOQ(TA)  – VOQ(25°C)

∆VOQ(∆V)  =

VCC / 5 V

x 100%                (4)

∆VOQ(∆T)  =

Sens(25°C)

(1)

and the per cent ratiometric change in sensitivity is defined as

Sens(VCC) / Sens(5V)

This calculation yields the device’s equivalent accuracy,

over the operating temperature range, in gauss.

∆Sens(∆V)  =

VCC / 5 V

x 100%               (5)

Sensitivity.  The presence of a south-pole magnetic field perpendicular to the package face (the branded surface) will increase the output voltage from its quiescent value toward the supply voltage rail by an amount proportional to the magnetic field applied.  Conversely, the application of a north pole will decrease the output voltage from its quiescent value.  This proportionality is specified as the sensitivity of the device and is defined as

Linearity and Symmetry.  The on-chip output stage is de- signed to provide a linear output to within 500 mV of either rail with a supply voltage of 5 V.  This is equivalent to approxi- mately ±800 gauss of ambient field.  Although application of stronger magnetic fields will not damage these devices, it will force the output into a non-linear region.  Linearity in per cent

is measured and defined as

VO(500G)  – VOQ

Sens =

VO(500G)

– V

O(-500G)                                        (2)

Lin+ =

2 (VO(250G)  – VOQ)

x 100%                    (6)

1000 G

The stability of sensitivity as a function of temperature is defined as

Lin– =

VO(-500G)  – VOQ

2 (VO(-250G)  – VOQ)

x 100%                   (7)

∆Sens(∆T)  =

Sens

(TA)

– Sens

(25°C)

x 100%              (3)

and output symmetry as

Sens(25°C)

Sym =

VO(500G)  – VOQ

VOQ – VO(-500G)

x 100%                     (8)

www.allegromicro.com                                                                                                                                                                             5

3515 AND 3516

RATIOMETRIC,

LINEAR

HALL-EFFECT SENSOR ICS

TYPICAL CHARACTERISTICS

| 9.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Vcc = 5.5 V |  |  |

| 1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Vcc = 5 V |  |  |  |  |  |  |  |

1.2

SUPPLY CURRENT IN mA

8.0

1.0

7.0

6.0

0.8

OUTPUT RESISTANCE IN Ω

0.6

-50

-25

0             25            50            75            100

AMBIENT TEMPERATURE IN °C

125

150

0.4

-50

-25

0             25            50             75           100

AMBIENT TEMPERATURE IN °C

125

150

Dwg. GH-039-2

Dwg. GH-060-1

2.55

| 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | A3515– |  |  |  |  |  |
|  |  |  |  |  | A3516– |  |  |  |  |
|  |  |  |  |  |  | Vcc = 5 V Io = -1 mA |  |  |  |

2.54

2.53

5.0

2.52

2.51

A3515–

SENSITIVITY IN mV/G

4.0

2.50

A3516–

3.0

2.49

2.48

2.47

Vcc = 5 V Io = -1 mA

2.0

1.0

QUIESCENT OUTPUT VOLTAGE IN VOLTS

2.46

2.45

-50

-25

0             25           50            75           100

AMBIENT TEMPERATURE IN °C

125

150

Dwg. GH-067

0

-50

-25

0             25            50             75           100

AMBIENT TEMPERATURE IN °C

125

150

Dwg. GH-066

115 Northeast Cutoff, Box 15036

6                                                                              Worcester, Massachusetts 01615-0036  (508) 853-5000

3515 AND 3516

RATIOMETRIC,

LINEAR

HALL-EFFECT SENSOR ICS

APPLICATIONS INFORMATION

Calibrated linear Hall devices, which can be used to deter- mine the actual flux density presented to the device in a particu- lar application, are available.

ELEMENT LOCATION

SUFFIX “UA”

above the supply voltage or pulled below the device ground.

For optimum performance, a 0.1 µF capacitor between the

and ground, should be added.

The ratiometric feature is especially valuable when these

converters typically derive their LSB step size by ratioing off a reference voltage line.  If the reference voltage varies, the LSB

sensing systems.  The A3515xUA and A3516xUA can eliminate this source of error by their ratiometric operation.  Because their gain and offsets are proportional to the supply voltage, if they are powered from the A/D reference voltage, the device output voltage will track changes in the LSB value.

These devices can withstand infrequent temperature excur- sions, beyond the Absolute Maximum Ratings, to

TA   = 170°C provided the junction temperature, TJ, does not exceed 200°C.

Extensive applications information on Hall-effect devices and magnets is also available in the “Hall-Effect IC Applications

| 0.0195" |  |  |
| --- | --- | --- |

0.50 mm

NOM

A

SURFACE        1

0.081"

2.06 mm

2             3

0.056"

1.42 mm

Dwg. MH-011-7A

Guide”, which can be found in the latest issue of the Allegro

MicroSystems Electronic Data Book, AMS-702 or Application

Allegro

Note 27701, or at

www.allegromicro.com

TYPICAL CURRENT-SENSING APPLICATION

B ≈ N x 6.9 G/A

Dwg. AH-005A

www.allegromicro.com                                                                                                                                                                             7

3515 AND 3516

RATIOMETRIC,

LINEAR

HALL-EFFECT SENSOR ICS

TYPICAL POSITION-SENSING APPLICATIONS

(Alnico 8, dimensions in inches)

+                                                                                                                          VCC                                             +

RELATIVE OUTPUT VOLTAGE

VCC

D

ELEMENT DEPTH

RELATIVE MAGNETIC FLUX DENSITY

BELOW PACKAGE FACE                                              N             S

0.21

N       S S       N

N       S S       N

D

RELATIVE OUTPUT VOLTAGE

0                                                                                                                                                                 VOQ

0.19

S  N

N   S

D

0                                                                                                                                                                 VOQ

0                           0.1                         0.2                         0.3                          0.4                          0.5

RELATIVE MAGNETIC FLUX DENSITY

-

-0.10

-0.08

-0.06           -0.04              0

0.04            0.06            0.08

GND

0.10

RELATIVE DISTANCE (TOTAL EFFECTIVE  AIR GAP)

Dwg. GH-047

RELATIVE DISTANCE

Dwg. GH-049

| + VCC + 0.19 |  |
| --- | --- |
| N S |  |
| D |  |

VCC

EFFECTIVE AIR GAP = 0.050

EFFECTIVE AIR GAP

N             S

RELATIVE OUTPUT VOLTAGE

D

0.21

RELATIVE MAGNETIC FLUX DENSITY

EFFECTIVE AIR GAP = 0.075

RELATIVE MAGNETIC FLUX DENSITY

RELATIVE OUTPUT VOLTAGE

0                                                                                                                                                                 VOQ

EFFECTIVE AIR GAP = 0.095

-

-0.4

-0.2

0                                  0.2

0.4

GND

0                                                                                                                                                                 VOQ

0                         0.05                        0.10                       0.15                        0.20                         0.25

RELATIVE DISTANCE

Dwg. GH-048

RELATIVE DISTANCE

Dwg. GH-050

115 Northeast Cutoff, Box 15036

8                                                                              Worcester, Massachusetts 01615-0036  (508) 853-5000

3515 AND 3516

RATIOMETRIC,

LINEAR

HALL-EFFECT SENSOR ICS

Dimensions in Inches

(controlling dimensions)

A3515xUA and A3516xUA

Dimensions in Millimeters

(for reference only)

0.164

0.159

4.17

4.04

45°

0.062

0.058

45°

1.57

1.47

0.122

0.117

45°

3.10

2.97

45°

0.085

MAX

1           2           3

0.031

2.16

MAX

1           2           3

0.79

0.640

0.600

0.0173

0.0138

16.26

15.24

0.44

0.35

SEE NOTE

0.0189

0.0142

SEE NOTE

0.48

0.36

0.050

BSC

Dwg. MH-014E in

1.27

BSC

Dwg. MH-014E mm

NOTES: 1.   Tolerances on package height and width represent allowable mold offsets. Dimensions given are measured at the widest point (parting line).

2.   Exact body and lead configuration at vendor’s option within limits shown.

3.   Height does not include mold gate flash.

4.   Recommended minimum PWB hole diameter to clear transition area is 0.035" (0.89 mm).

Radial Lead Form (order A351xxUA-LC)

1           2           3

0.108"

5.   Where no tolerance is specified, dimension is nominal.

6.   Supplied in bulk pack (500 pieces per bag).

0.620"

0.500" (15.7 mm

12.7 mm)

(2.74 mm)

0.100"

(2.5 mm)                 Dwg. MH-026

NOTE:  Lead-form dimensions are the nominals produced on the forming equipment. No dimensional tolerance is implied or guaranteed for bulk packaging (500 pieces per bag).

www.allegromicro.com                                                                                                                                                                             9

3515 AND 3516

RATIOMETRIC,

LINEAR

HALL-EFFECT SENSOR ICS

Horizontal-Mount Lead Form (order A351xxUA-TL)

Dimensions in Inches

(controlling dimensions)

Dimensions in Millimeters

(for reference only)

0.095

±0.005

2.41

±0.13

0.002

MAX

0.051

MAX

0.004

MAX

0°–8°

0.020

MIN

FLAT                                                                       Dwg. MH-015 in

0.10

MAX

0°–8°

0.51

MIN

FLAT                                                                       Dwg. MH-015 mm

0.050

BSC

1.27

BSC

0.062

1.57

0.038

Dwg. MA-013-3 in

0.97

Dwg. MA-013-3 mm

NOTE:  Supplied in bulk pack (500 devices per bag).

The products described herein are manufactured under one or more of the following U.S. patents: 5,045,920; 5,264,783; 5,442,283;

5,389,889; 5,581,179; 5,517,112; 5,619,137; 5,621,319; 5,650,719;

5,686,894; 5,694,038; 5,729,130; 5,917,320; and other patents pending.

Allegro MicroSystems, Inc. reserves the right to make, from time to time, such departures from the detail specifications as may be required to permit improvements in the performance, reliability, or manufacturability of its products.  Before placing an order, the user is cautioned to verify that the information being relied upon is current.

Allegro products are not authorized for use as critical components in life-support appliances, devices, or systems without express written approval.

The information included herein is believed to be accurate and reliable.  However, Allegro MicroSystems, Inc. assumes no responsibil- ity for its use; nor for any infringements of patents or other rights of third parties that may result from its use.

115 Northeast Cutoff, Box 15036

10                                                                            Worcester, Massachusetts 01615-0036  (508) 853-5000