3A,  Fast Transient Response, Low  Noise, LDO  Regulators

FEATURES 	DESCRIPTIO

■   Optimized for Fast Transient Response

■   Output Current: 3A

■   Dropout Voltage: 340mV at 3A

■   Low Noise: 40µVRMS (10Hz to 100kHz)

■   1mA Quiescent Current

■   Wide Input Voltage Range: 2.7V to 20V

■   No Protection  Diodes Needed

■   Controlled Quiescent Current in Dropout

■   Fixed Output Voltages: 1.5V, 1.8V, 2.5V, 3.3V

■   Adjustable Output from 1.21V to 20V

■    < 1µA Quiescent Current in Shutdown

■   Stable with 10µF Output Capacitor*

■   Stable with Ceramic Capacitors*

■   Reverse Battery Protection

■   No Reverse Current

■   Thermal  Limiting

APPLICATIO  S

■   3.3V to 2.5V Logic Power Supply

■   Post Regulator for Switching Supplies

The LT ®1764A is a low dropout regulator optimized for fast transient response. The device is capable of supplying

3A of output current with a dropout voltage of 340mV. Operating quiescent current is 1mA, dropping to < 1µA in shutdown. Quiescent current is well controlled; it does not rise in dropout as it does with many other regulators. In addition to fast transient response, the LT1764A has very low output voltage noise which makes the device ideal for sensitive RF supply applications.

Output voltage range is from 1.21V to 20V. The LT1764A regulators are stable with output capacitors as low as 10µF. Internal protection circuitry includes reverse battery pro- tection, current limiting, thermal limiting and reverse cur- rent protection.  The device is available in fixed output voltages of 1.5V, 1.8V, 2.5V, 3.3V and as an adjustable device with a 1.21V reference voltage. The LT1764A regu- lators are available in 5-lead TO-220 and DD packages, and

16-lead  FE packages.

, LT, LTC and LTM are registered trademarks of Linear Technology Corporation. All other trademarks are the property of their respective owners.

*See Applications Information Section.

TYPICAL APPLICATIO

3.3VIN  to 2.5VOUT Regulator

400

350

Dropout Voltage

VIN > 3V

+	IN

10µF*

OUT

+

10µF*

2.5V

3A

DROPOUT VOLTAGE (mV)

300

250

200

LT1764A-2.5

150

SHDN

SENSE

*TANTALUM,

GND

1764 TA01

CERAMIC OR

ALUMINUM ELECTROLYTIC

100

50

0

| 0 0.5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

1.0

1.5

2.0

2.5	3.0

LOAD CURRENT (A)

1764 TA02

1764afb

ABSOLUTE 	AXI	U	RAT 	GS (Note 1)

IN

IN Pin Voltage ........................................................ ± 20V OUT Pin Voltage .................................................... ± 20V Input to Output Differential Voltage (Note 12) ....... ± 20V SENSE Pin Voltage ...............................................  ± 20V ADJ Pin Voltage ...................................................... ±7V SHDN Pin Voltage ................................................. ± 20V

Output Short-Circuit Duration ......................... Indefinite

Operating Junction Temperature Range

E Grade ............................................. – 40°C to 125°C MP Grade ......................................... – 55°C to 125°C Storage Temperature Range .................  – 65°C to 150°C Lead Temperature (Soldering, 10 sec) .................. 300°C

PACKAGE/ORDER  I	FOR	ATIO

| 5 SENSE/ADJ* 4 OUT GND 2 IN 1 SHDN 5-LEAD PLASTIC DD *PIN 5 = SENSE FOR LT1764A-1.5/LT1764A-1.8/ LT1764A-2.5/LT1764A-3.3 = ADJ FOR LT1764A TJMAX = 150°C, θJA = 30°C/ W | SENSE/ 5 ADJ* 4 OUT 2 IN 1 SHDN GND 5-LEAD PLASTIC TO-220 *PIN 5 = SENSE FOR LT1764A-1.5/LT1764A-1.8/ LT1764A-2.5/LT1764A-3.3 = ADJ FOR LT1764A TJMAX = 150°C, θJA = 50°C/ W | TOP VIEW GND 1 16 GND NC 2 15 NC OUT 3 14 IN OUT 4 13 IN *PIN 6 = SENSE FOR LT1764A-1.5/ 17 LT1764A-1.8/LT1764A-2.5/ OUT 5 12 IN LT1764A-3.3 SENSE/ADJ* 6 11 NC = ADJ FOR LT1764A GND 7 10 SHDN GND 8 9 GND FE PACKAGE 16-LEAD PLASTIC TSSOP PIN 17 IS GND TJMAX = 150°C, θJA = 38°C/ W |  |
| --- | --- | --- | --- |
| ORDER PART NUMBER | ORDER PART NUMBER | ORDER PART NUMBER | FE PART MARKING |
| LT1764AEQ LT1764AEQ-1.5 LT1764AEQ-1.8 LT1764AEQ-2.5 LT1764AEQ-3.3 LT1764AMPQ | LT1764AET LT1764AET-1.5 LT1764AET-1.8 LT1764AET-2.5 LT1764AET-3.3 | LT1764AEFE LT1764AEFE-1.5 LT1764AEFE-1.8 LT1764AEFE-2.5 LT1764AEFE-3.3 | LT1764AEFE LT1764AEFE-1.5 LT1764AEFE-1.8 LT1764AEFE-2.5 LT1764AEFE-3.3 |
| Order Options Tape and Reel: Add #TR Lead Free: Add #PBF Lead Free Tape and Reel: Add #TRPBF Lead Free Part Marking: http://www.linear.com/leadfree/ |  |  |  |

Consult LTC Marketing for parts specified with wider operating temperature ranges.

ELECTRICAL CHARACTERISTICS

The ● denotes specifications which apply over the full operating temperature range, otherwise specifications are TA = 25°C. (Note 2)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Minimum Input Voltage (Notes 3, 11) | ILOAD = 0.5A ILOAD = 1.5A E Grade: ILOAD = 3A MP Grade: ILOAD = 3A | ● ● | 1.7 1.9 2.3 2.7 2.3 2.8 | V V V V |
| Regulated Output Voltage (Note 4) | LT1764A-1.5 VIN = 2.21V, ILOAD = 1mA 2.7V < VIN < 20V, 1mA < ILOAD < 3A | ● | 1.477 1.500 1.523 1.447 1.500 1.545 | V V |
|  | LT1764A-1.8 VIN = 2.3V, ILOAD = 1mA 2.8V < VIN < 20V, 1mA < ILOAD < 3A | ● | 1.773 1.800 1.827 1.737 1.800 1.854 | V V |

1764afb

ELECTRICAL CHARACTERISTICS

The ● denotes specifications which apply over the full operating temperature range, otherwise specifications are TA = 25°C. (Note 2)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
|  | LT1764A-2.5 VIN = 3V, ILOAD = 1mA 3.5V < VIN < 20V, 1mA < ILOAD < 3A | ● | 2.462 2.500 2.538 2.412 2.500 2.575 | V V |
|  | LT1764A-3.3 VIN = 3.8V, ILOAD = 1mA 4.3V < VIN < 20V, 1mA < ILOAD < 3A | ● | 3.250 3.300 3.350 3.183 3.300 3.400 | V V |
| ADJ Pin Voltage (Notes 3, 4) | LT1764A VIN = 2.21V, ILOAD = 1mA E Grade: 2.7V < VIN < 20V, 1mA < ILOAD < 3A MP Grade: 2.8V < VIN < 20V, 1mA < ILOAD < 3A | ● ● | 1.192 1.210 1.228 1.168 1.210 1.246 1.168 1.210 1.246 | V V V |
| Line Regulation | LT1764A-1.5 ∆VIN = 2.21V to 20V, ILOAD = 1mA LT1764A-1.8 ∆VIN = 2.3V to 20V, ILOAD = 1mA LT1764A-2.5 ∆VIN = 3V to 20V, ILOAD = 1mA LT1764A-3.3 ∆VIN = 3.8V to 20V, ILOAD = 1mA LT1764A (Note 3) ∆VIN = 2.21V to 20V, ILOAD = 1mA | ● ● ● ● ● | 2.5 10 3 10 4 10 4.5 10 2 10 | mV mV mV mV mV |
| Load Regulation | LT1764A-1.5 VIN = 2.7V, ∆ILOAD = 1mA to 3A VIN = 2.7V, ∆ILOAD = 1mA to 3A | ● | 3 7 23 | mV mV |
|  | LT1764A-1.8 VIN = 2.8V, ∆ILOAD = 1mA to 3A VIN = 2.8V, ∆ILOAD = 1mA to 3A | ● | 4 8 25 | mV mV |
|  | LT1764A-2.5 VIN = 3.5V, ∆ILOAD = 1mA to 3A VIN = 3.5V, ∆ILOAD = 1mA to 3A | ● | 4 10 30 | mV mV |
|  | LT1764A-3.3 VIN = 4.3V, ∆ILOAD = 1mA to 3A VIN = 4.3V, ∆ILOAD = 1mA to 3A | ● | 4 12 40 | mV mV |
|  | LT1764A (Note 3) VIN = 2.7V, ∆ILOAD = 1mA to 3A E Grade: VIN = 2.7V, ∆ILOAD = 1mA to 3A MP Grade: VIN = 2.8V, ∆ILOAD = 1mA to 3A | ● ● | 2 5 20 20 | mV mV mV |
| Dropout Voltage VIN = VOUT(NOMINAL) (Notes 5, 6, 11) | ILOAD = 1mA ILOAD = 1mA | ● | 0.02 0.05 0.10 | V V |
|  | ILOAD = 100mA ILOAD = 100mA | ● | 0.07 0.13 0.18 | V V |
|  | ILOAD = 500mA ILOAD = 500mA | ● | 0.14 0.20 0.27 | V V |
|  | ILOAD = 1.5A ILOAD = 1.5A | ● | 0.25 0.33 0.40 | V V |
|  | ILOAD = 3A ILOAD = 3A | ● | 0.34 0.45 0.66 | V V |
| GND Pin Current VIN = VOUT(NOMINAL) + 1V (Notes 5, 7) | ILOAD = 0mA ILOAD = 1mA ILOAD = 100mA ILOAD = 500mA ILOAD = 1.5A ILOAD = 3A | ● ● ● ● ● ● | 1 1.5 1.1 1.6 3.5 5 11 18 40 75 120 200 | mA mA mA mA mA mA |
| Output Voltage Noise | COUT = 10µF, ILOAD = 3A, BW = 10Hz to 100kHz |  | 40 | µVRMS |
| ADJ Pin Bias Current | (Notes 3, 8) |  | 3 10 | µA |
| Shutdown Threshold | VOUT = Off to On VOUT = On to Off | ● ● | 0.9 2 0.25 0.75 | V V |
| SHDN Pin Current (Note 9) | VSHDN = 0V VSHDN = 20V |  | 0.01 1 7 30 | µA µA |
| Quiescent Current in Shutdown | VIN = 6V, VSHDN = 0V |  | 0.01 1 | µA |
| Ripple Rejection | VIN – VOUT = 1.5V (Avg), VRIPPLE = 0.5VP-P, fRIPPLE = 120Hz, ILOAD = 1.5A |  | 55 63 | dB |

1764afb

ELECTRICAL CHARACTERISTICS

The ● denotes specifications which apply over the full operating temperature range, otherwise specifications are TA = 25°C. (Note 2)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Current Limit | VIN = 7V, VOUT = 0V |  | 4 | A |
|  | E Grade: LT1764A; LT1764A-1.5; VIN = 2.7V, ∆VOUT = – 0.1V | ● | 3.1 | A |
|  | MP Grade: LT1764A VIN = 2.8V, ∆VOUT = – 0.1V | ● | 3.1 | A |
| Input Reverse Leakage Current | VIN = – 20V, VOUT = 0V | ● | 1 | mA |
| Reverse Output Current (Note 10) | LT1764A-1.5 VOUT = 1.5V, VIN < 1.5V LT1764A-1.8 VOUT = 1.8V, VIN < 1.8V LT1764A-2.5 VOUT = 2.5V, VIN < 2.5V LT1764A-3.3 VOUT = 3.3V, VIN < 3.3V LT1764A (Note 3) VOUT = 1.21V, VIN < 1.21V |  | 600 1200 600 1200 600 1200 600 1200 300 600 | µA µA µA µA µA |

Note 1: Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. Exposure to any Absolute Maximum Rating condition for extended periods may affect device reliability and lifetime.

Note 2: The LT1764A regulators are tested and specified under pulse load

conditions such that TJ ≈ TA. The LT1764A (E grade) is 100% tested at

TA = 25°C; performance at – 40°C and 125°C is assured by design,

characterization and correlation with statistical process controls. The

LT1764A (MP grade) is 100% tested and guaranteed over the –55°C to

125°C temperature range.

Note 3: The LT1764A (adjustable version) is tested and specified for these conditions  with the ADJ pin connected to the OUT pin.

Note 4. Operating conditions are limited by maximum junction temperature. The regulated output voltage specification will not apply for all possible combinations of input voltage and output current. When operating at max- imum input voltage, the output current range must be limited. When operat- ing at maximum output current, the input voltage range must be limited.

Note 5: To satisfy requirements for minimum input voltage, the LT1764A (adjustable version) is tested and specified for these conditions with an external resistor divider (two 4.12k resistors) for an output voltage of

2.42V. The external resistor divider will add a 300µA DC load on the output. Note 6: Dropout voltage is the minimum input to output voltage differential needed to maintain regulation at a specified output current. In dropout, the

output voltage will be equal to: VIN – VDROPOUT.

Note 7: GND pin current is tested with VIN = VOUT(NOMINAL)  + 1V or VIN =

2.7V (E grade) or VIN = 2.8V (MP grade), whichever is greater, and a current

source load. The GND pin current will decrease at higher input voltages.

Note 8: ADJ pin bias current flows into the ADJ pin.

Note 9: SHDN pin current flows into the SHDN pin.

Note 10: Reverse output current is tested with the IN pin grounded and the OUT pin forced to the rated output voltage. This current flows into the OUT pin and out the GND pin.

Note 11. For the LT1764A, LT1764A-1.5 and LT1764A-1.8 dropout voltage will be limited by the minimum input voltage specification under some output voltage/load conditions.

Note 12. All combinations of absolute maximum input voltage and absolute maximum output voltage cannot be achieved. The absolute maximum differential from input to output is ± 20V. For example, with VIN = 20V, VOUT cannot be pulled below ground.

TYPICAL PERFOR 	A CE CHARACTERISTICS

600

500

400

300

DROPOUT VOLTAGE (mV)

200

100

Typical Dropout Voltage

| 700 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | TJ = 125°C |  |  |  |  |
|  |  | TJ | = 25°C |  |  |

600

500

400

300

200

GUARANTEED DROPOUT VOLTAGE (mV)

100

Guaranteed Dropout Voltage

= TEST POINTS

TJ ≤ 125°C

TJ ≤ 25°C

| 600 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | IL = | 3A |  |  |  |  |
|  |  | IL = 1 | .5A |  |  |  |
|  |  |  | IL = 0 | .5A |  |  |
|  |  |  |  | IL = 100mA |  |  |
|  |  |  |  |  | IL = 1mA |  |

500

400

DROPOUT VOLTAGE (mV)

300

200

100

Dropout Voltage

0

0	0.5	1.0	1.5	2.0

2.5	3.0

0

0	0.5	1.0

1.5

2.0

2.5

3.0

0

–50

–25	0

25	50

75	100   125

OUTPUT CURRENT (A)

1764 G01

OUTPUT CURRENT (A)

1764 G02

TEMPERATURE (°C)

1764 G03

1764afb

TYPICAL PERFOR 	A CE CHARACTERISTICS

1.4

1.2

1.0

0.8

0.6

QUIESCENT CURRENT (mA)

0.4

0.2

0

Quiescent Current

LT1764A-1.5/1.8/2.5/3.3

VIN = 6V

RL = ∞

IL = 0

VSHDN = VIN

LT1764A

| 1.54 IL = 1mA |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1.53

OUTPUT VOLTAGE (V)

1.52

1.51

1.50

1.49

1.48

1.47

1.46

LT1764A-1.5  Output Voltage

| 1.84 IL = 1 | mA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1.83

OUTPUT VOLTAGE (V)

1.82

1.81

1.80

1.79

1.78

1.77

1.76

LT1764A-1.8  Output Voltage

–50

–25	0

25	50

75	100   125

–50

–25	0

25	50

75    100   125

–50

–25	0

25	50

75    100   125

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

1764 G04

1764A G40

1756 G05

2.58

| 2.56 IL = 1 | mA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

OUTPUT VOLTAGE (V)

2.54

2.52

2.50

2.48

2.46

2.44

LT1764A-2.5  Output Voltage

3.38

3.36

OUTPUT VOLTAGE (V)

3.34

3.32

3.30

3.28

3.26

3.24

LT1764A-3.3  Output Voltage 	LT1764A ADJ Pin Voltage

| 1.230 IL = 1 | mA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1.225

1.220

1.215

1.210

1.205

ADJ PIN VOLTAGE (V)

1.200

1.195

2.42

–50

–25	0

25	50

75    100   125

3.22

| –50 IL = 1 | mA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

–25	0

25	50

75    100   125

1.190

–50

–25	0

25	50

75    100   125

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

1756 G06

1756 G07

1756 G08

| LT1764A-1.5 Quiescent Current TJ = 25°C RL = ∞ VSHDN = VIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

40

35

QUIESCENT CURRENT (mA)

30

25

20

15

10

5

LT1764A-1.8  Quiescent Current

40

35

QUIESCENT CURRENT (mA)

30

25

20

15

10

5

LT1764A-2.5  Quiescent Current

40

35

30

QUIESCENT CURRENT (mA)

25

20

15

10

5

0

0    1    2    3    4    5

6    7    8

9   10

0

| 0 1 2 3 TJ = 25°C RL = ∞ VSHDN = VIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4    5    6

7    8    9   10

0

0    1    2    3

4    5    6    7

| 8 9 10 TJ = 25°C RL = ∞ VSHDN = VIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

1764 G41

1764 G09

1764 G10

1764afb

TYPICAL PERFOR 	A CE CHARACTERISTICS

LT1764A-3.3  Quiescent Current	LT1764A Quiescent Current

LT1764A-1.5  GND Pin Current

35	1.4

QUIESCENT CURRENT (mA)

30	1.2

25	1.0

17.5

GND PIN CURRENT (mA)

15.0

12.5

TJ = 25°C VSHDN = VIN

*FOR VOUT = 1.5V

RL = 3Ω

IL = 500mA*

RL = 5Ω

IL = 300mA*

QUIESCENT CURRENT (mA)

| 15 0.6 |  |  |  |  |  |  | TJ = 25°C RL = ∞ VSHDN = VIN |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

7.5

10	0.4

5	0.2

5.0

2.5

RL = 15Ω

IL = 100mA*

0

0    1    2    3    4    5

6    7    8

9   10

0

| 0 2 4 6 TJ = 25°C RL = 4.3k VSHDN = VIN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

8   10  12

14  16  18  20

0

0    1    2    3

4    5    6

7    8    9   10

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

1764 G11

1764 G12

1764 G42

LT1764A-1.8  GND Pin Current	LT1764A-2.5  GND Pin Current	LT1764A-3.3  GND Pin Current

20.0

17.5

GND PIN CURRENT (mA)

15.0

12.5

10.0

RL = 3.6Ω

IL = 500mA*

TJ = 25°C VSHDN = VIN

*FOR VOUT = 1.8V

RL = 6Ω

IL = 300mA*

40

35

30

RL = 5Ω

25	IL = 500mA*

20

80

TJ = 25°C

VSHDN = VIN 	70

*FOR VOUT = 2.5V

GND PIN CURRENT (mA)

60

50

40

RL = 6.6Ω

IL = 500mA*

TJ = 25°C VSHDN = VIN

*FOR VOUT = 3.3V

7.5

5.0

2.5

0

RL = 18Ω

IL = 100mA*

15	RL = 25Ω

IL = 100mA*

GND PIN CURRENT (mA)

10

5

0

RL = 8.33Ω	30

IL = 300mA*

20

10

0

RL = 11Ω

IL = 300mA*

RL = 33Ω

IL = 100mA*

0	1    2

3    4    5    6

7    8    9   10

0	1    2

3    4    5    6

7    8    9   10

0	1    2

3    4    5    6

7    8    9   10

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

1764 G13

1764 G14

1764 G15

LT1764A GND Pin Current

15

150

LT1764A-1.5  GND Pin Current

GND PIN CURRENT (mA)

150

LT1764A-1.8  GND Pin Current

GND PIN CURRENT (mA)

GND PIN CURRENT (mA)

12	120

120

9	90	90

| 6 60 60 TJ = 25°C VSHDN = VIN *FOR VOUT = 1.21V |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | RL = 2.42Ω IL = 500mA* |  |  |  |  |
|  |  |  |  | RL IL | = 4. = 300 | 33Ω mA* |  |  |  |
|  |  | RL = 12.1Ω IL = 100mA* |  |  |  |  |  |  |  |

3	30	30

0

0    1    2    3	4    5

6    7    8

9   10

0

| 0 1 2 3 4 5 6 7 |  |  |  |  | TJ = 25°C VSHDN = VIN *FOR VOUT = 1.5V |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RL = 0.5Ω IL = 3A* RL = 1Ω IL = 1.5A* |  |  |  |  |  |  |
|  |  |  |  |  |  |  | RL = 2.14Ω IL = 0.7A* |  |  |

8    9   10

0

0    1    2    3

4    5	6    7

| 8 9 10 |  |  |  |  | TJ = 25°C VSHDN = VIN *FOR VOUT = 1.8V |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RL = 0.6Ω IL = 3A* |  |  |  |  |  |  |  |
|  |  |  | RL IL | = 1. = 1. | 2Ω A* | RL = 2.57Ω IL = 0.7A* |  |  |  |

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

1764 G16

1764A G43

1764 G17

1764afb

TYPICAL PERFOR 	A CE CHARACTERISTICS

LT1764A-2.5  GND Pin Current	LT1764A-3.3  GND Pin Current	LT1764A GND Pin Current

200

GND PIN CURRENT (mA)

160

TJ = 25°C VSHDN = VIN

GND PIN CURRENT (mA)

*FOR VOUT = 2.5V

200

160

TJ = 25°C VSHDN = VIN

*FOR VOUT = 3.3V

150

GND PIN CURRENT (mA)

120

120

RL = 0.83Ω

IL = 3A*

120

RL = 1.1Ω

IL = 3A*	90

80

RL = 1.66Ω

IL = 1.5A*

40

80

RL = 3.57Ω

IL = 0.7A*

40

RL = 2.2Ω

IL = 1.5A*

60

RL = 4.71Ω

IL = 0.7A*

30

0

0    1    2    3	4    5

6    7    8

9   10

0

0    1    2    3

4    5	6    7

8    9   10

0

0    1    2    3

4    5	6    7

| 8 9 10 |  |  |  |  | TJ = 25°C VSHDN = VIN *FOR VOUT = 1.21V |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | IL = 3A* |  |  |  |  |  |  |
|  |  | R | L = 0. | 81Ω |  |  |  |  |  |
|  |  |  |  |  |  |  | RL = 1.73Ω IL = 0.7A* |  |  |

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

INPUT VOLTAGE (V)

1764 G18

1764 G19

1764 G20

160

140

GND PIN CURRENT (mA)

120

100

80

60

40

20

0

GND Pin Current vs ILOAD

1.0 IL = 1mA

0.9

SHDN PIN THRESHOLD (V)

0.8

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

SHDN Pin Threshold

(On-to-Off)

1.0

0.9

SHDN PIN THRESHOLD (V)

0.8

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

SHDN Pin Threshold

(Off-to-On)

IL = 3A

IL = 1mA

| 0 0.5 1.0 VIN = VOUT(NOM) + 1V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

1.5

2.0

2.5

3.0

–50

–25	0

25	50	75

100   125

–50

–25	0

25	50	75

100   125

OUTPUT CURRENT (A)

TEMPERATURE (°C)

TEMPERATURE (°C)

1764 G21

1764 G22

1764 G23

| SHDN Pin Input Current SHDN Pin Input Current ADJ Pin Bias Current |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

9

8

7

6

5

4

3

SHDN PIN INPUT CURRENT (µA)

2

1

0

0    2    4    6

8   10  12

14  16  18  20

10

9

SHDN PIN INPUT CURRENT (µA)

8

7

6

5

4

3

2

1

0

–50

–25	0

25	50	75

100 125 VSHDN = 20V

4.0

3.5

3.0

2.5

2.0

1.5

1.0

0.5

0

–50

–25	0

25	50

ADJ PIN BIAS CURRENT (µA)

75    100   125

SHDN PIN VOLTAGE (V)

TEMPERATURE (°C)

TEMPERATURE (°C)

1764 G24

1764 G25

1756 G26

1764afb

TYPICAL PERFOR 	A CE CHARACTERISTICS

Current Limit	Current Limit	Reverse Output Current

| 6 6 5.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TJ = –50°C |  |  |  |
|  |  | TJ = 125°C |  |  |  |  |  |  |  |
|  |  |  | TJ = 25°C |  |  |  |  |  |  |

|  |  |  |  |  | VIN = 7V VOUT = 0V |  |
| --- | --- | --- | --- | --- | --- | --- |

REVERSE OUTPUT CURRENT (mA)

4.5

5	5

4.0

CURRENT LIMIT  (A)

CURRENT LIMIT  (A)

4	4	3.5

3.0

3	3	2.5

2.0

2	2

1.5

1	1	1.0

0.5

0	0	0

LT1764A-1.5

LT1764A-1.8

LT1764A-2.5

LT1764A-3.3

LT1764A

TJ = 25°C VIN = 0V

CURRENT FLOWS INTO OUTPUT PIN

VOUT = VADJ (LT1764A) VOUT = VFB

(LT1764A-1.5/1.8/-2.5/-3.3)

0    2    4

6    8   10

12  14  16

18  20

–50

–25	0

25	50

75	100   125

0    1    2    3

4    5    6

7    8    9   10

INPUT/OUTPUT DIFFERENTIAL (V)

1764 G27

TEMPERATURE (°C)

1764 G28

OUTPUT VOLTAGE (V)

1764 G29

1.0

REVERSE OUTPUT CURRENT (mA)

0.9

0.8

0.7

Reverse Output Current

VOUT = 1.21V (LT1764A) VOUT = 1.5V (LT1764A-1.5) VOUT = 1.8V (LT1764A-1.8) VOUT = 2.5V (LT1764A-2.5) VOUT = 3.3V (LT1764A-3.3)

VIN = 0V

Ripple Rejection	Ripple Rejection

| 80 75 |  |  |  | IL = 1.5A VIN = VOUT(NOM) + 1V + 0.5VP-P RIPPLE AT f = 120Hz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

70

70

RIPPLE REJECTION (dB)

60

0.6

0.5

0.4

0.3

0.2

0.1

0

LT1764A-1.5/1.8/-2.5/-3.3

LT1764A

50

40

30

20

IL = 1.5A

10  VIN = VOUT(NOM) + 1V

+ 50mVRMS RIPPLE

0

COUT = 100µF	65

TANTALUM +

10 × 1µF

RIPPLE REJECTION (dB)

CERAMIC	60

COUT = 10µF

TANTALUM	55

50

–50

–25	0

25	50	75

100   125

10	100

1k	10k

100k	1M

– 50   –25	0

25	50	75

100   125

TEMPERATURE (°C)

1764 G30

FREQUENCY (Hz)

1764 G31

TEMPERATURE (°C)

1764 G32

LT1764A Minimum  Input Voltage 	Load Regulation 	Output Noise Spectral Density

| 3.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | IL = 3 | A |
|  |  |  |  |  | IL = 1. | 5A |
|  | IL = 500mA |  |  | IL |  |  |
|  |  |  |  |  | = 100mA |  |

MINIMUM INPUT VOLTAGE (V)

2.5

2.0

10

5

LOAD REGULATION (mV)

0

–5

LT1764A-1.5

LT1764A

1

COUT = 10µF

ILOAD = 3A

LT1764A-3.3	LT1764A-2.5

1.5

1.0

–10

–15

LT1764A-1.8

LT1764A-2.5

LT1764A-3.3

0.1

LT1764A

LT1764A-1.8

0.5

0

–20

–25

–30

∆IL = 1mA TO 3A

VIN = 2.7V (LT1764A/LT1764A-1.5)

VIN = VOUT(NOM) + 1V (LT1764A-1.8/-2.5/-3.3)

OUTPUT NOISE SPECTRAL DENSITY (µV/√Hz)

0.01

LT1764A-1.5

–50

–25	0

25	50

75	100   125

–50

–25	0

25	50

75    100   125

10	100	1k	10k	100k

TEMPERATURE (°C)

1764 G33

TEMPERATURE (°C)

1764 G34

FREQUENCY (Hz)

1764 G35

1764afb

TYPICAL PERFOR 	A CE CHARACTERISTICS

RMS Output Noise vs Load Current

(10Hz to 100kHz)

LT1764A-3.3 10Hz to 100kHz

Output Noise

40

COUT = 10µF

35

OUTPUT NOISE (µVRMS)

30

25

20

LT1764A-3.3

LT1764A-2.5

LT1764A-1.8

VOUT

100µV/DIV

15

10

5

0

0.0001

0.001

LT1764A-1.5

0.01	0.1

LT1764A

1	10

COUT = 10µF 	1ms/DIV	1764A G37

IL = 3A

LOAD CURRENT (A)

1764 G36

LT1764A-3.3 Transient Response

LT1764A-3.3 Transient Response

OUTPUT VOLTAGE DEVIATION (V)

0.2

0.1

0

–0.1

–0.2

LOAD CURRENT (A)

1.00

0.75

0.50

0.25

0

VIN = 4.3V

CIN = 3.3µF TANTALUM

OUTPUT VOLTAGE DEVIATION (V)

COUT = 10µF TANTALUM

0.2

0.1

0

–0.1

–0.2

LOAD CURRENT (A)

3

2

1

0

VIN = 4.3V CIN = 33µF

COUT = 100µF TANTALUM

+ 10 × 1µF CERAMIC

0    2    4    6

8   10

12  14

16  18  20

0    2    4    6

8   10

12  14

16  18  20

TIME (µs)

TIME (µs)

1764 G38

1764 G39

1764afb

PI 	FU  CTIO  S

DD/TO-220/TSSOP

SHDN (Pin 1/1/10): Shutdown. The SHDN pin is used to put the LT1764A regulators into a low power shutdown state. The output will be off when the SHDN pin is pulled low. The SHDN pin can be driven either by 5V logic or open-collector logic with a pull-up resistor. The pull-up resistor is required to supply the pull-up current of the open-collector  gate, normally several microamperes, and the SHDN pin current, typically 7µA. If unused, the SHDN pin must  be connected to VIN.  The device  will be in

the low power shutdown  state if the SHDN pin is not

connected.

IN (Pin 2/Pin 2/Pins 12, 13, 14): Input. Power is supplied to the device through the IN pin. A bypass capacitor is required on this pin if the device is more than six inches away from the main input filter capacitor. In general, the output impedance of a battery rises with frequency, so it is advisable to include  a bypass  capacitor  in battery- powered circuits. A bypass capacitor in the range of 1µF to

10µF is sufficient. The LT1764A regulators are designed to withstand reverse voltages on the IN pin with respect to

ground and the OUT pin. In the case of a reverse input,

which can happen if a battery is plugged in backwards, the device will act as if there is a diode in series with its input. There will be no reverse current flow into the regulator and no reverse voltage will appear at the load. The device will protect both itself and the load.

NC (Pins 2, 11, 15) TSSOP Only: No Connect.

GND (Pin 3/Pin 3/Pins 1, 7, 8, 9, 16, 17): Ground.

Applications Information section for more information on output capacitance and reverse output characteristics.

SENSE (Pin  5/Pin  5/Pin  6): Sense. For fixed  voltage versions  of the LT1764A (LT1764A-1.5/LT1764A-1.8/ LT1764A-2.5/LT1764A-3.3), the SENSE pin is the input to the error amplifier. Optimum regulation will be ob- tained at the point where the SENSE pin is connected to the OUT pin of the regulator. In critical applications, small voltage drops are caused by the resistance (RP) of PC

traces between the regulator and the load. These may be

eliminated by connecting the SENSE pin to the output at the load as shown in Figure 1 (Kelvin Sense Connection). Note that the voltage drop across the external PC traces will add to the dropout voltage of the regulator. The SENSE pin bias current is 600µA at the nominal rated output voltage. The SENSE pin can be pulled below ground  (as in a dual supply system where the regulator  load is returned to a negative supply)  and still allow the device to start and operate.

ADJ (Pin 5/Pin 5/Pin 6): Adjust. For the adjustable LT1764A, this is the input to the error amplifier. This pin is internally clamped to ± 7V. It has a bias current of 3µA which flows into the pin. The ADJ pin voltage is 1.21V referenced to ground and the output voltage range is 1.21V to 20V.

2	4    RP

IN	OUT

LT1764A

+	1	5	+

OUT (Pin  4/Pin  4/Pins  3, 4, 5): Output. The output supplies power to the load. A minimum output capacitor of 10µF is required to prevent oscillations. Larger output capacitors will be required  for applications with large

VIN

SHDN

SENSE

GND

3

RP

LOAD

1764 F01

transient loads to limit  peak voltage transients. See the	Figure 1. Kelvin Sense Connection

1764afb

APPLICATIO 	S I  FOR	ATIO

The LT1764A series are 3A low dropout regulators opti- mized for fast transient response. The devices are capable of supplying 3A at a dropout voltage of 340mV. The low

VIN

IN		OUT LT1764A

VOUT

+

R2

VOUT = 1.21V ⎛1+ R2⎞ + (IADJ)(R2)

operating quiescent current (1mA) drops to less than 1µA

in shutdown. In addition to the low quiescent current, the

GND

ADJ

R1

⎜	⎟

⎝	1⎠

R

VADJ = 1.21V

LT1764A regulators incorporate several protection  fea-

tures which make them ideal for use in battery-powered systems. The devices are protected against both reverse

1764 F02

IADJ = 3µA AT 25°C

OUTPUT RANGE  = 1.21V TO 20V

input and reverse output  voltages. In battery  backup applications  where the output can be held up by a backup battery when the input is pulled to ground, the LT1764A-X acts like it has a diode in series with its output and prevents reverse current flow. Additionally, in dual supply applica- tions where the regulator load is returned to a negative supply, the output can be pulled below ground by as much as 20V and still allow the device to start and operate.

Adjustable Operation

The adjustable version  of the LT1764A has an output voltage range of 1.21V to 20V. The output voltage is set by the ratio of two external resistors as shown in Figure 2. The device servos the output to maintain the voltage at the ADJ pin at 1.21V referenced to ground. The current in R1 is then equal to 1.21V/R1 and the current in R2 is the current in R1 plus the ADJ pin bias current.  The ADJ pin bias current, 3µA at 25°C, flows through R2 into the ADJ pin. The output voltage can be calculated using the formula in Figure 2. The value of R1 should be less than 4.17k to minimize errors in the output voltage caused by the ADJ pin bias current. Note that in shutdown the output is turned off and the divider current will be zero.

The adjustable device is tested and specified with the ADJ pin tied to the OUT pin for an output voltage of 1.21V. Specifications for output voltages greater than 1.21V will be proportional to the ratio of the desired output voltage to

1.21V: VOUT/1.21V. For example, load regulation for an output current change of 1mA to 3A is – 3mV typical at VOUT = 1.21V. At VOUT = 5V, load regulation is:

(5V/1.21V)(–3mV)  = – 12.4mV

Figure 2. Adjustable Operation

Output Capacitors and Stability

The LT1764A regulator  is a feedback circuit.  Like any feedback circuit, frequency compensation is needed to make it stable. For the LT1764A, the frequency compensa- tion is both internal and external—the output capacitor. The size of the output capacitor, the type of the output capacitor, and the ESR of the particular output capacitor all affect the stability.

In addition to stability, the output capacitor also affects the high frequency transient response. The regulator loop has a finite band width. For high frequency transient loads, recovery from a transient is a combination of the output capacitor  and  the  bandwidth  of the  regulator.  The LT1764A was designed to be easy to use and accept a wide variety of output capacitors. However, the frequency compensation is affected by the output  capacitor and optimum frequency stability may require some ESR, espe- cially with ceramic capacitors.

For ease of use, low ESR polytantalum capacitors (POSCAP) are a good  choice for both the transient response and stability of the regulator. These capacitors have intrinsic ESR that improves the stability. Ceramic capacitors have extremely low ESR, and while they are a good choice in many cases, placing a small series resistance element will sometimes achieve optimum stability and minimize ring- ing. In all cases, a minimum  of 10µF is required while the

maximum  ESR allowable is 3Ω.

The place where ESR is most helpful with ceramics  is low

output voltage. At low output voltages, below 2.5V, some ESR helps the stability when ceramic output capacitors are used. Also, some ESR allows a smaller capacitor value to be used. When small signal ringing occurs with ceramics due to insufficient ESR, adding ESR or increas-

1764afb

APPLICATIO 	S I  FOR	ATIO

ing the capacitor value improves the stability and reduces the ringing. Table 1 gives some recommended values of ESR to minimize ringing  caused by fast, hard current transitions.

Table 1. Capacitor Minimum ESR

| VOUT | 10µF | 22µF | 47µF | 100µF |
| --- | --- | --- | --- | --- |
| 1.2V | 10mΩ | 5mΩ | 3mΩ | 0mΩ |
| 1.5V | 7mΩ | 5mΩ | 3mΩ | 0mΩ |
| 1.8V | 5mΩ | 5mΩ | 3mΩ | 0mΩ |
| 2.5V | 0mΩ | 0mΩ | 0mΩ | 0mΩ |
| 3.3V | 0mΩ | 0mΩ | 0mΩ | 0mΩ |
| ≥ 5V | 0mΩ | 0mΩ | 0mΩ | 0mΩ |

Figures 3 through 8 show the effect of ESR on the transient response of the regulator. These scope photos show the transient  response for the LT1764A at three different output voltages with various capacitors and various val- ues of ESR. The output load conditions are the same for all traces. In all cases there is a DC load of 1A. The load steps up to 2A at the first transition and steps back to 1A at the second transition.

At the  worst case  point of 1.2VOUT  with 10µF  COUT

(Figure 3), a minimum  amount of ESR is required. While

5mΩ is enough to eliminate most of the ringing, a value closer to 20mΩ  provides a more optimum response. At

2.5V output with 10µF COUT (Figure 4) the output rings

at the transitions with 0Ω ESR but still settles to within

10mV in 20µs after the 1A load step. Once again a small

value of ESR will provide a more optimum response.

At 5VOUT with 10µF COUT (Figure 5) the response is well

damped with 0Ω ESR.

With a COUT of 100µF at 0Ω ESR and an output of 1.2V

(Figure 6), the output rings although the amplitude is only

10mVp-p. With COUT of 100µF it takes only 5mΩ to 20mΩ

of ESR to provide good damping at 1.2V output. Perfor-

mance at 2.5V and 5V output with 100µF COUT shows sim- ilar characteristics  to the 10µF case (see Figures 7-8). At

2.5VOUT 5mΩ  to 20mΩ  can improve transient response.

At 5VOUT the response is well damped with 0Ω ESR.

Capacitor types with inherently higher ESR can be com-

bined with 0mΩ  ESR ceramic capacitors  to achieve both

good high frequency bypassing and fast settling time.

Figure 9 illustrates the improvement in transient response that can be seen when a parallel combination of ceramic and POSCAP capacitors are used. The output voltage is at the worst  case value of 1.2V. Trace A, is with a 10µF ceramic output capacitor and shows significant ringing

with a peak amplitude of 25mV. For Trace B, a 22µF/45mΩ

POSCAP is added in parallel with the 10µF ceramic.  The

output is well damped and settles to within 10mV in less than 5µs.

For Trace C, a 100µF/35mΩ  POSCAP is connected in

parallel with the 10µF ceramic capacitor. In this case the

peak output deviation is less than 20mV and the output settles in about 5µs. For improved transient response the value of the bulk capacitor (tantalum or aluminum electro- lytic) should be greater than twice the value of the ceramic capacitor.

Tantalum and Polytantalum Capacitors

There is a variety  of tantalum capacitor types available, with a wide range of ESR specifications.  Older types have

ESR specifications  in the hundreds of mΩ to several

Ohms. Some newer types of polytantalum with multi-

electrodes have maximum  ESR specifications  as low as

5mΩ. In general the lower the ESR specification, the larger

the size and the higher the price. Polytantalum capacitors

have better surge capability than older types and generally lower ESR. Some types such as the Sanyo TPE and TPB

series have ESR specifications  in the 20mΩ to 50mΩ

range, which provide near optimum transient response.

Aluminum Electrolytic Capacitors

Aluminum electrolytic capacitors can also be used with the LT1764. These capacitors can also be used in conjunction with ceramic capacitors. These tend to be the cheapest and lowest performance type of capacitors. Care must be used in selecting these capacitors as some types can have

ESR which can easily exceed the 3Ω maximum value.

1764afb

VOUT = 1.2V

0	IOUT = 1A WITH

1A PULSE

20mV/DIV

5	COUT = 10µF CERAMIC

50mV/DIV

10

20

50

VOUT = 1.2V

0	IOUT = 1A WITH

1A PULSE

COUT = 100µF CERAMIC

5

RESR (mΩ)

10

20

20µs/DIV

Figure 3

1764A F03

20µs/DIV

Figure 6

1764A F06

VOUT = 2.5V

0	IOUT = 1A WITH

1A PULSE

50mV/DIV

COUT = 10µF CERAMIC

5

10

20

VOUT = 2.5V

0	ILOAD = 1A WITH

1A PULSE

20mV/DIV

COUT = 100µF CERAMIC

5

RESR (mΩ)

10

50

20µs/DIV

Figure 4

1764A F04

20

20µs/DIV

Figure 7

1764A F07

VOUT = 5V

0	IOUT = 1A WITH

1A PULSE

50mV/DIV

COUT = 10µF CERAMIC

5

10

20

VOUT = 5V

0	ILOAD = 1A WITH

1A PULSE

20mV/DIV

COUT = 100µF CERAMIC

RESR (mΩ)

5

10

20

20µs/DIV

Figure 5

1764A F05

20µs/DIV

Figure 8

1764A F08

VOUT = 1.2V

IOUT = 1A WITH 1A PULSE

A 	COUT =

A = 10µF CERAMIC

20mV/DIV

B = 10µF CERAMIC IN PARALLEL WITH 22µF/

45mΩ POLY

RESR (mΩ)

B	C = 10µF CERAMIC IN PARALLEL WITH 100µF/

35mΩ POLY

C

20µs/DIV

Figure 9

1764A F09

RESR (mΩ)

RESR (mΩ)

RESR (mΩ)

1764afb

APPLICATIO 	S I	FOR	ATIO

Ceramic Capacitors

Extra consideration must be given to the use of ceramic capacitors. Ceramic capacitors are manufactured with a variety of dielectrics, each with different behavior over temperature  and  applied  voltage.  The most common dielectrics used are Z5U, Y5V, X5R and X7R. The Z5U and Y5V dielectrics are good for providing high capacitances in a small package, but exhibit strong voltage and tem- perature coefficients as shown in Figures 3 and 4. When used with a 5V regulator, a 10µF Y5V capacitor can exhibit an effective value as low as 1µF to 2µF over the operating temperature range. The X5R and X7R dielectrics result in more stable characteristics and are more suitable for use as the output capacitor. The X7R type has better stability across temperature, while the X5R is less expensive and is available in higher values.

Voltage and temperature  coefficients  are not the only sources of problems.  Some ceramic capacitors  have a piezoelectric response. A piezoelectric device generates voltage across its terminals due to mechanical stress, similar  to the way a piezoelectric  accelerometer  or micro- phone works. For a ceramic capacitor the stress can be induced by vibrations in the system or thermal transients.

“FREE” Resistance with PC Traces

The resistance values shown in Table 1 can easily be made using a small section of PC trace in series with the output capacitor. The wide range of noncritical ESR makes it easy to use PC trace. The trace width  should be sized to handle the RMS ripple current associated with the load.   The output capacitor only sources or sinks current for a few microseconds during fast output current transitions. There

| Table 2. PC Trace Resistors |  | BOTH CAPACITORS ARE 16V, 1210 CASE SIZE, 10µF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | X5R |  |  |  |
|  |  |  |  | Y5V |  |  |  |

|  | 10mΩ | 20mΩ | 30mΩ |
| --- | --- | --- | --- |
| 0.5oz CU Width Length | 0.011" (0.28mm) 0.102" (2.6mm) | 0.011" (0.28mm) 0.204" (5.2mm) | 0.011" (0.28mm) 0.307" (7.8mm) |
| 1.0oz CU Width Length | 0.006" (0.15mm) 0.110" (2.8mm) | 0.006" (0.15mm) 0.220" (5.6mm) | 0.006" (0.15mm) 0.330" (8.4mm) |
| 2.0oz CU Width Length | 0.006" (0.15mm) 0.224" (5.7mm) | 0.006" (0.15mm) 0.450" (11.4mm) | 0.006" (0.15mm) 0.670" (17mm) |

20

0

–20

–40

CHANGE IN VALUE (%)

–60

–80

–100

40

20

CHANGE IN VALUE (%)

0

–20

–40

–60

–80

–100

Y5V

BOTH CAPACITORS ARE 16V,

1210 CASE SIZE, 10µF

X5R

0	2	4	6

8	10

12    14    16

–50

–25	0

25	50

75	100   125

DC BIAS VOLTAGE (V)

1764 F10

TEMPERATURE (°C)

1764 F11

Figure 3. Ceramic Capacitor DC Bias Characteristics

Figure 4. Ceramic Capacitor Temperature Characteristics

1764afb

APPLICATIO 	S I	FOR	ATIO

is no DC current in the output capacitor. Worst case ripple current will occur if the output load is a high frequency (>100kHz) square wave with a high peak value and fast edges (< 1µs). Measured RMS value for this case is 0.5 times the peak-to-peak current change. Slower edges or lower frequency will significantly reduce the RMS ripple current in the capacitor.

This resistor  should  be made using  one of the inner layers of the PC board which are well defined. The resis- tivity is determined primarily by the sheet resistance of the copper laminate with no additional plating steps. Table 2 gives  some  sizes for 0.75A RMS current  for various copper thicknesses. More detailed information regarding resistors made from PC traces can be found in Application Note 69, Appendix A.

Overload Recovery

Like many IC power regulators, the LT1764A-X has safe operating area protection.  The safe area protection  de- creases the current limit as input-to-output  voltage in- creases and keeps the power  transistor  inside  a safe operating region for all values of input-to-output  voltage. The protection is designed to provide some output current at all values of input-to-output  voltage up to the device breakdown.

When power is first turned on, as the input voltage rises, the output follows the input, allowing the regulator to start up into very heavy loads. During the start-up, as the input voltage is rising, the input-to-output  voltage differential is small, allowing the regulator to supply large output cur- rents. With a high input voltage, a problem can occur wherein removal of an output short will not allow the output voltage to recover. Other regulators, such as the LT1085, also exhibit this phenomenon, so it is not unique to the LT1764A series.

The problem occurs with a heavy output load when the input voltage is high and the output voltage is low. Com- mon situations are immediately after the removal of a short circuit or when the SHDN pin is pulled high after the input voltage has already been turned on. The load line for such a load may intersect the output current curve at two points. If this happens, there are two stable output operating points  for the  regulator.  With this double

intersection,  the input  power supply  may need to be cycled down to zero and brought up again to make the output recover.

Output Voltage Noise

The LT1764A regulators  have been designed to provide low output voltage noise over the 10Hz to 100kHz band- width while operating at full load. Output voltage noise is

typically 50nV√Hz over this frequency bandwidth for the

LT1764A (adjustable version). For higher output voltages

(generated  by using a resistor divider), the output voltage noise will be gained up accordingly. This results in RMS noise over the 10Hz to 100kHz bandwidth of 15µVRMS for the LT1764A increasing to 37µVRMS for the LT1764A-3.3.

Higher values of output voltage noise may be measured when care is not exercised with regards to circuit layout and testing.  Crosstalk from nearby traces can induce unwanted noise onto the output of the LT1764A-X. Power supply  ripple  rejection  must also  be considered;  the LT1764A regulators do not have unlimited power supply rejection and will pass a small portion of the input noise through to the output.

Thermal Considerations

The power handling  capability of the device is limited by the maximum  rated junction  temperature (125°C). The power dissipated by the device is made up of two components:

1. Output current multiplied by the input/output voltage differential: (IOUT)(VIN – VOUT), and

2. GND  pin current  multiplied  by the  input voltage: (IGND)(VIN).

The GND pin current  can be found using the GND Pin Current curves in the Typical Performance Characteris- tics. Power dissipation will be equal to the sum of the two components listed above.

The LT1764A series regulators have internal thermal lim- iting designed to protect the device during overload con- ditions. For continuous normal conditions, the maximum junction  temperature  rating  of 125°C  must not be exceeded. It is important to give careful consideration to

1764afb

APPLICATIO 	S I	FOR	ATIO

all sources of thermal resistance from junction to ambient. Additional heat sources mounted nearby must also be considered.

For surface mount devices, heat sinking is accomplished by using the heat spreading capabilities of the PC board and its copper traces. Surface mount heatsinks and plated through-holes can also be used to spread the heat gener- ated by power devices.

The following table lists thermal resistance for several dif- ferent board sizes and copper areas. All measurements were taken in still air on 1/16" FR-4 board with one ounce copper.

Table 3. Q Package, 5-Lead DD

| COPPER AREA |  | BOARD AREA | THERMAL RESISTANCE (JUNCTION-TO-AMBIENT) |
| --- | --- | --- | --- |
| TOPSIDE* | BACKSIDE |  |  |
| 2500mm2 | 2500mm2 | 2500mm2 | 23°C/ W |
| 1000mm2 | 2500mm2 | 2500mm2 | 25°C/ W |
| 125mm2 | 2500mm2 | 2500mm2 | 33°C/ W |

*Device is mounted on topside.

T Package, 5-Lead TO-220

Thermal Resistance (Junction-to-Case)  = 2.5°C/W

Calculating Junction Temperature

Example: Given an output voltage of 3.3V, an input voltage range of 4V to 6V, an output current range of 0mA to

500mA and a maximum  ambient temperature of 50°C, what will the maximum junction temperature be?

The power dissipated by the device will be equal to: IOUT(MAX)(VIN(MAX)  – VOUT) + IGND(VIN(MAX))

where,

IOUT(MAX) = 500mA VIN(MAX) = 6V

IGND at (IOUT = 500mA, VIN = 6V) = 10mA

So,

P = 500mA(6V – 3.3V) + 10mA(6V) = 1.41W

Using a DD package, the thermal resistance will be in the range of 23°C/ W to 33°C/W depending on the copper area. So the junction temperature rise above ambient will be approximately equal to:

1.41W(28°C/ W) = 39.5°C

The maximum junction temperature will then be equal to the maximum junction temperature rise above ambient plus the maximum ambient temperature or:

TJMAX = 50°C + 39.5°C = 89.5°C

Protection Features

The LT1764A regulators incorporate several protection features which make them ideal for use in battery-powered circuits. In addition to the normal  protection  features associated with monolithic  regulators, such as current limiting  and thermal limiting,  the devices are protected against reverse input voltages, reverse output voltages and reverse voltages from output to input.

Current limit protection and thermal overload protection are intended to protect the device against current overload conditions at the output of the device. For normal opera- tion, the junction temperature should not exceed 125°C.

The input of the device will withstand reverse voltages of 20V. Current flow into the device will be limited to less than 1mA and no negative voltage will appear at the output. The device will protect both itself and the load. This provides protection against batteries which can be plugged in backward.

The output of the LT1764A-X can be pulled below ground without damaging the device. If the input is left open circuit or grounded, the output can be pulled below ground by

20V. For fixed voltage versions, the output will act like a

large resistor, typically 5k or higher, limiting current flow to typically less than 600µA. For adjustable versions, the output will act like an open circuit; no current will flow out of the pin. If the input is powered by a voltage source, the output will source the short-circuit current of the device and will protect itself by thermal limiting. In this case, grounding the SHDN pin will turn off the device and stop the output from sourcing the short-circuit current.

The ADJ pin of the adjustable device can be pulled above or below ground by as much as 7V without damaging the device. If the input is left open circuit or grounded, the ADJ pin will act like an open circuit when pulled below ground and like a large resistor (typically 5k) in series with a diode when pulled above ground.

1764afb

APPLICATIO 	S I	FOR	ATIO

In situations where the ADJ pin is connected to a resistor divider that would pull the ADJ pin above its 7V clamp voltage if the output is pulled high, the ADJ pin input current must be limited to less than 5mA. For example, a resistor  divider is used to provide  a regulated  1.5V output from the 1.21V reference when the output is forced to 20V. The top resistor of the resistor divider must be chosen to

limit the current into the ADJ pin to less than 5mA when the

will typically drop to less than 2µA. This can happen if the input of the device is connected to a discharged  (low voltage) battery and the output  is held up by either  a backup battery or a second regulator circuit. The state of the SHDN pin will have no effect on the reverse output current when the output is pulled above the input.

5.0

TJ = 25°C

ADJ pin is at 7V. The 13V difference between OUT and ADJ pins divided by the 5mA maximum current into the ADJ pin yields a minimum top resistor value of 2.6k.

In circuits  where a backup battery is required, several different input/output conditions can occur. The output voltage may be held up while the input is either pulled to ground, pulled to some intermediate voltage, or is left open circuit. Current flow back into the output will follow

4.5

4.0

3.5

3.0

2.5

2.0

1.5

1.0

REVERSE OUTPUT CURRENT (mA)

0.5

0

LT1764A-1.5

LT1764A-1.8

LT1764A

LT1764A-2.5

LT1764A-3.3

VIN = OV

CURRENT FLOWS INTO

OUTPUT PIN

VOUT = VADJ (LT1764A) VOUT = VFB (LT1764A-1.5

LT1764A-1.8, LT1764A-2.5, LT1764A-3.3)

the curve shown in Figure 5.

0    1    2    3    4    5    6    7    8    9   10

When the IN pin of the LT1764A-X is forced below the OUT

OUTPUT VOLTAGE (V)

1764 F12

pin or the OUT pin is pulled above the IN pin, input current	Figure 5. Reverse Output Current

TYPICAL APPLICATIO   S

SCR Preregulator Provides Efficiency Over Line Variations

NTE5437

L1

500µH

LT1764A-3.3

VOUT

L2

90V AC TO 140V AC

10V AC

AT 115VIN

10V AC

AT 115VIN

1N4148

1k

+

10000µF

34k*

IN SHDN

GND

OUT FB

+

22µF

3.3V

3A

“SYNC”

NTE5437

1N4002	1N4002

12.1k*

V+

TO ALL “V+”

POINTS

1N4002

+

2.4k

+

C1A

1N4148

200k

22µF

750Ω

1/2 LT1018

–

V+

0.1µF

V+

L1: COILTRONICS CTX500-2-52

L2: STANCOR P-8560

*1% FILM RESISTOR

0.033µF

750Ω

C1B

1/2 LT1018

1N4148

10k

V+

1µF

A1

LT1006

10k

10k

V+

LT1004

1.2V

1764 TA03

1764afb

TYPICAL APPLICATIO   S

Adjustable Current Source

R5

0.01Ω

IN	OUT

R1

C1	1k

LT1764A-1.8

VIN > 2.7V

10µF

LT1004-1.2

R2

R4	R6

SHDN	FB

GND

LOAD

R8

40.2k

2.2k

2.2k

100k

R3

2k	C3

1µF

R7

470Ω

ADJUST R1 FOR 0A TO 3A CONSTANT CURRENT

+

C2

3.3µF

2	8

1

1/2 LT1366

3

4

1764 TA04

PACKAGE DESCRIPTIO

Q Package

5-Lead Plastic DD Pak

(Reference LTC DWG # 05-08-1461)

0.060  (1.524)

0.256  (6.502)

0.060  (1.524)

0.183 (4.648)

0.060 (1.524) TYP

0.330 – 0.370 (8.382 – 9.398)

0.390 – 0.415   (9.906 – 10.541)

15° TYP

0.165 – 0.180 (4.191 – 4.572)

0.059 (1.499) TYP

0.045 – 0.055

(1.143 – 1.397)

+0.008

0.004

–0.004

(0.102 +0.203 )

–0.102

0.300 (7.620)

0.075  (1.905)

0.143 + 0.012

–0.020

+0.305

0.028 – 0.038

0.067 (1.70) BSC

0.013 – 0.023 (0.330 – 0.584)

0.095 – 0.115 (2.413 – 2.921)

0.050 ± 0.012 (1.270 ± 0.305)

BOTTOM VIEW OF DD PAK

HATCHED AREA IS SOLDER PLATED COPPER HEAT SINK

(3.632 –0.508 )

(0.711 – 0.965)

Q(DD5) 1098

1764afb

PACKAGE DESCRIPTIO

T Package

5-Lead Plastic TO-220 (Standard)

(Reference LTC DWG # 05-08-1421)

0.390 – 0.415 	 (9.906 – 10.541)

0.147 – 0.155 (3.734 – 3.937)

DIA

0.165 – 0.180 (4.191 – 4.572)

0.045 – 0.055 (1.143 – 1.397)

0.460 – 0.500 	 (11.684 – 12.700)

0.230 – 0.270 (5.842 – 6.858)

0.570 – 0.620 	 (14.478 – 15.748)

0.330 – 0.370 (8.382 – 9.398)

0.700 – 0.728 	 (17.78 – 18.491)

0.620 (15.75) TYP

SEATING PLANE

0.152 – 0.202

0.095 – 0.115 (2.413 – 2.921)

0.260 – 0.320 (6.60 – 8.13)

(3.861 – 5.131)

0.155 – 0.195*

(3.937 – 4.953)

BSC 0.067 (1.70)

0.028 – 0.038 (0.711 – 0.965)

0.135 – 0.165 (3.429 – 4.191)

0.013 – 0.023 (0.330 – 0.584)

* MEASURED AT THE SEATING PLANE

T5 (TO-220) 0399

FE Package

16-Lead Plastic TSSOP (4.4mm)

(Reference LTC DWG # 05-08-1663)

Exposed Pad Variation BB

3.58 (.141)

4.90 – 5.10* (.193 – .201)

3.58 (.141)

16   1514 13 12 1110   9

6.60 ± 0.10

4.50 ± 0.10

SEE NOTE 4

2.94  (.116)

2.94

6.40

0.45 ± 0.05

(.116)  (.252) BSC

1.05 ± 0.10

0.65 BSC RECOMMENDED SOLDER PAD LAYOUT

4.30 – 4.50* (.169 – .177)

0.25

REF

0° – 8°

1   2   3   4   5   6   7   8

1.10 	 (.0433) MAX

0.09 – 0.20 	 (.0035 – .0079)

NOTE:

0.50 – 0.75 (.020 – .030)

0.65 (.0256) BSC

0.195 – 0.30 (.0077 – .0118) TYP

0.05 – 0.15  (.002 – .006)

FE16 (BB) TSSOP 0204

1. CONTROLLING DIMENSION: MILLIMETERS

2. DIMENSIONS ARE IN  MILLIMETERS

4. RECOMMENDED MINIMUM  PCB METAL SIZE FOR EXPOSED PAD ATTACHMENT

(INCHES)	*DIMENSIONS  DO NOT INCLUDE MOLD FLASH. MOLD FLASH

3. DRAWING NOT TO SCALE

SHALL NOT EXCEED 0.150mm  (.006") PER SIDE

1764afb

19

Information furnished by Linear Technology Corporation is believed to be accurate and reliable. However, no responsibility  is assumed for its use. Linear Technology Corporation makes no represen- tation that the interconnection of its circuits as described herein will not infringe on existing patent rights.

TYPICAL APPLICATIO

Paralleling  of Regulators for Higher Output Current

R1

0.01Ω

+

IN	OUT

3.3V

+	6A

C1	LT1764A-3.3	C2

VIN > 3.7V

100µF

SHDN	FB GND

22µF

R2

0.01Ω

IN		OUT LT1764A

R6

6.65k

SHDN

SHDN

GND

ADJ

R7

4.12k

R3

2.2k

R4

2.2k	3

+

8	R5

1	1k

1/2 LT1366

2

–	4	C3

0.01µF

1764 TA05

RELATED PARTS

| PART NUMBER | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| LT1120 | 125mA Low Dropout Regulator with 20µA IQ | Includes 2.5V Reference and Comparator |
| LT1121 | 150mA Micropower Low Dropout Regulator | 30µA IQ, SOT-223 Package |
| LT1129 | 700mA Micropower Low Dropout Regulator | 50µA Quiescent Current |
| LT1175 | 500mA Negative Low Dropout Micropower Regulator | 45µA IQ, 0.26V Dropout Voltage, SOT-223 Package |
| LT1374 | 4.5A, 500kHz Step-Down Converter | 4.5A, 0.07Ω Internal Switch, SO-8 Package |
| LT1521 | 300mA Low Dropout Micropower Regulator with Shutdown | 15µA IQ, Reverse Battery Protection |
| LT1529 | 3A Low Dropout Regulator with 50µA IQ | 500mV Dropout Voltage |
| LT1573 | UltraFastTM Transient Response Low Dropout Regulator | Drives External PNP |
| LT1575 | UltraFast Transient Response Low Dropout Regulator | Drives External N-Channel MOSFET |
| LTC1735 | Synchronous Step-Down Converter | High Efficiency, OPTI-LOOP® Compensation |
| LT1761 Series | 100mA, Low Noise, Low Dropout Micropower Regulators in SOT-23 | 20µA Quiescent Current, 20µVRMS Noise, ThinSOTTM Package |
| LT1762 Series | 150mA, Low Noise, LDO Micropower Regulators | 25µA Quiescent Current, 20µVRMS Noise, MSOP Package |
| LT1763 Series | 500mA, Low Noise, LDO Micropower Regulators | 30µA Quiescent Current, 20µVRMS Noise, SO-8 Package |
| LT1962 | 300mA, Low Noise, LDO Micropower Regulator | 20µVRMS Noise, MSOP Package |
| LT1963A | 1.5A, Low Noise, Fast Transient Response LDO | 40µVRMS Noise, SOT-223 Package |
| LT1964 | 200mA, Low Noise, Negative LDO Micropower Regulator | 30µVRMS Noise, ThinSOT Package |

OPTI-LOOP is a registered trademark of Linear Technology Corporation.  UltraFast and ThinSOT are trademarks of Linear Technology Corporation.

Linear Technology Corporation

20

1630 McCarthy Blvd., Milpitas, CA 95035-7417

(408) 432-1900  ●  FAX: (408)  434-0507  ●    www.linear.com

1764afb

LT 0706 REV B • PRINTED IN USA

© LINEAR TECHNOLOGY CORPORATION 2002