Rev 0; 2/04

Parallel-Interface Elapsed Time Counter

General Description

The DS1318 parallel-interface  elapsed  time counter (ETC) is a 44-bit counter that maintains the amount of time that the device operates from main and/or backup power  or during  an  external  event.  The internal frequency of the counter clock is 4.096kHz, which pro- vides a 244µs resolution and a maximum count of over

136 years. A built-in power-sense circuit detects power failures, automatically switches to the backup  supply, and controls the timer. If an external event timer is desired,  the control input EXT can control the counter operation. An open-drain  output provides an interrupt, and a square-wave output provides a programmable square wave. The DS1318 is accessed  through a byte- wide parallel interface, and operates over the industrial temperature range.

Applications

Power Meters Industrial Controls Servers

Features

♦ Byte-Wide Parallel Interface

♦ 44-Bit Binary Counter Provides Timer with 244µs

Resolution

♦ Automatic Power-Fail Detect and Switch Circuitry Selects Power Source from the Primary Power and the Battery, and Write Protects the Internal Registers

♦ Internal Power-Fail Circuit Allows Timer to

Provide Primary or Battery Operation Times

♦ Timer can Alternately Provide an Event Timer of

Either an Active-High or Active-Low Pulse

♦ Interrupt Output Generated Periodically or When the Upper 32 Bits of the Counter Match an Alarm Register

♦ Square-Wave Output with 16 Selectable

Frequencies from 32.768kHz to 0.5Hz

♦ +3.3V Operation

♦ Industrial Temperature Range: -40°C to +85°C

Ordering Information

PART 	TEMP RANGE    PIN- PACKAGE

DS1318 	-40°C to +85°C    24 TSSOP,

4.4mm

TOP MARK

DS1318

Typical Operating Circuit

Pin  Configuration

CPU

RPU

IRQ WE OE CE

A3–A0

DQ–DQ7

DS1318

GND

VCC

VCC

SQW EXT

VBAT

X1

X2

0.1µF

-

EXTERNAL COUNTER ENABLE

(EVENT TIMER)

TOP VIEW

X1	1

X2	2

GND	3

EXT	4

A3	5

A2	6

A1	7

A0	8

DQ0	9

DQ1	10

DQ2	11

GND	12

DS1318

24	VCC

23	VBAT

22	IRQ

21	SQW

20	WE

19	OE

18	CE

17	DQ7

16	DQ6

15	DQ5

14	DQ4

13	DQ3

DS1318

TSSOP

ABSOLUTE MAXIMUM RATINGS

Voltage Range on any Pin Relative to Ground ......-0.3V to +6.0V Operating  Temperature  Range ...........................-40°C to +85°C Storage Temperature  Range .............................-55°C to +125°C

Soldering Temperature  ......................................See IPC/JEDEC J-STD-020A Specification

DS1318

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

RECOMMENDED DC OPERATING CONDITIONS

(VCC = VCC(MIN) to VCC(MAX), TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Supply Voltage | VCC | (Note 2) | 3.0 3.3 3.6 | V |
| Battery Voltage | VCC | (Note 2) | 1.6 3.3 3.7 | V |
| Logic 1 Voltage | VIH | (Note 2) | 0.7 x VCC + VCC 0.5 | V |
| Logic 0 Voltage | VIL | (Note 2) | -0.5 +0.3 x VCC | V |

DC ELECTRICAL CHARACTERISTICS

(VCC = VCC(MIN) to VCC(MAX), TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Logic 0 Output Current (VOL = 0.15 x VCC) | IOL |  | 3 | mA |
| Logic 1 Output Current (VOH = 0.85 x VCC) | IOH |  | 1 | mA |
| SQW, INT Logic 0 Output (VOL = 0.15 x VCC) | IOLSI |  | 5 | mA |
| Input Leakage | ILI | (Note 3) | 1 | µA |
| I/O Leakage | ILO | (Note 4) | -1 +1 | µA |
| Active Supply Current | ICCA | (Note 5) | 10 | mA |
| Standby Current | ICCS | (Note 6) | 100 150 | µA |
| Battery Input-Leakage Current | IBATLKG |  | 10 100 | nA |
| Power-Fail Voltage | VPF | (Note 2) | 2.70 2.97 | V |

DS1318

DC ELECTRICAL CHARACTERISTICS

(VCC = 0V, VBACKUP = 3.7V, TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Battery Input Current (ENOSC = 1) | IBAT | (Note 7) | 750 1100 | nA |
| Battery Input Current (ENOSC = 0) | IBATDR | (Note 7) | 100 | nA |

AC ELECTRICAL CHARACTERISTICS

(VCC = VCC(MIN) to VCC(MAX), TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Read Cycle Time | tRC |  | 80 | ns |
| Address Access Time | tAA |  | 80 | ns |
| CE to DQ Low-Z | tCEL |  | 0 | ns |
| CE Access Time | tCEA |  | 80 | ns |
| CE Data Off-Time | tCEZ |  | 30 | ns |
| OE to DQ Low-Z | tOEL |  | 0 | ns |
| OE Access Time | tOEA |  | 70 | ns |
| OE Data Off-Time | tOEZ |  | 30 | ns |
| Output Hold from Address | tOH |  | 5 | ns |
| Write Cycle Time | tWC |  | 80 | ns |
| Address Setup Time | tAS |  | 0 | ns |
| WE Pulse Width | tWEW |  | 40 | ns |
| CE Pulse Width | tCEW |  | 70 | ns |
| Data Setup Time | tDS |  | 40 | ns |
| Data Hold Time | tDH |  | 0 | ns |
| Address Hold Time | tAH |  | 0 | ns |
| WE Data Off-Time | tWEZ |  | 30 | ns |
| Write Recovery Time | tWR |  | 10 | ns |
| Oscillator Stop Flag (OSF) Delay | tOSF | (Note 8) | 4 | ms |

Read Cycle Timing

tRC

A0–A4

CE

tAA

tCEA

tCEZ 	tOH

tCEL

OE

tOEA

tOEZ

DQ0–DQ7

tOEL

VALID

Write Cycle Timing, Write-Enable Controlled

tWC

A0–A4

VALID 	VALID

tAS 	tAH

CE

tAS

tWEW

tWR

WE

tWEZ	tDS 	tDH

DQ0–DQ7

DATA OUTPUT

DATA INPUT

DATA INPUT

Write Cycle Timing, Chip-Enable Controlled

tWC

A0–A4

VALID 	VALID

tAS 	tCEW	tAH

CE

tAS

tWR

WE

tDS 	tDH

DS1318

DQ0–DQ7

DATA INPUT

DATA INPUT

Power-Up/Power-Down Timing

VCC

VPF(MAX)

VPF(MIN)

tVCCF

tVCCR

tREC

INPUTS

OUTPUTS

RECOGNIZED

VALID

DON'T CARE

HIGH-Z

RECOGNIZED

VALID

DS1318

POWER-UP/POWER-DOWN CHARACTERISTICS

(TA = -40°C to +85°C) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Recovery at Power-Up | tREC | (Note 9) | 150 | ms |
| VCC Fall Time; VPF(MAX) to VPF(MIN) | tVCCF |  | 300 | µs |
| VCC Rise Time; VPF(MIN) to VPF(MAX) | tVCCR |  | 0 | µs |

CAPACITANCE

(TA = +25°C)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Capacitance on All Input Pins | CIN |  | 10 | pF |
| Capacitance on IRQ, SQW, and DQ Pins | CIO |  | 10 | pF |

AC TEST CONDITIONS

| PARAMETER | TEST CONDITION |
| --- | --- |
| Input Pulse Levels | 0 to 2.7V |
| Output Load Including Scope and Jig | 25pF + 1TTL Gate |
| Input and Output Timing Measurement Reference Levels | VCC / 2 |
| Input-Pulse Rise and Fall Times | 4ns |

WARNING:

Under no circumstances are negative undershoots, of any amplitude,  allowed when device is in write protection.

Note 1: 	Limits at -40°C are guaranteed by design and not production tested.

Note 2: 	All voltages are referenced to ground.

Note 3: 	OE, CE, WE, EXT, and A3–A0.

Note 4: 	DQ7–DQ0, SQW, and IRQ, when the outputs are high impedance.

Note 5: 	Outputs open.

Note 6: 	Specified  with parallel bus inactive.

Note 7: 	Measured  with a 32,768kHz crystal attached  to the X1 and X2 pins.

Note 8: 	The parameter  tOSF is the period of time that the oscillator must be stopped  for the OSF flag to be set over the voltage range of 0V ≤ VCC ≤ VCC(MAX) and 1.3V ≤ VBACKUP ≤ 3.7V.

Note 9: 	This delay applies only if the oscillator is enabled  and running. If the ENOSC bit is 0, tREC is disabled, and the device is

immediately  accessible. If CE and OE are low on power-up,  the DQ outputs are active. Valid data out is not available until after tREC.

Typical Operating Characteristics

(VCC = +3.3V, TA = +25°C, unless otherwise noted.)

850

800

750

SUPPLY CURRENT (nA)

700

650

600

IBAT vs. TEMPERATURE VBAT = 3.0V

DS1318 toc01

VCC = 0

850

800

SUPPLY CURRENT (nA)

750

700

650

600

550

500

IBAT vs. VBAT

VCC = 0

32768.50

32768.45

32768.40

32768.35

32768.30

32768.25

32768.20

32768.15

FREQUENCY (Hz)

32768.10

32768.05

32768.00

DS1318 toc02

DS1318 toc03

| OSCILLATOR FREQUENCY vs. VOLTAGE |  |  |  |
| --- | --- | --- | --- |

-40

-20	0

20	40

60	80

1.5

1.9

2.3

2.7

3.1

3.5

2.0

2.5

3.0

3.5

4.0

DS1318

TEMPERATURE (°C)

VBACKUP (V)

INPUT VOLTAGE (V)

Functional Diagram

DS1318

X1

X2

EXT

VCC

VBAT

CE OE WE

A3–A0

DQ7–DQ0

32,768Hz CRYSTAL OSCILLATOR AND PRESCALER COUNTER

VCC LEVEL DETECT, POWER SWITCH,

AND WRITE PROTECT

BYTE-WIDE RAM INTERFACE

4,096Hz

4,096Hz

SUBSECONDS0

SUBSECONDS1

SECONDS0

SECONDS1

TRANSFER ENABLE (TE)

SECONDS2

SECONDS3

INTERNAL DIVIDERS AND COUNTERS

32,768Hz

CLOCK CONTROL CIRCUITRY

BUFFERED COUNTER REGISTERS, CONTROL AND STATUS REGISTERS

SUBSECONDS0

SUBSECONDS1

SECONDS0

SECONDS1

SECONDS2

SECONDS3

ALARM0

ALARM1

ALARM2

ALARM3

CONTROL A CONTROL B STATUS

SQUARE-WAVE RATE SELECTOR AND PRESCALER

DS1318

PF/ALMF MUX

IRQ

SQW

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | X1 | Connections for Standard 32.768kHz Quartz Crystal. The internal oscillator circuitry is designed for operation with a crystal having a 12.5pF specified load capacitance (CL). X1 is the input to the oscillator and can optionally be connected to an external 32.768kHz oscillator. The output of the internal oscillator, X2, is floated if an external oscillator is connected to X1. |
| 2 | X2 |  |
| 3, 12 | GND | Ground. This pin must be connected to ground for proper operation. |
| 4 | EXT | External Counter-Enable Input |
| 5–8 | A3–A0 | Address Bus Inputs |
| 9, 10, 11, 13–17 | DQ0–DQ7 | Bidirectional Data Pins |
| 18 | CE | Chip-Enable Input, Active Low |
| 19 | OE | Output-Enable Input, Active Low |
| 20 | WE | Write-Enable Input, Active Low |
| 21 | SQW | Square-Wave Output |
| 22 | IRQ | Interrupt Output. This active-low open-drain pin requires a pullup resistor. |
| 23 | VBAT | Battery/Backup Power-Supply Input |
| 24 | VCC | DC Power for Primary Power Supply |

Table 1. Operation Modes for Power-Supply Conditions

| VCC | CE | OE | WE | DQ0–DQ7 | A0–A4 | MODE | POWER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VCC > VPF | VIH | X | X | High-Z | X | Deselect | Standby |
|  | VIL | X | VIL | DIN | AIN | Write | Active |
|  | VIL | VIL | VIH | DOUT | AIN | Read | Active |
|  | VIL | VIH | VIH | High-Z | AIN | Read | Active |
| VSO < VCC < VPF | X | X | X | High-Z | X | Deselect | CMOS Standby |
| VCC < VSO < VPF | X | X | X | High-Z | X | Data | Battery Current |

DS1318

Detailed Description

The parallel-interface ETC contains a 44-bit up counter that maintains the amount of time the counter  is enabled. The resolution of the timer is 244µs. A control register selects which events enable and disable  the counter. The counter is double-buffered  into two regis- ter sets, and the TE bit controls the updating  of the user-readable copy.

The counter can be used to maintain the cumulative amount of time the primary power source or the battery powers the device.  In this mode, the counter starts when the internal power-switching  circuit  enables the selected  power source and stops when the circuit enables the other source.

The counter can also be used as an external event timer. In this mode, the counter starts when the signal EXT tog-

gles to the active sate and stops when it toggles to the inactive state. The active state of the EXT signal is con- figurable as high or low. EXT is ignored and the counter is disabled while the device is in power fail.

The interrupt output pin provides two maskable inter- rupt sources. A 32-bit alarm register allows an interrupt to be generated  whenever the upper  32 bits of the counter match the alarm register. A periodic  interrupt can also be generated from once every 244µs to once every 1/12,097,152Hz (24.27 days). The alarm and interrupt output operate when the device is operating from either supply.

Table 1 shows the factors that control the device oper- ation. VSO is the battery switchover voltage and is the lesser of VBAT and VPF. While the device is operating from the battery with the oscillator running, the battery

Table 2. Crystal Specifications*

| PARAMETER | SYMBOL | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| Nominal Frequency | fO | 32.768 | kHz |
| Series Resistance | ESR | 50 | kΩ |
| Load Capacitance | CL | 12.5 | pF |

*The crystal, traces, and crystal input pins should be isolated from RF generating  signals. Refer to Application Note 58: Crystal

Considerations for Dallas Real-Time Clocks for additional  specifications.

RTC

LOCAL GROUND PLANE (LAYER 2)

COUNTDOWN CHAIN 	X1

CRYSTAL

CL1 	CL2

X1	X2

CRYSTAL

RTC REGISTERS

NOTE: AVOID ROUTING SIGNAL LINES IN THE CROSSHATCHED AREA

(UPPER LEFT QUADRANT) OF

THE PACKAGE UNLESS THERE IS A GROUND PLANE BETWEEN THE

SIGNAL LINE AND THE DEVICE PACKAGE.

X2

GND

Figure 1. Oscillator Circuit Showing Internal Bias Network 	Figure 2. Layout Example

DS1318

input current is IBAT. The oscillator consumes most of the current. If the oscillator is disabled,  the data in the registers remain static, and the battery input current is IBATDR, which is primarily due to the leakage of the sta- tic memory cells.

The DS1318 uses a standard parallel byte-wide interface to access the register map. Table 1 summarizes the modes of operation at various power-supply conditions.

Oscillator Circuit

The DS1318 uses an external 32.768kHz crystal. The oscillator circuit does not require any external resistors or capacitors to operate. Table 2 specifies several crys- tal parameters for the external crystal, and Figure 1 shows a functional schematic  of the oscillator circuit. An enable bit in the control register controls the oscilla- tor. Oscillator startup times are highly dependent upon crystal characteristics,  PC board leakage, and layout. High ESR and excessive capacitive loads are the major contributors  to long startup times. A circuit  using a crystal with the recommended  characteristics  and proper layout usually starts within one second.

An external 32.768kHz oscillator can also drive the DS1318. In this configuration, the X1 pin is connected to the external oscillator signal and the X2 pin is floated.

Clock Accuracy

The accuracy of the clock is dependent upon the accu- racy of the crystal  and the accuracy  of the match between the capacitive load of the oscillator circuit and the capacitive  load for which the crystal was trimmed. Additional  error is added  by crystal frequency  drift caused  by temperature  shifts. External circuit  noise coupled into the oscillator circuit can result in the clock running fast. Figure 2 shows a typical PC board layout for isolation of the crystal and oscillator from noise. Refer to Application  Note 58: Crystal Considerations with Dallas Real-Time Clocks for more detailed information.

Table 3. Address Map

| ADDRESS | BIT 7 | BIT 6 | BIT 5 | BIT 4 | BIT 3 | BIT 2 | BIT 1 | BIT 0 | FUNCTION | RANGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00H | SS3 | SS2 | SS1 | SS0 | 0 | 0 | 0 | SQWS | Subseconds0 | 00–F0h |
| 01H | SS11 | SS10 | SS9 | SS8 | SS7 | SS6 | SS5 | SS4 | Subseconds1 | 00–FFh |
| 02H | S7 | S6 | S5 | S4 | S3 | S2 | S1 | S0 | Seconds0 | 00–FFh |
| 03H | S15 | S14 | S13 | S12 | S11 | S10 | S9 | S8 | Seconds1 | 00–FFh |
| 04H | S23 | S22 | S21 | S20 | S19 | S18 | S17 | S16 | Seconds2 | 00–FFh |
| 05H | S31 | S30 | S29 | S28 | S27 | S26 | S25 | S24 | Seconds3 | 00–FFh |
| 06H | ALM7 | ALM6 | ALM5 | ALM4 | ALM3 | ALM2 | ALM1 | ALM0 | Alarm0 | 00–FFh |
| 07H | ALM15 | ALM14 | ALM13 | ALM12 | ALM11 | ALM10 | ALM9 | ALM8 | Alarm1 | 00–FFh |
| 08H | ALM23 | ALM22 | ALM21 | ALM20 | ALM19 | ALM18 | ALM17 | ALM16 | Alarm2 | 00–FFh |
| 09H | ALM31 | ALM30 | ALM29 | ALM28 | ALM27 | ALM26 | ALM25 | ALM24 | Alarm3 | 00–FFh |
| 0AH | TE | ENOSC | CCFG1 | CCFG0 | EPOL | SQWE | PIE | AIE | ControlA | 00–FFh |
| 0BH | PRS3 | PRS2 | PRS1 | PRS0 | SRS3 | SRS2 | SRS1 | SRS0 | ControlB | 00–FFh |
| 0CH | OSF | UIP | 0 | 0 | 0 | 0 | PF | ALMF | Status | — |

Note: Unless otherwise specified, the state of the registers is not defined when power is first applied.

DS1318

Counter Operation

The binary time information is obtained by reading the appropriate  register bytes. Registers 02h through 05h contain the time in seconds from an arbitrary reference time determined  by the user. Registers 00h and 01h contain the fractional seconds  count. A buffered  copy of the clock  registers (A0–A5), updated  every 244µs, allows the user to read and write the registers while the internal registers continue to increment. However, it is possible to read or write inconsistent data, or for a write to corrupt the current buffered read copy, if an update occurs during the read or write. Several methods may be used to ensure that the data is accurate.

The clock registers can be read, with the least-signifi- cant byte (LSB) being read once at the beginning and again after the other registers have been read (i.e., A2–A5, A2). If the LSB register data has changed,  the registers should be re-read until the LSB register data matches. If the subseconds0 register is used, the user never has more than 244µs to read all the registers before a mismatch occurs.  In addition,  if the routine used to read the registers takes approximately 1.95ms to read the registers, it is possible that the subseconds0 register could roll over to the same value as previously read.

Other methods use the TE and UIP bits to synchronize accessing the clock registers to ensure that the data are valid. These methods are discussed in later sections.

Alarm

To use the alarm function, the user writes registers 06h through 09h with a time in seconds. When the current time in seconds becomes equal to the alarm value, the ALMF bit in the status register (0Ch) is set to 1. If the AIE bit in control register A is set to 1 by the user, then the IRQ pin is driven low when the ALMF bit is set to 1. The alarm and IRQ output operate when the device is running from either supply.

Periodic Flag

Writing a non-zero value into the periodic  flag rate- select bits in control register B enables the periodic flag operation. The periodic  flag is set to logic 1 when the internal counter reaches the selected value. Writing the PF bit to 0 resets the periodic flag. If the flag is not reset, it remains high. Once the PF bit is set, the inter- nal counter continues counting, and attempts to set the PF bit again when the count again matches the select- ed rate value. Clearing the PF bit has no effect on the internal counter. If the PIE bit in control register A is set to 1, the IRQ output goes low when the PF bit is set. The periodic  flag and IRQ output operates when the device is running from either supply.

Note that writing to the subseconds  or seconds regis- ters affects the setting of the PF flag and IRQ output. The square-wave output uses a separate prescaler and is not affected by changes  to the subseconds  or sec- onds bits.

Control Register A (0Ah)

| BIT 7 | BIT 6 | BIT 5 | BIT 4 | BIT 3 | BIT 2 | BIT 1 | BIT 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TE | ENOSC | CCFG1 | CCFG0 | EPOL | SQWE | PIE | AIE |

Special-Purpose Registers

The DS1318 has three additional  registers (control A, control B, and status) that control the clock,  alarms, square wave, and interrupt output. The subseconds0 register has a square-wave synchronization (SQWS) bit in the bit 0 location. Writing the SQWS bit to 1 clears the square-wave prescaler and holds it in reset. Only the frequencies below 4096Hz are reset. Writing the bit back to 0 takes the prescaler out of reset and starts the square wave running.

Bit 7: Transfer Enable (TE). When TE is set to logic 1, the DS1318 continues to update the user copy of the time value as it receives 4,096Hz clock pulses from the oscillator. To ensure reading  valid time data from the part, the user should set TE to logic 0 before reading registers 00–05h. TE must be enabled  (logic 1) for at least 244µs to ensure that a transfer occurs. Note that because  of the 244µs restriction, sequential values of the subseconds0  register cannot be read when TE is used.

It is possible  that TE could  be set to logic  0 while a transfer is taking place. In that case, the buffered data could be invalid. To prevent this, the UIP bit, described later, should be used. To write data to the clock regis- ters, the user should set TE to logic 0, write the regis- ters, and set TE to logic 1.

Bit 6: Enable Oscillator (ENOSC). When ENOSC is set to logic  1, the DS1318 crystal oscillator becomes enabled. Actual startup time for the oscillator depends on many external variables and is not a specified parameter.

Bits 5, 4: Clock Configuration 1, 0 (CCFG1, CCFG0). These bits determine which of the four possible modes the DS1318 uses to clock its timekeeping registers:

Bit 3: External Polarity (EPOL). This bit controls the polarity on the EXT pin input when the CCFG1 and CCFG0 bits are equal to 0 and 1, respectively.  When EPOL is set to logic 1, the registers count when the EXT pin is 1. When EPOL is set to logic  0, the registers count when the EXT pin is logic 0.

Bit 2: Square-Wave Enable (SQWE). When SQWE is set to logic 1, a frequency determined by the SRSx bits in control register B (0Bh) is output on the SQW pin. When SQWE is logic 0, the SQW pin is always 0. When the part is in power-fail, the SQW pin is always high- impedance.  The square-wave output uses a separate prescaler from the one used by PF, IRQ, UIP, and the up counter. The SQWS bit in control register A can be used to synchronize the square-wave output to within

244µs of the other events.

Bit 1: Periodic Interrupt Enable (PIE). When PIE is set to logic 1, the DS1318 sets the IRQ pin low whenever the PF flag is set to 1. When PIE is 0, the PF flag does not affect the IRQ pin.

Bit 0: Alarm Interrupt Enable (AIE). When AIE is set to logic 1, the DS1318 sets the IRQ pin low whenever the ALMF flag is set to 1. When AIE is 0, the ALMF flag does not affect the IRQ pin.

DS1318

| CCFG1 | CCFG0 | MODE |
| --- | --- | --- |
| 0 | 0 | Always clocks the registers (normal mode) |
| 0 | 1 | Clocks when the EXT pin is “active” and VCC is greater than VPF (event-timer mode, depends on EPOL bit) |
| 1 | 0 | Clocks registers when part is running on VCC |
| 1 | 1 | Clocks registers when part is running on VBAT |

Control Register B (0Bh)

| BIT 7 | BIT 6 | BIT 5 | BIT 4 | BIT 3 | BIT 2 | BIT 1 | BIT 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PRS3 | PRS2 | PRS1 | PRS0 | SRS3 | SRS2 | SRS1 | SRS0 |

Bits 7 to 4: Periodic Rate Select (PRS3–PRS0). When the oscillator is enabled (ENOSC = 1) the PF flag is set at the rates determined by the following table:

| Periodic Flag Frequency When ENOSC = 1 PRS3 | PRS2 | PRS1 | PRS0 | PERIODIC FLAG FREQUENCY |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | Periodic Flag Not Set |
| 0 | 0 | 0 | 1 | 4096Hz |
| 0 | 0 | 1 | 0 | 2048Hz |
| 0 | 0 | 1 | 1 | 1024Hz |
| 0 | 1 | 0 | 0 | 512Hz |
| 0 | 1 | 0 | 1 | 256Hz |
| 0 | 1 | 1 | 0 | 128Hz |
| 0 | 1 | 1 | 1 | 8Hz |
| 1 | 0 | 0 | 0 | 4Hz |
| 1 | 0 | 0 | 1 | 2Hz |
| 1 | 0 | 1 | 0 | 1Hz |
| 1 | 0 | 1 | 1 | 1/64Hz (Once per 1.067 Minutes) |
| 1 | 1 | 0 | 0 | 1/4096Hz (Once per 1.138 Hours) |
| 1 | 1 | 0 | 1 | 1/65536Hz (Once per 1.318 Days) |
| 1 | 1 | 1 | 0 | 1/524288Hz (Once per 0.8669 Weeks) |
| 1 | 1 | 1 | 1 | 1/2097152Hz (Once per 24.27 Days) |

Bits 3 to 0: Square-Wave Rate Select (SRS3–SRS0). When the oscillator is enabled  (ENOSC = 1) and run- ning, and the square-wave pin is enabled (SQWE = 1), the SQW pin outputs a square-wave signal determined by the SRS bits according to the following table:

Square-Wave Output Frequency When SQWE = 1, ENOSC = 1

DS1318

| SRS3 | SRS2 | SRS1 | SRS0 | SQUARE-WAVE OUTPUT FREQUENCY (Hz) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 32,768 |
| 0 | 0 | 0 | 1 | 8192 |
| 0 | 0 | 1 | 0 | 4096 |
| 0 | 0 | 1 | 1 | 2048 |
| 0 | 1 | 0 | 0 | 1024 |
| 0 | 1 | 0 | 1 | 512 |
| 0 | 1 | 1 | 0 | 256 |
| 0 | 1 | 1 | 1 | 128 |
| 1 | 0 | 0 | 0 | 64 |
| 1 | 0 | 0 | 1 | 32 |
| 1 | 0 | 1 | 0 | 16 |
| 1 | 0 | 1 | 1 | 8 |
| 1 | 1 | 0 | 0 | 4 |
| 1 | 1 | 0 | 1 | 2 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0.5 |

Status Register (0Ch)

| BIT 7 | BIT 6 | BIT 5 | BIT 4 | BIT 3 | BIT 2 | BIT 1 | BIT 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OSF | UIP | 0 | 0 | 0 | 0 | PF | ALMF |

Bit 7: Oscillator Stop Flag (OSF). A logic 1 in this bit indicates that the oscillator either is or was stopped for some period  of time and may be used to judge  the validity of the timekeeping data. This bit is set to logic 1 any time the oscillator stops. The following are exam- ples of conditions that can cause the OSF bit to be set:

1) 	The first time power is applied.

2) 	The voltage present on both VCC and VBAT is insufficient to support oscillation.

3) 	The ENOSC bit is turned off in battery-backed mode.

4) 	External influences on the crystal (i.e., noise, leak- age, etc.)

Any write to the status register while this flag is active clears the bit to 0.

Bit 6: Update-In-Progress Flag (UIP). A logic 1 in the update-in-progress  bit indicates that the internal clock registers may be in the process  of updating  the user registers. Writing to any seconds or subseconds  regis- ters when this bit is logic 1 may cause a collision with the internal update and corrupt one or more of the user registers until the next update occurs. If the UIP bit is read and is logic 0, the user has at least 60µs to write to the device without the possibility of causing a colli- sion with the internal update. The internal timekeeping update is gated by the falling edge of UIP.

Reading the subseconds  and or seconds  registers while UIP is logic 1 may result in reading inconstant val- ues. If the UIP bit is read and is logic 0, the user has at least 60µs to read from the device without the possibili- ty of getting inconstant values.

Bit 1: Periodic Flag (PF). The periodic flag bit is set to

1 at the rate determined by the PRS bits in register 0Bh. If the PF bit is already 1 when the selected frequency attempts to set it to 1 again, no change  occurs.  The user must clear  the PIF flag  faster than the part attempts to set it to see the desired PF rate. If the PIE bit in register 0Ah is also set to logic 1, the IRQ pin is driven low in response to PF transitioning  to 1. Any write to the status register while this flag is active clears the bit to 0.

Bit 0: Alarm Flag (ALMF). A logic 1 in the alarm flag bit indicates that the contents of the seconds registers matched the contents of the alarm registers. If the AIE bit in register 0Ah is also set to logic 1, the IRQ pin is driven low in response to ALMF transitioning to 1. Any write to the status register while this flag is active clears the bit to 0.

UIP  vs.  Update Timing

8kHz

4kHz

DS1318

UIP

60µs

Chip Information

TRANSISTOR COUNT: 10,517

PROCESS: CMOS

SUBSTRATE CONNECTED TO GROUND

Thermal Information

Theta-JA: 125°C/W Theta-JC: 26°C/W

Package Information

For the latest package  outline information, go to

www.maxim-ic.com/DallasPackInfo.

DS1318

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

14  	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  2004 Maxim Integrated  Products 	Printed USA 	is a registered  trademark  of Maxim Integrated  Products.