Rev 3; 1/06

I2C, 32-Bit Binary Counter Watchdog RTC with

Trickle Charger and  Reset Input/Output

General Description

The DS1374 is a 32-bit binary counter designed to contin- uously count time in seconds. An additional counter gen- erates a periodic alarm or serves as a watchdog timer. If disabled,  this counter can be used as 3 bytes of non- volatile (NV) RAM. Separate output pins are provided for an interrupt and a square wave at one of four selectable frequencies. A precision temperature-compensated refer- ence and comparator circuit monitor the status of VCC to detect power failures, provide a reset output, and auto- matically switch to the backup  supply when necessary. Additionally,  the reset pin is monitored as a pushbutton input for externally generating a reset. The device is pro- grammed serially through an I2C* serial interface.

Applications

Portable Instruments

Point-of-Sale Equipment

Medical Equipment

Features

♦ 32-Bit Binary Counter

♦ Second Binary Counter Provides Time-of-Day

Alarm, Watchdog Timer, or NV RAM

♦ Separate Square-Wave and Interrupt Output Pins

♦ I2C Serial Interface

♦ Automatic Power-Fail Detect and Switch Circuitry

♦ Single-Pin Pushbutton Reset Input/Open-Drain

Reset Output

♦ Low-Voltage Operation

♦ Trickle-Charge Capability

♦ -40°C to +85°C Operating Temperature Range

♦ 10-Pin μSOP, 16-Pin SO

♦ Available in a Surface-Mount Package with an

Integrated Crystal (DS1374C)

♦ Underwriters Laboratory (UL) Recognized

Typical Operating Circuit

Telecommunications

*Purchase of I2C components of Maxim Integrated Products, Inc., or one of its sublicensed Associated Companies, conveys a license under the Philips I2C Patent Rights to use these com- ponents in an I2C system, provided that the system conforms to the I2C Standard Specification as defined by Philips.

RPU = tr/CB

RPU VCC

CPU

INT

VCC

RPU

CRYSTAL

X1	X2

SCL SDA

DS1374

INT

VCC

VCC

SQW

PRIMARY

Pin Configurations appear at the end of the data sheet.

RST

N.O. PUSHBUTTON RESET

RST

GND

VBACKUP 	     BATTERY, RECHARGEABLE BATTERY, OR

SUPER CAPACITOR

DS1374

Ordering Information

PART TEMP RANGE VOLTAGE (V) PIN-PACKAGE TOP MARK**

DS1374C-18 -40°C to +85°C 1.8 16 SO (300 mils) DS1374C-18

DS1374C-18# -40°C to +85°C 1.8 16 SO (300 mils) DS1374C-18

DS1374C-3 -40°C to +85°C 3.0 16 SO (300 mils) DS1374C-3

DS1374C-3# -40°C to +85°C 3.0 16 SO (300 mils) DS1374C-3

DS1374C-33 -40°C to +85°C 3.3 16 SO (300 mils) DS1374C-33

DS1374C-33# -40°C to +85°C 3.3 16 SO (300 mils) DS1374C-33

DS1374C-33/T&R -40°C to +85°C 3.3 16 SO (300 mils)/Tape and Reel DS1374C-33

Ordering Information continued at end of data sheet.

#Denotes a RoHS-compliant device that may include lead that is exempt under RoHS requirements. The lead finish is JESD97 category e3, and is compatible with both lead-based and lead-free soldering processes.

**A "#" anywhere on the top mark denotes a RoHS-compliant package.

ABSOLUTE MAXIMUM RATINGS

Voltage Range on VCC Pin Relative to Ground .....-0.3V to +6.0V Voltage Range on SDA, SCL, and WDS

Relative to Ground ....................................-0.3V to VCC + 0.3V Operating  Temperature  Range ...........................-40°C to +85°C

Storage Temperature  Range .............................-55°C to +125°C Soldering Temperature  Range ........................See the Handling, PC Board Layout, and Assembly section.

DS1374

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

RECOMMENDED DC OPERATING CONDITIONS

(VCC = VCC(MIN) to VCC(MAX), TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Supply Voltage (Notes 2, 3) | VCC | DS1374-33 | 2.97 3.3 5.50 | V |
|  |  | DS1374-3 | 2.7 3.0 3.3 |  |
|  |  | DS1374-18 | 1.71 1.8 1.89 |  |
| Input Logic 1 | VIH | (Note 2) | 0.7 x VCC VCC + 0.3 | V |
| Input Logic 0 | VIL | (Note 2) | -0.3 +0.3 x VCC | V |
| Pullup Resistor Voltage (INT, SQW, SDA, SCL), VCC = 0V | VPU | (Note 2) | 5.5 | V |
| Power-Fail Voltage (Note 2) | VPF | DS1374-33 | 2.70 2.88 2.97 | V |
|  |  | DS1374-3 | 2.45 2.6 2.7 |  |
|  |  | DS1374-18 | 1.51 1.6 1.71 |  |
| Backup Supply Voltage (Notes 2, 3, 4) | VBACKUP | DS1374-33 | 1.3 3.0 VCC (MAX) | V |
|  |  | DS1374-3, DS1374-18 | 1.3 3.0 3.7 |  |

DS1374

DC ELECTRICAL CHARACTERISTICS

(VCC = VCC(MIN) to VCC(MAX) , TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Trickle-Charge Current-Limiting Resistors | R1 | (Note 5) | 250 | Ω |
|  | R2 | (Note 6) | 2000 |  |
|  | R3 | (Note 7) | 4000 |  |
| Input Leakage | ILI | (Note 8) | -1 +1 | µA |
| I/O Leakage | ILO | (Note 9) | -1 +1 |  |
| RST Pin I/O Leakage | ILORST | (Note 10) | -200 +1 |  |
| SDA Logic 0 Output (VOL = 0.4V) | IOLSDA |  | 3.0 | mA |
| RST, SQW, and INT Logic 0 Outputs (Note 11) | IOL1 | VCC > 2V; VOL = 0.4V | 3.0 | mA |
|  |  | 1.71V < VCC < 2V; VOL = 0.2 VCC | 3.0 |  |
|  |  | 1.3V < VCC < 1.71V; VOL = 0.2 VCC | 250 | µA |
| Active Supply Current (Notes 11, 12) | ICCA | DS1374-18 | 75 150 | µA |
|  |  | DS1374-3 | 110 200 |  |
|  |  | DS1374-33 | 180 300 |  |
| Standby Current (Notes 11, 13) | ICCS | DS1374-18 | 60 100 | µA |
|  |  | DS1374-3 | 80 125 |  |
|  |  | DS1374-33 | 115 175 |  |
| VBACKUP Leakage Current (VBACKUP = 3.7V) | IBACKUPLKG |  | 100 | nA |

DC ELECTRICAL CHARACTERISTICS

(VCC = 0V, VBACKUP = 3.7V, TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MAX TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| VBACKUP Current (OSC ON); SQW OFF | IBKOSC1 | (Note 14) | 400 700 | nA |
| VBACKUP Current (OSC ON); SQW ON (32kHz) | IBKOSC2 | (Notes 14, 15) | 600 1000 | nA |
| VBACKUP Data-Retention Current (OSC OFF) | IBACKUPDR |  | 25 100 | nA |

DS1374

AC ELECTRICAL CHARACTERISTICS

(VCC = VCC(MIN) to VCC(MAX), TA = -40°C to +85°C, unless otherwise noted.) (Note 1) (Figure 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX |  | UNITS |
| --- | --- | --- | --- | --- | --- |
| SCL Clock Frequency (Note 16) | fSCL | Fast mode | 100 400 |  | kHz |
|  |  | Standard mode | 0 100 |  |  |
| Bus Free Time Between STOP and START Conditions | tBUF | Fast mode | 1.3 |  | µs |
|  |  | Standard mode | 4.7 |  |  |
| Hold Time (Repeated) START Condition (Note 17) | tHD:STA | Fast mode | 0.6 |  | µs |
|  |  | Standard mode | 4.0 |  |  |
| Low Period of SCL Clock | tLOW | Fast mode | 1.3 |  | µs |
|  |  | Standard mode | 4.7 |  |  |
| High Period of SCL Clock | tHIGH | Fast mode | 0.6 |  | µs |
|  |  | Standard mode | 4.0 |  |  |
| Data Hold Time (Notes 18, 19) | tHD:DAT | Fast mode | 0 0.9 |  | µs |
|  |  | Standard mode | 0 0.9 |  |  |
| Data Setup Time (Note 20) | tSU:DAT | Fast mode | 100 |  | ns |
|  |  | Standard mode | 250 |  |  |
| Start Setup Time | tSU:STA | Fast mode | 0.6 |  | µs |
|  |  | Standard mode | 4.7 |  |  |
| Rise Time of Both SDA and SCL Signals (Note 16) | tR | Fast mode | 20 + 0.1CB | 300 | ns |
|  |  | Standard mode |  | 1000 |  |
| Fall Time of Both SDA and SCL Signals (Note 16) | tF | Fast mode | 20 + 0.1CB | 300 | ns |
|  |  | Standard mode |  | 300 |  |
| Setup Time for STOP Condition | tSU:STO | Fast mode | 0.6 |  | µs |
|  |  | Standard mode | 4.7 |  |  |
| Capacitive Load for Each Bus Line | CB | (Note 16) | 400 |  | pF |
| I/O Capacitance (SDA, SCL) | CI/O | (Note 21) | 10 |  | pF |
| Pulse Width of Spikes That Must be Suppressed by the Input Filter | tSP | Fast mode | 30 |  | ns |
| Pushbutton Debounce | PBDB | (Figure 2) | 250 |  | ms |
| Reset Active Time | tRST | (Figure 2) | 250 |  | ms |
| Oscillator Stop Flag (OSF) Delay | tOSF | (Note 22) | 100 |  | ms |

DS1374

POWER-UP/POWER-DOWN CHARACTERISTICS

(TA = -40°C to +85°C) (Figure 3)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| VCC Detect to Recognize Inputs (VCC Rising) | tRPU | (Note 23) | 250 | ms |
| VCC Fall Time; VPF(MAX) to VPF(MIN) | tF |  | 300 | µs |
| VCC Rise Time; VPF(MIN) to VPF(MAX) | tR |  | 0 | µs |

WARNING: Under no circumstances are negative undershoots, of any amplitude, allowed when the device is in write protection.

Note 1:    Limits at -40°C are guaranteed by design and not production tested.

Note 2:    All voltages are referenced to ground.

Note 3:    VBACKUP should not exceed VCC MAX or 3.7V, whichever  is greater.

Note 4:    The use of the 250Ω trickle-charge resistor is not allowed at VCC > 3.63V and should not be enabled.

Note 5:    Measured  at VCC = typ, VBACKUP = 0V, register 09h = A5h. Note 6:    Measured  at VCC = typ, VBACKUP = 0V, register 09h = A6h. Note 7:    Measured  at VCC = typ, VBACKUP = 0V, register 09h = A7h. Note 8:    SCL only.

Note 9:    SDA and SQW and INT.

Note 10:  The RST pin has an internal 50kΩ pullup resistor to VCC.

Note 11:  Trickle charger  disabled.

Note 12:  ICCA—SCL clocking  at max frequency  = 400kHz.

Note 13:  Specified  with I2C bus inactive.

Note 14:  Measured  with a 32.768kHz crystal attached  to the X1 and X2 pins.

Note 15:  WDSTR = 1. BBSQW = 1 is required  for operation when VCC is below the power-fail trip point (or absent).

Note 16:  CB—total capacitance of one bus line in pF.

Note 17:  After this period, the first clock pulse is generated.

Note 18:  The maximum tHD:DAT only has to be met if the device does not stretch the low period (tLOW) of the SCL signal.

Note 19:  A device must internally provide a hold time of at least 300ns for the SDA signal (referred to as the VIHMIN of the SCL sig- nal) to bridge  the undefined  region of the falling edge of SCL.

Note 20:  A fast-mode  device can be used in a standard-mode system, but the requirement  tSU:DAT ≥ to 250ns must be met. This is automatically the case if the device does not stretch the low period of the SCL signal. If such a device does stretch the low period of the SCL signal, it must output the next data bit to the SDA line tR max + tSU:DAT = 1000 + 250 = 1250ns before the SCL line is released.

Note 21: Guaranteed by design. Not production tested.

Note 22: The parameter tOSF is the period of time the oscillator must be stopped for the OSF flag to be set over the voltage range of

0V ≤ VCC ≤ VCC MAX and 1.3V ≤ VBACKUP ≤ 3.7V.

Note 23: This delay applies only if the oscillator is enabled and running. If the EOSC bit is 1, the startup time of the oscillator is added to this delay.

SDA

tBUF

tLOW

tR 	tF

tHD:STA

tSP

SCL

STOP	START

tHD:STA

tHD:DAT

tHIGH

tSU:DAT 	REPEATED START

tSU:STA

tSU:STO

Figure 1. Data Transfer on I2C Serial Bus

RST

PBDB

tRST

Figure 2. Pushbutton Reset Timing

VCC

VPF(MAX)

VPF(MIN)

VPF	VPF

tF 	tR

tRST

tRPU

RST

INPUTS

OUTPUTS

RECOGNIZED

VALID

DON'T CARE

HIGH-Z

RECOGNIZED

VALID

DS1374

Figure 3. Power-Up/Power-Down Timing

Typical Operating Characteristics

(VCC = +3.3V, TA = +25°C, unless otherwise noted.)

450

SUPPLY CURRENT (nA)

400

350

300

IBAT0SC1 vs. VBAT

SQUARE-WAVE OFF

DS1374 toc01

VCC = 0V

800

750

SUPPLY CURRENT (nA)

700

650

600

550

500

450

400

350

IBAT0SC2 vs. VBAT

SQUARE-WAVE  ON

DS1374 toc02

VCC = 0V

450

425

375

IBATOSC1 vs. TEMPERATURE VBAT = 3.0V

DS1374 toc03

VCC = 0V

1.3

1.8

2.3

2.8

3.3

3.8

4.3

4.8

5.3

1.3

1.8

2.3

2.8

3.3

3.8

4.3

4.8

5.3

-40

-20	0

20	40

60	80

275

250

SUPPLY CURRENT (μA)

225

200

175

150

125

100

75

VBAT (V)

ICCA vs. VCC

DS1374 toc04

(SQUARE-WAVE  ON)

VBAT (V)

32768.7

32768.6

32768.5

32768.4

FREQUENCY (Hz)

32768.3

32768.2

32768.1

TEMPERATURE (°C)

SUPPLY CURRENT (nA)

OSCILLATOR FREQUENCY vs. VBACKUP

DS1374 toc05

VCC = 0V

50

1.8

2.3

2.8    3.3

3.8    4.3

4.8

5.3

1.3

1.8

2.3

2.8

3.3

3.8

4.3

4.8

5.3

VCC (V)

VBACKUP (V)

1000

VCC FALLING vs. RST DELAY

DS1374 toc06

VCC = 3.0V TO 0V

100

RESET DELAY (μs)

10

1

0.1

0.01

0.10	1

10	100

DS1374

VCC FALLING (V/ms)

Pin  Description

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| µSOP | SO |  |  |
| 1, 2 | — | X1, X2 | Connections for a Standard 32.768kHz Quartz Crystal. The internal oscillator circuitry is designed for operation with a crystal having a specified load capacitance (CL) of 6pF. Pin X1 is the input to the oscillator and can optionally be connected to an external 32.768kHz oscillator. The output of the internal oscillator, pin X2, is floated if an external oscillator is connected to pin X1. |
| 3 | 13 | VBACKUP | Connection for a Secondary Power Supply. This supply is used to operate the oscillator and counters when VCC is absent. Supply voltage must be held between 1.3V and 3.7V (-18 and -3) or 1.3V and 5.5V (-33) for proper operation. This pin can be connected to a primary cell such as a lithium cell. Additionally, this pin can be connected to a rechargeable cell or a super cap when used with the trickle- charge feature. UL recognized to ensure against reverse charging when used with a lithium battery. This pin must be grounded if not used. |
| 4 | 14 | RST | Active-Low, Open-Drain Output with a Debounced Pushbutton Input. This pin can be activated by a pushbutton reset request, a watchdog alarm condition, or a power-fail event. It has an internal 50kΩ pullup resistor. No external resistors should be connected. If the crystal oscillator is disabled, the startup time of the oscillator is added to the tRST delay. |
| 5 | 15 | GND | Ground |
| 6 | 16 | SDA | Serial Data Input/Output. SDA is the input/output for the 2-wire serial interface. The SDA pin is open drain and requires an external pullup resistor. |
| 7 | 1 | SCL | Serial Clock Input. SCL is the clock input for the 2-wire serial interface and is used to synchronize data movement on the serial interface. |
| 8 | 2 | INT | Interupt. This pin is used to output the alarm interrupt or the watchdog reset signal. It is active-low open drain and requires an external pullup resistor. |
| 9 | 3 | SQW | Square-Wave Output. This pin is used to output the programmable square-wave signal. It is open drain and requires an external pullup resistor. |
| 10 | 4 | VCC | DC Power for Primary Power Supply |
| — | 5–12 | N.C. | No Connection. Must be connected to ground. |

X1

CLOCK DIVIDER

1Hz

4.096kHz

8.192kHz

MUX

SQW

X2	32.768kHz	1Hz/4.096kHz

INT

INT

VCC VBACKUP GND

SDA SCL

POWER CONTROL AND TRICKLE CHARGE

2-WIRE INTERFACE

32-BIT

COUNTER

ALARM/ WATCHDOG

STAT/CTRL/ TRICKLE

24-BIT COUNTER

CONTROL

RST CONTROL

RST

DS1374

DS1374

Figure 4. Functional Diagram

| Table 1. Crystal Specifications* PARAMETER | SYMBOL | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| Nominal Frequency | fO | 32.768 | kHz |
| Series Resistance | ESR | 45 | kΩ |
| Load Capacitance | CL | 6 | pF |

LOCAL GROUND PLANE (LAYER 2)

X1

CRYSTAL

X2

*The crystal, traces, and crystal input pins should be isolated from RF generating signals. Refer to Application Note 58: Crystal Considerations for Dallas Real-Time Clocks for addi- tional specifications.

GND

NOTE: AVOID ROUTING SIGNALS IN THE CROSSHATCHED AREA (UPPER LEFT-HAND QUADRANT)  OF THE PACKAGE UNLESS THERE IS A GROUND PLANE BETWEEN THE SIGNAL LINE AND THE PACKAGE.

Figure 6. Layout Example

DS1374

CL1 	CL2

X1	X2

CRYSTAL

COUNTDOWN CHAIN

RTC REGISTERS

Oscillator Circuit

The DS1374 uses an external 32.768kHz crystal. The oscillator circuit does not require any external resistors or capacitors to operate. Table 1 specifies several crys- tal parameters for the external crystal. Figure 5 shows a functional schematic of the oscillator circuit. The startup time is usually less than 1 second when using a crystal with the specified characteristics.

Clock Accuracy

Clock accuracy  is dependent  upon the accuracy  of the

DS1374

Figure 5. Oscillator Circuit Showing Internal Bias Network

Detailed Description The DS1374 is a real-time clock with an I2C serial inter- face. It provides elapsed seconds from a user-defined

starting point in a 32-bit counter (Figure 4). A 24-bit

counter can be configured  as either a watchdog counter or an alarm counter. An on-chip oscillator cir- cuit uses a customer-supplied  32.768kHz crystal to keep time. A power-control circuit  switches operation from VCC to VBACKUP and back when power on VCC is cycled. The oscillator and counters continue to operate when powered  by either supply.  If a rechargeable backup  supply  is used,  a trickle  charger  can be enabled to charge the backup supply while VCC is on.

crystal and the accuracy  of the match between the capacitive load of the oscillator circuit and the capacitive load for which the crystal was trimmed. Additional error is added  by crystal frequency drift caused by tempera- ture shifts. External circuit noise coupled into the oscilla- tor circuit can result in the clock running fast. Figure 6 shows a typical PC board layout for isolating the crystal and oscillator from noise. Refer to Application  Note 58: Crystal Considerations with Dallas Real-Time Clocks for detailed information.

DS1374C Only The DS1374C integrates a standard 32,768Hz crystal into the package. Typical accuracy at nominal VCC and

25°C is approximately 10ppm. See Application  Note 58

for information about crystal accuracy vs. temperature.

Power Control

The power-control  function is provided  by a precise, temperature-compensated  voltage reference and a comparator  circuit  that monitors the VCC level. The device is fully accessible  and data can be written and read when VCC is greater than VPF. However, when VCC falls below VPF, the internal clock  registers are blocked from any access. If VPF is less than VBACKUP, the device  power is switched  from VCC to VBACKUP when VCC drops  below VPF. If VPF is greater than VBACKUP, the device power is switched  from VCC to VBACKUP when VCC drops below VBACKUP. The regis- ters are maintained from the VBACKUP source until VCC

is returned to nominal levels (Table 1). After VCC returns above VPF, read and write access  is allowed after RST goes high (Figure 1).

| Table 2. Power Control SUPPLY CONDITION | READ/WRITE ACCESS | POWERED BY |
| --- | --- | --- |
| VCC < VPF, VCC < VBACKPUP | No | VBACKUP |
| VCC < VPF, VCC > VBACKUP | No | VCC |
| VCC > VPF, VCC < VBACKUP | Yes | VCC |
| VCC > VPF, VCC > VBACKUP | Yes | VCC |

Address Map

Table 3 shows the address map for the DS1374 regis- ters. During a multibyte access,  the address  pointer wraps around to location 00h when it reaches the end of the register space (08h). On an I2C START, STOP, or address pointer incrementing to location 00h, the current time is transferred to a second set of registers. These secondary registers read the time information, while the clock continues to run. This eliminates the need to reread the registers in case of an update of the main registers during a read.

Time-of-Day Counter

The time-of-day  counter is a 32-bit up counter that increments once per second when the oscillator is run- ning. The contents can be read or written by accessing the address range 00h–03h. When the counter is read, the current time of day is latched into a register, which is output on the serial data line while the counter contin- ues to increment.

Note: Writing to any TOD register will reset the 1Hz square wave output.

DS1374

Table 3. Address Map

| ADDRESS | BIT 7 | BIT 6 | BIT 5 | BIT 4 | BIT 3 | BIT 2 | BIT 1 | BIT 0 | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00H | TOD Counter Byte 0 |  |  |  |  |  |  |  | Time-of-Day Counter |
| 01H | TOD Counter Byte 1 |  |  |  |  |  |  |  | Time-of-Day Counter |
| 02H | TOD Counter Byte 2 |  |  |  |  |  |  |  | Time-of-Day Counter |
| 03H | TOD Counter Byte 3 |  |  |  |  |  |  |  | Time-of-Day Counter |
| 04H | WD/ALM Counter Byte 0 |  |  |  |  |  |  |  | Watchdog/Alarm Counter |
| 05H | WD/ALM Counter Byte 1 |  |  |  |  |  |  |  | Watchdog/Alarm Counter |
| 06H | WD/ALM Counter Byte 2 |  |  |  |  |  |  |  | Watchdog/Alarm Counter |
| 07H | EOSC | WACE | WD/ALM | BBSQW | WDSTR | RS2 | RS1 | AIE | Control |
| 08H | OSF | 0 | 0 | 0 | 0 | 0 | 0 | AF | Status |
| 09H | TCS3 | TCS2 | TCS1 | TCS0 | DS1 | DS0 | ROUT1 | ROUT0 | Trickle Charger |

Note: Unless otherwise specified, the state of the registers is not defined when power is first applied.

DS1374

Watchdog/Alarm Counter

The contents of the watchdog/alarm  counter, which is a separate 24-bit down counter, are accessed  in the address range 04h–06h. When this counter is written, the counter and a seed register are loaded with the desired value. When the counter is to be reloaded, it uses the value in the seed register. When the counter is read, the current counter value is latched into a register, which is output on the serial data line while the counter continues to decrement.

IIf the counter is not needed,  it can be disabled  and used as a 24-bit cache  of NV RAM by setting  the WACE bit in the control register to logic 0. If all 24 bits of the watchdog/alarm  counter are written to zero, the counter is disabled,  independent  of the WACE bit set- ting. When the watchdog  counter is is written to a nonzero value, and WACE is written to logic 1, the func- tion of the counter is determined by the WD/ALM bit.

When the WD/ALM bit in the control register is set to logic 0, the WD/ALM counter decrements every second until it reaches zero. At this point, the AF bit in the sta- tus register is set to 1 and the counter is reloaded and restarted. AF remains set until cleared by writing it to 0. If AIE = 1, the INT pin goes active whenever AF = 1. WDSTR does not affect operation when WD/ALM = 0.

When the WD/ALM bit is set to logic  1, the WD/ALM counter decrements every 1/4096 of a second (approx- imately every 244us) until it reaches zero. When any of the watchdog counters bytes are read, the seed value is reloaded  and the counter restarts. Writing to the watchdog counter updates the seed value and reloads the counter with the new seed value. When the counter reaches zero, the AF bit is set and the counter stops.

If WDSTR = 0, the RST pin pulses low for 250ms, and accesses to the device are inhibited. At the end of the

250ms pulse, the AF bit is cleared to zero, the RST pin becomes  high impedance,  and read/write access  to the device is enabled.

If WDSTR = 1 and the counter reaches zero, the AF bit is set and the counter stops. If AIE = 0, AF remains set until cleared by writing it to 0. If AIE = 1, the INT pin pulses low for 250ms. At the end of the 250ms pulse, the AF bit is cleared and INT becomes high impedance.

The 250ms pulse on INT or RST cannot be truncated by writing either AF or AIE to zero during the low time. If the INT counter is written during  the 250ms pulse, the counter starts decrementing upon the pulse completion.

The watchdog and alarm function operates from VCC or VBAT. When the AF bit is set, INT is pulled low when the device is powered by VCC or VBAT.

Note: WACE must be toggled  from logic 0 to logic 1 after the watchdog counter is written from a zero to a nonzero value.

Power-Up/Power-Down Reset and

Pushbutton Reset Functions

A precision temperature-compensated  reference and comparator circuit monitors the status of VCC. When an out-of-tolerance condition occurs, an internal power-fail signal is generated  that forces the RST pin low and blocks  read/write access  to the DS1374. When VCC returns to an in-tolerance condition, the RST pin is held low for 250ms to allow the power supply to stabilize. If the EOSC bit is set to a logic 1 (to disable the oscillator in battery-backup  mode), the reset signal is kept active for

250ms plus the startup time of the oscillator.

The DS1374 provides for a pushbutton switch to be con- nected to the RST output pin. When the DS1374 is not in a reset cycle, it continuously monitors the RST signal for a low-going edge. If an edge is detected,  the DS1374 debounces  the switch by pulling the RST pin low and inhibits read/write access. After the internal 250ms timer has expired,  the device continues to monitor the RST line. If the line is still low, the DS1374 continues to moni- tor the line, looking for a rising edge. Upon detecting release, the DS1374 forces the RST pin low and holds it low for an additional 250ms.

| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EOSC | WACE | WD/ALM | BBSQW | WDSTR | RS2 | RS1 | AIE |

DS1374

Special Purpose Registers

The DS1374 has two additional registers (07h–08h) that control the WD/ALM counter and the square-wave, inter- rupt, and reset outputs.

Control Register (07h) Bit 7/Enable Oscillator (EOSC).  When set to logic 0, the oscillator is started. When set to logic 1, the oscilla- tor is stopped. When this bit is set to logic 1, the oscilla- tor is stopped  and  the DS1374 is placed  into a low-power standby mode (IDDR). This bit is clear (logic

0) when power is first applied.  When the DS1374 is powered by VCC, the oscillator is always on regardless of the state of the EOSC bit.

Bit 6/WD/ALM Counter Enable (WACE). When set to logic 1, the WD/ALM counter is enabled. When set to logic 0, the WD/ALM counter is disabled,  and the 24 bits can be used as NV RAM. This bit is clear (logic 0) when power is first applied.

Bit 5/WD/ALM Counter Select (WD/ALM). When set to logic 0, the counter decrements  every second  until it reaches zero and is then reloaded and restarted. When set to logic 1, the WD/ALM counter decrements  every

1/4096 of a second (approximately every 244µs) until it reaches zero, sets the AF bit in the status register, and stops. If any of the WD/ALM counter registers are accessed before the counter reaches zero, the counter is reloaded  and restarted. This bit is clear (logic  0) when power is first applied.

Bit 4/Battery-Backed Square-Wave Enable (BBSQW). This bit, when set to logic 1, enables the square-wave output when VCC is absent and when the DS1374 is being powered by the VBACKUP pin. When BBSQW is

logic 0, the SQW pin goes high impedance  when VCC falls below the power-fail trip point. This bit is disabled (logic 0) when power is first applied.

Bit 3/Watchdog Reset Steering Bit (WDSTR). This bit selects which output pin the watchdog-reset  signal occurs  on. When the WDSTR bit is set to logic  0, a

250ms pulse occurs on the RST pin if WD/ALM = 1 and the WD/ALM counter reaches zero. The 250ms reset pulse occurs on the INT pin when the WDSTR bit is set to logic 1. This bit is logic 0 when power is first applied.

Bits 2, 1/Rate Select (RS2 and RS1). These bits con- trol the frequency of the square-wave output when the square wave has been enabled.  Table 4 shows the square-wave frequencies that can be selected with the RS bits. These bits are both set (logic 1) when power is first applied.

Bit 0/Alarm Interrupt Enable (AIE). When set to logic

1, this bit permits the alarm flag (AF) bit in the status register to assert INT (when INTCN = 1). When set to logic 0 or INTCN is set to logic 0, the AF bit does not initiate the INT signal. If the WD/ALM bit is set to logic 1 and the AF flag is set, writing AIE to zero does not trun- cate the 250ms pulse on the INT pin. The AIE bit is at logic 0 when power is first applied.  The INT output is available while the device is powered by either supply.

| Table 4. Square-Wave Output Frequency RS2 | RS1 | SQUARE-WAVE OUTPUT FREQUENCY |
| --- | --- | --- |
| 0 | 0 | 1Hz |
| 0 | 1 | 4.096kHz |
| 1 | 0 | 8.192kHz |
| 1 | 1 | 32.768kHz |

| Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OSF | 0 | 0 | 0 | 0 | 0 | 0 | AF |

Status Register (08h) Bit 7/Oscillator Stop Flag (OSF). A logic 1 in this bit indicates  that the oscillator either is stopped  or was stopped  for some period  of time and can be used to judge the validity of the timekeeping data. This bit is set to logic 1 any time the oscillator stops. The following are examples of conditions that can cause the OSF bit to be set:

1)  The first time power is applied.

2)  The voltage present on both VCC and VBACKUP are insufficient to support oscillation.

3)  The EOSC bit is turned off.

4)  External influences on the crystal (i.e., noise, leak- age, etc.).

This bit remains at logic 1 until written to logic 0.

Bit 0/Alarm Flag (AF). A logic 1 in the alarm flag bit indicates  that the WD/ALM counter reached  zero. If WD/ALM is set to zero and the AIE bit = 1, the INT pin goes low and stays low until AF is cleared.  AF is cleared  when written to logic  0. This bit can only be written to logic 0. Attempting to write logic 1 leaves the value unchanged.  If WD/ALM is set to 1 and the AIE bit = 1, the INT pin pulses low for 250ms when the WD/ALM counter reaches zero and sets AF = 1. At the pulse completion, the DS1374 clears the AF bit to zero. If the 250ms pulse is active, writing AF to zero does not truncate the pulse.

Trickle-Charge Register (10h) The simplified schematic  in Figure 7 shows the basic components  of the trickle charger.  The trickle-charge

select (TCS) bits (bits 4–7) control the selection of the trickle charger. To prevent accidental  enabling, only a pattern of 1010 enables the trickle charger.  All other patterns disable the trickle charger. The trickle charger is disabled  when power is first applied.  The diode select (DS) bits (bits 2, 3) select whether or not a diode is connected  between VCC and VBACKUP. If DS is 01, no diode is selected; if DS is 10, a diode is selected. The ROUT bits (bits 0, 1) select the value of the resistor connected between VCC and VBACKUP. Table 5 shows the resistor selected by the resistor select (ROUT) bits and the diode selected by the diode select (DS) bits.

Warning: The ROUT value of 250Ω must not be select- ed whenever VCC is greater than 3.63V.

The user determines  diode  and resistor selection according  to the maximum current desired for battery or super cap charging. The maximum charging current can be calculated as illustrated in the following example.

Assume that a system power supply of 3.3V is applied to VCC and a super cap is connected to VBACKUP. Also assume the trickle charger  has been enabled  with a diode and resistor R2 between VCC and VBACKUP. The maximum current IMAX  would therefore be calculated as follows:

IMAX = (3.3V - diode drop) / R2 ≈ (3.3V - 0.7V) / 2kΩ ≈

1.3mA

As the super cap changes, the voltage drop between VCC and VBACKUP decreases and therefore the charge current decreases.

DS1374

Table 5. Trickle Charge Register

| TCS3 | TCS2 | TCS1 | TCS0 | DS1 | DS0 | ROUT1 | ROUT0 | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | X | X | X | 0 | 0 | X | X | Disabled |
| X | X | X | X | 1 | 1 | X | X | Disabled |
| X | X | X | X | X | X | 0 | 0 | Disabled |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | No diode, 250Ω resistor |
| 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | One diode, 250Ω resistor |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | No diode, 2kΩ resistor |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | One diode, 2kΩ resistor |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | No diode, 4kΩ resistor |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | One diode, 4kΩ resistor |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Power-on reset value |

BIT 7

TCS3

BIT 6

TCS2

BIT 5

TCS1

BIT 4

TCS0

BIT 3

DS1

BIT 2

DS0

BIT 1

ROUT1

BIT 0

ROUT0

TCS0-3  = TRICKLE CHARGER SELECT DS0-1  = DIODE SELECT

TOUT0-1 = RESISTOR SELECT

1 OF 16 SELECT

NOTE: ONLY 1010b

ENABLES CHARGER

1 OF 2

SELECT

1 OF 3

SELECT

R1

250Ω

VCC

R2

2kΩ

R3

4kΩ

VBACKUP

Figure 7. Programmable Trickle Charger

SDA

MSB

SLAVE ADDRESS

R/W DIRECTION BIT

ACKNOWLEDGEMENT SIGNAL FROM RECEIVER

ACKNOWLEDGEMENT SIGNAL  FROM RECEIVER

SCL

1	2	6	7	8	9	1	2

3–7	8	9

START CONDITION

Figure 8. I2C Data Transfer Overview

ACK

REPEATED IF MORE BYTES ARE TRANSFERED

ACK

STOP CONDITION OR REPEATED START CONDITION

DS1374

I2C Serial Data Bus The  DS1374 supports the I2C bus protocol.  A device that sends data onto the bus is defined as a transmitter and a device receiving data is a receiver. The device that controls the message is called  a master. The devices that are controlled by the master are slaves. A master device that generates the serial clock  (SCL),

controls the bus access, and generates the START and STOP conditions  must control the bus. The DS1374 operates as a slave on the I2C bus. Connections to the bus are made through the open-drain  I/O lines SDA

and SCL. A standard mode (100kHz max clock  rate) and a fast mode (400kHz max clock rate) are defined within the bus specifications. The DS1374 works in both modes.

The following bus protocol has been defined (Figure 8):

•  Data transfer can be initiated only when the bus is not busy.

•  During data transfer, the data line must remain sta- ble whenever the clock line is high. Changes in the data line while the clock line is high can be interpret- ed as control signals.

R/W

SLAVE ADDRESS

REGISTER ADDRESS (n)

DATA (n)

DATA (n + 1) 	DATA (n + x)

SLAVE ADDRESS

DATA (n) 	DATA (n + 1)

DATA (n + 2)

DATA (n + x)

| S - START S | 1101000 | 0 | A | XXXXXXXX | A | XXXXXXXX |  | A | XXXXXXXX | A | XXXXXXXX | P |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

A - ACKNOWLEDGE P - STOP

R/W - READ/WRITE OR DIRECTION BIT

DATA TRANSFERRED (X+1 Bytes + Acknowledge)

S - START

| A - ACKNOWLEDGE P - STOP S | 1101000 | 1 | A |  | XXXXXXXX | A | XXXXXXXX | A | XXXXXXXX | A | XXXXXXXX | /A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R/W

/A - NOT ACKNOWLEDGE R/W - READ/WRITE OR DIRECTION BIT

DATA TRANSFERRED (X+1 Bytes + Acknowledge)

Figure 9. I2C Write Protocol

Figure 10. I2C Read Protocol

DS1374

Accordingly,  the following bus conditions  have been defined:

Bus not busy: Both data and clock  lines remain high.

Start data transfer: A change  in the state of the data line from high to low, while the clock  line is high, defines a START condition.

Stop data transfer: A change  in the state of the data line from low to high, while the clock  line is high, defines a STOP condition.

Data valid: The state of the data line represents valid data when, after a START condition, the data line is stable for the duration of the high period of the clock  signal. The data on the line must be changed during the low period of the clock signal. There is one clock pulse per bit of data.

Each data transfer is initiated with a START condi- tion and terminated with a STOP condition.  The number of data bytes transferred  between the START and the STOP conditions is not limited, and is determined  by the master device. The informa- tion is transferred  byte-wise and each receiver acknowledges  with a ninth bit. A standard mode (100kHz clock rate) and a fast mode (400kHz clock rate) are defined within the I2C bus specifications.

Acknowledge:  Each receiving  device,  when addressed,  is obliged  to generate an acknowledge after the reception of each byte. The master device must generate an extra clock pulse that is associat- ed with this acknowledge bit.

A device that acknowledges  must pull down the SDA line during  the acknowledge  clock  pulse in such a way that the SDA line is stable low during the high period  of the acknowledge-related  clock pulse. Setup and hold times must be considered. A master must signal an end of data to the slave by not generating an acknowledge bit on the last byte that has been clocked out of the slave. In this case,

the slave must leave the data line high to enable the master to generate the STOP condition.

Figures 9 and 10 detail how data transfer is accom- plished on the 2-wire bus. Depending  on the state of the R/W bit, two types of data transfer are possible:

Data transfer from a master transmitter to a slave receiver. The first byte transmitted by the master is the slave address. Next follows a number of data bytes. The slave returns an acknowledge bit after each received byte.

Data transfer from a slave transmitter to a mas- ter receiver. The master transmits the first byte (the slave address).  The slave then returns an acknowledge  bit. Next follows a number of data bytes transmitted by the slave to the master. The master  returns  an acknowledge  bit  after all received bytes other than the last byte. At the end of the last received  byte, a “not acknowledge”  is returned.

The master device generates the serial clock puls- es and the START and STOP conditions. A transfer is ended with a STOP condition or with a repeated START condition. Since a repeated START condi- tion is also the beginning of the next serial transfer, the bus is not released.

The DS1374 can operate in the following two modes:

Slave Receiver Mode (Write Mode): Serial data and clock data are received through SDA and SCL. After each byte is received, an acknowledge bit is transmitted. START and STOP conditions  are rec- ognized as the beginning and end of a serial trans- fer. Address recognition is performed by hardware after reception of the slave address  and direction bit.  The slave address  byte  is the first byte received after the master generates a START con- dition. The slave address  byte contains the 7-bit DS1374 address, which is 1101000, followed by the direction  bit (R/W), which is zero for a write. After receiving  and decoding  the slave address byte, the DS1374 outputs an acknowledge on SDA.

After the DS1374 acknowledges  the slave address

+ write bit, the master transmits a register address to the DS1374. This sets the register pointer on the DS1374, with the DS1374 acknowledging  the trans- fer. The master can then transmit zero or more bytes of data, with the DS1374 acknowledging each byte received. The register pointer increments after each data byte is transferred. The master gen- erates a STOP condition to terminate the data write.

Slave Transmitter Mode (Read Mode): The first byte is received and handled as in the slave receiv- er mode. However, in this mode, the direction  bit indicates  that the transfer direction  is reversed. Serial data is transmitted on SDA by the DS1374, while the serial clock is input on SCL. START and STOP conditions  are recognized  as the beginning and end of a serial transfer. Address recognition is performed by hardware after reception of the slave address  and direction  bit. The slave address  byte is the first byte received after the START condition is generated by the master. The slave address byte

contains  the  7-bit  DS1374  address,  which  is

1101000, followed by the direction bit (R/W), which is 1 for a read. After receiving and decoding  the slave  address  byte,  the DS1374 outputs  an acknowledge  on SDA. The DS1374 then begins to transmit data starting with the register address pointed  to by the register pointer. If the register

pointer is not written to before the initiation of a read mode, the first address that is read is the last one stored in the register pointer. The DS1374 must receive a not acknowledge to end a read.

Handling, PC Board Layout, and

Assembly

The DS1374C package  contains a quartz tuning-fork crystal. Pick-and-place  equipment  can be used, but precautions  should be taken to ensure that excessive shocks are avoided.  Ultrasonic cleaning  should be avoided to prevent damage to the crystal.

Avoid running signal traces under the package, unless a ground plane is placed between the package and the signal line. All no connect (N.C.) pins must be connect- ed to ground.

The SO package can be reflowed as long as the peak temperature does not exceed 240°C. Peak reflow tem- perature (≥ 230°C) duration should not exceed 10 sec- onds, and the total time above  200°C should not exceed 40 seconds (30 seconds nominal). Exposure to reflow is limited to 2 times maximum.

Moisture-sensitive packages are shipped from the facto- ry dry-packed.  Handling instructions listed on the pack- age label must be followed to prevent damage  during reflow. Refer to the IPC/JEDEC J-STD-020B standard for moisture-sensitive device (MSD) classifications.

Pin  Configurations

TOP VIEW

SCL	1

16 	SDA

X1    1

X2    2

VBACKUP   3

RST   4

GND    5

DS1374

μSOP

10    VCC

9     SQW

8     INT

7     SCL

6     SDA

INT SQW VCC

N.C.

N.C. N.C.

N.C.

2	15

3	14

4	DS1374C   13

5	12

6	11

7	10

8	9

GND RST VBACKUP

N.C.

N.C. N.C.

N.C.

DS1374

SO (0.300")

Ordering Information (continued)

PART TEMP RANGE VOLTAGE (V) PIN-PACKAGE TOP MARK*

DS1374U-18 -40°C to +85°C 1.8 10 µSOP DS1374-18

DS1374U-18+ -40°C to +85°C 1.8 10 µSOP DS1374-18

DS1374U-3 -40°C to +85°C 3.0 10 µSOP DS1374-3

DS1374U-3+ -40°C to +85°C 3.0 10 µSOP DS1374-3

DS1374U-3/T&R -40°C to +85°C 3.0 10 µSOP/Tape and Reel DS1374-3

DS1374U-3+T&R -40°C to +85°C 3.0 10 µSOP/Tape and Reel DS1374-3

DS1374U-33 -40°C to +85°C 3.3 10 µSOP DS1374-33

DS1374U-33+ -40°C to +85°C 3.3 10 µSOP DS1374-33

DS1374U-33/T&R -40°C to +85°C 3.3 10 µSOP/Tape and Reel DS1374-33

DS1374U-33+T&R -40°C to +85°C 3.3 10 µSOP/Tape and Reel DS1374-33

+ Denotes a lead(Pb)-free/RoHS-compliant device.

* A "+" anywhere on the top mark denotes a lead-free device.

Chip Information

TRANSISTOR COUNT: 11,036

| PROCESS: CMOS PACKAGE TYPE | PACKAGE CODE | DOCUMENT NO. |
| --- | --- | --- |
| 16 SO (0.300”) | W16-H2 | 21-0042 |
| 10 µSOP (3.0mm) | U10+2 | 21-0061 |

SUBSTRATE CONNECTED TO GROUND

Thermal Information

Theta-JA: 221°C/W (µSOP) Theta-JC: 39°C/W (µSOP) Theta-JA: 73°C/W (16 SO) Theta-JC: 23°C/W (16 SO)

Package Information For the latest package outline information and land patterns, go to  www.maxim-ic.com/packages.

DS1374

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600  	17

©  2006 Maxim Integrated  Products 	Maxim is a registered  trademark  of Maxim Integrated  Products. is a registered  trademark  of Dallas Semiconductor Corporation.