DS1411

Serial Port iButton Holder

www.maxim-ic.com

FEATURES

Small size, true-ground interface to an RS232

COM Port for reading and writing iButton®

devices

CE and FCC compliant

Works with bipolar logic signals of ± 5V to ±

12V (RS232C) as well as unipolar signals of

5V to 12V

Supports reading and writing at standard and

Overdrive speeds

Provides strong pullup to 5V for Temperature

SCHEMATIC (for illustration)

iButton, Crypto iButton and EEPROM

iButton

Communicates at data rates of 9.6k (default),

19.2k, 57.6k, and 115.2k bits/s

RS232

COM Port

RTS DTR

RXD

5V Reg.

4

DS1411-009 only

6

Powered entirely from an RS232 interface

Standard DB female connector for mating to the COM Port of a computer with DS9098P iButton Retainer for a single iButton

TXD GND

8

DS	5

2480B

7 	2

1

ID Chip

Includes DS2502 identification chip

(DS1411-009 version only)

ORDERING INFORMATION

DS1411-009	DB-9 Connector, with ID chip

DS1411-S09	DB-9 Connector, no ID chip

DESCRIPTION

The DS1411 is a sophisticated RS232-to-1-Wire® adapter and iButton holder that performs RS232-level conversion and actively generates the 1-Wire communication signals. Together with an adequate TMEX

software driver it enables an IBM-compatible PC or laptop computer to directly read all and write any non-EPROM iButton devices. The DS1411 has a DS9098P retainer for a single F5 MicroCan® and can replace the DS9097U-009 or DS9097U-S09, respectively, in applications that use only a single iButton.

1-Wire communication is supported by the adapter for standard as well as Overdrive speed. The serial port must support a data transmission rate of 9600 bits/s. Under software control, the adapter can be set up for RS232 data rates of 19200, 57600 and 115200 bits per second. The two higher rates are recommended for communication at Overdrive speed with Overdrive capable devices. Since the DS1411 operates in a byte mode, every data byte received from the RS232 port generates eight time slots on the 1- Wire bus.

The DS1411 is based on the DS2480B Serial 1-Wire Line Driver chip and therefore is not a plug-in replacement for the DS1412 Serial Port Adapter. The RXD/ TXD Polarity Select input of the DS2480B is tied to VDD (non-inverting).

iButton, 1-Wire, and MicroCan are registered trademarks of

Dallas Semiconductor.

1 of 1	070302