Rev 1; 9/06

Addressable Digital Potentiometer

General Description

The DS1805 addressable digital potentiometer contains a single 256-position digitally controlled potentiometer. Device control is achieved through a 2-wire serial inter- face. Device addressing  is provided  through three address inputs that allow up to eight devices on a sin- gle 2-wire bus. The exact wiper position of the poten- tiometer can  be  written or read.  The DS1805 is available in 16-pin SO and 14-pin TSSOP packages. The device is available in three standard resistance val- ues: 10kΩ, 50kΩ, and 100kΩ. The DS1805 is specified over the industrial temperature  range. The DS1805 provides a low-cost alternative for designs  based  on the DS1803, but require only a single potentiometer.

Features

| ♦ 3V or 5V Operation ♦ ♦ | Low Power Consumption One Digitally Controlled, 256-Position |
| --- | --- |
|  | Potentiometer |
| ♦ | Compatible with DS1803-Based Designs |
| ♦ | 14-Pin TSSOP (173mil) and 16-Pin SO (150mil) Available for Surface-Mount Applications |
| ♦ | Three Address Inputs |
| ♦ | Serial 2-Wire Bus |
| ♦ | Operating Temperature Range |

Applications

CCFL Inverters

PDAs and Cell Phones Portable Electronics Multimedia Products

Instrumentation and Industrial Controls

Industrial: -40°C to +85°C

♦ Standard Resistance Values

DS1805-010: 10kΩ DS1805-050*: 50kΩ DS1805-100*: 100kΩ

Pin  Configurations 	 	Ordering Information

TOP VIEW

H1    1

L1    2

W1    3

A2    4

A1    5

A0    6

GND   7

DS1805E

14   VCC

13    N.C.

12    N.C.

11    N.C.

10    N.C.

9    SDA

8     SCL

H1    1

N.C.    2

L1    3

W1    4

A2    5

A1    6

A0    7

GND   8

DS1805Z

16   VCC

15   N.C.

14   N.C.

13   N.C.

12 N.C. PART TEMP RANGE PIN-PACKAGE RESISTANCE (KΩ)

DS1805E-010 -40°C to +85°C 14 TSSOP (173mil) 10

DS1805E-050* -40°C to +85°C 14 TSSOP (173mil) 50

DS1805E-100* -40°C to +85°C 14 TSSOP (173mil) 100

DS1805Z-010 -40°C to +85°C 16 SO (150mil) 10

DS1805Z-050* -40°C to +85°C 16 SO (150mil) 50

DS1805Z-100* -40°C to +85°C 16 SO (150mil) 100

11   N.C.

10   SDA

9    SCL

Add “/T&R” for tape-and-reel orders.

*Future product.

14 TSSOP (173mil)

DS1805

16 SO (150mil)

ABSOLUTE MAXIMUM RATINGS

Voltage on Any Pin Relative to Ground .................-0.5V  to +6.0V Operating  Temperature  Range ...........................-40°C to +85°C

Storage Temperature  Range .............................-55°C to +125°C Soldering Temperature ............................................See IPC/JEDEC J-STD-020A Specification

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

DS1805

RECOMMENDED DC OPERATING CONDITIONS

(TA = -40°C to +85°C)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Supply Voltage | VCC | (Note 1) | 2.7 5.5 | V |
| Resistor Inputs | L, H, W | (Note 1) | -0.3 VCC + 0.3 | V |

DC ELECTRICAL CHARACTERISTICS

(VCC = 2.7V to 5.5V, TA = -40°C to +85°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Supply Current Active | ICC | (Note 2) | 200 | µA |
| Input Leakage | IIL |  | -1 +1 | µA |
| Wiper Resistance | RW |  | 400 1000 | Ω |
| Wiper Current | IW |  | 1 | mA |
| Input Logic 1 | VIH |  | 0.7VCC VCC + 0.3 | V |
| Input Logic 0 | VIL |  | GND - 0.3VCC 0.3 | V |
| Input Logic Levels A0, A1, A2 (Note 3) |  | Input logic 1 | 0.7VCC VCC + 0.3 | V |
|  |  | Input logic 0 | GND - 0.25V C C 0.3 |  |
| Input Current each I/O Pin (Note 4) |  | 0.4V < VI/O < 0.9VCC | -10 +10 | µA |
| Standby Current | ISTBY | (Note 5) | 20 40 | µA |
| Low-Level Output Voltage | VOL1 | 3mA sink current | 0 0.4 | V |
|  | VOL2 | 6mA sink current | 0 0.6 | V |
| I/O Capacitance | CI/0 |  | 10 | pF |
| Pulse Width of Spikes that Must be Suppressed by the Input Filter | tSP | Fast mode | 0 50 | ns |

DS1805

ANALOG RESISTOR CHARACTERISTICS

(VCC = 2.7V to 5.5V, TA = -40°C to +85°C)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| End-to-End Resistor Tolerance |  | (Note 6) | -20 +20 | % |
| Absolute Linearity |  | (Note 7) | -0.75 +0.75 | LSB |
| Relative Linearity |  | (Note 8) | -0.3 +0.3 | LSB |
| -3dB Cutoff Frequency | fCUTOFF | (Note 9) |  | Hz |
| Ratiometric Temperature Coefficient |  |  | 8 | ppm/°C |
| End-to-End Temperature Coefficient |  |  | 550 | ppm/°C |
| Capacitance | CI |  | 5 | pF |

AC ELECTRICAL CHARACTERISTICS

(VCC = 2.7V to 5.5V, TA = -40°C to +85°C)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| SCL Clock Frequency (Note 10) | fSCL | Fast mode | 0 400 | kHz |
|  |  | Standard mode | 0 100 |  |
| Bus Free Time Between STOP and START Condition (Note 10) | tBUF | Fast mode | 1.3 | µs |
|  |  | Standard mode | 4.7 |  |
| Hold Time (Repeated) START Condition (Notes 10, 11) | tHD:STA | Fast mode | 0.6 | µs |
|  |  | Standard mode | 4.0 |  |
| Low Period of SCL Clock (Note 10) | tLOW | Fast mode | 1.3 | µs |
|  |  | Standard mode | 4.7 |  |
| High Period of SCL Clock (Note 10) | tHIGH | Fast mode | 0.6 | µs |
|  |  | Standard mode | 4.0 |  |
| Data Hold Time (Notes 10, 12, 13) | tHD:DAT | Fast mode | 0 0.9 | µs |
|  |  | Standard mode | 0 0.9 |  |
| Data Setup Time (Note 10) | tSU:DAT | Fast mode | 100 | ns |
|  |  | Standard mode | 250 |  |
| Rise Time of Both SDA and SCL Signals (Notes 10, 14) | tR | Fast mode | 20 + 0.1CB 300 | ns |
|  |  | Standard mode | 20 + 0.1CB 1000 |  |
| Fall Time of Both SDA and SCL Signals (Notes 10, 14) | tF | Fast mode | 20 + 0.1CB 300 | ns |
|  |  | Standard mode | 20 + 0.1CB 300 |  |
| Setup Time for STOP Condition (Note 10) | tSU:STO | Fast mode | 0.6 | µs |
|  |  | Standard mode | 4.0 |  |
| Capacitive Load for Each Bus Line (Note 14) | CB |  | 400 | pF |

DS1805

AC ELECTRICAL CHARACTERISTICS

(VCC = 2.7V to 5.5V, TA = -40°C to +85°C)

Note 1:    All voltages are referenced to ground.

Note 2:    ICC specified with SDA pin open. SCL = 400kHz clock rate.

Note 3:    Address  inputs A0, A1, and A2 should be connected to either VCC or GND, depending on the desired address  selections.

Note 4:    I/O pins of fast mode devices  must not obstruct  the SDA and SCL lines if VCC is switched  off.

Note 5:    ISTBY specified with SDA = SCL = VCC = 5.0V.

Note 6:    Valid at +25°C only.

Note 7:    Absolute linearity is used to determine  wiper voltage versus expected voltage as determined by wiper position.

Note 8:    Relative linearity is used to determine  the change in voltage between successive tap positions.

Note 9:    -3dB cutoff frequency  characteristics for the DS1805 depend on potentiometer  total resistance:  DS1805-010, 1MHz; DS1805-50, 200kHz; DS1805-100, 100kHz.

Note 10:  A fast mode device can be used in a standard  mode system, but the requirement  tSU:DAT > 250ns must then be met. This will automatically be the case if the device does not stretch the low period of the SCL signal. If such a device does stretch

the LOW period of the SCL signal, it must output the next data bit to the SDA line tRMAX + tSU:DAT = 1000ns + 250ns =

1250ns before the SCL line is released.

Note 11:  After this period, the first clock pulse is generated.

Note 12:  The maximum tHD:DAT has only to be met if the device does not stretch the low period (tLOW) of the SCL signal.

Note 13:  A device must internally provide a hold time of at least 300ns for the SDA signal (referred to the VIHMIN of the SCL signal)

in order to bridge  the undefined  region of the falling edge of SCL.

Note 14:  CB—total capacitance of one bus line in picofarads, timing referenced to (0.9)(VCC) and (0.1)(VCC).

Typical Operating Characteristics

(VCC = 5.0V, TA = +25°C, unless otherwise noted.)

SUPPLY CURRENT

vs. TEMPERATURE

30

25

15

10

SUPPLY CURRENT (µA)

5

W-L RESISTANCE

vs. WIPER SETTING

10

DS1805 toc01

8

W-L RESISTANCE (kΩ)

6

4

2

350

DS1805 toc02

300

250

200

150

100

WIPER RESISTANCE (Ω)

50

WIPER RESISTANCE vs. WIPER VOLTAGE (10kΩ)

VCC = 5V

DS1805 toc03

VCC = 3V

0

| -40 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | VCC = 5V |  |  |  |
|  |  |  |  |  |  |  |  |  | VCC = 3V |  |  |  |

-20	0

20	40

60	80

0

| 0 25 50 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10kΩ POTENTIOMETER |  |  |  |  |  |

75 100 125 150 175 200 225 250

0

0	1	2	3	4	5

TEMPERATURE (°C)

WIPER SETTING

WIPER VOLTAGE (V)

VOLTAGE DIVIDER PERCENT CHANGE (FROM +25°C) vs. TEMPERATURE

END-TO-END RESISTANCE TEMPERATURE

ACTIVE SUPPLY CURRENT

DS1805 toc06

0.06

0.04

0.02

0

-0.02

% CHANGE (FROM +25°C)

WIPER = BFh

WIPER = 3Fh

DS1805 toc04

WIPER = 7Fh

WIPER = BFh

TC = 8.1ppm/°C

| 5 50 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SDA = VCC A0, A1, A2, L1 = GND W1, H1 = NO CONNECT |  |  |  |

DS1805 toc05

4	45

END-TO-END RESISTANCE % CHANGE

3	40

2	35

10kΩ, WORST CASE

1	30

0	25

ACTIVE SUPPLY CURRENT (µA)

-1	20

-2	15

-3	TC = 530ppm/°C	10

-0.04

-0.06

TC = 1.3ppm/°C

-4	5

-5	0

-40

-20	0

20	40	60	80

-40

-20	0	20

40	60	80

0	100

200

300

400

TEMPERATURE (°C)

TEMPERATURE (°C)

SCL FREQUENCY (kHz)

0.18

0.16

0.14

ABSOLUTE LINEARITY (LSB)

0.12

0.10

0.08

0.06

0.04

0.02

0

VOLTAGE-DIVIDER ABSOLUTE LINEARITY

vs. WIPER SETTING (10kΩ)

DS1805 toc07

0.06

0.05

0.04

0.03

0.02

0.01

RELATIVE LINEARITY (LSB)

0

-0.01

VOLTAGE-DIVIDER RELATIVE LINEARITY

vs. WIPER SETTING (10kΩ)

DS1805 toc08

0	50	100

150

200

250

0	50

100

150

200

250

WIPER SETTING

DS1805

WIPER SETTING

L1 	POTENTIOMETER-1	H1

256-TO-1  MULTIPLEXER

SRAM

REG-0 (8-BIT  REGISTER)

WIPER-1 (8-BIT REGISTER)	W1

| SCL SDA |  | COMMAND/ CONTROL UNIT |
| --- | --- | --- |

| 2-WIRE SERIAL INTERFACE DEVICE ADDRESS SELECTION |  | A0 |
| --- | --- | --- |
|  |  | A1 |
|  |  | A2 |

Figure 1. Functional Diagram

DS1805

| Pin Description PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| TSSOP | SO |  |  |
| 1 | 1 | H1 | High End of Potentiometer |
| 2 | 3 | L1 | Low End of Potentiometer |
| 3 | 4 | W1 | Wiper Terminal of Potentiometer |
| 6, 5, 4 | 7, 6, 5 | A0, A1, A2 | Address Select Inputs |
| 7 | 8 | GND | Ground |
| 8 | 9 | SCL | Serial Clock Input |
| 9 | 10 | SDA | Serial Data I/O |
| 10–13 | 2, 11–15 | N.C. | No Connection |
| 14 | 16 | VCC | 3V/5V Power-Supply Input |

Detailed Description

The DS1805 addressable digital potentiometer contains a single 256-position digitally controlled potentiometer. Device control is achieved through a 2-wire serial inter- face. Device addressing  is provided  through three address inputs that allow up to eight devices on a single

2-wire bus. The exact wiper position of the potentiometer can be written or read. The DS1805 is available in 16-pin SO and 14-pin TSSOP packages. The device is available in three standard resistance values: 10kΩ, 50kΩ, and

100kΩ. The DS1805 specified over the industrial temper- ature range. The DS1805 is provides a low-cost alterna- tive for designs based on the DS1803, but require only a single potentiometer.

Device Operation

The DS1805 is an addressable,  digitally  controlled device that has a single 256-position potentiometer. Figure 1 shows a block diagram of the part. Communication and control of the device is accom- plished through a 2-wire serial interface that has SDA and SDL signals. Device addressing  is attained using the device chip-select inputs A0, A1, and A2.

SDA

MSB

SLAVE ADDRESS

R/W DIRECTION BIT

ACKNOWLEDGEMENT SIGNAL FROM RECEIVER

ACKNOWLEDGEMENT SIGNAL  FROM RECEIVER

SCL

1	2	6	7	8	9	1	2

3–7 	8	9

START CONDITION

ACK

REPEATED IF MORE BYTES ARE TRANSFERED

ACK

STOP CONDITION OR REPEATED START CONDITION

Figure 2. 2-Wire Data Transfer Overview

The potentiometer is composed of a 256-position resis- tor array. Two 8-bit registers are provided  to ensure compatibility with DS1803-based designs. Register-0 is a general-purpose  SRAM byte, while register-1  is assigned to the potentiometer  and is used to set the wiper position on the resistor array. The wiper terminal is multiplexed to one of 256 positions on the resistor array based on its corresponding 8-bit register value. The high- est wiper setting, FFh, is 1 LSB away from H1 (resistor

DS1805

255), while the lowest setting, 00h, connects to L1.

The DS1805 is a volatile device that does not maintain the position of the wiper during power-down or loss of power. On power-up,  the wiper position is set to 00h (the low-end terminal). The user can then set the wiper value to a desired position.

Communication with the DS1805 takes place over the

2-wire serial interface consisting  of the bidirectional data terminal, SDA, and the serial clock  input, SCL. Complete details of the 2-wire interface are discussed in the 2-Wire Serial Data Bus section.

The 2-wire interface and address inputs A0, A1, and A2 allow operation of up to eight devices in a bus topology, with A0, A1, and A2 being the address of the device.

Application Considerations

The DS1805 is offered in three standard resistor values:

10kΩ, 50kΩ, and 100kΩ. The resolution of the poten- tiometer is defined as RTOT/256, where RTOT is the total resistor value of the potentiometer. The DS1805 is designed to operate using 3V or 5V power supplies over

the industrial (-40°C to +85°C) temperature range. Maximum input signal levels across the potentiometer cannot exceed the operating power supply of the device.

2-Wire Serial Data Bus

The DS1805 supports a bidirectional  2-wire bus and data transmission protocol. A device that sends data on the bus is called a transmitter, and a device receiving data is called a receiver. The device that controls the message is called a master. The devices that are con- trolled by the master are slaves. The bus must be con- trolled by a master device  that generates the serial clock (SCL), controls the bus access, and generates the START and STOP conditions. The DS1805 operates as a slave on the 2-wire bus. Connections to the bus are made through the open-drain I/O lines, SDA and SCL.

The following bus protocol has been defined (Figure 2):

•	Data transfer can be initiated only when the bus is not busy.

•	During data transfer, the data line must remain sta- ble whenever the clock line is high. Changes in the data line while the clock line is high will be interpret- ed as control signals.

Accordingly,  the following bus conditions  have been defined:

Bus not busy: Both data and clock lines remain high.

Start data transfer: A change in the state of the data line from high to low while the clock is high defines a START condition.

Stop data transfer: A change in the state of the data line from low to high while the clock line is high defines the STOP condition.

Data valid: The state of the data line represents valid data when, after a START condition, the data line is sta- ble for the duration of the high period of the clock signal. The data on the line must be changed  during the low period of the clock signal. There is one clock pulse per bit of data. Figure 2 details how data transfer is accom- plished on the 2-wire bus. Depending  upon the state of the R/W bit, two types of data transfer are possible.

Each data transfer is initiated with a START condition

| MSB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 1 0 1 A2 A1 A0 R/W |  |  |  |  |  |  |  |

DEVICE IDENTIFIER

DEVICE ADDRESS

LSB

and terminated with a STOP condition. The number of data bytes transferred between START and STOP con- ditions is not limited and is determined  by the master device.  The information is transferred byte-wise and each receiver acknowledges with a ninth bit.

DS1805

Within the bus specifications  a regular mode (100kHz clock  rate) and a fast mode (400kHz clock  rate) are defined. The DS1805 works in both modes.

Acknowledge: Each receiving device, when addressed, is obliged  to generate an acknowledge after the recep- tion of each byte. The master device must generate an extra clock pulse that is associated with this acknowl- edge bit.

A device that acknowledges  must pull down the SDA line during the acknowledge clock pulse in such a way that the SDA line is stable low during the high period of the acknowledge-related  clock pulse. Of course, setup and hold times must be taken into account.  A master must signal an end of data to the slave by not generat- ing an acknowledge  bit on the last byte that has been clocked  out of the slave. In this case, the slave must leave the data line high to enable the master to gener- ate the STOP condition.

Data transfer from a master transmitter to a slave receiver: The first byte transmitted by the master is the control byte (slave address). Next follows a number of data bytes. The slave returns an acknowledge bit after each received byte.

Data transfer from a slave transmitter to a master receiver: The first byte (the slave address) is transmit- ted by the master. The slave then returns an acknowl- edge bit. Next follows a number of data bytes transmitted  by the slave to the master. The master returns an acknowledge  bit after all received  bytes other than the last byte. At the end of the last received byte, a ‘not acknowledge’ is returned.

The master device  generates all of the serial clock pulses and the START and STOP conditions. A transfer

Figure 3. Control Byte

is ended  with a STOP condition  or with a repeated START condition. Since a repeated START condition is also the beginning  of the next serial transfer, the bus will not be released.

The DS1805 can operate in the following two modes:

Slave receiver mode: Serial data and clock  are received  through SDA and SCL. After each byte is received, an acknowledge bit is transmitted. START and STOP conditions are recognized  as the beginning  and end of a serial transfer. Address recognition is performed by hardware after reception of the slave address  and direction bit.

Slave transmitter mode: The first byte is received and handled as in the slave receiver mode. However, in this mode the direction  bit will indicate  that the transfer direction is reversed. Serial data is transmitted on SDA by the DS1805 while the serial clock  is input on SCL. START and STOP conditions  are recognized  as the beginning and end of a serial transfer.

Slave Address

A control byte is the first byte received  following the START condition from the master device.  The control byte consists of a four-bit control code; for the DS1805, this is set as 0101 binary for read/write operations. The next three bits of the control byte are the device select bits (A2, A1, A0). They are used by the master device to select which of eight devices  are to be accessed. The select bits are the three least significant bits (LSB) of the slave address. Additionally, A2, A1, and A0 can be changed any time during a powered condition of the part. The last bit of the control byte (R/W) defines the operation to be performed. When set to a one, a read operation is selected; when set to a zero a write opera- tion is selected. Figure 3 shows the control byte struc- ture for the DS1805.

Following the START condition,  the DS1805 monitors the SDA bus checking  the device type identifier being transmitted. Upon receiving  the 0101 address  code and appropriate  device select bits, the slave device outputs an acknowledge signal on the SDA line.

Command and Protocol

The DS1805’s command  and protocol  structure of the DS1805 allows the user to read or write to both the scratchpad  and potentiometer registers. Figures 4 and

5 show the command structures for the part. Potentiometer data values and control and command values are always transmitted  most significant  bit (MSB) first. During communications,  the receiving unit always generates the acknowledge.

Reading the DS1805

As shown in Figure 4, the DS1805 provides one read- command operation. This operation allows the user to read both potentiometers. Specifically, the R/W bit of the control byte is set equal to a one for a read operation. Communication to read the DS1805 begins with a START condition that is issued by the master device. The control byte from the master device follows the START condition. Once the control byte has been received by the DS1805, the part responds with an acknowledge.  The read/write bit of the control byte as stated should be set equal to one for reading the DS1805.

When the master has received the acknowledge from the DS1805, the master can then begin to receive poten- tiometer wiper data. The value of the register-0 wiper position will be the first returned from the DS1805. Once the eight bits of the register-0 wiper position have been transmitted, the master needs to issue an acknowledge, unless it is the only byte to be read, in which case the master issues a not acknowledge. If desired, the master can stop the communication transfer at this point by issu- ing the STOP condition.  However, if the value of the potentiometer-1 wiper position value is needed, commu-

Table 1. 2-Wire Command Words

| COMMAND | COMMAND VALUE |
| --- | --- |
| Write Register-0 | 101010 01 |
| Write Potentiometer-1 Register | 101010 10 |
| Write Both Registers | 101011 11 |

nication transfer can continue by clocking the remaining eight bits of the potentiometer-1 value, followed by a not acknowledge. Final communication transfer is terminated by issuing the STOP command. Figure 4 shows the flow of the read operation.

Writing to the DS1805

Figure 5 shows a data flow diagram  for writing the DS1805. The DS1805 has three write-command opera- tions. These include write reg-0, write pot-1, and write reg-0/pot-1. The write reg-0 command allows the user to write the value of scratchpad  register-0 and as an option the value of potentiometer-1. The write-1 command allows the user to write the value of potentiometer-1 only. The last write command, write-0/1, allows the user to write both registers to the same value with one command and one data value being issued.

All the write operations begin with a START condition. Following the START condition, the master device issues the control byte. The read/write bit of the control byte is set to zero for writing the DS1805. Once the control byte has been issued and the master receives the acknowl- edgment from the DS1805, the command byte is trans- mitted to the DS1805. As mentioned above, there exist three write operations that can be used with the DS1805. Figure 5 and Table 1 show the binary value of each write command.

Package Information

For the latest package outline information, go to

www.maxim-ic.com/packages.

OPTIONAL

| MSB |  |  |  |  |  |  |  | ACK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 1 0 1 A2 A1 A0 1 |  |  |  |  |  |  |  |  |

START

CONTROL BYTE

LSB	MSB

DATA BYTE

LSB	MSB

DATA BYTE

LSB

NACK

STOP

DS1805

REG-0	POT-1

ACK

R/W = 1

Figure 4. 2-Wire Read Protocols

REGISTER-0

MSB

START

CONTROL BYTE

| LSB MSB |  |  |  |  |  |  |  | ACK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 0 1 0 1 0 0 1 |  |  |  |  |  |  |  |  |

COMMAND BYTE

LSB	MSB

DATA BYTE

LSB

ACK

OPTIONAL

MSB

DATA BYTE

LSB

ACK

STOP

REG-0

POT-1

| R/W = 0 |  |  |  |  |  |  |  | ACK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 1 0 1 A2 A1 A0 0 |  |  |  |  |  |  |  |  |

WRITE POT-1

MSB

START

CONTROL BYTE

| LSB MSB |  |  |  |  |  |  |  | ACK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 0 1 0 1 0 1 0 |  |  |  |  |  |  |  |  |

COMMAND BYTE

| LSB MSB |  |  |  |  |  |  |  | ACK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POT-1 |  |  |  |  |  |  |  |  |

DATA BYTE

LSB

STOP

|  |  |  |  |  |  |  |  | ACK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 1 0 1 A2 A1 A0 0 |  |  |  |  |  |  |  |  |

R/W = 0

WRITE REGISTER-0 AND POT-1 (SAME VALUE)

MSB

START

CONTROL BYTE

| LSB MSB |  |  |  |  |  |  |  | ACK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 0 1 0 1 1 1 1 |  |  |  |  |  |  |  |  |

COMMAND BYTE

| LSB MSB |  |  |  |  |  |  |  | ACK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REG-0/POT-1 VALUE |  |  |  |  |  |  |  |  |

DATA BYTE

LSB

STOP

|  |  |  |  |  |  |  |  | ACK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 1 0 1 A2 A1 A0 0 |  |  |  |  |  |  |  |  |

R/W = 0

Figure 5. 2-Wire Write Protocols

SDA

tBUF

tLOW

tR 	tF

tHD:STA

tSP

SCL

STOP	START

tHD:STA

tHD:DAT

tHIGH

tSU:DAT 	REPEATED START

tSU:STA

tSU:STO

Figure 6. Timing Diagram

Revision History

Pages changed at Rev1: 9, 10

Package Information

For the latest package outline information, go to

www.maxim-ic.com/DallasPackInfo.

DS1805

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

10  	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  2002 Maxim Integrated  Products 	Printed USA 	is a registered  trademark  of Maxim Integrated  Products.