DS9092 iButton Probe

www.dalsemi.com

FEATURES

Simple, low-cost metal stampings form a read/write probe for the iButton    family

Probe guides the entry of the iButton

iButton slides over the surface to self-clean contacts

Accessible shallow probe cavity simplifies removal of debris such as mud

Flexible design supports panel mount or hand-grip mount with optional tactile feedback

Bright tarnish-resistant metal surface provides millions of operations

Panel-mount probe, pre-wired for easy installation

Hand-grip probe mates to RJ-11 jack for quick installation

DS9092 probes both F3 and F5 cans.  Extra consideration is required for operation with the F3 can.  The attachment method of the F3 can must not interfere with the cavity dimensions of the probe, otherwise the data signal will not make contact.

ORDERING INFORMATION

DS9092	Panel-mount probe, solid face

DS9092T	Panel-mount probe with tactile feedback

DS9092GT	Hand-grip mount with tactile feedback

PACKAGE DESCRIPTION

DESCRIPTION

The DS9092 iButton Probe provides the electrical contact necessary for the transfer of data to and from the DS19xx family of iButtons. The round probe shape provides a self-aligning interface that readily matches the circular rim of the iButton’s MicroCan package. Metal contacts resist wear and are easy to keep clean.

The DS9092 is available with a flat faceplate (standard) or with optional tactile feedback. The center contact of the standard reader has no moving parts, making this a more rugged interface for harsh

1 of 2	062706

iButton is a registered trademark of Dallas Semiconductor.

DS9092 environments. This type of probe is best suited for designs where the iButton is brought into contact with the reader.

The tactile feedback probe is ideal for situations where the iButton is stationary and the movable reader is brought in contact with it.

Both types of probes are available in a panel-mount version. The tactile feedback probe is also available in a grip-mount version. The panel-mount probes are fastened behind the panel with a push-on type spring nut.

The two 15 cm 22AWG wires are provided for easy connection to the system microcontroller. The hand- grip mount probe comes attached to a 10 cm handle and 1-meter cable which is terminated with an RJ11 jack.

STANDARD PROBE Figure 1

OPTIONAL TACTILE FEEDBACK Figure 2

OPTIONAL HAND-HELD WAND Figure 3

2 of 2