DS9100

Touch and Hold Probe Stampings

FEATURES

ﾧ	Compact two-piece, all-metal receptacle for

F5 MicroCanTM

ﾧ	Accepts two-thirds of the MicroCan (one- third of MicroCan will extend out)

ﾧ	Allows reading iButtons with either momentary or dwelled contact

ﾧ	Outer ring will hold MicroCan for dwelled contact

ﾧ	Two options for data contact: cantilever

(DS9100B) or coiled spring (DS9100C)

ﾧ	>10000 insertion/withdrawal cycles with no performance degradation

ﾧ	Redundancy of contacts and high contact force ensure reliability

ﾧ	Probe withstands high temperatures required

for PCB solder reflow operations

ﾧ	Material is stainless steel with selective tin-

lead plating for optimal solderability to printed circuit board

ﾧ	Cleaning fluids drain freely for quick clean up

ORDERING INFORMATION DS9100A	Outer Contact (Ground) DS9100B	Center Contact Cantilever (Data)

DS9100C	Center Contact Coiled Spring (Data)

DS9100A WITH DS9100B

Dimensions are in millimeters

DS9100C WITH CORRECT ORIENTATION TO SOLDER PAD

DESCRIPTION

The DS9100 Touch and Hold Probe Stampings' function is similar to the DS9098 iButton Retainer. An F5 MicroCan will fit completely into the DS9098, but the flange and about one-third of the can will extend out  if pressed into  the  DS9100.  As a  probe,  the  DS9100A together  with the  coiled spring DS9100C  allows  reading  iButtons  on  contact.  With  additional  pressure,  the  stiff  springs  of  the DS9100A’s outer ring will deflect and grip the MicroCan sufficiently to provide a continuous contact to both the can rim (ground) and the can lid (data). If reading on contact is not required, the cantilever type center contact DS9100B can be used rather than the DS9100C.

PRINTED CIRCUIT BOARD LAYOUTS

Compatible PCB layout.

Good for either center part DS9100B or DS9100C

Additional diminsions for ground contact. CENTER-DATA	OUTER RING-GROUND

COPPER CLADDING  0.3 kg/m2 or 	34   m

(  1 oz/sq. ft)

DS9100

Center piece optimized for cantilever DS9100B

Center piece optimized for coiled spring DS9100C Dimensions are in millimeters.

The DS9100 stampings are mounted on Side A of the printed circuit board.