19-0367; Rev 1; 5/06

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

General Description

The MAX1482 and MAX1483 are low-power  trans- ceivers for RS-485 and RS-422 communication.  Both feature slew-rate-limited drivers that minimize EMI and reduce  reflections caused  by improperly  terminated cables. Data rates are guaranteed up to 250kbps.

The MAX1482/MAX1483 draw only 20µA of supply cur- rent. Additionally,  they have a low-current shutdown mode that consumes only 0.1µA. Both parts operate from a single +5V supply.

Drivers are short-circuit current limited and are protect- ed against excessive power dissipation  by thermal shutdown circuitry that places the driver outputs into a high-impedance  state. The receiver input has a fail-safe feature that guarantees a logic-high  output if the input is open circuit.

The MAX1482 is full duplex and the MAX1483 is half duplex. Both parts have a 1⁄8-unit-load input impedance that guarantees up to 256 transceivers on the bus.

Applications Low-Power RS-485/RS-422 Networks Transceivers for EMI-Sensitive Applications Industrial-Control Local Area Networks

Large 256-Node LANs

MAX1482/MAX1483

Features

♦ Low 20µA Operating Current

♦ Slew-Rate Limited for Reduced EMI and

Reduced Reflections

♦ 0.1µA Low-Current Shutdown Mode

♦ Designed for RS-485 and RS-422 Applications

♦ Operate from a Single +5V Supply

♦ -7V to +12V Common-Mode Input Voltage Range

♦ Allows up to 256 Transceivers on the Bus— Guaranteed (1⁄8-unit load)

♦ Current Limiting and Thermal Shutdown for

Driver Overload Protection

Ordering Information PART TEMP RANGE PIN-PACKAGE PKG CODE

MAX1482CPD 0°C to +70°C 14 PDIP P14-3

MAX1482CSD 0°C to +70°C 14 SO S14-4

MAX1482EPD -40°C to +85°C 14 PDIP P14-3

MAX1482ESD -40°C to +85°C 14 SO S14-4

MAX1483CPA 0°C to +70°C 8 PDIP P8-1

MAX1483CSA 0°C to +70°C 8 SO S8-5

MAX1483CUA 0°C to +70°C 8 µMAX® U8-1

MAX1483EPA -40°C to +85°C 8 PDIP P8-1

MAX1483ESA -40°C to +85°C 8 SO S8-5

Typical Operating Circuits

Pin  Configurations

RO     1         R RE     2

DE     3

DI    4        D

+5V

8         VCC

7  B

Rt

6

A

5         GND

MAX1483

B Rt

A

DE

DI D

R                     RO

TOP VIEW

RO     1

RE     2

DE     3

DI    4

B   1

VCC   2

RO     3

R

D

DIP/SO

MAX1483

8   VCC

7    B

6    A

5    GND

8    A

7    GND

6    DI

DIAGRAMS  REFER TO PINS A AND B WHEN DE IS HIGH. TYPICAL OPERATING CIRCUIT SHOWN WITH DIP/SO PACKAGE.

RE     4

µMAX

5    DE

MAX1482 appears at end of data sheet.

MAX1482 appears at end of data sheet.

µMAX is a registered trademark of Maxim Integrated products, Inc.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

MAX1482/MAX1483

ABSOLUTE MAXIMUM RATINGS

Supply Voltage (VCC) ...............................................................7V Control Input Voltages (RE, DE) .................-0.5V  to (VCC + 0.5V) Driver Input Voltage (DI).............................-0.5V to (VCC + 0.5V) Driver Output Voltages ..........................................-7.5V to 12.5V Receiver Input Voltages (A, B) ..............................-7.5V to 12.5V Receiver Output Voltage (RO)....................-0.5V to (VCC + 0.5V) Continuous Power Dissipation  (TA = +70°C)

8-Pin Plastic DIP (derate 9.09mW/°C above +70°C) .....727mW

14-Pin Plastic DIP (derate 10.00mW/°C above +70°C) .800mW

8-Pin SO (derate 5.88mW/°C above +70°C)..................471mW

14-Pin SO (derate 8.33mW/°C above +70°C)................667mW

8-Pin µMAX (derate 4.10mW/°C above +70°C) .............330mW Operating  Temperature  Ranges

MAX148_C_ _ .......................................................0°C to +70°C MAX148_E_ _.....................................................-40°C to +85°C Storage Temperature  Range .............................-65°C to +160°C Lead Temperature  (soldering,  10sec) .............................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

DC ELECTRICAL CHARACTERISTICS

(VCC = 5V ±5%, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN | TYP | MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Differential Driver Output (no load) | VOD1 |  |  |  | 5 | V |

Differential Driver Output

(with load)

Change in Magnitude  of Driver Differential Output Voltage for Complementary Output States

VOD2

∆VOD

R = 50Ω (RS-422), Figure 1

R = 27Ω (RS-485), Figure 1

R = 27Ω or 50Ω, Figure 1

2                              5

V

1.5                            5

0.2            V

| Driver Common-Mode Output Voltage Change in Magnitude of Driver Common-Mode Output Voltage for Complementary Output States Three-State (high impedance) Output Current at Driver | VOC | R = 27Ω or 50Ω, Figure 1 |  | 3 | V |
| --- | --- | --- | --- | --- | --- |
|  | ∆VOD | R = 27Ω or 50Ω, Figure 1 |  | 0.2 | V |
|  | IOZD | MAX1482 only, -7V < VY and VZ < 12V |  | ±50 | µA |
| Logic Input High Voltage | VIH | DE, DI, –—– RE | 2.0 |  | V |
| Logic Input Low Voltage | VIL | DE, DI, –—– RE |  | 0.8 | V |
| Logic Input Current | IIN1 | DE, DI, –—– RE |  | ±2 | µA |

Input Current

(A, B)

IIN2

MAX1482,

DE = 0V, VCC = 0V or 5.25V

MAX1483,

DE = 0V, VCC = 0V or 5.25V

VIN = 12V VIN = -7V VIN = 12V

VIN = -7V

150

-100

µA

200

-150

Receiver Differential Threshold

Voltage

Receiver Input Hysteresis Receiver Output High Voltage Receiver Output Low Voltage

Three-State (high impedance) Output Current at Receiver

Receiver Input Resistance

VTH

∆VTH VOH VOL

IOZR

RIN

-7V ≤ VCM ≤ 12V VCM = 0V

IO = -4mA, VID = 200mV IO = 4mA, VID = -200mV

0.4V ≤ VO ≤ 2.4V

-7V ≤ VCM ≤ 12V

-0.2                        0.2           V

75                         mV

3.5                                         V

0.4           V

±1           µA

96                                            kΩ

Note 1:  All currents into device pins are positive; all currents out of device pins are negative. All voltages are referenced to device ground unless otherwise specified.

2

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

MAX1482/MAX1483

RS-485 Transceivers

DC ELECTRICAL CHARACTERISTICS (continued)

(VCC = 5V ±5%, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| No-Load Supply Current | ICC | MAX1482, RE = 0V or VCC | DE = VCC | 25 45 | µA |
|  |  |  | DE = 0V | 20 35 |  |
|  |  | MAX1483, RE = 0V or VCC | DE = VCC | 55 85 |  |
|  |  |  | DE = 0V | 20 35 |  |
| Supply Current in Shutdown | ISHDN | DE = 0V, RE = VCC |  | 0.1 10 | µA |
| Driver Short-Circuit Current | IOSD | DI = high or low, -7V ≤ VO ≤ 12V (Note 2) |  | 35 250 | mA |
| Receiver Short-Circuit Current | IOSR | 0V ≤ VO ≤ VCC |  | ±7 ±95 | mA |

SWITCHING CHARACTERISTICS

(VCC = 5V ±5%, TA = TMIN to TMAX, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Driver Input to Output | tPLH | Figures 3 and 5, RDIFF = 54Ω, CL1 = CL2 = 100pF | 2 | µs |
|  | tPHL |  | 2 |  |
| Driver Output Skew to Output | tSKEW | Figures 3 and 5, RDIFF = 54Ω, CL1 = CL2 = 100pF | 800 | ns |
| Driver Rise or Fall Time | tR, tF | Figures 3 and 5, RDIFF = 54Ω, CL1 = CL2 = 100pF | 0.25 2 | µs |
| Driver Enable to Output High | tZH | Figures 4 and 6, CL = 100pF, S2 closed | 0.2 2 | µs |
| Driver Enable to Output Low | tZL | Figures 4 and 6, CL = 100pF, S1 closed | 0.1 2 | µs |
| Driver Disable Time from Low | tLZ | Figures 4 and 6, CL = 15pF, S1 closed | 0.3 3.0 | µs |
| Driver Disable Time from High | tHZ | Figures 4 and 6, CL = 15pF, S2 closed | 0.3 3.0 | µs |
| Receiver Input to Output | tPLH, tPHL | Figures 3 and 7, RDIFF = 54Ω, CL1 = CL2 = 100pF | 0.25 2.25 | µs |
| ¦ tPLH - tPHL ¦ Differential Receiver Skew | tSKD | Figures 3 and 7, RDIFF = 54Ω, CL1 = CL2 = 100pF | 160 | ns |
| Receiver Enable to Output Low | tZL | Figures 2 and 8, CRL = 15pF, S1 closed | 90 | ns |
| Receiver Enable to Output High | tZH | Figures 2 and 8, CRL = 15pF, S2 closed | 90 | ns |
| Receiver Disable Time from Low | tLZ | Figures 2 and 8, CRL = 15pF, S1 closed | 90 | ns |
| Receiver Disable Time from High | tHZ | Figures 2 and 8, CRL = 15pF, S2 closed | 90 | ns |
| Maximum Data Rate | fMAX |  | 250 | kbps |
| Time to Shutdown | tSHDN | (Note 3) | 50 200 600 | ns |
| Driver Enable from Shutdown to Output High | tZH(SHDN) | Figures 4 and 6, CL = 100pF, S2 closed | 2 | µs |
| Driver Enable from Shutdown to Output Low | tZL(SHDN) | Figures 4 and 6, CL = 100pF, S1 closed | 2 | µs |
| Receiver Enable from Shutdown to Output High | tZH(SHDN) | Figures 2 and 8, CL = 15pF, S2 closed, A - B = 2V | 3 | µs |
| Receiver Enable from Shutdown to Output Low | tZL(SHDN) | Figures 2 and 8, CL = 15pF, S1 closed, B - A = 2V | 3 | µs |

Note 2:  Applies to peak current. See Typical Operating  Characteristics.

Note 3:  The MAX1482/MAX1483 are put into shutdown by bringing –—– high and DE low. If the inputs are in this state for less

RE

than 50ns, the parts are guaranteed not to enter shutdown. If the inputs are in this state for at least 600ns, the parts are guaranteed to have entered shutdown. See Low-Power Shutdown Mode section.

3

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

MAX1482/MAX1483

RECEIVER PROPAGATION DELAY (ns)

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

1400

1300

1200

1100

1000

900

800

MAX1482-01

DRIVER PROPAGATION DELAY (ns)

RECEIVER PROPAGATION DELAY

vs. TEMPERATURE

| 1200 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1100

1000

900

800

700

600

500

400

MAX1482-02

DRIVER PROPAGATION DELAY

vs. TEMPERATURE

MAX1482-03

DRIVER OUTPUT CURRENT vs. DIFFERENTIAL OUTPUT VOLTAGE

90

80

70

| 60 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

50

40

30

20

10

0

| -40 -20 0 20 40 60 80 100 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

-40    -20     0

20     40

60     80    100

0.5  1.0 1.5  2.0  2.5  3.0  3.5  4.0  4.5 5.0

TEMPERATURE (°C)

TEMPERATURE (°C)

OUTPUT VOLTAGE (V)

3.5

3.0

2.5

DIFFERENTIAL OUTPUT VOLTAGE (V)

2.0

MAX1482-04

| DRIVER DIFFERENTIAL OUTPUT VOLTAGE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R = 54Ω |  |  |

vs. TEMPERATURE

140

120

100

80

60

40

20

MAX1482-05

OUTPUT CURRENT (mA)

| OUTPUT CURRENT vs. DRIVER OUTPUT LOW VOLTAGE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

120

100

80

60

OUTPUT CURRENT (mA)

40

20

MAX1482-06

| OUTPUT CURRENT vs. DRIVER OUTPUT HIGH VOLTAGE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1.5

OUTPUT CURRENT (mA)

-40    -20     0      20     40     60     80    100

0

0         2         4        6

8       10      12

0

-7 -6  -5  -4  -3  -2  -1  0

1   2   3   4   5

TEMPERATURE (°C)

OUTPUT LOW VOLTAGE (V)

OUTPUT LOW VOLTAGE (V)

MAX1482

SUPPLY CURRENT

vs. TEMPERATURE

MAX1482-07

80

MAX1483

SUPPLY CURRENT

MAX1482-08

| vs. TEMPERATURE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DE = VCC |  |  |  |
|  |  |  | DE = GND |  |  |  |

80

70                                                                                                              70

60                                                                                                              60

50                                                                                                              50

SUPPLY CURRENT (µA)

40                                                                                                              40

30                       DE = VCC                                                                                                             30

SUPPLY CURRENT (µA)

20                                                                                                              20

DE = 0V

10                                                                                                              10

0

-40    -20     0

20     40

60     80    100

0

-40    -20     0

20     40

60     80    100

TEMPERATURE (°C)

TEMPERATURE (°C)

4

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

MAX1482/MAX1483

Pin  Description

| PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX1482 | MAX1483 |  |  |  |
| DIP/SO | DIP/SO | µMAX |  |  |
| 2 | 1 | 3 | RO | Receiver Output. With the receiver output enabled (RE low), RO is high if A > B by 200mV or when A and B are not connected, and RO is low if A < B by 200mV. |
| 3 | 2 | 4 | RE | Receiver Output Enable. When RE is low, RO is enabled. When RE is high, RO is high impedance. If RE is high and DE is low, the MAX1482/MAX1483 enter a low-power (0.1µA) shutdown state. |
| 4 | 3 | 5 | DE | Driver Output Enable. The driver outputs, A and B, (Y and Z for the MAX1482) are enabled by bringing DE high. When DE is low, the driver outputs are high impedance, and the devices can function as line receivers if RE is low. If RE is high and DE is low, the parts will enter a low-power (0.1µA) shutdown state. If the driver outputs are enabled, the devices function as line drivers. |
| 5 | 4 | 6 | DI | Driver Input. With DE high, a low on DI forces output Y low and output Z high, and a high on DI forces output Y high and output Z low. |
| 6, 7 | 5 | 7 | GND | Ground |
| 9 | — | — | Y | Noninverting Driver Output |
| 10 | — | — | Z | Inverting Driver Output |
| — | 6 | 8 | A | Noninverting Receiver Input and Noninverting Driver Output |
| 12 | — | — | A | Noninverting Receiver Input |
| — | 7 | 1 | B | Inverting Receiver Input and Inverting Driver Output |
| 11 | — | — | B | Inverting Receiver Input |
| 14 | 8 | 2 | VCC | Positive Supply: 4.75V to 5.25V |
| 1, 8, 13 | — | — | N.C. | No Connect—not internally connected |

5

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

MAX1482/MAX1483

Test Circuits

Y

VOD

R

R           VOC

RECEIVER  TEST POINT OUTPUT

CRL                                      1k

15pF

1k

VCC

S1

Z                                                                                                                                                                            S2

Figure 1.  Driver DC Test Load

Figure 2.  Receiver Timing Test Load

3V

DE                                           CL1

A

DI                    Y                              RDIFF                                                     RO

500Ω

VCC

S1

VID                                                                                                                                                     OUTPUT

B

Z                                                                                RE

CL2

UNDER TEST

CL

S2

Figure 3. Driver/Receiver Timing Test Circuit                                       Figure 4. Driver Timing Test Load

6

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

MAX1482/MAX1483

Switching Waveforms

3V

DI                 1.5V

0V

tPLH

tPHL

1.5V

1/2 VO

3V

DE                   1.5V                                                                                   1.5V

0V

tZL(SHDN), tZL                      tLZ

Z

VO

Y

1/2 VO

VO

VDIFF

= V (Y) - V (Z)

Y, Z

VOL

Y, Z

2.3V  OUTPUT NORMALLY LOW

OUTPUT NORMALLY HIGH

2.3V

VOL + 0.5V

VOH - 0.5V

VDIFF   0V

-VO

10%

tR

90%                            90%

tF

tSKEW = | tPLH - tPHL |

10%                               0V

tZH(SHDN), tZH                    tHZ

Figure 5.  Driver Propagation  Delays

Figure 6.  Driver Enable and Disable Times

VOH

RO

VOL

1.5V

OUTPUT

1.5V

3V RE

0V

V

1.5V                                                                                   1.5V

tZL(SHDN), tZL                      tLZ

A-B  VID

tPHL

tPLH

CC

RO                                               1.5V  OUTPUT NORMALLY LOW

VOL + 0.5V

-VID              0V

INPUT                                 0V

RO                                            1.5V

0V

OUTPUT NORMALLY HIGH

VOH - 0.5V

tZH(SHDN), tZH                    tHZ

Figure 7. Receiver Propagation Delays

Figure 8. Receiver Enable and Disable Times

7

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

TTL IN

100pF

Z                                   B

RECEIVER

tR, tF < 6ns        D                                                  R

OUT

Y        R = 54Ω               A

10dB/div

100pF

0Hz                              500kHz/div                         5MHz

Figure 9.  Driver Output Waveform and FFT, Transmitting

MAX1482/MAX1483

250kbps  (125kHz) Signal

| Figure 10. Receiver Propagation-Delay Test Circuit INPUTS |  |  | OUTPUT |
| --- | --- | --- | --- |
| RE | DE* | A-B | RO |
| 0 | 0 | > +0.2V | 1 |
| 0 | 0 | < -0.2V | 0 |
| 0 | 0 | Inputs open | 1 |
| 1 | 0 | X | High-Z |

Table 1. Transmitting                                         Table 2.  Receiving

| INPUTS |  |  | OUTPUTS |  |
| --- | --- | --- | --- | --- |
| RE | DE | DI | Z | Y |
| X | 1 | 1 | 0 | 1 |
| X | 1 | 0 | 1 | 0 |
| X | 0 | X | High-Z | High-Z |

X = Don't Care

High-Z = High Impedance

X = Don't Care

High-Z = High Impedance

* DE = 0 for MAX1483 and is a Don't Care for MAX1482.

Applications Information

The MAX1482/MAX1483 are low-power transceivers for RS-485 and RS-422 communications.  The MAX1482 and MAX1483 are specified  for data rates of at least

250kbps.  The MAX1482 is a full-duplex  transceiver while the MAX1483 is half duplex. When disabled,  the driver and receiver outputs are high impedance.

The 96kΩ, 1/8-unit-load receiver input impedance of the MAX1482/MAX1483 allows up to 256 transceivers on a bus, compared  to the 1-unit load (12kΩ input imped- ance) of standard RS-485 drivers (32 transceivers max- imum). Any combination  of MAX1482/MAX1483 and other RS-485 transceivers with a total of 32 unit loads or less can be put on the bus.

Reduced EMI  and Reflections The MAX1482/MAX1483 are slew-rate limited, minimiz- ing EMI and reducing reflections caused by improperly terminated cables. Figure 9 shows both the driver out- put waveform of a MAX1482/MAX1483 transmitting a

125kHz signal and the Fourier analysis of that signal.

High-frequency  harmonics have much lower ampli- tudes, and the potential for EMI is significantly reduced.

8

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

A

500mV/div

B

500mV/div

B RO

5V/div

A

5V/div

RO

500ns/div

MAX1482/MAX1483

500ns/div

Figure 11.  Receiver tPHL                                                                                      Figure 12.  Receiver tPLH

Low-Power Shutdown Mode A low-power shutdown mode is initiated by bringing RE high and DE low. The devices will not shut down unless both the driver and receiver are disabled.   In shut- down, the devices typically draw only 0.1µA of supply current.

RE and DE may be driven simultaneously; the parts are guaranteed not to enter shutdown if RE is high and DE is low for less than 50ns. If the inputs are in this state for at least 600ns, the parts are guaranteed to enter shut- down.

For the receiver, the tZH and tZL enable times assume the part was not in the low-power shutdown state. The tZH(SHDN) and tZL(SHDN) enable times assume the parts were shut down (see Electrical Characteristics).

It takes the receivers longer to become  enabled  from the low-power shutdown state (tZH(SHDN), tZL(SHDN)) than from the operating mode (tZH, tZL). (The parts are in operating mode if the RE , DE inputs equal a logical

0,1 or 1,1 or 0,0.)

Driver Output Protection Excessive output current and power dissipation caused by faults or by bus contention are prevented  by two mechanisms.  A foldback  current limit on the output stage provides immediate protection against short cir- cuits over the whole common-mode voltage range (see Typical Operating  Characteristics).  In addition,  a ther- mal shutdown circuit  forces the driver outputs into a high-impedance  state if the die temperature  rises excessively.

Propagation Delay Digital encoding  schemes depend  on the driver and receiver skew. Skew is defined  as the difference between the rising and falling propagation delay times. Typical propagation  delays are shown in Figures 11 and 12 using Figure 10’s test circuit.

The difference in receiver delay times, | tPLH - tPHL |, is typically under 160ns.

The driver skew times are typically 160ns (800ns max).

9

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

DI

RECEIVER INPUT VY-VZ

MAX1482/MAX1483

RO

2µs/div

Line Length vs.  Data Rate

The RS-485/RS-422 standard covers line lengths up to

4000 feet. For line lengths greater than 4000 feet, see

5                 Figure 16.

0                 Figure 13 shows the system differential voltage for the parts driving 4000 feet of 26AWG twisted-pair  wire at

110kHz into 120Ω loads. Even after 4000 feet of cable,

1                 the MAX1482/MAX1483 output shows virtually no dis- tortion.

0

Typical Applications

-1                The MAX1482/MAX1483 transceivers are designed  for bidirectional  data communications  on multipoint bus transmission lines. Figures 14 and 15 show typical net-

5                 work applications  circuits.  These parts can also be used as line repeaters, with cable lengths longer than

0

4000 feet, as shown in Figure 16.

To minimize reflections, the line should be terminated at both ends in its characteristic  impedance,  and stub lengths off the main line should be kept as short as

Figure 13.  System Differential Voltage at 250kbps  (125kHz)

Driving 4000 Feet of Cable

possible  (although the slew-rate-limited MAX1482 and MAX1483 are more tolerant of imperfect  termination than standard RS-485 ICs).

Isolated RS-485

For isolated RS-485 applications,  see the MAX253 and

MAX1480 data sheets.

DI                                                           B

D

120Ω                                                                                                                     120Ω

DE B

D

DI

DE                                                           A                             B                  A

B                  A                                   A

RO                                 R

RE

MAX1483

R                             RO RE

R                                                                    R D                                                           D

DI                 DE

RO  RE

DI                 DE

RO  RE

Figure 14. MAX1483 Typical Half-Duplex RS-485 Network

10

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

RO                         R

RE DE

DI                D

A                                                                                                                                                                                                 Y

MAX1482/MAX1483

120Ω                                                                                                                                                                        120Ω                           D              DI B                                                                                                                                                                                                      Z

DE Z                                                                                                                                                                                                 B

RE

120Ω                                                                                                                                                                 120Ω                 R                        RO

Y                                                                                                                                                                                                 A

Y              Z        B               A

Y               Z        B               A

R                                                                                        R

D                                                                            D                                                       MAX1482

DI            DE

RE RO

DI              DE  RE RO

NOTE:  RE AND DE ON.

Figure 15.  MAX1482 Full-Duplex RS-485 Network

Typical Operating Circuits

(continued)

DE        VCC

VCC    RE

RO                   R

RE

DE

MAX1482

A

B

120Ω                           DATA IN

DI          5

4          14

9      Y

D                    10

Z

MAX1482

Rt                        R                            RO

12    A

Z                                                                                                                 2                                                      Rt

DI            D

120Ω

Y

DATA OUT

RO

N.C. 1, 8, 13

R          11                                                                                                       D              DI

B

3           6, 7

RE  GND

GND       DE

NOTE:  RE AND DE ON.

Figure 16. Line Repeater for MAX1482

11

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

MAX1482/MAX1483

Pin  Configurations (continued)

Chip Information

TOP VIEW

TRANSISTOR COUNT: 294

MAX1482

N.C.    1

RO     2            R RE     3

DE     4

DI    5

D GND    6

GND  7

14  VCC

13  N.C.

12  A

11  B

10  Z

9   Y

8   N.C.

DIP/SO

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

| C α A 0.101mm e 0.004 in B A1 L | DIM | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MIN | MAX | MIN | MAX |
|  | A | 0.036 | 0.044 | 0.91 | 1.11 |
|  | A1 | 0.004 | 0.008 | 0.10 | 0.20 |
|  | B | 0.010 | 0.014 | 0.25 | 0.36 |
|  | C | 0.005 | 0.007 | 0.13 | 0.18 |
|  | D | 0.116 | 0.120 | 2.95 | 3.05 |
|  | E | 0.116 | 0.120 | 2.95 | 3.05 |
|  | e | 0.0 | 256 | 0. | 65 |
|  | H | 0.188 | 0.198 | 4.78 | 5.03 |
|  | L | 0.016 | 0.026 | 0.41 | 0.66 |
|  | α | 0° | 6° | 0° | 6° |
| E H 8-PIN µMAX MICROMAX SMALL OUTLINE PACKAGE D |  |  |  |  |  |

12

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

MAX1482/MAX1483

SOICN .EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | 0.053 | 0.069 | 1.35 | 1.75 |
| A1 | 0.004 | 0.010 | 0.10 | 0.25 |
| B | 0.014 | 0.019 | 0.35 | 0.49 |
| C | 0.007 | 0.010 | 0.19 | 0.25 |
| e | 0.050 BSC |  | 1.27 BSC |  |
| E | 0.150 | 0.157 | 3.80 | 4.00 |
| H | 0.228 | 0.244 | 5.80 | 6.20 |
| L | 0.016 | 0.050 | 0.40 | 1.27 |

|  | INCHES |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX | N | MS012 |
| D | 0.189 | 0.197 | 4.80 | 5.00 | 8 | AA |
| D | 0.337 | 0.344 | 8.55 | 8.75 | 14 | AB |
| D | 0.386 | 0.394 | 9.80 | 10.00 | 16 | AC |

| N E H VARIATIONS: 1 TOP VIEW D A C e B A1 0∞-8∞ L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, .150" SOIC |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0041 | REV. B | 1 1 |

13

20µA, 1⁄8-Unit-Load, Slew-Rate-Limited

RS-485 Transceivers

MAX1482/MAX1483

PDIPN.EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) |  |  |
| --- | --- | --- |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

14                                  Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  2006 Maxim Integrated  Products                                                               is a registered  trademark  of Maxim Integrated  Products, Inc.