19-1128; Rev 0; 9/96

Digitally Controlled CCFL  Backlight

Power Supplies

General Description

The MAX1610/MAX1611 are fully integrated,  high- efficiency  drivers for cold-cathode  fluorescent lamps (CCFLs). They operate from a 4.5V to 26V power source. An on-board,  high-switching-frequency  power MOSFET reduces external component count and mag- netics size. The MAX1610/MAX1611 protect  against open or shorted lamps. The CCFL can be driven from an isolated transformer secondary  winding  to improve efficiency  and avoid  flicker  at dim tube settings. Brightness is adjusted  by scaling the lamp current, or by operating  with a fixed lamp current and chopping the CCFL on and off at a rate faster than the eye can detect.

The MAX1610’s digital inputs increment, decrement, or clear an internal, 5-bit up/down  counter, which sets CCFL brightness.  The MAX1611 uses a System Management  Bus (SMBus) 2-wire serial interface to directly  set CCFL brightness.  Both devices  include micropower shutdown and a linear regulator that elimi- nates the need for a separate logic supply. The digital interface remains active in shutdown, preserving  the brightness setting.

Features

® Direct Digital Control of CCFL Brightness

® Low Supply Current:   3mA Max Operating

20µA Max Shutdown

® Low-Voltage Operation, Down to 4.5V

® Internal 26V, 0.7W Power Switch

® Protection Against Open or Shorted Lamps

® Supports Isolated Transformer Secondary

Winding

® SMBus Serial Interface (MAX1611)

® No Flicker at Low Brightness (internal 280Hz current chopping)

® High Power-to-Light Efficiency

® Selectable 290kHz/145kHz Switching Frequency

® Oscillator SYNC Input

® 16-Pin Narrow SO Package

Applications

Ordering Information

Notebook/Laptop Computers

PART

TEMP. RANGE 	PIN-PACKAGE

Point-of-Sale Terminals

MAX1610CSE

0°C to +70°C

16 Narrow SO

Portable Medical Equipment

Instrument Displays

MAX1611CSE 	0°C to +70°C

16 Narrow SO

MAX1610/MAX1611

Pin  Configurations

| TOP VIEW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UP | 1 |  |  | 16 | BATT | SDA | 1 |  |  | 16 | BATT |
|  | DN | 2 |  |  | 15 | LX | SCL | 2 |  |  | 15 | LX |
|  | SHDN | 3 |  |  | 14 | BST | SMBSUS | 3 |  |  | 14 | BST |
|  | SYNC | 4 | MAX1610 |  | 13 | GND | SYNC | 4 | MAX1611 |  | 13 | GND |
|  | SS | 5 |  |  | 12 | VL | SS | 5 |  |  | 12 | VL |
|  | CC | 6 |  |  | 11 | CS | CC | 6 |  |  | 11 | CS |
|  | CSAV | 7 |  |  | 10 | OTP | CSAV | 7 |  |  | 10 | OTP |
|  | MINDAC | 8 |  |  | 9 | REF | MINDAC | 8 |  |  | 9 | REF |
|  |  |  | SO |  |  |  |  |  | SO |  |  |  |

ABSOLUTE MAXIMUM RATINGS

BATT to GND ............................................................-0.3V to 28V BST to GND ..............................................................-0.3V to 30V BST to LX ....................................................................-0.3V to 6V LX to GND ................................................-0.6V to (BATT + 0.3V) VL to GND ...................................................................-0.3V to 6V CS, CSAV, CC, SYNC, REF, MINDAC,

SS, OTP to GND............................................-0.3V to (VL + 0.3V) SHDN, UP, DN to GND ...............................................-0.3V to 6V SMBSUS, SDA, SCL to GND ......................................-0.3V to 6V

BATT, LX Current .....................................................................1A SDA Current ........................................................................50mA VL Current ...........................................................................50mA Continuous Power Dissipation  (TA = +70°C)

SO (derate 8.70mW/°C above +70°C) .........................696mW

Operating  Temperature  Range

MAX1610CSE/MAX1611CSE ..............................0°C to +70°C Storage Temperature  Range .............................-65°C to +160°C Lead Temperature  (soldering,  10sec) .............................+300°C

MAX1610/MAX1611

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(TA = 0°C to +70°C, BATT = 8.2V, MINDAC = 0V, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- |
| SUPPLY AND REFERENCE |  |  |  |
| BATT Input Voltage Range |  | 4.75 26 | V |
| BATT Quiescent Supply Current, Operate Mode | BATT = 25V | 1.5 3 | mA |
| BATT Quiescent Supply Current, Shutdown Mode |  | 10 20 | A |
| VL Output Voltage, Operate Mode | 4.75V < BATT < 26V | 4.25 4.5 4.75 | V |
| VL Output Voltage, Shutdown Mode |  | 3.0 3.6 4.75 | V |
| REF Output Voltage | No load | 1.92 2.0 2.08 | V |
| REF Load Regulation | ISOURCE = 100A | 6 20 | mV |
| SWITCHING REGULATOR |  |  |  |
| BATT-to-LX Switch On-Resistance | BST - LX = 4.1V | 0.7 1.0 | Ω |
| LX Switch Off-Leakage Current |  | 10 | A |
| Oscillator Frequency | SYNC = REF | 250 290 330 | kHz |
|  | SYNC = GND | 125 145 165 |  |
| Oscillator SYNC Pin Synchronization Range |  | 240 350 | kHz |
| SYNC High Pulse Width |  | 200 | ns |
| SYNC Low Pulse Width |  | 200 | ns |
| SYNC Input Current | SYNC = GND or VL | -1 1 | A |
| SYNC Input Low Voltage |  | 0.5 | V |
| SYNC Input High Voltage |  | 4.0 | V |
| Power-Switch Maximum Duty Cycle | SYNC = REF | 89 91 | % |
| SS Source Current | SS = GND | 2.5 4.0 5.5 | A |
| SS Sink Current | SS = 0.5V | 2 | mA |

MAX1610/MAX1611

ELECTRICAL CHARACTERISTICS (continued)

(TA = 0°C to +70°C, BATT = 8.2V, MINDAC = 0V, unless otherwise noted. Typical values are at TA = +25°C.)

| PARAMETER | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| DAC AND ERROR AMPLIFIER |  |  |  |  |
| DAC Resolution | Guaranteed monotonic |  | 5 | Bits |
| MINDAC Input Voltage Range |  |  | 0 1 | V |
| MINDAC Input Bias Current |  |  | -1 1 | A |
| MINDAC Digital PWM Threshold |  |  | 3 | V |
| CSAV Input Voltage Range |  |  | 0 1.0 | V |
| CSAV Regulation Point | D/A at full scale |  | 232 247 260 | mV |
|  | D/A at 1LSB |  | 12 |  |
| CSAV Input Bias Current |  |  | -5 5 | A |
| CSAV to CC Voltage-to-Current Converter Transconductance | CC = 2V, CSAV = 1V, D/A at 1LSB |  | 85 | mho |
| CC Sink Current | CC = 2V, CSAV = 1V, D/A at 1LSB |  | 80 | A |
| CC Source Current | CC = 2V, CSAV = 0V, D/A at full scale |  | 20 | A |
| OPEN AND SHORTED TUBE PROTECTION |  |  |  |  |
| OTP Voltage Trip Point | Referred to REF | OTP rising | -20 20 | mV |
| OTP Input Bias Current | GND < OTP < VL |  | -1 1 | A |
| CS Overcurrent Cutoff Threshold |  |  | 500 | mV |
| MAX1610 LOGIC LEVELS |  |  |  |  |
| SHDN, UP, DN Input Low Voltage |  |  | 0.8 | V |
| SHDN, UP, DN Input High Voltage |  |  | 2.4 | V |
| SHDN, UP, DN Input Bias Current |  |  | -1 1 | A |
| MAX1611 LOGIC LEVELS |  |  |  |  |
| SMBSUS, SDA, SCL Input Low Voltage |  |  | 0.8 | V |
| SMBSUS, SDA, SCL Input High Voltage |  |  | 2.2 | V |
| SMBSUS, SDA, SCL Input Bias Current |  |  | -1 1 | A |
| SDA Output Low Sink Current | VSDA = 0.6V |  | 6 | mA |

MAX1610/MAX1611

TIMING CHARACTERISTICS—MAX1610

(Figure 1, TA = +25°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| UP, DN Pulse Width High | t1 |  | 1 | s |
| UP, DN Pulse Width Low | t2 |  | 1 | s |
| UP, DN Pulse Separation | t3 |  | 1 | s |
| Counter Reset Time | t4 |  | 1 | s |

TIMING CHARACTERISTICS—MAX1611

(Figures 2 and 3, TA = +25°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| SCL Serial Clock High Period | tHIGH |  | 4 | s |
| SCL Serial Clock Low Period | tLOW |  | 4.7 | s |
| SCL, SCA Rise Time | tR | (Note 1) | 1 | s |
| SCL, SDA Fall Time | tF | (Note 1) | 0.3 | s |
| Start Condition Setup Time | tSU:STA |  | 4.7 | s |
| Start Condition Hold Time | tHD:STA |  | 4 | s |
| SDA Valid to SCL Rising Edge Setup Time, Slave Clocking in Data | tSU:DAT |  | 500 | ns |
| SCL Falling Edge to SDA Transition | tHD:DAT | (Note 1) | 0 | ns |
| SCL Falling Edge to SDA Valid, Reading Out Data | tDV |  | 1 | s |

Note 1: Guaranteed  by design.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

2.2

2.1

REF OUTPUT VOLTAGE

vs. REF OUTPUT CURRENT

SHDN = VL, BATT = 5V

MAX1610/1611-TOC1

2.0

1.8

BATT SUPPLY CURRENT

MAX1610/1611-TOC2

vs. BATT VOLTAGE (SHDN = VL)

SHDN = VL, OTP = 3V

5.0

4.5

VL OUTPUT VOLTAGE

vs. VL OUTPUT CURRENT

MAX1610/1611-TOC3

SHDN = VL, OTP = 3V

REF OUTPUT VOLTAGE (V)

2.0

1.9

1.8

1.7

1.6

1.6

BATT CURRENT (mA)

1.4

1.2

VL VOLTAGE (V)

4.0

3.5

3.0

2.5

BATT = 5V

BATT = 12V

1.5

1

10	100	1000	10000

1.0

0

4	8	12	16	20	24	28

2.0

0

10	20	30	40

REF OUTPUT CURRENT (µA)

BATT (V)

VL OUTPUT CURRENT (mA)

| 3.70 SHDN = GND |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | BATT = 12V |  |  |  |
|  |  | BATT = 5V |  |  |

3.65

3.60

3.55

3.50

VL VOLTAGE (V)

3.45

3.40

3.35

3.30

VL OUTPUT VOLTAGE

MAX1610/1611-TOC4

vs. VL LOAD CURRENT

BATT SUPPLY CURRENT

MAX1610/1611-TOC5

vs. BATT VOLTAGE (SHDN = OV)

10

SHDN = OV

8

6

BATT CURRENT (  A)

4

2

0

| 5.0 NO LOAD ON VL, SHDN = OV |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

4.5

4.0

VL (V)

3.5

3.0

2.5

2.0

VL OUTPUT VOLTAGE

MAX1610/1611-TOC6

vs. BATT VOLTAGE (SHDN = OV)

0	200

400

600

800

1000

0	4	8

12	16	20	24	28

0	4	8

12	16	20	24	28

VL LOAD CURRENT (  A)

BATT (V)

BATT (V)

5.0

VL OUTPUT VOLTAGE

MAX1610/1611-TOC7

vs. BATT VOLTAGE (SHDN = VL)

4.5

4.0

VL (V)

3.5

3.0

MAX1610/MAX1611

2.5

2.0

| 0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NO LOAD ON VL, SHDN = VL |  |  |  |  |  |  |

4	8	12	16	20	24	28

BATT (V)

MAX1610/MAX1611

Pin  Description

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| MAX1610 | MAX1611 |  |  |
| 1 | — | UP | Logic-Level Input. A rising edge on UP increments the 5-bit counter for the 5-bit DAC. UP = DN = 1 presets the counter to mid-scale. |
| — | 1 | SDA | System Management Bus Serial Data Input and Open-Drain Output |
| 2 | — | DN | Logic-Level Input. A rising edge on DN decrements the 5-bit counter for the 5-bit DAC. UP = DN = 1 presets the counter to mid-scale. |
| — | 2 | SCL | System Management Bus Serial Clock Input |
| 3 | — | SHDN | Logic-Level Shutdown Input Pin. Applying a logic low to SHDN places the chip in a low- supply-current shutdown mode. |
| — | 3 | SMBSUS | System Management Bus Suspend Mode Input. SMBSUS Selects one of two chip- configuration settings, which are preprogrammed serially. |
| 4 | 4 | SYNC | Oscillator Synchronization Input. Tying SYNC to REF sets the oscillator frequency to 290kHz. Tying SYNC to GND or VL lowers the oscillator frequency to 145kHz. |
| 5 | 5 | SS | Soft-Start Pin. A 4µA current source feeds the capacitor placed on SS. The voltage on this pin limits the peak current in the switch. When the lamp is turned off, SS pulls to GND. |
| 6 | 6 | CC | Output of the Voltage-to-Current Converter; Input to the PWM Comparator, which sets the current limit. A capacitor placed at CC sets the current-regulator-loop bandwidth. |
| 7 | 7 | CSAV | Input to the Voltage-to-Current Converter, which averages the voltage on CSAV using the capacitor on CC. |
| 8 | 8 | MINDAC | The voltage at MINDAC sets the DAC’s minimum-scale output voltage. Tying MINDAC to VL enables the internal 280Hz current-chopping mode. |
| 9 | 9 | REF | 2.0V Reference Output. Bypass with 0.1F to GND. |
| 10 | 10 | OTP | Open-Tube Protection Comparator. As long as OTP exceeds the reference voltage, the N-channel BATT-to-LX switch is forced off. |
| 11 | 11 | CS | Low-Side Current-Sense Input. The current-mode regulator terminates the switch cycle when the voltage at CS exceeds REF - CC. |
| 12 | 12 | VL | Output of the Internal Linear Regulator. VL can be overdriven by a voltage greater than 4.75V to operate the chip from +5V ± 5%, and to conserve power. Bypass with 0.1F to GND. |
| 13 | 13 | GND | System Ground |
| 14 | 14 | BST | Power Input to the High-Side Gate Driver, which switches the internal N-channel MOSFET on and off. |
| 15 | 15 | LX | Ground Connection for the Internal High-Side Gate Driver; source-connection point for the internal N-channel MOSFET |
| 16 | 16 | BATT | 4.5V to 25V Battery-Voltage Input Point. Connects to the internal N-channel power MOSFET’s drain, and to the input of the internal linear regulator that powers the chip. |

t1	t2	t4

UP

t3

DN

Figure 1. MAX1610 UP and DN Signal Timing

START CONDITION

MOST SIGNIFICANT ADDRESS BIT (A6) CLOCKED INTO SLAVE

A5 CLOCKED INTO SLAVE

A4 CLOCKED INTO SLAVE

A3 CLOCKED INTO SLAVE

SCL

• • •

tHD:STA

tLOW	tHIGH

SDA

• • •

tSU:STA 	tSU:DAT

tHD:DAT

tSU:DAT

tHD:DAT

MAX1610/MAX1611

Figure 2. MAX1611 SMB Serial-Interface Timing—Address

RW BIT CLOCKED INTO SLAVE

ACKNOWLEDGED BIT CLOCK

INTO MASTER

MOST SIGNIFICANT BIT CLOCKED

SCL • • •

SDA • • •

SLAVE PULLING SDA LOW

tDV

tDV

Figure 3. MAX1611 SMB Serial-Interface Timing—Acknowledge

MAX1610/MAX1611

Detailed Description

Getting Started A cold-cathode fluorescent lamp (CCFL) has two termi- nals. For the CCFL to emit light, the two lamp terminals must be driven with a high-voltage  (approximately

550V AC RMS) and high-frequency  (approximately

45kHz) sine wave. The MAX1610/MAX1611 use a vary- ing DC input voltage to create this high-voltage,  high- frequency  sine-wave drive.  To select  the correct component  values for the MAX1610/MAX1611 circuit, several CCFL parameters and the minimum DC input voltage must be specified; these are listed in Table 1.

Table 3 shows the recommended component values to use with the circuit of Figure 4, depending  on the par- ticular CCFL parameters.  The C2 values in Table 3 have been selected  such that the normal operating voltage on the secondary of T1 is as close as possible to the CCFL strike voltage (where the strike voltage (VS) is assumed to be approximately  1.8 times the CCFL operating voltage (VL)).

Components  T1, C1, R2, Q1, and Q2 form a Royer oscillator. A Royer oscillator is a resonant tank circuit that oscillates at a frequency dependent on C1, the pri- mary magnetizing  inductance  of T1 (LP), and the

impedance seen by the T1 secondary. The MAX1610/MAX1611 regulate the current fed into the Royer oscillator by sensing the voltage on R1. For a given current through the Royer oscillator (IR1), the power delivered  to the CCFL depends  on the Royer oscillator frequency.  The R1 values in Table 3 have been selected to ensure that the power into the CCFL

does not exceed its maximum rating, despite T1, C1, and C2 component-value  variations. The Royer oscillator waveforms for the circuit of Figure 4 are shown in Figures

5 and 6.

Analog Circuitry The MAX1610/MAX1611 maintain fixed CCFL bright- ness with varying input voltages on BATT by regulating the current fed into the Royer oscillator. This current is sensed via resistor R1 between CSAV and GND. An internal switch from BATT-to-LX pulse-width modulates at a fixed frequency to servo the CSAV pin to its regula- tion voltage. The CSAV regulation  voltage can be adjusted  via the digital  interface to set CCFL bright- ness. The MAX1610 and MAX1611 differ only in the digital interface they use to adjust the internal 5-bit digi- tal-to-analog converter (DAC) that sets the CSAV regu- lation voltage. The minimum-scale (min-scale) CSAV regulation voltage is resistor adjustable using the MIN- DAC pin, setting the minimum CCFL brightness.  The D/A setting at MAX1610/MAX1611 power-up is preset to mid-scale (10000 binary) (Figure 7).

MINDAC Sets the Minimum Scale The MINDAC pin sets the lowest CCFL brightness level. The voltage at MINDAC is divided  by eight, and sets the minimum CSAV regulation voltage. For exam- ple, in the circuit  of Figure 4, R5 (150k) and R6 (51k) form a resistor divider  from REF, which sets MINDAC to 507mV (REF = 2.0V). This sets a minimum CSAV regulation  voltage of 63mV with a full-scale CSAV regulation voltage of 247mV.

VIN

16

+ C9

5

C4

6

BATT

SS

MAX1610

MAX1611*

12

VL

14

BST

15

LX

D3	C6

R7

L1

C7

D2

C2

CCFL

10	6

T1

1	2	3   4	5

D1 	R2

CC

C3

4

SYNC

9

REF

R5

8

OTP     10

11

CS

7

CSAV

13

R3	C1

C5 	Q1	Q2

R4

R1

MAX1610/MAX1611

C8 	MINDAC 	GND

R6

* DIGITAL INTERFACE NOT SHOWN

Figure 4. Typical Floating-Lamp Application Circuit

Table 1. Necessary CCFL Specifications

| SPECIFICATION | UNITS | SYMBOL | DESCRIPTION |
| --- | --- | --- | --- |
| CCFL Minimum Strike Voltage (“Kick-Off Voltage”) | VRMS | VS | Although CCFLs typically operate at 550VRMS, a higher voltage is required initially to light up the tube. |
| CCFL Typical Operating Voltage (“Lamp Voltage”) | VRMS | VL | Once a CCFL has been struck, the voltage required to maintain light output falls to approximately 550VRMS. Small tubes may operate on as little as 250VRMS. The operating voltage of the CCFL stays relatively constant, even as the tube’s brightness is varied. |
| CCFL Maximum Operating Current (“Lamp Current”) | mARMS | IL | The maximum root-mean-square AC current through a CCFL is almost always 5mARMS. No DC current is allowed through any CCFL. |
| CCFL Maximum Frequency (“Lamp Frequency”) | kHz | fL | The maximum AC-lamp-current frequency. |
| DC Power Source Minimum Input Voltage | V | VMIN | The minimum DC input voltage to the MAX1610/MAX1611 circuit determines the turns ratio required for the DC-AC conversion transformer. Decreasing the minimum input voltage increases the size of the transformer required for a given output power. |

Table 2. Typical Application Circuit Component Values

a)  Resistors

| b) Capacitors SYMBOL | VALUE | TOLER- ANCE | WORKING VOLTAGE | NOTES |
| --- | --- | --- | --- | --- |
| C1 | 0.1F | ±20% | ±25V | F ≤ 0.001 @ 1kHz |
| C2 | (Note 1) (pF) | ±10% | ±3kV | High voltage |
| C3, C5 | 27nF | ±20% | 25V |  |
| C4, C6, C7, C8 | 0.1F | -20% | 25V | Ceramic, larger values acceptable |
| C9 | 10F | -50% | 35V | Tantalum, low ESR |

MAX1610/MAX1611

| SYMBOL | VALUE | TOLERANCE | POWER RATING |
| --- | --- | --- | --- |
| R1 | (Note) | ±1% | 1/8W |
| R2 | 510Ω | ±10% | 1/8W |
| R3 | 51kΩ | ±5% | 1/16W |
| R4 | 8.2kΩ | ±5% | 1/16W |
| R5 | 150kΩ | ±5% | 1/16W |
| R6 | 51kΩ | ±5% | 1/16W |
| R7 | 20Ω | ±10% | 1/16W |

c) Other Components

| SYMBOL | DESCRIPTION | GENERIC PART | SURFACE-MOUNT PART | MANUFACTURER |
| --- | --- | --- | --- | --- |
| Q1, Q2 | 1A NPN switching transistor, VCEO ≥ 50V | 2N2222A | FMMT619, SOT23 | Zetex |
| D1, D3 | 50mA silicon diode, VBR ≥ 40V | 1N4148 | CMPD4448, SOT23 | Central |
| D2 | 1A Schottky diode, VBR ≥ 30V | 1N5818 | EC10QS04 | Nihon |
| L1 | 100µH, 1A inductor |  | CDR125-101 | Sumida |
| T1 | 6W Royer oscillator transformer, turns ratio 67:1, secondary (pins 10 and 6) : primary (pins 1 and 3), primary magnetizing inductance (LP) of 44µH ±20% |  | CTX110605 | Coiltronics |

Note:  Component  values depend on lamp characteristics. See Table 3 to select values.

Table 3. Selecting Circuit Values for Figure 4

| VL (VRMS) | IL (mARMS) | C2 | R1 | VCT (VMAX) | fROY (kHz) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MIN TYP MAX |
| 250 | 3 | 22pF | 1.21Ω | 3.63V | 50.3 58.6 71.8 |
| 250 | 5 | 43pF | 0.715Ω | 3.61V | 43.3 49.7 60.3 |
| 300 | 3 | 18pF | 1.18Ω | 4.30V | 52.1 61.0 75.1 |
| 300 | 5 | 36pF | 0.681Ω | 4.14V | 45.6 52.8 64.7 |
| 450 | 5 | 20pF | 0.732Ω | 6.55V | 51.1 59.7 73.3 |
| 500 | 5 | 18pF | 0.715Ω | 7.17V | 52.1 61.0 75.1 |
| 550 | 5 | 18pF | 0.665Ω | 7.29V | 52.5 61.8 76.7 |
| 600 | 5 | 15pF | 0.698Ω | 8.41V | 53.6 63.1 78.1 |

Note:  fROY = Royer oscillator damped resonant oscillation frequency. T1 primary magnetizing inductance (LP) = 44µH ±20%.

VCT = average voltage from the T1 center tap to the emitters of Q1 and Q2 (ignoring Q1, Q2 VCE,SAT).

C1 = 0.1µF ± 20%; C2 = ±10% tolerance; R1 = ±1% tolerance.

FIGURE 4 CIRCUIT, C2 = 15pF, IR1 = 462mA, CCFL VL = 500VRMS

BATT = 10V, IBATT = 0.20A,

MINDAC = 0.5V, D/A VALUE = 11111

6V T1

CENTER-TAP VOLTAGE

0V

1A C1

CURRENT

-1A

FIGURE 4 CIRCUIT, C2 = 15pF, R1 = 545  ,

CCFL VL = 500VRMS, BATT = 15V, MINDAC = 0.5V, D/A VALUE = 10000

3V

SS VOLTAGE

0V

6V T1

CENTER-TAP VOLTAGE

0V

5µs/div

Figure 5. Royer Oscillator Typical Operating Waveforms for

Circuit of Figure 4

10ms/div

Figure 6. Start-Up Waveforms for Circuit of Figure 4

MAX1610/MAX1611

REF / 8 = 250mV FULL-SCALE

CSAV REGULATION VOLTAGE

MID-SCALE

MIN-SCALE = MINDAC / 8

00000

00001

00010

00011

00100

00101

00110

00111

01000

01001

01010

01011

01100

01101

01110

01111

10000

10001

10010

10011

10100

10101

10110

10111

11000

11001

11010

11011

11100

11101

11110

11111

OmV

DAC CODE

NOTE: DAC CODE 00000  FORCES THE BATT-TO-LX  SWITCH  OFF REGARDLESS  OF CSAV OR MINDAC  VOLTAGE.

Figure 7. CSAV Regulation Voltage Range

Open-Tube Protection (OTP) Any real transformer used in a Royer oscillator will have a maximum-allowed secondary voltage. If the maximum- allowed secondary  voltage is exceeded,  the winding insulation can break down, leading to permanent trans- former damage. The maximum-allowed secondary volt- age can be exceeded either when the CCFL drive circuit is turned on without the CCFL being in place, or when the CCFL becomes disconnected  during normal opera- tion due to a mechanical failure. To protect against these fault conditions, use the OTP pin to sense the voltage on the transformer center tap (pin 2 of Figure 4). Whenever the voltage on OTP exceeds the REF reference voltage, the BATT-to-LX power switch is forced off.

For example, in Figure 4, the CTX110605 transformer has a maximum-allowed continuous secondary voltage of 1340VRMS. D1 and C5 detect  the peak voltage on the center tap of T1. R3 and R4 determine the limit on the center tap peak voltage. The relationship between the voltage on the center tap of T1 and the secondary voltage is diagrammed  in Figure 8. Neglecting  the Q1/Q2 saturation voltage and the voltage on the R1 current-sense resistor yields Equation 1:

V	2

Loop-Compensation Capacitor (CC) The BATT-to-LX switch turns on at fixed frequency, and turns off when the current-sense voltage on the CS pin exceeds CC - REF. As the CC pin voltage rises, the CS current limit rises as well. A transconductance  amplifier compares the voltage on CSAV to the desired regulation voltage and outputs a current proportional  to this error to the CC pin. A capacitor  from CC to GND sets the bandwidth of this regulation loop, as shown in Equation 2:

BW =   85

2 C3

where BW is the bandwidth of the CSAV regulation loop in kHz, and C3 is the capacitance  from CC to GND in nF.

Soft Start (SS) Soft start prevents the triggering  of OTP upon power- up. Placing a capacitor  from SS to GND soft starts the Royer oscillator by slowly raising the CS current-limit voltage. Internal circuitry  pulls SS to GND during power-on reset, or whenever the lamp is turned off (DAC

= 00000, shutdown mode, ON-1 = 0, or ON-0 = 0) (Figures 10 and 11). When SS is not pulled to GND, an

VCTPK  =

SEC

2N

internal 4µA current sources into the capacitor  at the SS pin. This pin is internally diode clamped  to REF so that it rises to a maximum voltage of about  2.7V.

where VSEC is the maximum root-mean-square voltage

allowed on the secondary, N is the secondary-to-prima-

ry turns ratio, and VCTPK is the peak voltage on the transformer center tap.

Block Diagram of the Analog Section Figure 9 shows a functional diagram of the analog cir- cuitry in the MAX1610/MAX1611. The chips have identi- cal analog circuitry,  and differ only in their digital interface.

Regardless of the voltage on CC, the CS current-sense voltage is never allowed to exceed the voltage on SS divided by 5.

Frequency Selection and Synchronization The SYNC pin performs two functions:  it sets the BATT- to-LX switching frequency, and it allows the BATT-to-LX switching frequency to be synchronized to an external oscillator.  SYNC tied to GND or VL sets a 145kHz switching frequency; SYNC tied to REF sets a 290kHz

T1 PRIMARY CENTER-TAP VOLTAGE (PIN 2)

VCT

2

N VCT

| 2 |  |
| --- | --- |
| 2 |  |

MAX1610/MAX1611

T1 SECONDARY VOLTAGE (PIN 10–PIN 6)

-N VCT

2

2

NOTE: VCT = AVERAGE VOLTAGE FROM THE T1 CENTER TO THE EMITTERS OF Q1 AND Q2 (IGNORING  Q1, Q2 VCE, SAT).     = 2 fROY.

Figure 8. Transformer Primary/Secondary Voltage Relationship

BATT BST

LX

DMOS POWER SWITCH

LEVEL SHIFTER

4.5V REG

VL GND

CS CSAV

CC	GM

5-BIT DAC

÷8

REF

MINDAC

(NOTE)

+

2.0V

-

SYNC

SS

OTP

OSC

UP (SDA) DN (SCL)

SHDN (SMBSUS)

÷5

5

DIGITAL INTERFACE

R

S    Q

4µA

MAX1610/MAX1611

( ) ARE FOR MAX1611

NOTE:  CIRCUITRY TO DETECT MINDAC = VL NOT SHOWN. SEE CHOPPING THE LAMP CURRENT SECTION.

Figure 9. Functional Diagram

MOST SIGNIFICANT ADDRESS BIT

START

LEAST SIGNIFICANT ADDRESS BIT

SLAVE ACKNOWLEDGE

MOST SIGNIFICANT

SLAVE ACKNOWLEDGE

LEAST SIGNIFICANT

CONDITION

R/W BIT

DATA BIT

DATA BIT

SCL

SDA

SHDNB-0

REGSEL	D4-0

D3-0    D2-0	D1-0    D0-0

SLAVE PULLS SDA LOW

STDBY-0

SLAVE PULLS SDA LOW

Figure 10. MAX1611 Serial-Interface Single-Byte Write Example (REGSEL = 0)

MOST SIGNIFICANT ADDRESS BIT

START

LEAST SIGNIFICANT ADDRESS BIT

SLAVE ACKNOWLEDGE

MOST SIGNIFICANT

SLAVE ACKNOWLEDGE

LEAST SIGNIFICANT

CONDITION

R/W BIT

DATA BIT

DATA BIT

SCL

SDA

REGSEL SHDNB-1

D4-1

D3-1    D2-1	D1-1    D0-1

SLAVE PULLS SDA LOW

STDBY-1

SLAVE PULLS SDA LOW

Figure 11. MAX1611 Serial-Interface Single-Byte Write Example (REGSEL = 1)

MAX1610/MAX1611

switching frequency. Any rising edge on SYNC restarts a BATT-to-LX switch cycle by forcing the switch on.

MAX1610 Digital Interface

The MAX1610 contains an internal 5-bit up/down counter that sets the value of the internal 5-bit DAC. At power-on, or when both the UP and DN pins are held high simulta- neously, the 5-bit up/down  counter is preset to 10000 binary, which corresponds  to mid-scale. A rising edge on UP increments the 5-bit up/down  counter. A rising edge on DN decrements the 5-bit up/down counter. The counter will not roll over on either underflow or overflow. For example, if the CCFL is at maximum intensity level, rising edges on UP will not change the output.

The SHDN pin provides a way to lower the MAX1610 supply  current to 10µA without resetting the 5-bit up/down counter. With SHDN = 1, the MAX1610 oper- ates normally with VL at 4.5V. When the BATT-to-LX power switch operates, an additional  3mA of current

(other than the supply current) is consumed  through the BST pin, requiring VL to source at least 4.5mA of current. With SHDN = 0, all analog circuitry  turns off, except  for a coarse regulator that can source up to

500µA from VL. The coarse regulator preserves the state of the internal logic and keeps the digital interface active during shutdown (SHDN = 0).

MAX1611 Digital Interface

A single byte of data written over the Intel System Management  Bus (SMBus™) controls the MAX1611. Figures 10 and 11 show example single-byte writes. The MAX1611 contains two 7-bit latches for storing configu- ration data. Only one of the 7-bit latches is active at a time. The MAX1611 responds only to its own address,

0101101 binary. The SMBSUS pin selects which of the two sets of configuration data is used. Figure 12 shows a schematic diagram of the MAX1611’s digital circuitry. Notice that the SMBSUS pin selects which one of the

CONTROL LOGIC

OTPOK

8

SCL

8-BIT

SHIFT REGISTER

SDA	IN	DATA   LE

7

LE	LE

7-BIT LATCH-0

7-BIT LATCH-1

VL

7	7

CLR   S A	B

SMBSUS

MULTIPLEXER

S	Y = A WHEN S IS LOW

Y

7

D_

SHDNB STDBY

5

OTPOK Q

PRE	R

REF

OTP OTP

COMPARATOR

5-BIT DAC

SS CIRCUITRY

BIAS GENERATORS

MAX1610/MAX1611

Figure 12. MAX1611 Serial-Interface Circuitry Block Diagram

MAX1610/MAX1611

Table 4. MAX1611 Configuration Byte with REGSEL = 0

| BIT | NAME | POR STATE* | DESCRIPTION |
| --- | --- | --- | --- |
| 7 | REGSEL | — | Register Select. A zero in this bit writes the remaining seven bits into the 7-bit latch-0 (Figure 13). |
| 6 | SHDNB-0 | 0 | Complete Shutdown. Pulling SMBSUS low with SHDNB-0 = 0 places the MAX1611 into a low-quiescent-current shutdown mode, with the reference off and the VL linear-regulator output switched to a low-current, coarse regulation mode. Pulling SMBSUS low with SHDNB-0 = 1 puts the MAX1611 into its normal operational mode, with the reference and internal VL linear regulator fully on. SHDNB-0 supersedes STDBY-0. As long as SHDNB-0 = 0 and SMBSUS = 0, it doesn't matter what STDBY-0 is; the MAX1611 still shuts down. |
| 5 | STDBY-0 | 0 | Standby, disables CCFL supply only. As long as SMBSUS stays low and STDBY-0 = 0, the internal power switch is kept off and SS is held shorted to GND; neither the internal refer- ence nor the linear regulator is affected. With STDBY = 1 and SMBSUS low, the MAX1611 operates normally. |
| 4 3 2 1 0 | D4-0 D3-0 D2-0 D1-0 D0-0 | 1 0 0 0 0 | DAC Input Data. With the SMBSUS pin low, bits D4-0 through D0-0 set the DAC. |

* Initial register state after power-up.

Table 5. MAX1611 Configuration Byte with REGSEL = 1

| BIT | NAME | POR STATE* | DESCRIPTION |
| --- | --- | --- | --- |
| 7 | REGSEL | — | Register Select. A one in this bit writes the remaining seven bits into the 7-bit latch-1 (Figure 13). |
| 6 | SHDNB-1 | 1 | Complete Shutdown. Pulling SMBSUS high with SHDNB-1 = 0 places the MAX1611 into a low-quiescent-current shutdown mode, with the reference off and the VL linear regulator output switched to a low-current coarse regulation mode. Pulling SMBSUS high with SHDNB-1 = 1 puts the MAX1611 into its normal operational mode, with the reference and internal VL linear regulator fully on. SHDNB-1 supersedes STDBY-1. As long as SHDNB-1 = 0 and SMBSUS = 0, it doesn’t matter what STDBY-1 is; the MAX1611 still shuts down. |
| 5 | STDBY-1 | 1 | Standby, disables CCFL supply only. As long as SMBSUS stays high and STDBY-1 = 0, the internal power switch is kept off and SS is held shorted to GND; neither the internal ref- erence nor the linear regulator is affected. With STDBY-1 = 1 and SMBSUS high, the MAX1611 operates normally. |
| 4 3 2 1 0 | D4-1 D3-1 D2-1 D1-1 D0-1 | 1 0 0 0 0 | DAC Input Data. With the SMBSUS pin high, bits D4-1 through D0-1 set the DAC. |

* Initial register state after power-up.

MOST SIGNIFICANT ADDRESS BIT

START

LEAST SIGNIFICANT ADDRESS BIT

SLAVE ACKNOWLEDGE

MOST SIGNIFICANT

CONDITION

R/W BIT

DATA BIT

SCL

SDA

SLAVE PULLS SDA LOW

OTPOK

DA4	DA3	DA2	DA1	DA0

MAX1611 DRIVES SDA

Figure 13. MAX1611 Serial-Interface Read Example

Table 6. MAX1611 Status Bits

| BIT | NAME | POR STATE* | FUNCTION |
| --- | --- | --- | --- |
| 7 | OTPOK | 1 | Latched Open-Tube Detection. OTPOK = 0 indicates that open-tube detection has been triggered. As soon as the voltage on the OTP pin exceeds REF, the OTPOK bit is cleared. Reset the OTPOK pin by entering shutdown or standby. |
| 6 5 | — — | — — | Unused. These bits always return a logic one. |
| 4 3 2 1 0 | DA4 DA3 DA2 DA1 DA0 |  | Displays the DAC setting selected by SMBSUS. |

* Initial register state after power-up.

MAX1610/MAX1611

two 7-bit registers is used. Tables 4 and 5 describe the data format for the configuration data.

Status information can be read from the MAX1611 using the SMBus read-byte protocol.  Figure 13 shows an example status read. Table 6 describes  the status information data format.

During shutdown (SMBSUS = 0 and SHDNB-0 = 0, or SMBSUS = 1 and SHDNB-1 = 0), the MAX1611 serial interface remains fully functional and can be used to set either the SHDNB-0 or SHDNB-1 bits in order to return the MAX1611 to its normal operational state.

Chopping the Lamp Current

Chopping the lamp current allows lower sustainable light levels without lamp flicker. Intensity is varied by control- ling the on-time duty cycle. Tying MINDAC to VL acti- vates a special mode, which allows the CCFL intensity to

be varied by turning the lamp on and off at a frequency faster than the eye can detect. The SS pin pulls to GND during off time and rises to 2.7V during on time. During on time, the CSAV pin regulates to REF / 8 (250mV). During off time, the BATT-to-LX power switch is forced off and the CC compensation node goes high imped- ance. Omit R5, R6, and C4 of the circuit in Figure 4.

In this mode, leave SS floating and increase the CC capacitance to 0.1µF. Also, insert a 330 resistor in series with D1 (Figure 4) to prevent the open-lamp detection cir- cuit from being tripped  by the repeated striking of the lamp. The SS pin will oscillate at the switching frequency divided by 1024 (283Hz with SYNC = REF). The intensity can be varied with the duty cycle at the SS pin. The duty cycle is set by the DAC in 3% increments. Duty cycle will vary with intensity. Full-scale yields a 100% duty cycle. DAC codes  00001, 00010, and 00011 all yield the

minimum 9% duty cycle. DAC code 00000 shuts off the lamp entirely (0% duty cycle).  Figure 14 shows the chopped waveforms with the DAC set to mid-scale.

Applications Information

Directly Regulating the Lamp Current The MAX1610/MAX1611 can directly regulate the CCFL current by tapping into the secondary of T1 (Figure 15). This allows more precise setting of the maximum lamp current (IL). The disadvantage  of this approach  is that the secondary-to-ground  voltage is twice that shown in Figure 4, increasing  the likelihood of the thermometer effect, where one end of the lamp is brighter than the other. Figure 15 uses the same component  values as Figure 4, except for R1, R40, D40, and D41. D40 and D41 are the same type of diode as D1. R1 should be

0.68 ±10% to set a peak current limit of about 735mA. Use a 107 ±1% resistor for R40 to set a lamp current of 5mARMS. This circuit accepts  a wide range of lamp types without component adjustments.

Component Suppliers Table 7 lists three different sources for C1. C1 requires a low dissipation factor to prevent overheating as energy is cycled  between C1 and the T1 magnetizing  induc-

tance in the Royer resonant tank. Table 8 lists suppliers for the high-voltage ballast capacitor, C2.

4V SS

VOLTAGE

0V BATT = 15V, MINDAC = VL, SS = OPEN, CC = 0.1 F,

C2 = 15pF, MID-SCALE SETTING, D/A VALUE = 10000

15V

T1

CENTER-TAP VOLTAGE

0V

500 s/div

Figure 14. Chopped Waveforms

VIN

16

+ C9

5

BATT

SS

MAX1610

MAX1611

12

VL

D3	C6

R7

14

BST	L1

C2

CCFL

10	6

T1

1	2	3    4	5

C4

6

CC

C3

4

SYNC

9

REF

R5

8

15

LX

OTP     10

11

CS

7

CSAV

13

C7

D2	D1

R3

C5

R4

R2

C1

Q1	Q2

R1

D40

R40

D41

MAX1610/MAX1611

C8 	MINDAC 	GND

R6

Figure 15. Directly Regulating the CCFL Current

MAX1610/MAX1611

Table 7. Capacitor C1 Supplier Information

| PART | SUPPLIER | LOCATION | PHONE | FAX | NOTES/CONTACT |
| --- | --- | --- | --- | --- | --- |
| SMD7.3104 | WIMA | Elmsford, NY | 914-347-2474 | 914-347-7230 | Dissipation factor (tan ) at 1kHz and 20°C ≤ 0.008. |
|  |  | Germany | (0621) 8785-0 | (0621) 8710457158 |  |
|  |  | Hong Kong | 5-70-11-51 | 58-06-84-74 |  |
| CHEV0025J104 | PACCOM Electronics | Redmond, WA | 206-883-9200 | 206-881-6959 | Dissipation factor (tan ) at 1kHz ≤ 0.002. |
| 4040N104M250 | NOVACAP | Valencia, CA | 805-295-5920 | 805-295-5928 | Dissipation factor (tan ) at 1kHz and 20°C ≤ 0.0015. |

Table 8. Capacitor C2 Supplier Information

| PART | SUPPLIER | LOCATION | PHONE | FAX |
| --- | --- | --- | --- | --- |
| 1808HA330KATMA | AVX/Kyocera | Olean, NY | 716-372-6611 | 716-372-6316 |
|  |  | Vancouver, WA | 206-696-2840 | 206-695-5836 |
|  |  | Germany | 08131 9004-0 | 08131 9004-44 |
|  |  | Hong Kong | 852-363-3303 | 852-765-8185 |
| GHM1040SL330J3K | Murata | Smyrna, GA | 404-436-1300 | 404-436-3030 |
|  |  | Germany | 49-911-66870 | 49-911-6687193 |
|  |  | Taiwan | 886-2-562-4218 | 886-2-536-6721 |
| 302C1812A330K | Metuchen Capacitors, Inc. | Old Bridge, NJ | 908-679-3366 | 908-679-3222 |
| 302R29N330K | Johanson Dielectrics | Sylmar, CA | 818-364-9800 | 818-364-6100 |

Chip Information

TRANSISTOR COUNT : 5457

Package Information

DIM

INCHES 	MILLIMETERS

D

0°-8° A

MIN

A 	0.053

A1 	0.004

B 	0.014

C 	0.007

E 	0.150

MAX

0.069

0.010

0.019

0.010

0.157

MIN

1.35

0.10

0.35

0.19

3.80

MAX

1.75

0.25

0.49

0.25

4.00

0.101mm

0.004in.

e 	0.050

1.27

e 	A1

B

H

C 	L 	L

0.228

0.016

0.244

0.050

5.80

0.40

6.20

1.27

INCHES	MILLIMETERS

Narrow SO

E 	H 	SMALL-OUTLINE PACKAGE

(0.150 in.)

DIM

D D D

PINS

8

14

16

MIN

0.189

0.337

0.386

MAX

0.197

0.344

0.394

MIN

4.80

8.55

9.80

MAX

5.00

8.75

10.00

21-0041A

MAX1610/MAX1611

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

20   	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  1996 Maxim Integrated  Products 	Printed USA 	  is a registered  trademark  of Maxim Integrated  Products.