19-1735; Rev 0; 7/01

1.5µA IQ, Step-Up DC-DC  Converters in Thin SOT23-5

General Description

The MAX1722/MAX1723/MAX1724 compact,  high-effi- ciency, step-up DC-DC converters are available in tiny, 5- pin thin SOT23 packages. They feature an extremely low

1.5µA quiescent supply current to ensure the highest pos- sible light-load efficiency. Optimized for operation from one to two alkaline or nickel-metal-hydride (NiMH) cells, or a single Li+ cell, these devices are ideal for applications where extremely low quiescent current and ultra-small size are critical.

Built-in synchronous rectification significantly improves efficiency and reduces size and cost by eliminating the need for an external Schottky diode. All three devices fea- ture a 0.5 N-channel power switch. The MAX1722/ MAX1724 also feature proprietary noise-reduction circuitry, which suppresses electromagnetic interference (EMI) caused by the inductor in many step-up applications. The family offers different combinations of fixed or adjustable outputs, shutdown, and EMI reduction (see Selector Guide).

Applications

Features

♦ Up to 90% Efficiency

♦ No External Diode or FETs Needed

♦ 1.5µA Quiescent Supply Current

♦ 0.1µA Logic-Controlled Shutdown

♦ ±1% Output Voltage Accuracy

♦ Fixed Output Voltage (MAX1724) or Adjustable

Output Voltage (MAX1722/MAX1723)

♦ Up to 150mA Output Current

♦ 0.8V to 5.5V Input Voltage Range

♦ 0.91V Guaranteed Startup (MAX1722/MAX1724)

♦ Internal EMI Suppression (MAX1722/MAX1724)

♦ Thin SOT23-5 Package (1.1mm max Height)

Ordering Information PART TEMP. RANGE PIN- TOP PACKAGE MARK

MAX1722EZK-T -40°C to +85°C 5 SOT23 ADQF

MAX1723EZK-T -40°C to +85°C 5 SOT23 ADQG

MAX1724EZK27-T -40°C to +85°C 5 SOT23 ADQH

MAX1724EZK30-T -40°C to +85°C 5 SOT23 ADQI

MAX1724EZK33-T -40°C to +85°C 5 SOT23 ADQJ

MAX1724EZK50-T -40°C to +85°C 5 SOT23 ADQK

Pagers

Remote Controls

Remote Wireless

Transmitters

Personal

Medical Devices

Digital Still Cameras

Single-Cell Battery- Powered Devices

Low-Power Hand-Held

Instruments

MP3 Players

Personal Digital

Assistants (PDA)

| Selector Guide PART | OUTPUT (V) | SHDN | LX DAMPING |
| --- | --- | --- | --- |
| MAX1722EZK | Adjustable | No | Yes |
| MAX1723EZK | Adjustable | Yes | No |
| MAX1724EZK27 | Fixed 2.7 | Yes | Yes |
| MAX1724EZK30 | Fixed 3.0 | Yes | Yes |
| MAX1724EZK33 | Fixed 3.3 | Yes | Yes |
| MAX1724EZK50 | Fixed 5.0 | Yes | Yes |

Typical Operating Circuit

10H

Pin  Configurations

TOP VIEW

IN

BATT

1 	5    LX

MAX1724

OUT

OUT

GND    2

MAX1722

OFF

SHDN

GND

UP TO 150mA

FB    3	4

OUT

MAX1722/MAX1723/MAX1724

THIN SOT23-5

Pin Configurations are continued at end of data sheet.

Maxim Integrated Products    1

ABSOLUTE MAXIMUM RATINGS

OUT, SHDN, BATT, LX to GND ................................-0.3V to +6V FB to GND ................................................-0.3V to (VOUT + 0.3V) OUT, LX Current.......................................................................1A Continuous Power Dissipation (TA = +70°C)

5-Pin Thin SOT23 (derate 7.1mW/°C above +70°C) ...571mW

Operating Temperature Range ...........................-40°C to +85°C Junction Temperature ......................................................+150°C Storage Temperature Range .............................-65°C to +150°C Lead Temperature (soldering,  10s) ................................ +300°C

MAX1722/MAX1723/MAX1724

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VBATT = 1.2V, VOUT = 3.3V (MAX1722/MAX1723),  VOUT = VOUT(NOM) (MAX1724),  SHDN = OUT, RL = , TA = 0°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Minimum Input Voltage |  | MAX1722/MAX1724 |  | 0.8 | V |
| Operating Input Voltage | VIN | TA = +25°C | MAX1722/MAX1724 | 0.91 5.5 | V |
|  |  |  | MAX1723 (Note 2) | 1.2 5.5 |  |
| Minimum Startup Input Voltage |  | TA = +25°C, RL = 3kΩ | MAX1722/MAX1724 | 0.83 0.91 | V |
|  |  |  | MAX1723 (Note 2) | 0.87 1.2 |  |
| Output Voltage | VOUT | MAX1724EZK27 | TA = +25°C | 2.673 2.7 2.727 | V |
|  |  |  | TA = 0°C to +85°C | 2.633 2.767 |  |
|  |  | MAX1724EZK30 | TA = +25°C | 2.970 3.0 3.030 |  |
|  |  |  | TA = 0°C to +85°C | 2.925 3.075 |  |
|  |  | MAX1724EZK33 | TA = +25°C | 3.267 3.3 3.333 |  |
|  |  |  | TA = 0°C to +85°C | 3.218 3.383 |  |
|  |  | MAX1724EZK50 | TA = +25°C | 4.950 5.0 5.050 |  |
|  |  |  | TA = 0°C to +85°C | 4.875 5.125 |  |
| Output Voltage Range | VOUT | MAX1722/MAX1723 |  | 2 5.5 | V |
| Feedback Voltage | VFB | MAX1722/MAX1723 | TA = +25°C | 1.223 1.235 1.247 | V |
|  |  |  | TA = 0°C to +85°C | 1.210 1.260 |  |
| Feedback Bias Current | IFB | MAX1722/MAX1723 | TA = +25°C | 1.5 20 | nA |
|  |  |  | TA = +85°C | 2.2 |  |
| N-Channel On-Resistance | RDS(ON) | VOUT forced to 3.3V |  | 0.5 1.0 | Ω |
| P-Channel On-Resistance | RDS(ON) | VOUT forced to 3.3V |  | 1.0 2.0 | Ω |
| N-Channel Switch Current Limit | ILIM | VOUT forced to 3.3V |  | 400 500 600 | mA |
| Switch Maximum On-Time | tON |  |  | 3.5 5 6.5 | µs |
| Synchronous Rectifier Zero- Crossing Current |  | VOUT forced to 3.3V |  | 5 20 35 | mA |
| Quiescent Current into OUT |  | (Notes 3, 4) |  | 1.5 3.6 | µA |
| Shutdown Current into OUT |  | MAX1723/MAX1724 (Notes 3, 4) | TA = +25°C | 0.01 0.5 | µA |
|  |  |  | TA = +85°C | 0.1 |  |
| Quiescent Current into BATT |  | MAX1722/MAX1724 (Note 4) | TA = +25°C | 0.001 0.5 | µA |
|  |  |  | TA = +85°C | 0.01 |  |

MAX1722/MAX1723/MAX1724

ELECTRICAL CHARACTERISTICS  (continued)

(VBATT = 1.2V, VOUT = 3.3V (MAX1722/MAX1723),  VOUT = VOUT(NOM) (MAX1724),  SHDN = OUT, RL = , TA = 0°C to +85°C, unless otherwise noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Shutdown Current into BATT |  | MAX1724 (Note 4) | TA = +25°C | 0.001 0.5 | µA |
|  |  |  | TA = +85°C | 0.01 |  |
| SHDN Voltage Threshold | VIL | MAX1723/MAX1724 |  | 75 400 | mV |
|  | VIH | MAX1723/MAX1724 |  | 500 800 |  |
| SHDN Input Bias Current |  | MAX1723/MAX1724, VSHDN = 5.5V | TA = +25°C | 2 100 | nA |
|  |  |  | TA = +85°C | 7 |  |

ELECTRICAL CHARACTERISTICS

(VBATT = 1.2V, VOUT = 3.3V (MAX1722/MAX1723),  VOUT = VOUT(NOM) (MAX1724),  SHDN = OUT, RL = , TA = -40°C to +85°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Output Voltage | VOUT | MAX1724EZK27 | 2.633 2.767 | V |
|  |  | MAX1724EZK30 | 2.925 3.075 |  |
|  |  | MAX1724EZK33 | 3.218 3.383 |  |
|  |  | MAX1724EZK50 | 4.875 5.125 |  |
| Output Voltage Range | VOUT | MAX1722/MAX1723 | 2 5.5 | V |
| Feedback Voltage | VFB | MAX1722/MAX1723 | 1.200 1.270 | V |
| N-Channel On-Resistance | RDS(ON) | VOUT forced to 3.3V | 1.0 | Ω |
| P-Channel On-Resistance | RDS(ON) | VOUT forced to 3.3V | 2.0 | Ω |
| N-Channel Switch Current Limit | ILIM | VOUT forced to 3.3V | 400 620 | mA |
| Switch Maximum On-Time | tON |  | 3.5 6.5 | µs |
| Synchronous Rectifier Zero- Crossing Current |  | VOUT forced to 3.3V | 5 35 | mA |
| Quiescent Current into OUT |  | (Notes 3,4) | 3.6 | µA |
| SHDN Voltage Threshold | VIL | MAX1723/MAX1724 | 75 | mV |
|  | VIH | MAX1723/MAX1724 | 800 |  |

Note 1: Limits are 100% production  tested at TA = +25°C. Limits over the operating temperature  range are guaranteed  by design.

Note 2: Guaranteed with the addition of a Schottky MBR0520L external diode between LX and OUT when using the MAX1723 with only one cell, and assumes a 0.3V voltage drop across the Schottky diode (see Figure 3).

Note 3: Supply current is measured with an ammeter between the output and OUT pin. This current  correlates directly with actual battery supply current, but is reduced  in value according to the step-up ratio and efficiency.

Note 4: VOUT forced to the following conditions  to inhibit switching: VOUT = 1.05 ✕ VOUT(NOM) (MAX1724), VOUT = 3.465V

(MAX1722/MAX1723).

Typical Operating Characteristics (Figure 3 (MAX1723), Figure 7 (MAX1722), Figure 8 (MAX1724), VBATT = VIN = 1.5V, L = 10µH, CIN = 10µF, COUT = 10µF, TA = +25°C, unless otherwise noted.)

100

EFFICIENCY vs. LOAD CURRENT (VOUT = 5.0V)

VIN = 3.3V	VIN = 4.0V VIN = 2.0V

MAX1722 toc01

100

EFFICIENCY vs. LOAD CURRENT (VOUT = 3.3V)

VIN = 2.5V

MAX1722 toc02

100

EFFICIENCY vs. LOAD CURRENT (VOUT = 2.5V)

90 	90 	VIN = 2.0V

80	80

90

80 	VIN = 1.5V

VIN = 2.0V

MAX1722 toc03

70	70	70

EFFICIENCY (%)

EFFICIENCY (%)

60 	VIN = 1.0V

L = DO1606

50

60

VIN = 1.5V

50

VIN

= 1.0V	60

L = DO1606

50

VIN = 1.0V 	L = DO1606

200

160

120

80

40

0

0.01	0.1	1	10	100	1000

LOAD CURRENT (mA)

MAXIMUM OUTPUT CURRENT

MAX1722 toc04

vs. INPUT VOLTAGE

2.4

2.2

2.0

STARTUP VOLTAGE (V)

1.8

1.6

1.4

1.2

1.0

0.8

0.6

0.01	0.1	1	10	100	1000

LOAD CURRENT (mA)

STARTUP VOLTAGE

vs. LOAD CURRENT

RESISTIVE LOAD VOUT = 5.0V

0.01	0.1	1	10	100	1000

LOAD CURRENT (mA)

QUIESCENT CURRENT INTO OUT

vs. OUTPUT VOLTAGE

2.0

MAX1722 toc05

1.8 	NO LOAD

MAX1722 toc06

1.6

1.4

QUIESCENT CURRENT (A)

1.2

1.0

0.8

0.6

0.4

0.2

0

| 0 1 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| VOUT = 2.5V |  |  |  |  |
|  |  |  | VOUT = 5.0V |  |
|  |  | VOUT = 3.3V |  |  |

3	4	5

0.01	0.1	1

10	100

1.0

1.5

2.0    2.5    3.0

3.5    4.0    4.5    5.0    5.5

INPUT VOLTAGE (V)

LOAD CURRENT (mA)

OUTPUT VOLTAGE (V)

1.2

STARTUP VOLTAGE vs.

TEMPERATURE

MAX1722 toc07

SWITCHING WAVEFORMS

1.0

0.8

0.6

0.4

STARTUP VOLTAGE (V)

0.2

NO LOAD

ILX

500mA/div

MAX1722 toc08

VOUT

50mV/div

VLX

2V/div

0

-40

-15	10

35	60	85

1s/div

TEMPERATURE (C)

MAX1722/MAX1723/MAX1724

EFFICIENCY (%)

IOUT(MAX) (mA)

IOUT = 50mA, VOUT = 5.0V, VIN = 3.3V

Typical Operating Characteristics (continued) (Figure 3 (MAX1723), Figure 7 (MAX1722), Figure 8 (MAX1724), VBATT = VIN = 1.5V, L = 10µH, CIN = 10µF, COUT = 10µF, TA = +25°C, unless otherwise noted.)

MAX1722 toc09

MAX1722 toc10

LOAD-TRANSIENT RESPONSE

SHUTDOWN RESPONSE

3.3V

50mA

0

5V	VOUT

A	2V/div

0

2V

B	VSHDN

1V/div

0

A: VOUT, 50mV/div

B: IOUT, 20mA/div

200s/div

1ms/div

VIN = 3.3V, VOUT = 5.0V, ROUT = 100

0.8

SHUTDOWN INPUT THRESHOLD

vs. TEMPERATURE

MAX1722 toc11

MAX1722/MAX1723/MAX1724

0.7

0.6

0.5

0.4

0.3

0.2

SHUTDOWN THRESHOLD (mV)

0.1

0

| -40 -15 10 35 60 85 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | RISING EDGE |  |  |
|  | FALLING EDGE |  |  |  |

TEMPERATURE (C)

Pin  Description

| PIN |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX1722 | MAX1723 | MAX1724 |  |  |
| 1 | — | 1 | BATT | Battery Input and Damping Switch Connection |
| — | 1 | 3 | SHDN | Shutdown Input. Drive high for normal operation. Drive low for shutdown. |
| 2 | 2 | 2 | GND | Ground |
| 3 | 3 | — | FB | Feedback Input to Set Output Voltage. Use a resistor-divider network to adjust the output voltage. See Setting the Output Voltage section. |
| 4 | 4 | 4 | OUT | Power Output. OUT also provides bootstrap power to the IC. |
| 5 | 5 | 5 | LX | Internal N-channel MOSFET Switch Drain and P-Channel Synchronous Rectifier Drain |

OUT

MAX1723

STARTUP CIRCUITRY

ZERO- CROSSING DETECTOR

P

SHDN

CONTROL LOGIC

DRIVER	LX

FB

ERROR

COMPARATOR	N

1.235V  REFERENCE

CURRENT LIMIT

GND

Figure 1. MAX1723 Simplified Functional Diagram

Detailed Description

MAX1722/MAX1723/MAX1724

The MAX1722/MAX1723/MAX1724 compact,  high-effi- ciency, step-up DC-DC converters are guaranteed to start up with voltages as low as 0.91V and operate with an input voltage down to 0.8V. Consuming only 1.5µA of quiescent current, these devices include a built-in syn- chronous rectifier that reduces cost by eliminating the need for an external diode and improves overall efficien- cy by minimizing losses in the circuit (see Synchronous Rectification section). The MAX1722/MAX1724 feature a clamp circuit that reduces EMI due to inductor ringing. The MAX1723/MAX1724 feature an active-low shutdown that reduces quiescent supply current to 0.1µA. The MAX1722/MAX1723 have an adjustable output voltage, while the MAX1724 is available with four fixed-output voltage options (see Selector Guide). Figure 1 is the MAX1723 simplified functional diagram and Figure 2 is the MAX1724 simplified functional diagram.

PFM  Control Scheme A forced discontinuous, current-limited, pulse-frequency- modulation (PFM) control scheme is a key feature of the

MAX1722/MAX1723/MAX1724. This scheme provides ultra-low quiescent current and high efficiency over a wide output current range. There is no oscillator; the inductor current is limited by the 0.5A N-channel cur- rent limit or by the 5µs switch maximum on-time. Following each on cycle,  the inductor  current must ramp to zero before another cycle may start. When the error comparator  senses that the output has fallen below the regulation threshold, another cycle begins.

Synchronous Rectification The internal synchronous rectifier eliminates the need for an external Schottky diode, thus reducing cost and board space.  While the inductor  discharges,  the P- channel MOSFET turns on and shunts the MOSFET body diode. As a result, the rectifier voltage drop is sig- nificantly reduced,  improving efficiency without the addition of external components.

Low-Voltage Startup Circuit The MAX1722/MAX1723/MAX1724 contain a low-volt- age startup circuit to control DC-DC operation until the output voltage exceeds 1.5V (typ). The minimum start-

BATT

DAMPING SWITCH

OUT

MAX1724

STARTUP CIRCUITRY

ZERO- CROSSING DETECTOR

R2

P

SHDN

CONTROL LOGIC

DRIVER	LX

ERROR

COMPARATOR	N

R1

1.235V  REFERENCE

CURRENT LIMIT

GND

Figure 2. MAX1724 Simplified Functional Diagram

1.2V TO VOUT

10F

SHDN

10H

LX OUT

D1

VOUT = 3.6V

R2

lacks a BATT pin; therefore, this circuit is powered through the OUT pin. Adding a Schottky diode in paral- lel with the P-channel synchronous rectifier allows for startup voltages as low as 1.2V for the MAX1723 (Figure 3). The external Schottky diode is not needed for input voltages greater than 1.8V. Once started, the output maintains the load as the battery voltage decreases below the startup voltage.

MAX1723

GND 	FB

Figure 3. MAX1723 Single-Cell Operation

2.37M

R1

1.24M

10F

Shutdown (MAX1723/MAX1724) The MAX1723/MAX1724 enter shutdown when the SHDN pin is driven low. During shutdown, the body diode of the P-channel MOSFET allows current to flow from the battery to the output. VOUT falls to approxi- mately VIN - 0.6V and LX remains high impedance. Shutdown can be pulled as high as 6V, regardless of the voltage at BATT or OUT. For normal operation, con-

up voltage is a function of load current (see Typical

MAX1722/MAX1723/MAX1724

Operating Characteristics). This circuit is powered from the BATT pin for the MAX1722/MAX1724, guaranteeing startup at input voltages as low as 0.91V. The MAX1723

nect SHDN to the input.

VOUT

VIN

MAX1722

MAX1724

PDRV

OUT

P

BATT

TIMING CIRCUIT

DAMPING SWITCH

DAMP

LX

NDRV	N

GND

Figure 4. Simplified Diagram of Damping Switch

1V/div

1V/div

1s/div

Figure 5. LX Ringing Without Damping Switch (MAX1723)

1s/div

Figure 6. LX Ringing With Damping Switch (MAX1722/MAX1724)

BATT/Damping Switch

(MAX1722/MAX1724) The MAX1722/MAX1724 include an internal damping switch (Figure 4) to minimize ringing at LX and reduce EMI. When the energy in the inductor is insufficient to supply  current to the output, the capacitance  and inductance  at LX form a resonant circuit that causes ringing. The damping switch supplies a path to quickly dissipate this energy, suppressing  the ringing at LX. This does not reduce  the output ripple,  but does

Design Procedure

Setting the Output Voltage

(MAX1722/MAX1723) The output voltage can be adjusted from 2V to 5.5V using external resistors R1 and R2 (Figure 7). Since FB leakage is 20nA (max), select feedback  resistor R1 in the 100k to 1M range. Calculate R2 as follows:

 VOUT	

reduce EMI with minimal impact on efficiency. Figures

MAX1722/MAX1723/MAX1724

5 and 6 show the LX node voltage waveform without and with the damping switch, respectively.

R2  

where VFB = 1.235V.

R1 



VFB

 1



INPUT

0.8V TO VOUT	10H

For maximum output current, choose the inductor value so that the controller reaches the current-limit before the maximum on-time is triggered:

VBATT tON(MAX)

10F

BATT

LX

OUT

OUTPUT

2V TO 5.5V

L 

ILIM

MAX1722

R2 	10F

where the maximum on-time is typically 5µs, and the current limit (ILIM) is typically 500mA (see Electrical Characteristics table).

FB

R1

GND

For larger inductor values, determine the peak inductor current (IPEAK) by:

Figure 7. Adjustable  Output Circuit

Inductor Selection

INPUT

IPEAK

VBATT tON(MAX)



L

The control  scheme  of the MAX1722/MAX1723/ MAX1724 permits flexibility in choosing an inductor. A

0.8V TO VOUT	10H

10µH inductor value performs well in most applications. Smaller inductance values typically offer smaller physi- cal size for a given series resistance, allowing the smallest overall circuit dimensions. Circuits using larger inductance  values may start up at lower battery volt- ages, provide higher efficiency, and exhibit less ripple,

C1

10F

BATT

MAX1724

LX OUT

OUTPUT VOUT (NOM)

C2

10F

but they may reduce the maximum output current. This

ON

occurs when the inductance is sufficiently large to pre- vent the maximum current limit (ILIM) from being reached  before the maximum on-time (tON(MAX)) expires.

OFF

SHDN

GND

Table 1. Suggested Inductors and

| Suppliers MANUFACTURER | INDUCTOR | PHONE WEBSITE |
| --- | --- | --- |
| Coilcraft | DO1608 Series DO1606 Series | 847-639-2361 www.coilcraft.com |
| Murata | LQH4C Series | 770-436-1300 www.murata.com |
| Sumida | CDRH4D18 Series CR32 Series CMD4D06 Series | 847-545-6700 www.sumida.com |
| Sumitomo/ Daidoo Electronics | CXLD140 Series | +81 (06) 6355-5733 www.daidoo.co.jp |
| Toko | 3DF Type D412F Type | 847-297-0070 www.toko.com |

Figure 8. MAX1724 Standard Application Circuit

The inductor’s incremental saturation current rating should be greater than the peak switching current. However, it is generally acceptable  to bias the inductor into saturation by as much as 20%, although this will slightly reduce effi- ciency. Table 1 lists suggested inductors and suppliers.

Maximum Output Current The maximum output current depends on the peak induc- tor current, the input voltage, the output voltage, and the overall efficiency ():

1

I	    I

 VBATT  

	

OUT(MAX)

MAX1722/MAX1723/MAX1724

2 PEAK  VOUT 

Table 2. Suggested Surface-Mount Capacitors and Manufacturers (C1 and C2)

| MANUFACTURER | CAPACITOR VALUE | DESCRIPTION | PHONE WEBSITE |
| --- | --- | --- | --- |
| AVX | 1F to 10F | X7R Ceramic | 843-448-9411 www.avxcorp.com |
|  | 10F to 330F | TAJ Tantalum Series TPS Tantalum Series |  |
| Kemet | 1F to 22F | X5R/X7R Ceramic | 864-963-6300 www.kemet.com |
|  | 10F to 330F | T494 Tantalum Series |  |
|  | 68F to 330F | T520 Tantalum Series |  |
| Sanyo | 33F to 330F | TPC Polymer Series | 408-749-9714 www.secc.co.jp |
| Taiyo Yuden | 33F to 330F | X5R/X7R Ceramic | 800-368-2496 www.t-yuden.org |
| TDK | 1F to 10F | X7R Ceramic | 847-803-6100 www.tdk.com |
| Vishay Sprague | 10F to 330F | 594D Tantalum Series 595D Tantalum Series | 203-452-5664 www.vishay.com |

For most applications, the peak inductor current equals the current limit. However, for applications using large inductor values or low input voltages, the maximum on- time limits the peak inductor current (see Inductor Selection section).

Capacitor Selection Choose input and output capacitors to supply the input and output peak currents with acceptable  voltage rip- ple. The input filter capacitor  (CIN) reduces peak cur- rents drawn from the battery and improves efficiency. Low equivalent series resistance (ESR) capacitors are recommended.  Ceramic capacitors  have the lowest ESR, but low ESR tantalum or polymer capacitors offer a good balance between cost and performance.

Output voltage ripple has two components: variations in the charge stored in the output capacitor with each LX pulse, and the voltage drop across the capacitor’s ESR caused by the current into and out of the capaci- tor:

VRIPPLE  VRIPPLEC  VRIPPLEESR

VRIPPLEESR  IPEAK RESRCOUT

where IPEAK is the peak inductor current (see Inductor Selection section). For ceramic capacitors,  the output voltage ripple is typically dominated by VRIPPLE(C). For example, a 10µF ceramic capacitor and a 10µH induc- tor typically provide 75mV of output ripple when step- ping up from 3.3V to 5V at 50mA. Low input-to-output voltage differences (i.e. two cells to 3.3V) require high- er output capacitor values.

Capacitance  and ESR variation of temperature should be considered  for best performance  in applications with wide operating temperature ranges. Table 2 lists suggested capacitors and suppliers.

PC Board Layout Considerations

Careful PC board layout is important for minimizing ground bounce and noise. Keep the IC’s GND pin and the ground leads of the input and output capacitors less than 0.2in (5mm) apart using a ground plane. In addition, 	keep 	all	connections 	to	FB (MAX1722/MAX1723 only) and LX as short as possible.

Chip Information

TRANSISTOR COUNT: 863

1 	L

V	



I	- I	

RIPPLEC

2   V

- V	C

  PEAK2

OUT2

   OUT	BATT 

MAX1722/MAX1723/MAX1724

OUT 

Pin  Configurations (continued)

TOP VIEW

SHDN

1 	5    LX

BATT

1 	5    LX

GND   2

MAX1723

GND   2

MAX1724

FB    3	4

OUT

SHDN

3 	4    OUT

THIN SOT23-5

THIN SOT23-5

MAX1722/MAX1723/MAX1724

Package Information

THIN SOT23.EPS

MAX1722/MAX1723/MAX1724

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

12    	Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600

©  2001 Maxim Integrated  Products 	Printed USA 	is a registered  trademark of Maxim Integrated  Products.