19-1295; Rev 7; 4/05

UCSP, Single-Supply, Low-Noise, Low-Distortion, Rail-to-Rail Op Amps

General Description

The MAX4249–MAX4257 low-noise, low-distortion oper- ational amplifiers offer rail-to-rail outputs and single- supply operation down to 2.4V. They draw 400µA of quiescent  supply current per amplifier while featuring ultra-low distortion (0.0002% THD), as well as low input voltage-noise  density  (7.9nV/√Hz) and  low input current-noise density (0.5fA/√Hz). These features make the devices an ideal choice for portable/battery-powered applications that require low distortion and/or low noise.

For additional  power  conservation,  the MAX4249/ MAX4251/MAX4253/MAX4256 offer a low-power shut- down mode that reduces  supply current to 0.5µA and puts the amplifiers’  outputs into a high-impedance state. The MAX4249-MAX4257’s outputs swing rail-to- rail and their input common-mode  voltage range includes  ground.  The MAX4250–MAX4254 are unity- gain stable with a gain-bandwidth  product  of 3MHz. The MAX4249/MAX4255/MAX4256/MAX4257 are inter- nally compensated for gains of 10V/V or greater with a gain-bandwidth  product of 22MHz. The single MAX4250/ MAX4255 are available in space-saving  5-pin SOT23 packages. The MAX4252 is available in an 8-bump chip- scale package (UCSP™) and the MAX4253 is available in a 10-bump UCSP. The MAX4250AAUK comes in a 5-pin SOT23 package  and is specified  for operation over the automotive (-40°C to +125°C) temperature range.

Applications

Wireless Communications Devices

PA Control

Portable/Battery-Powered Equipment

Medical Instrumentation

ADC Buffers

Digital Scales/Strain Gauges

Features

♦ Available in Space-Saving UCSP, SOT23, and

µMAX® Packages

♦ Low Distortion: 0.0002% THD (1kΩ load)

♦ 400µA Quiescent Supply Current per Amplifier

♦ Single-Supply Operation from 2.4V to 5.5V

♦ Input Common-Mode Voltage Range Includes

Ground

♦ Outputs Swing Within 8mV of Rails with a 10kΩ

Load

♦ 3MHz GBW Product, Unity-Gain Stable

(MAX4250–MAX4254)

22MHz GBW Product, Stable with AV ≥ 10V/V (MAX4249/MAX4255/MAX4256/MAX4257)

♦ Excellent DC Characteristics

VOS = 70µV IBIAS = 1pA

Large-Signal Voltage Gain = 116dB

♦ Low-Power Shutdown Mode

Reduces Supply Current to 0.5µA

Places Outputs in a High-Impedance State

♦ 400pF Capacitive-Load Handling Capability

Ordering Information

PART TEMP RANGE PIN- TOP PACKAGE MARK

MAX4249ESD -40°C to +85°C 14 SO —

MAX4249EUB -40°C to +85°C 10 µMAX —

MAX4250EUK-T -40°C to +85°C 5 SOT23-5 ACCI

MAX4250AAUK-T -40°C to +125°C 5 SOT23-5 AEYJ

Ordering Information continued at end of data sheet. Selector Guide appears at end of data sheet.

Pin  Configurations

TOP VIEW

(BUMPS ON BOTTOM)

1	2	3

A1	A2	A3	A4

A	OUTA	VDD 	OUTB

OUTB

INB-

INB+

SHDNB

B	INA-

MAX4252

INB-

B1

VDD

MAX4253

B4

VSS

C1	C2	C3	C4

C

INA+

VSS

INB+

OUTA

INA-

INA+

SHDNA

UCSP

Pin Configurations continued at end of data sheet.

UCSP

MAX4249–MAX4257

UCSP is a trademark and µMAX is a registered trademark of Maxim Integrated Products, Inc.

ABSOLUTE MAXIMUM RATINGS

Power-Supply Voltage (VDD to VSS) ......................+6.0V to -0.3V Analog Input Voltage (IN_+, IN_-)....(VDD + 0.3V) to (VSS - 0.3V) SHDN Input Voltage ......................................6.0V to (VSS - 0.3V) Output Short-Circuit  Duration to Either Supply ..........Continuous Continuous Power Dissipation  (TA = +70°C)

5-Pin SOT23 (derate 7.1mW/°C above +70°C)...........571mW

8-Bump UCSP (derate 4.7mW/°C above +70°C)........379mW

8-Pin µMAX (derate 4.5mW/°C above +70°C) ............362mW

8-Pin SO (derate 5.88mW/°C above +70°C)...............471mW

10-Bump UCSP (derate 6.1mW/°C above +70°C) ......484mW

10-Pin µMAX (derate 5.6mW/°C above +70°C) ...........444mW

14-Pin SO (derate 8.33mW/°C above +70°C)..............667mW Operating  Temperature  Range ...........................-40°C to +85°C MAX4250AAUK  .............................................-40°C to +125°C Junction Temperature ......................................................+150°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

Bump Temperature  (soldering)  (Note 1)

Infrared (15s) ................................................................+220°C Vapor Phase (60s) ........................................................+215°C

MAX4249–MAX4257

Note 1: 	This device  is constructed using a unique set of packaging techniques that impose a limit on the thermal profile the device can be exposed  to during  board-level solder attach and rework. This limit permits  only the use of the solder profiles rec- ommended in the industry-standard specification, JEDEC 020A,  paragraph 7.6, Table  3 for IR/VPR and  Convection Reflow. Preheating is required.  Hand or wave soldering  is not allowed.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VDD = 5V, VSS = 0, VCM = 0, VOUT = VDD/2, RL tied to VDD/2, SHDN = VDD, TA = TMIN to TMAX, unless otherwise noted. Typical val- ues are at TA = +25°C.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS |  |  |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage Range | VDD | (Note 4) |  |  |  |  | 2.4 5.5 | V |
| Quiescent Supply Current Per Amplifier | IQ | Normal mode | VDD = 3V |  |  |  | 400 | µA |
|  |  |  | VDD = 5V |  | E temperature |  | 420 575 |  |
|  |  |  |  |  | MAX4250AAUK |  | 675 |  |
|  |  |  | VDD = 5V, UCSP only |  |  |  | 420 655 |  |
|  |  | Shutdown mode (SHDN = VSS) (Note 2) |  |  |  |  | 0.5 1.5 |  |
| Input Offset Voltage (Note 5) | VOS | E temperature |  |  |  |  | ±0.07 ±0.75 | mV |
|  |  | MAX4250AAUK |  |  |  |  | ±1.85 |  |
| Input Offset Voltage Tempco | TCVOS |  |  |  |  |  | 0.3 | µV/°C |
| Input Bias Current | IB | (Note 6) |  | E temperature |  |  | ±1 ±100 | pA |
|  |  |  |  | MAX4250AAUK |  |  | ±10 | nA |
| Input Offset Current | IOS | (Note 6) |  | E temperature |  |  | ±1 ±100 | pA |
|  |  |  |  | MAX4250AAUK |  |  | ±10 | nA |
| Differential Input Resistance | RIN |  |  |  |  |  | 1000 | GΩ |
| Input Common-Mode Voltage Range | VCM | Guaranteed by CMRR test |  |  |  | E temperature | -0.2 VDD -1.1 | V |
|  |  |  |  |  |  | MAX4250AAUK | 0 VDD -1.1 |  |
| Common-Mode Rejection Ratio | CMRR | VSS - 0.2V ≤ VCM ≤ VDD - 1.1V |  |  |  | E temperature | 70 115 | dB |
|  |  |  |  |  |  | MAX4250AAUK | 68 |  |
| Power-Supply Rejection Ratio | PSRR | VDD = 2.4V to 5.5V |  |  |  | E temperature | 75 100 | dB |
|  |  |  |  |  |  | MAX4250AAUK | 72 |  |

MAX4249–MAX4257

ELECTRICAL CHARACTERISTICS (continued)

(VDD = 5V, VSS = 0, VCM = 0, VOUT = VDD/2, RL tied to VDD/2, SHDN = VDD, TA = TMIN to TMAX, unless otherwise noted. Typical val- ues are at TA = +25°C.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Large-Signal Voltage Gain | AV | RL = 10kΩ to VDD/2; VOUT = 25mV to VDD - 4.97V | E temperature |  | 80 116 | dB |
|  |  |  | MAX4250AAUK |  | 77 |  |
|  |  | RL = 1kΩ to VDD/2; VOUT = 150V to VDD - 4.75V | E temperature |  | 80 112 |  |
|  |  |  | MAX4250AAUK |  | 77 |  |
| Output Voltage Swing | VOUT | ¦VIN+ - VIN-¦ ≥ 10mV; RL = 10kΩ to VDD/2 | VDD - VOH | E | 8 25 | mV |
|  |  |  |  | A | 30 |  |
|  |  |  | VOL - VSS | E | 7 20 |  |
|  |  |  |  | A | 25 |  |
| Output Voltage Swing | VOUT | ¦VIN+ - VIN-¦ ≥ 10mV, RL = 1kΩ to VDD/2 | VDD - VOH | E | 77 200 | mV |
|  |  |  |  | A | 225 |  |
|  |  |  | VOL - VSS | E | 47 100 |  |
|  |  |  |  | A | 125 |  |
| Output Short-Circuit Current | ISC |  |  |  | 68 | mA |
| Output Leakage Current | ILEAK | Shutdown mode (SHDN = VSS), VOUT = VSS to VDD (Note 2) |  |  | 0.001 1.0 | µA |
| SHDN Logic Low | VIL | (Note 2) |  |  | 0.2 X VDD | V |
| SHDN Logic High | VIH | (Note 2) |  |  | 0.8 X VDD | V |
| SHDN Input Current | IIL/IIH | SHDN = VSS = VDD (Note 2) |  |  | 0.5 1.5 | µA |
| Input Capacitance |  |  |  |  | 11 | pF |
| Gain-Bandwidth Product | GBW | MAX4250–MAX4254 |  |  | 3 | MHz |
|  |  | MAX4249/MAX4255/MAX4256/MAX4257 |  |  | 22 |  |
| Slew Rate | SR | MAX4250–MAX4254 |  |  | 0.3 | V/µs |
|  |  | MAX4249/MAX4255/MAX4256/MAX4257 |  |  | 2.1 |  |
| Peak-to-Peak Input-Noise Voltage | enP-P | f = 0.1Hz to 10Hz |  |  | 760 | nVP-P |
| Input Voltage-Noise Density | en | f = 10Hz |  |  | 27 | nV/√Hz |
|  |  | f = 1kHz |  |  | 8.9 |  |
|  |  | f = 30kHz |  |  | 7.9 |  |
| Input Current-Noise Density | in | f = 1kHz |  |  | 0.5 | fA/√Hz |

MAX4249–MAX4257

ELECTRICAL CHARACTERISTICS (continued)

(VDD = 5V, VSS = 0, VCM = 0, VOUT = VDD/2, RL tied to VDD/2, SHDN = VDD, TA = TMIN to TMAX, unless otherwise noted. Typical val- ues are at TA = +25°C.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Total Harmonic Distortion Plus Noise | THD+N | MAX4250–MAX4254 AV = 1V/V, VOUT = 2VP-P, RL = 1kΩ to GND (Note 7) |  | f = 1kHz | 0.0004 | % |
|  |  |  |  | f = 20kHz | 0.006 |  |
|  |  | MAX4249/MAX4255/ MAX4256/MAX4257 AV = 1V/V, VOUT = 2VP-P, RL = 1kΩ to GND (Note 7) |  | f = 1kHz | 0.0012 |  |
|  |  |  |  | f = 20kHz | 0.007 |  |
| Capacitive-Load Stability |  | No sustained oscillations |  |  | 400 | pF |
| Gain Margin | GM | MAX4250–MAX4254, AV = 1V/V |  |  | 10 | dB |
|  |  | MAX4249/MAX4255/MAX4256/MAX4257, AV = 10V/V |  |  | 12.5 |  |
| Phase Margin | ΦM | MAX4250–MAX4254, AV = 1V/V |  |  | 74 | Degrees |
|  |  | MAX4249/MAX4255/MAX4256/MAX4257, AV = 10V/V |  |  | 68 |  |
| Settling Time |  | To 0.01%, VOUT = 2V step | MAX4250–MAX4254 |  | 6.7 | µs |
|  |  |  | MAX4249/MAX4255/ MAX4256/MAX4257 |  | 1.6 |  |
| Delay Time to Shutdown | tSH | IVDD = 5% of normal operation | MAX4251/MAX4253 |  | 0.8 | µs |
|  |  |  | MAX4249/MAX4256 |  | 1.2 |  |
| Delay Time to Enable | tEN | VOUT = 2.5V, VOUT settles to 0.1% | MAX4251/MAX4253 |  | 8 | µs |
|  |  |  | MAX4249/MAX4256 |  | 3.5 |  |
| Power-Up Delay Time | tPU | VDD = 0 to 5V step, VOUT stable to 0.1% |  |  | 6 | µs |

Note 2: SHDN is available on the MAX4249/MAX4251/MAX4253/MAX4256 only.

Note 3: All device specifications are 100% tested at TA = +25°C. Limits over temperature are guaranteed by design.

Note 4: Guaranteed by the PSRR test.

Note 5: Offset voltage prior to reflow on the UCSP.

Note 6: Guaranteed by design.

Note 7: Lowpass-filter bandwidth is 22kHz for f = 1kHz and 80kHz for f = 20kHz. Noise floor of test equipment = 10nV/√Hz.

Typical Operating Characteristics

(VDD =  5V, VSS =  0, VCM =  VOUT =  VDD/2, input  noise  floor  of test equipment  =10nV/√Hz  for all distortion  measurements, TA = +25°C, unless otherwise noted.)

MAX4251/MAX4256

INPUT OFFSET VOLTAGE DISTRIBUTION

MAX4249-57 TOC01

40

400 UNITS

250

OFFSET VOLTAGE

vs. TEMPERATURE

| 200 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | VDD = 3V VDD = 5V |  |  |

INPUT OFFSET VOLTAGE

MAX4249-57 TOC03

vs. INPUT COMMON-MODE  VOLTAGE

35	VCM = 0

TA = +25°C

30

25

20

NUMBER OF UNITS

15

10

5

-95

-75

-55

-35

-13

7

28

49

69

90

110

131

152

172

192

0

200

150

100

VOS (μV)

50

0

-50

-100

-150

-200

-250

| -40 VCM = 0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

-20	0

20	40	60	80

MAX4249-57 TOC02

INPUT OFFSET VOLTAGE (μV)

150

100

50

0

-50

-0.5

0.5

1.5

2.5	3.5	4.5

VOS (μV)

TEMPERATURE (°C)

INPUT COMMON-MODE VOLTAGE (V)

0.6

0.5

0.4

0.3

OUTPUT VOLTAGE (V)

0.2

0.1

OUTPUT VOLTAGE

VDD = 3V OR 5V VDIFF = ±10mV

VDD - VOH

VOL

MAX4249-57 TOC04

0.10

0.09

0.08

VDD - VOH (V)

0.06

0.05

0.03

0.02

OUTPUT VOLTAGE SWING (VOH)

MAX4249-57 TOC05

RL = 1kΩ

R  = 10k

0.06

0.05

0.04

VOL (V)

0.03

0.02

0.01

OUTPUT VOLTAGE SWING (VOL)

MAX4249 -57TOC06

RL = 1kΩ

R  = 10kΩ

L	Ω 	L

0

0    1    2

3    4    5    6    7    8    9   10

0

-40

-20	0

20	40

60	80

0

-40

-20	0

20	40

60	80

OUTPUT LOAD CURRENT (mA)

LARGE-SIGNAL VOLTAGE GAIN

TEMPERATURE (°C)

MAX4249-57 TOC08

LARGE-SIGNAL VOLTAGE GAIN

TEMPERATURE (°C)

MAX4249-57 TOC09

LARGE-SIGNAL VOLTAGE GAIN

140

130

120

AV (dB)

110

100

90

80

70

60

50

MAX4249-57 TOC07

RL = 200kΩ RL = 20kΩ RL = 2kΩ

VDD = 3V RL REFERENCED TO GND

140

130

120

AV (dB)

110

100

80

70

60

RL = 20kΩ 	RL = 200kΩ

VDD = 3V RL REFERENCED TO GND

140

130

120

110

AV (dB)

100

90

80

70

60

50

L

RL = 20kΩ

RL = 2kΩ

VDD = 5V RL REFERENCED TO GND

0	50

100

150	200	250

0	50	100	150	200	250

0	50

100

150	200	250

MAX4249–MAX4257

VOUT SWING FROM EITHER SUPPLY (mV)

VOUT SWING FROM EITHER SUPPLY (mV)

VOUT SWING FROM EITHER SUPPLY (mV)

150

130

120

110

LARGE-SIGNAL VOLTAGE GAIN

125

AV (dB)

120

115

LARGE-SIGNAL VOLTAGE GAIN

VDD = 5V

460

440

MAX4249-57 TOC11

420

SUPPLY CURRENT AND SHUTDOWN

MAX4249-57 TOC12 0.376

PER AMPLIFIER

0.375

100

80

VOUT = 10mV TO 4.99mV

400

SHDN = VDD

SUPPLY CURRENT (μA)

380

SHDN = VSS

0.374

50

| 0 50 RL = 20 | 0kΩ |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | RL = 20kΩ |  |  |
|  |  | RL | = 2kΩ |  |
|  |  |  | VD | D = 5V |
|  |  | RL REFE | RENCED T | O GND |

100

MAX4249-57 TOC10

150	200	250

100

-40

TO 4.75mV

-20	0

TO 4.975mV

20	40	60	80

340

-40

-20	0

20	40

60	80

0.373

420

400

380

360

VOUT SWING FROM EITHER SUPPLY (mV)

SUPPLY CURRENT AND SHUTDOWN SUPPLY CURRENT vs. SUPPLY VOLTAGE

| MAX4249-57 TOC13 PER AMPLIFIER |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SHDN = VDD |  |  |  |  |  |  |  |
|  |  |  | S |  |  |  |  |
|  |  |  |  | HDN = VSS |  |  |  |

SHUTDOWN SUPPLY CURRENT (μA)

0.5

0.4

0.3

0.2

2000

1000

SUPPLY CURRENT (μA)

400

TEMPERATURE (°C)

SUPPLY CURRENT

vs. OUTPUT VOLTAGE

VDD = 5V

VDD = 3V

160

MAX4249-57 TOC14

140

VOS (μV)

100

80

TEMPERATURE (°C)

INPUT OFFSET VOLTAGE

vs. SUPPLY VOLTAGE

MAX4249-57 TOC15

VCM = 0

VOUT = VDD/2

RL REFERENCED TO GND

RL = 10kΩ

RL = 1kΩ

320

1.8

2.3

2.8

0

3.3    3.8    4.3    4.8    5.3 5.5

100

0.001

0.01

0.1	1	5

RL = 100kΩ

40

1.8    2.3   2.8    3.3   3.8   4.3    4.8    5.3

SUPPLY VOLTAGE (V)

OUTPUT VOLTAGE (V)

SUPPLY VOLTAGE (V)

MAX4250–MAX4254

GAIN AND PHASE vs. FREQUENCY

60	MAX4249-57  TOC16    180

VDD = 3V, 5V

MAX4249/MAX4255/MAX4256/MAX4257

GAIN AND PHASE vs. FREQUENCY

60	MAX4249-57  TOC17  180

VDD = 3V, 5V

MAX4250–MAX4254

POWER-SUPPLY REJECTION RATIO

vs. FREQUENCY

0

MAX4249-57 TOC18

VDD = 3V, 5V

50

40

30

20

10

0

-10

-20

-30

-40

GAIN

PHASE

RL = 50kΩ CL = 20pF AV = 1000

144

108

PHASE (DEGREES)

72

36

0

-36

-72

-108

-144

-180

50

40

30

GAIN (dB)

20

10

0

-10

-20

-30

-40

RL = 50kΩ CL = 20pF AV = 1000

GAIN

PHASE

144

108

PHASE (DEGREES)

PSRR (dB)

72

36

0

-36

-72

-108

-144

-180

-10

-20

-30

-40

-50

-60

-70

-80

-90

-100

-110

PSRR+

PSRR-

MAX4249–MAX4257

AV (dB)

SUPPLY CURRENT (μA)

GAIN (dB)

SHUTDOWN SUPPLY CURRENT (μA)

100

1k		10k	100k	1M FREQUENCY (Hz)

10M

100

1k		10k	100k	1M FREQUENCY (Hz)

10M

1	10      100

1k	10k    100k     1M

FREQUENCY (Hz)

10M

100

OUTPUT IMPEDANCE

vs. FREQUENCY

AV = 10 (MAX4249/MAX4255/ MAX4256/MAX4257)

INPUT VOLTAGE-N0ISE DENSITY

MAX4249-57 TOC20

vs. FREQUENCY

MAX4249-57 TOC19

25

20

0.1Hz TO 10HzP-P  NOISE

MAX4249-57 TOC21

VDD = 3V OR 5V

Vn-EQUIVALENT INPUT NOISE-VOLTAGE (nV/√Hz)

10	15

200nV/div

1

0.1

1k	10k		100k	1M	10M  FREQUENCY (Hz)

MAX4250–MAX4254

10

5

0

10	100	1k	10k	100k

FREQUENCY (Hz)

MAX4249/MAX4255/MAX4256/MAX4257

VP-PNOISE = 760nVP-P

1s/div

MAX4250–MAX4254

TOTAL HARMONIC  DISTORTION PLUS NOISE

0

-20

VOUT = 2VP-P

MAX4249-57 TOC22

20

RL = 1kΩ

fO = 1kHz	0

MAX4249-57 TOC23

VOUT = 4VP-P

fO = 1kHz

10

AV = 10

fO = 3kHz

VIN

-40

-60

-80

-100

-120

-140

-160

fO

HD2   HD4

HD3

HD5

AV = 1

-20

-40

-60

AMPLITUDE (dBc)

-80

-100

-120

-140

HD2

HD3

VIN

11kΩ

100kΩ

THD+N (%)

1

0.1

0.01

0.001

FILTER BW = 30kHz 	VO

MAX4249-57 TOC24

RL

RL = 1kΩ

10	5k	10k	15k	20k

10	5k	10k	15k	20k

0	1	2

3	4	5

FREQUENCY (Hz)

MAX4250–MAX4254

TOTAL HARMONIC  DISTORTION PLUS NOISE

vs. OUTPUT VOLTAGE SWING (VDD = 3V)

10

FREQUENCY (Hz)

MAX4249/MAX4255/MAX4256/MAX4257

TOTAL HARMONIC  DISTORTION PLUS NOISE

vs. OUTPUT VOLTAGE SWING

1

OUTPUT VOLTAGE (VP-P)

MAX4250–MAX4254

TOTAL HARMONIC DISTORTION PLUS NOISE vs. FREQUENCY

1

VIN

1

11kΩ

THD+N (%)

100kΩ

MAX4249-57 TOC25

VOUT

RL

0.1

AV = 10

VIN

11kΩ

100kΩ

VOUT

MAX4249-57 TOC26

RL

0.1

VIN

R1	R2

MAX4249-57 TOC27

VOUT

RL

0.1

0.01

RL = 1kΩ

RL = 10kΩ

0.01

fO = 20kHz, FILTER BW = 80kHz

THD+N (%)

0.01

0.001

R1 = 560Ω, R2 = 53kΩ

R1 = 5.6kΩ, R2 = 53kΩ

AV = 100

AV = 10

0.001

AV = 10

fO = 3kHz

FILTER BW = 30kHz

RL = 100kΩ

0.001

fO = 3kHz, FILTER BW = 30kHz

0.0001

AV = 1

FILTER BW = 22kHz RL = 10kΩ TO GND VO = 2VP-P

0	1	2	3

0	1	2

3	4	5

10	100

1k	10k

MAX4249–MAX4257

THD+N (%)

AMPLITUDE (dBc)

OUTPUT IMPEDANCE (Ω)

OUTPUT VOLTAGE (VP-P)

OUTPUT VOLTAGE (VP-P)

FREQUENCY (Hz)

0.1

0.01

0.001

0.0001

MAX4250–MAX4254

MAX4249-57 TOC28

TOTAL HARMONIC DISTORTION PLUS NOISE vs. FREQUENCY

FILTER BW = 80kHz

AV = 1

RL = 1kΩ

VOUT = 2VP-P

RL TO VDD/2

RL TO GND RL TO VDD

1.5V

VOUT

200mV/div

0.5V

MAX4250–MAX4254

MAX4249-57 TOC29

LARGE-SIGNAL  PULSE RESPONSE

VDD = 3V RL = 10kΩ CL = 100pF

VIN = 1V PULSE

0.6V VOUT

20mV/div

0.5V

MAX4250–MAX4254

MAX4249-57 TOC30

SMALL-SIGNAL PULSE RESPONSE

VDD = 3V RL = 10kΩ CL = 100pF

VIN = 100V PULSE

10	100

1k	10k

2μs/div

2μs/div

FREQUENCY (Hz)

MAX4249/MAX4255/MAX4256/MAX4257

MAX4249-57 TOC31

LARGE-SIGNAL  PULSE RESPONSE

2V

MAX4249/MAX4255/MAX4256/MAX4257

MAX4249-57 TOC32

SMALL-SIGNAL PULSE RESPONSE

140

130

MAX4249-57 TOC33

| CHANNEL SEPARATION vs. FREQUENCY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

CHANNEL SEPARATION (dB)

120

VOUT

200mV/div

VDD = 3V

1.6V VOUT

50mV/div

1.5V

VDD

= 3V

110

100

90

MAX4249–MAX4257

THD+N(%)

RL = 10kΩ

1V	CL = 100pF

VIN = 100mV PULSE

AV = 10

2μs/div

RL = 10kΩ

CL = 100pF

VIN = 10mV PULSE AV = 10

2μs/div

80

70

0

1k	10k

100k	1M FREQUENCY (Hz)

10M

Pin  Description

| PIN/BUMP |  |  |  |  |  |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAX4250/ MAX4255 | MAX4251/ MAX4256 | MAX4252/ MAX4257 | MAX4252 | MAX4249/ MAX4253 |  |  | MAX4254 |  |  |
| 5-PIN SOT23 | 8-PIN SO/µMAX | 8-PIN SO/µMAX | 8-BUMP UCSP | 10-BUMP UCSP | 10-PIN µMAX | 14-PIN SO | 14-PIN SO |  |  |
| 1 | 6 | 1, 7 | A1, A3 | A1, C1 | 1, 9 | 1, 13 | 1, 7, 8, 14 | OUT, OUTA, OUTB, OUTC, OUTD | Amplifier Output |
| 2 | 4 | 4 | C2 | B4 | 4 | 4 | 11 | VSS | Negative Supply. Connect to ground for single- supply operation |
| 3 | 3 | 3, 5 | C1, C3 | A3, C3 | 3, 7 | 3, 11 | 3, 5, 10, 12 | IN+, INA+, INB+, INC+, IND+ | Noninverting Amplifier Input |
| 4 | 2 | 2, 6 | B1, B3 | A2, C2 | 2, 8 | 2, 12 | 2, 6, 9, 13 | IN-, INA-, INB-, INC-, IND- | Inverting Amplifier Input |
| 5 | 7 | 8 | A2 | B1 | 10 | 14 | 4 | VDD | Positive Supply |
| — | 8 | — | — | A4, C4 | 5, 6 | 6, 9 | — | SHDN, SHDNA, SHDNB | Shutdown Input, Connect to VDD or leave unconnected for normal operation (amplifier(s) enabled). |
| — | 1, 5 | — | — | — | — | 5, 7, 8, 10 | — | N.C. | No Connection. Not internally connected. |
| — | — | — | B2 | B2, B3 | — | — | — | — | Not populated with solder sphere |

MAX4249–MAX4257

Detailed Description

The MAX4249–MAX4257 single-supply  operational amplifiers feature ultra-low noise and distortion while consuming very little power. Their low distortion and low noise make them ideal for use as preamplifiers in wide dynamic-range  applications,  such as 16-bit analog-to- digital converters (see Typical Operating Circuit). Their high-input impedance and low noise are also useful for signal conditioning  of high-impedance  sources, such as piezoelectric transducers.

These devices  have true rail-to-rail output operation, drive loads as low as 1kΩ while maintaining DC accura-

cy, and can drive capacitive loads up to 400pF without oscillation. The input common-mode  voltage range extends from VDD - 1.1V to 200mV beyond the negative rail. The push-pull output stage maintains excellent DC characteristics, while delivering up to ±5mA of current.

The MAX4250–4254 are unity-gain stable, whereas, the MAX4249/MAX4255/MAX4256/MAX4257 have a higher slew rate and  are stable  for gains  ≥  10V/V. The MAX4249/MAX4251/MAX4253/MAX4256 feature a low- power shutdown mode, which reduces the supply cur- rent to 0.5µA and disables the outputs.

The MAX4250AAUK is specified  for operation over the automotive (-40°C to +125°C) temperature range.

Low Distortion Many factors can affect the noise and distortion that the device  contributes  to the input signal. The following guidelines  offer valuable information on the impact of design choices on Total Harmonic Distortion (THD).

Choosing proper feedback and gain resistor values for a particular application  can be a very important factor in reducing  THD. In general, the smaller the closed- loop gain, the smaller the THD generated,  especially when driving heavy resistive loads. Large-value feed- back resistors can significantly improve distortion. The THD of the part normally increases at approximately

20dB per decade, as a function of frequency. Operating  the device  near or above the full-power bandwidth significantly degrades distortion.

RF

RG

VIN

CZ

VOUT

Referencing the load to either supply also improves the part’s distortion performance, because only one of the MOSFETs of the push-pull output stage drives the out- put. Referencing the load to midsupply  increases the part’s distortion for a given load and feedback  setting. (See the Total Harmonic Distortion vs. Frequency graph in the Typical Operating Characteristics.)

For gains  ≥  10V/V, the decompensated  devices MAX4249/MAX4255/MAX4256/MAX4257 deliver the best distortion performance, since they have a higher slew rate and provide a higher amount of loop gain for a given closed-loop  gain setting. Capacitive  loads below 400pF, do not significantly  affect distortion results. Distortion performance  remains relatively con- stant over supply voltages.

Figure 1. Adding  Feed-Forward  Compensation

AV = 2V/V

RF = RG = 10kΩ

100mV

0

VIN =

50mV/div

VOUT =

100mV/div

Low Noise The amplifier’s input-referred, noise-voltage density is dominated by flicker noise at lower frequencies, and by thermal noise at higher frequencies. Because the ther- mal noise contribution is affected by the parallel combi- nation of the feedback  resistive network (RF || RG, Figure 1), these resistors should be reduced  in cases where the system bandwidth is large and thermal noise is dominant. This noise contribution  factor decreases, however, with increasing gain settings.

For example, the input noise-voltage density of the cir- cuit with RF = 100kΩ, RG = 11kΩ (AV = 10V/V) is en =

15nV/√Hz, en can be reduced to 9nV/√Hz by choosing RF  = 10kΩ, RG = 1.1kΩ (AV  = 10V/V), at the expense of greater current consumption  and potentially higher distortion. For a gain of 100V/V with RF  = 100kΩ, RG =

1.1kΩ, the en is low (9nV/√Hz).

2μs/div

Figure 2a. Pulse Response with No Feed-Forward

Compensation

AV = 2

RF = RG = 100kΩ

CZ = 11pF

100mV VIN

0

VOUT

2μs/div

50mV/div

100mV/div

MAX4249–MAX4257

Figure 2b. Pulse Response with 10pF Feed-Forward

Compensation

VIN

MAX4250

MAX4251

MAX4252

MAX4253

MAX4254

RISO

VOUT

CL

Using a Feed-Forward Compensation

Capacitor, CZ The amplifier’s input capacitance  is 11pF. If the resis- tance seen by the inverting input is large (feedback network), this can introduce a pole within the amplifier’s bandwidth,  resulting  in reduced  phase  margin. Compensate the reduced phase margin by introducing a feed-forward  capacitor  (CZ) between the inverting input and the output (Figure 1). This effectively cancels

the pole from the inverting  input of the amplifier. Choose the value of CZ as follows:

CZ = 11 x (RF / RG) [pF]

In the unity-gain stable MAX4250–MAX4254, the use of a proper  CZ  is most important for AV = 2V/V, and

Figure 3. Overdriven  Input Showing No Phase Reversal

4.25V

VOUT

0

4.45V

VIN

A V  =  -1V/V.  In  the  decompensated   MAX4249/ MAX4255/MAX4256/MAX4257, CZ  is most important for AV =  10V/V. Figures 2a and 2b show transient response both with and without CZ.

Using a slightly smaller CZ than suggested  by the for- mula above  achieves  a higher  bandwidth  at the expense of reduced phase and gain margin. As a gen- eral guideline, consider using CZ for cases where RG || RF is greater than 20kΩ  (MAX4250–MAX4254) or greater than 5k Ω  (MAX4249/MAX4255/MAX4256/ MAX4257).

0

20μs/div

AV = 1

VDD = 5V

RL  = 10kΩ

-200mV

Applications Information

The MAX4249–MAX4257 combine  good  driving capa- bility with ground-sensing  input and rail-to-rail output operation. With their low distortion, low noise, and low- power consumption, these devices are ideal for use in portable instrumentation systems and other low-power,

MAX4249–MAX4257

Figure 4. Rail-to-Rail Output Operation

5V

0 	VDD = 5V

RL  = 10kΩ

AV = 10

f = 1kHz

200μs/div

Figure 5. Capacitive-Load Driving Circuit

VOUT

1V/div

noise-sensitive applications.

Ground-Sensing and Rail-to-Rail Outputs The common-mode  input range  of these devices extends below ground,  and offers excellent common- mode rejection. These devices  are guaranteed  not to undergo  phase reversal when the input is overdriven (Figure 3).

Figure 4 showcases the true rail-to-rail output operation of the amplifier, configured with AV = 10V/V. The output swings to within 8mV of the supplies with a 10kΩ load, making the devices ideal in low-supply-voltage applica- tions.

Output Loading and Stability Even with their low quiescent current of 400µA, these amplifiers can drive 1kΩ loads while maintaining excel- lent DC accuracy.  Stability while driving heavy capaci- tive loads is another key feature.

160

140

120

RISO (Ω)

100

80

60

40

20

0

SHADED AREA INDICATES STABLE OPERATION

WITH NO NEED FOR ISOLATION RESISTOR.

4.5

4.0

3.5

3.0

2.5

UNITY-GAIN BANDWIDTH (MHz)

2.0

1.0

0.5

0

SHADED AREA INDICATES STABLE OPERATION

WITH NO NEED FOR ISOLATION RESISTOR.

10	100

1000

10,000

10	100

1000

10,000

CAPACITIVE LOADING (pF)

NOTE: USING AN ISOLATION RESISTOR REDUCES PEAKING.

CAPACITIVE LOAD (pF)

NOTE: RISO CHOSEN FOR PEAKING <2dB.

Figure 6. Isolation Resistance vs. Capacitive  Loading  to

Minimize Peaking (<2dB)

Figure 8. MAX4250–MAX4254  Unity-Gain Bandwidth  vs. Capacitive  Load

25

MAX4250–MAX4254  (AV = 1) MAX4249/MAX4255–MAX4257  (AV = 10)

20 	RISO = 0

SHADED AREA INDICATES

15 	STABLE OPERATION WITH NO NEED FOR ISOLATION RESISTOR.

PEAKING (dB)

10

5

0

For example, if the capacitive load is 1000pF, the corre- sponding isolation resistor is 150Ω. Figure 7 shows that peaking  occurs without the isolation resistor. Figure 8 shows the unity-gain bandwidth  vs. capacitive  load for the MAX4250–MAX4254.

Power Supplies and Layout

The MAX4249–MAX4257 operate from a single 2.4V to

5.5V power supply or from dual supplies  of ±1.20V to

±2.75V. For single-supply operation, bypass the power supply with a 0.1µF ceramic capacitor  placed close to the VDD pin. If operating  from dual supplies, bypass each supply to ground.

10	100

1000

10,000

Good layout improves performance  by decreasing  the

MAX4249–MAX4257

CAPACITIVE LOAD (pF)

Figure 7. Peaking vs. Capacitive  Load

These devices maintain stability while driving loads up to 400pF. To drive higher capacitive  loads, place  a small isolation resistor in series between the output of the amplifier and the capacitive  load (Figure 5). This resistor improves the amplifier’s phase margin by isolat- ing the capacitor  from the op amp’s output. Reference Figure 6 to select a resistance value that will ensure a load capacitance  that limits peaking  to <2dB  (25%).

amount of stray capacitance  and noise at the op amp’s inputs and output. To decrease stray capacitance,  min- imize PC board  trace lengths and resistor leads, and place external components close to the op amp’s pins.

UCSP  Applications Information

For the latest application details on UCSP construction, dimensions, tape carrier information, PC board  tech- niques, bump-pad  layout, and recommended  reflow temperature profile, as well as the latest information on reliability testing results, refer to the Application  Note: UCSP—A Wafer-Level Chip-Scale Package on Maxim’s web site at www.maxim-ic.com/ucsp.

Typical Operating Circuit

5V

50kΩ

2

7

6

MAX4256

AIN

VDD

MAX195

(16-BIT ADC)

DOUT

VIN

3

4     8

5kΩ

SHDN

VSS

SCLK CS REF

SERIAL INTERFACE

4.096V

MAX4249–MAX4257

-5V

SHDN

MAX4249–MAX4257

Selector Guide

| PART | GAIN BANDWIDTH (MHz) | MINIMUM STABLE GAIN (V/V) | NO. OF AMPLIFIERS PER PACKAGE | SHUTDOWN MODE | PIN-PACKAGE |
| --- | --- | --- | --- | --- | --- |
| MAX4249 | 22 | 10 | 2 | Yes | 10-pin µMAX, 14-pin SO |
| MAX4250/A | 3 | 1 | 1 | — | 5-pin SOT23 |
| MAX4251 | 3 | 1 | 1 | Yes | 8-pin µMAX/SO |
| MAX4252 | 3 | 1 | 2 | — | 8-pin µMAX/SO, 8-bump UCSP |
| MAX4253 | 3 | 1 | 2 | Yes | 10-pin µMAX, 14-pin SO, 10-bump UCSP |
| MAX4254 | 3 | 1 | 4 | — | 14-pin SO |
| MAX4255 | 22 | 10 | 1 | — | 5-pin SOT23 |
| MAX4256 | 22 | 10 | 1 | Yes | 8-pin µMAX/SO |
| MAX4257 | 22 | 10 | 2 | — | 8-pin µMAX/SO |

Ordering Information (continued) 	 	Chip Information

MAX4250/MAX4251/MAX4255/MAX4256 TRANSISTOR PART TEMP RANGE PIN- TOP PACKAGE MARK

MAX4251ESA -40°C to +85°C 8 SO —

MAX4251EUA -40°C to +85°C 8 µMAX —

MAX4252EBL-T -40°C to +85°C 8 UCSP-8 AAO

MAX4252ESA -40°C to +85°C 8 SO —

MAX4252EUA -40°C to +85°C 8 µMAX —

MAX4253EBC-T -40°C to +85°C 10 UCSP-10 AAK

MAX4253EUB -40°C to +85°C 10 µMAX —

MAX4253ESD -40°C to +85°C 14 SO —

MAX4254ESD -40°C to +85°C 14 SO —

MAX4255EUK-T -40°C to +85°C 5 SOT23-5 ACCJ

MAX4256ESA -40°C to +85°C 8 SO —

MAX4256EUA -40°C to +85°C 8 µMAX —

MAX4257ESA -40°C to +85°C 8 SO —

MAX4257EUA -40°C to +85°C 8 µMAX —

COUNT: 170

MAX4249/MAX4252/MAX4253/MAX4257 TRANSISTOR COUNT: 340

MAX4254 TRANSISTOR COUNT: 680

TOP VIEW

OUT

VDD

VSS

| IN+ | MAX4250 MAX4250A MAX4255 |  |
| --- | --- | --- |
| 1 |  | 5 |
| 2 |  |  |
| 3 |  | 4 |

SOT23

IN-

OUTA   1

INA-    2

INA+    3

VSS    4

SHDNA   5

MAX4249

MAX4253

μMAX

10    VDD

9     OUTB

8    INB-

7     INB+

6     SHDNB

OUTA   1

| INA- 2 | MAX4251 MAX4256 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

| N.C. |  |  | SHDN | OUTA |  | VDD |
| --- | --- | --- | --- | --- | --- | --- |
| IN- IN+ |  |  | VDD OUT | INA- INA+ |  | OUTB INB- |
| VSS |  |  | N.C. | VSS |  | INB+ |
|  | μMAX/SO |  |  |  | μMAX/SO |  |

INA+    3

VSS    4

N.C.    5

SHDNA   6

N.C.    7

MAX4249

MAX4253

14    VDD

13    OUTB

12   INB-

11    INB+

10   N.C.

9    SHDNB

8    N.C.

OUTA   1

| INA- 2 | MAX4252 MAX4257 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

INA+    3

VDD    4

INB+    5

INB-    6

OUTB   7

MAX4254

14    OUTD

13   IND-

12    IND+

11   VSS

10    INC+

9    INC-

8     OUTC

MAX4249–MAX4257

SO 	SO

SOT-23 5L .EPS

| Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

|  | PACKAGE OUTLINE, SOT-23, 5L |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 21-0057 | E | 1 1 |

4X S

8LUMAXD.EPS

8 	8

E      H Ø0.50±0.1

0.6±0.1

0.6±0.1

1

D

TOP VIEW

1

| BOTTOM VIEW | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | - | 0.043 | - | 1.10 |
| A1 | 0.002 | 0.006 | 0.05 | 0.15 |
| A2 | 0.030 | 0.037 | 0.75 | 0.95 |
| b | 0.010 | 0.014 | 0.25 | 0.36 |
| c | 0.005 | 0.007 | 0.13 | 0.18 |
| D | 0.116 | 0.120 | 2.95 | 3.05 |
| e | 0.0256 BSC |  | 0.65 BSC |  |
| E | 0.116 | 0.120 | 2.95 | 3.05 |
| H | 0.188 | 0.198 | 4.78 | 5.03 |
| L | 0.016 | 0.026 | 0.41 | 0.66 |
| α | 0∞ | 6∞ | 0∞ | 6∞ |
| S | 0.0207 BSC |  | 0.5250 BSC |  |

A2 	A1    A

e 	    	b 	L

FRONT VIEW 	SIDE VIEW

PROPRIETARY INFORMATION TITLE:

PACKAGE OUTLINE, 8L uMAX/uSOP

APPROVAL

DOCUMENT CONTROL NO.

REV.         1

MAX4249–MAX4257

21-0036 	J	1

e 	4X S

10LUMAX.EPS

10 	10

H

Ø0.50±0.1

0.6±0.1

1

0.6±0.1

TOP VIEW

1

BOTTOM VIEW

D2

A2 	A

E2

GAGE PLANE

| c | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | - | 0.043 | - | 1.10 |
| A1 | 0.002 | 0.006 | 0.05 | 0.15 |
| A2 | 0.030 | 0.037 | 0.75 | 0.95 |
| D1 | 0.116 | 0.120 | 2.95 | 3.05 |
| D2 | 0.114 | 0.118 | 2.89 | 3.00 |
| E1 | 0.116 | 0.120 | 2.95 | 3.05 |
| E2 | 0.114 | 0.118 | 2.89 | 3.00 |
| H | 0.187 | 0.199 | 4.75 | 5.05 |
| L | 0.0157 | 0.0275 | 0.40 | 0.70 |
| L1 | 0.037 REF |  | 0.940 REF |  |
| b | 0.007 | 0.0106 | 0.177 | 0.270 |
| e | 0.0197 BSC |  | 0.500 BSC |  |
| c | 0.0035 | 0.0078 | 0.090 | 0.200 |
| S | 0.0196 REF |  | 0.498 REF |  |
| α | 0° | 6° | 0° | 6° |

b

A1	α

D1

E1 		L L1

FRONT VIEW

SIDE VIEW

PROPRIETARY INFORMATION TITLE:

PACKAGE OUTLINE, 10L uMAX/uSOP

APPROVAL

DOCUMENT CONTROL NO.

REV.         1

MAX4249–MAX4257

9LUCSP, 3x3.EPS

21-0061	I	1

PACKAGE OUTLINE, 3x3 UCSP

1

21-0093	J	1

MAX4249–MAX4257

SOICN .EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | 0.053 | 0.069 | 1.35 | 1.75 |
| A1 | 0.004 | 0.010 | 0.10 | 0.25 |
| B | 0.014 | 0.019 | 0.35 | 0.49 |
| C | 0.007 | 0.010 | 0.19 | 0.25 |
| e | 0.050 BSC |  | 1.27 BSC |  |
| E | 0.150 | 0.157 | 3.80 | 4.00 |
| H | 0.228 | 0.244 | 5.80 | 6.20 |
| L | 0.016 | 0.050 | 0.40 | 1.27 |

|  | INCHES |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX | N | MS012 |
| D | 0.189 | 0.197 | 4.80 | 5.00 | 8 | AA |
| D | 0.337 | 0.344 | 8.55 | 8.75 | 14 | AB |
| D | 0.386 | 0.394 | 9.80 | 10.00 | 16 | AC |

| N E H VARIATIONS: 1 TOP VIEW D A C e B A1 0∞-8∞ L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, .150" SOIC |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0041 | REV. B | 1 1 |

MAX4249–MAX4257

12L, USPC.EPS

| Maxim cannot assume responsibility for use of any circuitry other than circuitry entirely embodied in a Maxim product. No circuit patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time. |  |  |
| --- | --- | --- |

Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600  	19

©  2005 Maxim Integrated  Products 	Printed USA 	is a registered  trademark  of Maxim Integrated  Products, Inc.