19-1472; Rev 0; 4/99

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

General Description

The MAX4370 is a circuit-breaker IC designed  to offer protection in hot-swap applications  using Maxim’s DualSpeed/BiLevel™ detection.  This controller, designed  to reside either on the backplane  or on the removable card, is used to protect a system from start- up damage when a card or board is inserted into a rack with the main system power supply  turned on. The card’s  discharged  filter capacitors  provide  a low impedance that can momentarily cause the main power supply to collapse. The MAX4370 prevents this start-up condition by providing inrush current regulation during a programmable start-up period, allowing the system to stabilize safely. In addition, two on-chip comparators provide DualSpeed/BiLevel short-circuit protection and overcurrent protection during normal operation.

The MAX4370 provides protection for a +3V to +12V single supply. An internal charge pump generates the controlled gate drive for an external N-channel MOS- FET power switch. The MAX4370 latches the switch off after a fault condition  until an external reset signal clears the device. Other features include a status pin to indicate a fault condition, an adjustable overcurrent response time, and a power-on reset comparator.

MAX4370†

Features

♦ DualSpeed/BiLevel Protection During Normal

Operation

♦ Inrush Current Regulated at Start-Up

♦ Resides Either on the Backplane or on the

Removable Card

♦ Programmable Start-Up Period and Response

Time

♦ Allows Safe Board Insertion and Removal from

Live Backplane

♦ Protection for +3V to +12V Single Supplies

♦ Latched Off After Fault Condition

♦ Status Output Pin

♦ Internal Charge Pump Generates Gate Drive for

External N-Channel MOSFET

Ordering Information

The MAX4370 is specified  for the extended-industrial

PART

TEMP. RANGE         PIN-PACKAGE

temperature range (-40°C to +85°C) and is available in

MAX4370ESA           -40°C to +85°C

8 SO

an 8-pin SO package.

Applications

Hot Board Insertion

Solid-State Circuit Breaker

Pin Configuration appears at end of data sheet.

Typical Operating Circuit

BACKPLANE

VCC

REMOVABLE CARD

M1

RSENSE

CBOARD

VOUT

STAT GND

STAT

ON

CSPD

ON CSPD

GND

CTIM

CTIM

DualSpeed/BiLevel is a trademark of Maxim Integrated Products.

†Patent Pending

Maxim Integrated Products   1

For free samples & the latest literature: http://www.maxim-ic.com, or phone 1-800-998-8800. For small orders, phone 1-800-835-8769.

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

ABSOLUTE MAXIMUM RATINGS

VIN to GND...........................................................................+15V STAT to GND ..........................................................-0.3V to +14V GATE to GND ..............................................-0.3V to (VIN + 8.5V) ON to GND (Note 1) ................................................. -1V to +14V CSPD to GND .............-0.3V to the lower of (VIN + 0.3V) or +12V VSEN, CTIM to GND ....................................-0.3V to (VIN + 0.3V) Current into ON...................................................................±2mA

Current into Any Other Pin ................................................±50mA Continuous Power Dissipation (TA = +70°C)

SO (derate 5.9mW/°C above +70°C) ...........................  471mW

Operating Temperature Range ...........................-40°C to +85°C Storage Temperature Range .............................-65°C to +150°C Lead Temperature (soldering,  10sec) ............................  +300°C

Note 1: ON can be pulled below ground. Limiting the current to 2mA ensures that this pin is never lower than about -0.8V.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VIN = +2.7V to +13.2V, TA = -40°C to +85°C, unless otherwise noted. Typical values are at VIN = +5V and TA = +25°C.) (Note 2)

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| POWER SUPPLIES |  |  |  |  |  |  |
| Input Voltage Range | VIN |  |  |  | 2.7 13.2 | V |
| Supply Current | IQ | ON = VIN |  |  | 0.6 1 | mA |
| CURRENT CONTROL |  |  |  |  |  |  |
| Slow Comparator Threshold | VSC,TH | VIN - VSEN | TA = +25°C |  | 45 50 55 | mV |
|  |  |  | TA = TMIN to TMAX |  | 43.5 56 |  |
| Slow Comparator Response Time | tCSPD | CSPD = floating |  |  | 10 20 40 | µs |
|  |  | 100nF on CSPD to GND |  |  | 10 20 40 | ms |
| Fast Comparator Threshold | VFC,TH | VIN - VSEN |  |  | 180 200 220 | mV |
| Fast Comparator Response Time | tFCD | 10mV overdrive, from overload condition to GATE discharging |  |  | 460 | ns |
| VSEN Input Bias Current | IB,VSEN | VSEN = VIN |  |  | 0.2 10 | µA |
| MOSFET DRIVER |  |  |  |  |  |  |
| Start-Up Period (Note 3) | tSTART | 100nF on CTIM |  |  | 21 31 41 | ms |
|  |  | CTIM = floating |  |  | 5.5 | µs |
| Gate Charge Current | IGATE | VGATE = VIN (Note 4) |  |  | 100 | µA |
| Turn-Off Time | tOFF | Time from current overload to VGATE < 0.1V, CGATE = 1000pF to GND (triggered by the fast comparator during normal operation) |  |  | 60 | µs |
| Gate Discharge Current | IGATE,DIS | During start-up (current regulation provided by fast comparator) |  |  | 80 | µA |
|  |  | During turn-off, triggered by a fault in normal operation or ON falling edge |  |  | 75 225 550 |  |
| Maximum Gate Voltage |  | Measured with respect to VIN; voltage at which internal clamp circuitry is triggered |  |  | 6.7 7.5 | V |
| Minimum Gate Drive Voltage |  | IGATE = 8.5µA, measured above VIN |  | VIN ≥ 5V | 5 | V |
|  |  |  |  | VIN ≥ 2.7V | 2.7 |  |
| Gate Overvoltage Threshold |  | Start-up is initiated only if VGATE is less than this voltage |  |  | 0.1 | V |

2

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

ELECTRICAL CHARACTERISTICS  (continued)

(VIN = +2.7V to +13.2V, TA = -40°C to +85°C, unless otherwise noted. Typical values are at VIN = +5V and TA = +25°C.) (Note 2)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| ON COMPARATOR |  |  |  |  |
| Threshold Voltage | VTH,ON | VIN = 5V, rising threshold | 0.575 0.6 0.625 | V |
| Hysteresis | VHYST |  | 3 | mV |
| Power-Supply Rejection Ratio | PSRR | 2.7V ≤ VIN ≤ 13.2V | 0.1 1 | mV/V |
| Propagation Delay | tD,COMP | 10mV overdrive | 10 | µs |
| Input Voltage Range | VON | Input can be driven to the absolute maximum limit without false output inversion | -0.1 13.2 | V |
| Input Bias Current | IB,ON |  | 0.001 1 | µA |
| ON Pulse Width Low | tRESTART | To restart after a fault | 20 | µs |
| DIGITAL OUTPUT (STAT) |  |  |  |  |
| Output Leakage Current |  | VSTAT ≤ +13.2V | 1 | µA |
| Output Voltage Low | VOL | ISINK = 1mA | 0.4 | V |
| VIN UNDERVOLTAGE LOC | OUT |  |  |  |
| Threshold | VUVLO | Start-up is initiated when this threshold is reached at VIN | 2.25 2.67 | V |
| Hysteresis | VUVLO,HYST |  | 100 | mV |
| UVLO to Start-Up Delay | tD,UVLO | Time which input voltage must exceed under- voltage lockout before start-up is initiated | 100 150 200 | ms |

Note 2:  All devices are 100% tested at TA = +25°C. All temperature  limits are guaranteed  by design.

Note 3:  The start-up period (tSTART) is the time during which the slow comparator  is ignored and the device acts as a current limiter by regulating the sense current with the fast comparator.  It is measured from ON rising above 0.6V to STAT rising.

Note 4:  The current available at GATE is a function of VGATE (see Typical Operating Characteristics.)

3

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

SUPPLY CURRENT (mA)

Typical Operating Characteristics

(Circuit of Figure 7, VIN = 5V, RSENSE = 100m, M1 = FDS6670A, CBOARD = 470µF, CGATE = 0, RS = 0, TA = +25°C, unless other- wise noted.)

1.0

0.9

0.8

0.7

0.6

0.5

0.3

0.2

0.1

0

MAX4370 toc01

SUPPLY CURRENT vs. INPUT VOLTAGE

1.0

0.9

SUPPLY CURRENT (mA)

0.8

0.7

0.6

0.5

0.3

0.2

0.1

0

MAX4370 toc02

SUPPLY CURRENT vs. TEMPERATURE

VIN = 12V VIN = 5V

VIN = 3V

51.0

50.8

50.6

50.4

VSC, TH (mV)

50.2

50.0

49.6

49.4

49.2

49.0

SLOW COMPARATOR  THRESHOLD

vs. INPUT VOLTAGE

MAX4370 toc03

TA = +25°C TA = -40°C

TA = +85°C

| 0 2 4 6 8 10 12 14 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ON = VIN IGATE = 10 A |  |  |  |  |  |
|  |  |  |  | ON = GND |  |  |

VIN (V)

-40       -15         10            35            60           85

TEMPERATURE (°C)

0         2         4         6         8        10       12       14

VIN (V)

SLOW COMPARATOR  RESPONSE TIME vs. INPUT VOLTAGE

MAX4370 toc05

| 24 24 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CSPD = 110nF TIME IN ms |  |  |  |  |
|  |  |  | CSPD = 0 TIME IN s |  |  |  |

23                                                                                       23

tCSPD (ms) VFC, TH (mV)

22                                                                                       22

tCSPD (  s)

21                                                                                       21

20                                                                                       20

19                                                                                       19

18                                                                                       18

210

108

206

204

202

200

198

196

194

192

190

MAX4370 toc06

FAST COMPARATOR  THRESHOLD

vs. INPUT VOLTAGE

700

600

500

tFCD (ns)

400

300

200

100

0

MAX4370 toc07

FAST COMPARATOR  RESPONSE TIME

vs. OVERDRIVE VOLTAGE

VIN = 12V VIN = 5V

VIN = 3V

0         2         4

6         8

VIN (V)

10       12       14

| 0 2 4 6 8 10 12 14 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TA = -40° |  | C |  |
|  |  |  |  | A = +25°C |  |  |  |
|  |  |  | T |  |  |  |  |
|  |  |  | TA = +85°C |  |  |  |  |

VIN (V)

1                        10                      100                    1000

VOD (mV)

500

490

480

470

tFCD (ns)

460

450

440

430

420

410

400

MAX4370 toc08

FAST COMPARATOR  RESPONSE TIME

vs. TEMPERATURE

VIN = 12V VIN = 5V

VIN = 3V

VOD = 10mV

350

330

tSTART (  s)

310

290

270

250

START-UP TIME

vs. INPUT VOLTAGE

CTIM = 100nF TIME IN ms

CTIM = 1nF TIME IN   s

MAX4370 toc09

35

33

tSTART (ms)

31

29

27

25

-40    -20     0        20       40       60       80      100

TEMPERATURE (°C)

0         2         4

6         8        10       12       14

VIN (V)

4

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

Typical Operating Characteristics (continued)

(Circuit of Figure 7, VIN = 5V, RSENSE = 100m, M1 = FDS6670A, CBOARD = 470µF, CGATE = 0, RS = 0, TA = +25°C, unless other- wise noted.)

120

100

GATE CHARGE CURRENT

vs. GATE VOLTAGE

VIN = 12V

MAX4370 toc10

150

125

GATE CHARGE CURRENT

vs. TEMPERATURE

MAX4370 toc11

VGATE = 0

VIN = 12V

GATE VOLTAGE vs. INPUT VOLTAGE

25

MAX4370 toc12

IGATE = 10   A

VGATE (V)

20                                         TA = +85°C

IGATE (  A)

80

VIN = 3.0V

60

40

20

VIN = 5.0V

100

IGATE (  A)

75

50

25

VIN = 5V VIN = 3V

15                       TA = +25°C

10

5

TA = -40°C

0

0      2      4

6      8     10    12    14    16    18    20

0

-40

-15

-10         35

60            85

0

0         2         4

6         8        10       12       14

| 400 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VIN = 3V |  |  |  |  |  |  |  |  |  |
|  |  | VIN = 5V |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | VIN = 12V |  |  |  |
|  |  |  |  |  | TRIGGERED BY A FAULT OR BY ON FALLING |  |  |  |  |  |

350

300

IGATE (  A)

250

VGATE (V)

MAX4370 toc13

GATE DISCHARGE CURRENT

vs. GATE VOLTAGE

400

350

300

IGATE (  A)

250

TEMPERATURE (°C)

GATE DISCHARGE CURRENT

vs. TEMPERATURE

ILOAD

(1A/div)

VIN (V)

MAX4370 toc14

START-UP TIME (CBOARD = 470 F)

MAX4370-15

ON VGATE

(2V/div)

200

150

100

50

0

0

2      4      6      8     10

12    14    16    18    20

200

150

100

50

| 0 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | VIN = 3V |  |  |
|  |  | VIN = 5V & 12V |  |  |
| VGATE = VIN TRIGGERED BY A |  | FAULT |  |  |
| OR BY O | N FALLING |  |  |  |

-40

-15         10

35            60            85

500  s/div

VOUT

(2V/div)

VGATE (V)

START-UP TIME (CBOARD = 0)

MAX4370-16

ON VGATE

TEMPERATURE (°C)

START-UP TIME

(EXTERNAL CGATE = 22nF,  CBOARD = 470 F)

MAX4370-17

ON

VGATE

CBOARD = 470   F, RSENSE = 100m   , CTIM = 10nF, CGATE = 0

TURN-OFF TIME (CBOARD = 470 F)

MAX4370-18

ON

ILOAD

(2V/div)

ILOAD

(1A/div)

(2V/div)

(1A/div)

0A

VOUT

(2V/div)

VOUT

(2V/div)

0V

VOUT

(2V/div)

VGATE

(2V/div)

100  s/div

CBOARD = 0, RSENSE = 100m   , CTIM = 10nF, CGATE = 0

1ms/div

CBOARD = 470   F, RSENSE = 100m   , CGATE = 22nF, CTIM = 10nF, RS = 0

50  s/div

CBOARD = 470   F, RSENSE = 100m   , CGATE = 0

5

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

Typical Operating Characteristics (continued)

(Circuit of Figure 7, VIN = 5V, RSENSE = 100m, M1 = FDS6670A, CBOARD = 470µF, CGATE = 0, RS = 0, TA = +25°C, unless other- wise noted.)

TURN-OFF TIME (CBOARD = 0)

MAX4370-19

ON

TURN-OFF TIME

(EXTERNAL CGATE = 22nF,  CBOARD = 470F)

MAX4370-20

ON

1000

MAX4370 toc21

TIME  TO CHARGE GATE

vs. CGATE

NO EXTERNAL MOSFET

VGATE

ILOAD

VGATE

TIME TO CHARGE GATE (ms)

ILOAD

100

(2V/div)

VOUT

(2V/div)

(1A/div)  (2V/div)

0A

VOUT

(2V/div)

(1A/div)

0A

10

1

0.1

VIN = 12V

TO VGATE = 17V

VIN = 3V

VIN = 5V

TO VGATE = 10V

0V                                                                                                                    0V

0.01

TO VGATE = 6V

1000

TIME TO DISCHARGE GATE (ms)

100

10

1

50s/div

CBOARD = 0, RSENSE = 100m, CGATE = 0, RS = 0

TIME  TO DISCHARGE GATE

vs. CGATE

MAX4370 toc22

NO EXTERNAL MOSFET

VIN = 12V

0.605

0.603

0.601

200s/div

CBOARD = 470F, RSENSE = 100m, CGATE = 22nF, RS = 0

MAX4370 toc23

ON COMPARATOR  THRESHOLD

vs. INPUT VOLTAGE

RISING

FALLING

0.001

0.01         0.1            1             10           100         1000

CGATE (nF)

MAX4370 toc24

ON COMPARATOR  THRESHOLD

vs. TEMPERATURE

0.6100

0.6075

0.6050

0.6025

0.1

0.01

VIN = 3V

ON COMPARATOR THRESHOLDD (V)

0.597

0.6000                                                                                     }

ON COMPARATOR THRESHOLD (V)

FALLING   RISING

0.5975                                                                                     }

0.001

0.01         0.1            1             10           100         1000

0.595

0         2

4         6         8        10       12       14

0.5950

-40

-15         10

35           60            85

CGATE (nF)

VIN (V)

TEMPERATURE (°C)

2.60

MAX4370 toc25

UVLO THRESHOLD VOLTAGE

vs. TEMPERATURE

160

MAX4370 toc26

UVLO DELAY vs. TEMPERATURE

2.55

2.50

2.45

155

UVLO DELAY (ms)

150

UVLO THRESHOLD (V)

2.40

| 2.35 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | RISING |  |
|  |  |  | FALLING |  |

145

2.30

-40

-15         10

35            60            85

140

| -40 |  |  |  |  |
| --- | --- | --- | --- | --- |
| VIN = 5V & 12V |  |  |  |  |
|  |  | VIN = 3V |  |  |

-15         10

35            60            85

TEMPERATURE (°C)

TEMPERATURE (°C)

6

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | VIN | Supply Voltage Input. Connect to 2.7V to 13.2V. |
| 2 | VSEN | Current-Sense Resistor Voltage Input. RSENSE is connected from VIN to VSEN. |
| 3 | GATE | Gate Drive Output. Connect to gate of external N-channel MOSFET. |
| 4 | GND | Ground |
| 5 | CSPD | Slow Comparator Speed Setting. Leave floating or connect the timing capacitor from CSPD to GND. See Slow Comparator Response Time section. |
| 6 | CTIM | Start-Up Timer Setting. Leave floating or connect the timing capacitor from CTIM to GND. See Start-Up Timing Capacitor section. |
| 7 | STAT | Status Output—open drain. High indicates start-up completed with no fault. See Table 1. |
| 8 | ON | ON Comparator Input. Connect high for normal operation; connect low to force the MOSFET off. Comparator threshold VTH,ON = 0.6V allows for precise control over shutdown feature. Pulse ON low for at least 20µs, then high to restart after a fault. |

CSPD

CSPD

VIN

2.45V

INPUT UVLO

6  A

MAX4370

6  A

SLOW COMPARATOR

FAST COMPARATOR

VSC, TH

50mV

VFC, TH

200mV

RSENSE

STAT

CTIM

150ms DELAY ON

VIN RISING

VIN

4  A

LOGIC CONTROL

DISCHARGE ENABLE

0.1V

GATE DRIVE

CHARGE PUMP

GATE OVLO

VSEN

M1

GATE                      N

ON

CTIM

ON COMPARATOR

0.6V

VOUT

GND

Figure 1. Functional Diagram

7

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

Detailed Description

The MAX4370 is a circuit-breaker IC designed for hot- swap applications  where a card  or board  is to be inserted into a rack with the main system power supply

backplane, the card is discharged filter capacitors pro- vide a low impedance,  which can momentarily cause the main power supply to collapse. The MAX4370 is designed  to reside either in the backplane  or in the removable card to provide inrush-current limiting and short-circuit protection. This is achieved using a charge pump as gate drive for an external N-channel MOSFET, an external current-sense resistor, and two on-chip comparators.  Figure 1 shows the device’s functional diagram.

The slow comparator response time and the start-up timer can be adjusted with external capacitors. The tim- ing components are optional; without them the part is set to its nominal values, as shown in the Electrical Characteristics.

Start-Up Period

ON

VGATE

VOUT

ILOAD

IFAST, SET

VTH

tON

tSTART

~VIN

CBOARD = LARGE CBOARD = 0

VGATE

VOUT

CTIM sets the start-up period. This mode starts when the power is first applied  to VIN if ON is connected to VIN, or at the rising edge of ON. In addition, the voltage at VIN must be above the undervoltage  lockout for

150ms (see Undervoltage Lockout).

During start-up, the slow comparator is disabled  and current limiting is provided two different ways:

1) Slow ramping of the current to the load by controlling the external MOSFET gate voltage.

2) Limiting the current to the load by regulating the volt- age across the external current-sense resistor.

Unlike other circuit-breaker ICs, the MAX4370 hot-swap controller regulates the current to a preset level instead of completely turning off if an overcurrent occurs during start-up.

In start-up mode, the gate drive current is limited to

100µA and decreases with the increase of the gate voltage (see Typical Operating Characteristics). This allows the MAX4370 to slowly enhance the MOSFET. If the fast comparator detects an overcurrent, the gate voltage is momentarily discharged  with a fixed 80µA current until the load current through the sense resistor (RSENSE) decreases below its threshold point. This effectively regulates the turn-on current during start-up. Figure 2 shows the start-up waveforms. STAT goes high at the end of the start-up period if no fault condi- tion is present.

Figure 2. Start-Up Waveforms

Normal Operation (DualSpeed/BiLevel) In normal operation (after the start-up period  has expired), protection is provided by turning off the exter- nal MOSFET when a fault condition  is encountered. DualSpeed/BiLevel fault protection incorporates two comparators  with different thresholds and response times to monitor the load current:

1) Slow Comparator. This comparator has an externally set response time (20µs to seconds) and a fixed

50mV threshold voltage. The slow comparator ignores low-amplitude  momentary current glitches. After an extended overcurrent condition, a fault is detected and the MOSFET gate is discharged.

2) Fast Comparator. This comparator  has a fixed response time and a higher 200mV threshold volt- age. The fast comparator  turns off the MOSFET immediately after it detects a large amplitude event such as a short circuit.

In each case, when a fault is encountered, the status pin (STAT) goes low and the MAX4370 stays latched off. Figure 3 shows the waveforms after a fault condi- tion.

8

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

STAT

VGATE

VOUT

ILOAD

~VIN

ILIM

td

tOFF

~VTH

MAX4370

50% (i.e., > 50% of the time the current is above the threshold level) will be considered  a fault condition even if it is never higher than the threshold for more than the slow comparator’s set response time.

Once the fault condition is detected, the STAT pin goes low and the device goes into latched mode. The GATE voltage discharge  rate depends  on the gate capaci- tance and the external capacitance at GATE.

Fast Comparator The fast comparator behaves differently according  to the operating mode. During start-up, the fast compara- tor is part of a simple current regulator. When the sensed current is above the threshold (VFC,TH =

200mV), the gate is discharged  with a 80µA current source. When the sensed current drops  below the threshold, the charge pump turns on again. The sensed current will rise and fall near the threshold due to the fast comparator and charge-pump  propagation  delay. The gate voltage will be roughly saw-tooth shaped, and the load current will present a 20% ripple. The ripple

Figure 3. Response to a Fault Condition

Slow Comparator The slow comparator is disabled  at start-up while the external MOSFET is turning on. This allows the part to ignore the higher-than-normal inrush current charging the board capacitors  (CBOARD) when a card is first plugged in.

If the slow comparator  detects an overload current while in normal operation (after start-up is completed), it turns off the external MOSFET by discharging  the gate capacitance  with a 200µA current. The slow com- parator threshold is set at 50mV and has a default delay of 20µs (CSPD floating), allowing it to ignore power-supply glitches and noise. The response time can be lengthened with an external capacitor at CSPD (Figure 8).

If the overcurrent condition is not continuous, the dura- tion above the threshold minus the duration below it must be greater than 20µs (or the external programmed value) for the device to trip. When the current is above the threshold, CSPD is charged  with a 6µA current source; when the current is below the threshold, CSPD is discharged  with a 6µA current source. A fault is detected  when CSPD is charged  to the trip point of

1.2V. A pulsing current with a duty cycle greater than

can be reduced  by adding  a capacitor  from GATE to GND. Once CBOARD  is completely charged,  the load current drops  to its normal operating levels. If the sensed current is still high after the start-up timer expires, the MOSFET gate is discharged completely.

In normal operation (after start-up), the fast comparator is used as an emergency off switch. If the load current reaches the fast comparator  threshold, the device immediately forces the MOSFET off completely by dis- charging  the GATE with a 200µA current. This can occur in the event of a serious current overload or a dead short. Given a 1000pF gate capacitance and 12V gate voltage, the MOSFET will be off in less than 60µs. Any additional capacitance  connected between GATE and GND to slow down the turn-on time also increases the turn-off time.

Latched Mode and Reset The MOSFET driver of the MAX4370 stays latched off after a fault condition  until it is reset by a negative- going pulse on the ON pin. Pulse ON low for 20µs (min), then high to restart after a fault. During start-up, a negative-going edge on ON will force the device to turn off the MOSFET and place the device in latched mode. Keep ON low for 20µs (min) to restart.

9

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

Status Output The status output is an open-drain output that goes low when the part is:

1) in start-up

2) forced off (on = GND)

3) in an overcurrent condition, or

4) latched off.

STAT is high only if the part is in normal mode and no faults are present (Table 1). Figure 4 shows the STAT timing diagram.

Over/Undervoltage Lockouts

The undervoltage lockout prevents the MAX4370 from

ON

CTIM

STAT

VIN

OV

1.2V OV

VIN

OV

FAULT CONDITION OR ON

FALLING EDGE

NO FAULT CONDITIONS PRESENT

turning on the external MOSFET until the input voltage at VIN exceeds the lockout threshold (2.25V min) for at least 150ms. The undervoltage  lockout protects  the external MOSFET from insufficient gate drive voltage. The 150ms timeout ensures that the board  is fully plugged  into the backplane  and that VIN is stable. Voltage transients at VIN with voltages below the UVLO will reset the device and initiate a start-up sequence.

The device also features a gate overvoltage lockout that prevents the device  from restarting after a fault condition  if the discharge  has not been completed. VGATE must be discharged  to below 0.1V before restarting. Since the MAX4370 does not monitor the output voltage, a start-up sequence can be initiated

Figure 4. Status Output (STAT) Timing Diagram

VIN

RSENSE

MAX4370

VSEN

VGD

while the board capacitance is still charged.

Gate Overvoltage Protection Newer-generation MOSFETs have an absolute maxi- mum rating of ±8V for the gate-to-source voltage (VGS). To protect these MOSFETs, the MAX4370 limits the gate-to-drain  (VGD) to +7.5V with an internal zener diode. No protection is provided  for negative VGD. If GATE can be discharged to GND faster than the output voltage, an external small-signal protection diode (D1)

GATE DRIVE CHARGE PUMP

GATE

VGS

D1

M1

N VOUT

CBOARD

can be used, as shown in Figure 5.

Table 1. Status Output Truth Table

Figure 5. External Gate-Source Protection

| PART IN START-UP | ON PIN | OVERCURRENT CONDITION ON VIN | PART IN LATCHED-OFF MODE DUE TO OVERCURRENT CONDITION | STAT PIN (STATUS) |
| --- | --- | --- | --- | --- |
| Yes | X | X | X | Low |
| No | Low | X | X | Low |
| No | High | Yes | X | Low |
| No | High | No | Yes | Low |
| No | High | No | No | High |

X = Don’t care

10

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

Applications Information

Component Selection

N-Channel MOSFET Select the external N-channel MOSFET according  to the application’s  current level. The MOSFET’s RDS(ON) should be chosen low enough to have a minimum volt- age drop at full load to limit the MOSFET power dissipa- tion. High RDS(ON) can cause output ripple if the board has pulsing loads, or it can trigger an external under- voltage reset monitor at full load. Determine the device’s power rating requirement to accommodate a short-circuit condition on the board during start-up (see MOSFET Thermal Considerations).

MOSFETs can typically withstand single-shot pulses with higher dissipation than the specified package rat- ing. Also, since part of the inrush current limiting is achieved by limiting the gate dV/dt, it is not necessary to use a MOSFET with low gate capacitance.  Table 2 lists some recommended  manufacturers and compo- nents.

MAX4370

capacitance).  Longer timings are determined by the value of the capacitor,  according  to Figure 6, and can be determined as follows:

tSTART (ms) = 0.31 · CTIM (nF)

Set the t START timer to allow the MOSFET to be enhanced and the load capacitor  to be completely charged.

There are two methods of completing  the start-up sequences. Case A describes a start-up sequence that does not use the current-limiting feature and slowly turns on the MOSFET by limiting the gate dV/dt. Case B uses the current-limiting feature and turns on the MOSFET as fast as possible while still preventing high inrush current.

1000

100

10

1

Sense Resistor

tSTART (ms)

The slow comparator threshold voltage is set at 50mV. Select a sense resistor that causes a 50mV voltage drop  at a current level above the maximum normal operating current. Typically, set the overload current at

1.2 to 1.5 times the nominal load current. The fast com- parator threshold is set at 200mV. This sets the fault current limit at four times the overload current limit.

Choose the sense-resistor power rating to accommo- date the overload current (Table 3):

PSENSE = (IOVERLOAD)2 · RSENSE

Start-Up Timing Capacitor (CTIM) The start-up period (tSTART) is determined by the capaci- tor connected at CTIM. This determines the maximum time allowed to completely turn on the MOSFET.

The default value for tSTART is chosen by leaving CTIM floating and is approximately 5.5µs. This is also the minimum value (not controlled and dependent on stray

0.1

0.01

0.001

0.01         0.1            1             10           100         1000

CAPACITANCE (nF)

Figure 6. Start-Up Period vs. CTIM

| Table 3. Current Levels vs. RSENSE RSENSE (mΩ) | OVERLOAD THRESHOLD SET BY SLOW COMPARATOR (A) | FAULT CURRENT THRESHOLD SET BY FAST COMPARATOR (A) |
| --- | --- | --- |
| 10 | 5 | 20 |
| 50 | 1 | 4 |
| 100 | 0.5 | 2 |

Table 2. Component Manufacturers

| Sense Resistors COMPONENT | MANUFACTURER | PHONE | INTERNET |
| --- | --- | --- | --- |
|  | Dale-Vishay | 402-564-3131 | www.vishay.com |
|  | IRC | 704-264-8861 | www.irctt.co |
| MOSFETs | Fairchild | 888-522-5372 | www.fairchildsemi.com |
|  | International Rectifier | 310-322-3331 | www.irf.com |
|  | Motorola | 602-244-3576 | www.mot-sps.com/ppd/ |

11

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

Case A: Slow Turn-On (without overcurrent)

There are two ways to turn on the MOSFET without reaching the fast comparator current limit:

1)  If the board  capacitance  (CBOARD) is low, the inrush current is low.

2)  If the capacitance  at GATE is high, the MOSFET

turns on slowly.

In both cases, the turn-on (tON) is determined only by the charge required to enhance the MOSFET—effec- tively, the small gate-charging  current limits the output voltage dv/dt. This time can be extended by connect- ing an external capacitor between GATE and GND, as shown in Figure 7. The turn-on time is dominated by the

charging CBOARD can be considered constant and the turn-on time is determined by:

tON = CBOARD · VIN / IFAST,SET

where the maximum load current IFAST,SET = VFC,TH / RSENSE. Figure 2 shows the waveforms and timing dia- grams for a turn-on transient with current regulation (see Start-Up Time with CBOARD  = 470µF in the Typical Operating Characteristics). When operating under this condition, an external gate capacitor  is not required. Adding an external capacitor at GATE reduces the regu- lated current ripple but increases the turn-off time by increasing the gate delay (td) (Figure 3).

RSENSE                                                                    VOUT

external gate capacitance  if its value is considerably higher than MOSFET gate capacitance.  Table 4 shows the timing required  to enhance the recommended MOSFET with or without an external capacitor at GATE; Figures 2 and 3 show the related waveforms and timing diagrams (see Start-Up Time with CBOARD  =  0 and

VIN

RPULL-UP

M1

RS*

CBOARD

Start-Up Time with External CGATE  in the Typical Operating Characteristics). Remember that a high gate capacitance also increases the turn-off time.

When using the MAX4370 without an external gate capacitor,  RS is not necessary. RS prevents MOSFET source oscillations that can occur when CGATE is high while CBOARD is low.

Case B: Fast Turn-On (with current limit)

In applications where the board capacitor (CBOARD) at

VOUT is high, the inrush current causes a voltage drop

CSPD

VIN

STAT ON CSPD

VSEN

MAX4370

GND

GATE

CTIM

CGATE

CTIM

across RSENSE that exceeds  the fast comparator threshold (VFC,TH = 200mV). In this case, the current

*OPTIONAL (SEE TEXT)

Figure 7. Operation with External Gate Capacitor

Table 4. MOSFET Turn-On Time (start-up without current limit)

(CBOARD = 0, turn-on with no load current, turn-off with 2A fault current)

| DEVICE | CGATE (nF) | MOSFET TURN-ON (tON) |  |  | MOSFET TURN-OFF (tOFF) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VIN = 3V | VIN = 5V | VIN = 12V | VIN = 3V | VIN = 5V | VIN = 12V |
| Fairchild FDS6670A | 0 | 220µs | 160µs | 190µs | 70µs | 130µs | 145µs |
|  | 22 | 2.3ms | 2ms | 3.2ms | 540µs | 1.1ms | 1.95ms |
| International Rectifier IRF7401 | 0 | 175µs | 130µs | 160µs | 75µs | 130µs | 160µs |
|  | 22 | 1.9ms | 1.8ms | 3.5ms | 540µs | 1.1ms | 2ms |
| Motorola MMSF5N03HD | 0 | 101µs | 74µs | 73µs | 33µs | 67µs | 85µs |
|  | 22 | 2ms | 1.8ms | 3.2ms | 470µs | 1ms | 1.95ms |

Electrical characteristics as specified  by the manufacturer’s  data sheet: FDS6670A: CISS = 3200pF, QT(MAX) = 50nC, RDS(ON) = 8.2m IRF7401: CISS = 1600pF, QT(MAX) = 48nC, RDS(ON) = 22m MMSF5N03HD: CISS = 1200pF, QT(MAX) = 21nC, RDS(ON) = 40m

12

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

1000

VREF

100

tCSPD (ms) = 0.2 · CSPD (nF)

R1

10

RESPONSE TIME (ms)

1                                                                                                                                                          NTC

ON

R2            0.6V

MAX4370

LOGIC CONTROL

0.1

0.01

0.01         0.1            1             10           100         1000

CSPD (nF)

R1 = R2 · (VREF / O.6 - 1)

R2 = VALUE OF THE NTC RESISTOR AT THE LIMIT  TEMPERATURE

VREF = ANY REFERENCE VOLTAGE AVAILABLE  OR VIN

Figure 8. Slow Comparator Response Time vs. CSPD

Figure 9. Temperature Monitoring and Protection

The actual turn-on time is determined by the longer of the two timings of Case A and Case B. Set the start-up timer (tSTART) at 2 · tON or longer to guarantee enough time for the output voltage to settle; also take into con- sideration device parameter variation.

Slow Comparator Response Time (CSPD) The slow comparator threshold is set at 50mV, and its response time is determined by the external capacitor connected to CSPD (Figure 8).

A minimum response time of 20µs (typ) is achieved by leaving this pin floating. This time is determined inter- nally and is not affected by stray capacitance at CSPD (up to 100pF).

Set the slow comparator response time to be longer than the normal operation load transients.

ON Comparator

The ON/OFF function of the MAX4370 is controlled by

RESET

VCC

BACKPLANE

REMOVABLE CARD

10k

VIN

ON

1M

VSEN     GATE

MAX4370

the  ON  comparator.  This  is  a  precision  voltage  com-

parator that can be used for temperature monitoring

(Figure 9) or as an additional undervoltage lockout. The

comparator threshold voltage is set at 0.6V with a 3mV

typical hysteresis.

The ON comparator initiates start-up when its input volt- age (VON) rises above the threshold voltage, and turns off the MOSFET when the voltage falls below the threshold. The ON comparator is also used to reset the MAX4370 after a fault condition.

The ON comparator input and the STAT output can be pulled to voltages up to 14V independently of VIN.

Figure 10. Fail-Safe Connector

In some applications, it is useful to use connectors with staggered  leads. In Figure 10, the ON pin forces the removable board to be powered up only when all con- nections are made.

13

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

Using the MAX4370 on  the Backplane The MAX4370 can be used on the backplane to regu- late current upon insertion of a removable card. This allows multiple cards with different input capacitance to be inserted into the same slot even if the card doesn’t have on-board hot-swap protection.

The MAX4370 current-limiting feature is active during the start-up period set by CTIM. The start-up period can be triggered  if VIN is connected  to ON through a trace on the card. Once tSTART has expired (timed out), the load capacitance has to be charged or a fault con- dition is detected. To ensure start-up with a fixed CTIM,

VIN

VIN

CTIM

VSEN      GATE

MAX4370

ON

BACKPLANE

REMOVABLE CARD

WITH NO HOT-INSERTION PROTECTION

VOUT

CBOARD

tSTART has to be longer than the time required  to                  charge  the board  capacitance.  The maximum load capacitance is calculated as follows:

CBOARD < tSTART · IFAST,SET / VIN

Input Transients The voltage at VIN must be above the UVLO during inrush and fault conditions. When a short condition occurs on the board, the fault current can be higher than the fast comparator current limit. The gate voltage

is discharged immediately, but note that the MOSFET is not completely off until VGS < VTH. If the main system power supply collapses below UVLO, the MAX4370 will force the device to restart once the supply has recov- ered. The main system power supply must be able to deliver this fault current without excessive voltage drop.

The MOSFET is turned off in a very short time; there- fore, the resulting di/dt can be considerable. The back- plane delivering the power to the external card must have a fairly low inductance  to limit the voltage tran- sients caused by the removal of a fault.

MOSFET Thermal Considerations During normal operation, the MOSFET dissipates little power; it is fully turned on and its RDS(ON) is minimal. The power dissipated  in normal operation is PD = (ILOAD)2 · RDS(ON). A considerable amount of power is dissipated  during the turn-on and turn-off transients. The design must take into consideration the worst-case scenario of a continuous short-circuit fault present on the board. Two cases must be considered:

1) The single turn-on with the device latched after a fault.

2) An external circuit forces a continuous automatic retry after the fault.

MOSFET manufacturers typically include the package normalized transient thermal resistance (rJA(t) or rJC(t)), which is determined by the start-up time and the retry duty cycle (d = tSTART / tRETRY). The following

Figure 11. Using the MAX4370 on the Backplane

equation is used to calculate  the required  transient thermal resistance:

RJA(t) = (TJ,MAX - TA) / PD,MAX(t)

where PDMAX(t) = VIN · IFAULT and the resulting RJA = RJA(t) / rJA(t). RJA is the thermal resistance deter- mined with a continuous load and by the layout or heatsink.

Layout Considerations To take full advantage of the switch response time to an output fault condition, it is important to keep all traces as short as possible and to maximize the high-current trace dimensions to reduce the effect of undesirable parasitic inductance.  Place the MAX4370 close to the card’s connector. Use a ground plane to minimize its impedance and inductance.

Minimize the current-sense resistor trace length (<10mm), and ensure accurate current sensing with Kelvin connections (Figure 12).

When the output is short circuited,  the voltage drop across the external MOSFET becomes large. Hence, the power dissipation across the switch increases, as does the die temperature. An efficient way to achieve good power dissipation on a surface-mount package is to lay out two copper pads directly under the package on both sides of the board. Connect the two pads to the ground plane through vias, and use enlarged cop- per mounting pads on the top side of the board.

14

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

HIGH-CURRENT PATH

MAX4370

SENSE RESISTOR

MAX4370

Figure 12. Kelvin Connections for the Current-Sense Resistors

Pin  Configuration

TOP VIEW

| VIN | MAX4370 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

VSEN GATE

GND

ON STAT CTIM

CSPD

SO

Chip Information

TRANSISTOR COUNT: 1792

15

SOICN.EPS

Current-Regulating Hot-Swap Controller with

DualSpeed/BiLevel Fault Protection

MAX4370

Package Information

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

16                                         Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  1999 Maxim Integrated  Products               Printed USA                 is a registered  trademark of Maxim Integrated  Products.