19-1202; Rev 4; 6/07

Low-Voltage, Single-Supply

Dual SPST/SPDT Analog Switches

General Description

The MAX4541–MAX4544 are precision,  dual analog switches designed to operate from a single +2.7V to +12V supply. Low power consumption (5µW) makes these parts ideal for battery-powered equipment. These switches offer low leakage currents (100pA max) and fast switching speeds (tON = 150ns max, tOFF = 100ns max).

When powered  from a +5V supply,  the MAX4541– MAX4544 offer 2Ω max matching between channels, 60Ω max on-resistance (RON), and 6Ω max RON flatness.

These switches also offer 5pC max charge injection and a minimum of 2000V ESD protection per Method 3015.7.

The MAX4541/MAX4542/MAX4543 are dual single-pole/ single-throw (SPST) devices and the MAX4544 is a single- pole/double-throw  (SPDT) device. The MAX4541 has two normally open (NO) switches and the MAX4542 has two normally closed (NC) switches. The MAX4543 has one NO and one NC switch and can be used as an SPDT, while the MAX4544 is an SPDT. The MAX4541/MAX4542/MAX4543 are available in tiny SOT23-8 and 8-pin TDFN packages, and the MAX4544 is available in a SOT23-6 package.

Applications

Features

♦ 60Ω max (33Ω typ) Low RON

♦ 2Ω max RON Matching Between Channels

♦ 6Ω max RON Flatness

♦ 5pC max Guaranteed Charge Injection

♦ +2.7V to +12V Single-Supply Operation

♦   < 5µW Low Power Consumption

♦ Low Leakage Current Over Temperature:

10nA max at +85°C

♦ Fast Switching: tON = 35ns, tOFF = 25ns

♦ Guaranteed Break-Before-Make

(MAX4543/MAX4544 Only)

♦ TTL/CMOS-Logic Compatible

♦ Pin Compatible with MAX323/MAX324/MAX325

♦ MAX4541/MAX4542/MAX4543 Available in SOT23-8

MAX4544 Available in SOT23-6

Ordering Information PIN- TOP PART TEMP RANGE PACKAGE MARK

MAX4541CUA 0°C to +70°C 8 µMAX® —

MAX4541CSA 0°C to +70°C 8 SO —

MAX4541CPA 0°C to +70°C 8 Plastic DIP —

MAX4541C/D 0°C to +70°C Dice* —

MAX4541EKA-T -40°C to +85°C 8 SOT23-8 AAAE

MAX4541ETA -40°C to +85°C 8 TDFN-EP** —

MAX4541EUA -40°C to +85°C 8 µMAX —

MAX4541ESA -40°C to +85°C 8 SO —

Battery-Operated Systems Sample-and-Hold Circuits Heads-Up Displays Guidance and Control

Systems

Audio and Video

Switching

Military Radios

Test Equipment

Communications Systems

+3V/+5V DACs and ADCs

PBX, PABX Cell Phones Ultrasound Pagers

Ordering Information  continued at end of data sheet.

*Dice are specified at TA = +25°C.

µMAX is a registered  trademark of Maxim Integrated  Products, Inc.

**EP = Exposed pad.

Pin  Configurations/Functional Diagrams/Truth Tables

TOP VIEW

MAX4544

MAX4544	MAX4544

PIN 6

6	5	4

NO    1

COM    2

NC    3

GND    4

8    V+

7   IN

6    N.C.

5    N.C.

IN    1

V+    2

| GND 3 MAX4544 |  |  |
| --- | --- | --- |
| LOGIC | NC | NO |
| 0 1 | ON OFF | OFF ON |

SOT23-6*

6    NO

5   COM

4    NC

AAAM

1	2	3

μMAX/SO/DIP/TDFN

*NOTE: SOT23-6 PACKAGE

HAS LETTERING NEAREST PIN 6

MAX4541–MAX4544

N.C. = NOT INTERNALLY CONNECTED

MAX4541/MAX4542/MAX4543

APPEAR AT END OF DATA SHEET.

SWITCHES SHOWN FOR "0" INPUT

ABSOLUTE MAXIMUM RATINGS

Voltage Referenced  to GND

V+ .........................................................................-0.3V to +13V IN_, COM_, NC_, NO_ (Note 1) ..................-0.3V  to (V+ + 0.3V) Continuous Current (any terminal)....................................±10mA

Peak Current, COM_, NO_, NC_

(pulsed at 1ms, 10% duty cycle max) ............................±20mA ESD per Method 3015.7 .................................................> 2000V Continuous Power Dissipation

SOT23-6 (derate 7.1mW/°C above +70°C)....................571mW SOT23-8 (derate 5.3mW/°C above +70°C)....................421mW

TDFN (derate 24.4mW/°C above +70°C).....................1951mW

µMAX (derate 4.10mW/°C above +70°C) ......................330mW Narrow SO (derate 5.88mW/°C above +70°C) ..............471mW Plastic DIP (derate 9.09mW/°C above +70°C) ..............727mW

Operating  Temperature  Ranges

MAX454_C_ _ .......................................................0°C to +70°C MAX454_E_ _.....................................................-40°C to +85°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

MAX4541–MAX4544

Note 1:   Signals on NC_, NO_, COM_, or IN_ exceeding V+ or V- are clamped by internal diodes. Limit forward diode current to maximum current rating.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS—Single +5V Supply

(V+ = +5V ±10%, GND = 0, VINH = 2.4V, VINL = 0.8V, TA = TMIN to TMAX, unless otherwise noted.) (Notes 2, 3)

| A MIN PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| ANALOG SWITCH |  |  |  |  |  |  |
| Analog Signal Range | VCOM_, VNO_, VNC_ | (Note 4) |  |  | 0 V+ | V |
| On-Resistance | RON | V+ = 4.5V, ICOM_ = 1.0mA, VNO_ or VNC_ = 3.5V | TA = +25°C |  | 30 60 | Ω |
|  |  |  | TA = TMIN to TMAX |  | 75 |  |
| On-Resistance Match Between Channels (Note 5) | ΔRON | V+ = 5V, ICOM_ = 1.0mA, VNO_ or VNC_ = 3.5V | TA = +25°C |  | 0.8 2 | Ω |
|  |  |  | TA = TMIN to TMAX |  | 4 |  |
| On-Resistance Flatness (Note 6) | RFLAT(ON) | V+ = 5V; ICOM_ = 1.0mA; VNO_ or VNC_ = 1V, 2V, 3V | TA = +25°C |  | 2 6 | Ω |
|  |  |  | TA = TMIN to TMAX |  | 8 |  |
| NO_ or NC_ Off-Leakage Current (Note 7) | INO(OFF) or INC(OFF) | V+ = 5.5V; VCOM_ = 1V, 4.5V; VNO_ or VNC_ = 4.5V, 1V | TA = +25°C |  | -0.1 ±0.01 +0.1 | nA |
|  |  |  | TA = TMIN to TMAX | C, E | -5 +5 |  |
| COM_ Off-Leakage Current (Note 7) | ICOM(OFF) | V+ = 5.5V; VCOM_ = 4.5V, 1V; VNO_ or VNC_ = 1V, 4.5V | TA = +25°C |  | -0.1 +0.1 | nA |
|  |  |  | TA = TMIN to TMAX | C, E | -5 +5 |  |
| COM_ On-Leakage Current (Note 7) | ICOM(ON) | V+ = 5.5V; VCOM_ = 1V, 4.5V or VNO_ or VNC_ = 1V, 4.5V, or floating | TA = +25°C |  | -0.2 +0.2 | nA |
|  |  |  | T = T to TMAX | C, E | -10 +10 |  |

MAX4541–MAX4544

ELECTRICAL CHARACTERISTICS—Single +5V Supply (continued)

(V+ = +5V ±10%, GND = 0, VINH = 2.4V, VINL = 0.8V, TA = TMIN to TMAX, unless otherwise noted.) (Notes 2, 3)

ON

OFF

| COM(ON) PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| DYNAMIC |  |  |  |  |  |
| Turn-On Time | t | VNO_ or VNC_ = 3V, Figure 2 | TA = +25°C | 35 100 | ns |
|  |  |  | TA = TMIN to TMAX | 240 |  |
| Turn-Off Time | t | VNO_ or VNC_ = 3V, Figure 2 | TA = +25°C | 25 75 | ns |
|  |  |  | TA = TMIN to TMAX | 150 |  |
| Break-Before-Make Time Delay (Note 4) | tD | MAX4543/MAX4544 only, RL = 300Ω, CL = 35pF |  | 2 10 | ns |
| Charge Injection (Note 4) | Q | CL = 1.0nF, VGEN = 0, RGEN = 0, TA = +25°C, Figure 4 |  | 1 5 | pC |
| Off-Isolation (Note 8) | OIRR | RL = 50Ω, CL = 5pF, f = 1MHz, TA = +25°C, Figure 5 |  | -76 | dB |
| Crosstalk (Note 9) |  | RL = 50Ω, CL = 5pF, f = 1MHz, TA = +25°C, Figure 6 |  | -90 | dB |
| NC_ or NO_ Capacitance | COFF | f = 1MHz, TA = +25°C, Figure 7 |  | 8 | pF |
| COM_ Off-Capacitance | CCOM(OFF) | f = 1MHz, TA = +25°C, Figure 7 |  | 8 | pF |
| COM_ On-Capacitance | C | f = 1MHz, TA = +25°C, Figure 7 | MAX4541/MAX4542/ MAX4543 | 13 | pF |
|  |  |  | MAX4544 | 20 |  |
| SUPPLY |  |  |  |  |  |
| Power-Supply Range | V+ |  |  | 2.7 12 | V |
| Positive Supply Current | I+ | V+ = 5.5V, VIN = 0 or V+, all channels on or off |  | -1 ±0.0001 +1 | µA |
| LOGIC INPUT |  |  |  |  |  |
| Input-Voltage Low | VINL |  |  | 0.8 | V |
| Input-Voltage High | VINH |  |  | 2.4 | V |

MAX4541–MAX4544

ELECTRICAL CHARACTERISTICS—Single +3.3V Supply

(V+ = +3.0V to +3.6V, GND = 0, VINH = 2.4V, VINL = 0.8V, TA = TMIN to TMAX, unless otherwise noted.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| ANALOG SWITCH |  |  |  |  |  |
| Analog Signal Range | VCOM_, VNO_, VNC_ | (Note 3) |  | 0 V+ | V |
| Channel On-Resistance | RON | V+ = 3V, ICOM_ = 1.0mA, VNO_ or VNC_ = 1.5V | TA = +25°C | 50 125 | Ω |
|  |  |  | TA = TMIN to TMAX | 275 |  |
| DYNAMIC |  |  |  |  |  |
| Turn-On Time (Note 3) | tON | VNO_ or VNC_ = 1.5V | TA = +25°C | 80 400 | ns |
|  |  |  | TA = TMIN to TMAX | 500 |  |
| Turn-Off Time (Note 3) | tOFF | VNO_ or VNC_ = 1.5V | TA = +25°C | 50 125 | ns |
|  |  |  | TA = TMIN to TMAX | 175 |  |
| Break-Before-Make Time Delay (Note 3) | tD | MAX4543/MAX4544 only, RL = 300Ω, CL = 35pF, TA = +25°C |  | 2 30 | ns |
| Charge Injection (Note 3) | Q | CL = 1.0nF, VGEN = 0, RGEN = 0, TA = +25°C |  | 1 5 | pC |
| SUPPLY |  |  |  |  |  |
| Positive Supply Current | I+ | V+ = 3.6V, VIN = 0 or V+, all channels on or off |  | -1 +1 | µA |

Note 2:   QFN and SOT-packaged parts are 100% tested at +25°C only and guaranteed by correlation at the full hot rated temperature.

Note 3:  The algebraic convention,  where the most negative value is a minimum and the most positive value a maximum, is used in this data sheet.

Note 4:  Guaranteed  by design.

Note 5:  ΔRON = ΔRON max - ΔRON min.

Note 6:  Flatness is defined as the difference  between the maximum and minimum value of on-resistance  as measured  over the specified analog signal range.

Note 7:  Leakage parameters  are 100% tested at maximum rated hot temperature  and guaranteed by correlation  at +25°C.

Note 8:  Off-isolation  = 20 x log10 [VCOM  ⁄  (VNC or VNO)], VCOM = output, VNC or VNO = input to off switch.

Note 9:  Between the two switches, MAX4541/MAX4542/MAX4543 only.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

MAX4541-01

| ON-RESISTANCE vs. COM_ VOLTAGE A: V+ = +3V B: V+ = +5V C: V+ = +12V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | A |  |  |  |  |
|  |  | B |  |  |  |
|  |  |  |  |  | C |

80

70

60

50

40

30

20

10

0

0	2	4	6	8	10	12

VCOM_ (V)

ON-RESISTANCE vs.

MAX4541-02

| COM VOLTAGE OVER TEMPERATURE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A |  |  |  |  |  |  |  |  |
|  | B C |  |  |  |  |  |  |  |  |
|  | D |  |  |  |  |  |  |  |  |
|  | E |  |  |  |  | A: TA = +125°C B: TA = +85°C C: TA = +25°C D: TA = -40°C E: TA = -55°C |  |  |  |

60

50

40

RON (Ω)

30

20

10

0

0	1	2	3	4	5

VCOM (V)

200

180

160

140

tON/tOFF (ns)

120

100

80

60

40

20

0

TURN-ON/OFF TIMES vs.

MAX4541-03

SUPPLY VOLTAGE

| V+ = +5V |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | tON |  |  |  |  |  |
| tOFF |  |  |  |  |  |  |

0	2	4	6	8	10	12

VSUPPLY (V)

100

90

80

70

60

50

40

30

20

10

0

TURN-ON/OFF TIMES

MAX4541-04

vs. TEMPERATURE

| V+ = +5V |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | tON |  |  |  |  |  |  |
|  |  |  |  |  |  | tOFF |  |  |

-55  -35  -15   5      25     45     65     85    105   125

TEMPERATURE (°C)

100

10

1

ON/OFF-LEAKAGE (nA)

0.1

0.01

0.001

ON/OFF-LEAKAGE CURRENT

vs. TEMPERATURE

V+ = +5.5V

ICOM(ON)

ICOM/IN(OFF)

THD (%)

-55	-30	-5	20     45	70      95    120 125

TEMPERATURE (°C)

CHARGE INJECTION vs. COM_ VOLTAGE

MAX4541-05

8

MAX4541-06

V- = +5V

7

6

Q (pC)

5

4

3

2

1

0

0	1	2	3	4	5

VCOM_ (V)

0

-10

-20

-30

-40

-50

-60

-70

-80

FREQUENCY RESPONSE

ON-LOSS

ON-PHASE

OFF-LOSS

MAX4541-07

225

180

135

PHASE (DEGREES)

90

I+ (nA)

45

0

-45

-90

-135

1000

900

800

700

600

500

400

300

200

MAX4541-08

| INPUT VOLTAGE vs. SUPPLY CURRENT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | IV+ |  |  |  |  |  |

TOTAL HARMONIC DISTORTION

MAX4541-09

vs. FREQUENCY

| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

MAX4541–MAX4544

tON/tOFF (ns)

RON (Ω)

LOSS (dB)

-90

-100

50Ω IN AND OUT V+ = +5V

0.1	1	10	100

FREQUENCY (MHz)

-180

-225

100

0

0	1	2	3	4	5

VIN (V)

0.1

10

100	1k	10k

FREQUENCY (Hz)

Pin  Description

NAME

| PIN |  |  |  |  |  |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAX4541 |  | MAX4542 |  | MAX4543 |  | MAX4544 |  |  |  |
| DIP/SO/ µMAX/ TDFN | SOT23-8 | DIP/SO/ µMAX/ TDFN | SOT23-8 | DIP/SO/ µMAX/ TDFN | SOT23-8 | DIP/SO/ µMAX/ TDFN | SOT23-6 |  |  |
| 1 | 1 | — | — | 1 | 1 | — | — | NO1 | Analog Switch 1—Normally Open |
| 2 | 8 | 2 | 8 | 2 | 8 | — | — | COM1 | Analog Switch 1—Common |
| 3 | 3 | 3 | 3 | 3 | 3 | — | — | IN2 | Digital Control Input 2 |
| 4 | 6 | 4 | 6 | 4 | 6 | 4 | 3 | GND | Ground—Negative Supply Input |
| 5 | 5 | — | — | — | — | — | — | NO2 | Analog Switch 2—Normally Open |
| 6 | 4 | 6 | 4 | 6 | 4 | — | — | COM2 | Analog Switch 2—Common |
| 7 | 7 | 7 | 7 | 7 | 7 | — | — | IN1 | Digital Control Input 1 |
| 8 | 2 | 8 | 2 | 8 | 2 | 8 | 2 | V+ | Positive Supply Voltage Input |
| — | — | 1 | 1 | — | — | — | — | NC1 | Analog Switch 1—Normally Closed |
| — | — | 5 | 5 | 5 | 5 | — | — | NC2 | Analog Switch 2—Normally Closed |
| — | — | — | — | — | — | 1 | 6 | NO | Analog Switch—Normally Open |
| — | — | — | — | — | — | 2 | 5 | COM | Analog Switch—Common |
| — | — | — | — | — | — | 3 | 4 | NC | Analog Switch—Normally Closed |
| — | — | — | — | — | — | 5, 6 | — | N.C. | Not Connected (Open Circuit) |
| — | — | — | — | — | — | 7 | 1 | IN | Digital Control Input |
| EP | — | EP | — | EP | — | EP | — | EP | Exposed Pad. Connect EP to V+. |

Applications Information 		Logic Levels The MAX4541–MAX4544 are TTL compatible  when powered  from a single +5V supply.  When powered

POSITIVE SUPPLY

D1

V+

NO	COM Vg

GND

D2

from other supply voltages, TTL compatibility  is guar- anteed and the logic  inputs can be driven rail-to-rail. For example, with a +12V supply, IN1 and IN2 can be driven low to 0 and high to 12V. With a +3.3V supply, IN1 and IN2 should be driven low to 0 and high to

3.3V.

Driving IN1 and IN2 rail-to-rail minimizes power con- sumption.

Analog Signal Levels Analog signals that range over the entire supply voltage (V+ to GND) can be switched with very little change in on-resistance  over the entire voltage range (see the Typical Operating Characteristics). All the switches are bidirectional,  so the NO_, NC_, and COM_ pins can be used as either inputs or outputs.

MAX4541–MAX4544

Figure 1. Overvoltage Protection Using Two External Blocking

Diodes

Power-Supply Sequencing and Overvoltage Protection

Do not exceed the absolute maximum ratings because

stresses beyond  the listed ratings may cause perma- nent damage to the devices.

Proper power-supply  sequencing  is recommended  for all CMOS devices.  Always apply V+ before applying analog signals or logic inputs, especially if the analog or logic signals are not current limited. If this sequenc- ing is not possible, and if the analog or logic inputs are not current limited to < 10mA, add a small-signal diode (D1) as shown in Figure 1. If the analog signal can dip below  GND, add  D2. Adding  protection  diodes reduces the analog signal range to a diode drop (about

0.7V) below V+ (for D1), and to a diode  drop above ground (for D2). Leakage is unaffected by adding  the diodes. On-resistance increases by a small amount at low supply  voltages. Maximum supply  voltage (V+) must not exceed 13V.

Adding  protection  diodes causes the logic thresholds to be shifted relative to the power-supply rails. This can be significant  when low supply voltages (+5V or less) are used. With a +5V supply,  TTL compatibility  is not guaranteed when protection diodes are added. Driving IN1 and IN2 all the way to the supply rails (i.e., to a diode drop higher than the V+ pin, or to a diode drop lower than the GND pin) is always acceptable.

Protection diodes  D1 and D2 also protect  against some overvoltage situations. With Figure 1’s circuit, if the supply voltage is below the absolute maximum rat- ing, and if a fault voltage up to the absolute maximum rating is applied  to an analog signal pin, no damage will result.

Test Circuits/Timing Diagrams

MAX4541

MAX4542

MAX4543

MAX4544

SWITCH INPUT

NO

or NC

V+

V+ COM

SWITCH OUTPUT

RL 	CL

VOUT

LOGIC INPUT

+3V

0

50%

tOFF

tr < 20ns tf < 20ns

LOGIC INPUT

IN

GND

0V

1kΩ

35pF

SWITCH	0

OUTPUT

VOUT

tON

0.9 x V0UT	0.9 x VOUT

CL INCLUDES FIXTURE AND STRAY CAPACITANCE. VOUT = VCOM  (    RL       )

RL + RON

LOGIC INPUT WAVEFORMS INVERTED FOR SWITCHES THAT HAVE THE OPPOSITE LOGIC SENSE.

MAX4541–MAX4544

Figure 2. Switching Time

MAX4543

+3V

LOGIC INPUT

NO1

NC2

IN1

IN2

GND

V+

V+

COM1

COM2

RL2

300Ω

VOUT2

CL2

35pF

RL1

300Ω

VOUT1

CL1

35pF

LOGIC INPUT

SWITCH OUTPUT 1 (VOUT1)

SWITCH OUTPUT 2 (VOUT2)

+3V

0

0

50%

0.9 x V0UT1

0.9 x VOUT2

CL INCLUDES FIXTURE AND STRAY CAPACITANCE.

0 	tD	tD

Figure 3a. Break-Before-Make Interval (MAX4543 Only)

MAX4544

+3V

V+

V+

NC 	V

LOGIC INPUT

+3V

0

50%

COM NO

IN

RL

300Ω

OUT

CL

35pF

LOGIC INPUT

GND

CL INCLUDES FIXTURE AND STRAY CAPACITANCE.

SWITCH OUTPUT (VOUT)

0.9 x VOUT

tD

Figure 3b. Break-Before-Make Interval (MAX4544 Only)

MAX4541

MAX4542

MAX4543

RGEN

V+

V+	VOUT

ΔVOUT

MAX4544

VGEN

NC

OR NO

GND

COM

CL

IN_

VOUT

IN

OFF

OFF ON

VIN = +3V

OFF IN

ON

Q = (ΔVOUT)(CL)

OFF

MAX4541–MAX4544

IN DEPENDS ON SWITCH CONFIGURATION;

INPUT POLARITY DETERMINED BY SENSE OF SWITCH.

Figure 4. Charge Injection

Test Circuits/Timing Diagrams (continued)

SIGNAL GENERATOR 0dBm

10nF    V+

V+ COM

MAX4541

MAX4542

MAX4543

MAX4544 	SIGNAL GENERATOR 0dBm

10nF     V+

V+ COM1

MAX4541

MAX4542

MAX4543

NO1	50Ω

ANALYZER

RL

NC

OR NO GND

0V OR IN	2.4V

0 OR 2.4V	IN1

ANALYZER	NO2

RL

GND

IN2	 	 COM2

0 OR

2.4V NC

Figure 5. Off-Isolation

Figure 6. Crosstalk

10nF	V+

V+ COM

MAX4541

MAX4542

MAX4543

MAX4544

CAPACITANCE

METER	NC OR NO

IN 	0 OR

2.4V

f = 1MHz

GND

MAX4541–MAX4544

Figure 7. Channel Off/On-Capacitance

MAX4541–MAX4544

PART TEMP RANGE PIN- TOP PACKAGE MARK

MAX4541EPA -40°C to +85°C 8 Plastic DIP —

MAX4542CUA 0°C to +70°C 8 µMAX —

MAX4542CSA 0°C to +70°C 8 SO —

MAX4542CPA 0°C to +70°C 8 Plastic DIP —

MAX4542C/D 0°C to +70°C Dice* —

MAX4542EKA-T -40°C to +85°C 8 SOT23-8 AAAF

MAX4542ETA -40°C to +85°C 8 TDFN-EP** —

MAX4542EUA -40°C to +85°C 8 µMAX —

MAX4542ESA -40°C to +85°C 8 SO —

MAX4542EPA -40°C to +85°C 8 Plastic DIP —

MAX4543CUA 0°C to +70°C 8 µMAX —

MAX4543CSA 0°C to +70°C 8 SO —

MAX4543CPA 0°C to +70°C 8 Plastic DIP —

MAX4543C/D 0°C to +70°C Dice* —

MAX4543EKA-T -40°C to +85°C 8 SOT23-8 AAAG

MAX4543ETA -40°C to +85°C 8 TDFN-EP** —

MAX4543EUA -40°C to +85°C 8 µMAX —

MAX4543ESA -40°C to +85°C 8 SO —

MAX4543EPA -40°C to +85°C 8 Plastic DIP —

MAX4544CUA 0°C to +70°C 8 µMAX —

MAX4544CSA 0°C to +70°C 8 SO —

MAX4544CPA 0°C to +70°C 8 Plastic DIP —

MAX4544C/D 0°C to +70°C Dice* —

MAX4544EUT-T -40°C to +85°C 6 SOT23-6 AAAM

MAX4544ETA -40°C to +85°C 8 TDFN-EP** —

MAX4544EUA -40°C to +85°C 8 µMAX —

MAX4544ESA -40°C to +85°C 8 SO —

MAX4544EPA -40°C to +85°C 8 Plastic DIP —

*Dice are specified at TA = +25°C.

**EP = Exposed pad.

TOP VIEW

MAX4541

MAX4542

MAX4543

NO1    1

COM1    2

IN2     3

GND    4

8     V+

7     IN1

6     COM2

5     NO2

NC1    1

COM1    2

IN2     3

GND    4

8     V+

7     IN1

6     COM2

5     NC2

NO1    1

COM1    2

IN2     3

GND    4

8     V+

7     IN1

6     COM2

5     NC2

μMAX/SO/DIP/TDFN

μMAX/SO/DIP/TDFN

μMAX/SO/DIP/TDFN

MAX4541

MAX4542

MAX4543

NO1    1

V+    2

IN2     3

COM2     4

8     COM1

7     IN1

6     GND

5     NO2

NC1    1

V+    2

IN2     3

COM2     4

8     COM1

7     IN1

6     GND

5     NC2

NO1    1

V+    2

IN2     3

COM2     4

8     COM1

7     IN1

6     GND

5     NC2

MAX4541–MAX4544

SOT23-8

SOT23-8

SWITCHES SHOWN FOR LOGIC "0" INPUT

SOT23-8

MAX4541–MAX4544

6, 8, &10L, DFN THIN.EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

MAX4541–MAX4544

| PACKAGE VARIATIONS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PKG. CODE | N | D2 | E2 | e | JEDEC SPEC | b | [(N/2)-1] x e |
| T633-2 | 6 | 1.50±0.10 | 2.30±0.10 | 0.95 BSC | MO229 / WEEA | 0.40±0.05 | 1.90 REF |
| T833-2 | 8 | 1.50±0.10 | 2.30±0.10 | 0.65 BSC | MO229 / WEEC | 0.30±0.05 | 1.95 REF |
| T833-3 | 8 | 1.50±0.10 | 2.30±0.10 | 0.65 BSC | MO229 / WEEC | 0.30±0.05 | 1.95 REF |
| T1033-1 | 10 | 1.50±0.10 | 2.30±0.10 | 0.50 BSC | MO229 / WEED-3 | 0.25±0.05 | 2.00 REF |
| T1033-2 | 10 | 1.50±0.10 | 2.30±0.10 | 0.50 BSC | MO229 / WEED-3 | 0.25±0.05 | 2.00 REF |
| T1433-1 | 14 | 1.70±0.10 | 2.30±0.10 | 0.40 BSC | - - - - | 0.20±0.05 | 2.40 REF |
| T1433-2 | 14 | 1.70±0.10 | 2.30±0.10 | 0.40 BSC | - - - - | 0.20±0.05 | 2.40 REF |

| COMMON DIMENSIONS |  |  |
| --- | --- | --- |
| SYMBOL | MIN. | MAX. |
| A | 0.70 | 0.80 |
| D | 2.90 | 3.10 |
| E | 2.90 | 3.10 |
| A1 | 0.00 | 0.05 |
| L | 0.20 | 0.40 |
| k | 0.25 MIN. |  |
| A2 | 0.20 REF. |  |

MAX4541–MAX4544

6LSOT.EPS

PACKAGE OUTLINE, SOT 6L BODY

1

21-0058	I	2

MAX4541–MAX4544

PACKAGE OUTLINE, SOT 6L BODY

2

21-0058	I	2

MAX4541–MAX4544

MARKING

SOT23, 8L.EPS

0

0

PACKAGE OUTLINE, SOT-23, 8L BODY

1

21-0078	G	1

Revision History

Pages changed at Rev 4: 1–16

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

16  	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 408-737-7600

©  2007 Maxim Integrated  Products 	is a registered  trademark  of Maxim Integrated  Products, Inc.