19-2627; Rev 2; 9/05

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

General Description

The MAX4717/MAX4718 low-voltage, low on-resistance (RON), dual single-pole/double  throw (SPDT) analog switches operate from a single +1.8V to +5.5V supply. These devices  are designed  for USB 1.1 and audio switching applications.

The MAX4717 features two 4.5Ω RON  (max) SPDT switches with 1.2Ω flatness and 0.3Ω matching between channels. The MAX4718 features one 4.5Ω RON (max) SPDT switch and one 20Ω RON (max) SPDT switch. The

20Ω switch has a guaranteed matching and flatness of

0.4Ω and 1.2Ω, respectively. These switches offer break- before-make switching (1ns) with tON  <80ns and tOFF

<40ns at +2.7V. The digital logic inputs are +1.8V logic compatible with a +2.7V to +3.6V supply.

These switches are packaged  in a chip-scale  package (UCSP™), significantly reducing  the required PC board area. The chip occupies  only a 2.0mm ✕  1.50mm area and has a 4 ✕ 3 bump array with a bump pitch of 0.5mm. These switches are also available in 10-pin µMAX® and

10-pin TDFN packages.

Applications

USB 1.1 Signal Switching Circuits Battery-Operated Equipment Audio/Video-Signal Routing Headphone Switching

Low-Voltage Data-Acquisition Systems

Sample-and-Hold Circuits

Cell Phones

PDAs

UCSP is a trademark of Maxim Integrated  Products, Inc.

µMAX is a registered trademark of Maxim Integrated Products, Inc.

MAX4717/MAX4718

Features

♦ USB 1.1 Signal Switching Compliant

(TID = 4000231)

♦ 2ns (max) Differential Skew

♦ -3dB Bandwidth: > 300MHz

♦ Low 15pF On-Channel Capacitance

♦ Single-Supply Operation from +1.8V to +5.5V

♦ 4.5Ω RON (max) Switches (MAX4717/MAX4718)

0.3Ω (max) RON Match (+3.0V Supply)

1.2Ω (max) Flatness (+3.0V Supply)

♦ 20Ω RON (max) Switch (MAX4718)

0.4Ω (max) RON Match (+3.0V Supply)

1.2Ω (max) Flatness (+3.0V Supply)

♦ Rail-to-Rail Signal Handling

♦ High Off-Isolation: -55dB (10MHz)

♦ Low Crosstalk: -80dB (10MHz)

♦ Low Distortion: 0.03%

♦ +1.8V CMOS-Logic Compatible

♦ < 0.5nA Leakage Current at +25°C

Ordering Information

PIN/BUMP- TOP PART TEMP RANGE PACKAGE MARK

MAX4717EUB -40°C to +85°C 10 µMAX —

MAX4717ETB -40°C to +85°C 10 TDFN-EP* ACV

MAX4717EBC-T -40°C to +85°C 12 UCSP-12 ABH

MAX4718EUB -40°C to +85°C 10 µMAX —

MAX4718ETB -40°C to +85°C 10 TDFN-EP* ACW

MAX4718EBC-T -40°C to +85°C 12 UCSP-12 ABI

*EP = Exposed paddle.

Pin  Configurations/Functional Diagrams/Truth Tables

TOP VIEW

(BUMP SIDE DOWN)

MAX4717/MAX4718

GND

MAX4717/MAX4718

MAX4717/MAX4718

NC1      C1                B1

A1      NC2

V+    1

10    NO2

V+    1                                           10    NO2

IN1      C2

COM1      C3

A2      IN2

A3      COM2

NO1    2

COM1    3

IN1    4

9    COM2

8    IN2

7    NC2

| NO1 2 PART | SPDT1 | SPDT2 |
| --- | --- | --- |
| MAX4717 | 4.5Ω | 4.5Ω |
| MAX4718 | 4.5Ω | 20Ω |

COM1    3

IN1    4

9     COM2

8     IN2

7     NC2

NO1

C4                B4                A4

V+

UCSP

NO2

NC1    5

TDFN

6    GND

| SWITCHES SHOWN FOR LOGIC "0" INPUT MAX4717/MAX4718 |  |  |
| --- | --- | --- |
| IN_ | NO_ | NC_ |
| 0 | OFF | ON |
| 1 | ON | OFF |

NC1    5

µMAX

6     GND

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

ABSOLUTE MAXIMUM RATINGS

(All voltages are referenced to GND.)

V+, IN_...................................................................-0.3V to +6.0V COM_, NO_, NC_ (Note 1) ...........................-0.3V to (V+ + 0.3V) Continuous Current COM_, NO_, NC_ ...........................±100mA Peak Current COM_, NO_, NC_

(pulsed at 1ms, 10% duty cycle) ................................±200mA Continuous Power Dissipation  (TA = +70°C)

10-Pin µMAX (derate 5.6mW/°C above +70°C) ...........444mW

10-Pin TDFN (derate 24.4mW/°C above +70°C) .......1951mW

12-Bump UCSP (derate 11.4mW/°C above +70°C) ....909mW

ESD Method 3015.7 .............................................................>2kV Operating  Temperature  Range ...........................-40°C to +85°C Junction Temperature ......................................................+150°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C Bump Temperature  (soldering)

Infrared (15s) ...............................................................+220°C Vapor Phase (60s) .......................................................+215°C

Note 1: Signals  on COM_, NO_, or NC_ exceeding V+ or GND  are clamped by internal  diodes.  Limit forward-diode current  to maximum current rating.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS—Single +3V Supply

(V+ = +2.7V  to +3.6V,  VIH = +1.4V,  VIL = +0.5V,  TA = TMIN to TMAX, unless otherwise  noted.  Typical  values are at V+ = +3.0V, TA = +25°C, unless otherwise noted.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Analog Signal Range | VCOM_, VNO_, VNC_ |  | TMIN to TMAX | 0 V+ | V |
| ANALOG SWITCH (Low RON—MAX4717/MAX4718 SPDT 1) |  |  |  |  |  |
| On-Resistance (Note 4) | RON | V+ = 2.7V, ICOM_ = 10mA; VNO_ or VNC_ = 1.5V | +25°C | 3.0 4.5 | Ω |
|  |  |  | TMIN to TMAX | 5 |  |
| On-Resistance Match Between Channels (Notes 4, 5) | ∆RON | V+ = 2.7V, ICOM_ = 10mA; VNO_ or VNC_ = 1.5V | +25°C | 0.1 0.3 | Ω |
|  |  |  | TMIN to TMAX | 0.4 |  |
| On-Resistance Flatness (Note 6) | RFLAT(ON) | V+ = 2.7V, ICOM_ = 10mA; VNO_ or VNC_ = 1.0V, 1.5V, 2.0V | +25°C | 0.6 1.2 | Ω |
|  |  |  | TMIN to TMAX | 1.5 |  |
| NO_, NC_ Off-Leakage Current (Note 7) | INO_(OFF), INC_(OFF) | V+ = 3.6V, VCOM_ = 0.3V, 3.3V; VNO_ or VNC_ = 3.3V, 0.3V | +25°C | -0.5 +0.01 +0.5 | nA |
|  |  |  | TMIN to TMAX | -1 +1 |  |
| COM_ On-Leakage Current (Note 7) | ICOM_(ON) | V+ = 3.6V, VCOM_ = 0.3V, 3.3V; VNO_ or VNC_ = 0.3V, 3.3V, or floating | +25°C | -1 +0.01 +1 | nA |
|  |  |  | TMIN to TMAX | -2 +2 |  |
| ANALOG SWITCH (High RON—MAX4718 SPDT 2) |  |  |  |  |  |
| On-Resistance (Note 4) | RON | V+ = 2.7V, ICOM_ = 10mA; VNO_ or VNC_ = 1.5V | +25°C | 15 20 | Ω |
|  |  |  | TMIN to TMAX | 25 |  |

2

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

ELECTRICAL CHARACTERISTICS—Single +3V Supply (continued)

(V+ = +2.7V  to +3.6V,  VIH = +1.4V,  VIL = +0.5V,  TA = TMIN to TMAX, unless otherwise  noted.  Typical  values are at V+ = +3.0V, TA = +25°C, unless otherwise noted.) (Notes 2, 3)

| MIN PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| On-Resistance Match Between Channels (Notes 4, 5) | ∆RON | V+ = 2.7V, ICOM_ = 10mA; VNO_ or VNC_ = 1.5V | +25°C | 0.15 0.4 | Ω |
|  |  |  | TMIN to TMAX | 0.5 |  |
| On-Resistance Flatness (Note 6) | RFLAT(ON) | V+ = 2.7V, ICOM_ = 10mA; VNO_ or VNC_ = 1.0V, 1.5V, 2.0V | +25°C | 0.6 1.2 | Ω |
|  |  |  | TMIN to TMAX | 1.5 |  |
| NO_, NC_ Off-Leakage Current (Note 7) | INO_(OFF), INC_(OFF) | V+ = 3.6V, VCOM_ = 0.3V, 3.3V; VNO_ or VNC_ = 3.3V, 0.3V | +25°C | -0.5 +0.01 +0.5 | nA |
|  |  |  | TMIN to TMAX | -1 +1 |  |
| COM_ On-Leakage Current (Note 7) | ICOM_(ON) | V+ = 3.6V, VCOM_ = 0.3V, 3.3V; VNO_ or VNC_ = 0.3V, 3.3V, or floating | +25°C | -1 +0.01 +1 | nA |
|  |  |  | T to TMAX | -2 +2 |  |
| DYNAMIC CHARACTERISTICS |  |  |  |  |  |
| Turn-On Time | tON | VNO_, VNC_ = 1.5V; RL = 300Ω, CL = 35pF, Figure 1; VIH = 1.5V, VIL = 0V | +25°C | 40 80 | ns |
|  |  |  | TMIN to TMAX | 100 |  |
| Turn-Off Time | tOFF | VNO_, VNC_ = 1.5V; RL = 300Ω, CL = 35pF, Figure 1; VIH = 1.5V, VIL = 0V | +25°C | 20 40 | ns |
|  |  |  | TMIN to TMAX | 50 |  |
| Break-Before-Make Time Delay (Note 7) | tBBM | VNO_, VNC_ = 1.5V; RL = 300Ω, CL = 35pF, Figure 2 | +25°C | 8 | ns |
|  |  |  | TMIN to TMAX | 1 |  |
| Skew (Note 7) | tSKEW | RS = 39Ω, CL = 50pF, Figure 3 | TMIN to TMAX | 0.15 2 | ns |
| Charge Injection | Q | VGEN = 1.5V, RGEN = 0Ω, CL = 1.0nF, Figure 4 | +25°C | 5 | pC |
| Off-Isolation | VISO | f = 10MHz; VNO_, VNC_ = 1VP-P; RL = 50Ω, CL = 5pF, Figure 5 | +25°C | -55 | dB |
|  |  | f = 1MHz; VNO_, VNC_ = 1VP-P; RL = 50Ω, CL = 5pF, Figure 5 |  | -80 |  |
| Crosstalk (Note 8) | VCT | f = 10MHz; VNO_, VNC_ = 1VP-P; RL = 50Ω, CL = 5pF, Figure 5 | +25°C | -80 | dB |
|  |  | f = 1MHz; VNO_, VNC_ = 1VP-P; RL = 50Ω, CL = 5pF, Figure 5 |  | -110 |  |
| On-Channel -3dB Bandwidth | BW | Signal = 0dBm, RL = 50Ω, CL = 5pF, Figure 5 | +25°C | >300 | MHz |
| Total Harmonic Distortion | THD | VCOM_ = 2VP-P, RL = 600Ω | +25°C | 0.03 | % |
| NO_, NC_ Off-Capacitance | CNO_(OFF), CNC_(OFF) | f = 1MHz, Figure 6 | +25°C | 9 | pF |

3

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

ELECTRICAL CHARACTERISTICS—Single +3V Supply (continued)

(V+ = +2.7V  to +3.6V,  VIH = +1.4V,  VIL = +0.5V,  TA = TMIN to TMAX, unless otherwise  noted.  Typical  values are at V+ = +3.0V, TA = +25°C, unless otherwise noted.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Switch On-Capacitance | C(ON) | f = 1MHz, Figure 6 | +25°C | 15 | pF |
| DIGITAL I/O |  |  |  |  |  |
| Input Logic High Voltage | VIH |  | TMIN to TMAX | 1.4 | V |
| Input Logic Low Voltage | VIL |  | TMIN to TMAX | 0.5 | V |
| Input Leakage Current | IIN | V+ = +3.6V, VIN_ = 0 or 5.5V | TMIN to TMAX | -100 +100 | nA |
| POWER SUPPLY |  |  |  |  |  |
| Power-Supply Range | V+ |  | TMIN to TMAX | 1.8 5.5 | V |
| Supply Current | I+ | V+ = +5.5V, VIN_ = 0V or V+ | TMIN to TMAX | 1 | µA |

ELECTRICAL CHARACTERISTICS—Single +5V Supply

(V+ = +4.2V  to +5.5V,  VIH = +2.0V,  VIL = +0.8V,  TA = TMIN to TMAX, unless otherwise  noted.  Typical  values are at V+ = +5.0V, TA = +25°C, unless otherwise noted.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Analog Signal Range | VCOM_, VNO_, VNC_ |  | TMIN to TMAX | 0 V+ | V |
| ANALOG SWITCH (Low RON—MAX4717/MAX4718 SPDT 1) |  |  |  |  |  |
| On-Resistance (Note 4) | RON | V+ = 4.2V, ICOM_ = 10mA; VNO_ or VNC_ = 3.5V | +25°C | 1.7 3 | Ω |
|  |  |  | TMIN to TMAX | 3.5 |  |
| On-Resistance Match Between Channels (Notes 4, 5) | ∆RON | V+ = 4.2V, ICOM_ = 10mA; VNO_ or VNC_ = 3.5V | +25°C | 0.1 0.3 | Ω |
|  |  |  | TMIN to TMAX | 0.4 |  |
| On-Resistance Flatness (Note 6) | RFLAT(ON) | V+ = 4.2V, ICOM_ = 10mA; VNO_ or VNC_ = 1.0V, 2.0V, 3.5V | +25°C | 0.4 1.2 | Ω |
|  |  |  | TMIN to TMAX | 1.5 |  |
| NO_, NC_ Off-Leakage Current (Note 7) | INO_(OFF), INC_(OFF) | V+ = 5.5V; VCOM_ = 1.0V, 4.5V; VNO_ or VNC_ = 1.0V, 4.5V | +25°C | -0.5 +0.01 +0.5 | nA |
|  |  |  | TMIN to TMAX | -1 +1 |  |
| COM_ On-Leakage Current (Note 7) | ICOM_(ON) | V+ = 5.5V; VCOM_ = 1.0V, 4.5V; VNO_ or VNC_ = 1.0V, 4.5V, or floating | +25°C | -1 +0.01 +1 | nA |
|  |  |  | TMIN to TMAX | -2 +2 |  |

4

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

ELECTRICAL CHARACTERISTICS—Single +5V Supply (continued)

(V+ = +4.2V  to +5.5V,  VIH = +2.0V,  VIL = +0.8V,  TA = TMIN to TMAX, unless otherwise  noted.  Typical  values are at V+ = +5.0V, TA = +25°C, unless otherwise noted.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| ANALOG SWITCH (High RON—MAX4718 SPDT 2) |  |  |  |  |  |
| On-Resistance (Note 4) | RON | V+ = 4.2V, ICOM_ = 10mA; VNO_ or VNC_ = 3.5V | +25°C | 12 20 | Ω |
|  |  |  | TMIN to TMAX | 25 |  |
| On-Resistance Match Between Channels (Notes 4, 5) | ∆RON | V+ = 4.2V, ICOM_ = 10mA; VNO_ or VNC_ = 3.5V | +25°C | 0.15 0.4 | Ω |
|  |  |  | TMIN to TMAX | 0.5 |  |
| On-Resistance Flatness (Note 6) | RFLAT(ON) | V+ = 4.2V, ICOM_ = 10mA; VNO_ or VNC_ = 1.0V, 2.0V, 4.5V | +25°C | 0.4 1.2 | Ω |
|  |  |  | TMIN to TMAX | 1.5 |  |
| NO_, NC_ Off-Leakage Current (Note 7) | INO_(OFF), INC_(OFF) | V+ = 5.5V; VCOM_ = 1.0V, 4.5V; VNO_ or VNC_= 1.0V, 4.5V | +25°C | -0.5 +0.01 +0.5 | nA |
|  |  |  | TMIN to TMAX | -1 +1 |  |
| COM_ On-Leakage Current (Note 7) | ICOM_(ON) | V+ = 5.5V, VCOM_ = 1.0V, 4.5V; VNO_ or VNC_ = 1.0V, 4.5V, or floating | +25°C | -1 +0.01 +1 | nA |
|  |  |  | TMIN to TMAX | -2 +2 |  |
| DYNAMIC CHARACTERISTICS |  |  |  |  |  |
| Turn-On Time | tON | VNO_, VNC_ = 3.0V; RL = 300Ω, CL = 35pF, Figure 1 | +25°C | 30 80 | ns |
|  |  |  | TMIN to TMAX | 100 |  |
| Turn-Off Time | tOFF | VNO_, VNC_ = 3.0V; RL = 300Ω, CL = 35pF, Figure 1 | +25°C | 20 40 | ns |
|  |  |  | TMIN to TMAX | 50 |  |
| Break-Before-Make Time Delay (Note 7) | tBBM | VNO_, VNC_ = 3.0V; RL = 300Ω, CL = 35pF, Figure 2 | +25°C | 8 | ns |
|  |  |  | TMIN to TMAX | 1 |  |
| Skew (Note 7) | tSKEW | RS = 39Ω, CL = 50pF, Figure 3 | TMIN to TMAX | 0.15 2 | ns |
| DIGITAL I/O |  |  |  |  |  |
| Input Logic High Voltage | VIH |  | TMIN to TMAX | 2.0 | V |
| Input Logic Low Voltage | VIL |  | TMIN to TMAX | 0.8 | V |
| Input Leakage Current | IIN | V+ = 5.5V, VIN_ = 0V or V+ | TMIN to TMAX | -100 +100 | nA |

5

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

ELECTRICAL CHARACTERISTICS—Single +5V Supply (continued)

(V+ = +4.2V  to +5.5V,  VIH = +2.0V,  VIL = +0.8V,  TA = TMIN to TMAX, unless otherwise  noted.  Typical  values are at V+ = +5.0V, TA = +25°C, unless otherwise noted.) (Notes 2, 3)

| PARAMETER | SYMBOL | CONDITIONS | TA | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| POWER SUPPLY |  |  |  |  |  |
| Power-Supply Range | V+ |  | TMIN to TMAX | 1.8 5.5 | V |
| Supply Current | I+ | V+ = 5.5V, VIN_ = 0V or V+ | TMIN to TMAX | 1 | µA |

Note 2:    UCSP and TDFN parts are 100% tested at +25°C only, and guaranteed by design over the specified temperature  range.

µMAX parts are 100% tested at TMAX and guaranteed by design over the specified temperature  range.

Note 3:    The algebraic convention used in this data sheet is where the most negative value is a minimum and the most positive

value is a maximum.

Note 4:    Guaranteed  by design for UCSP and TDFN parts.

Note 5:    ∆RON = RON(MAX) - RON(MIN).

Note 6:    Flatness is defined as the difference  between the maximum and minimum value of on-resistance  as measured  over the specified analog signal ranges.

Note 7:    Guaranteed  by design.

Note 8:    Between any two switches.

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

MAX4717/18 toc01

ON-RESISTANCE vs. COM_ VOLTAGE

10

LOW RON SWITCH

8

V+ = 1.8V

MAX4717/18 toc02

ON-RESISTANCE vs. COM_ VOLTAGE

6

V+ = 3V

LOW RON SWITCH

5

ON-RESISTANCE vs. COM_ VOLTAGE

5

V+ = 5V

LOW RON SWITCH

4

RON (Ω)

6

V+ = 2.5V

4                                     V+ = 3V  V+ = 4.2V

V+ = 5V

2

RON (Ω)

4

3         TA = +25°C

2

TA = +85°C

RON (Ω)

3

TA = +25°C

2

1

MAX4717/18 toc03

TA = +85°C

TA = -40°C

0                                                                                          1

TA = -40°C

0

0           1           2           3           4           5

0        0.5

1.0

1.5

2.0

2.5

3.0

0           1           2           3           4           5

VCOM_ (V)

VCOM_ (V)

VCOM_ (V)

6

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

Typical Operating Characteristics (continued)

(TA = +25°C, unless otherwise noted.)

ON-RESISTANCE vs. COM_  VOLTAGE

20

MAX4717/18 toc04

HIGH RON SWITCH

ON-RESISTANCE vs. COM_ VOLTAGE

15

V+ = 3V

MAX4717/18 toc06

ON-RESISTANCE vs. COM_ VOLTAGE

MAX4717/18 toc05

15

V+ = 5V

V+ = 1.8V

18

HIGH RON SWITCH

14

TA = +85°C

HIGH RON SWITCH

14

RON (Ω)

RON (Ω)

16                                                                                         13

V+ = 2.5V

TA = +85°C

RON (Ω)

13

14                                                V+ = 4.2V                                   12                                                                                         12

12

V+ = 5V

10

11                  TA = +25°C

10

TA = -40°C

11

TA = -40°C

10

TA = +25°C

0           1           2           3           4           5

0        0.5

1.0

1.5

2.0

2.5

3.0

0           1           2           3           4           5

VCOM_ (V)

VCOM_ (V)

VCOM_ (V)

500

LEAKAGE CURRENT (pA)

400

MAX4717/18 toc07

LEAKAGE CURRENT vs. TEMPERATURE

V+ = 3V

LOW RON SWITCH

1000

LEAKAGE CURRENT (pA)

800

LEAKAGE CURRENT vs. TEMPERATURE

V+ = 5V

LOW RON SWITCH

MAX4717/18 toc08

700

LEAKAGE CURRENT (pA)

500

LEAKAGE CURRENT vs. TEMPERATURE

V+ = 3V

HIGH RON SWITCH

MAX4717/18 toc09

COM_ ON-LEAKAGE

300

200

COM_ ON-LEAKAGE

COM_ OFF-LEAKAGE

600

400

COM_ ON-LEAKAGE COM_ OFF-LEAKAGE

300

COM_ OFF-LEAKAGE

100

200

100

0

-40

-15         10         35

CHARGE INJECTION (pC)

60         85

0

-40

-15         10         35

60         85

-100

-40

-15         10         35

60         85

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

900

LEAKAGE CURRENT (pA)

700

500

MAX4717/18 toc10

LEAKAGE CURRENT vs. TEMPERATURE

V+ = 5V

HIGH RON SWITCH

COM_ ON-LEAKAGE

CHARGE INJECTION vs. VCOM

50

40

CL = 1nF V+ = 5V

30

MAX4717/18 toc12

| SUPPLY CURRENT vs. TEMPERATURE |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | V+ = 5V |  |
|  |  | V+ = 3V |  |  |

MAX4717/18 toc11

6

5

SUPPLY CURRENT (nA)

4

3

300

100

COM_ OFF-LEAKAGE

20                  CL = 1nF

V+ = 3V                                                                              2

10                                                                                           1

-100

-40

-15         10         35

60         85

0

0           1           2           3           4           5

0

-40

-15         10         35

60         85

TEMPERATURE (°C)

VCOM_ (V)

TEMPERATURE (°C)

7

| 4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | V+ = 5V |  |  |
|  | V+ = 3V |  |  |  |

MAX4717/MAX4718

SUPPLY CURRENT (µA)

Typical Operating Characteristics (continued)

(TA = +25°C, unless otherwise noted.)

100

SUPPLY CURRENT vs. LOGIC LEVEL

MAX4717/18 toc13

2.0

| LOGIC THRESHOLD vs. SUPPLY VOLTAGE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | VTH+ |  |
|  |  |  |  |  |  | VTH- |  |

| 100 |  | LOW RON SWITCH |  |
| --- | --- | --- | --- |
|  |  | tON |  |
|  | tOFF |  |  |

TURN-ON/OFF TIME

MAX4717/18 toc15

vs. SUPPLY VOLTAGE

80                                                                                         1.6                                                                                        80

LOGIC THRESHOLD (V)

tON/tOFF (ns)

60                                                                                         1.2                                                                                        60

40                                                                                         0.8                                                                                        40

MAX4717/18 toc14

20                                                                                         0.4                                                                                        20

0

0           1           2           3           4           5

0

1.5

2.0

2.5

3.0

3.5

4.0

4.5

5.0

5.5

0

1.5

2.5

3.5

4.5

5.5

LOGIC LEVEL (V)

SUPPLY VOLTAGE (V)

SUPPLY VOLTAGE (V)

100

TURN-ON/OFF TIME

vs. SUPPLY VOLTAGE

TURN-ON/OFF TIME

vs. TEMPERATURE

MAX4717/18 toc16

60

MAX4717/18 toc17

LOW RON SWITCH

TURN-ON/OFF TIME

vs. TEMPERATURE

60

MAX4717/18 toc18

HIGH RON SWITCH

tON, V+ = 3.0V

80                                                                                         50

tON, V+ = 3.0V

50                                          t

ON, V+ = 5.0V

tON/tOFF (ns)

40

60

30

tON/tOFF (ns)

40

20

| 20 tOFF, V+ = 3.0V |  | HIGH RON SWITCH |  |
| --- | --- | --- | --- |
|  | tON |  |  |
|  |  | tOFF |  |

10

tON, V+ = 5.0V

tOFF, V+ = 5.0V

tON/tOFF (ns)

40

30

20

tOFF, V+ = 3.0V              tOFF, V+ = 5.0V

10

0

1.5

2.5

3.5

4.5

5.5

0

-40

-15         10         35

60         85

0

| -40 INPUT RISE/FALL TIME = 15ns FIGURE 4, CL = 50pF LOW RON SWITCH |  |  |  |  |
| --- | --- | --- | --- | --- |

-15         10

35         60         85

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

TEMPERATURE (°C)

3.0

2.5

2.0

RISE/FALL-TIME DELAY

MAX4717/18 toc19

vs. SUPPLY VOLTAGE

INPUT RISE/FALL TIME = 15ns

FIGURE 4, CL = 50pF LOW RON SWITCH

2.0

1.5

RISE/FALL-TIME  DELAY

vs. TEMPERATURE

INPUT RISE/FALL TIME = 15ns

FIGURE 4, CL = 50pF V+ = 4.2V

LOW RON SWITCH

400

300

MAX4717/18 toc20

MAX4717/18 toc21

RISE TIME TO FALL TIME MISMATCH

vs. SUPPLY VOLTAGE

1.5

1.0

OUTPUT RISE/FALL-TIME DELAY (ps)

0.5

RISE DELAY

FALL DELAY

1.0

OUTPUT RISE/FALL-TIME DELAY (ns)

0.5

RISE DELAY                      FALL DELAY

MISMATCH (ps)

200

100

0

1.5

2.5

3.5

4.5

5.5

0

-40

-15         10         35         60         85

0

1.5

2.5

3.5

4.5

5.5

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

SUPPLY VOLTAGE (V)

8

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

Typical Operating Characteristics (continued)

(TA = +25°C, unless otherwise noted.)

200

150

MAX4717/18 toc22

RISE TIME TO FALL TIME MISMATCH

vs. TEMPERATURE

INPUT RISE/FALL TIME = 15ns

FIGURE 4, CL = 50pF V+ = 4.2V

LOW RON SWITCH

400

300

MAX4717/18 toc23

SKEW vs. SUPPLY VOLTAGE

200

150

SKEW vs. TEMPERATURE

MAX4717/18 toc24

INPUT RISE/FALL TIME = 15ns

FIGURE 4, CL = 50pF V+ = 4.2V

MAX4717 ONLY

MISMATCH (ps)

100

SKEW (ps)

200

SKEW (ps)

100

| 50 100 50 INPUT RISE/FALL TIME = 15ns FIGURE 4, CL = 50pF MAX4717 ONLY |  |  |  |
| --- | --- | --- | --- |

0

-40

-15         10         35

60         85

0

1.5

2.5

3.5

4.5

5.5

0

-40

-15         10

35         60         85

TEMPERATURE (°C)

SUPPLY VOLTAGE (V)

TEMPERATURE (°C)

FREQUENCY RESPONSE

20

MAX4717/18 toc25

V+ = 3V/5V                               LOW RON SWITCH

0

FREQUENCY RESPONSE

20

MAX4717/18 toc26

V+ = 3V/5V                              HIGH RON SWITCH

0

-20

ON-LOSS (dB)

-40

ON-LOSS

-20

ON-LOSS (dB)

-40

ON-LOSS

-60

-80

OFF-ISOLATION

-60

-80

OFF-ISOLATION

-100

-120

CROSSTALK

-100

-120

CROSSTALK

-140

0.0001

0.01              1

100

-140

0.0001

0.01              1

100

FREQUENCY (MHz)

FREQUENCY (MHz)

TOTAL HARMONIC DISTORTION

vs. FREQUENCY

1

V+ = 3V

MAX4717/18 toc27

LOW RON SWITCH RL = 600Ω

TOTAL HARMONIC DISTORTION

vs. FREQUENCY

1

V+ = 3V

MAX4717/18 toc28

HIGH RON SWITCH RL = 600Ω

THD (%)

0.1

THD (%)

0.1

0.01

10

100

1k

FREQUENCY (Hz)

10k

100k

0.01

10

100

1k

FREQUENCY (Hz)

10k

100k

9

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

| Pin Description PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| UCSP | µMAX/ TDFN |  |  |
| A1 | 7 | NC2 | Analog Switch 2—Normally Closed Terminal |
| A2 | 8 | IN2 | Analog Switch 2—Digital Control Input |
| A3 | 9 | COM2 | Analog Switch 2—Common Terminal |
| A4 | 10 | NO2 | Analog Switch 2—Normally Open Terminal |
| B1 | 6 | GND | Ground. Connection. |
| B4 | 1 | V+ | Positive-Supply Voltage |
| C1 | 5 | NC1 | Analog Switch 1—Normally Closed Terminal |
| C2 | 4 | IN1 | Analog Switch 1—Digital Control Input |
| C3 | 3 | COM1 | Analog Switch 1—Common Terminal |
| C4 | 2 | NO1 | Analog Switch 1—Normally Open Terminal |
| — | — | EP | Exposed Pad (for TDFN package only). Connect to ground. |

Applications Information

Digital Control Inputs

The MAX4717/MAX4718 logic  inputs accept  up to

+5.5V regardless of supply voltage. For example, with a +3.3V supply, IN_ can be driven low to GND and high to +5.5V allowing for mixing of logic levels in a system. Driving the control logic  inputs rail-to-rail minimizes power consumption.  For a +3V supply  voltage, the logic  thresholds are 0.5V (low) and 1.4V (high); for a

+5V supply voltage, the logic thresholds are 0.8V (low)

and 2.0V (high).

Detailed Description

The MAX4717/MAX4718 high-speed, low-voltage, low on- resistance (RON), dual SPDT analog switches operate from a single +1.8V to +5.5V supply. The switches feature break-before-make  switching operation and fast switch- ing speeds (tON = 80ns (max), tOFF = 40ns (max)).

These switches have low 15pF on-channel  capaci- tance, which allows for 12Mbps  switching of the data signals for USB 1.0/1.1 applications.  The MAX4717 is designed to switch D+ and D- USB signals with a guar- anteed skew of less than 2ns (see Figure 4) as mea- sured from 50% of the input signal to 50% of the output signal.

Analog Signal Levels The on-resistance of the MAX4717/MAX4718 changes very little for analog input signals across the entire supply voltage range (see the Typical Operating Characteristics). The switches are bidirectional,  so the NO_, NC_, and COM_ pins can be either inputs or outputs.

Power-Supply Sequencing and

Overvoltage Protection Caution: Do not exceed the absolute maximum rat- ings because stresses beyond the listed ratings may cause permanent damage to the device.

Proper power-supply  sequencing  is recommended  for all CMOS devices.  Always apply V+ before applying analog signals, especially  if the analog signal is not current-limited.

UCSP  Application Information

For the latest application details on UCSP construction, dimensions,  tape carrier information, printed  circuit board techniques, bump-pad  layout, and recommend- ed reflow temperature profile as well as the latest infor- mation on reliability testing results, go to the Maxim web  site at www.maxim-ic.com/ucsp   to find  the Application  Note: USCP—A Wafer-Level Chip-Scale Package.

Chip Information

TRANSISTOR COUNT: 235

PROCESS: BiCMOS

10

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

Test Circuits/Timing Diagrams

MAX4717/ MAX4718

VN_

NO_

OR NC_

IN_

V+

V+

COM_

RL

VOUT

CL

LOGIC INPUT

VIH

VIL

50%

VOUT

tOFF

tr < 5ns tf < 5ns

LOGIC INPUT

GND

SWITCH       0V OUTPUT

tON

0.9 x V0UT                       0.9 x VOUT

CL INCLUDES FIXTURE AND STRAY CAPACITANCE.

VOUT = VN_ (  RL        )

R  + R

L        ON

IN DEPENDS ON SWITCH CONFIGURATION;

INPUT POLARITY DETERMINED BY SENSE OF SWITCH.

Figure 1. Switching Time

MAX4717/ MAX4718

V+

V+ NC_

LOGIC INPUT

VIH

VIL

50%

VN_                                                                   COM_

NO_

IN_

RL

300Ω

VOUT

CL

35pF

LOGIC INPUT

GND

CL INCLUDES FIXTURE AND STRAY CAPACITANCE.

VOUT                                                                                             0.9 x VOUT

tBBM

Figure 2. Break-Before-Make Interval

11

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

Test Circuits/Timing Diagrams (continued)

RS

IN+

RS

IN-

NC1 OR NO1

NC2 OR NO2

MAX4717

COM1

COM2

OUT+ CL

OUT- CL

RISE TIME DELAY = |tINRISE - tOUTRISE| FALL TIME DELAY = |tINFALL - tOUTFALL|

RISE TIME TO FALL TIME MISMATCH  = |tOUTFALL - tOUTRISE|

IN1            IN2

VIL TO VIH

tINFALL                                                                tINRISE

V+ VIN+

0V

V+ VIN-

50%

50%

90%

10%

10%

90%

0V

tOUTFALL                                                             tOUTRISE

V+ VOUT+

0V

V+ VOUT-

50%

50%

90%

10%

10%

90%

0V

tSKEW

Figure 3. Output Signal Skew

12

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

Test Circuits/Timing Diagrams (continued)

MAX4717/ MAX4718

RGEN

NC_

V+

V+                                                                      VOUT

COM_

∆VOUT

VGEN

OR NO_

GND

CL

IN_

VOUT

IN

OFF

OFF ON

VIL TO VIH

OFF IN

ON

Q = (∆VOUT)(CL)

OFF

LOGIC INPUT WAVEFORMS INVERTED FOR SWITCHES THAT HAVE THE OPPOSITE LOGIC SENSE.

Figure 4. Charge Injection

0V OR V+

50Ω

IN_ NC1

+5V  10nF

V+

COM1

MAX4717/ MAX4718

NO1*

VIN

VOUT

MEAS

50Ω

NETWORK ANALYZER

50Ω

REF

OFF-ISOLATION = 20log  VOUT

VIN

ON-LOSS = 20log  VOUT

VIN

VOUT

CROSSTALK = 20log    VIN

GND

50Ω                                50Ω

MEASUREMENTS ARE STANDARDIZED AGAINST SHORTS AT IC TERMINALS.

OFF-ISOLATION  IS MEASURED BETWEEN COM_   AND "OFF" NO_  OR NC_   TERMINAL ON EACH SWITCH. ON-LOSS  IS MEASURED BETWEEN COM_   AND "ON" NO_ OR NC_   TERMINAL ON EACH SWITCH. CROSSTALK IS MEASURED FROM ONE CHANNEL TO THE OTHER CHANNEL.

SIGNAL  DIRECTION  THROUGH SWITCH  IS REVERSED; WORST VALUES ARE RECORDED.

*FOR CROSSTALK THIS PIN IS NO2. NC2 AND COM2 ARE OPEN.

Figure 5. On-Loss, Off-Isolation,  and Crosstalk

10nF        V+

V+ COM_

MAX4717/ MAX4718

CAPACITANCE

METER                                     NC_ or

NO_

IN

VIL OR VIH

f = 1MHz

GND

Figure 6. Channel Off/On-Capacitance

13

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

12L, UCSP 4x3.EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

PACKAGE OUTLINE, 4x3 UCSP

1

21-0104               F      1

14

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

10LUMAX.EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | - | 0.043 | - | 1.10 |
| A1 | 0.002 | 0.006 | 0.05 | 0.15 |
| A2 | 0.030 | 0.037 | 0.75 | 0.95 |
| D1 | 0.116 | 0.120 | 2.95 | 3.05 |
| D2 | 0.114 | 0.118 | 2.89 | 3.00 |
| E1 | 0.116 | 0.120 | 2.95 | 3.05 |
| E2 | 0.114 | 0.118 | 2.89 | 3.00 |
| H | 0.187 | 0.199 | 4.75 | 5.05 |
| L | 0.0157 | 0.0275 | 0.40 | 0.70 |
| L1 | 0.037 REF |  | 0.940 REF |  |
| b | 0.007 | 0.0106 | 0.177 | 0.270 |
| e | 0.0197 BSC |  | 0.500 BSC |  |
| c | 0.0035 | 0.0078 | 0.090 | 0.200 |
| S | 0.0196 REF |  | 0.498 REF |  |
| α | 0° | 6° | 0° | 6° |

| e 4X S 10 10 H Ø0.50±0.1 0.6±0.1 1 1 0.6±0.1 TOP VIEW BOTTOM VIEW D2 E2 GAGE PLANE A2 A c b α E1 L A1 D1 L1 FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, 10L uMAX/uSOP |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0061 | REV. I | 1 1 |

15

4.5Ω/20Ω, 300MHz Bandwidth, Dual SPDT Analog Switches in UCSP

MAX4717/MAX4718

Package Information (continued) (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information go to  www.maxim-ic.com/packages.)

D2

A2                                                                                                    PIN 1 ID

N

PIN 1

INDEX AREA

b

E2

DETAIL A

0.35x0.35

e

6, 8, &10L, DFN THIN.EPS

[(N/2)-1] x e

REF.

| A1 k D |  |  |
| --- | --- | --- |
|  |  | E |

CL                                                                         CL

A

L                                                                                                L

e                                                   e

-DRAWING NOT TO SCALE-

PACKAGE OUTLINE,  6,8,10 & 14L, TDFN, EXPOSED PAD, 3x3x0.80  mm

1

21-0137               G      2

| COMMON DIMENSIONS |  |  |
| --- | --- | --- |
| SYMBOL | MIN. | MAX. |
| A | 0.70 | 0.80 |
| D | 2.90 | 3.10 |
| E | 2.90 | 3.10 |
| A1 | 0.00 | 0.05 |
| L | 0.20 | 0.40 |
| k | 0.25 MIN. |  |
| A2 | 0.20 REF. |  |

| PACKAGE VARIATIONS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PKG. CODE | N | D2 | E2 | e | JEDEC SPEC | b | [(N/2)-1] x e | DOWNBONDS ALLOWED |
| T633-1 | 6 | 1.50±0.10 | 2.30±0.10 | 0.95 BSC | MO229 / WEEA | 0.40±0.05 | 1.90 REF | NO |
| T633-2 | 6 | 1.50±0.10 | 2.30±0.10 | 0.95 BSC | MO229 / WEEA | 0.40±0.05 | 1.90 REF | NO |
| T833-1 | 8 | 1.50±0.10 | 2.30±0.10 | 0.65 BSC | MO229 / WEEC | 0.30±0.05 | 1.95 REF | NO |
| T833-2 | 8 | 1.50±0.10 | 2.30±0.10 | 0.65 BSC | MO229 / WEEC | 0.30±0.05 | 1.95 REF | NO |
| T833-3 | 8 | 1.50±0.10 | 2.30±0.10 | 0.65 BSC | MO229 / WEEC | 0.30±0.05 | 1.95 REF | YES |
| T1033-1 | 10 | 1.50±0.10 | 2.30±0.10 | 0.50 BSC | MO229 / WEED-3 | 0.25±0.05 | 2.00 REF | NO |
| T1433-1 | 14 | 1.70±0.10 | 2.30±0.10 | 0.40 BSC | - - - - | 0.20±0.05 | 2.40 REF | YES |
| T1433-2 | 14 | 1.70±0.10 | 2.30±0.10 | 0.40 BSC | - - - - | 0.20±0.05 | 2.40 REF | NO |

-DRAWING NOT TO SCALE-

PACKAGE OUTLINE,  6,8,10 & 14L, TDFN, EXPOSED PAD, 3x3x0.80  mm

2

21-0137               G      2

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

16                                         Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600

©  2005 Maxim Integrated  Products               Printed USA                           is a registered  trademark  of Maxim Integrated  Products, Inc.