19-0269; Rev 0; 9/94

5V/3.3V or Adjustable, Low-Dropout, Low IQ, 500mA Linear Regulators

General Description

The MAX603/MAX604 low-dropout,  low quiescent cur- rent, linear regulators supply 5V, 3.3V, or an adjustable output for currents up to 500mA. They are available in a

1.8W SO package.  Typical dropouts are 320mV at 5V and 500mA, or 240mV at 3.3V and 200mA. Quiescent currents are 15µA typ and 35µA max. Shutdown turns off all circuitry and puts the regulator in a 2µA off mode. A unique protection  scheme limits reverse currents when the input voltage falls below the output. Other fea- tures include  foldback  current limiting and thermal overload protection.

The output is preset at 3.3V for the MAX604 and 5V for the MAX603. In addition,  both devices  employ Dual Mode™ operation, allowing user-adjustable  outputs from 1.25V to 11V using external resistors. The input voltage supply range is 2.7V to 11.5V.

The MAX603/MAX604 feature a 500mA P-channel MOSFET pass transistor. This transistor allows the devices to draw less than 35µA over temperature, inde- pendent  of the output current. The supply  current remains low because the P-channel MOSFET pass tran- sistor draws no base currents (unlike the PNP transis- tors of conventional  bipolar  linear regulators).  Also, when the input-to-output  voltage differential becomes small, the internal P-channel MOSFET does not suffer from excessive base current losses that occur with sat- urated PNP transistors.

Applications

5V and 3.3V Regulators

1.25V to 11V Adjustable Regulators

Battery-Powered Devices Pagers and Cellular Phones Portable Instruments

Solar-Powered Instruments

Typical Operating Circuit

Features

♦ 500mA Output Current, with Foldback Current

Limiting

♦ High-Power (1.8W) 8-Pin SO Package

♦ Dual Mode™ Operation: Fixed or Adjustable

Output from 1.25V to 11V

♦ Large Input Range (2.7V to 11.5V)

♦ Internal 500mA P-Channel Pass Transistor

♦ 15µA Typical Quiescent Current

♦ 2µA (Max) Shutdown Mode

♦ Thermal Overload Protection

♦ Reverse-Current Protection

Ordering Information

PART TEMP. RANGE PIN-PACKAGE

MAX603CPA 0°C to +70°C 8 Plastic DIP

MAX603CSA 0°C to +70°C 8 SO

MAX603C/D 0°C to +70°C Dice*

MAX603EPA -40°C to +85°C 8 Plastic DIP

MAX603ESA -40°C to +85°C 8 SO

MAX603MJA -55°C to +125°C 8 CERDIP**

MAX604CPA 0°C to +70°C 8 Plastic DIP

MAX604CSA 0°C to +70°C 8 SO

MAX604C/D 0°C to +70°C Dice*

MAX604EPA -40°C to +85°C 8 Plastic DIP

MAX604ESA -40°C to +85°C 8 SO

MAX604MJA -55°C to +125°C 8 CERDIP**

*  Dice are tested at TA = +25°C, DC parameters only.

** Contact factory for availability.

Pin  Configuration

TOP VIEW

IN

MAX603

OUT

OUTPUT VOLTAGE

IN    1

GND   2

8     OUT

7     GND

BATTERY

CIN

10  F

OFF	COUT

10   F

GND   3

MAX603

MAX604

6     GND

GND

SET

OFF   4

5     SET

MAX603/MAX604

DIP/SO

™ Dual Mode is a trademark of Maxim Integrated Products.

Maxim Integrated Products   1

ABSOLUTE MAXIMUM RATINGS

Supply Voltage (IN or OUT to GND).......................-0.3V to +12V Output Short-Circuit  Duration ..............................................1 min Continuous Output Current ...............................................600mA SET, OFF Input Voltages ...........................-0.3V to the greater of (IN + 0.3V) or (OUT + 0.3V)

Continuous Power Dissipation  (TA = +70°C)

Plastic DIP (derate 9.09mW/°C above +70°C) ............727mW

SO (derate 23.6mW/°C above +70°C) .............................1.8W CERDIP (derate 8.00mW/°C above +70°C) .................640mW

Operating  Temperature  Ranges

MAX60_C_A ........................................................0°C to +70°C MAX60_E_A .....................................................-40°C to +85°C MAX60_MJA ..................................................-55°C to +125°C Junction Temperature ......................................................+150°C Storage Temperature  Range .............................-65°C to +160°C Lead Temperature  (soldering,  10sec) .............................+300°C

MAX603/MAX604

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VIN = 6V (MAX603) or 4.3V (MAX604), CIN = COUT = 10µF, OFF = VIN, SET = GND, TJ = TMIN to TMAX, unless otherwise noted. Typical values are at TJ = +25°C.) (Note 1)

| OUT PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Voltage | VIN | SET = OUT, RL = 1kΩ | MAX60_C | 2.7 11.5 | V |
|  |  |  | MAX60_E | 2.9 11.5 |  |
|  |  |  | MAX60_M | 3.0 11.5 |  |
| Output Voltage (Note 2) | VOUT | IOUT = 20µA to 500mA, 6.0V < VIN < 11.5V | MAX603 | 4.75 5.00 5.25 | V |
|  |  | IOUT = 20µA to 300mA, 4.3V < VIN < 11.5V | MAX604 | 3.15 3.30 3.45 |  |
| Load Regulation | ∆VLDR | IOUT = 1mA to 500mA | MAX603C/E | 60 100 | mV |
|  |  |  | MAX603M | 150 |  |
|  |  | IOUT = 1mA to 300mA | MAX604 | 30 100 |  |
| Line Regulation | ∆VLNR | (VOUT + 0.5V) ≤ VIN ≤ 11.5V, IOUT = 25mA |  | 7 40 | mV |
| Dropout Voltage (Note 3) | ∆VDO | IOUT = 200mA | MAX603 | 130 220 | mV |
|  |  | IOUT = 500mA |  | 320 550 |  |
|  |  | IOUT = 200mA | MAX604 | 240 410 |  |
|  |  | IOUT = 400mA |  | 480 820 |  |
| Quiescent Current | IQ | 3.0V ≤ VIN ≤ 11.5V, SET = OUT | MAX60_C/E | 15 35 | µA |
|  |  |  | MAX60_M | 40 |  |
| OFF Quiescent Current | IQ OFF | OFF ≤ 0.4V, RL = 1kΩ, (VOUT + 1V) ≤ VIN ≤ 11.5V | MAX60_C | 0.01 2 | µA |
|  |  |  | MAX60_E | 10 |  |
|  |  |  | MAX60_M | 20 |  |
| Minimum Load Current | IOUT MIN | VIN = 11.5V, SET = OUT | MAX60_C | 2 | µA |
|  |  |  | MAX60_E | 6 |  |
|  |  |  | MAX60_M | 20 |  |
| Foldback Current Limit (Note 4) | ILIM | V < 0.8V |  | 350 | mA |
|  |  | VOUT > 0.8V and VIN - VOUT > 0.7V |  | 1200 |  |
| Thermal Shutdown Temperature | TSD |  |  | 160 | °C |
| Thermal Shutdown Hysteresis | ∆TSD |  |  | 10 | °C |

MAX603/MAX604

ELECTRICAL CHARACTERISTICS (continued)

(VIN = 6V (MAX603) or 4.3V (MAX604), CIN = COUT = 10µF, OFF = VIN, SET = GND, TJ = TMIN to TMAX, unless otherwise noted. Typical values are at TJ = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Reverse-Current Protection Threshold (Note 5) | ∆VRTH | VOUT = 4.5V | MAX603 | 6 20 | mV |
|  |  | VOUT = 3.0V | MAX604 | 6 20 |  |
| Reverse Leakage Current | IRVL | VIN = 0V, VOUT = 4.5V (MAX603) VOUT = 3.0V (MAX604) | MAX60_C | 0.01 10 | µA |
|  |  |  | MAX60_E | 20 |  |
|  |  |  | MAX60_M | 100 |  |
| Start-Up Overshoot | VOSH | RL = 1kΩ, COUT = 10µF, OFF rise time ≤ 1µs |  | 2 | %VOUT |
| Time Required to Exit Shutdown | tSTART | VIN = 9V, RL = 18Ω, V OFF switched from |  | 200 | µs |
|  |  | 0V to VIN, time from 0% to 95% of VOUT |  |  |  |
| Dual-Mode SET Threshold | VSET TH | For internal feedback |  | 80 30 | mV |
|  |  | For external feedback |  | 150 80 |  |
| SET Reference Voltage | VSET | SET = OUT, RL = 1kΩ |  | 1.16 1.20 1.24 | V |
| SET Input Leakage Current | ISET | VSET = 1.5V or 0V |  | ±0.01 ±10 | nA |
| OUT Leakage Current | IOUT LKG | VIN = 11.5V, VOUT = 2V, SET = OUT | MAX60_C | 0.01 2 | µA |
|  |  |  | MAX60_E | 6 |  |
|  |  |  | MAX60_M | 20 |  |
| OFF Threshold Voltage | VIL OFF | Off |  | 0.4 | V |
|  | VIH OFF | On, SET = OUT, VIN = 4V |  | 2.0 |  |
|  |  | On, SET = OUT, VIN = 6V |  | 3.0 |  |
|  |  | On, SET = OUT, VIN = 11.5V |  | 4.0 |  |
| OFF Input Leakage Current | IOFF | V OFF = VIN or GND |  | ±0.01 ±10 | nA |
| Output Noise (Note 6) | en | 10Hz to 10kHz, SET = OUT, RL = 1kΩ, COUT = 10µF |  | 250 | µVRMS |

Note 1:  Electrical specifications are measured  by pulse testing and are guaranteed for a junction temperature  (TJ) equal to the operating  temperature  range. C and E grade parts may be operated  up to a TJ of +125°. Expect performance similar to M grade specifications. For TJ between +125°C and +150°C, the output voltage may drift more.

Note 2:  (VIN - VOUT) is limited to keep the product (IOUT x (VIN - VOUT)) from exceeding the package power dissipation  limits.

Note 3:  Dropout Voltage is (VIN - VOUT) when VOUT falls to 100mV below its nominal value at VIN = VOUT + 2V. For example, the MAX603 is tested by measuring the VOUT at VIN = 7V, then VIN is lowered until VOUT falls 100mV below the measured value. The difference (VIN - VOUT) is then measured and defined as VDO.

Note 4:  Foldback Current Limit was characterized by pulse testing to remain below the maximum junction temperature.

Note 5:  The Reverse-Current Protection Threshold is the output/input differential voltage (VOUT - VIN) at which reverse-current protection switchover occurs and the pass transistor is turned off.

Note 6:  Noise is tested using a bandpass amplifier with two poles at 10Hz and two poles at 10kHz.

Typical Operating Characteristics

(VIN = 7V for MAX603, VIN = 5.3V for MAX604, OFF = VIN, SET = GND, CIN = COUT = 10µF, RL = 1k, TJ = +25°C, unless otherwise noted.)

MAX603/4-TOC-01

OUTPUT VOLTAGE vs. LOAD CURRENT

MAX603/4-TOC-02

QUIESCENT CURRENT vs. LOAD CURRENT

OUTPUT VOLTAGE AND QUIESCENT CURRENT vs. SUPPLY VOLTAGE

1.01

1.00

0.99

0.97

0.95

VOUT = 3.3V, 5V, 10V OUTPUT VOLTAGE

AT 1mA

30

25

20

MAX603, VIN = 7V, VOUT = 5V

10

UPWARD CURVE IS THERMAL  EFFECT

0

6	24

MAX603, VOUT = 5V	22

5

20

4	18

MAX604, VOUT = 3.3V

16

14

OUTPUT VOLTAGE (V)

2	12

10

8

0	6

0.1	1

10	100

700

0.1	1

10	100

700

2    3    4    5    6    7    8

9    10

11  12

LOAD CURRENT (mA)

LOAD CURRENT (mA)

SUPPLY VOLTAGE (V)

QUIESCENT CURRENT (  A)

OUTPUT VOLTAGE vs.

TEMPERATURE

QUIESCENT CURRENT vs.

MAX603/4-TOC-05

TEMPERATURE

DROPOUT VOLTAGE vs. LOAD CURRENT

NORMALIZED OUTPUT VOLTAGE (%)

103

MAX603/4-TOC-04

102	20

QUIESCENT CURRENT (  A)

| 99 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

98	5

97

| 96 0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MAX603 |  |  |  |
|  |  |  |  |  |  | MAX604 |  |  |

0.8

DROPOUT VOLTAGE (V)

0.7

0.5

0.4

0.3

0.2

0.1

0

MAX604

MAX603/4-TOC-06

VOUT = 3.3V

MAX603

VOUT = 5V

MAX603, VOUT = 10V, SET EXTERNALLY

-55 -35

-15    5

25   45

65   85

105 125

-55 -35

-15    5

25   45

65   85

105 125

0     100

200

300  400

500

600

700

TEMPERATURE (°C)

TEMPERATURE (°C)

LOAD CURRENT (mA)

10Hz TO 10kHz  OUTPUT NOISE

LINE-TRANSIENT  RESPONSE

A

B

OUTPUT NOISE (1mV/div)

MAX603

VOUT = 5V

10ms/div

MAX603

VOUT = 5V

tR = 10 s, tF = 70 s

2ms/div

MAX603/MAX604

NORMALIZED OUTPUT VOLTAGE

QUIESCENT CURRENT (  A) MAX1603/4 TOC-03

A: VIN = 8V (HIGH), VIN = 7V (LOW) B: OUTPUT VOLTAGE (50mV/div)

Typical Operating Characteristics (continued)

(VIN = 7V for MAX603, VIN = 5.3V for MAX604, OFF = VIN, SET = GND, CIN = COUT = 10µF, RL = 1k, TJ = +25°C, unless otherwise noted.)

LOAD-TRANSIENT RESPONSE

OVERSHOOT AND TIME EXITING SHUTDOWN MODE

B

5V

A	A

MAX603

VOUT = 5V

B

2ms/div

0V

500 s/div

A: OUTPUT VOLTAGE (100mV/div)

B: IOUT = 500mA (HIGH), IOUT = 5mA (LOW)

A: OFF PIN VOLTAGE (1V/div) RISE TIME = 13 s

B: MAX603  OUTPUT VOLTAGE (1V/div)

DELAY = 4.936ms,  OVERSHOOT = 1%, RISE TIME = 55 s

Pin  Description

| PIN | NAME | DESCRIPTION |
| --- | --- | --- |
| 1 | IN | Regulator Input. Supply voltage can range from 2.7V to 11.5V. |
| 2, 3, 6, 7 | GND | Ground. These pins function as heatsinks, only in the SOIC package. All GND pins must be soldered to the circuit board for proper power dissipation. Connect to large copper pads or planes to channel heat from the IC. |
| 4 | OFF | Shutdown, active low. Switch logic levels in less than 1µs with the high level above the OFF threshold. |
| 5 | SET | Feedback for Setting the Output Voltage. Connect to GND to set the output voltage to the preselected 3.3V or 5V. Connect to an external resistor network for adjustable output operation. |
| 8 | OUT | Regulator Output. Fixed or adjustable from 1.25V to 11.0V. Sources up to 500mA for input voltages above 4V. |

VIN

CIN

1

IN

2   GND

3   GND

8

MAX603   OUT

MAX604	7

GND

GND   6

R1

COUT

VOUT

RL

10   F

4    	 OFF

SET   5

10  F

R2

MAX603/MAX604

Figure 1. Test Circuit

IN

SHUTDOWN

OFF

SHUTDOWN LOGIC

ERROR AMP

MOSFET DRIVER WITH FOLDBACK CURRENT LIMIT

REVERSE P 	CURRENT

PROTECTION

OUT SET

R1

GND

THERMAL SENSOR

1.20V

REFERENCE	DUAL-MODE COMPARATOR

80mV

R2

MAX603

MAX604

Figure 2.  Functional Diagram

MAX603/MAX604

Detailed Description

The MAX603/MAX604 are low-dropout,  low-quiescent- current linear regulators designed  primarily for battery- powered applications. They supply an adjustable 1.25V to 11V output or a preselected  5V (MAX603) or 3.3V (MAX604) output for load currents up to 500mA. As illustrated in Figure 2, they consist of a 1.20V reference, error amplifier, MOSFET driver, P-channel pass transis- tor, dual-mode comparator, and internal feedback  volt- age divider.

The 1.20V bandgap reference is connected to the error amplifier’s inverting input. The error amplifier compares this reference with the selected  feedback  voltage and amplifies the difference. The MOSFET driver reads the error signal and applies the appropriate  drive to the P- channel pass transistor. If the feedback voltage is lower than the reference, the pass transistor gate is pulled lower, allowing more current to pass and increasing the output voltage. If the feedback  voltage is too high, the pass transistor gate is pulled up, allowing less current to pass to the output.

The output voltage is fed back through either an internal resistor voltage divider connected  to the OUT pin, or an external resistor network connected  to the SET pin. The dual-mode comparator examines the SET voltage and selects the feedback  path used. If SET is below 80mV, internal feedback is used and the output voltage is regulat- ed to 5V for the MAX603 or 3.3V for the MAX604. Additional blocks include a foldback current limiter, reverse current protection, thermal sensor, and shutdown logic.

Internal P-Channel Pass Transistor The MAX603/MAX604 feature a 500mA P-channel MOSFET pass transistor.  This provides several advan- tages over similar designs using PNP pass transistors, including longer battery life.

The P-channel MOSFET requires no base drive, which reduces  quiescent  current considerably.  PNP based regulators waste considerable  amounts of current in dropout  when the pass transistor saturates. They also use high base-drive  currents under large loads. The MAX603/MAX604 do not suffer from these problems and consume only 15µA of quiescent  current under light and heavy loads, as well as in dropout.

Output Voltage Selection The MAX603/MAX604 feature dual-mode  operation. In preset voltage mode, the output of the MAX603 is set to

5V and the output of the MAX604 is set to 3.3V using internal, trimmed feedback  resistors. Select this mode by connecting SET to ground.

In adjustable mode, an output between 1.25V and 11V is selected using two external resistors connected as a voltage divider to SET (Figure 3). The output voltage is set by the following equation:

V	V	1 	R1

BATTERY

CIN

0.1  F to

10  F

IN

OFF

MAX603

MAX604

GND

OUT

SET

R1

COUT

10  F R2

OUTPUT VOLTAGE

RL

OUT 	SET	R2

where VSET = 1.20V. To simplify resistor selection:

Figure 3.  Adjustable Output Using External Feedback Resistors

R1	R2   VOUT   - 1

VSET

Since the input bias current at SET is nominally zero, large resistance values can be used for R1 and R2 to minimize power consumption without losing accuracy. Up to 1.5M is acceptable  for R2. Since the VSET tolerance is less than ±40mV, the output can be set using fixed resistors instead of trim pots.

In preset voltage mode, impedances between SET and ground should be less than 10k. Otherwise, spurious conditions  could cause the voltage at SET to exceed the 80mV dual-mode threshold.

Shutdown A low input on the OFF pin shuts down the MAX603/ MAX604. In the off mode, the pass transistor, control circuit, reference, and all biases are turned off, reduc- ing the supply current below 2µA. OFF should be con- nected to IN for normal operation.

Use a fast comparator, Schmitt trigger, or CMOS or TTL logic to drive the OFF pin in and out of shutdown. Rise times should be shorter than 1µs. Do not use slow RC circuits,  leave OFF open, or allow the input to linger between thresholds; these measures will prevent the output  from jumping  to the positive supply  rail in response to an indeterminate input state.

Since the OFF threshold varies with input supply volt- age (see Electrical Characteristics),  do not derive the drive voltage from 3.3V logic.  With VIN at 11.5V, the high OFF logic level needs to be above 4V.

Foldback Current Limiting

> 0.7V. For VIN - VOUT < 0.7V (dropout operation), there is no current limit. If the output voltage drops  below

0.8V, implying a short-circuit condition, the output cur- rent is limited to 350mA. The output can be shorted to ground for one minute without damaging  the device if the package  can  dissipate  V IN x 350mA without exceeding TJ = +150°C.

Thermal Overload Protection Thermal overload protection limits total power dissipa- tion in the MAX603/MAX604. When the junction temper- ature exceeds TJ = +160°C, the thermal sensor sends a signal to the shutdown logic, turning off the pass tran- sistor and allowing the IC to cool. The thermal sensor will turn the pass transistor on again after the IC’s junc- tion temperature cools by 10°C, resulting in a pulsed output during thermal overload conditions.

Thermal overload protection is designed  to protect the MAX603/MAX604 in the event of fault conditions.  For continual operation, the absolute maximum junction tem- perature rating of TJ = +150°C should not be exceeded.

Operating Region and Power Dissipation Maximum power dissipation  of the MAX603/MAX604 depends  on the thermal resistance of the case and cir- cuit board, the temperature difference between the die junction and ambient air, and the rate of air flow. The power dissipation across the device is P = IOUT (VIN - VOUT). The resulting maximum power dissipation is:

TJ - TA

P

The MAX603/MAX604 also include  a foldback  current limiter. It monitors and controls the pass transistor’s

MAX

JB  +   BA

MAX603/MAX604

gate voltage, estimating the output current and limiting it to 1.2A for output voltages above 0.8V and VIN - VOUT

where (TJ - TA) is the temperature difference between the MAX603/MAX604 die junction and the surrounding

MAX603/4  FIG 4

POWER DISSIPATION vs.

MAXIMUM OUTPUT CURRENT vs. SUPPLY VOLTAGE

POWER DISSIPATION (W)

1.6

1.5

MAX603, VOUT = 5V

TYPICAL DROPOUT VOLTAGE LIMIT

600

500

400

MAXIMUM CONTINUOUS CURRENT LIMIT

MAXIMUM SUPPLY VOLTAGE LIMIT

MAX603/4-FIG-04A

HIGH-POWER SOIC

1.1

1.0

PAPER EPOXY BOARD

1oz. COPPER TJ = +125°C

TA = +25°C STILL AIR

MAXIMUM OUTPUT CURRENT (mA)

100

0

OPERATING REGION AT TA = +25°C

PLASTIC DIP

CERAMIC DIP

0.2	1

10	20

(in2)

4	5	6	7

8	9    10   11   12    13

1.3

6.5

65	130 (cm2)

SUPPLY VOLTAGE (V)

COPPER GROUND PAD AREA

Figure 4. Typical Maximum Power Dissipation vs. Ground Pad

Size.

air, JB (or JC) is the thermal resistance of the package chosen, and BA  is the thermal resistance through the printed circuit board, copper traces and other materials to the surrounding air. The 8-pin SOIC package  for the MAX603/MAX604 features a special lead frame with a lower thermal resistance and higher allowable power

700

TYPICAL DROPOUT VOLTAGE LIMIT

600

500

400

300

MAXIMUM OUTPUT CURRENT (mA)

200

100

MAX604

MAXIMUM SUPPLY VOLTAGE LIMIT

MAX603/4-FIG-04B

MAXIMUM CONTINUOUS CURRENT LIMIT

HIGH-POWER SOIC

PLASTIC DIP OPERATING

REGION AT

dissipation.  The thermal resistance of this package  is

JB = 42°C/W, compared with JB = 110°C/W for an 8-

TA = +25°C

TJ = +125°C

0

CERAMIC DIP

pin plastic DIP package and JB = 125°C/W for an 8-pin

2    3   4    5

6    7   8

9   10

11 12  13

ceramic DIP package.

The GND pins of the MAX603/MAX604 SOIC package perform the dual function of providing an electrical con- nection to ground and channeling heat away. Connect all GND pins to ground  using a large pad or ground plane. Where this is impossible, place a copper  plane on an adjacent  layer. The pad  should exceed  the dimensions in Figure 4.

Figure 4 assumes the IC is an 8-pin SOIC package,  is

SUPPLY VOLTAGE (V)

Figure 5. Power Operating Regions: Maximum Output Current vs. Differential Supply Voltage

low, as shown in Figure 5. Maximum power dissipation depends on packaging, board layout, temperature, and air flow. The maximum output current is:

soldered directly to the pad, has a +125°C maximum junction temperature and a +25°C ambient air tempera-

IOUT max

PMAX

TJ - TA

ture, and has no other heat sources. Use larger pad sizes for other packages,  lower junction temperatures, higher ambient temperatures, or conditions where the IC

VIN - VOUT

where PMAX is derived from Figure 4.

100 C

MAX603/MAX604

is not soldered directly to the heat-sinking ground pad.

The MAX603/MAX604 can regulate currents up to

500mA and operate with input voltages up to 11.5V, but not simultaneously. High output currents can only be sustained when input-output  differential voltages are

Reverse-Current Protection The MAX603/MAX604 has a unique protection scheme that limits reverse currents when the input voltage falls below the output. It monitors the voltages on IN and OUT and switches the IC’s substrate and power bus to

IN

MAX603

OUT

OUTPUT VOLTAGE

Applications Information

Figure 6 illustrates the typical  application  for the

MAX603/MAX604.

Capacitor Selection and

BATTERY

CIN

MAX604

OFF	COUT

10   F

Regulator Stability

Normally, use 0.1µF to 10µF capacitors  on the input and 10µF on the output of the MAX603/MAX604. The

GND

SET

larger input capacitor  values provide  better supply- noise rejection and line-transient response. Improve load-transient  response, stability, and power-supply rejection by using large output capacitors.  For stable operation over the full temperature range and with load

Figure 6. 3.3V or 5V Linear-Regulator Application

POWER-SUPPLY REJECTION RATIO

90

80

MAX603/4-FIG-06

IOUT = 1mA

70

PSRR (dB)

60

50

40

30

currents up to 500mA, 10µF is recommended.  Using capacitors smaller than 3.3µF can result in oscillation.

Noise The MAX603/MAX604 exhibit 3mVp-p to 4mVp-p of noise during normal operation. This is negligible in most

cations that include  analog-to-digital  converters of greater than 12 bits, consider  the ADC’s power-supply rejection specifications. Refer to the output noise plot in the Typical Operating Characteristics.

PSRR  and Operation from Sources

Other than Batteries The MAX603/MAX604 are designed  to deliver low dropout voltages and low quiescent currents in battery-

20	VIN = 1Vp-p

FOR f < 400kHz

10	CIN = 0 F COUT = 10 F

0

IOUT = 100mA

powered systems. Achieving these objectives requires trading  off power-supply  noise rejection and swift response to supply  variations and load transients. Power-supply rejection is 80dB at low freqencies  and

MAX603/MAX604

100	101	102	103	104	105	106

FREQUENCY (Hz)

Figure 7. Power-Supply Rejection Ratio vs. Ripple Frequency

the more positive of the two. The control circuitry  can then remain functioning and turn the pass transistor off, limiting reverse currents back through the device. This feature allows a backup  regulator or battery pack  to maintain VOUT when the supply at IN fails.

Reverse-current protection  activates when the voltage on IN falls 6mV (20mV maximum) below the voltage on OUT. Before this happens, currents as high as several milliamperes can flow back  through the device.  After switchover,  typical  reverse currents are limited  to

0.01µA for as long as the condition exists.

rolls off above 10Hz. As the frequency increases above

10kHz, the output capacitor  is the major contributor  to the rejection of power-supply  noise (Figure 7). Do not use power supplies with ripple above 100kHz, especial- ly when the ripple exceeds 100mVp-p. When operating from sources other than batteries, improved  supply- noise rejection and transient response can be achieved by increasing the values of the input and output capaci- tors, and through passive filtering techniques.  The Typical Operating  Characteristics  show the MAX603/ MAX604 supply and load-transient responses.

Transient Considerations The Typical  Operating  Characteristics  show the MAX603/MAX604 load-transient response. Two compo- nents of the output response can be observed on the load-transient graphs—a DC shift from the output imped- ance due to the different load currents, and the transient response. Typical transients for step changes in the load current from 5mA to 500mA are 0.2V. Increasing the out- put capacitor’s value attenuates transient spikes.

Input-Output (Dropout) Voltage

A regulator’s minimum input-output  voltage differential,

Chip Topography

or dropout voltage, determines the lowest usable supply voltage. In battery-powered  systems, this will determine the useful end-of-life battery voltage. Because the MAX603/MAX604 use a P-channel MOSFET pass tran- sistor, their dropout voltage is a function of rDS(ON) multi- plied by the load current (see Electrical Characteristics).

Quickly stepping up the input voltage from the dropout voltage can result in overshoot. This occurs when the pass transistor is fully on at dropout and the IC is not given time to respond  to the supply voltage change. Prevent this by slowing the input voltage rise time.

IN	OUT

0.100" 	 (2.54mm)

MAX603/MAX604

OFF    GND	SET

0.104"

(2.64mm)

TRANSISTOR COUNT: 111

NO DIRECT SUBSTRATE CONNECTION. THE N-SUBSTRATE IS INTERNALLY SWITCHED BETWEEN THE MORE POSITIVE OF IN OR OUT.

MAX603/MAX604

Package Information

| 0° - 15° E D E1 A3 A A2 L A1 e B1 eA B eB D1 | DIM | INCHES |  |  |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MIN |  | MAX |  | MIN |  | MAX |  |
|  | A | – |  | 0.200 |  | – |  | 5.08 |  |
|  | A1 | 0.015 |  | – |  | 0.38 |  | – |  |
|  | A2 | 0.125 |  | 0.175 |  | 3.18 |  | 4.45 |  |
|  | A3 | 0.055 |  | 0.080 |  | 1.40 |  | 2.03 |  |
|  | B | 0.016 |  | 0.022 |  | 0.41 |  | 0.56 |  |
|  | B1 | 0.045 |  | 0.065 |  | 1.14 |  | 1.65 |  |
|  | C | 0.008 |  | 0.012 |  | 0.20 |  | 0.30 |  |
|  | D1 | 0.005 |  | 0.080 |  | 0.13 |  | 2.03 |  |
|  | E | 0.300 |  | 0.325 |  | 7.62 |  | 8.26 |  |
|  | E1 | 0.240 |  | 0.310 |  | 6.10 |  | 7.87 |  |
|  | e | 0.100 |  | – |  | 2.54 |  | – |  |
|  | eA | 0.300 |  | – |  | 7.62 |  | – |  |
|  | eB | – |  | 0.400 |  | – |  | 10.16 |  |
|  | L | 0.115 |  | 0.150 |  | 2.92 |  | 3.81 |  |
| P PACKAGE PLASTIC DUAL-IN-LINE | DIM | PINS | INCHES |  |  |  | MILLIMETERS |  |  |
|  |  |  | MIN |  | MAX |  | MIN |  | MAX |
|  | D | 8 | 0.348 |  | 0.390 |  | 8.84 |  | 9.91 |
|  | D | 14 | 0.735 |  | 0.765 |  | 18.67 |  | 19.43 |
|  | D | 16 | 0.745 |  | 0.765 |  | 18.92 |  | 19.43 |
|  | D | 18 | 0.885 |  | 0.915 |  | 22.48 |  | 23.24 |
|  | D | 20 | 1.015 |  | 1.045 |  | 25.78 |  | 26.54 |
|  | D | 24 | 1.14 |  | 1.265 |  | 28.96 |  | 32.13 |

| C |  |  |  |
| --- | --- | --- | --- |

| B C L D A 0°-8° 0.101mm 0.005in. e A1 | DIM | INCHES |  |  |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MIN |  | MAX |  | MIN |  | MAX |  |
|  | A | 0.053 |  | 0.069 |  | 1.35 |  | 1.75 |  |
|  | A1 | 0.004 |  | 0.010 |  | 0.10 |  | 0.25 |  |
|  | B | 0.014 |  | 0.019 |  | 0.35 |  | 0.49 |  |
|  | C | 0.007 |  | 0.010 |  | 0.19 |  | 0.25 |  |
|  | E | 0.150 |  | 0.157 |  | 3.80 |  | 4.00 |  |
|  | e | 0.050 |  |  |  | 1.27 |  |  |  |
|  | H | 0.228 |  | 0.244 |  | 5.80 |  | 6.20 |  |
|  | L | 0.016 |  | 0.050 |  | 0.40 |  | 1.27 |  |
| S PACKAGE E H SMALL OUTLINE | DIM | PINS | INCHES |  |  |  | MILLIMETERS |  |  |
|  |  |  | MIN |  | MAX |  | MIN |  | MAX |
|  | D | 8 | 0.189 |  | 0.197 |  | 4.80 |  | 5.00 |
|  | D | 14 | 0.337 |  | 0.344 |  | 8.55 |  | 8.75 |
|  | D | 16 | 0.386 |  | 0.394 |  | 9.80 |  | 10.00 |
| 21-0041A |  |  |  |  |  |  |  |  |  |

MAX603/MAX604

Package Information (continued)

e

| B1 E1 E A D Q 0°-15° L L1 C B | DIM | INCHES |  |  |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MIN |  | MAX |  | MIN |  | MAX |  |
|  | A | – |  | 0.200 |  | – |  | 5.08 |  |
|  | B | 0.014 |  | 0.023 |  | 0.36 |  | 0.58 |  |
|  | B1 | 0.038 |  | 0.065 |  | 0.97 |  | 1.65 |  |
|  | C | 0.008 |  | 0.015 |  | 0.20 |  | 0.38 |  |
|  | E | 0.220 |  | 0.310 |  | 5.59 |  | 7.87 |  |
|  | E1 | 0.290 |  | 0.320 |  | 7.37 |  | 8.13 |  |
|  | e | 0.100 |  |  |  | 2.54 |  |  |  |
|  | L | 0.125 |  | 0.200 |  | 3.18 |  | 5.08 |  |
|  | L1 | 0.150 |  | – |  | 3.81 |  | – |  |
|  | Q | 0.015 |  | 0.070 |  | 0.38 |  | 1.78 |  |
|  | S | – |  | 0.098 |  | – |  | 2.49 |  |
|  | S1 | 0.005 |  | – |  | 0.13 |  | – |  |
| S1 S J PACKAGE (0.300 in.) CERDIP DUAL-IN-LINE | DIM | PINS | INCHES |  |  |  | MILLIMETERS |  |  |
|  |  |  | MIN |  | MAX |  | MIN |  | MAX |
|  | D | 8 | – |  | 0.405 |  | – |  | 10.29 |
|  | D | 14 | – |  | 0.785 |  | – |  | 19.94 |
|  | D | 16 | – |  | 0.840 |  | – |  | 21.34 |
|  | D | 18 | – |  | 0.960 |  | – |  | 24.38 |
|  | D | 20 | – |  | 1.060 |  | – |  | 26.92 |
|  | D | 24 | – |  | 1.280 |  | – |  | 32.51 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

12   	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  1994 Maxim Integrated Products	Printed USA	is a registered trademark of Maxim Integrated Products.