19-0366; Rev 5; 2/07

3.0V/3.3V Adjustable Microprocessor

Supervisory Circuits

General Description

The MAX793/MAX794/MAX795 microprocessor  (µP)

supervisory circuits monitor and control the activities of

+3.0V/+3.3V µPs by providing backup-battery switchover, among other features such as low-line indication,  µP reset, write protection for CMOS RAM, and a watchdog (see the Selector Guide below). The backup-battery  volt- age can exceed VCC, permitting the use of 3.6V lithium batteries in systems using 3.0V to 3.3V for VCC.

The MAX793/MAX795 offer a choice of reset threshold voltage range (denoted by suffix letter): 3.00V to 3.15V (T), 2.85V to 3.00V (S), and 2.55V to 2.70V (R). The MAX794’s reset threshold is set externally with a resistor divider.  The MAX793/MAX794 are available in 16-pin DIP and narrow SO packages, and the MAX795 comes in 8-pin DIP and SO packages.

| Selector Guide FEATURE | MAX793 | MAX794 | MAX795 |
| --- | --- | --- | --- |
| Active-Low Reset | ✔ | ✔ | ✔ |
| Active-High Reset | ✔ | ✔ |  |
| Programmable Reset Threshold |  | ✔ |  |
| Low-Line Early Warning Output | ✔ | ✔ |  |
| Backup-Battery Switchover | ✔ | ✔ | ✔ |
| External Switch Driver | ✔ | ✔ | ✔ |
| Power-Fail Comparator | ✔ | ✔ |  |
| Battery OK Output | ✔ |  |  |
| Watchdog Input | ✔ | ✔ |  |
| Battery Freshness Seal | ✔ | ✔ |  |
| Manual Reset Input | ✔ | ✔ |  |
| Chip-Enable Gating | ✔ | ✔ | ✔ |
| Pin-Package | 16-DIP/SO | 16-DIP/SO | 8-DIP/SO |

Features

MAX793/MAX794/MAX795

♦ Precision Supply-Voltage Monitor:

Fixed Reset Trip Voltage (MAX793/MAX795)

Adjustable Reset Trip Voltage (MAX794)

♦ Guaranteed Reset Assertion to VCC = 1V

♦ Backup-Battery Power Switching—Battery

Voltage Can Exceed VCC

♦ On-Board Gating of Chip-Enable Signals—7ns

Max Propagation Delay

MAX793/MAX794 Only

♦ Battery Freshness Seal

♦ Battery OK Output (MAX793)

♦ Uncommitted Voltage Monitor for Power-Fail or

Low-Battery Warning

♦ Independent Watchdog Timer (1.6s timeout)

♦ Manual Reset Input

Ordering Information

PART*	TEMP RANGE

PIN- PACKAGE

PKG CODE

MAX793_CPE 	0°C to +70°C 	16 Plastic DIP 	P16-1

MAX793_CSE	0°C to +70°C 	16 Narrow SO	P16-1

Ordering Information continued on last page.

*The MAX793/MAX795 offer a choice of reset threshold voltage. Select the letter corresponding to the desired reset threshold voltage range (T = 3.00V to 3.15V,  S = 2.85V to 3.00V, R =

2.55V to 2.70V) and insert it into the blank to complete the part number.  The MAX794’s reset threshold is adjustable.

Devices are available in both leaded and lead-free packaging. Specify lead free by adding  the + symbol at the end of the part number when ordering.

Typical Operating Circuit

(OPTIONAL) Si9433DY

Applications

3.0V OR 3.3V

0.1µF

SILICONIX

PMOS

0.1µF

VCC   BATT ON   OUT	CMOS

Battery-Powered Computers and Controllers

Embedded Controllers

Intelligent Controllers

Critical µP Power Monitoring

Portable Equipment

3.6V

0.1µF

+5V SUPPLY

+5V FAILURE

BATT

WDO MR PFO

CE OUT

MAX793

CE IN WDI

LOWLINE

RAM

ADDRESS DECODER

VCC

VCC

A0-A15

I/O NMI

µP

MAX793/MAX794/MAX795

Pin Configurations appear at end of data sheet.

PFI

RESET BATT OK

GND

RESET

ABSOLUTE MAXIMUM RATINGS

Terminal Voltage (with respect to GND)

VCC ......................................................................-0.3V to +6.0V VBATT ...................................................................-0.3V to +6.0V All Other Inputs ..................-0.3V  to the higher of VCC or VBATT

Continuous Input Current

VCC .................................................................................200mA VBATT ................................................................................50mA GND ..................................................................................20mA

Output Current VOUT................................................................................200mA All Other Outputs ..............................................................20mA

Continuous Power Dissipation  (TA = +70°C)

8-Pin Plastic DIP (derate 9.09mW/°C above +70°C) .....727mW

8-Pin SO (derate 5.88mW/°C above +70°C)..................471mW

16-Pin Plastic DIP (derate 10.53mW/°C above +70°C) .842mW

16-Pin Narrow SO (derate 9.52mW/°C above +70°C) ...696mW Operating  Temperature  Ranges

MAX793_C_ _/MAX794C_ _/MAX795_C_ _ ......... 0°C to +70°C MAX793_E_ _/MAX794E_ _/MAX795_E_ _ ........-40°C to +85°C Storage Temperature  Range .............................-65°C to +160°C Lead Temperature  (soldering,  10s) .................................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = 3.17V to 5.5V for the MAX793T/MAX795T,  VCC = 3.02V to 5.5V for the MAX793S/MAX795S,  VCC = 2.72V to 5.5V for the

µA

MAX793R/MAX794/MAX795R,  VBATT = 3.6V, TA = TMIN to TMAX, unless otherwise noted.  Typical values are at TA = +25°C.)

| OUT PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Operating Voltage Range, VCC, VBATT (Note 1) |  | MAX79_C |  | 1.0 5.5 | V |
|  |  | MAX79_E |  | 1.1 5.5 |  |
| VCC Supply Current (excluding IOUT, ICE OUT) | ISUPPLY | MAX793/MAX794, MR = VCC | VCC < 3.6V | 46 60 |  |
|  |  |  | VCC < 5.5V | 62 80 |  |
|  |  | MAX795 | VCC < 3.6V | 35 50 | µA |
|  |  |  | VCC < 5.5V | 49 70 |  |
| VCC Supply Current in Battery-Backup Mode (excluding IOUT) | ISUPPLY | VCC = 2.1V, VBATT = 2.3V | MAX793/MAX794 | 32 45 | µA |
|  |  |  | MAX795 | 24 35 |  |
| BATT Supply Current (excluding IOUT) (Note 2) |  |  |  | 1 | µA |
| BATT Leakage Current, Freshness Seal Enabled |  | VCC = 0V, VOUT = 0V |  | 1 | µA |
| Battery Leakage Current (Note 3) |  |  |  | 0.5 | µA |
| OUT Output Voltage in Normal Mode | VOUT | IOUT = 75mA |  | VCC - 0.3 VCC - 0.125 | V |
|  |  | IOUT = 30mA (Note 4) |  | VCC - 0.12 VCC - 0.050 |  |
|  |  | IOUT = 250µA (Note 4) |  | VCC - 0.001 VCC - 0.5mV |  |
| OUT Output Voltage in Battery-Backup Mode | VOUT | VBATT = 2.3V | I = 250µA | V - 0.1 V - 0.034 | V |
|  |  |  | IOUT = 1mA | VBATT - 0.14 |  |
| Battery Switch Threshold (VCC falling) | VCC - VBATT | VSW > VCC > 1.75V (Note 5) |  | 20 65 | mV |
|  | V | VBATT > VCC (Note 6) | MAX793T/MAX795T | 2.69 2.82 2.95 | V |
|  |  |  | MAX793S/MAX795S | 2.55 2.68 2.80 |  |
|  |  |  | MAX793R/MAX795R/ MAX794 | 2.30 2.41 2.52 |  |
| Battery Switch Threshold (VCC rising) (Note 7) | VCC - VBATT | This value is identical to the reset threshold, VCC rising for VBATT > VRST |  |  |  |
|  |  | VBATT < VRST |  | 25 65 | mV |

BATT

BATT

MAX793/MAX794/MAX795

SW

MAX793/MAX794/MAX795

3.0V/3.3V Adjustable Microprocessor

Supervisory Circuits

ELECTRICAL CHARACTERISTICS (continued)

(VCC = 3.17V to 5.5V for the MAX793T/MAX795T,  VCC = 3.02V to 5.5V for the MAX793S/MAX795S,  VCC = 2.72V to 5.5V for the

MAX793R/MAX794/MAX795R,  VBATT = 3.6V, TA = TMIN to TMAX, unless otherwise noted.  Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Reset Threshold (Note 8) | VRST | VCC falling | MAX793T/MAX795T | 3.00 3.075 3.15 | V |
|  |  |  | MAX793S/MAX795S | 2.85 2.925 3.00 |  |
|  |  |  | MAX793R/MAX795R | 2.55 2.625 2.70 |  |
|  |  | VCC rising | MAX793T/MAX795T | 3.00 3.085 3.17 |  |
|  |  |  | MAX793S/MAX795S | 2.85 2.935 3.02 |  |
|  |  |  | MAX793R/MAX795R | 2.55 2.635 2.72 |  |
| RESET IN Threshold (MAX794 only) | VRST IN | VCC falling |  | 1.212 1.240 1.262 | V |
|  |  | VCC rising |  | 1.212 1.250 1.282 |  |
| RESET IN Leakage Current (MAX794 only) |  |  |  | -25 2 25 | nA |
| Reset Timeout Period | tRP | VCC < 3.6V |  | 140 200 280 | ms |
| LOWLINE-to-Reset Threshold, (V LOWLINE - VRST), VCC Falling | VLR | MAX793 |  | 30 45 60 | mV |
|  |  | MAX794 |  | 5 15 25 |  |
| Low-Line Comparator Hysteresis |  | MAX793 |  | 10 | mV |
|  |  | MAX794 |  | 10 | mV |
| LOWLINE Threshold, VCC Rising | VLL | MAX793T/MAX795T |  | 3.23 | V |
|  |  | MAX793S/MAX795S |  | 3.08 |  |
|  |  | MAX793R/MAX795R |  | 2.78 |  |
|  |  | MAX794 |  | 1.317 |  |
| PFI Input Threshold | VTH | VPFI falling |  | 1.212 1.240 1.262 | V |
|  |  | VPFI rising |  | 1.212 1.250 1.287 |  |
| PFI Input Current |  |  |  | -25 2 25 | nA |
| PFI Hysteresis, PFI Rising |  |  |  | 10 20 | mV |
| BATT OK Threshold (MAX793) | VBOK |  |  | 2.00 2.25 2.50 | V |
| INPUT AND OUTPUT LEVE | LS |  |  |  |  |
| RESET Output-Voltage High | VOH | ISOURCE = 300µA, VCC = VRST min |  | 0.8VCC 0.86VCC | V |
| BATT OK, BATT ON, WDO, LOWLINE Output-Voltage High | VOH | ISOURCE = 300µA, VCC = VRST max |  | 0.8VCC 0.86VCC | V |
| PFO Output-Voltage High | VOH | ISOURCE = 65µA, VCC = VRST max |  | 0.8VCC | V |
| BATT ON Output- Voltage High | VOH | ISOURCE = 100µA, VCC = 2.3V, VBATT = 3V |  | 0.8VBATT | V |
| RESET Output Leakage Current (Note 9) | ILEAK | VCC = VRST max |  | -1 -1 | µA |
| PFO Output Short to GND Current | ISC | VCC = 3.3V, V PFO = 0V |  | 180 500 | µA |
| PFO, RESET, RESET, WDO, LOWLINE Output-Voltage Low | VOL | ISINK = 1.2mA; RESET, LOWLINE tested with VCC = VRST min; RESET, BATTOK, WDO tested with VCC = VRST max |  | 0.08 0.2VCC | V |

MAX793/MAX794/MAX795

ELECTRICAL CHARACTERISTICS (continued)

(VCC = 3.17V to 5.5V for the MAX793T/MAX795T,  VCC = 3.02V to 5.5V for the MAX793S/MAX795S,  VCC = 2.72V to 5.5V for the

MAX793R/MAX794/MAX795R,  VBATT = 3.6V, TA = TMIN to TMAX, unless otherwise noted.  Typical values are at TA = +25°C.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| RESET Output-Voltage Low | VOL | MAX79_C, VBATT = VCC = 1.0V, ISINK = 40µA | 0.13 0.3 | V |
|  |  | MAX79_E, VBATT = VCC = 1.2V, ISINK = 200µA | 0.17 0.3 |  |
| BATT ON Output- Voltage Low | VOL | ISINK = 3.2mA, VCC = VRST max | 0.2VCC | V |
| All Inputs Including PFO (Note 10) | VIH | VRST max < VCC < 5.5V | 0.7VCC | V |
|  | VIL |  | 0.3VCC |  |
| MANUAL RESET INPUT |  |  |  |  |
| MR Pulse Width | tMR | MAX793/MAX794 only | 100 | ns |
| MR-to-Reset Delay | tMD | MAX793/MAX794 only | 75 250 | ns |
| MR Pullup Current |  | MAX793/MAX794 only, MR = 0V | 25 70 250 | µA |
| CHIP-ENABLE GATING |  |  |  |  |
| CE IN Leakage Current | ILEAK | Disable mode | ±10 | nA |
| CE IN-to-CE OUT Resistance |  | Enable mode, VCC = VRST max | 46 | Ω |
| CE IN-to-CE OUT Propagation Delay |  | VCC = VRST max, Figure 9 | 2 7 | ns |
| CE OUT Drive from CE IN | VOH | VCC = VRST max, IOUT = -1mA, V CE IN = VCC | 0.8VCC | V |
|  | VOL | VCC = VRST max, IOUT = 1.6mA, V CE IN = 0V | 0.2VCC |  |
| Reset to CE OUT High Delay |  |  | 10 | µs |
| CE OUT Output-Voltage High (reset active) | VOH | IOH = 500µA, VCC < 2.3V | 0.8VBATT | V |
| WATCHDOG (MAX793/MA | 794 only) |  |  |  |
| WDI Input Current |  | 0V < VCC < 5.5V | -1 0.01 1 | µA |
| Watchdog Timeout Period | tWD |  | 1.00 1.60 2.25 | s |
| WDI Pulse Width |  |  | 100 | ns |

Note 1:   VCC supply current, logic-input leakage, watchdog functionality  (MAX793/MAX794),  MR functionality  (MAX793/MAX794), PFI functionality  (MAX793/MAX794),  and state of RESET and RESET (MAX793/MAX794) tested at VBATT = 3.6V and VCC =

5.5V. The state of RESET is tested at VCC = VCC min.

Note 2:  Tested at VBATT = 3.6V, VCC = 3.5V and 0V. The battery current rises to 10µA over a narrow transition window around VCC

= 1.9V.

Note 3:  Leakage current into the battery is tested under the worst-case conditions at VCC = 5.5V, VBATT = 1.8V and VCC = 1.5V, VBATT = 1.0V.

Note 4:  Guaranteed by design.

Note 5:  When VSW > VCC > VBATT, OUT remains connected to VCC until VCC drops below VBATT. The VCC-to-VBATT comparator has a small 15mV typical hysteresis to prevent oscillation. For VCC < 1.75V (typical), OUT switches to BATT regardless of VBATT.

Note 6:  When VBATT > VCC > VSW, OUT remains connected to VCC until VCC drops below the battery switch threshold (VSW).

Note 7:  OUT switches from BATT to VCC when VCC rises above the reset threshold, if VBATT > VRST. In this case, switchover back

to VCC occurs at the exact voltage that causes reset to be asserted, however, switchover occurs 200ms prior to reset. If

VBATT < VRST, OUT switches from BATT to VCC when VCC exceeds VBATT.

Note 8:  The reset threshold tolerance is wider for VCC rising than for VCC falling to accommodate the 10mV typical hysteresis,

which prevents internal oscillation.

Note 9:  The leakage current into or out of the RESET pin is tested with RESET not asserted (RESET output high impedance).

Note 10: PFO is normally an output, but is used as an input when activating the battery freshness seal.

3.0V/3.3V/Adjustable Microprocessor

Supervisory Circuits

Typical Operating Characteristics

(TA = +25°C, unless otherwise noted.)

3.0

VCC-TO-OUT ON-RESISTANCE (Ω)

2.8

2.6

2.4

2.2

2.0

VCC-TO-OUT ON-RESISTANCE

vs. TEMPERATURE

| 160 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VBATT = 3.0V |  |  |  |
| VBATT = 3.6V |  |  |  |  |  |  |
|  |  | VBATT = 5V |  | IOUT = 250µA VCC = 0V |  |  |

BATT-TO-OUT ON-RESISTANCE (Ω)

140

MAX793 TOC1

120

100

BATT-TO-OUT ON-RESISTANCE

vs. TEMPERATURE

VCC SUPPLY CURRENT vs. TEMPERATURE (NORMAL OPERATING MODE)

70

MAX793 TOC3

MAX793/4, VCC = 5V

MAX793 TOC2

VCC SUPPLY CURRENT (µA)

60

MAX795, VCC = 5V

50

40	MAX793/4, VCC = 3.3V

1.8

1.6

1.4

1.2

1.0

30

80

| 20 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

60	10

40	0

MAX795, VCC

= 3.3V

VBATT = VCC = VOUT

| -40 |  |  |  | IOUT = 30mA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| VCC = 3.0V |  |  |  |  |  |  |
|  |  |  |  |  | VCC = 3.3V |  |
|  |  | VCC = 5V |  |  |  |  |

-20     0

20     40	60

80    100

-40

-20     0

20     40	60

80    100

-40

-20     0

20     40	60

80    100

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

0.10

SUPPLY CURRENT (µA)

0.08

0.06

0.04

0.02

BATTERY SUPPLY CURRENT vs. TEMPERATURE (BATTERY-BACKUP MODE)

MAX793 TOC4

VCC = 0V VBATT = 3.6V

250

RESET TIMEOUT PERIOD (ms)

200

150

100

50

RESET TIMEOUT PERIOD

vs. TEMPERATURE

RESET COMPARATOR PROPAGATION DELAY

MAX793 TOC6

vs. TEMPERATURE (VCC FALLING)

30

MAX793 TOC5

PROPAGATION DELAY (µs)

25

20

15

10

5

0

-40

-20     0

20     40	60

80    100

0

| -40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VCC RISING FROM OV TO VRST MAX |  |  |

-20     0

20     40	60

80    100

0

-40

-20     0

20     40	60

80    100

100

90

TEMPERATURE (°C)

MAX793

MAX793 TOC7

LOWLINE-TO-RESET  THRESHOLD

vs. TEMPERATURE

TEMPERATURE (°C)

MAX793/MAX794

LOWLINE COMPARATOR PROPAGATION DELAY

vs. TEMPERATURE

10

MAX793 TOC8

40mV OVERDRIVE

| 1.250 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

TEMPERATURE (°C)

MAX793/MAX794

MAX793 TOC9

PFI THRESHOLD vs. TEMPERATURE

PROPAGATION DELAY (µs)

80	8

70

60	6

50

40	4

30

20	2

LOWLINE-TO-RESET THRESHOLD (mV)

10

0	0

VCC RISING

VCC FALLING

1.245

PFI THRESHOLD (V)

1.240

1.235

1.230

| -40 |  |  |  | VCC FALLING |  |  |
| --- | --- | --- | --- | --- | --- | --- |

-20     0

20     40	60

80    100

-40

-20     0

20     40	60

80    100

-40

-20     0

20     40	60

80    100

MAX793/MAX794/MAX795

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

| Typical Operating Characteristics (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VCC = VRST MAX |  |  |

(TA = +25°C, unless otherwise noted.)

MAX794

RESET IN THRESHOLD AND LOWLINE-TO-RESET IN THRESHOLD vs. TEMPERATURE

MAX793

BATT OK THRESHOLD vs. TEMPERATURE

MAX793 TOC12

CE IN-TO-CE OUT ON-RESISTANCE

vs. TEMPERATURE

| 1.242 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | VRESET IN |  |  |  |  |  |
|  | VLOWLINE - VRST |  |  |  |  |  |
|  |  |  |  | VCC FALLING |  |  |

RESET IN THRESHOLD (V)

1.241

1.240

1.239

1.238

1.237

30	2.5	60

MAX793 TOC10

BATT OK THRESHOLD (V)

MAX793 TOC11

25	2.0	50

20	40

1.5

LOWLINE-TO-RESET IN THRESHOLD (mV)

CE IN-TO-CE OUT ON-RESISTANCE (Ω)

| 15 30 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VBATT FALLING |  |  |

1.0

10	20

0.5

5	10

1.236

-40

-20     0

20     40	60

0

80    100

0

-40

-20     0

20     40	60

80    100

0

| -40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VCC FALLING |  |  |

-20     0

20     40	60

80    100

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

1.70

1.65

1.60

WATCHDOG TIMEOUT PERIOD (sec)

1.55

MAX793/MAX794

MAX793 TOC13

WATCHDOG TIMEOUT PERIOD

vs. TEMPERATURE

MAX793/MAX794

BATTERY FRESHNESS SEAL LEAKAGE CURRENT vs. TEMPERATURE

20

MAX793 TOC14

VBATT = 5.5V VCC = 0V VOUT = 0V

LEAKAGE CURRENT (nA)

15

10

5

1.002

1.001

1.000

VRST (NORMALIZED)

0.999

0.998

0.997

RESET THRESHOLD

MAX793 TOC15

vs. TEMPERATURE (NORMALIZED)

1.50

-40

-20     0

20     40	60

| 80 100 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

0

-40

-20     0

20     40	60

80    100

0.996

-40

-20     0

20     40	60

80    100

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

MAX793/MAX794

MAX793 TOC16

PFI TO PFO PROPAGATION DELAY

vs. TEMPERATURE

10

PROPAGATION DELAY (µs)

8

6

4

2

0

| -40 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | VPFI FALLING 20mV OVERDRIVE |  |  |

-20     0

20     40	60

80    100

MAX793/MAX794/MAX795

TEMPERATURE (°C)

MAX793/MAX794/MAX795

Pin  Description

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| MAX793/ MAX794 | MAX795 |  |  |
| 1 | 1 | OUT | Supply Output for CMOS RAM. When VCC rises above the reset threshold or above VBATT, OUT is connected to VCC through an internal p-channel MOSFET switch. When VCC falls below VSW and VBATT, BATT connects to OUT. |
| 2 | 2 | VCC | Main Supply Input |
| 3 | — | BATT OK (MAX793) | Battery Status Output. High in normal operating mode when VBATT exceeds VBOK, other- wise low. VBATT is checked continuously. Disabled and logic low while VCC is below VSW. |
|  |  | RESET IN (MAX794) | Reset Input. Connect to an external resistor-divider to select the reset threshold. The reset threshold can be programmed anywhere in the VSW to 5.5V range. |
| 4 | — | PFI | Power-Fail Comparator Input. When PFI is less than VPFT or when VCC falls below VSW, PFO goes low; otherwise, PFO remains high (see Power-Fail Comparator section). Connect to VCC if unused. |
| 5 | 3 | BATT ON | Logic Output/External Bypass Switch-Driver Output. High when OUT switches to BATT. Low when OUT switches to VCC. Connect the base/gate of PNP/PMOS transistor to BATT ON for IOUT requirements exceeding 75mA. |
| 6 | 4 | GND | Ground |
| 7 | — | PFO | Power-Fail Comparator Output. When PFI is less than VPFT or when VCC falls below VSW, PFO goes low; otherwise, PFO remains high. PFO is also used to enable the bat- tery freshness seal (see Battery Freshness Seal, and Power-Fail Comparator sections). |
| 8 | — | MR | Manual Reset Input. A logic low on MR asserts reset. Reset remains asserted as long as MR is low and for 200ms after MR returns high. The active-low input has an internal 70µA pullup current. It can be driven from a TTL- or CMOS-logic line or shorted to ground with a switch. Leave open if unused. |
| 9 | — | WDO | Watchdog Output. WDO goes low if WDI remains either high or low for longer than the watchdog timeout period. WDO returns high on the next transition of WDI. WDO is a logic high for VSW < VCC < VRST, and low when VCC is below VSW. |
| 10 | — | WDI | Watchdog Input. If WDI remains either high or low for longer than the watchdog timeout period, the internal watchdog timer runs out and WDO goes low. WDO returns high on the next transition of WDI. Connect WDO to MR to generate a reset due to a watchdog fault. |
| 11 | 5 | CE IN | Chip-Enable Input. The input to the chip-enable gating circuit. Connect to GND if unused. |
| 12 | 6 | CE OUT | Chip-Enable Output. CE OUT goes low only when CE IN is low and reset is not asserted. If CE IN is low when reset is asserted, CE OUT remains low for 10µs or until CE IN goes high, whichever occurs first. CE OUT is pulled up to OUT. |
| 13 | — | RESET | Active-High Reset Output. Sources and sinks current. RESET is the inverse of RESET. |
| 14 | — | LOWLINE | Early Power-Fail Warning Output. Low when VCC falls to VLR. This output can be used to generate an NMI to provide early warning of imminent power failure. |
| 15 | 7 | RESET | Open-Drain, Active-Low Reset Output. Pulses low for 200ms when triggered, and stays low whenever VCC is below the reset threshold or when MR is a logic low. It remains low for 200ms after either VCC rises above the reset threshold, the watchdog triggers a reset (WDO connected to MR), or MR goes low to high. |
| 16 | 8 | BATT | Backup-Battery Input. When VCC falls below VSW and VBATT, OUT switches from VCC to BATT. When VCC rises above the reset threshold or above VBATT, OUT reconnects to VCC. VBATT can exceed VCC. Connect VCC, OUT, and BATT together if no battery is used. |

Detailed Description

General Timing Characteristics The MAX793/MAX794/MAX795 are designed  for 3.3V and 3V systems, and provide a number of supervisory functions (see the Selector Guide on the front page). Figures 1 and 2 show the typical timing relationships of the various outputs during  power-up and power-down with typical VCC rise and fall times.

Manual Reset Input (MAX793/MAX794) Many microprocessor-based  products  require manual- reset capability,  allowing the operator, a test technician, or external logic  circuitry  to initiate a reset. On the MAX793/MAX794, a logic low on MR asserts reset. Reset remains asserted while MR is low, and for tRP (200ms) after it returns high.  During the first half of the reset time-

out period (tRP), the state of MR is ignored if PFO is exter- nally forced low to facilitate enabling the battery fresh- ness seal.  MR has an internal 70µA pullup current, so it can be left open if it is not used.  This input can be driven with TTL- or CMOS-logic levels, or with open-drain/collec- tor outputs.  Connect a normally open momentary switch from MR to GND to create a manual-reset function; exter- nal debounce  circuitry is not required. If MR is driven from long cables or the device is used in a noisy environ- ment, connect a 0.1µF capacitor  from MR to ground to provide additional noise immunity.

Reset Outputs A microprocessor’s  (µP’s) reset input starts the µP in a known state.   These MAX793/MAX794/MAX795 µP supervisory circuits assert a reset to prevent code exe- cution  errors during  power-up,  power-down,  and

VSW

VRST

VLL

VCC

VLOWLINE (MAX793/MAX794) VRESET (PULLED  UP TO VCC)

VRESET (MAX793/MAX794)

5µs

tRP

tRP

VCE OUT	VBATT

VWDO

(MAX793/MAX794)

VBOK

(MAX793)

25µs

25µs

tRP/2

PFO (MAX793/MAX794)

BATT ON

SHOWN FOR VCC = 0V to 3.3V, VBATT = 3.6V, CE IN = GND. TYPICAL PROPAGATION DELAYS REFLECT A 40mV OVERDRIVE.

MAX794:  VRESET IN = VCC (VRST IN / VRST)

25µs

25µs

tRP/2

(PFO FOLLOWS PFI)

MAX793/MAX794/MAX795

Figure 1. Timing Diagram, VCC Rising

brownout conditions.   RESET is guaranteed  to be a

logic  low for 0V < VCC < VRST, provided  VBATT is greater than 1V. Without a backup  battery (VBATT = VCC = VOUT), RESET is guaranteed valid for VCC ≥ 1V. Once VCC exceeds the reset threshold, an internal timer keeps RESET low for the reset timeout period (tRP); after this interval, RESET becomes high imped-

ance (Figure 2). RESET is an open-drain  output, and

requires a pullup  resistor to VCC (Figure 3). Use a

4.7kΩ to 1MΩ pullup  resistor that provides sufficient

current to assure the proper logic levels to the µP.

If a brownout condition  occurs  (VCC dips  below the reset threshold), RESET goes low. Each time RESET is asserted, it stays low for the reset timeout period. Any time VCC goes below the reset threshold, the internal timer restarts.

The watchdog output (WDO) can also be used to initi- ate a reset. See the Watchdog Output section.

The RESET output is the inverse of the RESET output, and it can both source and sink current.

VCC

VLOWLINE

(MAX793/MAX794)

VRESET

(RESET PULLED UP TO VCC)

VRESET

(MAX793/MAX794)

VCE OUT

VWDO

(MAX793/MAX794)

VBOK

(MAX793)

VPFO

(MAX793/MAX794)

VLL    VRST

4µs

20µs

20µs

10µs

VSW

25µs

25µs

25µs

VBATT

25µs

VBATT ON

25µs

VBATT

MAX793/MAX794/MAX795

SHOWN FOR VCC = 3.3V to 0V, VBATT = 3.6V, CE IN = GND, PFI = VCC. TYPICAL DELAY TIMES REFLECT A 40mV OVERDRIVE

MAX794:  VRESET IN = VCC (VRST IN / VRST)

Figure 2. Timing Diagram, VCC Falling

3.3V

0.1µF

(OPTIONAL) Si9433DY SILICONIX

D 		S PMOS

0.1µF

VCC

VRST 	VRST

R1	VCC   BATT ON

OUT	CMOS RAM

RESET IN

R2

CE OUT

VCC

RESET

tRP

3.6V

0.1µF

BATT

MAX794

CE IN WDI

ADDRESS DECODER

A0-A15

I/O

PFO

PFO STATE LATCHED, FRESHNESS SEAL ENABLED.

WDO

MR

+5V SUPPLY

LOWLINE

VCC

4.7kΩ

NMI

(EXTERNALLY HELD AT 0V)

RESET PULLED UP TO VCC

+5V

FAILURE

PFO

PFI

GND

RESET

RESET

VRST = VRST IN  (R1  + 1)

R2

Figure 4.  Battery Freshness Seal Enable Timing

Using the standard  application  circuit  (Figure 3), the reset threshold can be programmed  anywhere in the range of VSW (the battery switch threshold) to 5.5V. Reset is asserted when VCC falls below VSW.

MAX793/MAX794/MAX795

Figure 3.  MAX794 Standard Application Circuit

Reset Threshold The MAX793T/MAX795T are intended for 3.3V systems with a ±5% power-supply tolerance and a 10% systems tolerance.  Except when MR is asserted, reset does not assert as long as the power supply  remains above

3.15V (3.3V - 5%). Reset is guaranteed to assert before the power supply falls below 3.0V (3.3V - 10%).

The MAX793S/MAX795S are designed  for 3.3V ±10% power supplies.  Except when MR is asserted, they are guaranteed  not to assert reset as long as the supply remains above 3.0V (3.0V is just above 3.3V - 10%). Reset is guaranteed to assert before the power supply falls below 2.85V (3.3V - 14%).

The MAX793R/MAX795R are optimized to monitor 3.0V

±10% power supplies.  Reset does not occur until VCC falls below 2.7V (3.0V - 10%), but is guaranteed  to occur before the supply falls below 2.55V (3.0V - 15%).

Program the MAX794’s reset threshold with an external voltage divider to RESET IN. The reset-threshold toler- ance is a combination  of the RESET IN tolerance and the tolerance of the resistors used to make the external voltage divider. Calculate the reset threshold as follows:

VRST =  VRST IN  (R1 / R2 + 1)

Battery Freshness Seal The MAX793/MAX794’s battery freshness seal discon- nects the backup battery from internal circuitry until it is needed.  This allows an OEM to ensure that the backup battery connected to BATT is fresh when the final prod- uct is put to use. To enable the freshness seal, connect a battery to BATT, ground PFO, bring  VCC above the reset threshold, and hold it there until reset is deassert- ed following the reset timeout period, then bring  VCC back  down again (Figure 4). Once the battery fresh- ness seal is enabled (disconnecting  the backup battery from the internal circuitry  and anything connected  to OUT), it remains enabled  until VCC is brought  above VRST. Note that connecting  PFO to MR does not inter- fere with battery freshness seal operation.

BATT OK  Output (MAX793) BATT OK indicates  the status of the backup  battery. When reset is not asserted, the MAX793 checks  the battery voltage continuously.  If VBATT is below VBOK (2.0V min), BATT OK goes low; otherwise, it remains pulled  up to VCC. BATT OK also goes low when VCC goes below VSW.

Watchdog Input (MAX793/MAX794) In the MAX793/MAX794, the watchdog circuit monitors the µP’s activity. If the µP does not toggle the watchdog input (WDI) within 1.6s, WDO goes low. The internal

1.6s timer is cleared and WDO returns high either when

VCC

VRST

VCC

MAX793/MAX794

4.7kΩ

RESET

tRP

WDO

MR

RESET

TO µP

WDO

VCC

WDI

WDO CONNECTED TO  µP INTERRUPT RESET PULLED UP TO VCC

tWD

WDO RESET

WDI

tRP 	tWP

∼10µs

tRP

Figure 5.  Watchdog  Timing Relationship

Figure 6.  Generating  a Reset on Each Watchdog  Fault

MAX793/MAX794/MAX795

a reset occurs or when a transition (low-to-high or high- to-low) takes place at WDI. As long as reset is assert- ed, the timer remains cleared and does not count. As soon as reset is released or WDI changes  state, the timer starts counting (Figure 5). WDI can detect pulses as short as 100ns. Unlike the 5V MAX690 family, the watchdog function cannot be disabled.

Watchdog Output (MAX793/MAX794) In the MAX793/MAX794, WDO remains high (WDO is pulled up to VCC) if there is a transition or pulse at WDI during the watchdog  timeout period. WDO goes low if no transition occurs at WDI during the watchdog timeout period. The watchdog function is disabled and WDO is a logic high when reset is asserted if VCC is above VSW. WDO is a logic low when VCC is below VSW.

If a system reset is desired  on every watchdog  fault, simply  diode-OR  connect  WDO to MR (Figure 6). When a watchdog fault occurs in this mode, WDO goes low, pulling MR low, which causes a reset pulse to be issued. Ten microseconds  after reset is asserted, the watchdog  timer clears and WDO returns high. This delay results in a 10µs pulse at WDO, allowing external circuitry to capture a watchdog fault indication.  A con- tinuous high or low on WDI causes 200ms reset pulses to be issued every 1.6s.

Chip-Enable Signal Gating Internal gating of chip-enable (CE) signals prevents erro- neous data from corrupting CMOS RAM in the event of an undervoltage condition. The MAX793/MAX794/MAX795 use a series transmission gate from CE IN to CE OUT During normal operation (reset not asserted), the CE transmission gate is enabled and passes all CE transi- tions. When reset is asserted, this path becomes dis- abled, preventing erroneous data from corrupting  the CMOS RAM. The short CE propagation delay from CE IN to CE OUT enables these µP supervisors to be used with most µPs. If CE IN is low when reset asserts, CE OUT remains low for typically 10µs to permit completion of the current write cycle.

Chip-Enable Input The CE transmission gate is disabled and CE IN is high impedance  (disabled  mode) while reset is asserted. During a power-down sequence when VCC passes the reset threshold, the CE transmission gate disables and CE IN immediately becomes high impedance if the volt- age at CE IN is high. If CE IN is low when reset asserts, the CE transmission gate disables at the moment CE IN goes high, or 10µs after reset asserts, whichever occurs  first (Figure 8). This permits the current write cycle to complete during power-down.

CE IN

MAX793

MAX794

MAX795

CHIP-ENABLE OUTPUT CONTROL

RESET

GENERATOR	P

N

OUT P

CE OUT

The propagation  delay through the CE transmission gate depends  on VCC, the source impedance  of the drive connected to CE IN, and the loading on CE OUT (see the Chip-Enable Propagation Delay vs. CE OUT Load Capacitance  graph  in the Typical Operating Characteristics).   The CE propagation  delay is produc- tion tested from the 50% point on CE IN to the 50%

point on CE OUT using a 50Ω driver and 50pF of load

capacitance  (Figure 9). For minimum propagation

delay, minimize the capacitive load at CE OUT and use a low-output-impedance  driver.

Chip-Enable Output When the CE transmission gate is enabled, the imped- ance of CE OUT is equivalent to a 46Ω resistor in series with the source driving  CE IN. In the disabled  mode, the transmission gate is off and an active pullup  con- nects CE OUT to OUT (Figure 8). This pullup turns off when the transmission gate is enabled.

Early Power-Fail Warning

(MAX793/MAX794)

Figure 7.  Chip-Enable  Transmission Gate

The CE transmission gate remains disabled and CE IN remains high impedance  (regardless of CE IN activity) for the first half of the reset timeout period (tRP / 2), any time a reset is generated. While disabled, CE IN is high impedance. When the CE transmission gate is enabled, the impedance  of CE IN appears  as a 46Ω resistor in series with the load at CE OUT.

Critical systems often require an early warning indicat- ing that power is failing.  This warning provides time for the µP to store vital data and take care of any additional “housekeeping”  functions, before the power supply gets too far out of tolerance for the µP to operate reli- ably.  The MAX793/MAX794 offer two methods  of achieving this early warning. If access to the unregulat- ed supply is feasible, the power-fail comparator  input (PFI) can be connected  to the unregulated  supply

VCC

CE OUT

VBATT

VRST

tRP/2

VRST

VSW

10µs

VRST

VBATT

CC

V

VRST

VSW

MAX793/MAX794/MAX795

tRP

RESET

(PULLED TO VCC)

CE IN

VBATT = 3.6V

RESET PULLED UP TO VCC

Figure 8. Chip-Enable Timing

3.6V

BATT

VCC

VCC

pulled to VCC. Use LOWLINE to provide an NMI to the

µP when power begins to fall.

In most battery-operated  portable  systems, reserve energy in the battery provides ample time to complete the shutdown  routine once the low-line warning is encountered  and before reset asserts.   If the system

25Ω EQUIVALENT SOURCE IMPEDANCE

50Ω CABLE

MAX793

MAX794

MAX795

must also contend with a more rapid VCC fall time, such as when the main battery is disconnected  or a high- side switch is opened  during  normal operation, use capacitance  on the VCC line to provide time to execute

50Ω

50Ω

CE IN

CE OUT

GND

50pF CL*

the shutdown routine (Figure 11).

First, calculate the worst-case time required for the sys- tem to perform its shutdown routine. Then, with the worst- case shutdown time, the worst-case load current, and the minimum low-line to reset threshold (VLR min), calculate the amount of capacitance  required  to allow the shut-

*CL  INCLUDES  LOAD CAPACITANCE  AND SCOPE PROBE CAPACITANCE.

Figure 9.  CE Propagation  Delay Test Circuit

through a voltage divider, with the power-fail compara- tor output (PFO) providing  the NMI to the µP (Figure

10). If there is no easy access to the unregulated sup- ply, the LOWLINE output can be used to generate an NMI to the µP (see LOWLINE Output section).

LOWLINE Output (MAX793/MAX794) The low-line comparator monitors VCC with a threshold voltage typically 45mV above the reset threshold (10mV of hysteresis) for the MAX793, and 15mV above RESET IN (4mV of hysteresis) for the MAX794. For normal operation (VCC above the reset threshold), LOWLINE is

down routine to complete before reset is asserted:

CHOLD > ILOAD x tSHDN / VLR

where ILOAD  is the current being  drained  from the capacitor,  VLR is the low-line to reset threshold differ- ence (VLL - VRST), and tSHDN  is the time required  for the system to complete an orderly shutdown routine.

Power-Fail Comparator (MAX793/MAX794) The MAX793/MAX794’s PFI input is compared  to an internal reference. If PFI is less than the power-fail threshold (VPFT), PFO goes low. The power-fail com- parator is intended for use as an undervoltage detector to signal a failing power supply (Figure 12). However, the comparator does not need to be dedicated  to this function because it is completely separate from the rest of the circuitry.

UNREGULATED

SUPPLY 	3.0V OR 3.3V

REGULATOR

VCC

REGULATOR

3.0V OR 3.3V CHOLD

VCC

LOWLINE

TO µP NMI

R1

PFI

MAX793

MAX794

PFO

TO µP NMI

CHOLD

> ILOAD

x tSHDN

MAX793

MAX794

VLR

R2

GND

GND

MAX793/MAX794/MAX795

Figure 10. Using the Power-Fail Comparator to Generate

Power-Fail Warning

Figure 11. Using LOWLINE to Provide Power-Fail Warning to the µP

VCC

R1

3.0V OR 3.3V

VIN

R1

VCC

3.0V OR 3.3V

PFI R2

MAX793

MAX794

PFO

PFI R2

MAX793

MAX794

PFO

MR

GND

GND

VCC

VIN

VCC

PFO

(V      +  V

) (  1

VL	VTRIP	0V

1  )     VCC

VIN

PFO

R1 + R2

VTRIP	VH

VIN

VTRIP = R2

PFT

PFH

+	–

R1      R2	R1

WHERE VPFT = 1.237V

VTRIP =   VPFT (

R2     )

VL = R2

(VPFT

) (  1  +   1

) – VCC

VPFH = 10mV

V   =  (V

+  V      )

( R1 + R2 )

(a)

R1      R2	R1

NOTE: VTRIP, VL ARE NEGATIVE

(b)

H 	PFT

PFH	R2

Figure 12.  Using the Power-Fail Comparator  to Monitor an Additional  Power Supply:  (a) VIN Is Negative, (b) VIN Is Positive

MAX793/MAX794/MAX795

The power-fail comparator turns off and PFO goes low when VCC falls below VSW on power-down. During the first half of the reset timeout period (tRP), PFO is forced high, irrespective of VPFI. At the beginning  of the sec- ond half of tRP, the power-fail comparator  is enabled and PFO follows PFI. If the comparator is unused, con- nect PFI to VCC a nd leave PFO unconnected. PFO can be connected to MR so that a low voltage on PFI gener- ates a reset (Figure 12b). In this configuration, when the monitored voltage causes PFI to fall below VPFT, PFO pulls MR low, causing  a reset to be asserted. Reset remains asserted as long as PFO holds MR low, and for 200ms after PFO pulls MR high when the moni- tored supply is above the programmed threshold.

Backup-Battery Switchover In the event of a brownout or power failure, it may be necessary to preserve the contents of RAM. With a backup battery installed at BATT, the devices automati- cally switch RAM to backup  power when VCC falls.  In order to allow the backup  battery (e.g., a 3.6V lithium cell) to have a higher voltage than VCC, this family of µP supervisors (designed  for 3.3V and 3V systems) does not always connect  BATT to OUT when V BATT is greater than VCC. BATT connects to OUT (through a

140Ω switch) either when VCC falls below VSW and

VBATT is greater than VCC, or when VCC falls below

1.75V (typ) regardless of the BATT voltage.

Switchover at VSW ensures that battery-backup mode is entered before VOUT gets too close to the 2.0V mini- mum required  to reliably retain data in most CMOS RAM, (switchover  at higher  V CC  voltages  would decrease  backup-battery  life). When VCC recovers, switchover is deferred either until VCC crosses VBATT if VBATT is below VRST, or when VCC rises above the reset threshold (VRST) if VBATT is above VRST. This power-up  switchover technique  prevents VCC from charging  the backup  battery through OUT when using an external transistor driven by BATT ON. OUT con- nects to VCC through a 4Ω (max) PMOS power switch when VCC crosses the reset threshold (Figure 13).

BATT ON (MAX793/MAX794) BATT ON is high when OUT is connected  to BATT. Although BATT ON can be used as a logic output to indicate  the battery switchover status, it is most often used as a gate or base drive for an external pass tran- sistor for high-current  applications  (see Driving an External Switch with BATT ON in the Applications Information section). When VCC exceeds  VRST on power-up,  BATT ON sinks 3.2mA at 0.4V.  In battery- backup mode, this terminal sources 100µA from BATT.

VCC

VOUT

VBATT = 3.6V

VRST

3.6V

3.3V

VSW

3.6V

3.3V

to VCC, the collector to OUT, and the base to BATT ON (Figure 14a). No current-limiting resistor is required, but a resistor connecting the base of the PNP to BATT ON can be used to limit the current drawn from VCC, pro- longing battery life in portable equipment.

If you are using a PMOS transistor, however, it must be connected  backwards  from the traditional method. Connect the gate to BATT ON, the drain to VCC, and the source to OUT (Figure 14b). This method orients the body  diode  from VCC to OUT and prevents the backup battery from discharging  through the FET when its gate is high. Two PMOS transistors in the Siliconix LITTLE FOOT® series are specified  with VGS down to

-2.7V. The Si9433DY has a maximum 100mΩ drain-

Figure 13.  Battery Switchover Timing

Table 1. Input and Output Status in

| Battery-Backup Mode PIN NAME | STATUS |
| --- | --- |
| OUT | Connected to BATT through an internal 140Ω switch |
| VCC | Disconnected from OUT |
| BATT ON | Pulled up to BATT |
| BATT OK | Logic low |
| PFI | Disabled |
| PFO | Logic low |
| MR | Disabled, but still pulled up to VCC |
| WDO | Logic low |
| WDI | Disabled |
| RESET | Logic low |
| RESET | Pulled up to VCC |
| BATT | Connected to OUT |
| LOWLINE | Logic low |
| CE IN | High impedance |
| CE OUT | Pulled to BATT |

source on-resistance with 2.7V of gate drive and a 2A drain-source  current. The Si9434DY specifies a 60mΩ drain-source on-resistance with 2.7V of gate drive and

a 5.1A drain-source current.

Using a Super Cap  as  a Backup

Power Source Super caps are capacitors with extremely high capaci- tance values (e.g., order of 0.47F) for their size.  Figure

15 shows two ways to use a super cap  as a backup power  source. The super  cap  can be connected through a diode to the 3V input (Figure 15a); or, if a 5V supply is also available, the super cap can be charged up to the 5V supply  (Figure 15b), allowing a longer backup  period.  Since VBATT can exceed  VCC while VCC is above the reset threshold, there are no special precautions  when using these µP supervisors with a super cap.

MAX793/MAX794/MAX795

Applications Information

These µP supervisory circuits are not short-circuit pro- tected. Shorting VOUT to ground,  excluding  power-up transients such as charging  a decoupling  capacitor, destroys the device.  Decouple  both VCC and BATT pins to ground by placing 0.1µF ceramic capacitors as close to the device as possible.

Driving an  External Switch with BATT ON BATT ON can be directly connected  to the base of a PNP transistor or the gate of a PMOS transistor. The PNP connection is straightforward:  connect the emitter

Operation without a

Backup Power Source These µP supervisors  were designed  for battery- backed  applications.  If a backup  battery is not used, connect BATT, OUT, and VCC together, or use a differ- ent µP supervisor. See the µP Supervisory Circuits table at the end of this data sheet.

Replacing the Backup Battery The backup  power source can be removed while VCC remains valid, without danger  of triggering  a reset pulse, provided  that BATT is decoupled  with a 0.1µF capacitor  to ground.  As long as VCC stays above the reset threshold,  battery-backup  mode  cannot be entered.

LITTLE FOOT is a registered trademark of Siliconix Inc.

3.0V OR 3.3V 	TO CMOS RAM

PMOS FET

BODY DIODE

D 		S G

VCC

BATT ON

OUT

VCC

BATT ON

OUT

MAX793

MAX794

MAX795

MAX793

MAX794

MAX795

GND

GND

(a)

(b)

Figure 14.  Driving an External Transistor with BATT ON

3.0V OR 3.3V

+5V

VCC

MAX793

OUT	TO STATIC

3.0V OR

3.3V

VCC

MAX793

OUT	TO STATIC

1N4148

MAX794

VCC

RAM

1N4148

MAX794

VCC

RAM

BATT

RESET	TO µP

BATT

RESET	TO µP

0.47F

GND

0.47F

GND

(a)

(b)

Figure 15.  Using a Super Cap as a Backup  Source

MAX793/MAX794/MAX795

Adding Hysteresis to the Power-Fail

Comparator (MAX793/MAX794)

The power-fail comparator has a typical input hystere- sis of 10mV. This is sufficient for most applications where a power-supply  line is being monitored through an external voltage divider (see the section Monitoring an Additional Power Supply).

If additional noise margin is desired, connect a resistor between PFO and PFI as shown in Figure 16a. Select the ratio of R1 and R2 such that PFI sees VPFT when VIN falls to its trip point (VTRIP). R3 adds the additional hysteresis and should typically be more than 10 times the value of R1 or R2.  The hysteresis window extends

both above (VH) and below (VL) the original trip point

(VTRIP).

Connecting an ordinary signal diode in series with R3, as shown in Figure 16b, causes the lower trip point (VL) to coincide with the trip point without hysteresis (VTRIP), so the entire hysteresis window occurs  above VTRIP. This method provides additional  noise margin without compromising  the accuracy  of the power-fail threshold when the monitored voltage is falling. It is useful for accurately detecting when a voltage falls past a thresh- old. The current through R1 and R2 should be at least

1µA to ensure that the 25nA (max over temperature) PFI input current does not shift the trip point. R3 should be larger than 82kΩ so it does not load down the PFO pin.  Capacitor C1 is optional, and adds noise rejection.

VIN

VIN

R1	VCC

R1	VCC

R2	R3

C1*

PFI

PFO

MAX793

MAX794

GND

R2	R3

C1*

PFI

PFO

MAX793

MAX794

GND

TO µP

*OPTIONAL

TO µP

*OPTIONAL

PFO

0V

0V

VL 	VH

VTRIP

VIN

PFO

0V

0V

V	= V

(R1 +  R2 )

VTRIP	VH

VIN

VTRIP = VPFT  (R1 +  R2)

R2

WHERE VPFT = 1.237V

TRIP

PFT	R2

1	1	1	VD

R1      R2      R3	R3

VH = (VPFT +  VPFH) (R1)    1

(

R1

1	1

1	1

+	+

R2      R3

)

1	VCC

(	)

VPFH = 10mV

VH = R1   (VPFT +  VPFH)  (	+	+	)  –

WHERE VPFT = 1.237V

(a)

VL = R1  VPFT

+	+	–

R1      R2      R3	R3

(b)

VPFH = 10mV

VD  = DIODE FORWARD VOLTAGE DROP

VL  = VTRIP

Figure 16.  Adding  Hysteresis to the Power-Fail Comparator: (a) Symmetrical  Hysteresis, (b) Hysteresis Only on Rising VIN

Monitoring an  Additional Power Supply These µP supervisors can monitor either positive or negative supplies  using a resistor voltage divider  to PFI. PFO can be used to generate an interrupt to the µP or to cause reset to assert (Figure 12).

Interfacing to µPs  with

Bidirectional Reset Pins Since the RESET output is open drain, the MAX793/ MAX794/MAX795 interface easily with µPs that have bidirectional  reset pins, such as the Motorola 68HC11.

VCC

RESET

VCC

RESET N

RESET

VCC

Connecting  the RESET output of the µP supervisor directly to the RESET input of the microcontroller with a single pullup  resistor allows either device  to assert reset (Figure 17).

Negative-Going VCC  Transients These supervisors are relatively immune to short-dura- tion negative-going VCC transients (glitches) while issu- ing resets to the µP during power-up, power-down, and brownout conditions.  Therefore, resetting the µP when VCC experiences only small glitches is usually not rec- ommended.

GENERATOR

MAX793

MAX794

MAX795

GND

µP

GND

MAX793/MAX794/MAX795

Figure 17. Interfacing to µPs with Bidirectional Reset I/O

Figure 18 shows maximum transient duration vs. reset- comparator  overdrive, for which reset pulses are not generated.  The graph  was produced  using negative- going VCC pulses, starting at 3.3V and ending below the reset threshold by the magnitude  indicated  (reset comparator  overdrive). The graph shows the maximum pulse width a negative-going  VCC transient can typically have without causing  a reset pulse to be issued.   As the amplitude of the transient increases (i.e., goes far- ther below the reset threshold), the maximum allowable pulse width decreases.  Typically, a VCC transient that goes 40mV below the reset threshold and lasts for 10µs or less does not cause a reset pulse to be issued.

A 0.1µF bypass  capacitor  mounted close to the VCC

| pin provides additional transient immunity. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Watchdog Software Considerations There is a way to help the watchdog timer monitor soft- ware execution more closely, which involves setting and resetting the watchdog  input at different points in the program  rather than pulsing  the watchdog  input high-low-high or low-high-low. This technique avoids a stuck loop, in which the watchdog timer would continue to be reset within the loop, keeping the watchdog from timing out. Figure 19 shows an example of a flow dia- gram where the I/O driving the watchdog  input is set high at the beginning  of the program,  set low at the beginning  of every subroutine or loop, then set high again when the program returns to the beginning. If the program  should hang in any subroutine, the problem would quickly be corrected, since the I/O is continually set low and the watchdog timer is allowed to time out, causing a reset or interrupt to be issued.

100

MAXIMUM  PULSE DURATION (µs)

90

80

70

60

50

40

30

20

10

0

10  20   30

MAX793-FIG 18

40   50   60  70   80   90 100

START

SET WDI HIGH

PROGRAM CODE

Subroutine or

Program Loop

RESET COMPARATOR OVERDRIVE, VRST - VCC (mV)

SET WDI LOW

MAX793/MAX794/MAX795

Figure 18. Maximum Transient Duration without Causing a

Reset Pulse vs. Reset Comparator Overdrive

RETURN

Figure 19. Watchdog Flow Diagram

_Ordering Information (continued) PART* TEMP RANGE PIN- PKG PACKAGE CODE

MAX793_EPE -40°C to +85°C 16 Plastic DIP P16-1

MAX793_ESE -40°C to +85°C 16 Narrow SO S16-1

MAX794CPE 0°C to +70°C 16 Plastic DIP P16-1

MAX794CSE 0°C to +70°C 16 Narrow SO S16-1

MAX794EPE -40°C to +85°C 16 Plastic DIP P16-1

MAX794ESE -40°C to +85°C 16 Narrow SO S16-1

MAX795_CPA 0°C to +70°C 8 Plastic DIP P8-1

MAX795_CSA 0°C to +70°C 8 SO S8-2

MAX795_EPA -40°C to +85°C 8 Plastic DIP P8-1

MAX795_ESA -40°C to +85°C 8 SO S8-2

Pin  Configurations

TOP VIEW

OUT   1

VCC    2

16   BATT

15    RESET

MAX793/MAX794/MAX795

*The MAX793/MAX795 offer a choice of reset threshold voltage. Select the letter corresponding to the desired reset threshold voltage range (T = 3.00V to 3.15V,  S = 2.85V to 3.00V, R =

2.55V to 2.70V) and insert it into the blank to complete  the part number.  The MAX794’s reset threshold is adjustable.

Devices are available in both leaded and lead-free packaging. Specify lead free by adding  the + symbol at the end of the part number when ordering.

Chip Information

TRANSISTOR COUNT: 1271

(RESET IN) BATT OK PFI

BATT ON GND PFO MR

OUT VCC

BATT ON

GND

( ) ARE FOR MAX794

3	14

4	MAX793 	13

MAX794

5	12

6	11

7	10

8	9

DIP/Narrow  SO

1	8

2	7

MAX795

3	6

4	5

DIP/SO

LOWLINE RESET

CE OUT CE IN WDI WDO

BATT RESET CE OUT CE IN

MAX793/MAX794/MAX795

PDIPN.EPS

| Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) |  |  |
| --- | --- | --- |

MAX793/MAX794/MAX795

3.0V/3.3V Adjustable Microprocessor

Supervisory Circuits

SOICN .EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | 0.053 | 0.069 | 1.35 | 1.75 |
| A1 | 0.004 | 0.010 | 0.10 | 0.25 |
| B | 0.014 | 0.019 | 0.35 | 0.49 |
| C | 0.007 | 0.010 | 0.19 | 0.25 |
| e | 0.050 BSC |  | 1.27 BSC |  |
| E | 0.150 | 0.157 | 3.80 | 4.00 |
| H | 0.228 | 0.244 | 5.80 | 6.20 |
| L | 0.016 | 0.050 | 0.40 | 1.27 |

|  | INCHES |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX | N | MS012 |
| D | 0.189 | 0.197 | 4.80 | 5.00 | 8 | AA |
| D | 0.337 | 0.344 | 8.55 | 8.75 | 14 | AB |
| D | 0.386 | 0.394 | 9.80 | 10.00 | 16 | AC |

| N E H VARIATIONS: 1 TOP VIEW D A C e B A1 0∞-8∞ L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, .150" SOIC |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0041 | REV. B | 1 1 |

Revision History

Pages changed at Rev 5: 1–4, 7, 11, 15, 16, 19, 20, 21

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

21    	Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 (408) 737-7600

©  2007 Maxim Integrated  Products 	is a registered  trademark  of Maxim Integrated  Products, Inc.