19-0487; Rev 5; 12/05

5-Pin Microprocessor Supervisory Circuits With

Watchdog Timer and Manual Reset

General Description

The MAX823/MAX824/MAX825* microprocessor  (µP) supervisory circuits combine  reset output, watchdog, and manual reset input functions in 5-pin SOT23 and SC70 packages. They significantly improve system relia- bility and accuracy compared to separate ICs or discrete components. The MAX823/MAX824/MAX825 are specifi- cally designed to ignore fast transients on VCC.

Seven preprogrammed  reset threshold voltages are available (see Reset Threshold Table). All three devices have an active-low reset output, which is guaranteed to be in the correct state for VCC down to 1V. The MAX823 also offers a watchdog  input and manual reset input. The MAX824 offers a watchdog  input and a comple- mentary active-high reset. The MAX825 offers a manual reset input and a complementary active-high reset. The Selector Guide explains the functions offered in this series of parts.

*Patents Pending

Applications

Computers and Controllers Embedded Controllers Intelligent Instruments Automotive Systems

Critical µP Monitoring

Portable/Battery-Powered Equipment

| Reset Threshold Table SUFFIX | RESET THRESHOLD (V) |
| --- | --- |
| L | 4.63 |
| M | 4.38 |
| T | 3.08 |
| S | 2.93 |
| R | 2.63 |
| Z (SC70 only) | 2.32 |
| Y (SC70 only) | 2.19 |

Features

♦ Precision Monitoring of +2.5V, +3V, +3.3V, and

+5V Power Supplies

♦ Operating Current: 6µA (MAX823L/M) (SC70)

2µA (MAX825T/S/R/Z/Y) (SC70)

♦ Fully Specified Over Temperature

♦ 140ms min Power-On Reset

♦ Guaranteed RESET Valid to VCC = 1V

♦ Power-Supply Transient Immunity

♦ Watchdog Timer with 1.6s Timeout

(MAX823/MAX824)

♦ Manual Reset Input (MAX823/MAX825)

♦ No External Components

Ordering Information

PART† TEMP. RANGE PIN-PACKAGE

MAX823_EXK-T -40°C to +85°C 5 SC70-5

MAX823_EUK-T -40°C to +125°C 5 SOT23-5

MAX824_EXK-T -40°C to +85°C 5 SC70-5

†Insert the desired suffix letter (from the Reset Threshold table) into the blank to complete the part number. All devices are available in tape-and-reel only. There is a 2500 piece minimum order increment. Devices are available in both leaded and lead-free packaging. Specify lead-free by replacing “-T” with “+T” when ordering.

Ordering Information continued at end of data sheet.

Selector Guide

| FUNCTION | MAX823 | MAX824 | MAX825 |
| --- | --- | --- | --- |
| Active-Low Reset | ✔ | ✔ | ✔ |
| Active-High Reset | — | ✔ | ✔ |
| Watchdog Input | ✔ | ✔ | — |
| Manual Reset Input | ✔ | — | ✔ |

Typical Operating Circuit appears at end of data sheet. Marking Information appears at end of data sheet.

Pin  Configurations

TOP VIEW

RESET    1	5

VCC

RESET

1	5   VCC

RESET     1	5

VCC

GND   2

MAX823

GND    2

MAX824

GND    2

MAX825

MR    3	4

WDI

RESET

3	4   WDI

RESET     3	4   MR

MAX823/MAX824/MAX825

SOT23/SC70

SOT23/SC70

SOT23/SC70

ABSOLUTE MAXIMUM RATINGS

VCC ........................................................................-0.3V to +6.0V All Other Pins..............................................-0.3V to (VCC + 0.3V) Input Current, All Pins Except RESET and RESET ..............20mA Output Current, RESET, RESET ..........................................20mA Continuous Power Dissipation  (TA = +70°C)

5-Pin SC70 (derate 3.1mW/°C above +70°C) ...............247mW

5-Pin SOT23 (derate 7.1mW/°C above +70°C).............571mW Operating  Temperature  Range

MAX82_EXK......................................................-40°C to +85°C MAX82_EUK ...................................................-40°C to +125°C Storage Temperature  Range .............................-65°C to +150°C Lead Temperature  (soldering,  10s) .................................+300°C

MAX823/MAX824/MAX825

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(VCC = +4.75V  to +5.5V  for MAX82_L, VCC = +4.5V  to +5.5V  for MAX82_M, VCC = +3.15V  to +3.6V  for MAX82_T, VCC = +3V to +3.6V for MAX82_S, VCC = +2.7V to +3.6V for MAX82_R, VCC = +2.38V to +2.75V for MAX82_Z, VCC = +2.25V to +2.75V for MAX82_Y, TA = TMIN to TMAX, TA = -40°C to +85°C  (SC70), TA = -40°C to +125°C  (SOT23), unless otherwise  noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Operating Voltage Range | VCC | TA = 0°C to +70°C |  | 1.0 5.5 | V |
|  |  | TA = TMIN to TMAX |  | 1.2 |  |
| Supply Current (SOT23 Only) | ISUPPLY | WDI and MR unconnected | MAX823L/M MAX824L/M | 10 24 | µA |
|  |  |  | MAX823T/S/R/Z/Y MAX824T/S/R/Z/Y | 5 12 |  |
|  |  | MR unconnected | MAX825L/M | 4.5 12 |  |
|  |  |  | MAX825T/S/R/Z/Y | 3 8 |  |
| Supply Current (SC70 Only) | ISUPPLY | WDI and MR unconnected | MAX823L/M MAX824L/M | 6 17 | µA |
|  |  |  | MAX823T/S/R/Z/Y MAX824T/S/R/Z/Y | 4 12 |  |
|  |  | MR unconnected | MAX825L/M | 3 8 |  |
|  |  |  | MAX825T/S/R/Z/Y | 2 6 |  |
| Reset Threshold | VRST | MAX82_L | TA = +25°C | 4.56 4.63 4.70 | V |
|  |  |  | TA = TMIN to TMAX | 4.50 4.75 |  |
|  |  | MAX82_M | TA = +25°C | 4.31 4.38 4.45 |  |
|  |  |  | TA = TMIN to TMAX | 4.25 4.50 |  |
|  |  | MAX82_T | TA = +25°C | 3.04 3.08 3.11 |  |
|  |  |  | TA = TMIN to TMAX | 3.00 3.15 |  |
|  |  | MAX82_S | TA = +25°C | 2.89 2.93 2.96 |  |
|  |  |  | TA = TMIN to TMAX | 2.85 3.00 |  |
|  |  | MAX82_R | TA = +25°C | 2.59 2.63 2.66 |  |
|  |  |  | TA = TMIN to TMAX | 2.55 2.70 |  |
|  |  | MAX82_Z (SC70 only) | TA = +25°C | 2.28 2.32 2.35 |  |
|  |  |  | TA = TMIN to TMAX | 2.25 2.38 |  |
|  |  | MAX82_Y (SC70 only) | TA = +25°C | 2.16 2.19 2.22 |  |
|  |  |  | TA = TMIN to TMAX | 2.13 2.25 |  |

MAX823/MAX824/MAX825

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +4.75V  to +5.5V  for MAX82_L, VCC = +4.5V  to +5.5V  for MAX82_M, VCC = +3.15V  to +3.6V  for MAX82_T, VCC = +3V to +3.6V  for MAX82_S, VCC = +2.7V  to +3.6V  for MAX82_R, VCC = +2.38V  to +2.75V  for MAX82_Z, VCC = +2.25V  to +2.75V  for MAX82_Y, TA = TMIN to TMAX, TA = -40°C to +85°C (SC70), TA = -40°C to +125°C  (SOT23), unless otherwise  noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| Reset Threshold Hysteresis |  | MAX82_L/M | 10 | mV |
|  |  | MAX82_T/S/R/Z/Y | 5 |  |
| Reset Threshold Temperature Coefficient |  |  | 40 | ppm/°C |
| Reset Timeout Period | tRP |  | 140 200 280 | ms |
| VCC to RESET Delay |  | VRST - VCC = 100mV | 20 | µs |
| RESET Output Voltage | VOH | MAX82_L/M, VCC = VRST max, ISOURCE = 120µA | VCC - 1.5 | V |
|  |  | MAX82_T/S/R/Z/Y, VCC = VRST max, ISOURCE = 30µA | 0.8 ✕ VCC |  |
|  | VOL | MAX82_L/M, VCC = VRST min, ISINK = 3.2mA | 0.4 |  |
|  |  | MAX82_T/S/R/Z/Y VCC = VRST min, ISINK = 1.2mA | 0.3 |  |
|  |  | TA = 0°C to +70°C, VCC = 1V, VCC falling, ISINK = 50µA | 0.3 |  |
|  |  | TA = TMIN to TMAX, VCC = 1.2V, VCC falling, VBATT = 0V, ISINK = 100µA |  |  |
| RESET Output Short-Circuit Current (Note 2) | ISOURCE | MAX82_L/M, RESET = 0V, VCC = 5.5V | 800 | µA |
|  |  | MAX82_T/S/R/Z/Y, RESET = 0V, VCC = 3.6V | 400 |  |
| RESET Output Voltage | VOH | VCC > 1.8V, ISOURCE = 150µA | 0.8 ✕ VCC | V |
|  | VOL | MAX824L/M, MAX825L/M, VCC = VRST max, ISINK = 3.2mA | 0.4 |  |
|  |  | MAX824T/S/R/Z/Y, MAX825T/S/R/Z/Y, VCC = VRST max, ISINK = 1.2mA | 0.3 |  |
| WATCHDOG INPUT (MAX823/MAX824) |  |  |  |  |
| Watchdog Timeout Period | tWD |  | 1.12 1.60 2.40 | s |
| WDI Pulse Width | tWDI | VIL = 0.4V, VIH = 0.8 ✕ VCC | 50 | ns |
| WDI Input Voltage (Note 3) | VIL |  | 0.3 ✕ VCC | V |
|  | VIH |  | 0.7 ✕ VCC |  |
| WDI Input Current (Note 4) |  | WDI = VCC, time average | 120 160 | µA |
|  |  | WDI = 0, time average | -20 -15 |  |

MAX823/MAX824/MAX825

ELECTRICAL CHARACTERISTICS (continued)

(VCC = +4.75V  to +5.5V  for MAX82_L, VCC = +4.5V  to +5.5V  for MAX82_M, VCC = +3.15V  to +3.6V  for MAX82_T, VCC = +3V to +3.6V  for MAX82_S, VCC = +2.7V  to +3.6V  for MAX82_R, VCC = +2.38V  to +2.75V  for MAX82_Z, VCC = +2.25V  to +2.75V  for MAX82_Y, TA = TMIN to TMAX, TA = -40°C to +85°C (SC70), TA = -40°C to +125°C  (SOT23), unless otherwise  noted. Typical values are at TA = +25°C.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| MANUAL RESET INPUT (MAX823/MAX825) |  |  |  |  |
| MR Input Voltage | VIL |  | 0.3 ✕ VCC | V |
|  | VIH |  | 0.7 ✕ VCC |  |
| MR Pulse Width |  |  | 1.0 | µs |
| MR Noise Immunity (pulse width with no reset) |  |  | 100 | ns |
| MR to Reset Delay |  |  | 500 | ns |
| MR Pullup Resistance (internal) |  |  | 35 52 75 | kΩ |

Note 1: Over-temperature limits are guaranteed by design and not production tested.

Note 2: The RESET short-circuit current is the maximum pullup current when RESET is driven low by a µP bidirectional reset pin.

Note 3: WDI is internally serviced  within the watchdog period if WDI is left unconnected.

Note 4: The WDI input current is specified as the average  input current when the WDI input is driven high or low. The WDI input is designed to drive a three-stated  output  device  with a 10µA maximum  leakage  current  and a maximum  capacitive load of

200pF. This output device must be able to source and sink at least 200µA when active.

Typical Operating Characteristics

MAX823_, VCC = +5V, TA = +25°C, unless otherwise noted.)

VCC SUPPLY CURRENT

vs. TEMPERATURE

9

MAX823/4/5 toc01

(SC70 ONLY)

8

SUPPLY CURRENT (µA)

7	MAX823L

6

5

MAX824Y

4

3

MAX825R

2

1

| 250 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

240

230

220

210

200

190

180

RESET TIMEOUT PERIOD (ms)

170

160

150

RESET TIMEOUT PERIOD

PROPAGATION DELAY (µs)

vs. TEMPERATURE

RESET COMPARATOR PROPAGATION DELAY

vs. TEMPERATURE

30

MAX823/4/5 toc03

VCC FALLING

25

MAX823/4/5 toc02

20

MAX823/4/5 toc06

15

10

5

0

-40

-20	0

20    40

60    80   100  120

-40    -20

0	20

40	60

80    100

-40    -20

0	20

40	60

80    100

TEMPERATURE (°C)

TEMPERATURE (°C)

TEMPERATURE (°C)

2.0

WATCHDOG TIMEOUT PERIOD (s)

1.9

1.8

1.7

1.6

1.5

1.4

1.3

1.2

1.1

1.0

WATCHDOG TIMEOUT PERIOD

vs. TEMPERATURE

MAX823/4/5 toc04

| 1.06 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

1.04

1.02

1.00

0.98

0.96

0.94

NORMALIZED RESET THRESHOLD VOLTAGE vs. TEMPERATURE

MAXIMUM VCC TRANSIENT DURATION

MAX823/4/5 toc05

vs. RESET THRESHOLD OVERDRIVE

50

45	(SC70 ONLY)

TRANSIENT DURATION (µs)

40

35

30

25	RESET OCCURS

20	ABOVE CURVE

15	MAX82_Y

10

MAX82_R

5	MAX82_L

0

NORMALIZED RESET THRESHOLD (V)

| -40 -20 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

0	20

40	60

80    100

-40    -20

0	20

40	60

80    100

0   20

40  60  80 100 120 140 160 180 200

MAX823/MAX824/MAX825

TEMPERATURE (°C)

TEMPERATURE (°C)

RESET THRESHOLD OVERDRIVE (mV),  VRST - VCC

Pin  Description

|  | PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- |
| MAX823 | MAX824 | MAX825 |  |  |
| 1 | 1 | 1 | RESET | Active-Low Reset Output. Pulses low for 200ms when triggered, and remains low whenever VCC is below the reset threshold or when MR is a logic low. It remains low for 200ms after one of the following occurs: VCC rises above the reset threshold, the watchdog triggers a reset, or MR goes low to high. |
| 2 | 2 | 2 | GND | Ground |
| 3 | — | 4 | MR | Manual Reset Input. A logic low on MR asserts reset. Reset remains asserted as long as MR is held low and for 200ms after MR returns high. The active-low input has an internal 52kΩ pullup resistor. It can be driven from a CMOS logic line or shorted to ground with a switch. Leave open or connect to VCC if unused. |
| — | 3 | 3 | RESET | Active-High Reset Output. Inverse of RESET. |
| 4 | 4 | — | WDI | Watchdog Input. If WDI remains either high or low for longer than the watch- dog timeout period, the internal watchdog timer runs out and a reset is trig- gered. The internal watchdog timer clears whenever reset is asserted, or whenever WDI sees a rising or falling edge. If WDI is left unconnected or is connected to a three-stated buffer output, the watchdog feature is disabled. |
| 5 | 5 | 5 | VCC | Supply Voltage |

VCC

VCC

1.25V

RESET GENERATOR

MAX823

MAX824

MAX825

RESET RESET

(MAX824/MAX825

ONLY)

MR (MAX823/MAX825

ONLY)

WDI (MAX823/MAX824

ONLY)

WATCHDOG TRANSITION DETECTOR

WATCHDOG TIMER

MAX823/MAX824/MAX825

GND

Figure 1. Functional Diagram

Detailed Description

RESET Output A microprocessor’s  (µP’s) reset input starts the µP in a known state. The MAX823/MAX824/MAX825 µP super- visory circuits assert a reset to prevent code-execution errors during  power-up,  power-down,  and brownout conditions.  RESET is guaranteed  to be a logic low for VCC down to 1V. Once VCC exceeds the reset thresh- old, an internal timer keeps RESET low for the specified reset timeout period  (tRP); after this interval, RESET returns high (Figure 2).

If a brownout condition  occurs  (VCC dips  below the reset threshold), RESET goes low. Each time RESET is asserted it stays low for the reset timeout period. Any time VCC goes below the reset threshold the internal timer restarts. RESET both sources and sinks current. RESET on the MAX824/MAX825 is the inverse of RESET.

Manual Reset Input (MAX823/MAX825) Many µP-based products require manual reset capabili- ty, allowing the operator, a test technician,  or external logic  circuitry  to initiate a reset. On the MAX823/ MAX825, a logic low on MR asserts reset. Reset remains asserted while MR is low, and for tRP (200ms nominal) after it returns high. MR has an internal 52kΩ pullup resistor, so it can be left open if not used. This input can be driven with CMOS logic levels or with open-drain/ collector outputs. Connect a normally open momentary switch from MR to GND to create a manual-reset func- tion; external debounce circuitry is not required. If MR is driven from long cables or the device is used in a noisy environment, connect  a 0.1µF capacitor  from MR to GND to provide additional noise immunity.

Watchdog Input (MAX823/MAX824) In the MAX823/MAX824, the watchdog circuit monitors the µP’s activity. If the µP does not toggle the watchdog input (WDI) within tWD (1.6s), reset asserts. The internal

1.6s timer is cleared by either a reset pulse or by tog- gling  WDI, which detects  pulses as short as 50ns. While reset is asserted, the timer remains cleared and does not count. As soon as reset is released, the timer starts counting (Figure 3).

Disable the watchdog  function by leaving WDI uncon- nected or by three-stating the driver connected to WDI. The watchdog  input is internally driven low during  the first 7/8 of the watchdog timeout period and high for the last 1/8 of the watchdog  timeout period. When WDI is left unconnected,  this internal driver clears the 1.6s timer every 1.4s. When WDI is three-stated or uncon- nected,  the maximum allowable leakage current is

10µA and the maximum allowable load capacitance  is

200pF.

Applications Information

Watchdog Input Current The MAX823/MAX824 WDI inputs are internally driven through a buffer and series resistor from the watchdog counter (Figure 1). When WDI is left unconnected,  the watchdog timer is serviced within the watchdog timeout period by a low-high-low pulse from the counter chain. For minimum watchdog input current (minimum overall power consumption), leave WDI low for the majority of the watchdog  timeout period,  pulsing  it low-high-low once within the first 7/8 of the watchdog timeout period to reset the watchdog timer. If WDI is externally driven high for the majority of the timeout period, up to 160µA can flow into WDI.

VCC

1V

VRST	VRST	1V

VCC

tRST

RESET

tRP

tRD

RESET*

tRP

tWD

tRP

RESET GND

tRP

tRD

WDI

*RESET ON THE MAX824/MAX825 IS THE INVERSE OF RESET.

MAX823/MAX824/MAX825

Figure 2. Reset Timing Diagram

Figure 3. MAX823/MAX824 Watchdog Timing Relationship

Interfacing to µPs  with

Bidirectional Reset Pins The RESET output maximum pullup current is 800µA for L/M versions (400µA for T/S/R/Z/Y versions). This allows

µPs with bidirectional  resets, such as the 68HC11, to force RESET low when the MAX823/MAX824/MAX825 are pulling RESET high (Figure 4).

Negative-Going VCC Transients These supervisors  are relatively immune to short- duration, negative-going VCC transients (glitches), which usually do not require the entire system to shut down. Resets are issued to the µP during  power-up, power- down, and brownout conditions.

The Typical Operating Characteristics show a graph of the MAX823_’s Maximum VCC Transient Duration vs. Reset Threshold Overdrive, for which reset pulses are not generated.  The graph was produced  using nega- tive-going VCC pulses, starting at 5V and ending below the reset threshold by the magnitude  indicated  (reset threshold overdrive). The graph  shows the maximum pulse width that a negative-going  VCC transient can typically  have without triggering  a reset pulse. As the amplitude of the transient increases (i.e., goes farther below the reset threshold), the maximum allowable pulse width decreases.

An optional 0.1µF bypass capacitor  mounted close to

VCC provides additional transient immunity.

Watchdog Software Considerations

(MAX823/MAX824) One way to help the watchdog  timer monitor software execution more closely is to set and reset the watchdog input at different points in the program,  rather than pulsing the watchdog  input high-low-high or low-high- low. This technique  avoids a stuck loop, in which the watchdog  timer would continue to be reset inside the loop, keeping the watchdog from timing out.

Figure 5 shows an example of a flow diagram where the I/O driving the watchdog input is set high at the begin- ning of the program, set low at the beginning  of every subroutine or loop, then set high again when the pro- gram returns to the beginning.  If the program  should hang in any subroutine, the problem would quickly be corrected,  since the I/O is continually set low and the watchdog  timer is allowed to time out, causing a reset or interrupt to be issued. As described  in the Watchdog Input Current section, this scheme results in higher time average WDI input current than does leaving WDI low for the majority of the timeout period  and periodically pulsing it low-high-low.

VCC

START

RESET

VCC

MAX823

MAX824

MAX825

VCC

ISOURCE MAX = 800µA L, M

400µA T, S, R, Z, Y

VCC

SET WDI HIGH

PROGRAM CODE

GENERATOR

RESET

µP

SUBROUTINE OR PROGRAM LOOP

SET WDI LOW

GND

GND

RETURN

MAX823/MAX824/MAX825

Figure 4. Interfacing to µPs with Bidirectional Resets

Figure 5. Watchdog Flow Diagram

Typical Operating Circuit

VCC

Chip Information

TRANSISTOR COUNT: 607

PROCESS TECHNOLOGY: BiCMOS

VCC

RESET

RESET

VCC

MAX823	µP

MANUAL RESET

MR

GND

WDI

I/O

GND

Ordering Information (continued)

PART† TEMP. RANGE PIN-PACKAGE

MAX824_EUK-T -40°C to +125°C 5 SOT23-5

MAX825_EXK-T -40°C to +85°C 5 SC70-5

MAX825_EUK-T -40°C to +125°C 5 SOT23-5

†Insert the desired suffix letter (from the Reset Threshold table) into the blank to complete the part number. All devices are available in tape-and-reel only. There is a 2,500 piece minimum order increment. Devices are available in both leaded and lead-free packaging. Specify lead-free by replacing “-T” with “+T” when ordering.

Marking Information

MAX823/MAX824/MAX825

MARKING INFORMATION (TOP) XXXX 	(SOT ONLY)

AAAI  = MAX823L AAAJ  = MAX823M AAAK = MAX823T AAAL = MAX823S AAAM = MAX823R AAAN = MAX824L AAAO = MAX824M AAAP = MAX824T AAAQ = MAX824S AAAR = MAX824R AAAS = MAX825L AAAT = MAX825M AAAU = MAX825T AAAV = MAX825S AAAW = MAX825R

MARKING INFORMATION (TOP) XXXX 	(SC70 ONLY)

AAL = MAX823L AAM = MAX823M AAP = MAX823R AAO = MAX823S AAN = MAX823T AAR = MAX823Y AAQ = MAX823Z AAS = MAX824L AAT = MAX824M AAW = MAX824R AAV = MAX824S AAU = MAX824T AAY = MAX824Y AAX = MAX824Z AAZ = MAX825L ABA = MAX825M ABD = MAX825R ABC = MAX825S ABB = MAX825T ABF = MAX825Y ABE = MAX825Z

MAX823/MAX824/MAX825

SOT-23 5L .EPS

Package Information (The package drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package outline information, go to  www.maxim-ic.com/packages.)

|  | PACKAGE OUTLINE, SOT-23, 5L |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 21-0057 | E | 1 1 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

10   	Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  2005 Maxim Integrated  Products 	Printed USA 	is a registered  trademark  of Maxim Integrated  Products, Inc.