19-2409; Rev 1; 9/02

Dual ECL and Dual/Quad PECL,  500ps, Ultra-High-Speed Comparators

General Description

The MAX9600/MAX9601/MAX9602 ultra-high-speed com- parators feature extremely low propagation  delay (500ps). These dual and quad comparators minimize propagation delay skew (10ps) and are designed for low propagation  delay dispersion (30ps). These features make them ideal for applications where high-fidelity track- ing of narrow pulses and low timing dispersion is critical.

The differential input stage accepts a wide range of signals in the common-mode range from (VEE + 3V) to (VCC - 2V). The outputs are complementary digital signals, compatible with ECL and PECL systems, and provide sufficient current to directly drive transmission lines terminated in 50Ω.

The MAX9600/MAX9601 dual-channel ECL and dual-chan- nel PECL output comparators incorporate latch enable (LE_, LE_), and hysteresis (HYS_). The complementary latch-enable control permits tracking, track-hold, or sample- hold mode of operations. The latch enables can be driven with standard ECL logic for MAX9600 and PECL logic for MAX9601. The MAX9602 quad-channel  PECL output comparator is ideal for high-density packaging  in limit- ed board space.

The MAX9600/MAX9601 are available in 20-pin TSSOP packages,  and the MAX9602 is offered in a 24-pin TSSOP package.  The MAX9600/MAX9601/MAX9602 are specified for operation from -40°C to +85°C.

Features

♦ 500ps Propagation Delay

♦ 30ps Propagation Delay Dispersion

♦ 4Gbps Tracking Frequency

♦ -2.2V to +3V Input Range with +5V/-5.2V Supplies

♦ -1.2V to +4V Input Range with +6V/-4.2V Supplies

♦ Differential ECL Outputs (MAX9600)

♦ Differential PECL Outputs (MAX9601/MAX9602)

♦ Latch Enable (MAX9600/MAX9601)

♦ Adjustable Hysteresis (MAX9600/MAX9601)

Ordering Information

PART TEMP RANGE PIN-PACKAGE

MAX9600EUP -40°C to +85°C 20 TSSOP

MAX9601EUP -40°C to +85°C 20 TSSOP

MAX9602EUG -40°C to +85°C 24 TSSOP

Selector Guide PART PIN-PACKAGE SELECTION

Dual ECL Output MAX9600EUP 20 TSSOP Comparator with Latch Enable and Hysteresis

Dual PECL Output MAX9601EUP 20 TSSOP Comparator with Latch Enable and Hysteresis

Quad PECL Output MAX9602EUG 24 TSSOP Comparator

Applications

VLSI and High-Speed Memory ATE High-Speed Instrumentation Scope/Logic Analyzer Front Ends High-Speed Triggering

Threshold and Peak Detection

Line Receiving/Signal Restoration

Pin Configurations  appear at end of data sheet.

Functional Diagrams

VCC   VEE GND

VCC   VEE VCCO_

VCC   VEE VCCO_

IN_+ 	Q_

IN_+ 	Q_

IN_+ 	Q_

IN_- HYS_

RHYS_

1/2 MAX9600

LE_  LE_

RL 	RL

ECL OUTPUT Q_

IN_- HYS_

RHYS_

1/2 MAX9601

LE_  LE_

RL 	RL

PECL OUTPUT Q_

IN_-

1/4 MAX9602

RL 	RL

PECL OUTPUT Q_

VT = -2V

VT = VCCO_ - 2V

VT = VCCO_ - 2V

MAX9600/MAX9601/MAX9602

THE OPEN-EMITTER  OUTPUTS REQUIRE EXTERNAL PULLDOWN  RESISTORS (RL). USE RESISTORS IN THE RANGE OF 50Ω TO 75Ω CONNECTED TO VT. CURRENT-CONTROLLED  HYSTERESIS REQUIRES A SINGLE EXTERNAL RESISTOR (RHYS_)  FROM HYS_ TO GND IN THE RANGE OF 10kΩ TO 35kΩ.

ABSOLUTE MAXIMUM RATINGS

VS = VCC - VEE ...................................................................12.0V VCC to GND (MAX9600) .......................................................6.8V VEE to GND (MAX9600)  ......................................................-6.5V Differential Input Voltage  ...................................................±6.5V Latch Differential Voltage ......................................................±4V Common-Mode  Input Voltage (VCM) .........................VEE to VCC VCCO_ to VEE

(MAX9601/MAX9602)....................(VEE - 0.3V) to (VCC + 0.3V) LE_, LE_ to GND

MAX9600 ....................................................(VEE - 0.3V) to 0.3V MAX9601 ..................................(VEE  - 0.3V) to (VCCO_ + 0.3V)

Input Current to Any Input Pin.............................................10mA HYS_ Current (MAX9600/MAX9601) ...................................-1mA Continuous Output Current .................................................50mA Continuous Power Dissipation (TA = +70°C)

20-Pin TSSOP (derate 10.9mW/oC above +70°C) ........879mW

24-Pin TSSOP (derate 12.2mW/°C above +70°C) ........975mW Operating Temperature Range ...........................-40°C to +85°C Junction Temperature ......................................................+150°C Storage Temperature Range .............................-65°C to +150°C Lead Temperature (soldering,  10s) .................................+300°C

MAX9600/MAX9601/MAX9602

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

DC ELECTRICAL CHARACTERISTICS

(VCC = 5V, VEE = -5.2V, VCM = 0V, HYS_ = open (MAX9600/MAX9601),  LE_ = low, LE_ = high (MAX9600/MAX9601),  GND = 0V, RL =  50Ω  to -2V (MAX9600),  VCCO_ =  5V, RL =  50Ω  to 3V (MAX9601/MAX9602),  TA =  TMIN to TMAX. Typical  values  are at TA = +25°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| INPUT (IN_+, IN_-) |  |  |  |  |  |  |
| Input Differential Voltage Range | VID | Guaranteed by input bias current tests |  |  | -5.2 +5.2 | V |
| Input Common-Mode Voltage | VCM | Guaranteed by input bias current tests |  |  | VEE + 3 VCC - 2 | V |
| Input Offset Voltage | VOS | TA = +25°C |  |  | ±1 ±5 | mV |
|  |  | TMIN ≤ TA ≤ TMAX |  |  | ±9 |  |
| Input Offset-Voltage Tempco | TCVOS |  |  |  | 8 | µV/°C |
| Input Offset-Voltage Channel Matching |  |  |  |  | 1 | mV |
| Input Bias Current | IB | VID = ±5.2V |  |  | 6 20 | µA |
| Input Bias-Current Tempco | TCIB |  |  |  | 10 | nA/°C |
| Input Offset Current | IOS |  |  |  | 0.3 ±5 | µA |
| Input Resistance | RIN | Differential mode (VID ≤ 10mV) |  |  | 10 | kΩ |
|  |  | Common mode (VEE + 3V) ≤ VCM ≤ (VCC - 2V) |  |  | 100 | MΩ |
| LATCH INPUT (LE_, LE_) |  |  |  |  |  |  |
| Latch Differential Input Voltage | VLD | Guaranteed by latch input current |  | MAX9600 | 0.4 2.0 | V |
|  |  |  |  | MAX9601 | 0.25 3.50 |  |
| Latch Input Voltage Range | VLR | MAX9600 |  |  | -2 0 | V |
|  |  | MAX9601 |  | VCCO_ ≥ 3.5V | VCCO_ VCCO_ - 3.5 |  |
|  |  |  |  | VCCO_ < 3.5V | 0 VCCO_ |  |
| Latch Input Current | ILE, ILE | MAX9600 |  |  | 5 20 | µA |
|  |  | MAX9601 |  |  | 5 20 |  |
| HYSTERESIS INPUT (HYS_) |  |  |  |  |  |  |
| Input-Referred Hysteresis |  | MAX9600/MAX9601 | RHYS = ∞ |  | 0 | mV |
|  |  |  | RHYS = 16.4kΩ |  | 30 |  |

MAX9600/MAX9601/MAX9602

DC ELECTRICAL CHARACTERISTICS  (continued)

(VCC = 5V, VEE = -5.2V, VCM = 0V, HYS_ = open (MAX9600/MAX9601),  LE_ = low, LE_ = high (MAX9600/MAX9601),  GND = 0V, RL =  50Ω  to -2V (MAX9600),  VCCO_ =  5V, RL =  50Ω  to 3V (MAX9601/MAX9602),  TA =  TMIN to TMAX. Typical  values  are at TA = +25°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| OUTPUT (Q_, Q ) |  |  |  |  |  |
| Logic Output High Voltage | VOH | TA = +25°C | MAX9600 | -1.10 -0.94 -0.75 | V |
|  |  |  | MAX9601/MAX9602 | VCCO_ VCCO_ VCCO_ - 1.10 - 0.94 - 0.75 |  |
|  |  | TA = TMIN | MAX9600 | -1.2 -1.02 -0.8 |  |
|  |  |  | MAX9601/MAX9602 | VCCO_ VCCO_ VCCO_ - 1.2 - 1.02 - 0.8 |  |
|  |  | TA = TMAX | MAX9600 | -1.05 -0.87 -0.70 |  |
|  |  |  | MAX9601/MAX9602 | VCCO_ VCCO_ VCCO_ - 1.05 - 0.87 - 0.70 |  |
| Logic Output Low Voltage | VOL | TA = +25°C | MAX9600 | -1.95 -1.72 -1.55 | V |
|  |  |  | MAX9601/MAX9602 | VCCO_ VCCO_ VCCO_ - 1.95 - 1.72 - 1.55 |  |
|  |  | TA = TMIN | MAX9600 | -2.0 -1.78 -1.6 |  |
|  |  |  | MAX9601/MAX9602 | VCCO_ VCCO_ VCCO_ - 2.0 - 1.78 - 1.6 |  |
|  |  | TA = TMAX | MAX9600 | -1.9 -1.66 -1.50 |  |
|  |  |  | MAX9601/MAX9602 | VCCO_ VCCO_ VCCO_ - 1.9 - 1.66 - 1.5 |  |
| SUPPLY |  |  |  |  |  |
| Positive Supply Voltage | VCC | Guaranteed by output swing tests |  | 4.3 5 6.3 | V |
| Negative Supply Voltage | VEE | Guaranteed by output swing tests |  | -6 -5.2 -4 | V |
| Supply Voltage Difference | VS | VS = (VCC - VEE), guaranteed by output swing tests |  | 9.5 11.5 | V |
| Logic Supply Voltage | VCCO_ | MAX9601/MAX9602 |  | 2.4 VCC | V |
| Positive Supply Current | ICC | (Note 2) | MAX9600 | 16 24 | mA |
|  |  |  | MAX9601 | 19 27 |  |
|  |  |  | MAX9602 | 28 39 |  |
| Negative Supply Current | IEE | (Note 2) | MAX9600 | 21 28 | mA |
|  |  |  | MAX9601 | 24 33 |  |
|  |  |  | MAX9602 | 38 49 |  |
| Power-Supply Dissipation | PDISS | (Note 2) | MAX9600 | 190 266 | mW |
|  |  |  | MAX9601 | 220 307 |  |
|  |  |  | MAX9602 | 338 450 |  |
| Common-Mode Rejection Ratio | CMRR | (VEE + 3V) ≤ VCM ≤ (VCC - 2V) |  | 70 | dB |
| Power-Supply Rejection Ratio | PSRR | 4.3V ≤ VCC ≤ 6.3V, -6V ≤ VEE ≤ -4V, 9.5V ≤ VS ≤ 11.5V |  | 65 | dB |

MAX9600/MAX9601/MAX9602

AC ELECTRICAL CHARACTERISTICS

(VCC = 5V, VEE = -5.2V, VCM = 0V, HYS_ = open  (MAX9600/MAX9601),  LE_ = low, LE_ = high  (MAX9600/MAX9601),  CL = 5pF, GND = 0V, RL = 50Ω to -2V (MAX9600), VCCO_ = 5V, RL = 50Ω to 3V (MAX9601/MAX9602),  TA = TMIN to TMAX. Typical values are at TA = +25°C, unless otherwise noted.) (Note 1)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Tracking Frequency Toggle Rate | fMAX | VOUT = 550mVP-P, input overdrive = 100mV |  | 4 | Gbps |
| Minimum Pulse Width | tPW | VOUT = 550mVP-P, input overdrive = 100mV |  | 250 | ps |
| Propagation Delay | tPD-, tPD+ | Input overdrive = 100mV, Figure 1, (Note 3) |  | 500 700 | ps |
| Propagation Delay Tempco | TCtPD |  |  | 0.5 | ps/°C |
| Propagation Delay Skew | tPDSKEW | Input overdrive = 100mV (Note 4) |  | 10 | ps |
| Propagation Delay Match |  | Input overdrive = 100mV (Note 5) |  | 40 | ps |
| Propagation Delay Dispersion Overdrive |  | 10mV to 100mV |  | 15 | ps |
|  |  | 100mV to 2V |  | 40 |  |
| Propagation Delay Dispersion Common-Mode Voltage |  | VIN = 1VP-P input overdrive = 100mV | (VEE + 3V) ≤ VCM ≤ (VCC - 2V) | 10 | ps |
| Propagation Delay Dispersion Input Slew Rate |  |  | 0.2V/ns to 10V/ns | 40 |  |
| Propagation Delay Dispersion Duty Cycle |  |  | 10% to 90% at 250MHz | 30 |  |
| Propagation Delay Dispersion Pulse Width |  |  | 350ps to 1ns | 20 |  |
| Unit-to-Unit Propagation Delay Match |  | Input overdrive = 100mV |  | 50 | ps |
| Output Jitter |  | VIN = 2VP-P; 50MHz |  | 300 | fs |
| Input Capacitance | CIN | IN_+ or IN_, with respect to GND |  | 2 | pF |
| Latch Setup Time | tLS | Figure 1, (Notes 3, 6) |  | 250 80 | ps |
| Latch Hold Time | tLH | Figure 1, (Notes 3, 6) |  | 300 85 | ps |
| Minimum Pulse Width | tLPW | Figure 1 |  | 250 | ps |
| Latch to Output Delay | tLPD | Figure 1 |  | 200 | ps |
| Rise Time and Fall Time | tR, tF | 20% to 80%, Figure 1 |  | 200 | ps |

Note 1: All devices are 100% production  tested at TA = +25°C. Specifications over temperature  are guaranteed  by design.

Note 2: Does not include output state current in Q_, Q_.

Note 3: Guaranteed by design.

Note 4: Propagation  delay skew (tPDSKEW) is for a single channel and is the difference between the propagation delay to the high- to-low output transition vs. the low-to-high output transition.

Note 5: Propagation delay match is the difference of tPD- or tPD+ of one channel to the tPD- or tPD+ of another channel of the same device.

Note 6: Latch setup and hold-timing  specifications are for a differentially  driven latch signal.

Typical Operating Characteristics (VCC = 5V, VEE = -5.2V, VCM = 0V, HYS _ = open (MAX9600/MAX9601),  LE_ = low, LE_ = high (MAX9600/MAX9601),  CL = 5pF, GND = 0V, RL = 50Ω  to -2V (MAX9600),  VCCO_ = 5V, RL = 50Ω  to 3V (MAX9601/MAX9602),  input  slew rate = 2V/ns, duty  cycle  = 50%,

TA = TMIN to TMAX. Typical values are at TA = +25°C, unless otherwise noted.) (Note 1)

MAX9600/1/2 toc01

PROPAGATION DELAY vs. INPUT OVERDRIVE (VCC = 10mV TO 100mV)

530

PROPAGATION DELAY vs. INPUT OVERDRIVE (VOD = 0.1V  TO 2V)

560

3500

PROPAGATION DELAY

MAX9600/1/2 toc03

vs. SOURCE IMPEDANCE

520

510

500

PROPAGATION DELAY (ps)

490

480

470

| 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

20     30

40     50

60     70

80     90

100

540

PROPAGATION DELAY (ps)

520

500

480

460

440

420

400

| 0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

0.2   0.4   0.6   0.8   1.0   1.2   1.4   1.6   1.8   2.0

3000

MAX9600/1/2 toc02

2500

2000

1500

PROPAGATION DELAY (ps)

1000

500

0

0

100

MAX9600/1/2 toc05

200

300

400

500

| 600 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

INPUT OVERDRIVE (mV)

INPUT OVERDRIVE (V)

SOURCE IMPEDANCE (Ω)

1500

1250

1000

750

PROPAGATION DELAY (ps)

500

250

0

PROPAGATION DELAY

vs. CAPACITIVE LOAD

MAX9600/1/2 toc04

| 550 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

540

PROPAGATION DELAY (ps)

530

520

510

500

490

480

470

PROPAGATION DELAY

vs. TEMPERATURE

| 530 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

520

510

500

PROPAGATION DELAY (ps)

490

480

470

PROPAGATION DELAY

MAX9600/1/2 toc05

vs. COMMON-MODE  VOLTAGE

| 0 5 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

15	20

25	30

-50

-25	0	25

50	75

100

-3	-2	-1	0	1	2	3

MAX9600/MAX9601/MAX9602

CAPACITIVE LOAD (pF)

TEMPERATURE (°C)

COMMON-MODE  VOLTAGE (V)

Typical Operating Characteristics (continued) (VCC = 5V, VEE = -5.2V, VCM = 0V, HYS _ = open (MAX9600/MAX9601), LE_ = low, LE_ = high (MAX9600/MAX9601), CL = 5pF, GND = 0V, RL = 50Ω  to -2V (MAX9600),  VCCO_ = 5V, RL = 50Ω  to 3V (MAX9601/MAX9602),  input  slew rate = 2V/ns, duty  cycle  = 50%,

TA = TMIN to TMAX. Typical values are at TA = +25°C, unless otherwise noted.) (Note 1)

| 550 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

540

530

520

510

500

490

480

470

460

450

PROPAGATION DELAY

vs. PULSE WIDTH

| 540 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

530

520

510

500

PROPAGATION DELAY (ps)

490

480

470

PROPAGATION DELAY

MAX9600/1/2 toc07

vs. INPUT SLEW RATE

| 550 | FREQUENCY = 250MHz |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

540

MAX9600/1/2 toc08

PROPAGATION DELAY (ps)

530

520

510

500

490

480

470

460

450

PROPAGATION DELAY

MAX9600/1/2 toc09

vs. DUTY CYCLE

300

400

500

600     700

800

900

1000

0      1      2      3

4      5      6

7      8      9     10

0     10

20    30    40    50

60    70    80

90   100

PULSE WIDTH (ps)

INPUT SLEW RATE (V/ns)

DUTY CYCLE (%)

| 300 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

INPUT OFFSET VOLTAGE (µV)

200

100

0

-100

-200

-300

INPUT OFFSET VOLTAGE

HYSTERESIS (mV)

vs. TEMPERATURE

HYSTERESIS

vs. RHYS  TO GND

MAX9600/1/2 toc10

| 70 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

60

50

MAX9600/1/2 toc11

40

30

20

10

0

HYSTERESIS

vs. TEMPERATURE

35

MAX9600/1/2 toc12

RHYS = 16.4kΩ

34

33

HYSTERESIS (mV)

32

31

30

29

28

27

26

25

-50

-25	0

25	50

75	100

10	15

20	25

30	35	40

-50

-25	0

25	50

75	100

MAX9600/MAX9601/MAX9602

PROPAGATION DELAY (ps)

TEMPERATURE (°C)

RHYS (kΩ)

TEMPERATURE (°C)

Typical Operating Characteristics (continued) (VCC = 5V, VEE = -5.2V, VCM = 0V, HYS _ = open (MAX9600/MAX9601), LE_ = low, LE_ = high (MAX9600/MAX9601), CL = 5pF, GND = 0V, RL = 50Ω  to -2V (MAX9600),  VCCO_ = 5V, RL = 50Ω  to 3V (MAX9601/MAX9602),  input  slew rate = 2V/ns, duty  cycle  = 50%,

TA = TMIN to TMAX. Typical values are at TA = +25°C, unless otherwise noted.) (Note 1)

| 8.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

7.5

7.0

6.5

6.0

MAX9600/1/2 toc13

5.5

INPUT BIAS CURRENT (µA)

5.0

4.5

4.0

INPUT BIAS CURRENT

vs. TEMPERATURE

INPUT BIAS CURRENT

vs. INPUT VOLTAGE DIFFERENTIAL

20

TA = +85°C

INPUT BIAS CURRENT (µA)

15

10

TA = +25°C

5

0

TA = -40°C

-5

-0.75

MAX9600/1/2 toc14

OUTPUT VOLTAGE HIGH (V)

-0.80

-0.85

-0.90

-0.95

-1.00

-1.05

OUTPUT VOLTAGE HIGH

MAX9600/1/2 toc15

vs. TEMPERATURE

RL = 200Ω

RL = 100Ω

RL = 50Ω

-50

-25	0	25	50

75	100

-6	-4	-2	0	2	4	6

-50

-25	0

25	50

75	100

TEMPERATURE (°C)

INPUT VOLTAGE DIFFERENTIAL (V)

TEMPERATURE (°C)

-1.55

OUTPUT VOLTAGE LOW

vs. TEMPERATURE

MAX9600/1/2 toc17

OUTPUT RESPONSE TO 100MHz INPUT

MAX9600/1/2 toc18

OUTPUT RESPONSE TO 4Gbps INPUT

OUTPUT VOLTAGE LOW (V)

-1.60

-1.65

VIN

50mV/div

VIN

50mV/div

-1.70

-1.75

QOUT

200mV/div

QOUT - QOUT

200mV/div

-1.80

| -50 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | RL = 100Ω |  |  |  |
| RL = 200Ω |  |  |  |  |  |
|  |  |  | RL = 50Ω |  |  |

MAX9600/1/2 toc16

-25	0

25	50

75	100

2ns/div

200ps/div

MAX9600/MAX9601/MAX9602

TEMPERATURE (°C)

Timing Diagram

LE_

LATCH

VLD

VLE + VLE

2

COMPARE

LATCH

VLR (MAX)

LE_

IN_-

tLPD

tLPW

tFW(MIN)

tLS

tLH

VLR (MIN)

VID

VCM

IN_+

tPD- 	tPD+

Q_

VOH

Q_

– VOL

VOH + VOL

2

80%	80%

20%	20%

tR	tF

MAX9600/MAX9601/MAX9602

Figure 1. MAX9600/MAX9601/MAX9602  Timing Diagram

Pin  Descriptions

MAX9600/MAX9601

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| MAX9600 | MAX9601 |  |  |
| 1 | 1 | QA | Channel A Output |
| 2 | 2 | QA | Channel A Complementary Output |
| 3 | — | GND | Channel A Output Ground |
| — | 3 | VCCOA | Channel A Output Driver Positive Supply |
| 4 | 4 | LEA | Channel A Latch-Enable Input |
| 5 | 5 | LEA | Channel A Latch-Enable Complementary Input |
| 6, 15 | 6, 15 | VEE | Negative Supply Voltage |
| 7, 14 | 7, 14 | VCC | Positive Supply Voltage |
| 8 | 8 | HYSA | Channel A Hysteresis Input |
| 9 | 9 | INA- | Channel A Minus Input |
| 10 | 10 | INA+ | Channel A Plus Input |
| 11 | 11 | INB+ | Channel B Plus Input |
| 12 | 12 | INB- | Channel B Minus Input |
| 13 | 13 | HYSB | Channel B Hysteresis Input |
| 16 | 16 | LEB | Channel B Latch-Enable Complementary Input |
| 17 | 17 | LEB | Channel B Latch-Enable Input |
| 18 | — | GND | Channel B Output Ground |
| — | 18 | VCCOB | Channel B Output Driver Positive Supply |
| 19 | 19 | QB | Channel B Complementary Output |
| 20 | 20 | QB | Channel B Output |

Pin  Descriptions (continued)

MAX9602

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | INA+ | Channel A Plus Input |
| 2 | INA- | Channel A Minus Input |
| 3, 9 | VEE | Negative Supply Voltage |
| 4 | INB+ | Channel B Plus Input |
| 5 | INB- | Channel B Minus Input |
| 6, 12 | VCC | Positive Supply Voltage |
| 7 | INC+ | Channel C Plus Input |
| 8 | INC- | Channel C Minus Input |
| 10 | IND+ | Channel D Plus Input |
| 11 | IND- | Channel D Minus Input |
| 13 | QD | Channel D Complementary Output |
| 14 | QD | Channel D Output |
| 15 | VCCOD | Channel D Output Driver Positive Supply |
| 16 | QC | Channel C Complementary Output |
| 17 | QC | Channel C Output |
| 18 | VCCOC | Channel C Output Driver Positive Supply |
| 19 | QB | Channel B Complementary Output |
| 20 | QB | Channel B Output |
| 21 | VCCOB | Channel B Output Driver Positive Supply |
| 22 | QA | Channel A Complementary Output |
| 23 | QA | Channel A Output |
| 24 | VCCOA | Channel A Output Driver Positive Supply |

Detailed Description

The MAX9600/MAX9601/MAX9602 ultra-high-speed com- parators feature extremely low propagation  delay (500ps). These dual and quad comparators minimize channel-to-channel skew (10ps) and are designed for low propagation delay dispersion. These features make them ideal for applications where high-fidelity tracking of nar- row pulses and low timing dispersion is critical. The devices operate from either standard supply levels of

-5.2V/+5V or shifted levels of -4.2V/+6V.

The differential input stage accepts a wide range of sig- nals in the common-mode range from (VEE + 3V) to (VCC

- 2V) with a CMRR of 70dB (typ). The outputs are com- plementary digital signals, compatible  with ECL and PECL systems, and provide sufficient current to directly drive transmission lines terminated in 50Ω. The ultra-fast

INPUT

50mV/div

OUTPUT

200mV/div

500ps/div

0V

-0.9V

-1.7V

MAX9600/MAX9601/MAX9602

operation makes signal processing possible at a data rate up to 4Gbps. Figure 2 shows a 1Gbps (500MHz) example with an input-signal level of 100mVP-P.

Figure 2. Signal Processed at 500MHz with Input-Signal  Level of 100mVRMS.

MAX9600/MAX9601/MAX9602

The MAX9600/MAX9601 incorporate latch-enable and hysteresis control. Hysteresis rejects noise and pre- vents oscillations on low-slew input signals. The latch- enable control permits tracking or sampling mode of operations. Drive the complementary latch enable with standard ECL logic for MAX9600 and PECL logic for MAX9601. The MAX9602 quad-channel  PECL output comparator does not include the latch-enable or hys- teresis control functions.

Applications Information

Layout Special layout precautions exist due to the large gain- bandwidth  characteristic  of the MAX9600/MAX9601/ MAX9602. Use a printed circuit board with a good, low-

inductance  ground  plane. Mount 0.01µF ceramic decoupling  capacitors  as close to the power-supply inputs as possible. Minimize lead lengths on the inputs and outputs to avoid unwanted parasitic  feedback around the comparators. Use surface-mount chip com- ponents to minimize lead inductance. Pay close atten- tion to the bandwidth of the decoupling and terminating components.

Use microstrip layout and terminations at the input and output. Avoid discontinuities in differential impedance. Maximize common-mode noise immunity by maintain- ing the distance between differential traces and avoid sharp corners. Minimize the number of vias to prevent impedance  discontinuities. Match the electrical length of the traces to minimize skew.

Input Slew-Rate Requirements As with all high-speed  comparators,  the high gain- bandwidth product of these devices can create oscilla- tion problems  when the input  goes  through the threshold region. This is typically due to parasitic paths, which cause positive feedback  to occur.  For clean switching without oscillation or steps in the output waveform for the MAX9600/MAX9601, use an input with a slew rate of 5V/µs or faster. For the MAX9602, use a slew rate of 25V/µs or faster. The tendency of the part to oscillate is a function of the layout and source imped- ance of the circuit employed. Poor layout and larger source impedance  increases the minimum slew-rate requirement. Adding  hysteresis accommodates slower inputs (see the Hysteresis section).

Hysteresis (MAX9600/MAX9601) Hysteresis can be introduced  to prevent oscillation or multiple transitions due to noise. The MAX9600/ MAX9601 feature current-controlled hysteresis, which is set by placing a resistor between HYS_ and GND. The value of the current-setting resistor is determined by the

output voltage of 2.5V at HYS_ divided by the desired hysteresis current level in the range of 0 to 200µA. RHYS of 10kΩ to 35kΩ resistors provides hysteresis of

60mV to 5mV (see the Hysteresis vs. RHYS to GND graph in the Typical Operating Characteristics section). For a zero hysteresis (0µA hysteresis current), leave HYS_ open or connect it to VCC.

Propagation Delay Dispersion Propagation delay dispersion is defined as a variation in propagation  delay as a function of change in input conditions. In an automatic test system pin-driver elec- tronics, for example, the dispersion determines the maximum edge resolution.

Many factors can affect the dispersion, such as common- mode voltage, overdrive, input slew rate, duty cycle, and pulse width. The typical propagation delay dispersions of the MAX9600/MAX9601/MAX9602 are less than 10ps to

40ps (see the Typical Operating Characteristics and

Electrical Characteristics sections).

Comparators with Latch Enable

(MAX9600/MAX9601) The latch-enable function allows the comparator to be used in a sampling mode. When LE_ is low (LE_ is high), the comparator tracks the input signal. When LE_ is dri- ven high (LE_ is low), the outputs are forced to an unam- biguous logic state, dependent on the input conditions at the time of the latch input transition. If the latch-enable function is not used, connect the appropriate LE_ input to a low ECL/PECL logic, and its complementary LE_ input to a high ECL/PECL logic level (see Table 1).

The input range of the MAX9600 differential latch- enable inputs is 400mV to 2V. The logic-input  swing excursion must fall within an input-voltage range (VLR) of -2V to 0 to work properly.  The input range of the MAX9601 differential latch-enable inputs is 250mV to

3.5V. The logic-input swing excursion must fall within an input-voltage  range (VLR) of 0 to 3.5V for (VCCO_ <

3.5V) or VLR of (VCCO_ - 3.5V) to VCCO_ for (VCCO_ ≥

3.5V) to work properly.

| Table 1. Latch-Enable Truth Table LATCH-ENABLE INPUT |  | OPERATION |
| --- | --- | --- |
| LE_ | LE |  |
| 0 | 1 | Compare Mode. Output follows input state. |
| 1 | 0 | Latch Mode. Output latches to last known output state. |
| 0 | 0 | Invalid condition, output is in unknown state. |
| 1 | 1 |  |

Timing Information (MAX9600/MAX9601) The timing diagram (Figure 1) illustrates the operation of a comparator with latch enable. The top line of the diagram illustrates a latch-enable pulse. Initially, the latch-enable input (LE, LE_) is differentially high, which places the comparator in latch mode. When the input signal (IN_+, IN_-) switches from low to high, the output (Q_, Q_) remains latched to the previous low state. When the latch-enable  input goes differentially low, starting the compare function, the output responds to the input and transitions to high after a time (tLPD). The leading edges of the subsequent input signal switch the comparator after time interval tPD+ or tPD- (depend- ing on the direction of the input transitions) until a high latch-enable pulse places  the device  in latch mode again. The input signal must occur at minimum time (tLS) before the latch rising edge, and must maintain its state for at least tLH after the rising edge. A minimum latch-pulse width (tLPW) of 250ps (typ) is needed for proper latch operation.

ECL/PCL The MAX9600/MAX9601/MAX9602 outputs are emitter followers that require external resistive connections to a voltage source (VT) more negative than the lowest VOL for proper static and dynamic operation. When properly terminated, the outputs provide appropriate levels, VOL or V OH , for ECL (MAX9600) or PECL (MAX9601/ MAX9602). Output-current polarity always sinks into the termination scheme during proper operation.

ECL-output signal levels are referenced to GND, and

PECL-output signals are referenced to VCCO_.

Chip Information

MAX9600 TRANSISTOR COUNT: 558

MAX9601 TRANSISTOR COUNT: 600

MAX9602 TRANSISTOR COUNT: 608

PROCESS: Bipolar

Pin  Configurations

TOP VIEW

QA   1

QA   2

GND   3

LEA   4

LEA   5

VEE      6

VCC    7

HYSA   8

INA-    9

INA+  10

MAX9600

TSSOP-20

20    QB

19    QB

18   GND

17    LEB

16    LEB

15   VEE

14   VCC

13   HYSB

12   INB-

11   INB+

QA   1

QA   2

VCCOA    3

LEA   4

LEA   5

VEE      6

VCC    7

HYSA   8

INA-    9

INA+  10

MAX9601

TSSOP-20

20    QB

19    QB

18   VCCOB

17    LEB

16    LEB

15   VEE

14   VCC

13   HYSB

12   INB-

11   INB+

INA+    1

INA-    2

VEE      3

INB+    4

INB-    5

VCC    6

INC+    7

INC-    8

VEE      9

IND+  10

IND-   11

VCC   12

MAX9602

24   VCCOA

23    QA

22    QA

21   VCCOB

20    QB

19    QB

18   VCCOC

17    QC

16    QC

15   VCCOD

14    QD

13    QD

MAX9600/MAX9601/MAX9602

TSSOP-24

MAX9600/MAX9601/MAX9602

TSSOP4.40mm.EPS

| Package Information (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) |  |
| --- | --- |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

12    	Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 408-737-7600

©  2002 Maxim Integrated  Products 	Printed USA 	is a registered  trademark of Maxim Integrated  Products.