www.ti.com

FEATURES

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

DGG OR DL PACKAGE

Member of the Texas Instruments Widebus™

Family

(TOP VIEW)

Max tpd of 5.8 ns at 3.3 V

24-mA Output Drive at 3.3 V

1DIR   1

1B1    2

48    1OE

47    1A1

Control Inputs VIH

/VIL

Levels Are Referenced

1B2    3

GND   4

46    1A2

45    GND

to VCCA Voltage

Latch-Up Performance Exceeds 250 mA Per

JESD 17

NOTE: New and improved versions of the SN74ALVC164245 are available. The new part numbers are SN74LVC16T245 and SN74LVCH16T245 and should be considered for new designs.

DESCRIPTION/ORDERING INFORMATION

This 16-bit (dual-octal) noninverting bus transceiver contains two separate supply rails. B port has VCCB, which is set to operate at 3.3 V and 5 V. A port has VCCA, which is set to operate at 2.5 V and 3.3 V. This allows for translation from a 2.5-V to a 3.3-V environment, and vice versa, or from a 3.3-V to a 5-V environment, and vice versa.

The SN74ALVC164245 is designed for asynchronous communication between data buses. The control circuitry (1DIR, 2DIR, 1OE, and 2OE) is powered by VCCA.

To ensure the high-impedance state during power up or power down, the output-enable (OE) input should be tied to VCC through a pullup resistor; the minimum value    of    the    resistor    is    determined    by    the current-sinking capability of the driver.

1B3    5

1B4    6

(3.3 V, 5 V) VCCB     7

1B5    8

1B6    9

GND   10

1B7    11

1B8    12

2B1    13

2B2    14

GND   15

2B3    16

2B4    17

(3.3 V, 5 V) VCCB     18

2B5    19

2B6    20

GND   21

2B7    22

2B8    23

2DIR   24

44    1A3

43    1A4

42    VCCA (2.5 V, 3.3 V)

41    1A5

40    1A6

39    GND

38    1A7

37    1A8

36    2A1

35    2A2

34    GND

33    2A3

32    2A4

31    VCCA (2.5 V, 3.3 V)

30    2A5

29    2A6

28    GND

27    2A7

26    2A8

25    2OE

ORDERING INFORMATION

| TA | PACKAGE (1) |  | ORDERABLE PART NUMBER | TOP-SIDE MARKING |
| --- | --- | --- | --- | --- |
| –40 C to 85 C | FBGA – GRD | Tape and reel | 74ALVC164245GRDR | VC4245 |
|  | FBGA – ZRD (Pb-free) |  | 74ALVC164245ZRDR |  |
|  | SSOP – DL | Tube of 25 | SN74ALVC164245DL | ALVC164245 |
|  |  | Reel of 1000 | SN74ALVC164245DLR |  |
|  |  |  | 74ALVC164245DLRG4 |  |
|  | TSSOP – DGG | Reel of 2000 | SN74ALVC164245DGGR | ALVC164245 |
|  |  |  | 74ALVC164245DGGRG4 |  |
|  |  | Reel of 250 | SN74ALVC164245DGGT |  |
|  |  |  | 74ALVC164245DGGTE4 |  |
|  | VFBGA – GQL | Reel of 1000 | SN74ALVC164245KR | VC4245 |
|  | VFBGA – ZQL (Pb-free) |  | 74ALVC164245ZQLR |  |

(1)   Package drawings, standard packing quantities, thermal data, symbolization, and PCB design guidelines are available at www.ti.com/sc/package.

Please be aware that an important notice concerning availability, standard warranty, and use in critical applications of Texas

Instruments semiconductor products and disclaimers thereto appears at the end of this data sheet.

Widebus is a trademark of Texas Instruments.

PRODUCTION DATA information is current as of publication date.                                                                                        Copyright © 1994–2005, Texas Instruments Incorporated

Products  conform  to  specifications  per  the  terms  of  the  Texas

Instruments  standard  warranty.  Production  processing  does  not necessarily include testing of all parameters.

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

DESCRIPTION/ORDERING INFORMATION (CONTINUED)

www.ti.com

The logic levels of the direction-control (DIR) input and the output-enable (OE) input activate either the B-port outputs or the A-port outputs or place both output ports into the high-impedance mode. The device transmits data from the A bus to the B bus when the B-port outputs are activated, and from the B bus to the A bus when the A-port outputs are activated. The input circuitry on both A and B ports always is active and must have a logic HIGH or LOW level applied to prevent excess ICC and ICCZ.

GQL OR ZQL PACKAGE (TOP VIEW)

| TERMINAL ASSIGNMENTS (1) | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| A | 1DIR | NC | NC | NC | NC | 1OE |
| B | 1B2 | 1B1 | GND | GND | 1A1 | 1A2 |
| C | 1B4 | 1B3 | VCCB | VCCA | 1A3 | 1A4 |
| D | 1B6 | 1B5 | GND | GND | 1A5 | 1A6 |
| E | 1B8 | 1B7 |  |  | 1A7 | 1A8 |
| F | 2B1 | 2B2 |  |  | 2A2 | 2A1 |
| G | 2B3 | 2B4 | GND | GND | 2A4 | 2A3 |
| H | 2B5 | 2B6 | VCCB | VCCA | 2A6 | 2A5 |
| J | 2B7 | 2B8 | GND | GND | 2A8 | 2A7 |
| K | 2DIR | NC | NC | NC | NC | 2OE |

(56-Ball GQL/ZQL Package)

1  2  3  4  5  6

A B C D E F G H J

K

GRD OR ZRD PACKAGE (TOP VIEW)

1     2     3      4     5     6

| (1) NC – No internal connection | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| A | 1B1 | NC | 1DIR | 1OE | NC | 1A1 |
| B | 1B3 | 1B2 | NC | NC | 1A2 | 1A3 |
| C | 1B5 | 1B4 | VCCB | VCCA | 1A4 | 1A5 |
| D | 1B7 | 1B6 | GND | GND | 1A6 | 1A7 |
| E | 2B1 | 1B8 | GND | GND | 1A8 | 2A1 |
| F | 2B3 | 2B2 | GND | GND | 2A2 | 2A3 |
| G | 2B5 | 2B4 | VCCB | VCCA | 2A4 | 2A5 |
| H | 2B7 | 2B6 | NC | NC | 2A6 | 2A7 |
| J | 2B8 | NC | 2DIR | 2OE | NC | 2A8 |

TERMINAL ASSIGNMENTS (1)

(54-Ball GRD/ZRD Package)

A B C D

E

F G

H                                                                                              (1)   NC – No internal connection

J

FUNCTION TABLE (1)

(EACH 8-BIT SECTION)

| CONTROL INPUTS | OUTPUT CIRCUITS |  | OPERATION |
| --- | --- | --- | --- |
| OE DIR | A PORT | B PORT |  |
| L L L H H X | Enabled Hi-Z Hi-Z | Hi-Z Enabled Hi-Z | B data to A bus A data to B bus Isolation |

(1)   Input circuits of the data I/Os always are active.

2

www.ti.com

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

LOGIC DIAGRAM (POSITIVE LOGIC)

1DIR    1

24

2DIR

48

1OE

25

2OE

1A1    47

36

2A1

2   1B1

13   2B1

To Seven Other Channels

To Seven Other Channels

Absolute Maximum Ratings (1)

over operating free-air temperature range for VCCB at 5 V and VCCA at 3.3 V (unless otherwise noted)

|  |  | MIN MAX | UNIT |
| --- | --- | --- | --- |
| VCC Supply voltage range | VCCA | –0.5 4.6 | V |
|  | VCCB | –0.5 6 |  |
| VI Input voltage range | Except I/O ports (2) | –0.5 6 | V |
|  | I/O port A (3) | –0.5 VCCA + 0.5 |  |
|  | I/O port B (2) | –0.5 VCCB + 0.5 |  |
| IIK Input clamp current | VI < 0 | –50 | mA |
| IOK Output clamp current | VO < 0 | –50 | mA |
| IO Continuous output current |  | 50 | mA |
| Continuous current through each VCC or GND |  | 100 | mA |
| JA Package thermal impedance (4) | DGG package | 70 | C/W |
|  | DL package | 63 |  |
|  | GQL/ZQL package | 42 |  |
|  | GRD/ZRD package | 36 |  |
| Tstg Storage temperature range |  | –65 150 | C |

(1)   Stresses beyond those listed under "absolute maximum ratings" may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated under "recommended operating conditions" is not implied. Exposure to absolute-maximum-rated conditions for extended periods may affect device reliability.

(2)   This value is limited to 6 V maximum. (3)   This value is limited to 4.6 V maximum.

(4)   The package thermal impedance is calculated in accordance with JESD 51-7.

3

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

Recommended Operating Conditions (1)

for VCCB at 3.3 V and 5 V

www.ti.com

|  |  | MIN MAX | UNIT |
| --- | --- | --- | --- |
| VCCB Supply voltage |  | 3 5.5 | V |
| VIH High-level input voltage |  | 2 | V |
| VIL Low-level input voltage | VCCB = 3 V to 3.6 V | 0.7 | V |
|  | VCCB = 4.5 V to 5.5 V | 0.8 |  |
| VIB Input voltage |  | 0 VCCB | V |
| VOB Output voltage |  | 0 VCCB | V |
| IOH High-level output current |  | –24 | mA |
| IOL Low-level output current |  | 24 | mA |
| t/ v Input transition rise or fall rate |  | 10 | ns/V |
| TA Operating free-air temperature |  | –40 85 | C |

(1)   All unused inputs of the device must be held at VCC or GND to ensure proper device operation. Refer to the TI application report,

Implications of Slow or Floating CMOS Inputs, literature number SCBA004.

Recommended Operating Conditions (1)

for VCCA at 2.5 V and 3.3 V

|  |  | MIN MAX | UNIT |
| --- | --- | --- | --- |
| VCCA Supply voltage |  | 2.3 3.6 | V |
| VIH High-level input voltage | VCCA = 2.3 V to 2.7 V | 1.7 | V |
|  | VCCA = 3 V to 3.6 V | 2 |  |
| VIL Low-level input voltage | VCCA = 2.3 V to 2.7 V | 0.7 | V |
|  | VCCA = 3 V to 3.6 V | 0.8 |  |
| VIA Input voltage |  | 0 VCCA | V |
| VOA Output voltage |  | 0 VCCA | V |
| IOH High-level output current | VCCA = 2.3 V | –18 | mA |
|  | VCCA = 3 V | –24 |  |
| IOL Low-level output current | VCCA = 2.3 V | 18 | mA |
|  | VCCA = 3 V | 24 |  |
| t/ v Input transition rise or fall rate |  | 10 | ns/V |
| TA Operating free-air temperature |  | –40 85 | C |

(1)   All unused inputs of the device must be held at VCC or GND to ensure proper device operation. Refer to the TI application report,

Implications of Slow or Floating CMOS Inputs, literature number SCBA004.

4

www.ti.com

Electrical Characteristics

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

over recommended operating free-air temperature range for VCCA = 2.7 V to 3.6 V and VCCB = 4.5 V to 5.5 V (unless otherwise noted)

| PARAMETER |  | TEST CONDITIONS | VCCA | VCCB | MIN TYP (1) MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VOH | B to A | IOH = –100 A | 2.7 V to 3.6 V |  | VCC – 0.2 | V |
|  |  | IOH = –12 mA | 2.7 V |  | 2.2 |  |
|  |  |  | 3 V |  | 2.4 |  |
|  |  | IOH = –24 mA | 3 V |  | 2 |  |
|  | A to B | IOH = –100 A |  | 4.5 V | 4.3 |  |
|  |  |  |  | 5.5 V | 5.3 |  |
|  |  | IOH = –24 mA |  | 4.5 V | 3.7 |  |
|  |  |  |  | 5.5 V | 4.7 |  |
| VOL | B to A | IOL = 100 A | 2.7 V to 3.6 V |  | 0.2 | V |
|  |  | IOL = 12 mA | 2.7 V |  | 0.4 |  |
|  |  | IOL = 24 mA | 3 V |  | 0.55 |  |
|  | A to B | IOL = 100 A |  | 4.5 V to 5.5 V | 0.2 |  |
|  |  | IOL = 24 mA |  | 4.5 V to 5.5 V | 0.55 |  |
| II | Control inputs | VI = VCCA/VCCB or GND | 3.6 V | 5.5 V | 5 | A |
| IOZ (2) | A or B port | VO = VCCA/VCCB or GND | 3.6 V | 5.5 V | 10 | A |
| ICC |  | VI = VCCA/VCCB or GND, IO = 0 | 3.6 V | 5.5 V | 40 | A |
| ICC (3) |  | One input at VCCA/VCCB – 0.6 V, Other inputs at VCCA/VCCB or GND | 3 V to 3.6 V | 4.5 V to 5.5 V | 750 | A |
| Ci | Control inputs | VI = VCCA/VCCB or GND | 3.3 V | 5 V | 6.5 | pF |
| Cio | A or B port | VO = VCCA/VCCB or GND | 3.3 V | 3.3 V | 8.5 | pF |

(1)   Typical values are measured at VCCA = 3.3 V and VCCB = 5 V, TA = 25 C. (2)   For I/O ports, the parameter IOZ includes the input leakage current.

(3)   This is the increase in supply current for each input that is at one of the specified TTL voltage levels, rather than at 0 or the associated

VCC.

Electrical Characteristics

over recommended operating free-air temperature range for VCCA = 2.3 V to 2.7 V and VCCB = 3 V to 3.6 V (unless otherwise noted)

| PARAMETER |  | TEST CONDITIONS | VCCA | VCCB | MIN MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VOH | B to A | IOH = –100 A | 2.3 V to 2.7 V | 3 V to 3.6 V | VCCA – 0.2 | V |
|  |  | IOH = –8 mA | 2.3 V | 3 V to 3.6 V | 1.7 |  |
|  |  | IOH = –12 mA | 2.7 V | 3 V to 3.6 V | 1.8 |  |
|  | A to B | IOH = –100 A | 2.3 V to 2.7 V | 3 V to 3.6 V | VCCB – 0.2 |  |
|  |  | IOH = –18 mA | 2.3 V to 2.7 V | 3 V | 2.2 |  |
| VOL | B to A | IOL = 100 A | 2.3 V to 2.7 V | 3 V to 3.6 V | 0.2 | V |
|  |  | IOL = 12 mA | 2.3 V | 3 V to 3.6 V | 0.6 |  |
|  | A to B | IOL = 100 A | 2.3 V to 2.7 V | 3 V to 3.6 V | 0.2 |  |
|  |  | IOL = 18 mA | 2.3 V | 3 V | 0.55 |  |
| II | Control inputs | VI = VCCA/VCCB or GND | 2.3 V to 2.7 V | 3 V to 3.6 V | 5 | A |
| IOZ (1) | A or B port | VO = VCCA/VCCB or GND | 2.3 V to 2.7 V | 3 V to 3.6 V | 10 | A |
| ICC |  | VI = VCCA/VCCB or GND, IO = 0 | 2.3 V to 2.7 V | 3 V to 3.6 V | 20 | A |
| ICC (2) |  | One input at VCCA/VCCB – 0.6 V, Other inputs at VCCA/VCCB or GND | 2.3 V to 2.7 V | 3 V to 3.6 V | 750 | A |

(1)   For I/O ports, the parameter IOZ includes the input leakage current.

(2)   This is the increase in supply current for each input that is at one of the specified TTL voltage levels, rather than at 0 or the associated

VCC.

5

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

Switching Characteristics

www.ti.com

over recommended operating free-air temperature range (unless otherwise noted) (see Figure 1 through Figure 4)

| PARAMETER | FROM (INPUT) | TO (OUTPUT) | VCCB = 3.3 V 0.3 V | VCCB = 5 V 0.5 V |  | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCCA = 2.5 V 0.2 V | VCCA = 2.7 V | VCCA = 3.3 V 0.3 V |  |
|  |  |  | MIN MAX | MIN MAX | MIN MAX |  |
| tpd | A | B | 7.6 | 5.9 | 1 5.8 | ns |
|  | B | A | 7.6 | 6.7 | 1.2 5.8 |  |
| ten | OE | B | 11.5 | 9.3 | 1 8.9 | ns |
| tdis | OE | B | 10.5 | 9.2 | 2.1 9.5 | ns |
| ten | OE | A | 12.3 | 10.2 | 2 9.1 | ns |
| tdis | OE | A | 9.3 | 9 | 2.9 8.6 | ns |

Operating Characteristics

TA = 25 C

| PARAMETER |  | TEST CONDITIONS | VCCB = 3.3 V | VCCB = 5 V | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VCCA = 2.5 V | VCCA = 3.3 V |  |
|  |  |  | TYP | TYP |  |
| Cpd Power dissipation capacitance | Outputs enabled (B) | CL = 50 pF, f = 10 MHz | 55 | 56 | pF |
|  | Outputs disabled (B) |  | 27 | 6 |  |
|  | Outputs enabled (A) | CL = 50 pF, f = 10 MHz | 118 | 56 |  |
|  | Outputs disabled (A) |  | 58 | 6 |  |

6

www.ti.com

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

POWER-UP CONSIDERATIONS(1)

TI level-translation devices offer an opportunity for successful mixed-voltage signal design. A proper power-up sequence always should be followed to avoid excessive supply current, bus contention, oscillations, or other anomalies caused by improperly biased device pins. Take these precautions to guard against such power-up problems:

1.  Connect ground before any supply voltage is applied.

2.  Power up the control side of the device (VCCA for all four of these devices).

3.  Tie OE to VCCA with a pullup resistor so that it ramps with VCCA.

4.  Depending on the direction of the data path, DIR can be high or low. If DIR high is needed (A data to B bus), ramp it with VCCA. Otherwise, keep DIR low.

(1)   Refer to the TI application report, Texas Instruments Voltage-Level-Translation Devices, literature number SCEA021.

7

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

PARAMETER MEASUREMENT INFORMATION VCCA = 2.5 V    0.2 V to VCCB = 3.3 V    0.3 V

www.ti.com

From Output

Under Test

CL = 30 pF (see Note A)

500

500

| VCCB = 6 V S1 Open TEST | S1 |
| --- | --- |
| tpd tPLZ/tPZL tPHZ/tPZH | Open VCCB = 6 V GND |

GND

LOAD CIRCUIT

Output Control (low-level enabling)

VCCA/2

VCCA/2

VCCA

0 V

Input

VCCA/2                          VCCA/2

tPLH                                                                               tPHL

VCCA

0 V

Output

Waveform 1

S1 at 6 V (see Note B)

Output

tPZL

tPZH

tPLZ

1.5 V                 VOL + 0.3 V

tPHZ

VCCB

VOLB

Output

1.5 V                          1.5 V VOLTAGE WAVEFORMS

VOHB

VOLB

Waveform 2

S1 at GND (see Note B)

1.5 V

VOLTAGE WAVEFORMS

VOH

VOHB

− 0.3 V

0 V

PROPAGATION DELAY TIMES

ENABLE AND DISABLE TIMES

NOTES:  A.  CL includes probe and jig capacitance.

B.  Waveform 1 is for an output with internal conditions such that the output is low, except when disabled by the output control.

Waveform 2 is for an output with internal conditions such that the output is high, except when disabled by the output control. C.  All input pulses are supplied by generators having the following characteristics: PRR 10 MHz, ZO = 50   , tr 2 ns, tf 2 ns. D.  The outputs are measured one at a time, with one transition per measurement.

E.  tPLZ and tPHZ are the same as tdis. F.  tPZL and tPZH are the same as ten. G.  tPLH and tPHL are the same as tpd.

Figure 1. Load Circuit and Voltage Waveforms

8

www.ti.com

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

PARAMETER MEASUREMENT INFORMATION VCCB = 3.3 V    0.3 V to VCCA = 2.5 V    0.2 V

From Output

Under Test

CL = 30 pF (see Note A)

500

500

2  VCCA

S1            Open

GND

LOAD CIRCUIT

2.7 V

Output Control (low-level enabling)

Output

Waveform 1

tPZL

1.5 V

| 1.5 V TEST | S1 |
| --- | --- |
| tpd tPLZ/tPZL tPHZ/tPZH | Open 2  VCCA GND |

tPLZ

2.7 V

0 V

VCCA

Input

tPLH

1.5 V                             1.5 V

tPHL

0 V

VOHA

S1 at 2  VCCA

(see Note B)

Output

tPZH

VCCA/2

VOL + 0.15 V VOLA

tPHZ

VOHA

Output

V      /2                       V      /2

Waveform 2

VCCA/2

VOH − 0.15 V

CCA

VOLTAGE WAVEFORMS PROPAGATION DELAY TIMES

CCA

VOLA

S1 at GND

(see Note B)

0 V VOLTAGE WAVEFORMS

ENABLE AND DISABLE TIMES

NOTES:  A.  CL includes probe and jig capacitance.

B.  Waveform 1 is for an output with internal conditions such that the output is low, except when disabled by the output control.

Waveform 2 is for an output with internal conditions such that the output is high, except when disabled by the output control. C.  All input pulses are supplied by generators having the following characteristics: PRR  10 MHz, ZO = 50   , tr  2 ns, tf  2 ns. D.  The outputs are measured one at a time, with one transition per measurement.

E.  tPLZ and tPHZ are the same as tdis. F.  tPZL and tPZH are the same as ten. G.  tPLH and tPHL are the same as tpd.

Figure 2. Load Circuit and Voltage Waveforms

9

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

PARAMETER MEASUREMENT INFORMATION VCCA = 3.3 V    0.3 V to VCCB = 5 V    0.5 V

2     VCCB

www.ti.com

From Output

Under Test

CL = 50 pF (see Note A)

500

500

S1            Open

GND

LOAD CIRCUIT

Output Control (low-level enabling)

1.5 V

1.5 V

| 2.7 V TEST | S1 |
| --- | --- |
| tpd tPLZ/tPZL tPHZ/tPZH | Open 2 VCCB GND |

0 V

Input

tPLH

1.5 V                            1.5 V

tPHL

2.7 V

0 V

VOH

Output

Waveform 1

S1 at 2     VCCB

(see Note B)

Output

Waveform 2

tPZL

tPZH

50% VCCB

50% V

tPLZ

20% VCCB

tPHZ

80% VCCB

VCCB

VOL

VOH

Output

50% VCCB                     50% VCCB

VOL

VOLTAGE WAVEFORMS PROPAGATION DELAY TIMES

S1 at GND (see Note B)

CCB

0 V

VOLTAGE WAVEFORMS ENABLE AND DISABLE TIMES

NOTES:  A.  CL includes probe and jig capacitance.

B.  Waveform 1 is for an output with internal conditions such that the output is low, except when disabled by the output control.

Waveform 2 is for an output with internal conditions such that the output is high, except when disabled by the output control.

C.  All input pulses are supplied by generators having the following characteristics: PRR  10 MHz, ZO = 50   , tr  2.5 ns, tf  2.5 ns. D.  The outputs are measured one at a time, with one transition per measurement.

E.  tPLZ and tPHZ are the same as tdis. F.  tPZL and tPZH are the same as ten. G.  tPLH and tPHL are the same as tpd.

Figure 3. Load Circuit and Voltage Waveforms

10

www.ti.com

SN74ALVC164245

16-BIT 2.5-V TO 3.3-V/3.3-V TO 5-V LEVEL-SHIFTING TRANSCEIVER

WITH 3-STATE OUTPUTS

SCAS416P – MARCH 1994 – REVISED NOVEMBER 2005

PARAMETER MEASUREMENT INFORMATION VCCB = 5 V    0.5 V to VCCA = 2.7 V and 3.3 V    0.3 V

From Output

Under Test

CL = 50 pF (see Note A)

500

500

VCCA = 6 V S1            Open

GND

LOAD CIRCUIT

| Output Control (low-level enabling) TEST | S1 |
| --- | --- |
| tpd tPLZ/tPZL tPHZ/tPZH | Open VCCA = 6 V GND |

Output

3 V                     Waveform 1

tPZL

1.5 V

1.5 V

tPLZ

3 V

0 V

3 V

Input

Output

1.5 V                             1.5 V

tPLH                                                                             tPHL

1.5 V                          1.5 V

0 V

VOHA

VOLA

S1 at 6 V (see Note B)

Output

Waveform 2

S1 at GND (see Note B)

tPZH

1.5 V                  VOL + 0.3 V

tPHZ

VOH − 0.3 V

1.5 V

VOLA

VOHA

0 V

VOLTAGE WAVEFORMS PROPAGATION DELAY TIMES

VOLTAGE WAVEFORMS ENABLE AND DISABLE TIMES

NOTES:  A.  CL includes probe and jig capacitance.

B.  Waveform 1 is for an output with internal conditions such that the output is low, except when disabled by the output control.

Waveform 2 is for an output with internal conditions such that the output is high, except when disabled by the output control.

C.  All input pulses are supplied by generators having the following characteristics: PRR  10 MHz, ZO = 50   , tr  2.5 ns, tf  2.5 ns. D.  The outputs are measured one at a time, with one transition per measurement.

E.  tPLZ and tPHZ are the same as tdis. F.  tPZL and tPZH are the same as ten. G.  tPLH and tPHL are the same as tpd.

Figure 4. Load Circuit and Voltage Waveforms

11

PACKAGE OPTION ADDENDUM

www.ti.com                                                                                                                                                                                                                18-Sep-2008

PACKAGING INFORMATION

Orderable Device Status (1) Package Package Pins Package Eco Plan (2) Lead/Ball Finish MSL Peak Temp (3) Type Drawing Qty

74ALVC164245DGGRE4 ACTIVE TSSOP DGG 48 2000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

74ALVC164245DGGRG4 ACTIVE TSSOP DGG 48 2000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

74ALVC164245DGGTE4 ACTIVE TSSOP DGG 48 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

74ALVC164245DGGTG4 ACTIVE TSSOP DGG 48 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

74ALVC164245DLG4 ACTIVE SSOP DL 48 25 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

74ALVC164245DLRG4 ACTIVE SSOP DL 48 1000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

74ALVC164245GRDR ACTIVE BGA MI GRD 54 1000 TBD SNPB Level-1-240C-UNLIM CROSTA R JUNI OR

74ALVC164245ZQLR ACTIVE BGA MI ZQL 56 1000 Green (RoHS & SNAGCU Level-1-260C-UNLIM CROSTA no Sb/Br) R JUNI OR

74ALVC164245ZRDR ACTIVE BGA MI ZRD 54 1000 Green (RoHS & SNAGCU Level-1-260C-UNLIM CROSTA no Sb/Br) R JUNI OR

SN74ALVC164245DGGR ACTIVE TSSOP DGG 48 2000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

SN74ALVC164245DGGT ACTIVE TSSOP DGG 48 250 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

SN74ALVC164245DL ACTIVE SSOP DL 48 25 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

SN74ALVC164245DLR ACTIVE SSOP DL 48 1000 Green (RoHS & CU NIPDAU Level-1-260C-UNLIM no Sb/Br)

SN74ALVC164245KR ACTIVE BGA MI GQL 56 1000 TBD SNPB Level-1-240C-UNLIM CROSTA R JUNI OR

(1) The marketing status values are defined as follows:

ACTIVE: Product device recommended for new designs.

LIFEBUY: TI has announced that the device will be discontinued, and a lifetime-buy period is in effect.

NRND: Not recommended for new designs. Device is in production to support existing customers, but TI does not recommend using this part in a new design.

PREVIEW: Device has been announced but is not in production. Samples may or may not be available.

OBSOLETE: TI has discontinued the production of the device.

(2) Eco Plan - The planned eco-friendly classification: Pb-Free (RoHS), Pb-Free (RoHS Exempt), or Green (RoHS & no Sb/Br) - please check http://www.ti.com/productcontent for the latest availability information and additional product content details.

TBD: The Pb-Free/Green conversion plan has not been defined.

Pb-Free (RoHS): TI's terms "Lead-Free" or "Pb-Free" mean semiconductor products that are compatible with the current RoHS requirements for all 6 substances, including the requirement that lead not exceed 0.1% by weight in homogeneous materials. Where designed to be soldered at high temperatures, TI Pb-Free products are suitable for use in specified lead-free processes.

Pb-Free (RoHS Exempt): This component has a RoHS exemption for either 1) lead-based flip-chip solder bumps used between the die and package, or 2) lead-based die adhesive used between the die and leadframe. The component is otherwise considered Pb-Free (RoHS compatible) as defined above.

Addendum-Page 1

PACKAGE OPTION ADDENDUM

www.ti.com                                                                                                                                                                                                                18-Sep-2008

Green (RoHS & no Sb/Br): TI defines "Green" to mean Pb-Free (RoHS compatible), and free of Bromine (Br) and Antimony (Sb) based flame retardants (Br or Sb do not exceed 0.1% by weight in homogeneous material)

(3)  MSL, Peak Temp. -- The Moisture Sensitivity Level rating according to the JEDEC industry standard classifications, and peak solder temperature.

Important Information and Disclaimer:The information provided on this page represents TI's knowledge and belief as of the date that it is provided. TI bases its knowledge and belief on information provided by third parties, and makes no representation or warranty as to the accuracy of such information. Efforts are underway to better integrate information from third parties. TI has taken and continues to take reasonable steps to provide representative and accurate information but may not have conducted destructive testing or chemical analysis on incoming materials and chemicals. TI and TI suppliers consider certain information to be proprietary, and thus CAS numbers and other limited information may not be available for release.

In no event shall TI's liability arising out of such information exceed the total purchase price of the TI part(s) at issue in this document sold by TI

to Customer on an annual basis.

OTHER QUALIFIED VERSIONS OF SN74ALVC164245 :

• Enhanced Product: SN74ALVC164245-EP

NOTE: Qualified Version Definitions:

• Enhanced Product - Supports Defense, Aerospace and Medical Applications

Addendum-Page 2

PACKAGE MATERIALS INFORMATION

www.ti.com                                                                                                                                                                                                                                 8-May-2009

TAPE AND REEL INFORMATION

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Reel Diameter (mm) | Reel Width W1 (mm) | A0 (mm) | B0 (mm) | K0 (mm) | P1 (mm) | W (mm) | Pin1 Quadrant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 74ALVC164245GRDR C | BGA MI ROSTA R JUNI OR | GRD | 54 | 1000 | 330.0 | 16.4 | 5.8 | 8.3 | 1.55 | 8.0 | 16.0 | Q1 |
| 74ALVC164245ZQLR C | BGA MI ROSTA R JUNI OR | ZQL | 56 | 1000 | 330.0 | 16.4 | 4.8 | 7.3 | 1.45 | 8.0 | 16.0 | Q1 |
| 74ALVC164245ZRDR C | BGA MI ROSTA R JUNI OR | ZRD | 54 | 1000 | 330.0 | 16.4 | 5.8 | 8.3 | 1.55 | 8.0 | 16.0 | Q1 |
| SN74ALVC164245DGGR | TSSOP | DGG | 48 | 2000 | 330.0 | 24.4 | 8.6 | 15.8 | 1.8 | 12.0 | 24.0 | Q1 |
| SN74ALVC164245DGGT | TSSOP | DGG | 48 | 250 | 330.0 | 24.4 | 8.6 | 15.8 | 1.8 | 12.0 | 24.0 | Q1 |
| SN74ALVC164245DLR | SSOP | DL | 48 | 1000 | 330.0 | 32.4 | 11.35 | 16.2 | 3.1 | 16.0 | 32.0 | Q1 |
| SN74ALVC164245KR C | BGA MI ROSTA R JUNI OR | GQL | 56 | 1000 | 330.0 | 16.4 | 4.8 | 7.3 | 1.45 | 8.0 | 16.0 | Q1 |

Pack Materials-Page 1

PACKAGE MATERIALS INFORMATION

www.ti.com                                                                                                                                                                                                                                 8-May-2009

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Length (mm) | Width (mm) | Height (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 74ALVC164245GRDR | BGA MICROSTAR JUNIOR | GRD | 54 | 1000 | 346.0 | 346.0 | 33.0 |
| 74ALVC164245ZQLR | BGA MICROSTAR JUNIOR | ZQL | 56 | 1000 | 346.0 | 346.0 | 33.0 |
| 74ALVC164245ZRDR | BGA MICROSTAR JUNIOR | ZRD | 54 | 1000 | 346.0 | 346.0 | 33.0 |
| SN74ALVC164245DGGR | TSSOP | DGG | 48 | 2000 | 346.0 | 346.0 | 41.0 |
| SN74ALVC164245DGGT | TSSOP | DGG | 48 | 250 | 346.0 | 346.0 | 41.0 |
| SN74ALVC164245DLR | SSOP | DL | 48 | 1000 | 346.0 | 346.0 | 49.0 |
| SN74ALVC164245KR | BGA MICROSTAR JUNIOR | GQL | 56 | 1000 | 346.0 | 346.0 | 33.0 |

Pack Materials-Page 2

MECHANICAL DATA

MTSS003D – JANUARY 1995 – REVISED JANUARY 1998

DGG (R-PDSO-G**)                                                                                    PLASTIC SMALL-OUTLINE PACKAGE

48 PINS SHOWN

0,50

48

0,17

0,08  M

25

6,20      8,30

6,00

7,90

0,15 NOM

Gage Plane

1                                                                                    24

A

0– 8

0,25

0,75

0,50

1,20 MAX

0,15

0,05

Seating Plane

0,10

| PINS ** DIM | 48 | 56 | 64 |
| --- | --- | --- | --- |
| A MAX | 12,60 | 14,10 | 17,10 |
| A MIN | 12,40 | 13,90 | 16,90 |

4040078 / F 12/97

NOTES:  A.  All linear dimensions are in millimeters.

B.  This drawing is subject to change without notice.

C.  Body dimensions do not include mold protrusion not to exceed 0,15. D.  Falls within JEDEC MO-153

POST OFFICE BOX 655303  DALLAS, TEXAS 75265

MECHANICALDATA

MSSO001C – JANUARY 1995 – REVISED DECEMBER 2001

DL (R-PDSO-G**)                                                                                       PLASTIC SMALL-OUTLINE PACKAGE

48 PINS SHOWN

0.025 (0,635)

| 0.0135 (0,343) |  | 0.005 (0,13) M |
| --- | --- | --- |

0.008 (0,203)

48                                                                       25

0.010 (0,25)

0.005 (0,13)

0.299 (7,59)

0.291 (7,39)

0.420 (10,67)

0.395 (10,03)

Gage Plane

1                                                                      24

A

0– 8

0.010 (0,25)

0.040 (1,02)

0.020 (0,51)

0.110 (2,79) MAX

0.008 (0,20) MIN

Seating Plane

0.004 (0,10)

| PINS ** DIM | 28 | 48 | 56 |
| --- | --- | --- | --- |
| A MAX | 0.380 (9,65) | 0.630 (16,00) | 0.730 (18,54) |
| A MIN | 0.370 (9,40) | 0.620 (15,75) | 0.720 (18,29) |

4040048 / E 12/01

NOTES:  A.  All linear dimensions are in inches (millimeters).

B.  This drawing is subject to change without notice.

C.  Body dimensions do not include mold flash or protrusion not to exceed 0.006 (0,15). D.  Falls within JEDEC MO-118

POST OFFICE BOX 655303  DALLAS, TEXAS 75265

IMPORTANT NOTICE

Texas Instruments Incorporated and its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products and services at any time and to discontinue any product or service without notice. Customers should obtain the latest relevant information before placing orders and should verify that such information is current and complete. All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment.

TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI components. To minimize the risks associated with customer products and applications, customers should provide adequate design and operating safeguards.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products or services are used. Information published by TI regarding third-party products or services does not constitute a license from TI to use such products or services or a warranty or endorsement thereof. Use of such information may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

Reproduction of TI information in TI data books or data sheets is permissible only if reproduction is without alteration and is accompanied by all associated warranties, conditions, limitations, and notices. Reproduction of this information with alteration is an unfair and deceptive business practice. TI is not responsible or liable for such altered documentation. Information of third parties may be subject to additional restrictions.

Resale of TI products or services with statements different from or beyond the parameters stated by TI for that product or service voids all express and any implied warranties for the associated TI product or service and is an unfair and deceptive business practice. TI is not responsible or liable for any such statements.

TI products are not authorized for use in safety-critical applications (such as life support) where a failure of the TI product would reasonably be expected to cause severe personal injury or death, unless officers of the parties have executed an agreement specifically governing

such use. Buyers represent that they have all necessary expertise in the safety and regulatory ramifications of their applications, and acknowledge and agree that they are solely responsible for all legal, regulatory and safety-related requirements concerning their products and any use of TI products in such safety-critical applications, notwithstanding any applications-related information or support that may be provided by TI. Further, Buyers must fully indemnify TI and its representatives against any damages arising out of the use of TI products in such safety-critical applications.

TI products are neither designed nor intended for use in military/aerospace applications or environments unless the TI products are specifically designated by TI as military-grade or "enhanced plastic." Only products designated by TI as military-grade meet military specifications. Buyers acknowledge and agree that any such use of TI products which TI has not designated as military-grade is solely at the Buyer's risk, and that they are solely responsible for compliance with all legal and regulatory requirements in connection with such use.

TI products are neither designed nor intended for use in automotive applications or environments unless the specific TI products are designated by TI as compliant with ISO/TS 16949 requirements. Buyers acknowledge and agree that, if they use any non-designated products in automotive applications, TI will not be responsible for any failure to meet such requirements.

Following are URLs where you can obtain information on other Texas Instruments products and application solutions:

| Products |  | Applications |  |
| --- | --- | --- | --- |
| Amplifiers | amplifier.ti.com | Audio | www.ti.com/audio |
| Data Converters | dataconverter.ti.com | Automotive | www.ti.com/automotive |
| DLP® Products | www.dlp.com | Broadband | www.ti.com/broadband |
| DSP | dsp.ti.com | Digital Control | www.ti.com/digitalcontrol |
| Clocks and Timers | www.ti.com/clocks | Medical | www.ti.com/medical |
| Interface | interface.ti.com | Military | www.ti.com/military |
| Logic | logic.ti.com | Optical Networking | www.ti.com/opticalnetwork |
| Power Mgmt | power.ti.com | Security | www.ti.com/security |
| Microcontrollers | microcontroller.ti.com | Telephony | www.ti.com/telephony |
| RFID | www.ti-rfid.com | Video & Imaging | www.ti.com/video |
| RF/IF and ZigBee® Solutions | www.ti.com/lprf | Wireless | www.ti.com/wireless |

Mailing Address: Texas Instruments, Post Office Box 655303, Dallas, Texas 75265

Copyright © 2009, Texas Instruments Incorporated