19-1089; Rev 3; 3/03

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

General Description

The MAX4130–MAX4134 family of operational amplifiers combines 10MHz gain-bandwidth product and excellent DC accuracy  with Rail-to-Rail® operation at the inputs and outputs. These devices require only 900µA per amplifier, and operate from either a single supply (+2.7V to +6.5V) or dual supplies (±1.35V to ±3.25V) with a common-mode voltage range that extends 250mV beyond VEE and VCC. They are capable of driving 250Ω

loads and are unity-gain stable. In addition,  the MAX4131/ MAX4133 feature a shutdown mode in which the outputs are placed  in a high-impedance  state and the supply current is reduced to only 25µA per amplifier.

With their rail-to-rail input common-mode range and output swing, the MAX4130–MAX4134 are ideal for low- voltage, single-supply operation. Although the minimum operating voltage is specified  at 2.7V, the devices typically operate down to 1.8V. In addition, low offset voltage and high speed make them the ideal signal- conditioning  stages for precision, low-voltage data- acquisition systems. The MAX4130 is offered in the space-saving  5-pin SOT23 package.  The MAX4131 is offered in the ultra-small 6-bump, 1mm x 1.5mm chip- scale package (UCSP™).

| Selector Guide PART | AMPS PER PACKAGE | SHUTDOWN MODE | PIN- PACKAGE |
| --- | --- | --- | --- |
| MAX4130 | 1 | — | 5 SOT23 |
| MAX4131 | 1 | Yes | 6 UCSP, 8 µMAX/SO |
| MAX4132 | 2 | — | 8 µMAX/SO |
| MAX4133 | 2 | Yes | 14 SO |
| MAX4134 | 4 | — | 14 SO |

MAX4130–MAX4134

Features

♦ 6-Bump UCSP (MAX4131)

♦ +2.7V to +6.5V Single-Supply Operation

♦ Rail-to-Rail Input Common-Mode Voltage Range

♦ Rail-to-Rail Output Voltage Swing

♦ 10MHz Gain-Bandwidth Product

♦ 900µA Quiescent Current per Amplifier

♦ 25µA Shutdown Function (MAX4131/MAX4133)

♦ 200µV Offset Voltage

♦ No Phase Reversal for Overdriven Inputs

♦ Drive 250Ω Loads

♦ Stable with 160pF Capacitive Loads

♦ Unity-Gain Stable

Ordering Information

PIN- TOP PART TEMP RANGE PACKAGE MARK

MAX4130EUK-T -40°C to +85°C 5 SOT23-5 AABB

MAX4131C/D 0°C to +70°C Dice* —

MAX4131EBT-T -40°C to +85°C 6 UCSP-6 AAX

MAX4131EUA -40°C to +85°C 8 µMAX —

MAX4131ESA -40°C to +85°C 8 SO —

*Dice are specified  at TA = +25°C. DC parameters only.

Ordering Information continued at end of data sheet.

Typical Operating Circuit

Applications

Battery-Powered Instruments

MAX147

3                                         1

+3V

Portable Equipment

Data-Acquisition Systems

Signal Conditioning

Low-Power, Low-Voltage Applications

Pin Configurations appear at end of data sheet.

Rail-to-Rail is a registered trademark of Nippon Motorola, Ltd. UCSP is a trademark of Maxim Integrated Products, Inc.

SERIAL INTERFACE

SHDN

6    DOUT

8    SCLK

7    CS

VDD

AIN    2

VREF   4

GND    5

MAX4131

Maxim Integrated Products    1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

ABSOLUTE MAXIMUM RATINGS

Supply Voltage (VCC - VEE) ...................................................7.5V IN+, IN-, SHDN Voltage ...................(VCC  + 0.3V) to (VEE - 0.3V) Output Short-Circuit Duration (Note 1) .......................Continuous (short to either supply)

Continuous Power Dissipation (TA = +70°C)

5-Pin SOT23 (derate 7.1mW/°C above +70°C)............571mW

6-Bump UCSP (derate 2.9mW/°C above +70°C).........308mW

8-Pin SO (derate 5.88mW/°C above +70°C)................471mW

8-Pin µMAX (derate 4.10mW/°C above +70°C) ...........330mW

14-Pin SO (derate 8.00mW/°C above +70°C)..............640mW Operating Temperature Range

MAX413_E_ _ ...................................................-40°C to +85°C Maximum Junction Temperature .....................................+150°C Storage Temperature Range .............................-65°C to +160°C Lead Temperature (soldering,  10s) .................................+300°C Bump Reflow Temperature  .........................................+235°C

Note 1:   Provided that the maximum package  power-dissipation rating is not exceeded.

Stresses beyond those listed under “Absolute Maximum Ratings” may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

DC ELECTRICAL CHARACTERISTICS

(VCC =  +2.7V  to +6.5V,  VEE =  0V, VCM =  0V, VOUT =  VCC/2, RL tied  to VCC/2, SHDN ≥ 2V (or open),  TA  = +25°C,  unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Offset Voltage | VOS | VCM = VEE to VCC | MAX4130EUK | ±0.35 ±1.50 | mV |
|  |  |  | MAX4131ESA | ±0.20 ±0.60 |  |
|  |  |  | MAX4131EUA/ MAX4131EBT | ±0.35 ±1.20 |  |
|  |  |  | MAX4132ESA/ MAX4133ESD | ±0.25 ±0.75 |  |
|  |  |  | MAX4132EUA | ±0.40 ±1.50 |  |
|  |  |  | MAX4134ESD | ±0.35 ±1.50 |  |
| Input Bias Current | IB | VCM = VEE to VCC |  | ±50 ±150 | nA |
| Input Offset Current | IOS | VCM = VEE to VCC | MAX4131EBT | ±15 | nA |
|  |  |  | All other packages | ±12 |  |
| Differential Input Voltage | RIN | -1.5V < VDIFF < 1.5V |  | 500 | kΩ |
| Common-Mode Input Voltage Range | CMVR |  |  | VEE - VCC - 0.25 0.25 | V |
| Common-Mode Rejection Ratio | CMRR | VEE - 0.25V < VCM < VCC + 0.25V | MAX4130EUK | 67 90 | dB |
|  |  |  | MAX4131ESA | 78 98 |  |
|  |  |  | MAX4131EUA/ MAX4131EBT | 68 88 |  |
|  |  |  | MAX4132ESA/ MAX4133ESD | 74 94 |  |
|  |  |  | MAX4132EUA | 66 86 |  |
|  |  |  | MAX4134ESD | 64 84 |  |
| Power-Supply Rejection Ratio | PSRR | VCC = 2.7V to 6.5V |  | 78 100 | dB |
| Output Resistance | ROUT | AV = 1 |  | 0.1 | Ω |
| Off-Leakage Current | ILKG | SHDN < 0.8V, VOUT = 0V to VCC |  | ±0.1 ±1 | µA |

2

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

DC ELECTRICAL CHARACTERISTICS (continued)

(VCC =  +2.7V  to +6.5V,  VEE =  0V, VCM =  0V, VOUT =  VCC/2, RL tied  to VCC/2, SHDN ≥ 2V (or open),  TA  = +25°C,  unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| Large-Signal Voltage Gain | AV | VCC = 2.7V | VOUT = 0.25V to 2.45V, RL = 100kΩ |  | 92 108 | dB |
|  |  |  | VOUT = 0.4V to 2.3V, RL = 250Ω |  | 72 82 |  |
|  |  | VCC = 5V | VOUT = 0.25V to 4.75V, RL = 100kΩ |  | 94 108 |  |
|  |  |  | VOUT = 0.4V to 4.6V, RL = 250Ω |  | 75 86 |  |
| Output Voltage Swing | VO | MAX4130/ MAX4131 | RL = 100kΩ | VCC - VOH | 12 20 | mV |
|  |  |  |  | VOL - VEE | 20 35 |  |
|  |  |  | RL = 250Ω | VCC - VOH | 240 290 |  |
|  |  |  |  | VOL - VEE | 125 170 |  |
|  |  | MAX4132/ MAX4133/ MAX4134 | RL = 100kΩ | VCC - VOH | 15 30 |  |
|  |  |  |  | VOL - VEE | 25 40 |  |
|  |  |  | RL = 250Ω | VCC - VOH | 280 330 |  |
|  |  |  |  | VOL - VEE | 180 230 |  |
| Output Short-Circuit Current | ISC |  |  |  | 50 | mA |
| SHDN Logic Threshold | VIL | MAX4131–MAX4134 |  | Low | 0.8 | V |
|  | VIH |  |  | High | 2.0 |  |
| SHDN Input Current | IIL | MAX4131–MAX4134 |  |  | ±1 ±3 | µA |
| Operating Supply Voltage Range | VCC |  |  |  | 2.7 6.5 | V |
| Supply Current per Amplifier | ICC | VCM = VOUT = VCC/2 |  | VCC = 2.7V | 900 1050 | µA |
|  |  |  |  | VCC = 5V | 1000 1150 |  |
| Shutdown Supply Current per Amplifier | ISHDN | SHDN > 0.8V, MAX4131– MAX4134 |  | VCC = 2.7V | 25 40 | µA |
|  |  |  |  | VCC = 5V | 40 60 |  |

DC ELECTRICAL CHARACTERISTICS

(VCC = +2.7V to +6.5V, VEE = 0V, VCM = 0V, VOUT = VCC/2, RL tied to VCC/2, SHDN ≥ 2V (or open), TA = -40°C to +85°C, unless otherwise noted.) (Note 2)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Input Offset Voltage | VOS | VCM = VEE to VCC | MAX4130EUK | ±3.50 | mV |
|  |  |  | MAX4131ESA | ±0.75 |  |
|  |  |  | MAX4131EUA/MAX4131EBT | ±4.40 |  |
|  |  |  | MAX4132ESA/MAX4133ESD | ±0.95 |  |
|  |  |  | MAX4132EUA | ±4.70 |  |
|  |  |  | MAX4134ESD | ±4.00 |  |
| Input Offset Voltage Tempco | TCVOS |  |  | ±2 | µV/°C |
| Input Bias Current | IB | VCM = VEE to VCC |  | ±160 | nA |

3

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

DC ELECTRICAL CHARACTERISTICS

(VCC = +2.7V to +6.5V, VEE = 0V, VCM = 0V, VOUT = VCC/2, RL tied to VCC/2, SHDN ≥ 2V (or open), TA = -40°C to +85°C, unless otherwise noted.) (Note 2)

| PARAMETER | SYMBOL | CONDITIONS |  |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input Offset Current | IOS | VCM = VEE to VCC |  | MAX4131EBT |  | ±24 | nA |
|  |  |  |  | All other packages |  | ±18 |  |
| Common-Mode Input Voltage Range | CMVR |  |  |  |  | VEE - VCC - 0.20 0.20 | V |
| Common-Mode Rejection Ratio | CMRR | VEE - 0.2V < VCM < VCC + 0.2V | MAX4130EUK |  |  | 62 | dB |
|  |  |  | MAX4131ESA |  |  | 76 |  |
|  |  |  | MAX4131EUA/MAX4131EBT |  |  | 60 |  |
|  |  |  | MAX4132ESA/MAX4133ESD |  |  | 74 |  |
|  |  |  | MAX4132EUA |  |  | 58 |  |
|  |  |  | MAX4134ESD |  |  | 60 |  |
| Power-Supply Rejection Ratio | PSRR | VCC = 2.7V to 6.5V |  |  |  | 74 | dB |
| Off-Leakage Current | ILKG | SHDN < 0.8V, VOUT = 0V to VCC |  |  |  | ±12 | µA |
| Large-Signal Voltage Gain | AV | VCC = 2.7V | VOUT = 0.25V to 2.45V, RL = 100kΩ |  |  | 84 | dB |
|  |  |  | VOUT = 0.4V to 2.3V, RL = 250Ω |  |  | 66 |  |
|  |  | VCC = 5V | VOUT = 0.25V to 4.75V, RL = 100kΩ |  |  | 86 |  |
|  |  |  | VOUT = 0.4V to 4.6V, RL = 250Ω |  |  | 68 |  |
| Output Voltage Swing | VO | MAX4130/ MAX4131ESA/ MAX4131EUA | RL = 100kΩ |  | VCC - VOH | 25 | mV |
|  |  |  |  |  | VOL - VEE | 40 |  |
|  |  |  | RL = 250Ω |  | VCC - VOH | 300 |  |
|  |  |  |  |  | VOL - VEE | 190 |  |
|  |  | MAX4131EBT | RL = 100kΩ |  | VCC - VOH | 25 |  |
|  |  |  |  |  | VOL - VEE | 40 |  |
|  |  |  | RL = 250Ω |  | VCC - VOH | 350 |  |
|  |  |  |  |  | VOL - VEE | 190 |  |
|  |  | MAX4132/ MAX4133/ MAX4134 | RL = 100kΩ |  | VCC - VOH | 35 |  |
|  |  |  |  |  | VOL - VEE | 50 |  |
|  |  |  | RL = 250Ω |  | VCC - VOH | 350 |  |
|  |  |  |  |  | VOL - VEE | 250 |  |

4

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

DC ELECTRICAL CHARACTERISTICS (continued)

(VCC = +2.7V to +6.5V, VEE = 0V, VCM = 0V, VOUT = VCC/2, RL tied to VCC/2, SHDN ≥ 2V (or open), TA = -40°C to +85°C, unless otherwise noted.) (Note 2)

| PARAMETER | SYMBOL | CONDITIONS |  |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- | --- |
| SHDN Logic Threshold | VIL | MAX4131–MAX4134 |  | Low | 0.8 | V |
|  | VIH |  |  | High | 2.0 |  |
| SHDN Input Current | IIL | MAX4131–MAX4134 |  |  | ±3 | µA |
| Operating Supply Voltage Range | VCC |  |  |  | 2.7 6.5 | V |
| Supply Current per Amplifier | ICC | VCM = VOUT = VCC/2 | MAX4131EBT | VCC = 2.7V | 1200 | µA |
|  |  |  |  | VCC = 5V | 1350 |  |
|  |  |  | All other devices | VCC = 2.7V | 1100 |  |
|  |  |  |  | VCC = 5V | 1200 |  |
| Shutdown Supply Current per Amplifier | ISHDN | SHDN > 0.8V, MAX4131–MAX4134 |  | VCC = 2.7V | 50 | µA |
|  |  |  |  | VCC = 5V | 70 |  |

Note 2: All devices are 100% tested at TA = +25°C. MAX4130EUK/MAX4131EBT temperature  limits are guaranteed  by design.

AC ELECTRICAL CHARACTERISTICS

(VCC = +2.7V to +6.5V, VEE = 0V, SHDN ≥ 2V (or open), TA = +25°C, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Gain-Bandwidth Product | GBWP |  |  | 10 | MHz |
| Phase Margin | θM |  |  | 62 | Degrees |
| Gain Margin | GM |  |  | 12 | dB |
| Total Harmonic Distortion | THD | f = 10kHz, VOUT = 2VP-P (AV = 1) |  | 0.003 | % |
| Slew Rate | SR |  |  | 4 | V/µs |
| Settling Time to 0.01% | tS | AV = 1, VOUT = 2V step |  | 2.0 | µs |
| Turn-On Time | tON | VCC = 0V to 3V step, VOUT = VCC/2 |  | 1 | µs |
| SHDN Delay |  | MAX4131–MAX4134, VCC = 3V, VOUT = VCC/2 | Enable | 1 | µs |
|  |  |  | Disable | 0.2 |  |
| Input Capacitance | CIN |  |  | 3 | pF |
| Input Noise-Voltage Density | en | f = 1kHz |  | 22 | nV/√Hz |
| Input Noise-Current Density | in | f = 1kHz |  | 0.4 | pA/√Hz |
| Capacitive-Load Stability | CL | AV = 1 |  | 160 | pF |

5

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

GAIN (dB)

Typical Operating Characteristics

(VCC = +5V, VEE = 0V, VCM = VCC / 2, TA = +25°C, unless otherwise noted.)

GAIN AND PHASE

vs. FREQUENCY

60

MAX4130/34-01

180

144

GAIN AND PHASE

vs. FREQUENCY (WITH CLOAD)

MAX4130/34-02

GAIN

180

144

POWER-SUPPLY REJECTION

vs. FREQUENCY

MAX4130/34-03

0       AV = +1

40

20

0

-20

-40

AV = +1000

NO LOAD

PHASE

GAIN

108

72

36

0

-36

PHASE (DEGREES)

-72

-108

-144

-180

40

60

20

0

-20

-40

AV = +1000

GAIN (dB)

RL = ∞

CL = 160pF

PHASE

108

72

36

0

-36

PHASE (DEGREES)

PSR (dB)

-72

-108

-144

-180

-20

-40

-60

-80

-100

100     1k      10k

100k

1M    10M

100M

100     1k      10k

100k

1M    10M

100M

10    100    1k

10k

100k   1M   10M

100M

FREQUENCY (Hz)

FREQUENCY (Hz)

FREQUENCY (Hz)

100

OUTPUT IMPEDANCE

vs. FREQUENCY

MAX4130/34-05

1150

SUPPLY CURRENT PER AMPLIFIER

vs. TEMPERATURE

MAX4130/34-07

SHUTDOWN SUPPLY CURRENT

vs. TEMPERATURE

| 50 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VCC = +6.5V |  |  |  |  |  |
|  |  |  | VCC = +2.7V |  |  |  |  |  |

AV = +1

10

1

1100

1050

1000

VCC

MAX4130/34-06

= +6.5V                           45

35

30

25

OUTPUT IMPEDANCE (Ω)

0.10

0.01

100     1k

10k

100k    1M    10M

100M

950

SUPPLY CURRENT (µA)

900

850

800

VCC = +2.7V

-40  -25  -10   5    20   35   50   65   80   95

20

SUPPLY CURRENT (µA)

15

10

5

0

-40  -25  -10   5    20   35   50   65   80   95

FREQUENCY (Hz)

TEMPERATURE (°C)

TEMPERATURE (°C)

MAX4130/34-08

OUTPUT LEAKAGE CURRENT

vs. TEMPERATURE

20

AV = +1

SHDN = 0V

LEAKAGE CURRENT (µA)

15

10              VOUT SHORT    VCC = +6.5V TO VEE       VCC = +2.7V

5

0

40

30

20

10

0

-10

-20

-30

INPUT BIAS CURRENT

vs. COMMON-MODE  VOLTAGE

VCC = +2.7V

MAX4130/34-10

VCC = +6.5V

60

40

20

0

INPUT BIAS CURRENT (nA)

-20

INPUT BIAS CURRENT

vs. TEMPERATURE

MAX4130/34-11

VCC = +6.5V, VCM = VCC

VCC = +2.7V, VCM = VCC

-5

-10

VOUT SHORT TO VCC

VCC = +6.5V VCC = +2.7V

INPUT BIAS CURRENT (nA)

-40

-50

-60

-40

-60

VCC = +6.5V, VCM = VEE

-40  -25  -10   5    20   35   50   65   80   95

TEMPERATURE (°C)

0        1        2        3        4        5        6

COMMON-MODE VOLTAGE (V)

-40  -25  -10   5    20   35   50   65   80   95

TEMPERATURE (°C)

6

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

Typical Operating Characteristics (continued)

(VCC = +5V, VEE = 0V, VCM = VCC / 2, TA = +25°C, unless otherwise noted.)

3.00

2.25

1.50

0.75

0

VOLTAGE (mV)

-0.75

-1.50

-2.25

-3.00

MAX4130/34-09

INPUT OFFSET VOLTAGE

vs. TEMPERATURE

|  |  |  |  |  |  | SOT23 PACKAGE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SO PACKAGE |  |  |  |  |  |

-40  -25  -10   5    20   35   50   65   80   95

TEMPERATURE (°C)

1.9

1.8

1.7

1.6

1.5

1.4

MINIMUM OPERATING VOLTAGE (V)

1.3

1.2

MAX4130/34-21

MINIMUM OPERATING VOLTAGE

vs. TEMPERATURE

-40  -25  -10   5    20   35   50   65   80   95

TEMPERATURE (°C)

120

COMMON-MODE  REJECTION (dB)

115

110

105

100

95

90

85

80

MAX4130/34-12

COMMON-MODE  REJECTION

vs. TEMPERATURE

|  |  | VCM = 0V TO 5.0V |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VCM = 0.2V TO 5.2V |  |  |  |  |  |  |  |
|  |  | VCM = 0.3V TO 5.3V |  |  |  |  |  |  |  |

-40  -25  -10   5    20   35   50   65   80   95

TEMPERATURE (°C)

120

LARGE-SIGNAL GAIN

vs. OUTPUT VOLTAGE

130

LARGE-SIGNAL GAIN

vs. OUTPUT VOLTAGE

MAX4130/34-16

120

LARGE-SIGNAL GAIN

vs. OUTPUT VOLTAGE

MAX4130/34-18

R  = 100kΩ

115

110

105

GAIN (dB)

100

95

90

85

80

75

70

VCC = +6.5V

RL TO VCC

MAX4130/34-19

RL = 100kΩ

RL = 10kΩ

RL = 2kΩ

RL = 500Ω

120

GAIN (dB)

110

100

90

80

70

VCC = +6.5V

RL TO VEE

RL = 100kΩ

RL = 10kΩ

RL = 2kΩ

RL = 500Ω

115

110

105

100

95

GAIN (dB)

90

85

80

75

70

VCC = +2.7V RL TO VCC

L

RL = 10kΩ RL = 2kΩ RL = 500Ω

0       100     200

300

400

500

600

0       100     200

300

400

500

600

0       100     200

300

400

500

600

120

110

GAIN (dB)

100

90

80

70

60

OUTPUT VOLTAGE: EITHER SUPPLY (mV)

LARGE-SIGNAL GAIN

vs. OUTPUT VOLTAGE

VCC = +2.7V RL TO VEE

RL = 100kΩ

RL = 500Ω

RL = 2kΩ

RL = 10kΩ

120

115

MAX4130/34-15

110

105

100

GAIN (dB)

95

90

85

80

OUTPUT VOLTAGE: EITHER SUPPLY (mV)

LARGE-SIGNAL GAIN

vs. TEMPERATURE

|  |  |  |  |  | VOUT(P-P) = VCC -1V RL = 500Ω |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VCC = +6.5V, RL TO VEE |  |  |  |  |  |  |  |  |  |
|  |  |  |  | VCC = +2.7V, RL TO VEE |  |  |  |  |  |
|  |  |  |  |  |  | VCC = +6.5V, RL TO VCC |  |  |  |
| VCC = +2.7V, RL TO VCC |  |  |  |  |  |  |  |  |  |

-40  -25  -10   5    20   35   50   65   80   95

MAX4130/34-17

130

125

120

115

110

GAIN (dB)

105

100

95

90

85

OUTPUT VOLTAGE: EITHER SUPPLY (mV)

LARGE-SIGNAL GAIN

vs. TEMPERATURE

MAX4130/34-20

VCC = +6.5V, RL TO VCC OR VEE

VCC = +2.7V, RL TO  VCC OR VEE

RL = 100kΩ

VOUT(P-P) = VCC - 0.6V

-40  -25  -10   5    20   35   50   65   80   95

0       100     200

300

400

500

600

OUTPUT VOLTAGE: EITHER SUPPLY (mV)

TEMPERATURE (°C)

TEMPERATURE (°C)

7

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

VOUT - VEE (mV)

Typical Operating Characteristics (continued)

(VCC = +5V, VEE = 0V, VCM = VCC / 2, TA = +25°C, unless otherwise noted.)

160

140

120

100

80

60

40

20

0

MINIMUM OUTPUT VOLTAGE

vs. TEMPERATURE

RL TO VCC

VCC = +6.5V, RL = 500Ω

VCC = +2.7V, RL = 500Ω

VCC = +6.5V, RL = 100kΩ

VCC = +2.7V, RL = 100kΩ

MAX4130/34-13

300

250

VCC - VOUT (mV)

200

150

100

50

0

MAXIMUM OUTPUT VOLTAGE

vs. TEMPERATURE

MAX4130/34-14

RL TO VEE               VCC = +6.5V, RL = 500Ω

VCC = +2.7V, RL = 500Ω

VCC = +6.5V, RL = 100kΩ  (TOP)

VCC = +2.7V, RL = 100kΩ (BOTTOM)

0.030

0.025

0.020

0.015

THD AND NOISE (%)

0.010

0.005

0

TOTAL HARMONIC DISTORTION AND NOISE vs. FREQUENCY

2Vp-p SIGNAL

MAX4130/34-22A

500kHz LOWPASS FILTER RL = 10kΩ TO VCC / 2

-40  -25  -10   5    20   35   50   65   80   95

TEMPERATURE (°C)

-40  -25  -10   5    20   35   50   65   80   95

TEMPERATURE (°C)

10            100           1k

FREQUENCY (Hz)

MAX4131

10k

100k

0.1

TOTAL HARMONIC DISTORTION AND NOISE

vs. PEAK-TO-PEAK SIGNAL AMPLITUDE

AV = +1

10kHz SINE WAVE

RL TO VCC / 2

MAX4130/34-23

140

CHANNEL SEPARATION (dB)

130

CHANNEL SEPARATION

vs. FREQUENCY

MAX4130/34-25a

VS = +5V

SMALL-SIGNAL TRANSIENT RESPONSE (NONINVERTING)

MAX4130/34-24

AV = +1

VOLTAGE (50mV/div)

500kHz LOWPASS FILTER                                                                                                                                       IN

120

THD + NOISE (%)

0.01

RL = 250Ω

RL = 2kΩ

RL = 10kΩ

RL = 100kΩ

110

100

90

OUT

0.001

4.0

4.2

4.4

4.6

4.8

5.0

80

1k            10k

100k          1M

10M

TIME (200ns/div)

PEAK-TO-PEAK SIGNAL AMPLITUDE (V)

MAX4131

MAX4130/34-25

SMALL-SIGNAL TRANSIENT RESPONSE (INVERTING)

FREQUENCY (Hz)

MAX4131

LARGE-SIGNAL TRANSIENT RESPONSE (NONINVERTING)

MAX4130/34-27

MAX4131

MAX4130/34-26

LARGE-SIGNAL TRANSIENT RESPONSE (INVERTING)

AV = -1

AV = +1

AV = -1

VOLTAGE (50mV/div)

VOLTAGE (2V/div)

VOLTAGE (2V/div)

IN                                                                                          IN                                                                                        IN

OUT

OUT

OUT

TIME (200ns/div)

TIME (2µs/div)

TIME (2µs/div)

8

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

Pin  Description

| PIN |  |  |  |  |  | NAME | FUNCTION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MAX4130 | MAX4131 |  | MAX4132 | MAX4133 | MAX4134 |  |  |
|  | SO/µMAX | UCSP |  |  |  |  |  |
| 1 | 6 | A2 | — | — | — | OUT | Output |
| 2 | 4 | A1 | 4 | 4 | 11 | VEE | Negative Supply. Connect to GND for single-supply operation. |
| 3 | 3 | B1 | — | — | — | IN+ | Noninverting Input |
| 4 | 2 | B2 | — | — | — | IN- | Inverting Input |
| 5 | 7 | A3 | 8 | 14 | 4 | VCC | Positive Supply |
| — | 1, 5 | — | — | 5, 7, 8, 10 | — | N.C. | No Connection. Not internally connected. |
| — | 8 | B3 | — |  | — | SHDN | Shutdown Control. Connect to VCC or leave floating to enable amplifier. |
| — | — | — | 1, 7 | 1, 13 | 1, 7 | OUT1, OUT2 | Outputs for Amps 1 and 2 |
| — | — | — | 2, 6 | 2, 12 | 2, 6 | IN1-, IN2- | Inverting Inputs for Amps 1 and 2 |
| — | — | — | 3, 5 | 3, 11 | 3, 5 | IN1+, IN2+ | Noninverting Inputs for Amps 1 and 2 |
| — | — | — | — | 6, 9 | — | SHDN1, SHDN2 | Shutdown Control. Provides independent shutdown control for amps 1 and 2. Connect to VCC or leave floating to enable amplifiers. |
| — | — | — | — | — | 8, 14 | OUT3, OUT4 | Outputs for Amps 3 and 4 |
| — | — | — | — | — | 9, 13 | IN3-, IN4- | Inverting Inputs for Amps 3 and 4 |
| — | — | — | — | — | 10, 12 | IN3+, IN4+ | Noninverting Inputs for Amps 3 and 4 |

R3

R3 = R1     R2

MAX4130

MAX4131

MAX4132

MAX4133

MAX4134

R3

R3 = R1     R2

MAX4130

MAX4131

MAX4132

MAX4133

MAX4134

R1                                               R2

R1                                               R2

Figure 1a. Reducing Offset Error Due to Bias Current

(Noninverting)

Figure 1b. Reducing Offset Error Due to Bias Current

(Inverting)

9

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

Applications Information

Rail-to-Rail Input Stage Devices in the MAX4130–MAX4134 family of high- speed  amplifiers have rail-to-rail input and output stages designed  for low-voltage, single-supply  opera- tion. The input stage consists of separate NPN and PNP differential stages that combine  to provide an input common-mode range that extends 0.2V beyond the supply rails. The PNP stage is active for input volt- ages close to the negative rail, and the NPN stage is active for input voltages near the positive rail. The input offset voltage is typically below 200µV. The switchover transition region, which occurs near VCC / 2, has been extended to minimize the slight degradation  in com- mon-mode rejection ratio caused by the mismatch of the input pairs. Their low offset voltage, high band- width, and rail-to-rail common-mode range make these

Reduce the offset error caused by input bias currents flowing through external source impedances by match-

ing the effective impedance  seen by each input (Figures 1a, 1b). High source impedances,  together with input capacitance, can create a parasitic pole that produces an underdamped signal response. Reducing the input impedance  or placing  a small (2pF to 10pF) capacitor  across the feedback  resistor improves response.

The MAX4130–MAX4134s’ inputs are protected  from large differential input voltages by 1kΩ series resistors and back-to-back  triple diodes  across the inputs (Figure 2). For differential input voltages less than 1.8V, input resistance is typically 500kΩ. For differential input voltages greater than 1.8V, input resistance is approxi- mately 2kΩ. The input bias current is given by the fol- lowing equation:

op amps excellent choices for precision, low-voltage

VDIFF

- 1.8V

data-acquisition systems.

Since the input stage switches between the NPN and PNP pairs, the input bias current changes polarity as the input voltage passes through the transition region.

IBIAS  =

2kΩ

1kΩ

1kΩ

Figure 2. Input Protection Circuit

10

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

Rail-to-Rail Output Stage The minimum output voltage is within millivolts of ground for single-supply  operation where the load is referenced to ground (VEE). Figure 3 shows the input voltage range and output voltage swing of a MAX4131 connected  as a voltage follower. With a +3V supply and the load tied to ground, the output swings from

0.00V to 2.90V. The maximum output voltage swing depends on the load, but will be within 150mV of a +3V supply, even with the maximum load (500Ω to ground).

MAX4130–MAX4134

Driving a capacitive load can cause instability in most high-speed  op amps, especially those with low quies- cent current. The MAX4130–MAX4134 have a high tol- erance for capacitive  loads. They are stable with capacitive loads up to 160pF. Figure 4 gives the stable operating region for capacitive loads. Figures 5 and 6 show the response with capacitive  loads and the results of adding an isolation resistor in series with the output (Figure 7). The resistor improves the circuit’s phase margin by isolating the load capacitor  from the op amp’s output.

VOLTAGE (1V/div)

IN

OUT

VCC = 3V, RL = 10kΩ to VEE

TIME (1µs/div)

500

450

LOAD CAPACITANCE (pF)

400

350

300

250

200

150

100

50

0

RL TO VEE

VOUT = VCC/2

0.1                  1

UNSTABLE OPERATING REGION

STABLE OPERATING REGION

10

100

LOAD RESISTANCE (kΩ)

Figure 3. Rail-to-Rail Input/Output  Voltage Range

VOLTAGE (50mV/div)

Figure 4. Capacitive-Load Stability

VCC = 5V RL = 10kΩ CL = 130pF

VOLTAGE (50mV/div)

IN

VCC = 5V

CL = 1000pF

RS = 39Ω

IN

OUT

OUT

TIME (200ns/div)

TIME (500ns/div)

Figure 5. MAX4131 Small-Signal Transient Response with

Capacitive  Load

Figure 6. MAX4131 Transient Response to Capacitive  Load with Isolation Resistor

11

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

Power-Up and Shutdown Mode The MAX4130–MAX4134 amplifiers typically settle with- in 1µs after power-up. Figures 9 and 10 show the out- put voltage and supply current on power-up, using the test circuit of Figure 8.

The MAX4131 and MAX4133 have a shutdown option. When the shutdown pin (SHDN) is pulled low, the sup- ply current drops below 25µA per amplifier and the

amplifiers are disabled  with the outputs in a high- impedance state. Pulling SHDN high or leaving it float-

ing  enables  the amplifier.  In the dual-amplifier MAX4133, the shutdown functions operate indepen- dently. Figures 11 and 12 show the output voltage and supply current responses of the MAX4131 to a shut- down pulse, using the test circuit of Figure 8.

VCC

0V TO 2.7V STEP FOR POWER-UP TEST;  2.7V FOR SHUTDOWN ENABLE TEST

*0.1µF

2kΩ

OUT

RS

CL

2kΩ

10Ω

SHDN

0V TO 2.7V STEP FOR SHUTDOWN TEST

10kΩ

SUPPLY-CURRENT MONITORING POINT

*FOR SHUTDOWN TEST ONLY.

Figure 7. Capacitive-Load Driving Circuit

Figure 8. Power-Up/Shutdown Test Circuit

VOLTAGE (1V/div)

VCC

VCC

(1V/div)

OUT

IEE

(500µA/div)

TIME (5µs/div)

TIME (5µs/div)

Figure 9. Power-Up Output Voltage

Figure 10. Power-Up Supply Current

12

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

VOLTAGE (1V/div)

SHDN

SHDN

1V/div

OUT

SUPPLY CURRENT

500µA/div

TIME (1µs/div)

TIME (1µs/div)

Figure 11. Shutdown Output Voltage

MAX4130–MAX4134

Figure 12. Shutdown Enable/Disable  Supply Current

Power Supplies and Layout The MAX4130–MAX4134 operate from a single +2.7V to +6.5V power supply, or from dual supplies of ±1.35V to ±3.25V. For single-supply  operation, bypass the power supply with a 0.1µF ceramic capacitor in parallel with at least 1µF. For dual supplies, bypass each sup- ply to ground.

Good layout improves performance by decreasing the amount of stray capacitance  at the op amp’s inputs and outputs. Decrease stray capacitance  by placing external components close to the op amp’s pins, mini- mizing trace lengths and resistor leads.

UCSP  Applications Information

For the latest application details on UCSP construction, dimensions, tape carrier information, PC board tech- niques, bump-pad  layout, and the recommended reflow temperature profile, as well as the latest informa- tion on reliability testing results, go to Maxim’s website at www.maxim-ic.com/ucsp and search for the Application  Note: UCSP–A Wafer-Level Chip-Scale Package.

13

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

Pin  Configurations

TOP VIEW

TOP VIEW

BUMPS ON BOTTOM

OUT

1                                      5     VCC

IN+

B1            A1

VEE

VEE      2

MAX4130

IN-

B2            A2

OUT

IN+    3                                     4

IN-

SHDN

MAX4131

B3            A3

VCC

| SOT23 | MAX4131 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

| UCSP | MAX4132 |  |
| --- | --- | --- |
| 1 |  | 8 |
| 2 |  | 7 |
| 3 |  | 6 |
| 4 |  | 5 |

| N.C. |  |  | SHDN | OUT1 |  | VCC |
| --- | --- | --- | --- | --- | --- | --- |
| IN1- IN1+ |  |  | VCC OUT | IN1- IN1+ |  | OUT2 IN2- |
| VEE |  |  | N.C. | VEE |  | IN2+ |
|  | SO/µMAX |  |  |  | SO/µMAX |  |

OUT1  1

IN1- 2

IN1+   3

VEE    4

N.C.   5

SHDN1   6

N.C.    7

MAX4133

14    VCC

13    OUT2

12   IN2-

11   IN2+

10   N.C.

9     SHDN2

8     N.C.

OUT1   1

IN1-  2

IN1+    3

VCC    4

IN2+    5

IN2-  6

OUT2   7

MAX4134

14    OUT4

13   IN4-

12   IN4+

11   VEE

10   IN3+

9     IN3-

8     OUT3

SO                                                                                                                SO

14

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

Ordering Information (continued)

MAX4130–MAX4134

| Chip Information MAX4130 TRANSISTOR COUNT: | 170 |
| --- | --- |
| MAX4131 TRANSISTOR COUNT: | 170 |
| MAX4132 TRANSISTOR COUNT: | 340 |
| MAX4134 TRANSISTOR COUNT: | 680 |

PART TEMP RANGE PIN- TOP PACKAGE MARK

MAX4132ESA -40°C to +85°C 8 SO —

MAX4132EUA -40°C to +85°C 8 µMAX —

MAX4133C/D 0°C to +70°C Dice* —

MAX4133ESD -40°C to +85°C 14 SO —

MAX4134ESD -40°C to +85°C 14 SO —

*Dice are specified  at TA = +25°C, DC parameters only.

SOT-23 5L .EPS

Package Information (The package  drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package  outline information, go to  www.maxim-ic.com/packages.)

|  | PACKAGE OUTLINE, SOT-23, 5L |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | 21-0057 | E | 1 1 |

15

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

SOT23, 8L .EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) SYMBOL | MIN | MAX |
| --- | --- | --- |
| A | 0.90 | 1.45 |
| A1 | 0.00 | 0.15 |
| A2 | 0.90 | 1.30 |
| b | 0.28 | 0.45 |
| C | 0.09 | 0.20 |
| D | 2.80 | 3.00 |
| E | 2.60 | 3.00 |
| E1 | 1.50 | 1.75 |
| L | 0.30 | 0.60 |
| L2 | 0.25 BSC. |  |
| e | 0.65 BSC. |  |
| e1 | 1.95 REF. |  |
| 0 | 0 | 8 |

| SEE DETAIL "A" b e CL CL E CL E1 PIN 1 I.D. DOT (SEE NOTE 6) e1 D C CL L2 A A2 A1 GAUGE PLANE SEATING PLANE C L 0 NOTE: 1. ALL DIMENSIONS ARE IN MILLIMETERS. 2. FOOT LENGTH MEASURED FROM LEAD TIP TO UPPER RADIUS OF DETAIL "A" HEEL OF THE LEAD PARALLEL TO SEATING PLANE C. 3. PACKAGE OUTLINE EXCLUSIVE OF MOLD FLASH & METAL BURR. 4. PACKAGE OUTLINE INCLUSIVE OF SOLDER PLATING. |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5. COPLANARITY 4 MILS. MAX. 6. PIN 1 I.D. DOT IS 0.3 MM MIN. LOCATED ABOVE PIN 1. 7. SOLDER THICKNESS MEASURED AT FLAT SECTION OF LEAD BETWEEN 0.08mm AND 0.15mm FROM LEAD TIP. 8. MEETS JEDEC MO178. | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, SOT-23, 8L BODY |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0078 | REV. D | 1 1 |

16

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

6L, UCSP.EPS

Package Information (continued) (The package  drawing(s)  in this data sheet may not reflect the most current specifications. For the latest package  outline information, go to  www.maxim-ic.com/packages.)

PACKAGE OUTLINE, 3x2 UCSP

1

21-0097                G      1

17

Single/Dual/Quad, Wide-Bandwidth, Low-Power, Single-Supply, Rail-to-Rail I/O Op Amps

MAX4130–MAX4134

SOICN .EPS

| Package Information (continued) (The package drawing(s) in this data sheet may not reflect the most current specifications. For the latest package outline information, go to www.maxim-ic.com/packages.) | INCHES |  | MILLIMETERS |  |
| --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX |
| A | 0.053 | 0.069 | 1.35 | 1.75 |
| A1 | 0.004 | 0.010 | 0.10 | 0.25 |
| B | 0.014 | 0.019 | 0.35 | 0.49 |
| C | 0.007 | 0.010 | 0.19 | 0.25 |
| e | 0.050 BSC |  | 1.27 BSC |  |
| E | 0.150 | 0.157 | 3.80 | 4.00 |
| H | 0.228 | 0.244 | 5.80 | 6.20 |
| L | 0.016 | 0.050 | 0.40 | 1.27 |

|  | INCHES |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DIM | MIN | MAX | MIN | MAX | N | MS012 |
| D | 0.189 | 0.197 | 4.80 | 5.00 | 8 | AA |
| D | 0.337 | 0.344 | 8.55 | 8.75 | 14 | AB |
| D | 0.386 | 0.394 | 9.80 | 10.00 | 16 | AC |

| N E H VARIATIONS: 1 TOP VIEW D A C e B A1 0 -8 L FRONT VIEW SIDE VIEW |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PROPRIETARY INFORMATION |  |  |  |
|  | TITLE: PACKAGE OUTLINE, .150" SOIC |  |  |  |
|  | APPROVAL | DOCUMENT CONTROL NO. 21-0041 | REV. B | 1 1 |

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses are implied. Maxim reserves the right to change the circuitry and specifications without notice at any time.

18                                     Maxim Integrated Products, 120 San Gabriel Drive, Sunnyvale, CA   94086 (408) 737-7600

©  2003 Maxim Integrated  Products               Printed USA                 is a registered  trademark of Maxim Integrated  Products.