19-0143; Rev 1; 2/95

Digitally Adjustable LCD Bias Supply

General Description

The MAX749 generates negative LCD-bias contrast voltages from 2V to 6V inputs. Full-scale output voltage can be scaled to -100V or greater, and is digitally adjustable in 64 equal steps by an internal digital-to- analog converter (DAC). Only seven small surface- mount components are required to build a complete supply. The output voltage can also be adjusted using a PWM signal or a potentiometer.

A unique current-limited control scheme reduces supply current and maximizes efficiency, while a high switching frequency (up to 500kHz) minimizes the size of external components. Quiescent current is only 60µA max and is reduced to under 15µA in shutdown mode. While shut down, the MAX749 retains the voltage set point, simpli- fying software control. The MAX749 drives either an external P-channel MOSFET or a PNP transistor.

MAX749

Features

♦ +2.0V to +6.0V Input Voltage Range

♦ Flexible Control of Output Voltage: Digital Control

Potentiometer Adjustment

PWM Control

♦ Output Voltage Range Set by One Resistor

♦ Low, 60µA Max Quiescent Current

♦ 15µA Max Shutdown Mode

♦ Small Size – 8-Pin SO and Plastic DIP Packages

Applications

Notebook Computers Laptop Computers Palmtop Computers Personal Digital Assistants

Communicating Computers

Portable Data-Collection Terminals

Ordering Information

PART TEMP. RANGE PIN-PACKAGE

MAX749CPA 0°C to +70°C 8 Plastic DIP

MAX749CSA 0°C to +70°C 8 SO

MAX749C/D 0°C to +70°C Dice*

MAX749EPA -40°C to +85°C 8 Plastic DIP

MAX749ESA -40°C to +85°C 8 SO

* Contact factory for dice specifications.

Typical Operating Circuit

Pin  Configuration

VIN +5V

TOP VIEW

0.1 F

1  V+

2

CS   8

7

RSENSE

V+  1

8   CS

DIGITAL ADJUST

ADJ MAX749 DHI

3                                  6

-VOUT

ADJ  2

MAX749

7   DHI

ON/OFF

CTRL

4  FB

DLOW

GND   5

CTRL   3

FB   4

6   DLOW

5   GND

RFB

DIP/SO

CCOMP

Maxim Integrated Products     1

Call toll free 1-800-998-8800 for free samples or literature.

Digitally Adjustable LCD Bias Supply

MAX749

ABSOLUTE MAXIMUM RATINGS

V+ ................................................................................-0.3V, +7V CTRL, ADJ, FB, DLOW, DHI, CS .....................-0.3V, (V+ + 0.3V) Continuous Power Dissipation (TA = +70°C)

Plastic DIP (derate 9.09mW/°C above +70°C) ............727mW

SO (derate 5.88mW/°C above +70°C) .........................471mW

Operating Temperature Ranges: MAX749C_A........................................................0°C to +70°C MAX749E_A .....................................................-40°C to +85°C

Storage Temperature Range .............................-65°C to +160°C Lead Temperature (soldering, 10sec) .............................+300°C

Stresses beyond those listed under “Absolute Maximum Ratings" may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated in the operational sections of the specifications is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(2V < V+ < 6V, TA = TMIN to TMAX, unless otherwise noted.)

| PARAMETER | SYMBOL | CONDITIONS | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- |
| V+ Voltage |  |  | 2 6 | V |
| FB Source Current | IFBS | On power-up or reset, VFB = 0V (Note 1) | 12.80 13.33 13.86 | µA |
| Zero-Count FB Current |  | VFB = 0V | 0.45 0.55 | IFBS |
| Full-Count FB Current |  | VFB = 0V | 1.43 1.53 | IFBS |
| FB Offset Voltage |  |  | ±15 | mV |
| DAC Step Size (Note 2) |  | Monotonicity guaranteed, VFB = 0V | 1.00 1.56 2.12 | %IFBS |
| DAC Linearity (Note 2) |  | VFB = 0V | ±1 | %IFBS |
| Supply Rejection |  | V+ = 2V to 6V, full-count current | 1.5 | %IFBS |
| Switching Frequency |  |  | 100 to 500 | kHz |
| Logic Input Current |  | 0V < VIN < V+, CTRL, ADJ | ±100 | nA |
| Logic High Threshold (Note 3) | VIH | CTRL, ADJ | 1.6 | V |
| Logic Low Threshold (Note 3) | VIL | CTRL, ADJ | 0.4 | V |
| Quiescent Current |  |  | 60 | µA |
| Shutdown Current |  |  | 15 | µA |
| V+ to CS Voltage |  | Current-limit trip voltage | 110 140 180 | mV |
| DHI Source Current |  | V+ = 2V, VDHI = 1V | 24 50 | mA |
| DHI Drive Level |  | No load | V+ - 50mV V+ | V |
| DLOW On Resistance |  | V+ = 2V, VDLOW = 0.5V | 5 10 | Ω |

Note 1: The device is in regulation when VFB = 0V (see Figures 3 - 6).

Note 2: These tests performed at V+ = 3.3V. Operation over supply range is guaranteed by supply rejection test of full-count current.

Note 3: VIH is guaranteed by design to be 1.8V min for V+ = 2V to 6V for TA = TMIN to TMAX. VIL is guaranteed by design from

TA = TMIN to TMAX.

TIMING CHARACTERISTICS

| PARAMETER | SYMBOL | CONDITIONS | TA = +25°C MIN TYP MAX | TA = TMIN to TMAX MIN MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Minimum Reset Pulse Width | tR | V+ = 2V | 125 300 | 400 | ns |
|  |  | V+ = 5V | 25 85 | 100 |  |
| Minimum Reset Setup | tRS | Not tested | 0 | 0 | ns |
| Minimum Reset Hold | tRH | Not tested | 0 | 0 | ns |
| Minimum ADJ High Pulse Width | tSH | V+ = 2V | 15 85 | 100 | ns |
|  |  | V+ = 5V | 10 85 | 100 |  |
| Minimum ADJ Low Pulse Width | tSL | V+ = 2V | 170 400 | 500 | ns |
|  |  | V+ = 5V | 60 150 | 200 |  |
| Minimum ADJ Low to CTRL Low | tSD | V+ = 2V | 70 200 | 250 | ns |
|  |  | V+ = 5V | 20 85 | 100 |  |

2

Digitally Adjustable LCD Bias Supply

MAX749

Typical Operating Characteristics

(TA = +25°C, L = 47µH, unless otherwise noted.)

EFFICIENCY vs. OUTPUT CURRENT – PNP

85

80

EFFICIENCY (%)

75

70

EFFICIENCY vs. OUTPUT CURRENT – PNP

MAX749TOC2-A

80

78

76

74

72

EFFICIENCY (%)

70

68

EFFICIENCY vs. OUTPUT CURRENT – MOSFET

85

MAX749TOC2-C

-24V

-12V

MAX749TOC2-B

80

-5V

EFFICIENCY (%)

75

V+ = 5V RSENSE = 0.25

TRANSISTOR: SMD10P05L

70

66

65

0        10       20       30       40       50       60

| 64 |  |  |  |  | -12V -5V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | -24V |  |  |  |  |  |
|  |  |  | V+ = 3V RBASE = 470 RSENSE = 0.25 TRANSISTOR: ZTX750 |  |  |  |

| 0 10 20 30 40 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | -12V |  |  |  |  |
|  |  |  |  |  |  |  | - | 5V |  |
|  |  | -24V |  |  |  |  |  |  |  |
|  |  |  |  | V+ = 3V RBASE = 160 RSENSE = 0.25 TRANSISTOR = ZTX750 |  |  |  |  |  |

50  60  70  80

90 100

65

0     5   10

15  20

25  30

35  40   45  50

OUTPUT CURRENT (mA)

85

EFFICIENCY  vs. OUTPUT VOLTAGE

OUTPUT CURRENT (mA)

85

EFFICIENCY  vs. OUTPUT VOLTAGE

MAX749-TOC1-B

OUTPUT CURRENT (mA)

-20mA

-5mA

80

-5mA

EFFICIENCY (   )

75

MAX749-TOC1-A

-20mA

EFFICIENCY (   )

-40mA                          80

75

-40mA

V+ = 3V

70           RBASE = 470

70        V+ = 5V RSENSE

= 0.25

RSENSE = 0.25

TRANSISTOR : ZTX750

65

-24 -22 -20 -18 -16 -14 -12 -10  -8   -6   -4

OUTPUT VOLTAGE (V)

TRANSISTOR : SMD10P05L

65

-24 -22 -20 -18 -16 -14 -12 -10  -8   -6    -4

OUTPUT VOLTAGE (V)

400

LOAD CURRENT vs. INPUT VOLTAGE

500

MAX749-TOC3-B

LOAD CURRENT vs. INPUT VOLTAGE

RBASE = 470

MAX749-TOC3-A

-5V

RBASE = 160

LOAD CURRENT (mA)

300        R

= 0.25

= ZTX750

LOAD CURRENT (mA)

400

350

RSENSE = 0.25

TRANSISTOR = ZTX750

-5V

250

200

150

100

50

0

TRANSISTOR

-12V

-24V

-48V

300

250

200

100

50

0

-12V

-24V

-48V

2               3              4               5              6

INPUT VOLTAGE (V)

2               3              4               5              6

INPUT VOLTAGE (V)

3

Digitally Adjustable LCD Bias Supply

MAX749

Typical Operating Characteristics (continued)

(TA = +25°C, L = 47µH, unless otherwise noted.)

LOAD-TRANSIENT RESPONSE

LINE-TRANSIENT RESPONSE

OUTPUT VOLTAGE

100mVAC/div

OUTPUT VOLTAGE

100mVAC/div

LOAD CURRENT

VOUT = -15V TRANSISTOR = ZTX750

50 s/div

VOUT = -15V TRANSISTOR = ZTX750

10mA/div

0mA

INPUT VOLTAGE

VOUT = -15V ILOAD = 5mA

50ms/div

1 V/div

0V

TRANSISTOR = ZTX750

Pin  Description

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1 | V+ | +2V to +6V Input Voltage to power the MAX749 and external circuitry. When using an external P-channel MOSFET, V+ must exceed the MOSFET’s gate threshold voltage. |
| 2 | ADJ | Logic Input. When CTRL is high, a rising edge on ADJ increments an internal counter. When CTRL is low, the counter is reset to mid-scale when ADJ is high. When ADJ is low, the counter does not change (regardless of activity on CTRL) as long as V+ is applied. |
| 3 | CTRL | Logic Input. When CTRL and ADJ are low, the MAX749 is shut down, but the counter is not reset. When CTRL is low, the counter is reset to mid-scale when ADJ is high. The device is always on when CTRL is high. |
| 4 | FB | Feedback Input for output full-scale voltage selection. -VOUT(MAX) = (RFB) x (20µA) where RFB is connected from FB to -VOUT. The device is in regulation when VFB = 0V. |
| 5 | GND | Ground |
| 6 | DLOW | Output Driver Low. Connect to DHI when using an external P-channel MOSFET. When using an external PNP transistor, connect a resistor RBASE from DLOW to the base of the PNP to set the maxi- mum base-drive current. |
| 7 | DHI | Output Driver High. Connect to the gate of the external P-channel transistor, or to the base of the external PNP transistor. |
| 8 | CS | Current-Sense Input. The external transistor is turned off when current through the sense resistor, RSENSE, brings CS below V+ by 140mV (typ). |

4

Digitally Adjustable LCD Bias Supply

+2V TO +6V INPUT

0.1 F

22  F

6.2V

POWER-ON RESET

RESET

6-BIT COUNTER

V+

6-BIT

CURRENT-OUTPUT DAC

6.66 A TO 20 A

MAX749

RSENSE

CTRL ADJ

LOGIC

INCREMENT

REF

CS

ON/OFF

BIAS

SWITCH- MODE POWER SUPPLY

DHI DLOW

RBASE

470

Q1

ZTX750

L1

47 H

D1

1N5819

MAX749

GND

RFB

FB

CCOMP

VOUT

(NEGATIVE)

22 F

30V

Figure 1.   Block Diagram, Showing External Circuitry Using a PNP Transistor

Detailed Description

The MAX749 is a negative-output inverting power con- troller that can drive an external PNP transistor or P- channel MOSFET. An external resistor and an internal DAC control the output voltage (Figure 1).

The MAX749 is designed to operate from 2V to 6V inputs, ideal for operation from low-voltage batteries. In systems with higher-voltage batteries, such as notebook comput- ers, the MAX749 may also be operated from the regulat- ed +5V supply. A high-efficiency +5V regulator, such as the MAX782, is an ideal source for the MAX749. In this example, the MAX749 efficiency (80%) is compounded with the MAX782 efficiency (95%): 80% x 95% = 76%, which is still high.

Operating Principle The MAX749 and the external components shown in the Typical Operating Circuit form a flyback converter. When the external transistor is on, current flows through the current-sense resistor, the transistor, and the coil. Energy is stored in the core of the coil during this phase, and the diode does not conduct.  When the transistor

turns off, current flows from the output through the diode and the coil, driving the output negative. Feedback con- trol adjusts the external transistor’s timing to provide a regulated negative output voltage.

The MAX749’s unique control scheme combines the ultra-low supply current of pulse-skipping,  pulse-fre- quency modulation (PFM) converters with the high full- load efficiency characteristic of pulse-width modulation (PWM) converters. This control scheme allows the device to achieve high efficiency over a wide range of loads. The current-sense function and high operating frequency allow the use of tiny external components.

Switching control is accomplished  through the combi- nation of a current limit in the switch plus on- and off- time limits (Figure 2).

Once turned on, the transistor stays on until either:

- the maximum on-time one-shot turns it off

(8µs later), or

- the switch current reaches its limit (as determined by the current-sense resistor and the current comparator).

5

Digitally Adjustable LCD Bias Supply

+2V TO +6V INPUT

0.1 F

MAX749

Q            TRIG MINIMUM

OFF-TIME ONE-SHOT

CURRENT COMPARATOR

V+

140mV

RSENSE

22 F

S

MAXIMUM ON-TIME ONE-SHOT

TRIG             Q

FLIP-FLOP

Q R

DHI DLOW

VOLTAGE COMPARATOR

RBASE

470

Q1

ZTX750

L1

47 H

D1

1N5819

MAX749

REF

GND

6-BIT

CURRENT-OUTPUT DAC

FB                   RFB

CCOMP

22 F

30V

VOUT

(NEGATIVE)

Figure 2.   Switch-Mode Power-Supply Section Block Diagram

Once turned off, a one-shot holds the switch off for a minimum of 1µs, and the switch either stays off (if the output is in regulation), or turns on again (if the output is out of regulation).

With light loads, the transistor switches for one or more cycles and then turns off, much like a traditional PFM converter. With heavy loads, the transistor stays on until the switch current reaches the current limit; it then shuts off for 1µs, and immediately turns on again until the next time the switch current reaches its limit. This cycle repeats until the output is in regulation.

Output Voltage Control The output voltage is set using a single external resistor and the internal current-output DAC (Figure 1). The full- scale output voltage is set by selecting the feedback resistor, RFB. The output voltage is controlled from 33% to 100% of the full-scale output by an internal 64-step DAC/counter.

On power-up or after a reset, the counter sets the DAC

output to mid-range. Each rising edge of ADJ incre-

ments the DAC output. When incremented beyond full scale, the counter rolls over and sets the DAC to the minimum value. In this way, a single pulse applied  to ADJ increases the DAC set point by one step, and 63 pulses decrease the set point by one step.

Table 1 is the logic table for the CTRL and ADJ inputs, which control the internal DAC and counter. Figures 3-7 show various timing specifications and different ways of incrementing and resetting the DAC, and of placing it in the low-power standby mode. As long as the timing specifications  for ADJ and CTRL are observed, any sequence of operations can be implemented.

| Table 1. Input Truth Table ADJ | CTRL | RESULT |
| --- | --- | --- |
| Low | Low | Shut down |
| High | Low | Reset counter to mid-range. The device is not shut down. |
| X | High | On |
|  | High | Increment the counter |

6

Digitally Adjustable LCD Bias Supply

ADJ CTRL

tR                                                               tSD

MAX749

In Figure 3, the MAX749 is reset when it is taken out of shutdown, which sets the output at mid-scale. Figure 4 shows how to increment the counter. Figure 5 illustrates a reset without shutting the device down.

Figure 7 provides an example of a sequence of opera- tions: Starting from shutdown, the device is turned on, incremented, reset to mid-scale without being shut

SHUTDOWN  RESET                                  ON                                    SHUTDOWN

Figure 3.  Shutdown-Reset-On-Shutdown Sequence of Operation. The device is not shut down during reset.

ADJ CTRL

down, incremented again, and finally shut down.

Shutdown Mode When CTRL and ADJ are both low, the MAX749 is shut down (Table 1):  The internal reference and biasing cir- cuitry turn off, the output voltage drops to zero, and the supply current drops to 15µA. The MAX749 retains its DAC setting, simplifying software control.

Reset Mode If ADJ is high when CTRL is low, the DAC set point is reset to mid-scale and the MAX749 is not shut down.

tSH

tSL

HIGH

Mid-scale is 32 steps from the minimum, 31 steps from the maximum.

Figure 4.   Count-Up Operation

ADJ CTRL

tRS                                                    tRH

tR

ON                                          RESET                                       ON

Figure 5.  Reset Sequence without Shutdown. The device is not shut down during reset.

Design Procedure

and  Component Selection

Setting the Output Voltage The MAX749’s output voltage is set using an external resistor and the internal current-output DAC. The full- scale output voltage is set by selecting the feedback resistor RFB according to the formula:

-VOUT(MAX) = RFB x 20µA (Figure 1). The device is in regulation when VFB = 0V.

DAC Adjustment On power-up or after a reset, the counter sets the DAC output to mid-range, and -VOUT = RFB x 13.33µA. Each rising edge of ADJ increments the counter (and there- fore the DAC output) in the direction of -VOUT(MAX)  by one count. When incremented beyond -VOUT(MAX), the

ADJ CTRL

tRH

tR

ADJ CTRL

INCREMENT

RESET

INCREMENT

SHUTDOWN                 RESET                                ON

SHUTDOWN

ON                                            SHUTDOWN

Figure 6.  Reset Sequence with Shutdown

Figure 7.  Control Sequence Example (see Output Voltage

Control section)

7

Digitally Adjustable LCD Bias Supply

MAX749

+4.5V to +6V INPUT

V+

MAX749

0.1 F

CS DHI

22 F

RSENSE

Q1

SMD10P05L

Current-Sense Resistor The current-sense resistor limits the peak switch cur- rent to 140mV/RSENSE, where RSENSE is the value of the current-sense resistor, and 140mV is the typical cur- rent-sense comparator threshold (see V+ to CS Voltage in the Electrical Characteristics).

To maximize efficiency and reduce the size and cost of the external components, minimize the peak current. However, since the output current is a function of the peak

CTRL

ADJ

DLOW

L1

47 H

D1

1N5819

VOUT

current (Figures 9a-9e), the limit should not be set too low.

No calculations are required to choose the proper cur- rent-sense resistor; simply follow this two-step procedure:

1. Determine:

- the minimum input voltage, VIN(MIN),

VOUT(MIN)

GND

= -R1(13.33  A)

R1           R2

CCOMP

(NEGATIVE)

22 F

30V

- the maximum output voltage, VOUT(MAX), and

- the maximum output current, IOUT(MAX).

For example, assume that the output voltage must be adjustable to -24V (VOUT(MAX)  = -24V) at up to 30mA (IOUT(MAX)  = 30mA). The supply voltage ranges from

VOUT(MAX) = -(R1+R2)(13.33   A)

Figure 8.  Using a Potentiometer to Adjust the Output Voltage

counter rolls over and sets the DAC to -VOUT(MIN), where -VOUT(MIN)  = RFB x 6.66µA. In other words, a sin- gle rising edge of ADJ increments the DAC output by one, and 63 rising edges of ADJ decrement the DAC output by one.

Potentiometer Adjustment It is also possible to adjust the output voltage using a potentiometer instead of the internal DAC (Figure 8).  On power-up (V+ applied), the internal current source is set to mid-scale, or 13.33µA. Choose R1 and R2 with the fol- lowing equations:

R1 = -VOUT(MIN)/13.33µA

R2 = -VOUT(MAX)/13.33µA - R1.

Where the potentiometer can be varied from 0 (producing VOUT(MIN)) to R2 (producing VOUT(MAX)). Notice that ADJ is connected to ground, allowing the device to be shut down.

PWM Adjustment A positive pulse-width modulated (PWM) logic signal (e.g., from a microcontroller) can control the MAX749’s output voltage. Use the PWM signal to pull up the FB pin through a suitable resistor. An RC network on the PWM output would also be required. In this configura- tion, the longer the PWM signal remains high, the more negative the MAX749’s output will be driven.

4.75V to 6V (VIN(MIN) = 4.75V).

2. In Figures 9a-9e, locate the graph drawn for the appropriate  output voltage (which is either the desired output voltage or, if that is not shown, the graph for the nearest voltage more negative than the desired output). On this graph find the curve for the highest RSENSE (the lowest current limit) with an out- put current that is adequate at the lowest input voltage.

In this example, select the -24V output graph, Figure 9d. We then want a curve where IOUT is 30mA with a 4.75V input. The 0.3 RSENSE graph shows 25mA of output cur- rent with a 4.75V input, so we look next at the 0.25

RSENSE graph. It shows IOUT = 30mA for VIN = 4.75V and VOUT = -24V. Therefore select RSENSE = 0.25. This pro- vides a current limit in the range 440mA to 720mA.

Alternatively, a 0.2 sense resistor can be used. This gives a current limit in the range 550mA to 900mA, but enables over 40mA to be generated at -24V with input voltages down to 4.5V. A 0.2 resistor may be easier to obtain than an 0.25 resistor.

The theoretical design curves shown in Figures 9a-9e assume the minimum (worst-case) value for the current- limit comparator threshold. Having selected the cur- rent-sense resistor, the maximum current limit is given by 180mV/RSENSE. Use the maximum current-limit fig- ure when choosing the transistor, coil, and diode.

IRC (see Table 2) makes surface-mount resistors with pre- ferred values including: 0.1, 0.2, 0.3, 0.5, and 1.0.

8

Digitally Adjustable LCD Bias Supply

Choosing an Inductor Practical inductor values range from 22µH to 100µH, and 47µH is normally a good choice. Inductors with a

MAXIMUM OUTPUT CURRENT (mA)

100

VOUT

MAX749-Fig 11

= -15V

0.2

ferrite core or equivalent are recommended. The induc-

tor’s saturation current rating – the current at which the core begins to saturate and the inductance falls to 80% or 90% of its nominal value – should ideally equal the current limit (see Current-Sense Resistor section). However, because  the current is limited by  the MAX749, the inductor can safely be driven into satura- tion with only a slight impact on efficiency.

For highest efficiency, use a coil with low resistance, preferably under 300m. To minimize radiated noise, use a toroid, pot-core, or shielded inductor.

L = 47  H

80

60

40

20

0

2              3             4

0.25

MAX749

RSENSE (  )

0.3

0.5

1.0

5             6

INPUT VOLTAGE (V)

Figure 9c.   Maximum Output Current vs. Input Voltage, VOUT = -15V

250

200

VOUT = -5V L = 47 H

0.2

MAX749-Fig 9

0.25

60

50           VOUT = -24V L = 47 H

MAX749-Fig 12

0.2

150

100

MAXIMUM OUTPUT CURRENT (mA)

50

0

2

3              4              5             6

INPUT VOLTAGE (V)

RSENSE (  )

0.3

0.5

1.0

40

30

20

MAXIMUM OUTPUT CURRENT (mA)

10

0

2             3              4              5

INPUT VOLTAGE (V)

RSENSE (  )

0.25

0.3

0.5

1.0

MAX749-Fig 13

6

Figure 9a.  Maximum Output Current vs. Input Voltage, VOUT = -5V

Figure 9d.   Maximum Output Current vs. Input Voltage, VOUT = -24V

140

120

100

80

60

40

MAXIMUM OUTPUT CURRENT (mA)

20

VOUT = -12V L = 47  H

0.2

MAX-749-Fig10

0.25

RSENSE (  )

0.3

0.5

1.0

25

VOUT = -48V

20            L = 47 H

15

10

MAXIMUM OUTPUT CURRENT (mA)

5

0.2

0.25

RSENSE (  )

0.3

0.5

1.0

0

2              3              4             5              6

INPUT VOLTAGE (V)

0

2              3              4              5             6

INPUT VOLTAGE (V)

Figure 9b.   Maximum Output Current vs. Input Voltage, VOUT = -12V

Figure 9e.  Maximum Output Current vs. Input Voltage, VOUT = -48V

9

Digitally Adjustable LCD Bias Supply

MAX749

The Sumida CD54-470N (47µH, 720mA, 370m) is suit- able for a wide range of applications,  and the larger CD105-470N (47µH, 1.17A, 170m) permits higher cur- rent levels and efficiencies.

| Table 2. Component Suppliers SUPPLIER | PHONE | FAX |
| --- | --- | --- |
| INDUCTORS |  |  |
| Coiltronics | (305) 781-8900 | (305) 782-4163 |
| Gowanda | (716) 532-2234 | (716) 532-2702 |
| Sumida USA | (708) 956-0666 | (708) 956-0702 |
| Sumida Japan | 81-3-3607-511 | 81-3-3607-5428 |
| CAPACITORS |  |  |
| Kemet | (803) 963-6300 | (803) 963-6322 |
| Matsuo | (714) 969-2491 | (714) 960-6492 |
| Nichicon | (708) 843-7500 | (708) 843-2798 |
| Sprague | (603) 224-1961 | (603) 224-1430 |
| Sanyo USA | (619) 661-6322 |  |
| Sanyo Japan | 81-3-3837-6242 |  |
| United Chemi-Con | (714) 255-9500 | (714) 255-9400 |
| DIODES |  |  |
| Motorola | (800) 521-6274 |  |
| Nihon USA | (805) 867-2555 | (805) 867-2698 |
| Nihon Japan | 81-3-3494-7411 | 81-3-3494-7414 |
| POWER TRANSISTORS - MOSFETS |  |  |
| Harris | (407) 724-3739 | (407) 724-3937 |
| International Rectifier | (213) 772-2000 | (213) 772-9028 |
| Siliconix | (408) 988-8000 | (408) 727-5414 |
| POWER TRANSISTORS - PNP TRANSISTORS |  |  |
| Zetex USA | (516) 543-7100 | (516) 864-7630 |
| Zetex UK | 44 (61) 727 5105 | 44 (61) 627 5467 |
| CURRENT-SENSE RESISTORS |  |  |
| IRC | (512) 992-7900 | (512) 992-3377 |

Diode Selection The MAX749’s high switching frequency demands a high- speed rectifier. Schottky diodes such as the 1N5817-

1N5822 family are recommended. Choose a diode with an average current rating approximately equal to the peak current, as determined by 180mV/RSENSE and a break-

down voltage greater than V+ + I-VOUTMAXI.

External Switching Transistor The MAX749 can drive a PNP transistor or a P-channel logic-level MOSFET. The choice of a power switch is dictated by the input voltage range, cost, and efficiency.

MOSFETs provide the highest efficiency because they do not draw any DC gate-drive current (see Typical Operating Characteristics graphs). However, a gate- source voltage of several volts is needed to turn on a MOSFET, so a 5V or greater input supply is required (although this restriction may change as lower-thresh- old P-channel MOSFETs become available). PNP tran- sistors, meanwhile, may be used over the entire 2V to

6V operating voltage range of the MAX749.

When using a MOSFET, connect DHI and DLOW to its gate (see Typical Operating Circuit). When using a PNP transistor, connect DHI to its base, and connect a resis- tor between the base and DLOW (RBASE) (Figure 1). The PNP transistor is turned off quickly by the direct pull-up of DHI, and turned on by the base current provided through RBASE. This resistor limits the transistor’s base- drive current to (VIN - 140mV -  VBE)/RBASE, where VIN is the input voltage, 140mV is the drop across RSENSE, VBE is the transistor’s base-emitter voltage, and RBASE is the current-limiting resistor. For maximum efficiency, make RBASE as large as possible, but small enough so that the transistor is always driven into saturation.

Highest efficiency with a PNP transistor comes from using a device with a low collector-emitter saturation voltage and a high current gain. Use a fast-switching type. For example the Zetex ZTX792A has switching speeds of 40ns (tON) and 500ns (tOFF).

The transistor must have a collector-to-emitter (PNP) or drain-to-source (MOSFET) voltage rating greater than the input-to-output voltage differential (VIN - VOUT). In either case the transistor must have a current rating that exceeds the peak current set by the current-sense resistor.

PNP transistors are generally less expensive than P- channel MOSFETs. Table 2 lists some suppliers of switching transistors suitable for use with the MAX749.

Base Resistor The base resistor, RBASE in Figure 1, controls the amount of base current in the PNP transistor. A low value for RBASE increases base drive, which provides higher output cur- rents and compensates for lower input voltages, but decreases efficiency. Conversely, a high RBASE value increases efficiency but reduces the output capability, especially at low voltages. When using high-gain transis- tors, e.g. the Zetex ZTX750 or ZTX792, typical values for RBASE are in the 150 to 510 range, but will depend on the required input voltage range and output current (see Typical Operating Characteristics). Lower-gain transistors require lower values for RBASE and are less efficient. Larger RBASE values are suitable if less output power is required.

10

Digitally Adjustable LCD Bias Supply

Capacitors

Output Filter Capacitor A 22µF, 30V surface-mount (SMT) tantalum output filter capacitor  typically maintains 100mVp-p output ripple when generating -24V at 40mA from a 5V input. Smaller capacitors, down to 10µF, may be used for light loads in applications  that can tolerate higher output ripple. Surface-mount capacitors  are generally preferred because they lack the inductance and resistance of the leads of their through-hole equivalents.

Input Bypass Capacitor A 22µF tantalum capacitor  in parallel with a 0.1µF ceramic normally provides sufficient bypassing. Mount the 0.1µF capacitor very close to the IC. Larger capacitors may be needed if the incoming supply has high impedance.  Less bypass capacitance  is accept- able if the circuit is run off a low-impedance  supply. Begin prototyping with a large bypass capacitor; when the circuit is working, reduce the bypass to the smallest value that gives good results. Although bench power supplies have low impedance  at DC, they often have high impedance  at the frequencies used by switching DC-DC converters.

The effective series resistance (ESR) of both the bypass and filter capacitors affects efficiency. Best per- formance is obtained by doubling  up on the filter capacitors or using low-ESR types.

The smallest low-ESR SMT capacitors  currently avail- able are Sprague 595D series, which are about half the size of competing  products.  Sanyo OS-CON organic semiconductor through-hole capacitors also exhibit low ESR, and are especially useful when operation below

0°C is required. Table 2 lists the phone numbers of these and other manufacturers.

MAX749

Compensation Capacitor The high value of the feedback resistor makes the feed- back loop susceptible to phase lag if parasitic capaci- tance is present at the FB pin. To compensate for this, it may be necessary to connect a capacitor,  CCOMP, in parallel with RFB. Although CCOMP  is normally not required, the value of CCOMP  depends upon the value of RFB and on the individual circuit layout—typical val- ues range from 0pF to 220pF.

PC Layout and  Grounding Due to high current levels and fast switching wave- forms, proper PC board layout is essential. In particular, keep all leads short, especially the lead connected  to the FB pin and those connecting  Q1, L1, and D1 together. Mount the RFB resistor very close to the IC.

Use a star ground configuration:   Connect the ground lead of the input bypass capacitor, the output capaci- tor, and the inductor at a common point next to the GND pin of the MAX749. Additionally, connect the posi- tive lead of the input bypass capacitor as close as pos- sible to the V+ pin of the IC.

Chip Topography

0.070"

(0.1178mm)

V+                            V+    CS

ADJ

CTRL

DHI   0.808"

(0.2032mm)

DLOW

FB                            GND

TRANSISTOR COUNT: 521; SUBSTRATE CONNECTED TO GND.

11

Digitally Adjustable LCD Bias Supply

MAX749

Package Information

| 0° - 15° E D E1 A3 A A2 L A1 C e B1 eA B eB D1 |  | DIM | INCHES |  |  |  | MILLIMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MIN |  | MAX |  | MIN |  | MAX |  |
|  |  | A | – |  | 0.200 |  | – |  | 5.08 |  |
|  |  | A1 | 0.015 |  | – |  | 0.38 |  | – |  |
|  |  | A2 | 0.125 |  | 0.175 |  | 3.18 |  | 4.45 |  |
|  |  | A3 | 0.055 |  | 0.080 |  | 1.40 |  | 2.03 |  |
|  |  | B | 0.016 |  | 0.022 |  | 0.41 |  | 0.56 |  |
|  |  | B1 | 0.045 |  | 0.065 |  | 1.14 |  | 1.65 |  |
|  |  | C | 0.008 |  | 0.012 |  | 0.20 |  | 0.30 |  |
|  |  | D1 | 0.005 |  | 0.080 |  | 0.13 |  | 2.03 |  |
|  |  | E | 0.300 |  | 0.325 |  | 7.62 |  | 8.26 |  |
|  |  | E1 | 0.240 |  | 0.310 |  | 6.10 |  | 7.87 |  |
|  |  | e | 0.100 |  | – |  | 2.54 |  | – |  |
|  |  | eA | 0.300 |  | – |  | 7.62 |  | – |  |
|  |  | eB | – |  | 0.400 |  | – |  | 10.16 |  |
|  |  | L | 0.115 |  | 0.150 |  | 2.92 |  | 3.81 |  |
| Plastic DIP PLASTIC DUAL-IN-LINE PACKAGE (0.300 in.) | PKG. | DIM | PINS | INCHES |  |  |  | MILLIMETERS |  |  |
|  |  |  |  | MIN |  | MAX |  | MIN |  | MAX |
|  | P | D | 8 | 0.348 |  | 0.390 |  | 8.84 |  | 9.91 |
|  | P | D | 14 | 0.735 |  | 0.765 |  | 18.67 |  | 19.43 |
|  | P | D | 16 | 0.745 |  | 0.765 |  | 18.92 |  | 19.43 |
|  | P | D | 18 | 0.885 |  | 0.915 |  | 22.48 |  | 23.24 |
|  | P | D | 20 | 1.015 |  | 1.045 |  | 25.78 |  | 26.54 |
|  | N | D | 24 | 1.14 |  | 1.265 |  | 28.96 |  | 32.13 |
| 21-0043A |  |  |  |  |  |  |  |  |  |  |

DIM

INCHES            MILLIMETERS

D

0°-8° A

MIN

A      0.053

A1     0.004

B      0.014

C      0.007

E      0.150

MAX

0.069

0.010

0.019

0.010

0.157

MIN

1.35

0.10

0.35

0.19

3.80

MAX

1.75

0.25

0.49

0.25

4.00

0.101mm

0.004in.

e               0.050

1.27

e                              A1

B

H

C                   L                              L

0.228

0.016

0.244

0.050

5.80

0.40

6.20

1.27

INCHES       MILLIMETERS

Narrow SO

E     H                          SMALL-OUTLINE PACKAGE (0.150 in.)

DIM   PINS

D        8

D       14

D       16

MIN

0.189

0.337

0.386

MAX

0.197

0.344

0.394

MIN

4.80

8.55

9.80

MAX

5.00

8.75

10.00

21-0041A

12