19-4333; Rev 4; 12/05

Microprocessor Supervisory Circuits

General Description

The MAX690A/MAX692A/MAX802L/MAX802M/MAX805L reduce  the complexity  and number of components required for power-supply monitoring and battery-control functions in microprocessor  (µP) systems.  They signifi- cantly improve system reliability and accuracy compared to separate ICs or discrete components.

These parts provide four functions:

1)  A reset output during power-up, power-down,  and brownout conditions.

2)  Battery-backup  switching  for CMOS RAM, CMOS

µP, or other low-power logic.

3)  A reset pulse if the optional watchdog timer has not been toggled within 1.6sec.

4)  A 1.25V threshold detector for power-fail warning or low-battery detection, or to monitor a power supply other than +5V.

The parts differ in their reset-voltage threshold levels and reset outputs.  The MAX690A/MAX802L/MAX805L generate a reset pulse when the supply voltage drops below 4.65V, and the MAX692A/MAX802M generate a reset below 4.40V.  The MAX802L/MAX802M guaran- tee power-fail accuracies to ±2%.  The MAX805L is the same as the MAX690A except that RESET is provided instead of RESET.

All parts are available in 8-pin DIP and SO packages. The MAX690A/MAX802L are pin compatible  with the MAX690 and MAX694.  The MAX692A/MAX802M are pin compatible with the MAX692.

Applications

Battery-Powered Computers and Controllers

Intelligent Instruments

Automotive Systems

Critical µP Power Monitoring

Typical Operating Circuit

UNREGULATED DC   REGULATED +5V

VCC

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

Features

♦ Precision Supply-Voltage Monitor:

4.65V for MAX690A/MAX802L/MAX805L

4.40V for MAX692A/MAX802M

♦ Reset Time Delay – 200ms

♦ Watchdog Timer – 1.6sec Timeout

♦ Battery-Backup Power Switching

♦ 200µA Quiescent Supply Current

♦ 50nA Quiescent Supply Current in Battery- Backup Mode

♦ Voltage Monitor for Power-Fail or Low-Battery

Warning

♦ Power-Fail Accuracy Guaranteed to ±2% (MAX802L/M)

♦ Guaranteed RESET Assertion to VCC = 1V

♦ 8-Pin SO and DIP Packages

Ordering Information

PART TEMP. RANGE PIN-PACKAGE

MAX690ACPA 0°C to +70°C 8 Plastic DIP

MAX690ACSA 0°C to +70°C 8 SO

MAX690AC/D 0°C to +70°C Dice*

MAX690AEPA -40°C to +85°C 8 Plastic DIP

MAX690AESA -40°C to +85°C 8 SO

MAX690AMJA -55°C to +125°C 8 CERDIP**

Ordering Information  continued on last page.

* Dice are specified at TA = +25°C

** Contact factory for availability and processing to MIL-STD-883. Devices in PDIP and SO packages are available in both leaded and lead-free packaging. Specify lead free by adding  the + symbol at the end of the part number when ordering. Lead free not available for CERDIP package.

Pin  Configurations

TOP VIEW

O.1F      R1

VCC

PFI

RESET PFO

RESET NMI  P

V OUT      1

V CC     2

MAX690A MAX692A

8    V BATT

7    RESET (RESET)

R2            3.6V

WDI

BUS

I/O LINE   GND

GND    3

MAX802L

6    WDI

LITHIUM

VBATT

VOUT

MAX802M

BATTERY

MAX690A MAX802L

GND

VCC

CMOS RAM

GND

PFI    4

MAX805L

DIP/SO

5    PFO

(   ) ARE FOR MAX805L ONLY.

Maxim Integrated Products   1

For pricing, delivery, and ordering information, please contact Maxim/Dallas Direct! at

1-888-629-4642, or visit Maxim’s website at www.maxim-ic.com.

Microprocessor Supervisory Circuits

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

ABSOLUTE MAXIMUM RATINGS

Terminal Voltage (with respect to GND)

VCC   ...............................................  -0.3V to 6.0V VBATT ..............................................  -0.3V to 6.0V All Other Inputs (Note 1)  ................  -0.3V to (VCC + 0.3V)

Input Current

VCC ...................................................... 200mA VBATT  ..................................................... 50mA GND ...................................................... 20mA

Output Current

VOUT ...................   Short-Circuit  Protected for up to 10sec

All Other Outputs  ........................................ 20mA

Rate of Rise, VCC, VBATT  ................................  100V/µs

Continuous Power Dissipation

Plastic DIP (derate 9.09mW/°C above +70°C)  ........  727mW SO (derate 5.88mW/°C above +70°C)   ................  471mW CERDIP (derate 8.00mW/°C above +70°C)  ...........  640mW

Operating  Temperature  Ranges:

MAX69_AC_ _, MAX80_ _ C_ _ ..................  0°C to +70°C MAX69_AE_ _, MAX80_ _ E_ _ ................  -40°C to +85°C MAX69_AMJA, MAX805LMJA ................  -55°C to +125°C Storage Temperature  Range ..................  -65°C to +160°C Lead Temperature  (soldering,  10sec)  ..................  +300°C

Note 1:  The input voltage limits on PFI and WDI may be exceeded if the current into these pins is limited to less than 10mA.

Stresses beyond those listed under "Absolute Maximum Ratings" may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions  beyond those indicated  in the operational sections of the specifications  is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

ELECTRICAL CHARACTERISTICS

(V CC  =  4.75V  to  5.5V  for  MAX690A/MAX802L/MAX805L,  V CC  =  4.5V  to  5.5V  for  MAX692A/MAX802M,  V BATT  =  2.8V, TA = TMIN to TMAX, unless otherwise noted.)

BATT

OUT

CC                     CC

| CC RT PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| Operating Voltage Range, VCC, VBATT (Note 2) |  | MAX69_AC, MAX802_C |  | 1.0 5.5 | V |
|  |  | MAX805LC |  | 1.1 5.5 |  |
|  |  | MAX69_AE/M, MAX80_ _E |  | 1.2 5.5 |  |
| Supply Current (Excluding IOUT) | ISUPPLY | MAX69_AC, MAX802_C |  | 200 350 | µA |
|  |  | MAX69_AE/M, MAX802_E, MAX805LE/M |  | 200 500 |  |
| ISUPPLY in Battery-Backup Mode (Excluding IOUT) |  | VCC = 0V, VBATT = 2.8V | TA = +25°C | 0.05 1.0 | µA |
|  |  |  | TA = TMIN to TMAX | 5.0 |  |
| V Standby Current (Note 3) |  | 5.5V > VCC> VBATT +0.2V | TA = +25°C | -0.1 0.02 | µA |
|  |  |  | TA = TMIN to TMAX | -1.0 0.02 |  |
| VOUT Output |  | IOUT = 5mA |  | VCC - 0.05 VCC - 0.025 | V |
|  |  | I = 50mA |  | V - 0.5 V - 0.25 |  |
| VOUT in Battery-Backup Mode |  | IOUT = 250µA, VCC < VBATT - 0.2V |  | VBATT - 0.1 VBATT - 0.02 | V |
| Battery Switch Threshold, VCC to VBATT |  | V < V | Power-up | 20 | mV |
|  |  |  | Power-down | -20 |  |
| Battery Switchover Hysteresis |  |  |  | 40 | mV |
| Reset Threshold | VRT | MAX690A, MAX802L, MAX805L |  | 4.50 4.65 4.75 | V |
|  |  | MAX692A, MAX802M |  | 4.25 4.40 4.50 |  |
|  |  | MAX802L, TA = +25°C, VCC falling |  | 4.55 4.70 |  |
|  |  | MAX802M, TA = +25°C, VCC falling |  | 4.30 4.45 |  |
| Reset Threshold Hysteresis |  |  |  | 40 | mV |
| Reset Pulse Width | tRS |  |  | 140 200 280 | ms |
| RESET Output Voltage |  | ISOURCE = 800µA |  | VCC - 1.5 | V |
|  |  | ISINK = 3.2mA |  | 0.4 |  |
|  |  | MAX69_AC, MAX802_C, VCC = 1.0V ISINK = 50µA |  | 0.3 |  |
|  |  | MAX69_AE/M, MAX802_E, VCC = 1.2V, ISINK = 100µA |  | 0.3 |  |

2

Microprocessor Supervisory Circuits

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

ELECTRICAL CHARACTERISTICS (continued)

(V CC  =  4.75V  to  5.5V  for  MAX690A/MAX802L/MAX805L,  V CC  =  4.5V  to  5.5V  for  MAX692A/MAX802M,  V BATT  =  2.8V, TA = TMIN to TMAX, unless otherwise noted.)

| SOURCE CC PARAMETER | SYMBOL | CONDITIONS |  | MIN TYP MAX | UNITS |
| --- | --- | --- | --- | --- | --- |
| RESET Output Voltage |  | MAX805LC, ISOURCE = 4µA, VCC = 1.1V |  | 0.8 | V |
|  |  | MAX805LE/M, ISOURCE = 4µA, VCC = 1.2V |  | 0.9 |  |
|  |  | MAX805L, I = 800µA |  | V - 1.5 |  |
|  |  | MAX805L, ISINK = 3.2mA |  | 0.4 |  |
| Watchdog Timeout | tWD |  |  | 1.00 1.60 2.25 | sec |
| WDI Pulse Width | tWP | VIL = 0.4V, VIH = (0.8) (VCC) |  | 50 | ns |
| WDI Input Threshold (Note 4) |  | VCC = 5V | Logic low | 0.8 | V |
|  |  |  | Logic high | 3.5 |  |
| WDI Input Current |  | WDI = VCC |  | 50 150 | µA |
|  |  | WDI = 0V |  | -150 -50 |  |
| PFI Input Threshold |  | MAX69_A, MAX805L, VCC = 5V |  | 1.20 1.25 1.30 | V |
|  |  | MAX802_C/E, VCC = 5V |  | 1.225 1.250 1.275 |  |
| PFI Input Current |  |  |  | -25 0.01 25 | nA |
| PFO Output Voltage |  | ISOURCE = 800µA |  | VCC - 1.5 | V |
|  |  | ISINK = 3.2mA |  | 0.4 |  |

Note 2:  Either VCC or VBATT can go to 0V, if the other is greater than 2.0V.

Note 3:  "-" = battery-charging current, "+" = battery-discharging current.

Note 4:  WDI is guaranteed to be in an intermediate, non-logic  level state if WDI is floating and VCC is in the operating  voltage range.

WDI is internally biased to 35% of VCC with an input impedance of 50k.

3

Microprocessor Supervisory Circuits

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

VOUT (V)

Typical Operating Characteristics

5.00

4.95

OUTPUT VOLTAGE

vs. LOAD CURRENT

VCC = +5V VBATT = +2.8V TA = +25°C

2.80

2.78

OUTPUT VOLTAGE

vs. LOAD CURRENT

VCC = 0V VBATT = +2.8V TA = +25°C

VCC

MAX690A

RESET OUTPUT VOLTAGE

vs. SUPPLY VOLTAGE

VBATT  = OV TA = +25°C

1V/div

+5V

4.90

4.85

VOUT (V)

2.76

2.74

0V

VCC

VCC                   2k

1V/div

+5V

4.80

SLOPE = 5

2.72

SLOPE = 80

RESET

RESET

GND

RESET

330pF

4.75

0

10         20

30        40         50

2.70

0

0.2

0.4

0.6

0.8

0V

1.0

500ms/div

IOUT (mA)

IOUT (mA)

MAX690A

RESET RESPONSE TIME

POWER-FAIL COMPARATOR RESPONSE TIME

POWER-FAIL COMPARATOR RESPONSE TIME

+5V VCC

+4V

+5V

+5V

PFI

+5V

PFO

VCC = +5V TA = +25°C

+5V

VCC

VCC                   10k

1V/div

TA = +25°C

PFO

PFI

+1.25V

PFO      1k

30pF

+0V

PFO

0V

+1.25V

30pF          1k

+3V

RESET

RESET GND

RESET

30pF

+1.30V PFI

0V

VCC = +5V TA = +25°C

+1.2V

PFI

+1.20V

+1.3V

2s/div

400ns/div

400ns/div

VCC

MAX805L

RESET OUTPUT VOLTAGE

vs. SUPPLY VOLTAGE

1V/div

+5V

+5V

MAX805L

RESET RESPONSE TIME

VCC                                                                                                            +4V

VCC

RESET

0V RESET

0V

VCC

VBATT

10k

RESET

GND

500ms/div

330 pF

1V/div

+5V         0V

0V

GN  D

10k

330pF

2s/div

+4V

4

Microprocessor Supervisory Circuits

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

Pin  Description

| PIN |  | NAME | FUNCTION |
| --- | --- | --- | --- |
| MAX690A/MAX692A MAX802L/MAX802M | MAX805L |  |  |
| 1 | 1 | VOUT | Supply Output for CMOS RAM. When VCC is above the reset threshold, VOUT connects to VCC through a P-channel MOSFET switch. When VCC is below the reset threshold, the higher of VCC or VBATT will be connected to VOUT. |
| 2 | 2 | VCC | +5V Supply Input |
| 3 | 3 | GND | Ground |
| 4 | 4 | PFI | Power-Fail Comparator Input. When PFI is less than 1.25V, PFO goes low. Connect PFI to GND or VCC when not used. |
| 5 | 5 | PFO | Power-Fail Output. When PFI is less than 1.25V, PFO goes low; otherwise PFO stays high. |
| 6 | 6 | WDI | Watchdog Input. If WDI remains high or low for 1.6sec, the internal watchdog timer runs out and reset is triggered. If WDI is left floating or connected to a high-impedance three-state buffer, the watchdog feature is disabled. The internal watchdog timer clears whenever reset is asserted, WDI is three-stated, or WDI sees a rising or falling edge. |
| 7 |  | RESET | Reset Output. Whenever RESET is triggered, it pulses low for 200ms. It stays low when VCC is below the reset threshold (4.65V in the MAX690A/MAX802L and 4.4V in the MAX692A/MAX802M) and remains low for 200ms after VCC rises above the reset threshold. A watchdog timeout also triggers RESET. |
| – | 7 | RESET | Active-High Reset Output is the inverse of RESET. When RESET is asserted, the RESET output voltage = VCC or VBATT, whichever is higher. |
| 8 | 8 | VBATT | Backup-Battery Input. When VCC falls below the reset threshold, VBATT will be switched to VOUT if VBATT is 20mV greater than VCC. When VCC rises to 20mV above VBATT, VOUT will be reconnected to VCC. The 40mV hysteresis prevents repeated switching if VCC falls slowly. |

5

Microprocessor Supervisory Circuits

VBATT

VCC

BATTERY-SWITCHOVER CIRCUITRY

VOUT

VCC

+5V

0V

RESET GENERATOR

RESET (RESET)

VOUT

3.0V

0V

tRS

WDI

PFI

3.5V

0.8V

1.25V

WATCHDOG TIMER

MAX690A MAX692A MAX802L MAX802M MAX805L

PFO

+5V

RESET

0V

+5V (RESET)

0V

+5V PFO

0V

3.0V

1.25V

(  ) ARE FOR MAX805L ONLY.                     GND

(  ) ARE FOR MAX805L ONLY.

VBATT = PFI = 3.0V IOUT = 0mA

Figure 1.  Block Diagram

Detailed Description

Reset Output A microprocessor’s  (µP’s) reset input starts the µP in a known state. When the µP is in an unknown state, it should

be held in reset.  The MAX690A/MAX692A/MAX802L/

MAX802M assert reset during power-up and prevent code execution errors during power-down or brownout condi- tions.

On power-up, once VCC reaches 1V, RESET is guaran- teed to be a logic low.   As VCC rises, RESET remains low.  When VCC exceeds the reset threshold, an internal timer keeps RESET low for a time equal to the reset pulse width; after this interval, RESET goes high (Figure

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

2).   If a brownout condition occurs (if VCC dips below the reset threshold), RESET is triggered.   Each time RESET is triggered, it stays low for the reset pulse width interval.  Any time VCC goes below the reset threshold, the internal timer restarts the pulse.  If a brownout con- dition interrupts a previously initiated reset pulse, the reset pulse continues for another 200ms.   On power- down, once VCC goes below the threshold, RESET is guaranteed to be logic low until VCC droops below 1V.

RESET is also triggered  by a watchdog  timeout.   If a high or low is continuously applied  to the WDI pin for

1.6sec, RESET pulses low.  As long as RESET is assert-

Figure 2.  Timing Diagram

ed, the watchdog  timer remains clear.   When RESET comes high, the watchdog resumes timing and must be serviced  within 1.6sec.   If WDI is tied high or low, a RESET pulse is triggered every 1.8sec (tWD plus tRS).

The MAX805L active-high RESET output is the inverse of the MAX690A/MAX692A/MAX802L/MAX802M RESET output, and is guaranteed to be valid with VCC down to

1.1V.  Some µPs, such as Intel’s 80C51, require an active-high reset pulse.

Watchdog Input

The watchdog  circuit monitors the µP’s activity.   If the

µP does not toggle  the watchdog  input (WDI) within

1.6sec, a reset pulse is triggered.   The internal 1.6sec timer is cleared by either a reset pulse or by open cir- cuiting the WDI input.   As long as reset is asserted or the WDI input is open circuited,  the timer remains cleared  and does not count.   As soon as reset is released or WDI is driven high or low, the timer starts counting.  It can detect pulses as short as 50ns.

Power-Fail Comparator The PFI input is compared  to an internal 1.25V refer- ence.   If PFI is less than 1.25V, PFO goes low.   The

power-fail comparator is intended for use as an under-

voltage detector  to signal a failing power supply;  it need not be dedicated  to this function though, as it is

6

Microprocessor Supervisory Circuits

VBATT                                                                                                                                       VCC

+5V

SW1

SW2

D1      D2

SUBSTRATE

SW3

SW4

VCC                                   VOUT

MAX690A MAX802L

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

TO STATIC RAM

MAX690A

MAX802L  MAX805L         D3

VBATT

MAX805L

RESET (RESET)

TO P

| MAX692A MAX802M CONDITION | SW1/SW2 | SW3/SW4 |
| --- | --- | --- |
| VCC > Reset Threshold | Open | Closed |
| VCC < Reset Threshold and VCC > VBATT | Open | Closed |
| VCC < Reset Threshold and VCC < VBATT | Closed | Open |

VOUT

+

0.1F

GND

( ) ARE FOR MAX805L ONLY.

RESET THRESHOLD  = 4.65V  IN MAX690A/MAX802L/MAX805L. RESET THRESHOLD  = 4.4V IN MAX692A/MAX802M

Figure 3.  Backup-Battery Switchover Block Diagram

completely separate from the rest of the circuitry.   The external voltage divider drives PFI to sense the unregu-

lated DC input to the +5V regulator  (see Typical Operating  Circuit).   The voltage-divider  ratio can be chosen such that the voltage at PFI falls below 1.25V just before the +5V regulator drops out.  PFO then trig- gers an interrupt which signals the µP to prepare  for power-down.

To conserve backup-battery  power, the power-fail detector  comparator  is turned off and PFO is forced low when VBATT connects to VOUT.

Backup-Battery Switchover In the event of a brownout or power failure, it may be necessary to preserve the contents of RAM.  With a

backup battery installed at VBATT, the devices automati- cally switch RAM to backup power when VCC fails.

As long as VCC exceeds the reset threshold, VOUT connects to VCC through a 5 PMOS power switch.  Once VCC falls below the reset threshold, VCC or VBATT (whichever is high- er) switches to VOUT.  Unlike the MAX690/MAX692, the MAX690A/MAX692A/MAX802L/MAX802M/MAX805L  don’t

always connect VBATT to VOUT when VBATT is greater than VCC. VBATT connects to VOUT (through an 80 switch) only when VCC is below the reset threshold  and VBATT is greater than VCC.

Figure 4.  Using a SuperCap  as a Backup  Power Source with a

MAX690A/MAX802L/MAX805L and a +5V ±5% Supply

When VCC exceeds the reset threshold, it is connected to the MAX690A/MAX692A/MAX802L/MAX802M/MAX805L substrate, regardless of the voltage applied  to VBATT (Figure 3).  During this time, the diode (D1) between VBATT and the substrate will conduct current from VBATT to VCC if VBATT is 0.6V or greater than VCC.

Table 1.   Input and Output Status in Battery-Backup Mode

| BATT SIGNAL | STATUS |
| --- | --- |
| VCC | Disconnected from VOUT |
| VOUT | Connected to VBATT through an internal 80Ω PMOS switch |
| V | Connected to VOUT. Current drawn from the battery is less than 1µA, as long as VCC < VBATT - 1V. |
| PFI | Power-fail comparator is disabled. |
| PFO | Logic low |
| RESET | Logic low |
| RESET | Logic high (MAX805L only) |
| WDI | Watchdog timer is disabled |

7

Microprocessor Supervisory Circuits

+5V

VIN

+5V

VCC                                   VOUT

R1

TO STATIC RAM

VCC

100k

+

0.1F

VBATT

MAX692A MAX802M

GND

RESET

TO P

R2                                                        R3

C1*

PFI

PFO

MAX690

MAX692A MAX802L MAX802M MAX805L

GND

TO P

*OPTIONAL

Figure 5.  Using a SuperCap™ as a Backup  Power Source with the MAX692A/MAX802M  and a +5V ±10% Supply

+5V

PFO

When VBATT connects to VOUT, backup mode is activat- ed and the internal circuitry is powered from the battery (Table 1).   When VCC is just below VBATT, the current

0V

0V

    R 2      

VL   VTRIP   VH

VIN

drawn from VBATT is typically 30µA.  When VCC drops to

VTRIP   1.25/               

more than 1V below VBATT, the internal switchover com- parator shuts off and the supply current falls to less

VIH   1.25/

 R 2   R 2 

    R2 || R3     

 R   R   || R   

than 1µA.

1      2       3

VL  1.25   5  1.25   1.25

Applications Information

Using a SuperCap™ as  a Backup

Power Source

SuperCaps are capacitors  with extremely high capaci- tance values, on the order of 0.1F.  Figure 4 shows a SuperCap used as a backup power source.  Do not allow the SuperCap’s voltage to exceed the maximum reset threshold by more than 0.6V.  In Figure 4’s circuit, the SuperCap rapidly charges to within a diode drop of VCC. However, after a long time, the diode leakage current will pull the SuperCap voltage up to VCC.  When using a SuperCap with the MAX690A/MAX802L/MAX805L, VCC may not exceed 4.75V + 0.6V = 5.35V.

Use the SuperCap circuit of Figure 5 with a MAX692A or MAX802M and  a ±10%  supply.   This circuit ensures that the SuperCap  only charges  to VCC  -

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

0.5V.   At the maximum VCC  of 5.5V, the SuperCap charges  up to 5.0V, only 0.5V above the maximum reset threshold—well within the requisite 0.6V.

R1               R3            R2

Figure 6.  Adding  Hysteresis to the Power-Fail Comparator

Allowable Backup Power-Source

Batteries

Lithium batteries work very well as backup batteries due to very low self-discharge  rates and high energy density. Single lithium batteries with open-circuit voltages of 3.0V to

3.6V are ideal.   Any battery with an open-circuit  voltage less than the minimum reset threshold plus 0.3V can be connected   directly   to   the   V BATT   input   of   the MAX690A/MAX692A/MAX802L/MAX802M/MAX805L  with

Table 2.  Allowable Backup-Battery Voltages

| (see Using a SuperCap as a Backup Power Source PART NO. | MAXIMUM BACKUP-BATTERY VOLTAGE (V) |
| --- | --- |
| MAX690A/ MAX802L/MAX805L | 4.80 |
| MAX692A/ MAX802M | 4.55 |

section for use with a SuperCap)

™SuperCap is a trademark of Baknor Industries.

8

Microprocessor Supervisory Circuits

no additional circuitry (see the Typical Operating Circuit). However, batteries with open-circuit  voltages that are greater cannot be used for backup, as current is sourced into the substrate through the diode (D1 in Figure 3) when VCC is close to the reset threshold.

Operation

Without a Backup Power Source

If a backup  power source is not used, ground  VBATT and connect  VOUT to VCC.  Since there is no need to switch over to any backup  power source, VOUT does not need to be switched.   A direct connection  to VCC eliminates any voltage drops across the switch which may  push VOUT below VCC.

Replacing the Backup Battery

The backup battery can be removed while VCC remains valid, without danger of triggering  RESET/RESET. As long as VCC stays above the reset threshold, battery- backup  mode cannot be entered.  In other switchover ICs where battery-backup  mode is entered whenever VBATT gets close to VCC, an unconnected  VBATT pin

+5V

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

accumulates    leakage    charge    and    triggers

RESET/RESET in error.

Adding Hysteresis to the

Power-Fail Comparator

Hysteresis adds a noise margin to the power-fail com- parator and prevents repeated triggering of PFO when VIN is close to its trip point.  Figure 6 shows how to add hysteresis to the power-fail comparator.   Select the ratio of R1 and R2 such that PFI sees 1.25V when VIN falls to its trip point (VTRIP).  R3 adds the hysteresis.  It will typically be an order of magnitude greater than R1 or R2 (about 10 times either R1 or R2). The current through R1 and R2 should be at least 1µA to ensure that the 25nA (max) PFI input current does not shift the trip point.  R3 should be larger than 10k so it does not

load down the PFO pin.  Capacitor C1 adds additional

noise rejection.

Monitoring a Negative Voltage The power-fail comparator  can be used to monitor a negative supply rail using the circuit of Figure 7.  When

the negative rail is good  (a negative voltage of large

magnitude),  PFO is low.   When the negative rail is degraded  (a negative voltage of lesser magnitude), PFO goes high.   This circuit’s accuracy  is affected by

R1

R2

+5V

PFI

V-

VCC

MAX690A MAX692A MAX802L MAX802M MAX805L

GND

PFO

the PFI threshold tolerance, the VCC line, and the resis- tors.

Interfacing to µPs  with

Bidirectional Reset Pins

µPs with bidirectional  reset pins, such as the Motorola

68HC11 series, can contend with the MAX690A/MAX692A/MAX802L/MAX802M RESET out- put.   If, for example, the RESET output is driven high and the µP wants to pull it low, indeterminate logic lev- els may result.  To correct this, connect a 4.7k resis- tor between the RESET output and the µP reset I/O, as in Figure 8.  Buffer the RESET output to other system components

PFO

0V

VTRIP                                        0V V-

5  1.25     1.25 -  VTRIP

NOTER:1 V     IS NERG2 ATIVE

TRIP

Figure 7. Monitoring a Negative Voltage

9

Microprocessor Supervisory Circuits

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

BUFFERED RESET TO OTHER SYSTEM COMPONENTS

VCC

| 4.7k |  |  |  |
| --- | --- | --- | --- |
| VCC MAX690A MAX692A MAX802L R MAX802M GND |  |  |  |
|  |  | ESET |  |

RESET

GND

Figure 8.  Interfacing  to µPs with Bidirectional Reset I/O

10

Microprocessor Supervisory Circuits

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

                                                                                                      P Supervisory Circuits

Minimum    Nominal

Nominal       Reset          Watchdog                                    Power-

Reset           Pulse          Timeout     Backup-   CE -        Fail           Manual-  Watch-    Low-         Active-    Battery- Part                     Threshold    Width          Period        Battery     Write       Com-        Reset      dog         Line          High        On Number                (V)                (ms)            (sec)          Switch     Protect   parator     Input       Output    Output      Reset      Output

MAX690A/692A     4.65/4.40          140                 1.6                  ✔                             ✔

MAX691A/693A     4.65/4.40          140/adj.          1.6/adj.           ✔             ✔/10ns      ✔                             ✔            ✔              ✔            ✔ MAX696                  Adj.                  35/adj.            1.6/adj.           ✔                             ✔                             ✔            ✔              ✔            ✔ MAX697                  Adj.                  35/adj.            1.6/adj.                                ✔            ✔                             ✔            ✔              ✔

MAX700                  4.65/adj.           200                 -                                                                                   ✔                                             ✔

MAX703/704           4.65/4.40          140                 -                      ✔                             ✔              ✔

MAX705/706           4.65/4.40          140                 1.6                                                          ✔              ✔            ✔

MAX706P                2.63                  140                 1.6                                                          ✔              ✔            ✔                             ✔

MAX706R/S/T         2.63/2.93/         140                 1.6                                                          ✔              ✔            ✔

3.08

MAX707/708           4.65/4.40          140                 -                                                              ✔              ✔                                             ✔

MAX708R/S/T         2.63/2.93/         140                 -                                                              ✔              ✔                                             ✔

3.08

MAX709L/M/          4.65/4.40/         140                 -

R/S/T                        2.63/2.93/3.08

| MAX791 | 4.65 | 140 | 1 | ✔ | ✔/10ns | ✔ | ✔ | ✔ | ✔ | ✔ | ✔ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MAX792L/M/ | 4.65/4.40/ | 140 | 1 |  | ✔/10ns | ✔ | ✔ | ✔ | ✔ | ✔ |  |
| R/S/T 2.63/2.93/3.08 |  |  |  |  |  |  |  |  |  |  |  |
| MAX800L/M | 4.60/4.40 | 140 | 1.6/adj. | ✔ | ✔/10ns | ✔/2% |  | ✔ | ✔ | ✔ | ✔ |
| MAX802L/M | 4.60/4.40 | 140 | 1.6 | ✔ |  | ✔/2% |  |  |  |  |  |
| MAX805L | 4.65 | 140 | 1.6 | ✔ |  | ✔ |  |  |  | ✔ |  |
| MAX813L | 4.65 | 140 | 1.6 |  |  | ✔ | ✔ | ✔ |  | ✔ |  |
| MAX820L/M/ | 4.65/4.40/ | 140 | 1 |  | ✔/10ns | ✔/2% | ✔ | ✔ | ✔ | ✔ |  |
| R/S/T 2.63/2.93/3.08 |  |  |  |  |  |  |  |  |  |  |  |
| MAX1232 | 4.37/4.62 | 250 | 0.15/0.60/1.2 |  |  |  | ✔ |  |  | ✔ |  |
| MAX1259 | - | - | - | ✔ |  | ✔ |  |  |  |  |  |

11

Microprocessor Supervisory Circuits

MAX690A/MAX692A/MAX802L/MAX802M/MAX805L

Ordering Information (continued) PART TEMP. RANGE PIN-PACKAGE

MAX692ACPA 0°C to +70°C 8 Plastic DIP

MAX692ACSA 0°C to +70°C 8 SO

MAX692AC/D 0°C to +70°C Dice*

MAX692AEPA -40°C to +85°C 8 Plastic DIP

MAX692AESA -40°C to +85°C 8 SO

MAX692AMJA -55°C to +125°C 8 CERDIP**

MAX802LCPA 0°C to +70°C 8 Plastic DIP

MAX802LCSA 0°C to +70°C 8 SO

MAX802LEPA -40°C to +85°C 8 Plastic DIP

MAX802LESA -40°C to +85°C 8 SO

MAX802MCPA 0°C to +70°C 8 Plastic DIP

MAX802MCSA 0°C to +70°C 8 SO

MAX802MEPA -40°C to +85°C 8 Plastic DIP

MAX802MESA -40°C to +85°C 8 SO

MAX805LCPA 0°C to +70°C 8 Plastic DIP

MAX805LCSA 0°C to +70°C 8 SO

MAX805LC/D 0°C to +70°C Dice*

MAX805LEPA -40°C to +85°C 8 Plastic DIP

MAX805LESA -40°C to +85°C 8 SO

MAX805LMJA -55°C to +125°C 8 CERDIP**

Chip Topography

V CC          V OUT

V BATT

RESET  (RESET)

0.061"

WDI

(1.55mm)

GND

PFI

PFO

0.078"    (1.98mm)

(  ) ARE FOR MAX805L ONLY. TRANSISTOR COUNT: 573;

SUBSTRATE MUST BE LEFT UNCONNECTED.

* Dice are specified at TA = +25°C.

**Contact factory for availability and processing to MIL-STD-883. Devices in PDIP and SO packages are available in both leaded and lead-free packaging. Specify lead free by adding  the + symbol at the end of the part number when ordering. Lead free not available for CERDIP package.

Maxim cannot assume responsibility  for use of any circuitry  other than circuitry  entirely embodied  in a Maxim product.  No circuit  patent licenses  are implied. Maxim reserves the right to change the circuitry and specifications  without notice at any time.

12                                  Maxim Integrated Products, 120  San  Gabriel Drive, Sunnyvale, CA  94086 (408) 737-7600

©  2005 Maxim Integrated  Products               Printed USA                           is a registered  trademark  of Maxim Integrated  Products, Inc.